RingMon Ver 1.0c

Copyright(c) 1990 .... All rights reserved

Written by

Gary L. Bouchard

SysOp

The Computer Store BBS

St. Augustine, Florida

(904)797-4824

RingMon is a simple TSR program that allows you to monitor your modem

for a ring detect, and alert you to the fact that someone is calling.

The idea for RingMon came from Pcboard ver 14.5, and the ability that

the program has of allowing you on-line with the modem on hook, and

having Pcboard alert you to a caller. There are many times that I might

have to take the board off-line to do non-critical housekeeping, but

would like the BBS to be available to users. I don't have a telephone

connected to my BBS modem, so the only way I could tell if a caller was

trying to get on was the Ring Detect light. If I didn't happen to see the

Ring detect lite flashing I never knew that was a caller.

This program is 7k, and terminates to monitor the Ring Detect of your modem,

alerting you to a caller with an audible sound thru your PC speaker, just

like PCBoard. To be able to remove the program from memory if you desire, you

may use TurboPower's Mark.com and Release.com programs set into your

board.bat file or leave it in until you reboot. I do recommend that you

remove the program from memory if your going to be working with a

program that uses the comm ports, just so theres no interference with

your application. Mark & Release should come in very handy for that.

The program will only install once per session to prevent multiple

copies in memory, so that should help. I am working on an Assembler

version that should make this considerably smaller, to be released

in a few weeks. The program will continue to ring your speaker until

the caller either hangs up, or a change occurrs in DSR status on

your modem (A ready to recieve signal). This usually happens when the

modem asnwers the phone, and gives a Clear to send signal.

Once the change is made the program goes dormant again until next time.

RingMon was written in Borlands Turbo Pascal ver 5.5 on an IBM PS/2

Model 60 with 3 megs RAM and 230megs of disk.

Special Thanks to Joe Agrella, SysOp of Trade80 BBS in Fort Lauderdale,

Florida for letting me pick his brain on Turbo Pascal.

If you have any comments or suggestions or problem/bug reports, please

call my BBS listed at the top of this page. You can always find the

latest versions of my programs there free of charge...which brings up

another point...

RingMon is a FREE program. There is no charge whatsoever involved with

the use of this program. I maintain the rights to the source, and the

program cannot be modified for redistribution without my consent. There

is no warranty either expressed or implied attached to this program, and

I cannot be responsible in any way, shape or form for any damage resulting

from the use of this program (though that is not likely to occurr!).