U.S. Robotics Courier Modems (HST, V.32 or V.32bis, and Dual Standard)

Recommended settings for maximum throughput

NOTE:  These are recommended settings only.  Your individual communications

program and equipment may require different settings.  Where there is more

than one setting for a given AT command, either can be used depending on

personal preference or specific equipment (HST or V.32 modem, Tone or Pulse

dialing, etc).

Items with an asterisk (*) denote IMPORTANT items to maximize speed.

INSURE THESE ARE SET to get the most throughput and highest cps.

---------

DIP switch settings -- for MOST comm programs.  Check your manual.

DIP    Setting

10     Off

9     On

8     On

7     Off

6     Off

5     On

4     Off

3     On

2     Off

1     Offs

---------

AT Command Settings

B1      HST mode (for HST modems)

B0      V.32 or V.32bis mode (for V.32/V.32bis modems)

C1      Transmitter enabled (Default)

E1      Local echo of command mode ON (Default)

F1      Full duplex mode (Default)

K0      Modem clock used for call duration (Default)

L2      Speaker volume medium (Default)

M1      Speaker on until carrier detect (Default)

Q0      Connection result codes displayed (Default)

V1      Connection result codes displayed VERBAL (Default)

X6      Result code options

--------

&A3      Display protocol in result code

&A1      Display ARQ result codes (Default)

&B1  *   Fixed DTE rate -- does not follow connect rate

&C1      CD carrier detect when carrier is present

&D2      Modem accepts commands on DTR data terminal ready

&G0      No guard tone for U.S. and Canadian systems (Default)

&H1  *   Hardware (RTS/CTS) transmit data flow control

&I0  *   Disable receive data software flow control (Default)

&J0      Normal single line phone connection (Default)

&K3  *   Disable MNP Level 5 data compression/enable MNP 2-4 error correction

&K1      Auto enable data compression (Default)

{Use &K1 only if your modem does not support &K3}

&L0      Disable leased line operation (Default)

&M4  *   Normal/ARQ mode -- Use ARQ error control if connection can be made

if not made, use normal mode (Default)

&N0  *   Data rate NORMAL, negotiate for highest connect (Default)

&P0      Pulse dialing ratio for North America (Default)

&R2  *   RTS hardware flow control -- pass data on RTS high

&S0      DSR override always on (Default)

---------

To send these commands, I recommend the following strings

NOTE:  The &W command writes the settings to NRAM non-volatile RAM

that will be returned as "defaults" whenever the modem is powered up or

reset with the ATZ command

ATB1C1E1F1K0L2M1Q0V1X6&W <return>

AT&A3&B1&C1&D2&G0&H1&I0&J0&K3&L0&M4&N0&P0&R2&S0&W <return)

---------

For WHATEVER communications program you will be using (Qmodem, Procomm Plus,

Telix, etc) the INITIALIZATION string should be ATZ^M.  The ^M represents the

command for <return>.

You should set your communications software for a data rate of 38,400 baud or

19,200 baud.  If your modem (internal) or serial port (external) has the

16550AN or 16550AFN UART chip installed, use the 38,400 setting.  If you do

not have this UART chip, TRY the 38,400 setting -- if you seem to have a lot

of errors (particularly on sending files), then drop back to the 19,200

setting.

Most COMM programs will have a set-up utility that will make MOST of these

changes for you.  Use the ATI4 command to see the current modem settings

and the ATI5 command to see the NRAM settings, and modify those that need

to be changed.  For example, if you only need to change from &K1 to &K3,

you can send the command AT&K3&W <return> and it will only modify that

setting.

PLEASE, read the manual for your modem.  The U.S. Robotics modems can only

give you the highest throughput IF you understand and properly configure

the settings.