NBSETUP - NetBios SETUP Program Documentation

(C) Copyright 1988 ARTISOFT Inc.

The NBSETUP program is used to setup (alter) NETBIOS session and

NCB parameters so that more users can log in into a server.  If

you have an ARTISOFT NETBIOS, you will be able to perform this

function when you install the NETBIOS software (LANBIOS or

AILANBIOS).

Primarily, you will use NBSETUP if your NETBIOS has no facility

for altering the maximum number of configured sessions and NCB's,

although it may be run with any NETBIOS.  This document does not

attempt to describe the significance of NCB's and sessions in their

relation to the LANtastic network.  You should consult the

LANtastic Network Operating System User's Manual for an

explanation of these.

Although there is no NETBIOS standard, most NETBIOS manufacturers

conform to either the IBM PC Network Adapter NETBIOS or the IBM

Token Ring NETBIOS.  When either of these NETBIOS's is run, the

maximum number of configured sessions is set to 6 and the maximum

number of configured NCB's is set to 12.  This default translates

to 6 concurrent logins on a LANtastic server.

You can run NBSETUP to change these default values.  NBSETUP must,

however, be run before any network software is started.  The reason

for this is that NBSETUP performs a NETBIOS reset to alter the

default parameters and such a reset would cause the network

software to stop working (actually, NBSETUP will not run if either

REDIR or SERVER are started).

The syntax for NBSETUP is

NBSETUP [parameters]

where parameters are optional command line parameters which specify

values that sessions and NCB's should be set to.

Parameters may be separated by either spaces or slashes (/).

Parameters which take on values may be followed by either = or :.

The command line parameters are as follows:

______

OPTION:     VERBOSE

VERBOSE displays detailed information about what values the NBSETUP

parameters are set to.  Examples of this would include the command

line used to invoke NBSETUP and the maximum number of NCB's and

sessions the adapter has been set to.

You can use VERBOSE to verify that the number of NCB's and

sessions was set to the values that you wanted.

EXAMPLE:

NBSETUP VERBOSE MAX

______

OPTION:     MAX

MAX will set the number of NCB's and sessions to the maximum the

NETBIOS will support.  Many NETBIOS implementations support only

32 maximum sessions and 32 maximum NCB's (all ARTISOFT NETBIOS's

support at least 128 sessions and NCB's).  You can use MAX along

with the VERBOSE option to determine the maximum number of NCB's

and sessions your NETBIOS interface supports.

You may mix the MAX option with other options such as NCBS= or

SESSIONS=.

EXAMPLES:

NBSETUP VERBOSE MAX

NBSETUP MAX NCBS=45

______

OPTION:     NCBS=

NCBS= will set the maximum number of NCB's that the NETBIOS will

recognize. The value you specify for NCBs must be in a range that

your NETBIOS supports.  Most NETBIOS's will reset to both their

default NCB and session values when either value is not in range,

or either value is 0.

EXAMPLE:

NBSETUP NCBS=34

NBSETUP NCBS=0

______

OPTION:     SESSIONS=

SESSIONS= will set the maximum number of sessions that the

NETBIOS will recognize.  The value you specify for sessions must

be in a range that your NETBIOS supports.  Most NETBIOS's will

reset to both their default NCB and session values when either

value is not in range, or either value is 0.

EXAMPLES:

NBSETUP SESSIONS=100

NBSETUP/NCBS:32/SESSIONS:32

NBSETUP SESSIONS=0

______

OPTION:     CHECK

CHECK will not alter any NETBIOS parameters but will display the

same items as the VERBOSE command.  You can use CHECK to see what

the current NETBIOS parameters are set to.

EXAMPLE:

NBSETUP CHECK

______

OPTION: HELP or ?

HELP or ? will display a short description of all the command line

parameters.

EXAMPLE:

NBSETUP ?

You may mix several parameters on the command line.  The order of

the parameters is not important.  NBSETUP performs the following

actions sequentially before resetting the NETBIOS:

1.  The number of NCB's and sessions are set to the values

that the adapter currently has.

2.  If the MAX parameter was specified, then the number of

NCBs and sessions is set to the maximum that the NETBIOS

can support.

3.  If NCBS= or SESSIONS= parameters were specified, then

the NCB's or sessions are set to the corresponding

values.

The above sequence, for example, implies that you can mix MAX with

NCBS=10 and, if the maximum number of sessions were 128, then that

would be equivalent to specifying NCBS=10 SESSIONS=128.  The above

sequence also implies that if no options are present then the

NETBIOS is reset with the same parameters that it

already had.  Here are some examples of various command lines:

NBSETUP

NBSETUP SESSIONS=40 NCBS=40

NBSETUP SESSIONS=40 NCBS=40 VERBOSE

NBSETUP MAX VERBOSE SESSIONS=10

NBSETUP VERBOSE

You can also redirect NBSETUP's output to a file or device.  This

may be useful if you want to log verbose information.  For

example,

NBSETUP MAX VERBOSE >NBSETUP.LOG

NBSETUP Messages

================

NBSETUP will display two types of messages: informative and error.

Informative messages require no direct action.  Error messages

imply that NBSETUP was not successful.  If you run NBSETUP in a

batch file, you can test for errors using the IF ERRORLEVEL

command.  The following error levels may be returned:

1     No NETBIOS present

2     NETBIOS is already in use

Informative Messages

--------------------

MESSAGE: LANtastic (tm) NBsetup V1.0 - (C) Copyright 1988

ARTISOFT Inc.

Meaning: This message is displayed whenever NBSETUP is invoked.

The version number may vary from the above version number.

MESSAGE:         ---- NETBIOS parameters have been altered ----

Meaning: This message is displayed after the NETBIOS parameters

have been successfully altered.

MESSAGE:       ---- NETBIOS parameters have NOT been altered ----

Meaning: This message is displayed if the NETBIOS parameters have

not been altered.  The message may appear after an error

or when the CHECK parameter is used.

MESSAGE: Command line:

ccc cccc ccccc

NETBIOS parameters:

Configured maximum sessions sss

Configured maximum NCBs nnn

Meaning: This message is displayed when you specify the VERBOSE

command line option.  The "ccc cccc ccccc" denotes the

command line you invoked NBSETUP with.  The number sss is

the number of sessions that the NETBIOS currently

supports.  The number nnn is the number of NCB's the

NETBIOS currently supports.

MESSAGE: Valid parameters are

VERBOSE     Displays NETBIOS information after setting it

MAX         Sets maximum NCBs and sessions that NETBIOS

supports

NCBS=n      Sets maximum NCBs to n

SESSIONS=n  Sets maximum sessions to n

CHECK       Displays current NETBIOS information but does

not set it

? or HELP   Displays this help text

Meaning: This message is displayed when you specify the HELP or ?

command line option.

Error Messages

--------------

ERROR:      NETBIOS interface is not installed

Meaning:   You do not have a NETBIOS interface installed.

Remedy:    You must first install your NETBIOS software. If you

are using an Artisoft NETBIOS, then you must run

NBSETUP after running LANBIOS or AILANBIOS.

ERROR:     NETBIOS interface is in use - cannot be set up now

Meaning:   You have already started software which is currently

using the NETBIOS interface.  NBSETUP will not change

NETBIOS parameters while other software is using the

NETBIOS.

Remedy:    You should run NBSETUP right after you bring up the

NETBIOS interface.  In particular, you should run

NBSETUP before REDIR or SERVER.