����������������������������������������������������������͸

�                                                          ���

�          S L I C K   A N S W E R  M A C H I N E          ���

�                   >>>> SlickWare <<<<                    ���

�                     Downloaded from:                     ���

�             Host Line BBS of New Orleans, La             ���

�                 Home of Simple Software                  ���

�                      P.O.B. 8184                         ���

�                   N. O., La., 70182                      ���

�                                                          ���

����������������������������������������������������������͵��

�  Voice (504) 288-6550               BBS (504) 283-9785   ���

����������������������������������������������������������;��

����������������������S�A�M���V�3.2��������������������������

(S)lick (A)nswer (M)achine is about the simplest BBS imaginable. It

was designed to do ONLY one thing, ANSWER your telephone and give a

short and simple text message to the caller. It will then hang up

and recycle itself. It will be up to you to decide how to use it

best.

You are hereby granted a limited license to use SAM for a period of

10 days for evaluation purposes only. If after that time you wish

to continue using SAM a nominal registration fee of $10.00 is

required. The shareware version is limited to displaying no more

than 15 lines of text. The registered version will send text files

of unlimited size and will have our advertisment removed.

SAM was written in Zales new PowerBasic V2.10 and is quite a big

improvement over Borlands older discontinued Turbo Basic Ver 1.1.

SAM was designed to run on any IBM or close clone using MS/PC DOS

Ver 2.1 or later. RAM requirments are minimal at 128k. SAM will

function on most modems using Hayse commands and addressing all

four standard communications ports.

3F8 - COM1 - IRQ4

2F8 - COM2 - IRQ3

3E8 - COM3 - IRQ4

2E8 - COM4 - IRQ3

To operate, place SAM.EXE in the desired HD directory or floppy,

and at the prompt type SAM. If everything is configured correctly, you

will first hear the telephone go off hook for a few seconds, then

the screen will display the opening message followed by a time line

appearing with time updating every second on the second.

SAM will automatically check to see if SAM.CFG and SAM.TXT exist and

if not, SAM will create them. The default SAM.CFG file is set for

COM1 at N81, 1200baud. You can change this with any simple ascii text

editor or WP in ascii mode. The same goes for the message you wish to

send via SAM.TXT.

SAM comes with the following TWO files:

SAM.DOC   This file you are reading

SAM.EXE   Executable Program

An explanation of SAM.CFG follows:

Contents: 11 lines, simple ascii text file

------------------------------------------------------------------------------

COM1                     [either COM1: or COM2:]

1200                     [highest baud your modem supports]

N                        [No Pariety]

8                        [8 bits]

1                        [1 stop bit]

RS                       [Clear to Send delay in milleseconds]

CS0                      [Data set Ready delay in milleseconds]

ATZ                      [modem reset string]

ATM1H1                   [modem go off hook command]

ATH0&D2V0X4S7=10S0=1     [modem init string]

1                       [special delay value in seconds]

------------------------------------------------------------------------------

NOTE: 99% of all calls we receive are modem related problems. SAM

will work 90% of all modems manufactured in the past 24 months

with the parameters specified. However, there is a lot of weird

iron out there. Some modems can be handled by making simple

adjustments to the three adjustable timing parameters in CFG

file. You know when SAM is going to work correctly as you will

hear the dial tone as the line goes off hook for a few seconds

before the "Waiting for a call..." message appears.

Another problem area is the so called Hayse compatable modems.

Some recognize all string commands SOME DON'T. Try removing

the &C1 and &D2 if problems persist. ANY one command the modem

cant handle will usually result in a failed command, period.

SAMPLE of SAM.TXT, simple ascii text file, limited to 15 lines.

------------------------------------------------------------------------------

OurLine  BBS is Temporarily OFF

Line While we are doing general

maintenance  to our  system. We     [Edit with any text editor]

expect to be ON-LINE within the     [or WP in ascii mode.     ]

next Hour.

Your participation  in  the BBS

and  OUR   SIG  is appreciated!

* THANKS AGAIN *

Sysops

------------------------------------------------------------------------------

DISCLAIMER

THIS PROGRAM IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND,

EITHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

PARTICULAR PURPOSE.  THE AUTHOR WILL NOT BE LIABLE TO USERS

OF THIS PROGRAM FOR ANY DAMAGES, INCLUDING LOST PROFITS OR

LOST SAVINGS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES,

ARISING OUT OF USE OF THIS PROGRAM.

The Registered version of SAM is $10.00

Name_____________________________________________________________

Mailing Address__________________________________________________

City & Statre ___________________________________________________

ZIP _____________________________ Phone _________________________

Shipped on 5.25 DSDD floppy unless other noted here: ____________

Send $10 check, cash or money order to:

********************************************

Simple Software Company

P.O. Box 8184

New Orleans, La.  70182

Voice Support 504-288-6550  BBS 504-283-9785

EOF()