Moved from HPREAD.C, so program compiles faster.

HPREAD reads data sent by infrared pulses from an HP calculator (such

as an HP-28S) to a Radio Shack $3.49 GP1U52X Infrared Receiver

(catalog #276-137).  The output pin of the receiver is connected to a

a bit of an input port on an IBM PC or compatible.  HPREAD then reads

from the specified port and bit and writes the received data to

stdout.  The data is terminated by a pause of more than three

seconds.  HPREAD also writes the number of bytes transferred and the

number of unrecoverable errors to stderr.  For example:

C>hpread >foo

1043 bytes transferred (17 fixed) with 0 errors

writes the file "foo" with the transferred data.  By default, HPREAD

converts any 0x04 or 0x0A characters that the HP sends into a new

line sequence (carriage return, line feed).  If the "/n" option is

specified, no translation of the data is performed.

By default, HPREAD assumes that the receiver is connected to the

parallel printer port's ERROR* line (pin 15 on the DB-25).  Some

parallel printers supply +5V power on one of the lines of the

printer-end connector.  For example, my printer (a C.Itoh 8510A)

provides +5V at 50 mA on pin 18 of the 36 pin Centronics connector

(NOT the DB-25).  This can truly minimize the external hardware for

the detector.  Just put a switch on the ERROR* line to the computer

to select either the printer's ERROR* line or the output of the

detector, and connect the power and ground of the detector to the

printer's ground and +5V lines.  (For this to work, of course, the

printer must be on.)  Schematic:

SPDT switch

Printer (DB-25) pin 15 -----*      (used to go to port pin 15)

Detector Vout (pin 1) ------*<---o------ Parallel port pin 15

Detector GND (pin 3) ------------------- Parallel port pin 18

Detector Vcc (pin 2) ------------------- Printer supplied +5v

Metal case of detector ----------------- Detector GND (pin 3)

If an easy source of +5V is not available, then you only need three

more parts to supply it.  An AC adapter that puts out DC anywhere in

the range of +6V to +30V (whatever current it puts out is enough), a

0.047 uF or 0.1 uF capacitor, and a 7805 or 78L05 voltage regulator.

You can get all of this from (you guessed it) Radio Shack.  (No, I do

not have stock in Radio Shack.  It's just it seems there are more of

them than McDonald's.)  Connect the output of the AC adapter to the

input of the voltage regulator with the capacitor across that (and

near the regulator).  Make sure to check the polarity of the AC

adapter---some put the positive lead and some put the negative lead

on the outside of the connector.  Connect the output of the regulator

to the detector's power and ground.  An output capacitor is not

needed, since the detector has a built in 5 uF capacitor for power

buffering.  Schematic:

SPDT switch

Printer (DB-25) pin 15 -----*      (used to go to port pin 15)

Detector Vout (pin 1) ------*<---o------ Parallel port pin 15

Detector GND (pin 3) ------------------- Parallel port pin 18

Adapter +DC--------|---Regulator In

0.047 uF ===  Regulator Out---- Detector Vcc (pin 2)

Adapter -DC--------|---Regulator GND---- Detector GND (pin 3)

Metal case of detector ----------------- Detector GND (pin 3)

Wherever power comes from, be sure to connect the metal case of the

detector to ground to reduce noise in the detector.

If a different port or bit is used, then the port and bit mask (and

possibly an invert option) must be specified on the HPREAD command

line.  For example:

C>hpread 22e2 1 /i >foo

1043 bytes transferred (13 fixed) with 0 errors

reads the data from port 0x22e2, bit 0, with the input bit inverted.

Both the port and the mask must be specified in hexadecimal.  The bit

mask must be one of 1, 2, 4, 8, 10, 20, 40, or 80.  If there are

other operations necessary to initialize the port before it can be

used, then another program could be written to do the initialization,

or the operations could be added to this program in main() after

memory is allocated, and the program recompiled.

If a byte is received with no errors, or it is received with errors

and is correctable, then the byte is written to stdout and counted in

the bytes transferred.  If the byte required correction, then it is

also counted as a "fixed" byte.  The main use of the "fixed" count is

to judge the quality of the link.  Fixed bytes are almost certainly

good.

If a byte (or at least the illusion of start bits) is received, and

the data is in error and not correctable, then nothing is written to

stdout and the byte counts as an error.  If there are trailing

errors, i.e. errors with no good bytes that follow them, then

imbedded errors and trailing errors are counted separately.  If a

transfer results in no imbedded errors, but some trailing errors,

then the transfer may very well be good, since the trailing errors

might just be noise received while waiting for a silence of three

seconds.  For example:

519 bytes transferred (8 fixed) with 0 imbedded errors and 1 trailing error

HPREAD has been tested and works on 6 MHz 286 systems and on 8 MHz

8088 systems.  It is not known at this time whether this program will

work on 4.77 MHz 8088 systems or not.  The program utilizes the

system timer which runs at the same speed on all machines (1.193182

MHz), regardless of the processor's clock speed.  HPREAD should work

on any system faster than those mentioned above (for example, 8 MHz

8086 systems, 16 MHz 386 systems, 1000 GHz 786 systems, etc.).

There are two options for faster machines: /f (fast) does the data

decoding and error correction between bytes, and /e (enable) enables

interrupts between bytes.  /f reduces the memory requirements of the

program by 128K, and /e prevents the clock from losing (much) time by

allowing timer tick interrupts during the transfer.  If either option

is not appropriate for your machine, you will simply get errors

reported for the transfer.  Even with /e, you can still lose the

three seconds it takes to determine that the transmission is over.

If you have an AT compatible machine, then you can use the /c option

to restore the current clock from the real time battery backed-up

clock after the transfer is complete.  If /c is specified, but there

is an error reading the time from the real time clock (for example,

if it's an XT), then HPREAD will warn you that it did not set the

current clock:

!warning: unable read the time from the real time clock

If you have an XT compatible machine with an add-on battery backed-up

clock, then that clock came with a program that sets the current

clock (typically executed from AUTOEXEC.BAT).  You can write a batch

file that executes HPREAD, and then executes the program to reset the

current clock.

If more than 32,768 (32K) bytes are sent, then only the first 32,768

bytes are written to stdout.

The command format and options can be displayed using the "/h" (help)

option.  If /h is specified, no other action is performed.

When used in a batch file, HPREAD returns the following errorlevels:

0 - No error or /h specified.

1 - Error in data, or data exceeded MAXFER (32,768) bytes.

2 - Error writing data to stdout.

3 - Could not allocate a MAXFER byte buffer, or if /f not specified,

could not allocate a 4*MAXFER byte buffer in the far heap.

4 - Invalid arguments.