=========

O R B S

=========

Orbital Prediction Program

for

Earth  Orbiting Satellites

Version  1.4.3

USER REFERENCE

7/22/90

Ron Dunbar, W0PN

=========

O R B S

=========

INTRODUCTION:

ORBS is a menu-driven system which produces printed listings showing

an earth-orbiting satellite's azimuth, elevation, range as well as

other approprate data at specified intervals of time as seen from an

observer's location.  Perhaps the easiest way to give you an idea of

ORB's features is to start with the opening menu:

__________________________________________________________________

O R B S

Version 1.4.3

ORBITAL PREDICTION PROGRAM WITH TIME WINDOWS

7/22/1990

__________________________________________________________________

Turbo BASIC                                      Ron Dunbar W0PN

1.  Generate orbital predictions.

2.  Update Elements.

3.  Modify beacon frequency file.

4.  Modify observers data file.

5.  Exit to operating system.

Your choice..

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Let's investigate the menu items one at a time...

1.  GENERATE ORBITAL PREDICTIONS:

If you choose this function, the next screen you'll see will look

something like this:

____________________________________________________________________________

O R B S

ORBITAL PREDICTION PROGRAM WITH TIME WINDOWS

Ron Dunbar, W0PN         7/22/1990

______________________________________  _____________________________________

Satellite        ID   ELset  Date       Satellite        ID   ELset  Date

____________________________________________________________________________

1.  AO-10        14129  530   6/26/90   12. MIR          16609  775   7/5/90

2.  UO-11        14781  754   7/1/90    13. HUBBLE       20580  116   7/6/90

3.  RS-10/11     18129  245   7/6/90    14. NOAA-9       15427  601   7/5/90

4.  AO-13        19216  135   7/3/90    15. NOAA-10      16969  447   7/2/90

5.  FO-20        20480  91    7/4/90    16. MET-2/16     18312  451   7/4/90

6.  UO-14        20437  174   7/4/90    17. MET-2/17     18820  309   7/4/90

7.  AO-16        20439  90    7/4/90    18. MET-3/2      19336  518   7/4/90

8.  DO-17        20440  94    7/4/90    19. NOAA-11      19531  301   7/4/90

9.  WO-18        20441  95    7/4/90    20. MET-2/18     19851  253   7/4/90

10. LO-19        20442  94    7/4/90    21. MET-3/3      20305  160   7/4/90

11. SALYUT 7     13138  364   7/6/90    22. MET-2/19     20670  7     7/2/90

<Enter> to abort, or SELECT SATELLITE #?

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Now choose the satellite you're interested in, answer a few questions,

and the printed listing is produced in record speed, thanks to Turbo

BASIC's nimble compiled code and Tom Clark's QUICKSTEP algorithm

trickery (that's the infamous W3IWI, you know) which speeds up the

calculations when the satellite is below the horizon.

As you answer the questions mentioned above, you'll note that another

unique feature has been added; TIME WINDOWS.  You have the option of

ignoring all satellite passes outside a selected timeframe.. after all,

there's little value in spending time calculating and producing a

listing covering times when you can't be there to see/hear/work that

satellite, is there?

TIME WINDOWS will allow you to span 2400 UTC with your window time;

you can specify a window such as 2200 to 0400, and it will be handled

properly.  The TIME WINDOW feature is particularly useful for listing

the 'good' visual passes of MIR and SALYUT-7, for instance, because

you can only see them during the period between local civil twilight

and the time when they are enveloped in the earth's shadow.

Now let's see what a typical listing looks like.  We'll use PACSAT:

*** O R B S ***

AO-16 Orbital Predictions                 Page # 1

for W0PN at Lat: 40.145  Lon:105.144

Element set 94 is  8.4 days old

Listing covers the period 206.0000 to 207.0000 or  24.00 hrs.

This listing run at 14:18:46 on 07-24-1990.

PARAMETER           REFERENCE           STARTING

---------           ---------           --------

Catalog number       20439

Epoch               90197.64903266      90206.00000000

Inclination          98.7017             [ No change ]

R. A. A. N.         273.7876            282.1178

Eccentricity        0.00125860           [ No change ]

Arg. Perigee         69.5283             45.1486

Mean Anomaly        290.7248             44.4046

Mean Motion         14.28763438         14.28768449

Drag Correction      3E-006              [ No change ]

Orbit number         2507                2627

S.M.A.,(km)          7173.9590           7173.9422

Perigee Ht, km        786.7698            786.7531

Apogee  Ht, km        804.8281            804.8113

Freq (MHz)           437.05  (F1)        2401.142  (F2)

*************************************************************

Printing only the window between 0330:00 and 0500:00 UTC.

*************************************************************

U T C    AZ  EL  437.050 2401.142  RANGE HEIGHT  LAT  LON  M/A SQT F2-TRACK

hhmm:ss  deg deg  DPLR Hz  DPLR Hz    km    km    deg  deg  256 ang 2401.142

- - - Orbit # 2629 - - - Day # 90206 - - - Wednsday, 7/25/90 - - -

0442:00  153   1    +9711   +53353   3127   788  16.2  93.4 235   0 2401.089

0444:00  150  10    +9605   +52769   2336   787  23.3  95.2 240   0 2401.089

0446:00  143  24    +9066   +49811   1590   787  30.3  97.0 246   0 2401.092

0448:00  117  48    +6958   +38227   1017   787  37.4  99.1 251   0 2401.104

- - - Orbit # 2630 - - - Day # 90206 - - - Wednsday, 7/25/90 - - -

0450:00   32  51     +290    +1593    993   787  44.2 101.4   0   0 2401.140

0452:00    3  25    -6647   -36517   1540   787  51.1 104.3   5   0 2401.179

0454:00  355  11    -8986   -49367   2280   787  58.1 108.0  10   0 2401.191

0456:00  352   2    -9583   -52650   3069   787  64.8 113.3  15   0 2401.195

End of AO-16 listing.

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

As you can see by the information between the lines of asterisks, we

specified a 'time window' of 0330 to 0500 UTC.  Note that the

printout shows TWO different orbit numbers (2629 and 2630).  From the

oberver's point of view (yours), this was a single pass (note the

times and elevations), but the standard convention calls for the

orbit number to increment at perigee, indicated by the MA/256 column

showing a transition from 255 to 0 in this example.

Note that two 'beacon' frequencies were present for PACSAT in the

FREQS.ORB file.. 437.050 and 2401.142 MHz.  The doppler associated with

each frequency is listed in the 4th and 5th columns of the printout...

the column heading shows the frequency the doppler is computed for.

The RANGE, HEIGHT, LAT and LONG all refer to the satellites position.

The LAT and LONG are the point on the earth above which the satellite is

currently positioned (the Sub-Satellite Point, or SSP).  The RANGE is

the distance from the observer in meters, and the HEIGHT is the

satellite's altitude above the earth in meters.  LAT and LONG are in

degrees, of course.

The "M/A 256" column represents the satellites position in its orbit

(Mean Anomaly) divided in 256 parts per orbit (0 - 255).  Perigee = 0

and apogee = 128.  The orbit number is incremented at perigee passage,

represented by M/A 256 going from 255 to 0.

The "SQT" column represents the 'squint' angle from the satellites

antennas to you.. in other words, if SQT = 10 degrees, the satellites

antennas are pointed 10 degrees off 'bore sight' to your location.

NOTE: this feature has not been completely implemented yet, so it will

always indicate '0' degrees squint angle.

Note the rightmost column labeled "F2 TRACK".  The numbers in this

column represent the frequency you must transmit on to 'hit' the F2

TRACK frequency (435.025) as seen at the spacecraft.. this listed

frequency takes the doppler into consideration and is a handy feature

when you need to uplink at a specific frequency.  One of the last

questions you are asked before the program goes off to compute all this

stuff is whether you would like to manually enter an alternate frequency

for F2.  This lets you temporarily override the frequencies contained in

the FREQS.ORB default file.  (More on that later).  The "F2-TRACK"

column won't appear on the printout if no frequency is specified for F2.

Ok, now you've run your listing, and you've just downloaded the AMSAT

Orbital Data Bulletin and captured this week's latest Kepler elements

into your STUFF.LOG file.  Use your favorite word processor in the NON-

DOCUMENT MODE (please) to edit STUFF.LOG and get rid of everything

except the elements.  You can leave in all the carefully constructed

headers, asterisks, etc., but its best to clean out the stuff not

relating to the Bulletins.

Be sure to check the bulletin for typos.. sometimes the ':' following

the keywords gets lost, or 'e' (as in  "1.687e-04") turns up missing.

The program will catch the first error, but not the second one.  It

can show up as subsurface apogee and perigee heights in the parameter

listing on the first page.. that's why that listing is printed for you!

If you come up with anything that looks weird, check and correct the

ELEMENTS file, then redo the update operation.

All bulletins should be combined into one file which MUST be named

ELEMENTS  (no .EXTension, please).  Now move that file into the same

directory that ORBS resides in (I call it \ORBS, for want of a better

name.. I have trouble remembering obscure names, birthdays, etc.).

If you spell ELEMENTS wrong or forget to move it into the \ORBS direc-

tory, you'll get a RUNTIME ERROR 53: which says you spelled ELEMENTS

wrong or forgot to move it to the \ORBS directory.. don't DO that!

(See HELPFUL HINTS at the end for a .BAT program to do the moving.)

2.  UPDATE ELEMENTS:

Now, we're ready to choose item (2) on the ORBS menu.

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=               ORBITAL ELEMENT UPDATE PROGRAM                      =

=                          9/22/1990                                =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Scanning input file... please wait.

Total number of satellites read from the ELEMENTS file = 22

Total number of satellites written to the KEPLER.ORB file = 22

The frequency data in FREQS.ORB will be inserted into the

records contained in KEPLER.ORB.

FREQS.ORB contains 5 records for input.

5 insertions completed.

End of update..

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Now, wasn't that a lot easier than typing in 14 numbers for 22 dif-

ferent satellites?  You didn't have to do anything but sit there and

wonder what the computer was doing (especially when it was messing

around with your hard disk).  You've just updated all the Keplerian

elements for all the satellites contained in the bulletin in about

one minute or less!

What actually happened was that (1) the program ORBUPDAT.TBC was called

by the menu program.  (2) The file ELEMENTS was read in, (3) the beacon

frequency data from the file FREQS.ORB was merged with the ELEMENTS

data, and (4) the combined data was written to the KEPLER.ORB file

which is used by the tracking program ORBP.TBC.

KEPLER.ORB is the source of the data that is displayed as the list of

satellites from which you choose when you do menu item (1), "Generate

orbital predictions".  This function does perform rudimentary editing

of the data contained in the ELEMENTS file as it is processed... it

will flag you of any gross errors encountered.

Let's suppose that a new satellite has been placed in orbit, and it

contains a beacon or two which you want to monitor.  You're ready for

the next menu choice...

3.  MODIFY BEACON FREQUENCY FILE:

When you make this choice from the menu, you'll see this....

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=                    Modify FREQS.ORB Entries                       =

=                                                                   =

=                            9/22/1990                              =

=                                                                   =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Existing entries will be presented for possible modification, followed

by blank entry(s) to allow you to add to the list of satellite beacon

frequencies; the frequency specified can be any you wish to track; it

doesn't have to be strictly a beacon.  Make certain you enter the

satellite's CATALOG number accurately.. the record is keyed on it.

Satellite name:  Oscar-10

Catalog number:  14129

Frequency 1:  145.810

Frequency 2:  145.987

Accept, Delete, Modify this entry (A/d/m):

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

IMPORTANT:... Note the prompt line.. the 'A' is in UPPER CASE.  This is

the convention I use to indicate the DEFAULT you will get if you just

hit the ENTER key.  You may also type an 'a' if you wish to follow

directions.  If you Accept this entry, the next satellite contained in

the FREQS.ORB file will be presented in order, and so on until the

entries are exhausted.

If you choose to DELETE the entry, it goes away.. simple.  If you choose

to MODIFY it, BE VERY CAREFUL that you get the "Catalog number" right!

You can misspell the name, but the catalog number is the key to matching

this data up with it's counterpart contained in the ELEMENTS file.

If you choose MODIFY, the cursor moves to the field following "Satellite

name:" and waits for your action.  If you merely hit ENTER, the data in

that field remains unchanged and the cursor moves on to the next field.

If you modify a field, any data remaining in the field beyond the cursor

is erased before the cursor advances to the next field.  When you ENTER

following the last field (enter '0' if you don't want one of the beacon

frequencies anymore), you are asked "Is data correct (Y/n):".  If you

made an error, you can redo it by answering 'n', otherwise, a 'y' or an

ENTER will accept the modifications. When you have responded to the last

entry and hit the ENTER key to accept the data, the following appears:

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=                    Modify FREQS.ORB Entries                       =

=                                                                   =

=                            9/22/1990                              =

=                                                                   =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Existing entries will be presented for possible modification, followed

by blank entry(s) to allow you to add to the list of satellite beacon

frequencies; the frequency specified can be any you wish to track.  It

does not have to be strictly a beacon.  Make certain you enter the

satellite's CATALOG number accurately.. the record is keyed on it.

Satellite name:

Catalog number:

Frequency 1:

Frequency 2:

Make an entry (y/N):

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

You have noticed, of course, that all the fields are empty, and the

prompt is asking whether you wish to make a new entry.  Remember to get

that "Catalog number" right!

I suppose that you'd like to get YOUR call letters and location stuck

in here somewhere.. that brings us to....

....the next main menu function... (4):

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=                     Modify ORB.CFG Entries                        =

=                                                                   =

=                            9/22/1990                              =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

The ORBS.CFG file contains 'observer specific' data used by the prediction

program.  The existing entry will be displayed for possible modification.

Latitude is entered as decimal degrees (nn.nnn); South latitudes are

preceeded by a '-' (-nn.nnn).  Longitude is entered as decimal degrees

West longitude.  Observer Altitude is entered in meters, not feet.

Minimum elevation is that elevation (in degrees) above which you wish the

printout to commence (usually 0 or -1).

Observer name/call:  W0PN

Observer latitude:  46.860

Observer W. longitude:  92.459

Observer altitude:  435

Min sat elevation (deg): -1

Accept or Modify this entry (A/m):

Most of the fields are pretty self-explanatory.  The Station latitude

and longitude must be in decimal degrees, not degrees, minutes, etc.

The longitude is specified as WEST longitude.. none of that '-179.999'

stuff allowed.  The altitude is in METERS, not feet or furlongs.

If you don't want to forever run listings with my call and coordinates

on them, I suggest you choose to MODIFY this entry.  When you have

completed your task (operates the same as the previous function), you

will receive this screen:

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

=                                                                   =

=                     Modify ORB.CFG Entries                        =

=                                                                   =

=                            9/22/1990                              =

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

The ORBS.CFG file contains 'observer specific' data used by the prediction

program.  The existing entry will be displayed for possible modification.

Latitude is entered as decimal degrees (nn.nnn); South latitudes are

preceeded by a '-' (-nn.nnn).  Longitude is entered as decimal degrees

West longitude.  Observer Altitude is entered in meters, not feet.

Minimum elevation is that elevation (in degrees) above which you wish the

printout to commence (usually 0 or -1).

Observer name/call:  W3IWII

Observer latitude:  66.871

Observer W. longitude:  74.103

Observer altitude:  250

Min sat elevation (deg): -1

Is data correct (Y/n):

= = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = = =

Again, you have a chance to correct any typos.

The only remaining main menu function is......

(5) Exit to operating system.

That one ALWAYS works exactly as it's supposed to, even after extensive

and rigorous testing conditions (read: exiting to fix another bug!).

Please forward your comments, suggestions and abuse to me via AMSAT

Telemail (RDUNBAR), or you can try to reach me directly:

Ron Dunbar, W0PN         (Res) 303-776-7899

8902 Quail Road          (Bus) 303-443-4321

Longmont, CO  80503

..and I thank you for your support!

Supplemental:

The ARChived file of which this document file is a part, contains

the following files:

ELEMENTS          ORBP    .TBC

FREQS   .ORB      ORBS    .CFG

KEPLER  .ORB      ORBS    .DOC

MODCFG  .TBC      ORBS    .EXE

MODFREQ .TBC      ORBUPDAT.TBC

ELEMENTS:

Contains a recent version of the AMSAT Orbital Data Bulletin which

is prepared weekly for AMSAT NA by Dick Campbell, N3FKV.  This file is

normally captured directly from a bulletin board system (BBS).  ORBS

allows you to input this unedited data file directly, thereby

avoiding the formidable task of entering the data manually.  You can

also download current Bulletins by calling AMSAT's BBS.  This

service is also being provided by others; just ask around.

A sample of the first few lines of a typical Orbital Data Bulletin:

*******************************************************************

*      A M S A T   O R B I T A L   D A T A   B U L L E T I N      *

*                         ORBS227O  20JUN90                       *

*                  Prepared for AMSAT-NA by N3FKV                 *

*******************************************************************

Orbital Elements for OSCARs

Satellite: oscar-9                          |Yes, you can create a file

Catalog number: 12888                       |with your favorite word pro-

Epoch time:      87218.25756376             |cessor in NON-DOCUMENT mode

Element set:     1062                       |IF you take particular care

Inclination:       97.6454 deg              |to duplicate the KEYWORDS

RA of node:       237.0035 deg              |exactly as shown at left;

Eccentricity:    0.0003676                  |the upper-case letters MUST

Arg of perigee:    58.3920 deg              |be upper-case and the colons

Mean anomaly:     301.7665 deg              |MUST be positioned as shown.

Mean motion:   15.29888737 rev/day          |There MUST be at least one

Decay rate:      2.007e-05 rev/day^2        |space between the keyword

Epoch rev:           32439                  |and the value following.

|The words following the value

Satellite: oscar-10                         |are NOT required; i.e.; 'deg',

Catalog number: 14129                       |'rev/day', etc.

.....etc.................

FREQS.ORB:

Contains user defined beacon frequencies for selected satellites.

When you choose "Update Elements." from the ORBS menu, the ORBUPDAT

routine merges the data from the FREQS.ORB file with the data from

the ELEMENTS file to produce the file KEPLER.ORB, which is used

directly by the ORBS tracking program, ORBP.TBC.

Typical contents of the file FREQS.ORB could be as follows:

"Oscar-10",14129,145.810,145.987

"Oscar-11",14781,145.826,435.026

"rs-10/11",12999,145.857,145.907

"Oscar-13",19216,145.811,435.025

Each line (or record) is terminated in a <CR/LF> to indicate the end

of the record.  You need not worry about constructing this file

yourself, because ORBS provides you with a menu selection which does

the hard work for you.  IMPORTANT...Do NOT attempt to update this file

with a word processor.. use ONLY the built in maintenance function.

ORBS.CFG:

Contains observer specific data.  As supplied, ORB.CFG contains the

following:

"W0PN",40.145,105.242,1612,-1

This entry is followed by a <CR/LF> to indicate the end of the record.

LATE NOTES & HELPFUL HINTS:

--------------------------

Julian search time:

When you're running a listing, you will notice a highlighted 6 digit

number spinning in the upper right corner of your screen.. the 6 digit

number is followed by a single '0' or '1' which appears to change

randomly.  The label to the left of the number says "Julian search

time:".  This is the instantaneous day of the year and fractional part

of the day that is being used in the determination of the satellite's

position.  The 0/1 following indicates whether the satellite is below

or above the observer's horizon.  It's just a 'fuzzy blanket' to let

you know the computer is actually doing something between printouts.

Sidereal Time table:

I neglected to mention that the sidereal time tables have been

eliminated because that figure is now calculated internally by ORBP.TBC.

Moving the ELEMENTS file from your communications subdirectory to the

ORBS subdirectory:

I use the following batch file to perform the relocation of the ELEMENTS

file from my communication subdirectory over to the ORBS subdirectory.

Note that the previous ELEMENTS file is renamed ELEMENTS.OLD, just in case

something is drastically wrong with the new data and I want to get the old

stuff back (simply a matter of deleting the defective ELEMENTS file and

renaming ELEMENTS.OLD to ELEMENTS).

= = = = = =

REM Move ELEMENTS to \ORBS directory from \PROCOMM

echo off

cd\procomm

if NOT EXIST ELEMENTS goto nofile

cd\orbs

del elements.old

ren elements elements.old

cd\procomm

copy elements \orbs\elements

del  elements

goto exit

REM

:nofile

echo No ELEMENTS file found.. update aborted.

pause

REM

:exit

echo on

= = = = = =

7/22/1990

ORBS Release 1.4.3 is a minor update which modified the "Satellite

Selection" screen to allow more satellites to be displayed; color was

also added to all the screen displays.  As a further convenience, you

may now either SELECT one of the satellites, or if you change your mind,

hitting a lone <Enter> will take you back to the Main Menu.  This lets

you visually check the entries (without producing a printout) following

an update of the Keplerians, for instance.

The ELEMENT UPDATE's editing functions have been beefed up.  In

addition to detecting missing/erroneous keywords, grossly erroneous

values will now be detected as well.  Previously, a Julian date such

as 872269 would have been accepted, resulting in a runtime error

when the data had to be used in calculations.  Rough range-checking

will now catch most typical typos.  The values and ranges tested

are:

Epoch time:     > 78000 AND <99367

Inclination:    >=  0 AND <= 180

RA of node:     >=  0 AND <= 360

Eccentricity:   >=  0 AND <= 1

Arg of perigee: >=  0 AND <= 360

Mean anomaly:   >=  0 AND <= 360

Decay rate:     >= -1 AND <= +1

To install ORBS, merely copy all the files (into a specific

directory if you're using a hard disk), and execute ORBS.  The menus

will guide you through the rest.

It is recommended that all files except  ORBS.DOC be placed in a

single subdirectory and executed from there.  I know it's difficult

to force yourself, but DO try to read the documentation...

Enjoy ORBS, and thanks for trying it..

73, Ron  W0PN