C Communications Library ( CCL )

USER REFERENCE MANUAL

Version 1.1

10 April 1991

Copyright (C) 1991

By MarshallSoft

All rights reserved

MarshallSoft

PO Box 4543

Hunstville, AL 35815

(205) 881-4630

Page 1

Table of Contents

Chapter                                     Page

Introduction.................................3

Registration.................................4

Serial COM Ports.............................5

RS232........................................6

National INS8250 UART........................7

Revision History.............................7

Warranty.....................................7

Using the Library............................8

Compiling & Linking.......................9

Problems.................................10

Library Reference...........................11

SioBaud..................................12

SioBrkKey................................13

SioBrkSig................................14

SioCrtWrite..............................15

SioDTR...................................16

SioDelay.................................17

SioDone..................................18

SioError.................................19

SioGetc..................................20

SioKeyPress..............................21

SioKeyRead...............................22

SioLine..................................23

SioModem.................................24

SioParms.................................25

SioPutc..................................26

SioRTS...................................27

SioReset.................................28

SioRxBuf.................................29

SioRxFlush...............................30

SioRxQue.................................31

SioTimer.................................32

SioUnGetc................................33

Function Summary............................34

Error Code Summary..........................35

Page 2

Introduction

The   C   Communications  Library  (  CCL  )  is  an  asynchronous

communications   library   designed   for   experienced   software

developers programming in Microsoft C or Turbo C.  An IBM PC/XT/AT

or compatible is required.  The CCL features:

o  16 communications functions + 6 support functions.

o  Receiver is interrupt driven.

o  Runs from 300 baud to 115,200 baud.

o  Supports COM1, COM2, COM3, and COM4.

o  Adjustable receive queues from 8 bytes to 16 KB.

o  Control-BREAK error exit.

o  11 comm error conditions trapped.

o  Supports all memory models.

o  Allows 2 ports to run concurrently.

o  Complete modem control & status.

o  Written in hand optimized assembly language.

o  Low overhead & very reliable !

A typical application  program  using  CCL  might  look  like  the

following code outline:

+-------------------------------------------------------+

| #include "ccl.h"                                      |

|                                                       |

| char far Buffer[1024];                                |

| main()                                                |

| {                                                     |

|  SioRxBuf(Port,Buffer,Size1024);                      |

|  SioParms(Port,NoParity,OneStopBit,WordLength8);      |

|  SioReset(Port,Baud2400);                             |

|                                                       |

|  ...application code...                               |

|                                                       |

|  SioDone(Port);                                       |

| }                                                     |

+-------------------------------------------------------+

A simple terminal emulator program CALLPGM.C is provided in source

code  form  as an example of the use of CCL functions. CALLPGM can

be used to call up bulletin board services or mainframe computers,

and features ASCII file transfer using XON / XOFF flow control.

If you find any problems with the C Communications Library or have

any suggestions for improvement, please call or write.   A  Pascal

version  of  CCL  (  to be called PCL ) may be available if demand

warrants.

Custom versions of CCL can be developed for special needs.  Custom

communication packages can also be developed. Call for prices.

Page 3

Registration

The  shareware  version  of  CCL.LIB  is  provided so that you may

personally determine the usefulness of the product  for  yourself.

If you can use CCL.LIB, please register your use with us.

MarshallSoft

PO Box 4543

Huntsville, AL

35815-4543

Please  pay  by  check  in  US  dollars.   Payment  must accompany

purchase orders. Print the file CCL.INV if an invoice  is  needed.

The registered package is $35 postpaid and includes:

o  CCL.LIB      CCL Library - without shareware screen.

o  CCL.ASM      CCL source code.

o  CCL.DOC      CCL User Reference Manual.

o  CALLPLUS.C   Source for terminal emulator with XMODEM & YMODEM.

o  CALLPLUS.DOC Documentation file for CALLPLUS.

o  LZW.C        Source for Lempel-Ziv-Welch file compression code.

o  CRC.C        Source for 16 bit cyclic redundancy check code.

o  Backbone bound printed User Reference Manual.

o  Telephone support for one year.

o  All future updates are $10

CCL.ASM  is  the  source  code for the library. The source code is

copyrighted by MarshallSoft. The user is granted a license to  use

the  CCL  object code in his own application only.  CCL.ASM is not

shareware and may not be sold or given away to anyone.

CALLPLUS.C is a terminal emulator program based on CALLPGM.C  that

features  ASCII,  XMODEM,  XMODEM/CRC,  XMODEM/1K  and YMODEM file

transfer.  CALLPLUS.C is documented in CALLPLUS.DOC.

LZW.C   is   an   implementation   of  the  Lempel-Ziv-Welch  file

compression algorithm.   It  is  used  in  many  file  compression

utilities available today. LZW can compress files to as much as 40

percent of their original size.

CRC.C is an implementation of the 16 bit Cyclic  Redundancy  Check

algorithm   as   specified   by   the  Comsultative  Committee  in

International Telegraphy and Telephoney ( CCITT ). It is  used  in

XMODEM/CRC  and  in  YMODEM.  It will detect 99.99 percent of data

errors.

The  registered  user  will  receive  the latest version of CCL by

return mail.  A 5.25" diskette is provided unless a 3.5"  diskette

is requested when ordering.

Page 4

Serial COM Ports

IBM PC compatible computers can have up to four serial ports.  The

BIOS   table   located  at  address  40:0000  has  room  for  four

communication port addresses: COM1 to COM4. During  boot  up,  the

COM1  and  COM2  addresses  are placed in the BIOS table providing

that the hardware is present. Unfortunately,  the  COM3  and  COM4

addresses  are not placed in the table in DOS through version 3.3.

This can be corrected by using DEBUG  to  assemble  the  following

program  SETCOM3  which  should be added to the AUTOEXEC.BAT file.

When executed, this program will  place  the  standard  COM3  port

address  03E8 in the BIOS table. For COM4, change 03E8 to 02E8 and

[0004] to [0006].

PUSH DS

MOV AX,0040

MOV DS,AX

MOV AX,03E8

MOV [0004],AX

POP DS

MOV AX,4C00

INT 21

SETCOM3.COM and SETCOM4.COM are provided to setup  COM3  and  COM4

addresses.   Please  note that you must have the physical hardware

present !

Page 5

RS-232C

RS-232 is the name of the serial data interface standard  used  to

connect  computers  to  modems.  Most IBM compatible computers are

built with at least one serial port and use either DB9 ( 9  pin  )

or DB25 ( 25 pin ) connectors.

A  summary  of  these  pins  and their function follows.  For more

detailed information, refer to one of the many books dealing  with

RS-232 interfacing.

Signal Ground Pin 7 (DB25), Pin 5 (DB9)

The SG line is used as the common signal ground, and  must  always

be connected.

Transmit Data Pin 2 (DB25), Pin 3 (DB9)

The TX line is used to carry data from the computer to the modem.

Receive Data Pin 3 (DB25), Pin 2 (DB9)

The RX line is used to carry data from the modem to the computer.

Data Terminal Ready Pin 20 (DB25), Pin 4 (DB9)

The  DTR  line is used by the computer to signal the modem that it

is ready.

Data Set Ready Pin 6 (DB25), Pin 6 (DB9)

The DSR line is used by the modem to signal the computer  that  it

is ready.

Request to Send Pin 4 (DB25), Pin 7 (DB9)

The  RTS  line  is  used  to "turn the line around" in half duplex

modems, but is not necessary in full duplex modems.

Clear to Send Pin 5 (DB25), Pin 8 (DB9)

The  CTS  line, like the RTS line, in not necessary in full duplex

modems.

Data Carrier Detect Pin 8 (DB25), Pin 1 (DB9)

The DCD line is used by the modem to signal the  computer  that  a

data carrier signal is present.

Ring Indicator Pin 22 (DB25), Pin 9 (DB9)

The RI line is asserted when a 'ring' occurs.

Page 6

National INS8250

The  C  Communications  Library  is based on the standard National

INS8250 UART. The 8250 consists of 6 register ports based  at  the

following standard addresses:

COM1 = 3F8H  COM2 = 2F8H  COM3 = 3E8H  COM4 = 2E8H

If  you  are not familiar with the INS8250, several good books are

available.  Although a knowledge of the 8250 is not  necessary  to

use  CCL,  a  general  knowledge  of  the  theory  of operation of

Univeral  Asynchronous  Receiver  /  Transmitters  (  UARTs  )  is

recommended.

Offset R/W Register

0       R/W   Receiver ( read ) / Transmitter ( write )

1       R/W   Interrupt Enable

2       R     Interrupt Identification

3       R/W   Data Format ( Line Control )

4       R/W   RS-232 ( Modem ) Control

5       R/W   Line Status

6       R/W   RS-232 ( Modem ) Status

Revision History

Version 1.0 -- 14 January 1991 -- original release.

Version 1.1 -- 11 March 1991

o  Added SioUnGetc() function to library.

o  Added ASCII file transfer to CALLPGM & CALLPLUS.

o  Minor documentation changes.

Warranty

The user of this software assumes all liability for its use. In no

case shall MarshallSoft be liable for any damages,  including  any

incidental or consequential damages.

Page 7

Using the Library

The  first  thing  to  do  is  to  copy all the files from the CCL

distribution disk to a working disk,  and  put  the  original  CCL

distribution disk in a safe place.

The CCL has been tested on a TANDY 1000 ( IBM PC clone ), a  TANDY

3000  (  IBM  AT  clone  ), a TANDY 1400LT ( IBM XT clone ), and a

Gateway 2000 Cache ( 25 MHZ 80386-DX ).

The C Communications Library  is  provided  as  the  library  file

CCL.LIB.   It  has  been  tested with both Microsoft C and Turbo C

compilers.  CCL.LIB may be used with any  memory  model.  However,

your   application  must  be  certain  to  include  CCL.H  and  to

specifically declare the receive buffers as FAR ( see SioRxBuf ).

Please examine the CCL.H file. Note that COM1 is defined  as  port

zero,  not  port  one.  The user must assume the responsibilty for

passing the correct information when calling CCL functions.   Note

that  function  prototypes  are  provided  in CCL.  But since some

earlier C compilers don't support function prototypes,  CCL.H2  is

identical to CCL.H except for the prototypes.

For  an  example of CCL use, examine the terminal emulator program

CALLPGM.C.  It uses most CCL functions. The  user  should  compile

CALLPGM.C and link with CCL.LIB as a test of the library.

If you have two computers, then you can connect them together with

a  null modem cable and run CALLPGM on both machines.  Whatever is

typed on one machine should appear on the other, and  vice  versa.

Depending  on  the  design  of the null modem cable, CALLPGM.C may

need to be modified so that it does not  wait  for  DSR.  This  is

clearly documented in the CALLPGM.C code.

If you have a modem, then use CALLPGM  to  call  up  any  bulletin

board  system  (  BBS  ).   There  are  many  free BBSs around the

country. Look in any issue of "Computer Shopper"  (  available  in

bookstores,  computer  shops, and many grocery stores ) for a list

of current systems.

CCL functions can be called from any  language  supporting  the  C

language  calling  convention  and  FAR arrays and functions.  See

your compiler manual.

Page 8

Compiling and Linking

CCL has been tested with both Microsoft C and  Turbo  C.  Other  C

compilers  may  work  as well provided that they recognize the FAR

keyword, or can compile LARGE model programs.

Any memory model can be used with CCL since all functions  in  CCL

are  declared  as  FAR.   Recall  that  the  receive  buffer ( see

function SioRxBuf ) must also be declared as FAR.   If  two  ports

are  to  be  run  simultaneously,  be sure to use seperate receive

buffers.

Registered users may wish to assemble CCL.ASM.  Use the /MX switch

in order to disable automatic conversion from lower case to  upper

case.   If  the  /MX  switch  is  not  used, then all CCL function

references in C code must be in upper case. To assemble using  the

Microsoft assembler:

MASM CCL /MX;

Then make CCL.OBJ into a library file:

LIB CCL +CCL,CCL.LST,CCL.LIB;

When  compiling with the Microsoft compiler, use the /Ze switch to

enable recognition of the FAR keyword. To compile and  link  using

Microsoft C:

cl -Ze callpgm.c /link ccl.lib

To compile and link using Turbo C:

tcc callpgm.c ccl.lib

Page 9

Problems

If  you cannot get your application to run properly, first compile

and run the terminal emulator program  CALLPGM  provided  on  your

distribution disk.

If  CALLPGM  runs  correctly,  then  you  have  made a programming

mistake in  your  application.   MarshallSoft  cannot  debug  your

application,  especially  over  the  telephone!  However, consider

each of  the  following  when  searching  for  an  error  in  your

application.

1.  Have you included the file CCL.H in your application ?

2.  Have you declared your receive buffer to be FAR  ?  Your  code

should include something like:

char far Buffer[1024];

SioRxBuf(Port,Buffer,Size1024);

3.  Is your receive buffer large enough ? If you are using 1K data

blocks in  YMODEM,  then your receive buffer should be  1K  or

more.

4.  Have  you  selected too high a baud rate ?   Always start with

the slowest  baud rate.  If only one COM port  is  being  run,

you should be able to run at 38400 baud.

5.  Are  you  attempting  to  run  another  application   in   the

background ?  Try running  without any other programs  running

in the background.

6.  If you are running two COM ports simultaneously, are you using

seperate receive buffers ? ( you should ).

7.  Did SioReset return a zero value ?  If not, then you must call

SioReset again. See CALLPGM.C for an example.

If  CALLPGM  does  not  run,  then  either  there  is  a  physical

connection problem or your computer isn't  as  compatible  as  you

thought!   Registered users can always call (205) 881 - 4630 after

5 PM CST Monday through Friday for help.

Page 10

Library Reference

The  remainder  of  this manual list all the CCL functions.  Every

library function will return a value as follows:

1.  Negative values for error conditions. See last  page  of  this

manual for a list of error values and their meanings.

2.  Non-negative values when returning data ( eg: SioLine ).

3.  Zero otherwise.

When  debugging an application, be sure to test all return values.

Use SioError to print the associated text for errors.

/*** example code segment ***/

int code;                /* MUST be 'int', not 'char' */

code = SioFunction();    /* any CCL function */

if(code<0)

{SioError(code);

/* ...do some stuff... */

}

Page 11

SioBaud

Function

Sets the baud rate of the selected port.

Syntax

int SioBaud(Port,BaudCode)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

int BaudCode;         /* Baud code */

Remarks

The  SioBaud  function  sets  the  baud rate without resetting the

port. It is used to change the baud rate after calling SioReset.

Baud Code        Baud Rate        CCL.H Name

0                 300            Baud300

1                 600            Baud600

2                1200            Baud1200

3                2400            Baud2400

4                4800            Baud4800

5                9600            Baud9600

6               19200            Baud19200

7               38400            Baud38400

8               57600            Baud57600

9              115200            Baud115200

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad  port selected.  Value  must  be  0  to 3.

-11 : Bad baud rate code. See above code values.

Example

#include "ccl.h"

/* do auto baud detect */

for(Code=0;Code<10;Code++)

{SioBaud(Port,Code);

SioPutc(Port,'A');

if(SioGetc(Port,18)=='A'))

{puts("Baud rate detected");

/*...do something here...*/

}

}

Page 12

SioBrkKey

Function

Return non-zero if the Control-BREAK key was pressed.

Syntax

int SioBrkKey()

Remarks

The SioBrkKey function returns a TRUE value ( non-zero  )  if  the

Control-BREAK  key  was  pressed,  else  it  returns  a zero.  Use

SioBrkKey as a safety exit from a polling loop.   Don't  mix  this

function up with SioBrkSig.

Returns

-1 : Control-BREAK was pressed.

0 : Control-BREAK was NOT pressed.

Example

#include "ccl.h"

int c;

while(1)

{if(SioBrkKey())

{puts("User typed Contrl-BREAK");

SioDone(Port);

exit(1);

}

if(SioKeyPress())

{c = SioKeyRead();

SioPutc(Port,c);

}

c = SioGetc(Port,0);

if(c!=-1)  SioCrtWrite(c);

}

Page 13

SioBrkSig

Function

Asserts, cancels, or detects BREAK signal.

Syntax

int SioBrkSig(Port,Cmd)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char Cmd;             /* ASSERT, CANCEL, or DETECT */

Remarks

The SioBrkSig function controls the BREAK bit in the  line  status

register. The legal commands are:

ASSERT ('A') to assert BREAK

CANCEL ('C') to cancel BREAK

DETECT ('D') to detect BREAK

ASSERT,  CANCEL,  and  DETECT  are defined in CCL.H. See CALLPGM.C

for an example of the use of SioBrkSig.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-6 : Illegal command. Expected 'A', 'C', or 'D'.

>0 : BREAK signal detected ( DETECT only )

Example

/* Assert BREAK for 1 second */

SioBrkSig(Port,ASSERT);

SioDelay(18);

SioBrkSig(Port,CANCEL);

/* Detect BREAK */

if(SioBrkSig(Port,DETECT))

{puts("BREAK signal detected");

/* ...do some more stuff... */

}

Page 14

SioCrtWrite

Function

Write character to the screen.

Syntax

int SioCrtWrite(ch)

char ch;             /* character to write */

Remarks

The SioCrtWrite function uses the BIOS to write a single character

to the screen at the current cursor location.

SioCrtWrite  is  faster  than a call to the C library and for this

reason is included in CCL.

Returns

zero

Example

#include "ccl.h"

if( (c = SioGetc(COM1,18)) != -1)

{/* echo to screen */

SioCrtWrite(c);

}

Page 15

SioDTR

Function

Set, clear, or read the Data Terminal Ready ( DTR ) bit.

Syntax

int SioDTR(Port,Cmd)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char Cmd;             /* DTR command ( SET, CLEAR, or READ ) */

Remarks

The  SioDTR  function controls the Data Terminal Ready ( DTR ) bit

in the modem control register. Commands ( defined in CCL.H ) are:

SET ('S')  to set DTR ( ON )

CLEAR ('C')  to clear DTR ( OFF )

READ ('R')  to read DTR

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-5 : Not one of 'S', 'C', or 'R'.

0 : DTR is OFF (READ Command).

>0 : DTR is ON (READ Command).

Example

#include "ccl.h"

/* turn DTR on for modem connected to COM4 */

SioDTR(COM4,SET);

Page 16

SioDelay

Function

Delays one or more tics.

Syntax

int SioDelay(tics)

int tics;                     /* # tics */

Remarks

The  SioDelay  function  is  used to delay one or more timer tics,

where each timer tic is approximately 55 milliseconds ( 18 to  the

second ). See SioTimer also.

Returns

zero

Example

#include "ccl.h"

SioDelay(5*18);               /* delay 5 seconds */

Page 17

SioDone

Function

Terminates further serial processing.

Syntax

int SioDone(Port)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

Remarks

The SioDone function terminates further serial processing. SioDone

MUST  be called before exiting your application so that interrupts

can be restored to their original state. Failure to  do  this  can

crash  the operating system.  If you forget to call SioDone before

exiting, be sure to re-boot your computer.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

#include "ccl.h"

/* terminate processing for COM3 */

SioDone(COM3);

Page 18

SioError

Function

Displays error in text.

Syntax

int SioError(Code)

int Code;             /* Error code returned from a CCL function */

Remarks

The SioError function displays the error in text corresponding  to

the   error   code.    During   development  of  a  communications

application, it is a good idea to always test  return  codes,  and

print out their descriptions with SioError.

Returns

none

Example

#include "ccl.h"

code = SioReset(Port,Baud4800);

if(code<0) SioError(code);

Page 19

SioGetc

Function

Reads the next character from the serial line.

Syntax

int SioGetc(Port,Tics)

int Port;             /* COM1 to COM4 */

int Tics;             /* # timer tics */

Remarks

The SioGetc function reads the selected serial port. The  function

will  wait  for  the  number  of  system  tics given by the 'Tics'

argument before returning 'timed out'. There are 18  tics  to  the

second.

To specify no waiting, call SioGetc with Tics = 0.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-1 : If timed out.

>0 : Character read.

Example

#include "ccl.h"

int c;   /* MUST be 'int', not 'char' !!! */

if( (c=SioGetc(COM1,1)) != -1) printf("Character is '%c'\n", c);

else puts("Timed out");

Page 20

SioKeyPress

Function

Detects if keyboard has been pressed.

Syntax

int SioKeyPress()

Remarks

The  SioKeyPress function uses the BIOS to test the keyboard for a

key press.

Returns

zero

Example

#include "ccl.h"

if( SioKeyPress())

{

/* ...do something... */

}

Page 21

SioKeyRead

Function

Reads the keyboard.

Syntax

int SioKeyRead()

Remarks

The SioKeyRead function uses the BIOS to read the keyboard.   Will

wait until a character is typed.

SioKeyRead is faster than using the C library.

Returns

character typed.

Example

#include "ccl.h"

if(SioKeyPress())

{

c = SioKeyRead();

}

Page 22

SioLine

Function

Reads the line status register.

Syntax

int SioLine(Port)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

Remarks

The   SioLine  function  reads  the  line  status  register.   The

individual bit masks are as follows:

0x20  = Transmitter Buffer Empty.

0x10  = Break detected.

0x08  = Framming error.

0x04  = Parity error.

0x02  = Overrun error.

0x01  = Data ready.

The above are documented in the file CCL.H.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

>0 : Line status ( rightmost byte of word ).

Example

#include "ccl.h"

int rc;

rc = LineStatus(Port);

if(rc & (FramingError|ParityError|OverrunError))

{if(rc & FramingError) puts("Framing Error");

if(rc & ParityError)  puts("Parity Error");

if(rc & OverrunError) puts("Overrun Error");

}

else puts("No error");

Page 23

SioModem

Function

Reads the modem status register.

Syntax

int SioModem(Port, Mask)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char Mask;            /* Modem function mask */

Remarks

The   SioModem   function  reads  the  modem  register.   The  bit

definitions for the function mask are as follows:

Bit    CCL.H Name   Function

7      DCD         Data Carrier Detect

6      RI          Ring Indicator

5      DSR         Data Set Ready

4      CTS         Clear To Send

3      DeltaDCD    Delta DCD   ( DCD has changed )

2      DeltaRI     Delta RI    ( RI has changed )

1      DeltaDSR    Delta DSR   ( DSR has changed )

0      DeltaCTS    Delta CTS   ( CTS has changed )

Bits  4 through 7 represent the absolute state of their respective

RS-232 inputs. Bits 0 through 3 repesent a change in the state  of

their respective RS-232 inputs since last read.

The above definitions are also in the CCL.H file for use  by  your

application program.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

>0 : Modem status ( rightmost byte of word ).

Example

#include "ccl.h"

/* any change in DCD or CTS ? */

if(delta=SioModem(Port,DeltaDCD|DeltaCTS))

{status = SioModem(Port,DCD|CTS);

if(delta&DeltaDCD) {if(status&DCD) c='T'; else c='F';

printf("DCD=%c\n",c);

}

if(delta&DeltaCTS) {if(status&CTS) c='T'; else c='F';

printf("CTS=%c\n",c);

}

}

Page 24

SioParms

Function

Sets parity, stop bits, and word length.

Syntax

int SioParms(Port,ParityCode,StopBitsCode,WordLengthCode)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

int ParityCode;       /* parity code [0,1,2] */

int StopBitsCode;     /* stop bits code  [0,1] */

int WordLengthCode;   /* word length code [0,1,2,3] */

Remarks

The SioParms function sets the parity, stop bits, and word length.

If  the default parity ( none ), stop bits ( 1 ), or word length (

8 ) is not  acceptable,  then  they  can  be  changed  by  calling

SioParms.   SioParms  can be called either before or after calling

SioReset. See file CCL.H.

Value      Description    CCL.H Name

ParityCode:   *0        no parity      NoParity

1        odd parity     OddParity

2        even parity    EvenParity

StopBitsCode:  *0        1 stop bit     OneStopBit

1        2 stop bits    TwoStopBits

WordLengthCode: 0        5 data bits    WordLength5

1        6 data bits    WordLength6

2        7 data bits    WordLength7

*3        8 data bits    WordLength8

* = Default

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-7 : Bad parity code selected. Value must be 0 to 2.

-8 : Bad stop bits code. Value must be 0 or 1.

-9 : Bad word length code. Value must be 0 to 3.

Example

#include "ccl.h"

SioParms(COM1,NoParity,OneStopBit,WordLength8);

Page 25

SioPutc

Function

Transmit a character over a serial line.

Syntax

int SioPutc(Port,c)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char c;               /* character to send */

Remarks

The SioPutc function transmits one  character  over  the  selected

serial line.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

#include "ccl.h"

char crc;

char buffer[128];

crc = 0;

for(i=0;i<128;i++)

{crc = crcupdate( buffer[i], crc);

SioPutc(Port, buffer[i]);

}

SioPutc(crc);

Page 26

SioRTS

Function

Sets, clears, or reads the Request  to  Send ( RTS ) line.

Syntax

int SioRTS(Port,Cmd)

int Port;             /* COM1 to COM4 */

char Cmd;             /* RTS command ( SET, CLEAR, or READ ) */

Remarks

The SioRTS function controls the Request to Send ( RTS  )  bit  in

the modem control register. Commands ( defined in CCL.H ) are:

SET ('S')  set RTS ( ON )

CLEAR ('C')  clear RTS ( OFF )

READ ('R')  read RTS

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-5 : Command is not one of 'S', 'C', or 'R'.

0 : RTS is OFF (READ Command).

>0 : RTS is ON  (READ Command).

Example

#include "ccl.h"

/* turn off RTS for modem */

SioRTS(Port,CLEAR);

Page 27

SioReset

Function

Initialize a serial port for processing.

Syntax

int SioReset(Port,BaudCode)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

int BaudCode;         /* baud code */

Remarks

The  SioReset  function  initializes  the  selected  serial  port.

SioReset  should  be called after calling SioParm and SioRxBuf but

before making any other calls to CCL. SioReset  uses  the  parity,

stop  bits,  and  word  length value previously set if SioParm was

called, otherwise the default values ( see SioParm ) are used.

Recall  that  COM1  and  COM3  share the same interrupt vector and

therefore cannot operate  simultaneously.   Similiarly,  COM2  and

COM4  cannot  operate simultaneously. Any other combination of two

ports can be used.

See SioBaud for a list of the baud rate codes, or see "CCL.H".

Returns

-4 : Bad port selected. Value must be 0 to 3.

-11 : Bad baud rate code selected. Value must be 0 to 9.

Example

#include "ccl.h"

char far Buffer[128];

int rc;

SioRxBuf(COM1,Buffer,Size128);

rc = SioReset(Com1,Baud38400);

if(rc==0) puts("RESET ok");

else if(rc<0) SioError(rc);

else

{if(rc&OverrunError) puts("Overrun Error");

if(rc&ParityError) puts("Parity Error");

if(rc&FramingError) puts("Framing Error");

if(rc&BreakDetected) puts("Break Detected");

}

Page 28

SioRxBuf

Function

Sets up receive buffers.

Syntax

int  SioRxBuf(Port,Buffer,SizeCode)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char far *Buffer;     /* Receive buffer */

int  SizeCode;        /* Buffer size code */

Remarks

The SioRxBuf function passes the address and size of  the  receive

buffer  to CCL. Recall that CCL requires a receive buffer for each

port in simultaneous  operation  since  the  receive  function  is

interrupt  driven.   SioRxBuf passes the receive buffer to CCL for

both the primary ( COM1/COM3 ) and secondary ( COM2/COM4 )  ports.

It  must be called before any incoming characters can be received.

SioRxBuf should be called before SioReset. Buffer size  codes  are

listed in "CCL.H".

Size Code       Buffer Size    CCL.H Name

0              8 bytes        Size8

1             16 bytes        Size16

2             32 bytes        Size32

3             64 bytes        Size64

4            128 bytes        Size128

5            256 bytes        Size256

6            512 bytes        Size512

7           1024 bytes        Size1024

8           2048 bytes        Size2048

9           4096 bytes        Size4096

10           8192 bytes        Size8192

11          16384 bytes        Size16384

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

-10 : Bad buffer size code. Value must be between 0 and 11.

Example

#include "ccl.h"

char far RxBuf[128];

SioRxBuf( COM1, RxBuf, 128);

Page 29

SioRxFlush

Function

To flush the receive buffer associated with the specified port.

Syntax

int SioRxFlush(Port)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

Remarks

The  SioRxFlush function will delete any characters in the receive

buffer for the  specified  port.   After  execution,  the  receive

buffer  will  be  empty.  Call  SioRxBuf after resetting a port in

order to delete any spurious characters.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

#include "ccl.h"

char far buffer[1024];

/* setup receive buffer & reset port */

SioRxBuf(COM1,buffer,Size1024);

SioReset(COM1,Baud115200);

/* flush any spurious character */

SioRxFlush(COM1);

/* ready for serial processing ! */

Page 30

SioRxQue

Function

Returns the number of characters in the receive queue.

Syntax

int SioRxQue(Port)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

Remarks

The SioRxQue function will return the number of characters in  the

receive queue. It can be used to implement XON/XOFF flow control.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

#include "ccl.h"

#define XON 0x11

#define XOFF 0x13

int count;

char last = XON;

char far Buffer[128];

SioRxBuf(COM1,Buffer,Size128);

/* implement XON / XOFF */

count = SioRxQue(COM1);

if((last==XON)&&(count>120))

{SioPutc(COM1,XOFF);

last = XOFF;

}

if((last==XOFF)&&(count<8))

{SioPutc(COM1,XON);

last = XON;

}

Page 31

SioTimer

Function

Returns the number of system clock tics since midnight.

Syntax

long SioTimer()

Remarks

The SioTimer function will return the number of system clock  tics

since  midnight,  at  18.2065  tics  per second.  This function is

usefull for timeing various functions. Also see SioDelay.

Returns

timer tics

Example

#include "ccl.h"

long time;

time = SioTimer();

/* do some stuff */

printf("Elasped time = %ld tics\n", SioTimer() - time );

Page 32

SioUnGetc

Function

"Un-gets" the last character read with SioGetc().

Syntax

int SioUnGetc(Port,c)

int Port;             /* Port selected (COM1,COM2,COM3,or COM4) */

char c;               /* character to unget */

Remarks

The SioUnGetc function returns ( pushes ) the character back  into

the  serial  input  buffer.  The character pushed will be the next

character returned by SioGetc. Only one character  can  be  pushed

back. This function works just like the "ungetc" function in the C

language.

Returns

-2 : Port not enabled. Must call SioReset first.

-4 : Bad port selected. Value must be 0 to 3.

Example

#include "ccl.h"

SioUnGetc(Port,c);

Page 33

Function Sumary

+-------------+----------+----------+----------+----------+

| Function    |   Arg1   |   Arg2   |   Arg3   |   Arg4   |

+-------------+----------+----------+----------+----------+

| SioBaud     | Port     | BaudCode |          |          |

+-------------+----------+----------+----------+----------+

| SioBrkKey   |          |          |          |          |

+-------------+----------+----------+----------+----------+

| SioBrkSig   | Port     | Cmd      |          |          |

+-------------+----------+----------+----------+----------+

| SioCrtWrite | Ch       |          |          |          |

+-------------+----------+----------+----------+----------+

| SioDTR      | Port     | Cmd      |          |          |

+-------------+----------+----------+----------+----------+

| SioDelay    | Tics     |          |          |          |

+-------------+----------+----------+----------+----------+

| SioDone     | Port     |          |          |          |

+-------------+----------+----------+----------+----------+

| SioError    | Code     |          |          |          |

+-------------+----------+----------+----------+----------+

| SioGetc     | Port     | Tics     |          |          |

+-------------+----------+----------+----------+----------+

| SioKeyPress |          |          |          |          |

+-------------+----------+----------+----------+----------+

| SioKeyRead  |          |          |          |          |

+-------------+----------+----------+----------+----------+

| SioLine     | Port     |          |          |          |

+-------------+----------+----------+----------+----------+

| SioModem    | Port     | Mask     |          |          |

+-------------+----------+----------+----------+----------+

| SioParms    | Port     | Parity   | StopBits |WordLength|

+-------------+----------+----------+----------+----------+

| SioPutc     | Port     | Ch       |          |          |

+-------------+----------+----------+----------+----------+

| SioRTS      | Port     | Cmd      |          |          |

+-------------+----------+----------+----------+----------+

| SioReset    | Port     | BaudCode |          |          |

+-------------+----------+----------+----------+----------+

| SioRxBuf    | Port     | Buffer   | SizeCode |          |

+-------------+----------+----------+----------+----------+

| SioRxFlush  | Port     |          |          |          |

+-------------+----------+----------+----------+----------+

| SioRxQue    | Port     |          |          |          |

+-------------+----------+----------+----------+----------+

| SioTimer    |          |          |          |          |

+-------------+----------+----------+----------+----------+

| SioUnGetc   | Port     | Ch       |          |          |

+-------------+----------+----------+----------+----------+

Page 34

Error Code Summary

Code        Description

0          No error.

-1          Timeout waiting for input. Only returned by SioGetc.

-2          Port not enabled. Must call SioReset first.

-3          No buffer available. Must call SioRxBuf before

calling SioReset.

-4          Bad port specified. Must be 0 to 3. Recall that COM1

is port 0, COM2 is port 1, etc.

-5          Expected 'S', 'C', or 'R' as second argument.

-6          Expected 'A', 'C', or 'D' as second argument.

-7          Bad parity code specified. Must be 0 to 7.

-8          Bad stop bits code specified. Must be 0 or 1.

-9          Bad wordlength code specified. Must be 0 to 3.

-10          Bad buffer size code specified. Must be 0 to 11.

-11          Bad baud rate code. Must be 0 to 9.

Page 35