Cport

C Language Serial Communications Library

Version 1.30

30 Oct 91

Copyright (c) 1991  Bri Productions

1

Table of Contents

Introduction

Acknowledgments.........................5

General Description.....................5

Disclaimer..............................5

Registration............................5

Contacting the Author...................6

A typedef...............................6

Terminology.............................6

Control Functions

Description.............................7

ComOpen.................................8

ComOpenS...............................12

ComActive..............................15

ComClose...............................17

ComCloseAll............................18

ComParam...............................18

ComBaud................................19

ComMode................................21

ComHandshake...........................22

ComTx..................................24

Input Functions

Description............................25

ComGetc................................26

ComGets................................27

ComIn..................................28

ComFlushRx.............................29

2

ComLenRx...............................30

ComPeek................................31

Output Functions

Description............................33

ComPutc................................34

ComPuts................................35

ComOut.................................36

ComFlushTx.............................37

ComLenTx...............................38

Status Functions

Description............................39

ComError...............................40

ComStatus..............................41

ComRts.................................42

ComDtr.................................43

ComOut1................................44

Data Integrity

Description............................46

ComChecksum............................47

ComCrc16...............................48

Miscellaneous Functions

Description............................50

ComSetBreak............................51

ComClrBreak............................51

ComPutScrtch...........................52

ComGetScrtch...........................53

3

Appendix A - Things to Look Out For............55

Appendix B - Handshaking Schemes...............56

Appendix C - The UART Chip.....................58

Appendix D - The RS-232 Standard ?!!...........59

Appendix E - Registration Form.................60

4

Acknowledgments.

I would like to thank Earl Jensen and Steve Hill for technical

review, Diana Hill for proofreading the documentation, Chip

Curry and Luxtron Corp for beta testing, John Kerwin for

technical advice, and Ton Kneist and No*l Gaskell for

facilitating development.

General Description.

Cport is a C language interrupt-driven serial communication

library for IBM and compatible computers. Cport is compatible

with most if not all DOS based C compilers.

Some of Cport's features are:

� Supports COM 1 to COM 4 and special configurations.

� Limited simultaneous communications.

� Baud rates from 50 to 115200 baud.

� Built in hardware and software handshaking.

� Both transmitter and receiver are interrupt-driven.

� Adjustable transmit and receive queues.

� Queues as large as 65534 bytes each.

� Transmit and receive queues are handled internally.

� Supports the NS16550 UART automatically.

� Written in assembly language for optimum speed and efficiency.

DISCLAIMER.

Cport is provided AS IS. Bri Productions specifically disclaims

any and all warranties, expressed or implied, including fitness

for a particular purpose. Use this product at your own risk.

Registration.

Cport is a user supported software product. Distribution of

this product, unaltered and without charge, is encouraged. If

you find this product useful, you are encouraged to register

your copy. This allows Bri Productions to continue to provide

and support low cost, high quality shareware.

The registration fee is $35.00 US dollars and includes libraries

for small, medium, compact and large memory models and complete

source code. A registration form is provided in Appendix E.

5

Contacting the author.

Bri Productions may be contacted by any of the following means:

CompuServe      -       76635,2246

U.S. mail       -       Bri Productions

P.O.Box 7121

Fremont, CA 94537-7121

CompuServe is a trademark of CompuServe Inc.

A typedef.

You will see the data type 'byte' throughout the Cport library.

This is a typedef defined in cport.h. Byte is an unsigned

character used for byte size values.

Terminology

This section defines some of the terminology used in this

and other Cport documentations.

'Remote device' refers to the device that the computer is

communicating with. The actual device may be a modem, serial

printer or other communications device.

If a serial port is 'open' it is currently performing or ready to

perform serial communications. All 'open' ports are either 'active'

or on the 'active stack'.

If a serial port is 'active' it is currently recognized by

Cport functions. For example, calling ComBaud will set the

baud rate of the 'active' serial port and calling ComGetc

will fetch a character/byte from the 'active' serial port's

receive queue.

If a serial port is on the 'active stack', it continues to

transmit and receive but is not recognized by the Cport

functions.

6

Control Functions

Control functions manage the parameters and the modes in

which the serial port will operate. There are two sets of

two functions, at least one from each set is mandatory:

ComOpen or ComOpenS, and ComClose or ComCloseAll.

ComOpen and/or ComOpenS open a serial port and sets the

parameters and modes. ComOpen takes discrete parameters while

ComOpenS takes a pointer to a structure of parameters.

ComClose and ComCloseAll close the active and all opened

serial port(s) respectively. ComCloseAll is generally used

in simultaneous communications for conveniently ensuring

all serial ports are closed. For single serial port

communications ComClose is sufficient. Failing to close

all opened serial ports prior to a program's termination

will cause the program to crash.

If more that one serial port is open, ComActive can be used

to make any of the opened serial ports the active serial port.

ComBaud and ComMode modify one or more of the parameters/modes

of the serial port. ComParam reports on the current parameters

and handshaking under which the currently active serial port

is operating.

ComHandshake determines if and what type of handshaking

scheme will be enabled and sets the receiver threshold. ComTx

turns the transmitter on and off, allowing you to implement

your own handshaking scheme. See Appendix B for a description

of handshaking.

7

----------------------------------------------------------------

ComOpen

----------------------------------------------------------------

Function

Opens serial port and makes it active.

Syntax

#include "cport.h"

int ComOpen(unsigned com, int baud, byte mode,

unsigned rxQ, unsigned txQ);

Parameters

com   - Constant that defines the serial port to be

opened. The upper 4 bits define the interrupt

vector while the lower 12 bits define the

port address.Possibilities are:

* COM1  (PORT0 | IRQ4)

* COM2  (PORT1 | IRQ3)

* COM3  (PORT2 | IRQ4)

* COM4  (PORT3 | IRQ3)

PORT0 0x03F8

PORT1 0x02F8

PORT2 0x03E8

PORT3 0x02E8

IRQ2  0xA000

IRQ3  0xB000

IRQ4  0xC000

IRQ5  0xD000

IRQ6  0xE000

IRQ7  0xF000

BIOS0 (*(int far *)0x400000l)

BIOS1 (*(int far *)0x400002l)

BIOS2 (*(int far *)0x400004l)

BIOS3 (*(int far *)0x400006l)

* The COMx constants are usually sufficient

baud  - Constant defining the baud rate to which the

serial port will be initialized. Possibilities

are:

B115200    (1)

B57600     (2)

B38400     (3)

B19200     (6)

B9600     (12)

8

B7200     (16)

B4800     (24)

B3600     (32)

B2400     (48)

B2000     (58)

B1800     (64)

B1200     (96)

B600     (192)

B300     (384)

B150     (768)

B110    (1047)

B75     (1536)

B50     (2304)

mode  - Bit ORed constant defining the word

length, number of stop bits and the

parity that the serial port will use.

Possibilities are:

W8  (0x03)  -  8 bit words

W7  (0x02)  -  7 bit words

W6  (0x01)  -  6 bit words

W5  (0x00)  -  5 bit words

S1  (0x00)  -  1 stop bit

S2  (0x04)  -  2 stop bits

NONE   (0x00) -  No parity

ODD    (0x08) -  Odd parity

EVEN   (0x18) -  Even parity

MARK   (0x28) -  Mark parity

SPACE  (0x38) -  Space parity

* rxQ   - Size, in bytes, of the receive queue.

* txQ   - Size in bytes, of the transmit queue.

* Limited to 32K in small and medium

memory models.

Remarks

ComOpen opens the serial port 'com' and makes it the

active serial port. If a serial port has been previously

opened, it will be pushed into the active stack.

The 4 most significant bits in 'com' represent the

interrupt vector, while the lower 12 bits represent

the port address. Constants are provided in cport.h

to facilitate the formulation of the 'com' parameter.

The COMx values will work in most cases, however, the

IRQx values can be ORed with the PORTx or BIOSx

values to produce other combinations. The PORTx

values are actual port addresses where the serial

9

ports are normally located. The BIOSx values are

actually macros that fetch the port address from the

BIOS data area. Usually the BIOSx and PORTx values

will be the same but it is not guaranteed. Quite

often the values at BIOS2 and BIOS3 (com 3 and com 4)

will be set to 0 regardless if com3 or com4 exist.

The BIOSx macros can also be used to set the values

in the BIOS data area as well, for example:

if(!BIOS2)

BIOS2 = PORT2;

The parameter 'baud' is the baud rate divisor which

determines the baud rate. Constants are provided in

cport.h for the most common baud rates. The 'baud'

rate divisor is defined as f / (16 * baud rate) where

f is 1.8432e+6.

Achieving baud rates above 19200 is largely dependent

on many variables such as machine speed, cable length,

handshaking activity, degree bidirectional transfers,

number of open serial ports, the type UART(s) used

(see Appendix C) and activity of other hardware interrupts.

The parameter 'mode' determines the word length,

number of stop bits and parity of the exchanged data.

Bits 0 & 1 determine the word length, bit 2

determines the number of stop bits, bit 3 enables

parity and bits 4 & 5 determine the type of parity

check. Constants which can be ORed together are

provided in cport.h.

'RxQ' and 'txQ' set the size of the receive and transmit

queues respectively. 'RxQ' should be at least 4 times

larger than the largest string or block of data the

programmer intends to fetch from the queue at once. '

TxQ' can be set as small as 1, but is not recommended.

In the small and medium memory models, where all data

is limited to 64K, the size queues are limited. In

the compact and large memory models, each queue can

be as large as 64K -1 (65534 bytes).

Return value

If successful, ComOpen returns 0. In the event of an

error, it returns one of the following error codes:

OPENED    (1) -  Port already opened

BAD_COM   (2) -  Bad 'com' parameter.

NO_UART   (3) -  No UART chip detected.

RCV_ALC   (4) -  Receive queue allocation error.

TX_ALC    (5) -  Transmit queue allocation error.

10

See also

ComOpenS ComClose ComCloseAll ComBaud ComMode ComActive

Example

Open COM1 at 9600 baud with 8 bit words, 1 stop bit,

no parity, a 1024 byte receiver and a 512 byte

transmitter.

{

int rv;

rv = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

if(rv)

{

...error handler

}

...more code

}

----------------------------------------------------------------

ComOpenS

----------------------------------------------------------------

Function

Opens serial port and makes it active.

Syntax

#include "cport.h"

int ComOpenS(const C_param *param);

Parameters

param   - Pointer to the structure of type 'C_param'.

The structure 'C_param' is defined as follows:

struct C_param{

unsigned com;

int      baud;

byte     mode;

unsigned rxQ;

unsigned txQ;

byte     htype;

11

unsigned thresh;

};

com   - Constant that defines the serial port to be

opened. The upper 4 bits define the interrupt

vector while the lower 12 bits define the

port address.Possibilities are:

* COM1  (PORT0 | IRQ4)

* COM2  (PORT1 | IRQ3)

* COM3  (PORT2 | IRQ4)

* COM4  (PORT3 | IRQ3)

PORT0 0x03F8

PORT1 0x02F8

PORT2 0x03E8

PORT3 0x02E8

IRQ2  0xA000

IRQ3  0xB000

IRQ4  0xC000

IRQ5  0xD000

IRQ6  0xE000

IRQ7  0xF000

BIOS0 (*(int far *)0x400000l)

BIOS1 (*(int far *)0x400002l)

BIOS2 (*(int far *)0x400004l)

BIOS3 (*(int far *)0x400006l)

* The COMx constants are usually sufficient

baud  - Constant defining the baud rate to which

the serial port will be initialized.

Possibilities are:

B115200    (1)

B57600     (2)

B38400     (3)

B19200     (6)

B9600     (12)

B7200     (16)

B4800     (24)

B3600     (32)

B2400     (48)

B2000     (58)

B1800     (64)

B1200     (96)

B600     (192)

B300     (384)

B150     (768)

B110    (1047)

B75     (1536)

B50     (2304)

12

mode  - Bit ORed constant defining the word

length, number of stop bits and the

parity that the serial port will use.

Possibilities are:

W8  (0x03)  -  8 bit words

W7  (0x02)  -  7 bit words

W6  (0x01)  -  6 bit words

W5  (0x00)  -  5 bit words

S1  (0x00)  -  1 stop bit

S2  (0x04)  -  2 stop bits

NONE   (0x00) -  No parity)

ODD    (0x08) -  Odd parity)

EVEN   (0x18) -  Even parity)

MARK   (0x28) -  Mark parity)

SPACE  (0x38) -  Space parity)

* rxQ  - Size, in bytes, of the receive queue.

* txQ   - Size in bytes, of the transmit queue.

* Limited to 32K in small and medium

memory models.

htype - Type of handshaking to be used. Possibilities are:

OFF   (0)    - No handshaking

DTR   (0x01) - The receiver uses DTR

RTS   (0x02) - The receiver uses RTS

S_RX  (0x04) - The receiver uses XON and XOFF

characters

S_TX  (0x08) - XON and XOFF characters are

honored by the transmitter

CTS   (0x10) - CTS is checked by the transmitter

DSR   (0x20) - DSR is checked by the transmitter

DCD   (0x80) - DCD is checked by the transmitter

SOFT  (0x0C) - Same as (S_RX | S_TX)

* HARD  (0x12) - Same as (RTS | CTS)

HARD1 (0x21) - Same as (DTR | DSR)

* HARD2 (0x12) - Same as (RTS | CTS)

* The constants HARD and HARD2 have the same value. HARD

is retained to maintain compatibility with older versions

of Cport.

NOTE: See Appendix B for a more detailed discussion on

handshaking schemes.

thresh - Receive queue threshold.

13

Remarks

ComOpenS is an alternative to ComOpen for opening

a serial port. It takes a pointer to a structure of

parameters instead of taking discrete parameters.

It also can set the handshaking without making a

separate call to ComHandshake.If a serial port has

been previously opened, it will be pushed into the

active stack.

The 4 most significant bits in 'com' represent the

interrupt vector, while the lower 12 bits represent

the port address. Constants are provided in cport.h

to facilitate the formulation of the 'com' parameter.

The COMx values will work in most cases, however, the

IRQx values can be ORed with the PORTx or BIOSx

values to produce other combinations. The PORTx

values are actual port addresses where the serial

ports are normally located. The BIOSx values are

actually macros that fetch the port address from the

BIOS data area. Usually the BIOSx and PORTx values

will be the same but it is not guaranteed. Quite

often the values at BIOS2 and BIOS3 (com 3 and com 4)

will be set to 0 regardless if com3 or com4 exist.

The parameter 'baud' is the baud rate divisor which

determines the baud rate. Constants are provided in

cport.h for the most common baud rates. The 'baud'

rate divisor is defined as f / (16 * baud rate) where

f is 1.8432e+6.

Achieving baud rates above 19200 is largely dependent

on many variables such as machine speed, cable length,

handshaking activity, degree bidirectional transfers,

number of open serial ports, the type UART used

(see Appendix C) and activity of other hardware interrupts.

The parameter 'mode' determines the word length,

number of stop bits and parity of the exchanged data.

Bits 0 & 1 determine the word length, bit 2

determines the number of stop bits, bit 3 enables

parity and bits 4 & 5 determine the type of parity

check. Constants which can be ORed together are

provided in cport.h.

'RxQ' and 'txQ' set the size of the receive and transmit

queues respectively. 'RxQ' should be at least 4 times

larger than the largest string or block of data the

programmer intends to fetch from the queue at once.

'TxQ' can be set as small as 1, but is not recommended.

In the small and medium memory models, where all data

is limited to 64K, the size queues are limited. In

the compact and large memory models, each queue can

be as large as 64K -1 (65534).

14

The parameter 'htype' defines the type of flow

control (handshaking) to be used by the transmitter

and receiver. 'Threshold' determines when the

receiver will assert of disassert the selected

handshaking.

Return value

If successful, ComOpenS returns 0. In the event of an

error, it returns one of the following error codes:

OPENED    (1) -  Port already opened

BAD_COM   (2) -  Bad 'com' parameter.

NO_UART   (3) -  No UART chip detected.

RCV_ALC   (4) -  Receive queue allocation error.

TX_ALC    (5) -  Transmit queue allocation error.

See also

ComOpen ComParam ComHandshake ComClose ComCloseAll

Example

Open COM2 with the same parameters as COM1.

{

struct C_param param;

ComActive(COM1);

ComParam(&param);

param.com = COM2;

ComOpenS(&param);

}

----------------------------------------------------------------

ComActive

----------------------------------------------------------------

Function

Activates a currently opened serial port.

Syntax

#include "cport.h"

int ComActive(unsigned com);

15

Parameters

com - Opened serial port which is to be made active.

Remarks

ComActive activates the opened serial port specified

by 'com'. The value of 'com' is the same the 'com'

parameter which was passed to ComOpen or ComOpenS

when the serial port was opened.

The specified serial port is taken off the active

stack and the previously active serial port is pushed

onto the active stack. For example:

before:

active stack + 1 = COM1

active stack + 0 = COM2

active = COM3

after: ComActive(COM1);

active stack + 1 = COM2

active stack + 0 = COM3

active = COM1

Return value

If successful, ComActive returns 0. Otherwise it

will return an error code. Possibilities are:

BAD_COM  (2) - bad 'com' parameter

NOT_OPEN (8) - 'com' is not open

See Also

ComOpen ComOpenS ComClose

Example

If there is data available on COM1, get it and send it

out on COM2.

{

byte buf[128];

unsigned len;

ComActive(COM1);

if(ComLenRx())

16

{

len = ComIn(buf, 128);

ComActive(COM2);

ComOut(buf, len);

}

}

----------------------------------------------------------------

ComClose

----------------------------------------------------------------

Function

Closes the currently active serial port.

Syntax

#include "cport.h"

void ComClose(void);

Remarks

ComClose closes the currently active serial port. The

serial port on the top of the active stack (most recently

active) subsequently becomes the active serial port.

Failing to close all serial ports before a program

is terminated will cause the program to crash.

See also

ComCloseAll ComOpen ComOpenS

Example

Close the only opened serial port prior to terminating.

{

...closing code

ComClose();     /* only one serial port open */

exit(0)

}

17

----------------------------------------------------------------

ComCloseAll

----------------------------------------------------------------

Function

Closes all the currently opened serial ports.

Syntax

#include "cport.h"

void ComCloseAll(void);

Remarks

ComCloseAll closes all the currently opened serial ports.

This includes the active serial port and any serial

ports on the active stack.

Failing to close all serial ports before a program

is terminated will cause the program to crash.

See also

ComClose ComOpen ComOpenS

Example

Make sure all serial ports are closed prior to

terminating.

{

...closing code

ComCloseAll();

exit(0)

}

----------------------------------------------------------------

ComParam

----------------------------------------------------------------

Function

Retrieves the parameters of the currently active serial

port.

18

Syntax

#include "cport.h"

void ComParam(const struct C_param *param);

Remarks

ComParam retrieves the parameters and handshaking

status of the currently active serial port and stores

them in a structure of type 'C_param'. The structure

'C_param' is defined as follows:

struct C_param{

unsigned com;

int      baud;

byte     mode;

unsigned rxQ;

unsigned txQ;

byte     htype;

unsigned thresh;

};

See ComOpenS for a discussion of the individual

elements.

See also

ComOpen ComOpenS ComMode ComBaud

Example

Open COM2 with the same parameters as COM1.

{

struct C_param param;

ComActive(COM1);

ComParam(&param);

param.com = COM2;

ComOpenS(&param);

}

----------------------------------------------------------------

ComBaud

----------------------------------------------------------------

Function

Changes the baud rate of the currently active serial

port.

19

Syntax

#include "cport.h"

void ComBaud(int baud);

Parameters

baud  - Constant defining the baud rate to which

the serial port will be initialized.

Possibilities are:

B115200    (1)

B57600     (2)

B38400     (3)

B19200     (6)

B9600     (12)

B7200     (16)

B4800     (24)

B3600     (32)

B2400     (48)

B2000     (58)

B1800     (64)

B1200     (96)

B600     (192)

B300     (384)

B150     (768)

B110    (1047)

B75     (1536)

B50     (2304)

Remarks

The parameter 'baud' is the baud rate divisor which

determines the baud rate. Constants are provided in

cport.h for the most common baud rates. The 'baud'

rate divisor is defined as f / (16 * baud rate) where

f is 1.8432e+6.

Achieving baud rates above 19200 are largely dependent

on many variables such as machine speed, cable length,

handshaking activity, degree bidirectional transfers,

the type of UART used, number of opened serial ports and

activity of other hardware interrupts.

See also

ComOpen ComOpenS ComParam ComMode

Example

Set the currently active serial port to the next baud

rate in the table 'baud'.

20

{

static int indx = 0;

int baud[4] = { B9600,

B2400,

B1200,

B300

};

indx++;

indx %= 4;

ComBaud(baud[indx]);

}

----------------------------------------------------------------

ComMode

----------------------------------------------------------------

Function

Changes the word length, number of stop bits and parity

of the active serial port.

Syntax

#include "cport.h"

void ComMode(int mode);

Parameters

mode  - Bit ORed constant defining the word length,

number of stop bits and the parity that the

serial port will use. Possibilities are:

W8  (0x03)  -  8 bit words

W7  (0x02)  -  7 bit words

W6  (0x01)  -  6 bit words

W5  (0x00)  -  5 bit words

S1  (0x00)  -  1 stop bit

S2  (0x04)  -  2 stop bits

NONE   (0x00) -  No parity)

ODD    (0x08) -  Odd parity)

EVEN   (0x18) -  Even parity)

MARK   (0x28) -  Mark parity)

SPACE  (0x38) -  Space parity)

21

Remarks

The parameter 'mode' determines the word length,

number of stop bits and parity of the exchanged data.

Bits 0 & 1 determine the word length, bit 2

determines the number of stop bits, bit 3 enables

parity and bits 4 & 5 determine the type of parity

check. Constants that can be ORed together are

provided in cport.h.

See also

ComOpen ComOpenS ComParam ComBaud

Example

Toggle the currently active serial port to the next mode

in the table 'mode'.

{

static int indx = 0;

int mode[2] = { W8|S1|NONE,

W7|S1|EVEN,

};

indx ^= 1;

ComMode(mode[indx]);

}

----------------------------------------------------------------

ComHandshake

----------------------------------------------------------------

Function

Enables or disables flow control (handshaking).

Syntax

#include "cport.h"

void ComHandshake(byte htype, unsigned thresh);

Parameters

htype - Type of handshaking to be used. Possibilities are:

OFF   (0)    - No handshaking

DTR   (0x01) - The receiver uses DTR

RTS   (0x02) - The receiver uses RTS

S_RX  (0x04) - The receiver uses XON and XOFF

characters

22

CTS   (0x10) - CTS is checked by the transmitter

DSR   (0x20) - DSR is checked by the transmitter

DCD   (0x80) - DCD is checked by the transmitter

S_TX  (0x08) - XON and XOFF characters are

honored by the transmitter

SOFT  (0x0C) - Same as (S_RX | S_TX)

* HARD  (0x12) - Same as (RTS | CTS)

HARD1 (0x21) - Same as (DTR | DSR)

* HARD2 (0x12) - Same as (RTS | CTS)

* The constants HARD and HARD2 have the same value. HARD

is retained to maintain compatibility with older versions

of Cport.

NOTE: See Appendix B for a more detailed discussion on

handshaking schemes.

thresh - Receive queue threshold.

Remarks

ComHandshake enables or disables software and/or

hardware handshaking. In software handshaking, XON

and XOFF characters are transmitted and received to

suspend and resume communications. In hardware

handshaking uses any or all of DTR, DSR, RTS, CTS

and DCD lines to suspend and resume communications.

'Thresh' determines when the receiver will assert and

disassert.

See also

ComParam ComOpenS ComTx

Example

If COM1 is successfully opened, turn on some hardware

handshaking and set the receiver threshold to 3/4 of the

receiver's full size. The transmitter will not transmit

unless both DSR and DCD are true. The receiver will

disassert DTR if the receive queue length exceeds the

threshold.

{

int rv;

rv = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

if(rv)

{

...error handler

}

ComHandshake(DSR|DTR|DCD, RCVQ_SIZE * 3 / 4);

23

...more code

}

----------------------------------------------------------------

ComTx

----------------------------------------------------------------

Function

Turns the transmitter on or off

Syntax

#include "cport.h"

void ComTx(byte on_off);

Parameters

on_off - Determines whether the transmitter is to be

turned on or off. Constants are provided in

cport.h. Possibilities are:

ON  (1) - Turns the transmitter on

OFF (0) - Turns the transmitter off

Remarks

ComTx allows you to turn the transmitter on and off

to suspend and resume transmissions. This function

can be very useful when implementing your own

handshaking scheme.

See also

ComHandshake ComOpenS ComParam

Example

If the transmitter is off ('off') and needs to be

turned on ('turn_on'), turn it on. If the transmitter

is on and needs to be turned off, turn it off.

{

if(off && turn_on)

ComTx(ON);

else if(!off && turn_off)

ComTx(OFF);

}

24

Input Functions

Input fuctions interface with the active serial port's receiver

and the internal receive queue.

There are four functions for fetching from the receive

queue. ComGetc and ComGets can be used to fetch control codes

as well as normal ascii characters, while ComIn is intended

for fetching binary data. ComPeek returns a copy of the head

character/byte without removing it from the queue.

ComFlushRx flushes the receiver while ComLenRx finds

the number of characters/bytes in the queue.

25

----------------------------------------------------------------

ComGetc

----------------------------------------------------------------

Function

Fetches a character from the active serial port's

receive queue.

Syntax

#include "cport.h"

char ComGetc(void);

Remarks

ComGetc fetches the head character from the active

serial port's receive queue. If there are no characters

in the receive queue, ComGetc returns a null character

('\0').

Return value

On Success, ComGetc returns the character from the

receive queue. If it fails it returns a '\0'.

See also

ComPutc ComGets ComIn ComPeek

Example

Get a character from the serial port and test if it

is a start of header control character.

#define SOH 1

{

char c;

c = ComGetc();

if(c == SOH)

{

...SOH code

}

else

return c;

}

26

----------------------------------------------------------------

ComGets

----------------------------------------------------------------

Function

Fetches a string of characters from the active serial

port's  receive queue.

Syntax

#include "cport.h"

char *ComGets(char *str, int max_c, char term_c);

Parameters

*str  - Pointer to the buffer where the string of

characters will be stored.

max_c - Maximum number of characters to fetch.

term_c - Termination character. ComGets will return

if this character is encountered.

Remarks

ComGets fetches a string of characters from the

active serial port's receive queue. The string will

be stored in 'str'. 'str' must be at least 'max_c' + 1

long to allow for the null terminator.

ComGets will return on the first occurrence of one of

three conditions: 1) the character 'term_c' is

encountered; 2) 'max_c' characters are reached; or 3)

No more characters are available in the receive queue.

Return value

ComGets returns a pointer to the null terminated

string.

See also

ComPuts ComGetc ComIn ComLenRx

Example

Get a line of max 80 characters from the receive queue

and store it in 'str'. The string will be echoed to the

display.

27

{

char str[81];

if(ComLenRx() > 80)

puts(ComGets(str, 80, '\n'));

}

----------------------------------------------------------------

ComIn

----------------------------------------------------------------

Function

Fetches a block of bytes from the active serial port's

receive queue.

Syntax

#include "cport.h"

unsigned ComIn(void *abyte, unsigned num_byte);

Parameters

*abyte - Pointer to the buffer where the block of bytes

will be stored.

num_byte - Number of bytes to get from the receive queue.

Remarks

ComIn fetches a block of bytes from active serial port's

the receive queue and stores it in 'abyte'. ComIn will

always fetch 'num_byte' bytes unless not enough bytes are

available. ComIn is well suited for receiving binary data.

Return value

ComIn returns the number of bytes fetched.

See also

ComOut ComGetc ComGets ComLenRx

Example

Get a block of 1024 bytes from the serial port.

#define BLK_SZ 1024

{

28

byte block[BLK_SZ];

unsigned rs;

if(ComLenRx() > BLK_SZ)

{

rs = ComIn(block, BLK_SZ);

if(rs < BLK_SZ)

{

...error handler

}

}

}

----------------------------------------------------------------

ComFlushRx

----------------------------------------------------------------

Function

Flushes the active serial port's receiver and receive

queue.

Syntax

#include "cport.h"

void ComFlushRx(void);

Remarks

ComFlushRx flushes the active serial port's receive

queue and the UART of any existing characters/bytes.

See also

ComLenRx ComFlushTx

Example

If a receive flush command is in order, flush the

receiver.

#define ...

#define RCV_FLSH 2

#define ...

{

int command;

...fetch command

switch(command)

{

29

case ...:

...case code

break;

case RCV_FLSH:

ComFlushRx();

break;

case ...:

...case code

break;

}

}

----------------------------------------------------------------

ComLenRx

----------------------------------------------------------------

Function

Calculates the number of characters in the active serial

port's receive queue.

Syntax

#include "cport.h"

unsigned ComLenRx(void);

Remarks

ComLenRx calculates the number of characters in the

active serial port's receive queue. Valid numbers are 0

to 65534.

Return value

ComLenRx returns the number of characters presently

residing in the receive queue.

See also

ComFlushRx ComLenTx

Example

If there are more than 80 characters available, get a

line.

{

char str[81];

30

if(ComLenRx() > 80)

puts(ComGets(str, 80, '\n'));

}

----------------------------------------------------------------

ComPeek

----------------------------------------------------------------

Function

Fetches a copy of the head character in the active

serial port's receive queue.

Syntax

#include "cport.h"

char ComPeek(void);

Remarks

ComPeek fetches a copy of the head character in the

active serial port's receive queue. ComPeek will not

remove the character from the queue.

Return value

ComPeek returns the copy of the head character.

See also

ComGetc

Example

Determine if the head character is a control character.

#define CTRL_A 1

#define CTRL_Z 26

{

char peek_c;

char str[81];

peek_c = ComPeek();

if(peek_c >= CTRL_A && peek_c <= CTRL_Z)

{

switch(ComGetc())

{

31

...command processor

}

}

else

puts(ComGets(str, 80, '\n');

}

32

Output Functions

Output functions interface with the active serial port's

transmitter and it's internal transmit queue.

There are three functions for putting characters/bytes into

the transmit queue, all of which return the actual number of

characters/bytes successfully put. ComPutc and ComPuts can be

used to put control characters as well as normal ascii characters

into the transmit queue. ComOut is intended for putting binary

data into the transmit queue.

ComFlushTx flushes the transmitter while ComLenTx finds the

number of characters in the queue.

33

----------------------------------------------------------------

ComPutc

----------------------------------------------------------------

Function

Puts a character into the active serial port's transmit

queue.

Syntax

#include "cport.h"

int ComPutc(char c);

Parameters

c - Character to be put into the transmit queue.

Remarks

ComPutc puts a character into the active serial port's

transmit queue to be transmitted. If the transmit queue

is full, the character is not put and ComPutc returns a 0.

Return value

On success, ComPutc returns a 1. If it fails, it

returns 0.

See also

ComGetc ComPuts ComOut

Example

Send an BS to the transmitter.

#define BS 8

{

if(!ComPutc(BS))

{

...error handler

}

34

----------------------------------------------------------------

ComPuts

----------------------------------------------------------------

Function

Puts a string of characters into the active serial

port's transmit queue.

Syntax

#include "cport.h"

int ComPuts(const char *str);

Parameters

*str  - Pointer to the string to be put into the

transmit queue.

Remarks

ComPuts puts the string 'str' into the active serial

port's transmit queue to be transmitted. If the transmit

queue becomes full, ComPuts will not attempt to put any

more characters in the queue.

Return value

ComPuts returns the actual number of characters put

into the transmit queue.

See also

ComGets ComPutc ComOut

Example

Send the string "hello world" to the transmitter.

{

char *str = "hello world";

if(ComPuts(str) < strlen(str))

{

...error handler

}

}

35

----------------------------------------------------------------

ComOut

----------------------------------------------------------------

Function

Puts a block of bytes into the active serial port's

transmit queue.

Syntax

#include "cport.h"

unsigned ComOut(const void *abyte, unsigned num_byte);

Parameters

*abyte - Pointer to the buffer of the block to be put

into the transmit queue.

num_byte - Number of bytes to be put into the transmit

queue.

Remarks

ComOut puts the block of bytes pointed to by 'abyte'

into the active serial port's transmit queue to be

transmitted.

Return value

ComOutQ returns the number of bytes actually put into

the transmit queue.

See also

ComIn ComPutc ComPuts

Example

Send a block of 128 bytes to the transmitter.

#define BLK_SZ 128

{

byte block[BLK_SZ];

if(ComOut(block, BLK_SZ) < BLK_SZ)

{

...error handler

}

}

}

36

----------------------------------------------------------------

ComFlushTx

----------------------------------------------------------------

Function

Flushes the active serial port's transmit queue.

Syntax

#include "cport.h"

void ComFlushTx(void);

Remarks

ComFlushTx flushes any existing characters in the

active serial port's transmit queue.

See also

ComFlushRx ComLenTx

Example

If a transmit flush command is in order, flush the

transmitter.

#define

#define TX_FLSH 3

#define

{

int command;

...fetch command

switch(command)

{

case ...:

...case code

break;

case TX_FLSH:

ComFlushTx();

break;

case ...:

...case code

break;

}

}

37

----------------------------------------------------------------

ComLenTx

----------------------------------------------------------------

Function

Calculates the number of characters in the active

serial port's transmit queue.

Syntax

#include "cport.h"

unsigned ComLenTx(void);

Remarks

ComLenTx calculates the number of characters in the

active serial port's receive queue. Valid numbers are

0 to 65534.

Return value

ComLenTx returns the number of characters presently

in the transmit queue.

See also

ComLenRx ComFlushTx

Example

Check if the transmitter has reached 3/4 of its

capacity.

{

if(ComLenTx() > (TX_SIZE * 3 / 4))

{

...pause

}

}

38

Status Functions

Status functions are for obtaining or setting various status of

the active serial port.

There are two functions for obtaining status: ComError detects

line errors in the serial port; and ComStatus returns the status

of the input hardware lines.

ComDtr, ComRts and ComOut1 manipulate the output hardware

lines

39

----------------------------------------------------------------

ComError

----------------------------------------------------------------

Function

Determines if any errors have occurred on the active

serial port's since the last call to ComError.

Syntax

#include "cport.h"

unsigned ComError(void);

Remarks

ComError determines if any errors are pending on

the active serial port. After a call to ComError, the

errors are cleared. ComError returns an error

word in which the bits are defined as follows:

OVERUN  (0x002)  -  overrun error

PARITY  (0x004)  -  parity error

FRAMING (0x008)  -  framing error

BREAK   (0x010)  -  break detect

TIMOUT  (0x080)  -  timeout

TXFULL  (0x100)  -  transmit queue overflow

RXFULL  (0x200)  -  receive queue overflow

Return value

If an error has occurred, ComError returns an error

word. If no errors have occurred it returns a 0.

See also

ComStatus

Example

Determine the source of the error if one has occurred.

{

unsigned com_err;

com_err = ComError();

if(com_err)

{

switch(com_err)

{

40

case OVERUN:

...over run handler

break;

case PARITY:

...parity error handler

break;

case ...

}

}

----------------------------------------------------------------

ComStatus

----------------------------------------------------------------

Function

Retrieves the status of the active serial port's

hardware lines.

Syntax

#include "cport.h"

byte ComStatus(void);

Remarks

ComStatus retrieves the status of the active serial

port's hardware lines. ComStatus returns a status byte

in which the bits are defined as follows:

DCTS (0x01) - change in clear to send.

DDSR (0x02) - change in data set ready.

TERI (0x04) - trailing edge ring indicator.

DDCD (0x08) - change in data carrier detect.

CTS  (0x10) - clear to send.

DSR  (0x20) - data set ready.

RI   (0x40) - ring indicator.

DCD  (0x80) - data carrier detect.

Return value

ComStatus returns the status byte.

See also

ComError ComRts ComDtr

41

Example

Test if the modem is powered up and ready.

{

ComDtr(ON);

delay(40);

if(!(ComStatus() & DSR))

{

puts("modem not responding");

...what to do code

}

}

----------------------------------------------------------------

ComRts

----------------------------------------------------------------

Function

Sets or clears the active serial port's Request to Send

line.

Syntax

#include "cport.h"

void ComRts(byte on_off);

Parameters

on_off - Determines whether RTS is to be set or

cleared. Constants are provided in cport.h.

Possibilities are:

ON  (1) - Sets RTS.

OFF (0) - Clears RTS.

Remarks

ComRts sets or clears the active serial port's RTS line

depending on the value of 'on_off'. In many cases, this

signal will be looked at by the remote device to determine

if it is safe to transmit. This gives the programmer the

ability to turn off the remote device's transmitter.

NOTE: ComRts should never be called when RTS hardware

handshaking is enabled.

42

See also

ComDtr ComOut1 ComStatus

Example

If the receiver is off ('rx_off') but ready to

receive, assert RTS. If it is on but not ready, turn

off RTS.

{

if(rx_off && rx_rdy)

ComRts(ON);

else if(!rx_off && !rx_rdy)

ComRts(OFF);

}

----------------------------------------------------------------

ComDtr

----------------------------------------------------------------

Function

Sets or clears the active serial port's  Data Terminal

Ready line.

Syntax

#include "cport.h"

void ComDtr(byte on_off);

Parameters

on_off - Determines whether DTR is to be set or

cleared. Constants are provided in cport.h.

Possibilities are:

ON  (1) - Sets DTR.

OFF (0) - Clears DTR.

Remarks

ComDtr set or clears the active serial port's DTR

line depending on the value of 'on_off'. Setting DTR

informs the remote equipment that the computer is ready

to establish communications.

NOTE: ComRts should never be called when DTR hardware

handshaking is enabled.

43

See also

ComRts ComOut1 ComStatus

Example

If an break is detected, turn off DTR.

{

unsigned com_err;

int brk_flg = 0;

com_err = ComError();

if(com_err)

{

switch(com_err)

{

case ...

case BREAK:

ComDtr(OFF);

brk_flg = 1;

break;

case ...

}

}

}

----------------------------------------------------------------

ComOut1

----------------------------------------------------------------

Function

Sets or clears the active serial port's Out 1 line.

Syntax

#include "cport.h"

void ComOut1(byte on_off);

Parameters

on_off - Determines whether OUT1 is to be set or

cleared. Constants are provided in cport.h.

Possibilities are:

ON  (1) - Sets OUT1.

OFF (0) - Clears OUT1.

44

Remarks

ComOut1 sets or clears the active serial port's

general purpose OUT1 line depending on the value of

'on_off'. OUT1 is a general purpose output sometimes

utilized by device for a special feature or reset.

See also

ComRts ComDtr ComStatus

Example

Reset a smart modem.

#define ComSMSet() ComOut1(ON)

#define ComSMClr() ComOut1(OFF)

{

ComSMRes();

delay(50);

ComSMClr();

}

45

Data Integrity

Two functions are currently provided for checking the integrity

of data. They are: ComChecksum and ComCrc16. ComChecksum and

ComCrc16 perform a checksum calculation and a 16 bit cyclic

redundancy check respectively, on a block of bytes. Notice also

that these functions take the same arguments as the functions

ComOut and ComIn.

46

----------------------------------------------------------------

ComChecksum

----------------------------------------------------------------

Function

Performs a checksum calculation on a block of bytes.

Syntax

#include "cport.h"

byte ComChecksum(const void *abyte, unsigned n_byte);

Parameters

*abyte - Pointer to the block on which the checksum

is to be performed

num_byte - Number of bytes in the block.

Remarks

ComChecksum performs a one byte checksum on the block

of bytes pointed to by 'abyte'. The checksum is

calculated by summing all the bytes in the block and

ignoring the carry outs.

Return value

ComChecksum returns the one byte checksum.

See also

ComCrc16 ComIn ComOut

Example

Verify the actual checksum matches the received

checksum.

{

struct Xmodem{

byte num;

byte _num;

byte data[128];

byte checksum;

}block;

ComIn(&block, sizeof(struct Xmodem));

47

if(ComChecksum(block.data, 128) != block.checksum)

{

...bad block handler

}

else...

}

----------------------------------------------------------------

ComCrc16

----------------------------------------------------------------

Function

Performs a 16 bit cyclic redundancy check on a block of

bytes.

Syntax

#include "cport.h"

unsigned ComCrc16(const void *abyte, unsigned n_byte);

Parameters

*abyte - Pointer to the block on which the 16 bit

cyclic redundancy check is to be performed.

num_byte - Number of bytes in the block.

Remarks

ComCrc16 performs a 16 bit cyclic redundancy check on

the block of bytes pointed to by 'abyte'. The 16 bit

CRC is calculated using a table of precalculated

values.

Return value

ComCrc16 returns the two byte CRC value.

See also

ComChecksum ComIn ComOut

48

Example

Verify the actual CRC matches the received CRC.

{

struct Xmodem{

byte num;

byte _num;

byte data[128];

unsigned crc16;

}block;

ComIn(&block, sizeof(struct Xmodem));

if(ComCrc16(block.data, 128) != block.crc16)

{

...bad block handler

}

else...

}

49

Misc Functions

There are two pair of misc functions: ComSetBreak and

ComClrBreak set and clear a break condition; ComPutScrtch

and ComGetScrtch write to and read from the scratch register.

50

----------------------------------------------------------------

ComSetBreak

----------------------------------------------------------------

Function

Set a break condition on the active serial port.

Syntax

#include "cport.h"

void ComSetBreak(void);

Remarks

ComSetBreak sets a break condition on the active serial

port. The break condition will stay in effect until a

subsequent call to ComClrBreak is made.

See also

ComClrBreak

Example

If the an abort condition has been declared, set a

break condition. When it clears, release the break

condition.

{

int abrt_flg;

int brk_flg;

if(abrt_flg && !brk_flg)

ComSetBreak();

else if(!abrt_flg && brk_flg)

ComClrBreak();

}

----------------------------------------------------------------

ComClrBreak

----------------------------------------------------------------

Function

Clears a previously set break condition on the active

serial port.

51

Syntax

#include "cport.h"

void ClrBreak(void);

Remarks

ComClrBreak clears a previously set break condition

on the active serial port.

See also

ComSetBreak

Example

If the an abort condition has been declared, set a

break condition. When it clears, release the break

condition.

{

int abrt_flg;

int brk_flg;

if(abrt_flg && !brk_flg)

ComSetBreak();

else if(!abrt_flg && brk_flg)

ComClrBreak();

}

----------------------------------------------------------------

ComPutScrtch

----------------------------------------------------------------

Function

Writes a byte to the active serial port's scratch

register.

Syntax

#include "cport.h"

void ComPutScrtch(byte abyte);

Parameters

abyte - The byte to be written to the scratch register.

52

Remarks

ComPutScrtch writes the byte value 'abyte' to the

active serial port's UART scratch register. Not all

UARTs have a scratch register.

See also

ComGetScrtch

Example

Use the scratch register to store a status byte.

#define ComSavStatus() ComPutScrtch(ComStatus())

#define ComOldStatus() ComGetScrtch()

{

unsigned old_stat;

ComSavStatus();

...later on

old_stat = ComOldStatus();

}

----------------------------------------------------------------

ComGetScrtch

----------------------------------------------------------------

Function

Fetches the byte stored in the active serial port's

scratch register.

Syntax

#include "cport.h"

byte ComGetScrtch(void);

Remarks

ComGetScratch reads the byte in the active serial

port's UART scratch register. Not all UARTs have a

scratch register.

Return value

ComGetScrtch returns the byte read from the UART

scratch register.

53

See also

ComPutScrtch

Example

Use the scratch register to store a status byte.

#define ComSavStatus() ComPutScrtch(ComStatus())

#define ComOldStatus() ComGetScrtch()

{

unsigned old_stat;

ComSavStatus();

...later on

old_stat = ComOldStatus();

}

54

Appendix A

Things to look out for:

1) Since Cport is driven by hardware interrupts, input and

output functions will work unpredictably inside a debugger.

If a break point is set and the program is run, everything

fuctions until the break point is reached. At that point the

interrupts will probably cease and the transmitter and

receiver will freeze. Needless to say, single-stepping is out

of the question.

2) Baud rates above 19200 require special attention because they

are sensitive to many factors. Some of these factors are:

� Cable length

� Machine speed

� Type of UART used

� Activity of handshaking

� Number of open serial ports

� Degree of bidirectional transfer (transmitting and receiving

simultaneously)

� Other interrupt activity

The last factor, interrupt activity, includes keyboard

activity which can be a major impedance. During critical high

speed transfers, some interrupts can be temporarily masked

out (disabled) at the PIC to preclude them from impeding the

communications interrupt handler.

3) In the small and medium memory models, the near heap where

the queue buffers are allocated is limited. The data segment

(globals etc.) + the stack + allocated memory (queue buffers)

must not exceed 64K bytes. If extremely large queue(s) are

required, the compact or large model where each queue can be

as large as 65534 bytes must be used.

4) The functions ComOpen, ComOpenS, ComClose and ComCloseAll

make calls to the C functions malloc() and free().

55

Appendix B

Handshaking schemes

There are number of built-in options available for implementing

handshaking. The options can be divided into two categories.

One is hardware handshaking and the other is software handshaking.

In hardware handshaking, one or more of the hardware lines are

used. In software handshaking, XON and XOFF characters are used.

Hardware handshaking

In hardware handshaking, any combination of the hardware signals

RTS, DTR, DSR, CTS, and DCD can be used to achieve handshaking.

RTS and/or DTR are used by the receiver to signal the remote device

to suspend or resume transmitting. Similarly, DSR, CTS, and/or

DCD can be used by the remote device's receiver to signal

the transmitter to suspend or resume transmitting.

Software handshaking

In software handshaking, the control characters XON (ctrl Q)

and XOFF (ctrl S) are used to achieve handshaking. When

receiver software handshaking (S_RX) is enabled, XON and XOFF

characters are transmitted to signal the remote device to

suspend or resume it's transmission. Notice that even though

the XON and XOFF characters are sent by the transmitter, it

is the receiver which exercises the handshaking in order

to preclude a receive queue overflow. In transmit software

handshaking (S_TX), the remote device sends XON and XOFF

characters to suspend and resume the computer's transmission.

Notice that even though the XON and XOFF characters are received,

it is the transmitter which is affected. Both receive and

transmit handshaking can be enabled simultaneously.

The receiver

When receive software and/or hardware handshaking (DTR and/or

RTS) are enabled the receiver monitors the length of the receive

queue. If the length exceeds the threshold set by the function

ComHandshake or ComOpenS, the receiver will take the appropriate

action to signal the remote device to suspend transmitting. When

the length of the receive queue subsequently falls below half of

the threshold, the appropriate action is taken to signal the

remote device to resume transmitting.

The transmitter

When transmit software and/or hardware handshaking (DSR, CTS,

56

and/or DCD) are enabled and the computer is signaled to suspend

transmitting with the corresponding handshaking  enabled, the

transmitter will stop transmitting. Subsequently, when the

computer is signaled to resume transmitting, the transmitter will

pick up where it left off.

User handshaking

Another option for implementing handshaking is for the programmer

implement their own handshaking scheme. To accomplish this,

Cport provides the function ComTx. ComTx enables the programmer

to turn the transmitter on and off. When the transmitter is

turned off, transmitting halts immediately. If any characters

still reside in the transmit queue, they will remain there until

the transmitter is turned back on or the transmit queue is

flushed. More characters may be put into the transmit queue

while the transmitter is turned off.

Cooperation

In any handshaking scheme it is imperative that the computer and

the remote device recognize the handshaking scheme. For instance,

if the remote device recognizes hardware handshaking and the

computer does not, the computer will ignore any requests by the

remote device to suspend transmitting. If the remote device

recognized software handshaking and the computer did not, bogus

XON and XOFF characters might be found to be corrupting the

received data.

57

Appendix C

The UART (Universal Asynchronous Receiver/ Transmitter) chip

The UART, a single integrated circuit chip, is the backbone

of serial communications in computers today. Serial

communications in IBM and compatible computers is based on the

National Semiconductor family of UARTs. These chips include:

INS8250

INS8250-B

INS8250A

INS82C50A

NS16450

NS16C450

NS16550A

The latest and greatest is the NS16550A which features transmitter

and receiver FIFO's. If a NS16550 UART is detected when a serial

port is opened, Cport will automatically take advantage of it's

advanced features. This feature greatly improves performance.

58

Appendix D

The RS-232 Standard ?!!

Voltages, logic levels and pin assignments of the RS-232

standard are more or less adhered to. However, the way the

RS-232 standard prescribes data transfers to be controlled,

more commonly know as handshaking, does not work for computers.

This can lead to much confusion and frustration when trying

to connect two serial communications devices from different

manufacturers.

The book "The RS-232 Solution" by Joe Campbell addresses

this problem and includes a number of case studies. It is

recommended reading for both the novice and expert programmer.

59

Appendix E

Cport v1.30 Registration Form

Name: ______________________________________________________

Company: ___________________________________________________

Address: ___________________________________________________

City: ____________________________  State: _________________

Zip Code: _________________  Phone # : _____________________

CompuServe # : _____________________________________________

How did you acquire Cport ? ________________________________

What functions do you find most useful ?____________________

____________________________________________________________

What functions do you find least useful ?___________________

____________________________________________________________

What functions, not in Cport, would you like to see ?_______

____________________________________________________________

What other types of libraries would you find useful ? _____

__Game Port  __Expanded Memory  __IEEE/GPIB  __Sound effects

Others______________________________________________________

Other comments ?____________________________________________

Registration                                      $ 35.00

California residents add sales tax ( 8.25% )      $________

Shipping inside continental U.S.   ( $ 2.00 )

Shipping outside continental U.S.  ( $ 5.00 )     $________

Total:   $________

All Payments Must be in U.S. Dollars

Make check or money order payable to:    Bri Productions

P.O. Box 7121

Fremont, CA 94537-7121

60