A

Comparison

of

===========================================================

MM   MM  IIIIIII    CCCC   RRRRRR     OOO             CCCC

M M M M     I      C    C  R     R   O   O           C    C

M  M  M     I     C        R     R  O     O         C

M     M     I     C        RRRRRR   O     O  -----  C

M     M     I     C        R   R    O     O         C

M     M     I      C    C  R    R    O   O           C    C

M     M  IIIIIII    CCCC   R     R    OOO             CCCC

===========================================================

A N D

===========================================================

SSSSS   MM   MM     A     L        L                 CCCC

S     S  M M M M    A A    L        L                C    C

S        M  M  M   A   A   L        L               C

SSSSS   M     M  A     A  L        L        -----  C

S  M     M  AAAAAAA  L        L               C

S     S  M     M  A     A  L        L                C    C

SSSSS   M     M  A     A  LLLLLLL  LLLLLLL           CCCC

===========================================================

Copyright 1990 Dave Dunfield

All rights reserved

MICRO-C .vs. SMALL-C                                             Page: 1

1. INTRODUCTION

The most common reaction of people hearing about MICRO-C  for  the

first time is something like "Humph... Another  version  of  the  old

SMALL-C compiler". This couldn't be further from the truth.

MICRO-C IS NOT SMALL-C!!!

MICRO-C is a completely  new  implementation  of  a  'C'  compiler

suitable for use on very small systems. It offers several  advantages

over SMALL-C:

- It was designed from the  ground  up  to  be  easily  portable  to

different processors & computer platforms. Sample code  generators

are included for 8080, 80x86, 6809 and 68HC11 cpu's, and an entire

section of the manual is devoted to porting the compiler.

Although SMALL-C claims  to  be  easily  portable,  much  of  it's

arcitecture is oriented toward the original 8080 processor. For an

example of  how  this  affects  the  compiler,  compare  the  code

generated by the 8086 version of SMALL-C to reference stack (auto)

variables to that produced by MICRO-C.

- It is a more complete implementation of the 'C' language, see  the

features comparison later in this document.

- It produces faster, more compact code. See the  benchmark  results

later in this document.

- MICRO-C employs  a  fully  tokenized  parser,  allowing  statement

analysis to be performed  on  16  bit  "tokens"  instead  of  text

strings as is  done  in  SMALL-C.  This  results  in  MUCH  faster

compilation.

- Although it is higher in functionality than SMALL-C,  the  MICRO-C

compiler is much smaller than SMALL-C (Nearly half the size). This

allows it to run on very small computer systems.

2. DETAILED COMPARISON OF COMPILERS

The following pages contain a detailed comparison of  the  SMALL-C

version 2 compiler, and MICRO-C.

MICRO-C .vs. SMALL-C                                             Page: 2

2.1 'C' Implementation

Here is a  table  detailing  features  implemented  in  SMALL-C

version 1, SMALL-C version 2, and MICRO-C:

=============================================================

Data types              | SMALL-C V1 | SMALL-C V2 | MICRO-C |

------------------------+------------------------------------

int                     |    Yes     |    Yes     |   Yes   |

char                    |    Yes     |    Yes     |   Yes   |

unsigned int            |     No     |     No     |   Yes   |

unsigned char           |     No     |     No     |   Yes   |

pointers                |    Yes     |    Yes     |   Yes   |

pointers to pointers    |     No     |     No     |   Yes   |

Single dimension arrays |    Yes     |    Yes     |   Yes   |

Multi dimension arrays  |     No     |     No     |   Yes   |

arrays of pointers      |     No     |     No     |   Yes   |

Typecast                |     No     |     No     |   Yes   |

========================+====================================

Control Structures      | SMALL-C V1 | SMALL-C V2 | MICRO-C |

------------------------+------------------------------------

if/else                 |    Yes     |    Yes     |   Yes   |

while                   |    Yes     |    Yes     |   Yes   |

do/while                |     No     |    Yes     |   Yes   |

for                     |     No     |    Yes     |   Yes   |

switch/case             |     No     |    Yes     |   Yes   |

goto                    |     No     |    Yes     |   Yes   |

Conditional Expressions |     No     |    Yes     |   Yes   |

========================+====================================

Pre-processor           | SMALL-C V1 | SMALL-C V2 | MICRO-C |

------------------------+------------------------------------

#define (basic)         |    Yes     |    Yes     |   Yes   |

#define (Parameterized) |     No     |     No     |   Yes   |

#undef                  |     No     |     No     |   Yes   |

#include (single level) |    Yes     |    Yes     |   Yes   |

#include (nested)       |     No     |    Yes     |   Yes   |

#ifdef/#ifndef          |     No     |    Yes     |   Yes   |

#asm (inline assembler) |    Yes     |    Yes     |   Yes   |

========================+====================================

Misc features           | SMALL-C V1 | SMALL-C V2 | MICRO-C |

------------------------+------------------------------------

Optimization            |     No     |    Yes     |   Yes   |

Multiple memory models  |     No     |     No     |   Yes   |

Make/touch utility      |     No     |     No     |   Yes   |

Source linker           |     No     |     No     |   Yes   |

8080 code generator     |    Yes     |    Yes     |   Yes   |

8086 code generator     |     No     |    Yes     |   Yes   |

6809 code generator     |     No     |     No     |   Yes   |

68HC11 code generator   |     No     |     No     |   Yes   |

========================+====================================

Library features        | SMALL-C V1 | SMALL-C V2 | MICRO-C |

------------------------+------------------------------------

Complete standard lib   |     No     |    Yes     |   Yes   |

Interrupt serial I/O    |     No     |     No     |   Yes   |

Windowing library       |     No     |     No     |   Yes   |

TSR support             |     No     |     No     |   Yes   |

========================+====================================

MICRO-C .vs. SMALL-C                                             Page: 3

2.2 Generated code quality

To test the actual performace  of  code  generated  by  MICRO-C

against  code  generated  by  SMALL-C,  I  used  this  "Sieve   of

Eratosthenes" prime number generator program, taken from the  BYTE

benchmarks:

/*

* The classic "Sieve of Eratosthenes" prime number program.

* from BYTE, January 1983.

*/

#include <stdio.h>

#include "timer.c"          /* The timer subroutines */

#define SIZE  8190

#define LOOP  100

#define TRUE  1

#define FALSE 0

char flags [SIZE + 1];

main()

{

int i, prime, k, count, iter;

printf("BYTE Sieve Benchmark - %d iterations\n",LOOP);

/* Start timer and execute loop */

startimer(timestamp);

for(iter = 1; iter <= LOOP; ++iter) {

count = 0;                          /* prime counter */

for(i = 0; i <= SIZE; ++i)          /* set all flags true */

flags [i] = TRUE;

for(i = 0; i <= SIZE; ++i) {

if(flags [i]) {                 /* found a prime */

prime = i + i + 3;          /* twice index + 3 */

/*              printf ("\n%d", prime);     */

for(k = i + prime; k <= SIZE; k+= prime)

flags [k] = FALSE;          /* kill all multiple */

++count; } } }                  /* primes found */

elapsed(timestamp);                     /* Calculate time taken */

/* Report results */

printf("%d primes.\n\n", count);        /* primes found on 100th pass */

printf("Elapsed time: %02d:%02d:%02d.%02d\n",

timestamp[1], timestamp[0], timestamp[3], timestamp[2]);

}

MICRO-C .vs. SMALL-C                                             Page: 4

2.3 Speed of compilation

To show the relative compilation times,  I  wrote  this  simple

MICRO-C program, using the timing routines shown below, to execute

the compilers & report the time taken:

#include <stdio.h>

#include "timer.c"          /* The timer subroutines */

main()

{

startimer(timestamp);

system("CC SIEVE");             /* Execute the SMALL-C compiler */

elapsed(timestamp);

printf("Elapsed time: %02d:%02d:%02d.%02d\n\n",

timestamp[1], timestamp[0], timestamp[3], timestamp[2]);

startimer(timestamp);

system("MCC SIEVE.C SIEVE.ASM");/* Execute the MICRO-C compiler */

elapsed(timestamp);

printf("Elapsed time: %02d:%02d:%02d.%02d\n",

timestamp[1], timestamp[0], timestamp[3], timestamp[2]);

}

NOTE: The results from the above program will include the  time

taken to load the compilers. To minimize this, all tests were  run

from a RAMdisk. This eliminates disk seek time, and  reduces  load

time to the speed of a memory to memory copy.

MICRO-C .vs. SMALL-C                                             Page: 5

2.4 Timing the tests

These subroutines were used to time  the  programs,  using  the

MS-DOS internal clock which has a resolution of 1/100 of a second:

char timestamp[4];          /* Stores initial timestamp */

/*

* Record system time for later calculation

*/

startimer()

{ ;

#asm

MOV     AH,2CH      ; Get time function

INT     21H         ; Ask DOS

MOV     SI,4[BP]    ; Get pointer to timestamp

MOV     [SI],CX     ; Record hours & minites

MOV     2[SI],DX    ; Record seconds & hundreds

#endasm

}

/*

* Calculate elapsed time since timestamp recorded

*/

elapsed()

{ ;

#asm

MOV     AH,2CH      ; Get time function

INT     21H         ; Ask DOS

MOV     SI,4[BP]    ; Pointer to timestamp

SUB     DL,2[SI]    ; Convert 100ths

JNC     ELAP1       ; No borrow

ADD     DL,100      ; Re-adjust

DEC     DH          ; Reduce seconds

ELAP1:  MOV     2[SI],DL    ; Save elapsed hundreds

SUB     DH,3[SI]    ; Convert seconds

JNS     ELAP2       ; No borrow

ADD     DH,60       ; Re-adjust

DEC     CL          ; Reduce minites

ELAP2:  MOV     3[SI],DH    ; Save elapsed seconds

SUB     CL,[SI]     ; Convert minites

JNS     ELAP3       ; No borrow

ADD     CL,60       ; Re-adjust

DEC     CH          ; Adjust hours

ELAP3:  MOV     [SI],CL     ; Save elapsed minites

SUB     CH,1[SI]    ; Convrt hours

MOV     1[SI],CH    ; Save elapsed hours

#endasm

}

MICRO-C .vs. SMALL-C                                             Page: 6

2.5 Test results

After compiling the programs, I ran  the  various  tests  on  a

standard 8-Mhz IBM PC/AT, with these results:

E:\>dir

Volume in drive E is VDISK  V3.3

Directory of  E:\

CC       EXE    40626   9-10-90  /* SMALL-C compiler */

MCC      COM    22316   9-10-90  /* MICRO-C compiler */

TIMER    C       1094   9-10-90  /* The timer subroutines */

TIMECOMP C        527   9-10-90  /* The compiler timer */

SIEVE    C       1149   9-10-90  /* The test program */

SIEVE-S  EXE    16946   9-10-90  /* SMALL-C produced this */

SIEVE-M  COM     1651   9-10-90  /* MICRO-C produced this */

TIMECOMP COM     1924   9-10-90  /* Executable compiler timer */

8 File(s)    301056 bytes free

E:\>sieve-s     /* The SMALL-C version: 1 min, 28.98 secs */

BYTE Sieve Benchmark - 100 iterations

1899 primes.

Elapsed time: 00:01:28.98

E:\>sieve-m     /* The MICRO-C version: 1 min, 4.59 secs */

BYTE Sieve Benchmark - 100 iterations

1899 primes.

Elapsed time: 00:01:04.59

E:\>

**NOTE: Before running TIMECOMP, I created a "generic" STDIO.H

file which was suitable for both compilers.

E:\>timecomp    /* Times to compile the program */

Small-C Compiler, Version 2.1, (Rev. 75)

Copyright 1982, 1983, 1985 J. E. Hendrix

Elapsed time: 00:00:04.72       /* SMALL-C 4.72 secs */

MICRO-C 2.0 Compiler

Copyright 1988,1990 Dave Dunfield

All rights reserved.

Elapsed time: 00:00:01.32       /* MICRO-C 1.32 secs */

E:\>

MICRO-C .vs. SMALL-C                                             Page: 7

3. OTHER ADVANTAGES OF MICRO-C

In addition to being a superiour and more portable  compiler,  The

MICRO-C package also offers these advantages:

3.1 Utilities

MICRO-C Comes with the following utilities (all  with  complete

MICRO-C source code):

CC      - Command Coordinator, combines pre-processor, compiler,

optimizer, assembler and linker into a single command.

Command line parameters allow very flexible operation.

MAKE    - Automates building of larger (multi module) programs.

TOUCH   - Set timestamp of file to current or specified date.

(Used with MAKEs dependancy checking)

SLINK   - Source LINKer, Allows pre-compiled (assembler) source to

be kept in a library and included as needed to resolve

external references. Useful when MICRO-C is used as cross

compiler for systems not supporting an object linker.

SINDEX  - Automatically indexes a source library for use by SLINK.

SCONVERT- Converts assembly language source for use by SLINK.

EXE2BIN - Converts MS-DOS EXE files to a binary image, useful for

users of later versions of DOS which no longer include

this utility.

MCP     - The MICRO-C Pre-processor

MCC     - The MICRO-C Compiler

MCO     - The MICRO-C Optimizer

MICRO-C .vs. SMALL-C                                             Page: 8

3.2 Example Programs

MICRO-C comes with the following  example  programs  (all  with

complete MICRO-C source code):

HELLO   - Standard 'C' demo program

PRIME   - Prime number generator

FIBO    - Calculates FIBONACCI series

TYPE4   - Display files with tabs every 4 spaces

TEXTNUM - Convert a number into a text description

EQUIP   - Display equipment installed in IBM/PC

ROBOFACE- Simple example of drawing lines and circles

HANOI   - Solves the TOWERS OF HANOI problem

TTT3D   - Three Dimensional Tic Tac Toe game

FORTUNE - A "fortune cookie" program

MTERM   - A POP-UP (TSR) ANSI terminal with XMODEM

ASM86   - An 8086 assembler

BASIC   - A BASIC interpreter

CALC    - A POP-UP (TSR) hex/decimal calculator

Features automatic entry of the last result into the

underlying DOS application.

CASTLE  - A very large text adventure game

CHATTR  - Change ANY attribute of dos files

COMEXT  - Extract comments from 'C' sources

CSET    - A POP-UP (TSR) display of the IBM/PC character set

DUMP    - Hex/Octal file dump utility

HEXED   - Screen (window) hexidecimal file editor

INSTALL - Menu driven automatic install program

OBSCURE - Scramble 'C' source but it still compiles

RLTEXT  - RunLength encoder for text files

HFTEXT  - Huffman encoder for text files

WINDEMO - Demo of MICRO-C windowing library

MICRO-C .vs. SMALL-C

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

2. DETAILED COMPARISON OF COMPILERS                                    1

2.1 'C' Implementation                                              2

2.2 Generated code quality                                          3

2.3 Speed of compilation                                            4

2.4 Timing the tests                                                5

2.5 Test results                                                    6

3. OTHER ADVANTAGES OF MICRO-C                                         7

3.1 Utilities                                                       7

3.2 Example Programs                                                8