NOPAUSE.CPP and NOPAUSE2.CPP

By Thomas Astin  CIS: 73407,3427  Prodigy: RNVN08A

(C) Copyright 1991.  All rights reserved.

Introduction

------------

This code demonstrates how to "disable" the pause/lock feature provided on

most 286/386 class machines supporting a 101/102 keyboard. When Pause is

pressed the BIOS loops until any other key is pressed thereby allowing the

user to suspend any program currently running.

NOPAUSE and NOPAUSE each demonstrate a way to disable the Pause feature. The

word "disabled" is used loosely because the Pause key is not actually

disabled. Either the Pause key scan codes are kept from the BIOS or the BIOS

data area containing a Pause/Lock bit is constantly cleared. Due to limited

testing on multiple machines it is hard to say if this code will behave

correctly on all machines. Please see notes in source code for further

information.

Method #1 - NOPAUSE.CPP

-----------------------

NOPAUSE demonstrates by way of intercepting keyboard hardware int 0x09.

Method #1 - NOPAUSE.CPP

-----------------------

NOPAUSE2 demonstrates by way of intercepting clock tick interrupt 0x1C. The

code in the intercepting handler clears a Pause/Lock bit located in the BIOS

data area. NOTE: This worked on the testing machine because it was known that

the Pause/Lock loop in the testing machine's bios tested the Pause/Lock bit

and droped out of the loop if the bit was clear. This may not work the same on

other machines.