�����������������������������������������������������������������������������Ŀ

� TOSHIBA.DOC  Notes on graphics programmimg of TOSHIBA printers.    11-22-91 �

�              Uncle Mikie, P.O.Box 176, Covington, OH, 45318  (513) 473-5587 �

�������������������������������������������������������������������������������

WARNING: The two control code sets for Toshiba printers have NOT been tested.

Information from Toshiba, entitled "Software Developer Technical Reference",

shows several unique problems with these printers:

1) Bit-data must be formatted as 4 bytes of 6 bits.  The manual does

not say whether bits are 'UP' or 'DN', or whether bytes are 'UP'

or 'DN'.    Adjacent pins will not fire (vertically?).

2) The amount of data is 'a' format and refers to the number of

columns (horizontal dots).    Use FOUR digits, even if the

first is a zero.

3) The vertical motion command is:

ESC   ~  L  0  3  d1 d2 d3   Set Line spacing to d/120".

27 126 76 48 51  d1 d2 d3   d1,d2,d3 are ACSII decimal digits.

To simplify things, use:  ESC ~ L 0 3 0 1 6

this will give you 16/120" or 2/15" spacing for graphics.

4) There are other vertical and horizontal move commands that use

a hexadecimal digit for 10's, and decimal for 1's.   Forget them!

5) The IBM emulation available is the Graphics printer or Proprinter

which is a poor way to use a 24-pin printer.

NOTE: Qume uses the same Toshiba control codes.