�����������������������������������������������������������������������������Ŀ

� IBM_PPDS.DOC Notes on graphics programmimg of IBM lasers.          11-17-91 �

�              Uncle Mikie, P.O.Box 176, Covington, OH, 45318  (513) 473-5587 �

�������������������������������������������������������������������������������

When IBM set up the control codes for their laser printers, they made it

compatible with the Graphics Printer and Proprinter.   Unfortunately, when they

made the NEW raster graphics command, they made one command with SIX variables.

This does not fit the format of PRTCODE.ASC, so we put 'PPDS' in the file as a

trigger into a seperate procedure.

The raster graphics command is:

ESC [ h nb np nh nv nf bw bd <data>

where: ESC [ h          are ASCII 27, 91, 104.

nb np nh nv nf   are two byte integers, low byte first.

bw bd            are one byte parameters.

nb               total bytes of parameters and data.

np               bytes of parameters.

nh               horizontal dpi; 75, 100, 150, 300.

nv               vertical dpi; 75, 100, 150, 300.

nf               BlockF parameter: number of bytes per line.

(You can send several lines of bit-data?)

bw               Write mode (default is 0):

0: OR                4: AND with inverted

1: Overpaint         5: AND

2: Inverted then OR  6: Subtract

3: XOR               7: OR result inverted

bd               Decompression algorithm:

0: No compression

1: IBM MMR decompression

2: G4 decompression

255: Use previous value

<data>           bit-graphics data.

Oddly, nh, nv, and nf are shown as 'required' parameters.  Yet

each is given a default value (300, 300, 0), and the decription

of np says that '0' indicates 'use previous values'!

CLARIFICATION: nb is the number of bytes following nb, thus

including np.   Also, np is the number of parameter bytes

after np.   Thus, the number of data bytes is:

nd = nb - np - 2

I have been able to verify that you can set parameters

with the long version of this command, and no bit-data.

Then use the short version (parameters = 0) to send each

line of data.   Further, BlockF can be set to any size

larger than your line length and it will not cause any

harm.

If you have problems with PPDS, please contact IBM/Lexmark.