�����������������������������������������������������������������������������Ŀ

� INKJETS.DOT Programming notes concerning ink jet quirks.           11-1-91  �

�              Uncle Mikie, P.O.Box 176, Covington, OH, 45318  (513) 473-5587 �

�������������������������������������������������������������������������������

Ink jet printers pose several problems for the structure of PRTCODE.ASC.

HEWLETT-PACKARD PAINTJET and XL:

Hewlett-Packard PaintJets are adequately supported if you understand the

way that they use 'color planes'.   The control code set for the PaintJet is

currently set up for three color planes.   This assumes that you create your

own dot color patterns.    But then you get a black & white image to print.

We have two ways to handle this:

1) Shift to the 'DeskJet' code set.   Bad because we would like to

set up the printer with one code set and leave it alone.

2) Create a GIB command to initiate 1-plane graphics (B&W), and

a GIC command to initiate 3-plane graphics (color).

This is probably the smoother way to handle it.

3) The current way, initialize 3-plane graphics, but just use the

GLB command for black & white.  The PaintJet treats a single

plane of data as black even when 2, 3, or 4 planes are selected.

ANOTHER PROBLEM: If you print three-color graphics at less than the default

width (8.00 or 10.50 inch) then you MUST use the 'Picture Width' command.

This is designated 'GW' in PRTCODE.ASC:

ESC * r # S

where: # = ASCII dot width of the picture, computed as:

1440 dots = 8 inches at ANY resolution!

# = 180 *(inches wide)   Independant of resolution!!!

If you do not define the picture width, the addition space to the

right edge will be assumed zero = black !!!

CANON COLOR INKJET:

Canon color ink jets (PJ-1080) create another problem.   For raster

graphics, they send a command code and then follow it with n bytes of

red-data, n bytes of green data, and n bytes of blue data.   There is no

obvious way to designate this in the code set, and yet it poses no

programming problem, once you know what which printers work this way.

Currently, I am handling it with bit 4 of the printer flags.

SHARP COLOR INKJET:

The Sharp JX-730 poses the worst of problems.   Raster graphics require

four lines of four (CMYX) colors, then send a 'ESC A' command.  The 'GL'

command is:

ESC I m a <bit-data>

where: a is the ASCII number of bytes (three ASCII digits).

m designates color and dot line as:

m = 48 + 4*color + line

where: line = 0, 1, 2, 3

color = 0: black

1: magenta

2: yellow

3: cyan

In this scheme, you send this command 16 times for the four lines, then

the 'ESC A' will print them and advance 1/45 inch vertically.  Wonderful?

I have no idea how to designate this code set.   By default, I included

a different graphics command and ignored the raster mode.

SUGGESTIONS ARE WELCOME!