�����������������������������������������������������������������������������Ŀ

� LASERS.DOC   Notes on graphics programmimg of laser printers.      11-16-91 �

�              Uncle Mikie, P.O.Box 176, Covington, OH, 45318  (513) 473-5587 �

�������������������������������������������������������������������������������

In the early days of the laser printer, they could only make 100 to 150

dpi, and printer memory was very expensive.   These early lasers were set up

to do IBM Graphics Printer (Epson MX) graphics but adjusted to 150x75 dpi or

whatever the manufacturer thought would be adequate.

A few years later, lasers could do 300 dpi, but printer memory was still

costly, so they emulated the IBM Proprinter (Epson FX).   These emulations

were actually done at 75x75, 150x75, and 300x75 dpi.  Several of these lasers

are in PRTMODEL, and are indicated with a HEAD value of 300.   How each laser

will handle your Proprinter graphics is not entirely predictable.  However,

most will print your image correctly, but 20% narrower and 4% shorter.

Then came the Hewlett-Packard LaserJet.   From here on we have a good

laser standard that we can depand on.   All graphic densities are one to one,

75x75, 100x100, 150x150, and 300x300.    Further, the graphics line command

includes an automatic line advance of the correct number of dots for each

density.

But then in 1990, IBM introduced their 4019 and 4029 laser printers.

The IBM lasers use PPDS, which is an extension from the Graphics printer and

Proprinter.   This laser properly simulates horizontal densities by printing

variable numbers of dots per graphics bit to get exactly 60, 120, and 240

dpi horizontally.   Also, the 72 dpi vertically is simulated by printing five

or four dots per graphics bit.    You can safely use these emulations if it

helps your programming.

The new part of IBM PPDS for the laser is the "ESC [ h" command.   It

provides great flexibility, but is a long and complicated when fit into the

PRTCODE format.   We get around this by creating GI commands from "ESC [ h"

then use the simplest form of the same command for the GL command.   No GE

command is necessary.

ALL LASERS: You must ask your USER how much memory is installed in the

laser before choosing graphics density.    Most will obey the following limits:

256 K:    Proprinter, full page.

512 K:    150 by 150 dpi, full page.

1024 K:    300 by 300 dpi, half page.

2048 K:    300 by 300 dpi, full page.

If you have ideas how SETPRINT can test laser memory, let me know.  I do

not like the idea of loading the laser with graphics until it chokes!  The user

gets a bad feeling from flashing red lights and error signals.