INSTRUCTIONS for adding a PRINTER MODEL to PRTMODEL.ASC      Nov.1991

------------ --- ------ - ------- ----- -- ------------

Most of the following information can be found in your printer manual

in the section called 'Programming' or 'Control Codes'.   Unfortunately,

printer manuals vary greatly in the type of information given, and where

it is hidden.   It is acceptable to guess at answers since the printer

test will help you to find the correct values.

EASY WAY: Find a printer that uses the same graphics codes

and copy the information to a new line.   Then change the

manufacturer and model the new entry.

Or you can edit the PRTMODEL.ASC file following this format:

MANUFACTURER: The company name, up to 16 characters.

MODEL: The printer model, up to 20 characters.

TYPE: Enter 'DM' for dot-matrix, 'IJ' for ink jet, or

'LS' for lasers.   Unusual types are shown by the

way that they are programmed.

DIRECTION: Enter the direction that bits are printed from

each byte.   For 8, 16, and 24-pin printers, the bits

in each byte are printed bottom 'UP', or top down('DN').

For one pin printers, the bits in each byte are

printed to the LEFT ('LF'), or to the RIGHT ('RT').

COLOR: IBM standard printers (most) will print a dot with

the bit set (B&W); Apple prints bits off (W&B).

Graphics data sent to the printer usually must be

converted to CYAN-MAGENTA-YELLOW-BLACK ('CMY').  But,

most ink jets want RED-GREEN-BLUE ('RGB') data.

CODE SET: Find the control code set in PRTCODE.ASC that

matches your printer.  Code sets are in numeric order.

PINS: Enter the number of pins used for graphics only.

This is pretty obvious for 8-pin and 24-pin printers

but beware!   Lasers, ink jets, and 18-pin printers

may use 8-pin control codes for graphics!  Otherwise,

lasers and ink jets are usually 1-pin printers.

COLORS: This is the actual number of inks, usually four for

CMY printers and three for RGB printers.

WIDTH: Printers cannot print graphics across the full width

of the paper.   Enter the maximum print width(inches).

LENGTH: Enter a length if the printer usually feeds cut sheet.

Enter zero for all continuous feed printers.

HEAD SPACING: This is the vertical spacing of the pins in

the print head.   Few manuals tell this.   Put in the

appropriate guess and let SETPRINT adjust it:

Ink jet:   maximum density (150, 180 or 300)

8-pin:      72 per inch

16-pin:    144 per inch

24-pin:    180 per inch

Lasers:    300 per inch

FLAGS: These are bit flags for non-standard features:

Bit 0 = 1 means horizontal density is non-standard.

Bit 1 = 2 if GL requires horizontal dots (else bytes).

Bit 2 = 4 if continuous paper is possible (ink jets).

Bit 3 = 8 if dot size proportional to density (lasers).

Bit 4 =16 if GL command is followed by n red-bytes, n

green-bytes, and n blue-bytes.

Bit 5 =32 if GL command includes automatic line advance

proportional to selected density.

INSTRUCTIONS for adding control codes to PRTCODE.ASC:

------------ --- ------ ------- ----- -- ------------

The PRTCODE.ASC file contains the sets of control codes that your graphics

program will use to send graphics to your printer.    PRTMODEL.ASC relates

the printer make and model to the correct set of codes.  PRTCODE.ASC has

the actual codes.   You can modify a set of control codes or add a new set

with a wordprocessor.  If your wordprocessor saves in a special format, you

may need to print the new version to disk and then rename the file.

HEADER: Each set of codes starts with header such as:

'Epson JX series',30,'CMY'

The first item is the name of the code set and must

be 6 to 30 characters.   The second item is the set

number which should be the next in order in the file.

The last item is as follows:

B&W for standard black on white printers.

W&B for Apple printers (bit meaning reversed).

CMY for color using Cyan-Magenta-Yellow data.

RBG for color using Red-Green-Blue data (HP).

COMMANDS: Each control code is in the following form:

'FUN',size,'000 000 000 000 x 000'

FUN is the function code from the list below.

size is the dots per inch or increment per inch.

000 is a list of ASCII codes from your manual.

x will be one of the following to show what type

of number should be inserted in the code:

a: an ASCII number of several digits.

b: a one byte binary value.

i: a two byte Intel integer (low byte first).

m: a two byte Motorola integer (high byte first).

FUNCTION CODES: The FUN in each code is chosen from the

following list.   The simplest possible set is one

GL code and one of VL or VM codes.

GL: Most dot-matrix printers use one code sequence to

send graphics data.  The size is the horizontal

dots per inch.   The number inserted in the code

is the number of bytes for 1 and 8-pin printers,

and the columns for 16 and 24-pin printers.

GI: Lasers and ink jets often set the graphics density

and then switch to a graphics mode.  size is the

density and no numbers are inserted.   If density

is set here, then the GL command has a size of 1.

This often contains several commands up to 16 ASCII

numbers.

GW: Hewlett-Packard PaintJets require a graphics width

command.   Calculate the width in the units of 'size'

independant of density.

GE: If a GI command is needed then a GE will be used to

end graphics mode.  size is usually 1, and no value

is inserted in the code.

VERTICAL MOVEMENT: One of the following two commands is

needed for 8, 16, and 24-pin graphics.  1-pin codes

do not need these commands if a vertical move is

automatic at the end of each graphics line.

VL: Vertical Line sets the vertical amount for any

LF codes until changed.   size is increments per

inch.   The amount is inserted in types a or b.

VM: Vertical Move is like VL but must be sent at the

end of each graphics line.   size is increments

per inch.  The amount is a or b type as above.

COLORS: Most printers set the ribbon color in a command.

CC: Selects CYAN ribbon or ink.

CM: Selects MAGENTA ribbon or ink.

CY: Selects YELLOW ribbon or ink.

CX: Selects BLACK ribbon or ink.

A few printers want red-green-blue graphics data.

CB: Selects BLUE ribbon or ink.

CG: Selects GREEN ribbon or ink.

CR: Selects RED ribbon or ink.

If the color is within the GL command then use the

following with size and number the same as GL:

GLR: red graphics data.      GLC: cyan graphics data.

GLG: green graphics data.    GLM: magenta graphics data.

GLB: blue graphics data.     GLY: yellow graphics data.

GLX: black graphics data.

TESTING:

--------

Each code set MUST be tested on a printer that uses that code set.  If

it is a printer that emulates another manufacturer, then we need to test it

on the printer that ORIGINATED that code set!