NAME

setdtr -- set state of data terminal ready flag

setrts -- set state of ready to send flag

SYNOPSIS

void setdtr(port, mode);

void setrts(port, mode);

int port;        0, 1, 2, or 3 (for COM1-COM4)

int mode;        TRUE to set flag, FALSE to clear flag

DESCRIPTION

These are assembly language functions which provide direct

access to the serial communications chips for the fastest

access possible.  Each function sets a particular

communications output bit to the condition stated by "mode".

The port parameter may be specified as integers 0-3, or

SER1 to SER4 as defined in <smdefs.h>.

EXAMPLE

int sport;        /* serial port */

sport = SER2;     /* let it point to COM2 */

setdtr(sport, YES);  /* take the modem offhook */