NAME

usec_clock     -- read the system clock in microseconds

restart_uclock -- re-initialize the microsecond clock

usec_delay     -- delay a given number of microseconds

usec_difftime  -- calculate the difference between two times

usec_timeout   -- test for a timeout condition

msec_pause     -- pause a given number of milliseconds

SYNOPSIS

(uclock_t) r = usec_clock();

uclock_t r  = returns zero if first call, thereafter

returns microseconds elapsed

(void) restart_uclock();

(uclock_t) end = usec_delay(usec);

uclock_t end   = usec_clock() value at end of delay

uclock_t usec  = number of microseconds to delay

(uclock_t) diff = usec_difftime(start, end);

uclock_t diff  =  microseconds elapsed between start & end

uclock_t start =  start time (from usec_clock())

uclock_t end   =  ending time (from usec_clock())

LOGICAL stat = usec_timeout(start, end, usecs);

LOGICAL stat   = TRUE if time limit exceeded, else FALSE

uclock_t start = start time (from usec_clock())

uclock_t end   = ending time (from usec_clock())

uclock_t usecs = time limit in microseconds

(void) msec_pause(msec)

long msec =      number of milliseconds to pause

DESCRIPTION

These functions implement timing functions with an accuracy of

one microsecond on IBM and closely compatible computers. To

function properly, these functions require the host system to use

either an 8253 or 8254 counter/timer chip based at I/O address 40h

to generate the 18.2 Hz system interrupt used by DOS. These

functions further assume that DOS uses the four bytes at 40:6C for

its timekeeping function. These have proven to be safe assumptions

for all IBM PC's and PS/2's, Compaq's, as well as most other clone

computers and single-board computers designed for MS/PC-DOS

compatibility. The sole exception is msec_pause() which requires

the counter/timer chip but makes no other assumptions.

The key function is usec_clock() which is a microsecond-accurate

replacement for the normal clock() function. Just as the time

returned by clock() may be converted to seconds by dividing by

CLK_TCK, values returned by usec_clock may be similarly converted

by dividing by UCLK_TCK, defined in RBS.H.

uclock functions,  page 2

The usec_clock() function relies on data which rolls over at

midnight each day and can handle this. However, if a program is to

run continuously for many days, it becomes necessary to take

action to prevent internal overflow. This may be done by resetting

the internal registers used by usec_clock() at least once per day

by calling restart_uclock().

Two delay functions are provided; usec_delay() which is passed a

number of microseconds, and msec_pause() which is passed a number

of milliseconds. Other than the time units, one difference between

the two is that msec_pause() is slightly more portable as noted

above. Another difference is that usec_delay() returns the value

returned by usec_clock() when it finished. This is done in order

to be able to chain time measurements.

Two differential time functions are provided; usec_difftime(), a

macro which simply returns the difference between two usec_clock()

readings, and usec_timeout() which tests for an expired time limit.

EXAMPLE

uclock_t time_1, time_2;

int i;

time_1 = usec_clock(); /* take an initial clock reading */

for (i = 1; i < 1000; ++i)

{

/* do things here but we don't know for how long */

}

time_2 = usec_delay(500000L); /* pause 0.5 second, note time */

do {

/* do things here for 1.2345 seconds */

if (5000000L > usec_difftime(usec_clock() - time_1))

break; /* abort if total time is over 5 sec. */

} while (!timeout(time_2, usec_clock(), 1234500L));