NAME

ticker -- timer interrupt service routine package

SYNOPSIS

void installtick(&counter);

void removetick();

unsigned int counter;     integer to decrement

DESCRIPTION

"ticker" is not actually a C function, but a routine which

is installed on interrupt 1CH.  At each interrupt (18.21

times per second) this routine decrements an integer (16 bits)

whose address was passed during installation.  The C program

can load and test this integer at any time to perform time-based

operations.  When the variable is decremented to zero, it

is not decremented any further; i.e., no underflow occurs.

Therefore, the integer is actually an unsigned integer with

a maximum count of 65535, or (65535/18.2) seconds.

installtick() installs the ticker on interrupt 1CH and

saves the address of the passed variable. removetick()

restores the interrupt 1CH vector to its original state.

Any other routines already installed on this vector are

executed after ticker, since the ticker chains to the

original vector after execution.

EXAMPLE

int  counter;

main() {

installtick(&counter);

counter = 18 * 10;  /* about 10 seconds */

while(counter > 0); /* delay here */

removetick();

}

CAVEAT

It is imperative to call removetick() BEFORE terminating

the program,  Otherwise, the system will crash!