NAME

vga_colorset -- set a DAC color value

vga_colorget -- get a DAC color value

SYNOPSIS

void vga_colorset(number, red, green, blue);

void vga_colorget(number, &red, &green, &blue);

int number, red, green, blue;

DESCRIPTION

These functions allow the manipulation of the Digital to

Analog registers in a VGA video system.  It is assumed that

proper tests for VGA mode have been previously used before

invoking these functions.  The values red, green, and blue

will always be in the range of 0 (off) to 63 (highest).

Any of over 256,000 colors can be made by adjusting the

6-bit combinations of each color.  Setting all three

to equal values will makes various shades of gray, though

this does depend somewhat on the monitor itself.

The number of the color palette should be one of the numbers

in the Turbo-C header file beginning with "EGA_".

WARNING:  TURBO-C GRAPHICS VGA MODE IS REQUIRED!

EXAMPLE

int red, green, blue

// first, save existing DAC values:

vga_colorget(EGA_LIGHTMAGENTA, &red, &green, &blue);

// now, alter the light magenta color to something less obnoxious:

vga_colorset(EGA_LIGHTMAGENTA, 10, 40, 60);

// assuming that init_graph was previously called:

setcolor(EGA_LIGHTMAGENTA);

outtextxy(100, 100, "Hello World");

// restore original DAC values and watch the above line change

vga_colorset(EGA_LIGHTMAGENTA, red, green, blue);