NAME

vid_wrpix -- write a pixel to the screen

vid_rdpix -- read a pixel from the screen

SYNOPSIS

(void) vid_wrpix(x, y, clr);

clr = vid_rdpix(x, y);

int x, y;      x and y screen coordinates

int clr;       one of the 4 color values for current palette

DESCRIPTION

These two functions manipulate the CGA card, or EGA card in

CGA graphics mode.

vid_wrpix() will set the pixel at location x,y to the color

clr, which is 0, 1, 2, or 3.  The real color will be as

set via vid_palette().  For black and white 640 graphics,

use 0 or 1 only.

vid_rdpix() will return the color value (0-3) at the specified

x,y location.

If using black&white 640 x 200 graphics mode on a CGA, then

the color value for vid_wrpix should be 0 or 1.  0 will set

the pixel to black (which is the background color), and 1 will

set the pixel to the color which has been previously selected

by vid_border()  (the default is, of course, white).  In BW640

mode, therefore, the pixels can only be turned on or off.  There

is no possibility of multi-color displays.

These functions do not support the extra modes of the EGA or PGA

adapters.

EXAMPLE

vmode(CG320);   /* set 320 color graphics mode */

vid_border(BLUE);  /* set border and graphics background color */

vid_palette(1);    /* select palette 1 */

vid_wrpix(10, 10, 2)  /* write a pixel at x10, y10, color 2,

which is magenta in palette 1 */