NAME

chkdcd  -- check state of carrier detect flag

chkdsr  -- check state of data set ready flag

chkcts  -- check state of clear to send flag

chkring -- check state of ring indicator flag

SYNOPSIS

r = chkdcd(port);

r = chkdsr(port);

r = chkcts(port);

r = chkring(port);

int r;           TRUE or FALSE, depending upon state

int port;        0, 1, 2, or 3 (for COM1-COM4)

DESCRIPTION

These are assembly language functions which provide direct

access to the serial communications chips for the fastest

access possible.  Each function checks a particular

communications flag and returns its state as TRUE (on) or

FALSE (off).

The port parameter may be specified as integers 0-3, or

SER1 to SER4 as defined in <smdefs.h>.

EXAMPLE

int sport;        /* serial port */

sport = SER2;     /* let it point to COM2 */

if(chkdcd(sport)) puts("We have a carrier detected");

else puts("The modem is apparently idle");