Video 7 Super VGA.

Now Headland Technologies

Earlier V7/Headland boards use Chips&Technologies and Cirrus chips.

V7VRAM

V71024i

Both use the HT-208 (V7VGA) chip.

100h (R/W?): Microchannel ID low

bit 0-7  Card ID bit 0-7

101h (R/W?): Microchannel ID high

bit 0-7  Card ID bit 8-15

102h (R/W): Alt Video Subsystem Enable

bit 0  Enable Video if set

Must be armed by 3C4h index 0FCh bit 7

or in setup mode (46E8h bit4) to change.

3C2h (W): Misc Output register

bit 5  Bit 1 of Bank no.

Note:  This register can be read at 3CCh.

3C3h (R/W): Video Subsystem Enable

bit 0  Enable Microchannel Video if set

Must be armed by 3C4h index 0FCh bit 7 to change.

3C4h index  6  (R/W): Extension Control

bit   0  (Read Only) Extensions enabled if set

bit 0-7  (Write Only) 0EAh Enables extensions, 0AEh disables.

3C4h index  7  (R/W): Reset Horizontal Character Counter

3C4h index 80h (R/W): Test

3C4h index 81h (R/W): Test

3C4h index 82h (R/W): Test

3C4h index 83h (R/W): Attribute Control Index

3C4h index 8Eh-8Fh (R): Chip Version

bit 0-7  Chip version:

80h-FFh: VEGA VGA Chip,

70h: V7VGA chip revision 1,2 or 3

71h: V7VGA chip revision 4

50h-59h: V7VGA Version 5

41h-49h: 1024i.

3C4h index 94h (R/W): Pointer Pattern Address

bit 0-7  Bit 6-13 of the start address of the Pointer Pattern

3C4h index 9Ch (R/W): Pointer Horizontal Position High

bit 0-2  Bit 8-10 of the X coordinate of the Pointer

3C4h index 9Dh (R/W): Pointer Horizontal Position Low

bit 0-7  Bit 0-7 of the X coordinate of the Pointer

3C4h index 9Eh (R/W): Pointer Vertical Position High

bit 0-1  Bit 8-9 of the Y coordinate of the Pointer

3C4h index 9Fh (R/W): Pointer Vertical Position Low

bit 0-7  Bit 0-7 of the Y coordinate of the Pointer

3C4h index A0h (R/W): GC Memory Latch 0

bit 0-7  Plane 0 Memory Latch Data

3C4h index A1h (R/W): GC Memory Latch 1

bit 0-7  Plane 1 Memory Latch Data

3C4h index A2h (R/W): GC Memory Latch 2

bit 0-7  Plane 2 Memory Latch Data

3C4h index A3h (R/W): GC Memory Latch 3

bit 0-7  Plane 3 Memory Latch Data

3C4h index A4h (R/W): Clock Select

3C4h index A5h (R/W): Cursor Attributes

bit 0  Cursor blink enabled if clear

3  Text Cursor Mode is XOR if set, Replace if clear

7  Graphics Cursor Enabled if set

3C4h index B0h-BFh (R/W): Scratch Registers

bit 0-7  Scratch

3C4h index E0h (R/W): Miscellaneous Control      (Version 4+)

bit 0-6  Reserved

7  Enables Split Bank Mode

3C4h index E8h (R/W): Single/Write Bank Register (Version 4+)

bit 4-7  Single/Write Bank no.

3C4h index E9h (R/W): Read Bank Register         (Version 4+)

bit 4-7  Read Bank no.

Only Active if Split mode enabled (3C4h index E0h bit 7)

3C4h index EAh (W): Switch Strobe

Note:  A write to this register copies the switch positions to

the Switch Readback Register (3C4h index F7h).

3C4h index EBh (R/W): Emulation Control

3C4h index ECh (R/W): Foreground Latch 0

bit 0-7  Foreground Latch for plane 0. When in Dither Foreground mode

(3C4h index FEh bit 2-3 = 2) the data in this register

replaces the data written from the processor.

3C4h index EDh (R/W): Foreground Latch 1

bit 0-7  Foreground Latch for plane 1.

3C4h index EEh (R/W): Foreground Latch 2

bit 0-7  Foreground Latch for plane 2.

3C4h index EFh (R/W): Foreground Latch 3

bit 0-7  Foreground Latch for plane 3.

3C4h index F0h (R/W): Fast Foreground Latch Load

bit 0-7  The Foreground Latches (3C4h index ECh to EFh) for the four

memory planes can be loaded by writing to this register.

The writes will cycle through planes 0-3.

A read will restart at plane 0.

3C4h index F1h (R/W): Fast Latch Load State

bit 0-1  Background Latch Load State. Determines which of the

four memory latches will be loaded by a write to 3C4h

index F2h. Each write to index F2h will increment this

value and each read from index F2h will reset it to 0.

2-3  Unused

4-5  Foreground Latch Load State. Determines which of the

four Foreground latches (3C4h index ECh to EFh) will

be loaded by the next write to 3C4h index F0h.

Each write to index F0h will increment this value

and each read from index F0h will reset it to 0.

6-7  Unused

3C4h index F2h (R/W): Fast Background Latch Load

bit 0-7  The Memory Data Latches for the four memory planes can be

loaded by writing to this register. The writes will cycle

through planes 0-3. A read will restart at plane 0.

3C4h index F3h (R/W): Masked Write Control   (Only with VRAM)

bit 0  Enables Masked Writes if set

1  If set rotated CPU byte is used as WriteMask, else

Masked Write Mask register is used.

3C4h index F4h (R/W): Masked Write Mask      (Only with VRAM)

bit 0-7  If Masked Writes enabled by 3C4h index F3h bit 0

Only the bits set here will be updated in Video memory.

3C4h index F5h (R/W): Foreground/Background Pattern

bit 0-7

3C4h index F6h (R/W): 1MB RAM Bank Select

bit 0-1  Write Bank no bit 2-3 if 256 color, bit 0-1 else.

2-3  Read Bank no bit 2-3 if 256 color, bit 0-1 else.

4-5  CRTC Bank (Adress bit 16-17)

6  Display address Wraps Around at bank boundry if set

7  Split Screen Wraps around at bank boundry if set

3C4h index F7h (R/W): Switch Readback

bit 0-7  Switch positions as read by the last write to the Switch

Strobe Register (3C4h index EAh)

3C4h index F8h (R/W): Extended Clock Control

3C4h index F9h (R/W): Page Select

bit 0  bit 16 of Video Memory Address. (Only needed if in a

256 color mode, and 3C4h index FCh bit 1-2 = 1).

3C4h index FAh (R/W): Extended Foreground Color

bit 0-3  Foreground expansion color.

Bit 0 is written to plane 0 etc.

3C4h index FBh (R/W): Extended Background Color

bit 0-3  Background expansion color

Bit 0 is written to plane 0 etc.

3C4h index FCh (R/W): Compatibility Control

bit 0  Enable Extended Attribute functions if set

Extended attributes allows underlining using a mask

in plane 3 for each character.

1  256-Color Paging Enabled if set.

2  256-Color 64K/128K paging Select.

128K pages if set, 64K pages else.

3-6  Reserved.

7  If set allows enabling VGA via 102h bit 0 or 3C3h bit 0.

3C4h index FDh (R/W): Extended Timing Select

3C4h index FEh (R/W): Foreground/Background Control

bit 0  Unused

1  Foreground/background source select

Source is CPU data if set, 3C4h index F5h else.

2-3  Foreground/background mode select

0  Standard VGA mode

1  Color Expansion Mode

A monochrome bitmap is expanded to color.

For each bit of of data written from the processor

a zero bit causes the background color (3C4h index FBh)

to be written in the corresponding pixel, and a 1 bit

causes the foreground color (3C4h index FAh) to be written.

2  Dithered foreground. The data from the processor is

replaced by data from four Foreground Latches (3C4h index

ECh to EFh). The normal VGA Read Latches function as normal.

3  Invalid

4-7  Unused

3C4h index FFh (R/W): 16 bit Interface Control

bit 0  16 bit memory if set

1  16 bit I/O if set

2  Fast Write Enabled if set

3  16 bit ROM access if set

4  Enable bank selection

5-6  Cursor Pattern Page Select

7  (Read only) Card in 8 or 16 bit slot

3d4h index 1Fh (R): Identification register

bit 0-7  Returns bit 0-7 of the Start Adress High Register

(3d4h index 0Ch) xored with 0EAh.

3d4h index 22h (R): Graphics Controller Data Latch

bit 0-7  Data from one of 4 bit-planes selected through the

RMS field of the Read Map Select Register (3CEh index 4)

3d4h index 24h (R): Graphics Controller Data

bit 0-4  Attribute Index. Same as 3C1 bit 0-4.

5  Palette source.  Same as 3C1h bit 5

7  Does the Attribute Controller point to Index

or Data .

3d4h index 30h-3Fh (W): Clear Vertical Display

bit 0  if set speeds up video memory access by increasing the

vertical Retrace Period.

46E8h (W): Rom Map & Video Subsystem

bit 0-2  Enable VGA PC/AT

3  Enable VGA PC/AT if set

4  Enter VGA Setup Mode if set

In Setup Mode only registers 102h, 3C3h and 46E8h

are active.

4BC4h,4BC5 used.

Bank Select:

Two methods exists:

All models can use method 1:

For 2 and 4 color modes bit 16 of the address is in 3C2h bit 5.

For 16 color modes seperate read and write banks can be selected

through 3C4h index F6h.

For 256 color modes the bank fields in 3C4h index F6h select

bit 18&19 of the video memory address.

Bit 16 is selected through 3C4h index F9h bit 0.

Bit 17 is selected through Miscellanous Output Register

(3C2h/3CCh bit 5).

Bit 16&17 are shared by read and write operations.

For Version 5 and above seperate bank registers are available.

Both read and write banks exists.

ID Video 7 VGA Chip Set:

if odd(port[$3CC]) then base:=$3d4 else base:=$3b4;

port[base]:=12;old_value:=port[base+1];

port[base+1]:=$55;new_value:=port[base+1];

port[base]:=$1f;id:=port[base+1];

port[base]:=12;port[base+1]:=old_value;

if id=byte($55 xor $ea) then VIDEO7_VGA !!!

ID Specific Video7 VGA Chip:

port[$3C4]:=$8E;

case port[$3C5] of

$80..$FF:VEGA_VGA;

$70..$7F:V7VGA_FASTWRITE/VRAM; {Revision 3}

$50..$59:V7VGA_Version5;

$41..$49:Video7_1024i;

end;

Video Modes:

40h  T   80   43   2

41h  T  132   25   2

42h  T  132   43   2

43h  T   80   60  16

44h  T  100   60  16

45h  T  132   28  16

60h  G  752  410  16 planar

61h  G  720  540  16 planar

62h  G  800  600  16 planar

63h  G 1024  768   2 planar

64h  G 1024  768   4 planar odd/even

65h  G 1024  768  16 planar

66h  G  640  400 256 packed    V1024 w/VRAM

67h  G  640  480 256 packed

68h  G  720  540 256 packed    VRAM only

69h  G  800  600 256 packed    VRAM only

BIOS extensions:

----------106F00-----------------------------

INT 10 - VIDEO - INSTALLATION CHECK (Video7 VGA,VEGA VGA)

AX = 6F00h

Return: BX = 5637h ('V7') indicates Video7 VGA/VEGA VGA extensions are present

----------106F01-----------------------------

INT 10 - VIDEO - GET MONITOR INFO (Video7 VGA,VEGA VGA)

AX = 6F01h

Return: AL = monitor type code (VEGA VGA only)

AH = status register information

bit  0 = display enable

0 = display enabled

1 = vertical or horizontal retrace in progress

bit  1 = light pen flip flop set

bit  2 = light pen switch activated

bit  3 = vertical retrace if set

bit  4 = monitor resolution

0 = high resolution (>200 lines)

1 = low resolution (<=200 lines)

bit  5 = display type

0 = color

1 = monochrome

bits6,7= diagnostic bits

Note: bits 0-3 are the same as the EGA/VGA status register bits 0-3

----------106F04-----------------------------

INT 10 - VIDEO - GET MODE AND SCREEN RESOLUTION (Video7 VGA, VEGA VGA)

AX = 6F04h

Return: AL = current video mode (see AX=6F05h)

BX = horizontal columns (text) or pixels (graphics)

CX = vertical columns (text) or pixels (graphics)

SeeAlso: AX=6F05h

----------106F05-----------------------------

INT 10 - VIDEO - SET VIDEO MODE (Video7 VGA, VEGA EXTENDED EGA/VGA)

AX = 6F05h

BL = mode (see below)

SeeAlso: AH=00h,AX=0070h

Values for video mode:

text/ text pixel	 pixel   colors disp	 scrn  system

grph resol  box  resoltn 	       page addr

00h-13h = standard IBM modes (see AH=00h)

40h = T   80x43  8x8                            Video7/VEGA VGA

41h = T  132x25  8x14                           Video7/VEGA VGA

42h = T  132x43  8x8                            Video7/VEGA VGA

43h = T   80x60  8x8                            Video7/VEGA VGA

44h = T  100x60  8x8                            Video7/VEGA VGA

45h = T  132x28  8x8                            Video7/VEGA VGA

60h = G               752x410   16              Video7 VGA, VEGA VGA

61h = G               720x540   16              Video7 VGA, VEGA VGA

= G               720x540   16              Northgate

62h = G               800x600   16              Video7 VGA, VEGA Ext EGA

63h = G               1024x768  2               Video7 VGA

64h = G               1024x768  4               Video7 VGA

65h = G               1024x768  16              Video7 VGA, VEGA Ext EGA

66h = G               640x400   256             Video7 VGA, VEGA Ext VGA

= G               640x400   256             Northgate

67h = G               640x480   256             Video7 VGA, VEGA Ext VGA

68h = G               720x540   256             Video7 VGA, VEGA Ext VGA

69h = G               800x600   256             Video7 VGA, VEGA Ext VGA

70h = G               752x410   16gray          Video7 VGA, VEGA VGA

71h = G               720x540   16gray          Video7 VGA, VEGA VGA

72h = G               800x600   16gray          Video7 VGA

73h = G               1024x768  2gray           Video7 VGA

74h = G               1024x768  4gray           Video7 VGA

75h = G               1024x768  16gray          Video7 VGA

76h = G               640x400   256gray         Video7 VGA

77h = G               640x480   256gray         Video7 VGA

78h = G               720x540   256gray         Video7 VGA

79h = G               800x600   256gray         (future)

SeeAlso: AH=00h,AX=0070h,AX=007Eh,AX=6F04h

----------106F06-----------------------------

INT 10 - VIDEO - SELECT AUTOSWITCH MODE (V7VGA,VEGA VGA)

AX = 6F06h

BL = Autoswitch mode select

00h select EGA/VGA-only modes

01h select Autoswitched VGA/EGA/CGA/MGA modes

02h select 'bootup' CGA/MGA modes

BH = enable/disable (00h enable, 01h = disable selection)

----------106F07-----------------------------

INT 10 - VIDEO - GET VIDEO MEMORY CONFIGURATION (V7VGA,VEGA VGA)

AX = 6F07h

Return: AL = 6Fh

AH = bits 0-6 = number of 256K blocks of video memory

bit 7    = DRAM/VRAM (0: DRAM, 1: VRAM)

BH = chip revision (SR8F) (S/C Chip in VEGA VGA)

BL = chip revision (SR8E) (G/A Chip in VEGA VGA)

CX = 0000h

SeeAlso: AH=12h/BL=10h