IBM Color Graphics Adapter

3D8h (W): Mode Control register

bit 0  80x25 Alpha mode if set, 40x25 else

1  Graphics mode if set, alpha else.

2  BW mode if set, color else

3  Enable video signal if set

4  640x200 Graphics mode if set, 320x200 else

5  if set bit 7 of the attribute controlls background, else blink

3D9h (W): Color Select register

The function of this register depends on the active mode.

Text modes:      320x200 modes:         640x200 mode:

0  Blue border      Blue background        Blue foreground

1  Green border     Green background       Green foreground

2  Red border       Red background         Red foreground

3  Bright border    Bright background      Bright foreground

4  Backgr. color    Alt. intens. colors

5  No func.         Selects palette

Palette 0 is Green, red and brown,

palette 1 is Cyan, magenta and white.

3DAh (R): Status register

bit 0  If set retrace active and the video memory may be

accessed by the PC.

1  Lightpen trigger has occured if set

2  Lightpen switch is off if set

3  Vertical retrace active if set.

Video RAM can be accessed for the next 1.25 ms.

Video Modes:

00h T   40  25  2   (8x8)

01h T   40  25  2   (8x8)

02h T   80  25  16  (8x8)

03h T   80  25  16  (8x8)

04h G  320 200  4

05h G  320 200  4

06h G  640 200  2