Everex Micro Enhancer.

Ev678 and Ev236 use Trident chips.

3C2h (r/W)): Misc Output Register

bit 5  Bit 1 of Bank No.

3C4h index  8  (R/W):

bit 7  Bit 0 of Bank no.

Bank switching.

Everex uses a two bit bank no. with bit 0 in 3C4h index 8 bit 7

and bit 1 in 3C2h bit 5.

ID Everex Micro Enhancer:

RP.AX=$7000;

RP.BX:=0;

INTR($10,RP):

IF RP.AL=$70 THEN

BEGIN

NR:=RP.DX SHR 4;

IF (NR=$678) or (nr=$236) then TRIDENT

else EVEREX Ev(Nr).

END;

BIOS extensions:

----------100070-----------------------------

INT 10 - VIDEO - Everex Micro Enhancer EGA/Viewpoint VGA - EXTENDED MODE SET

AX = 0070h

BL = mode (see below)

SeeAlso: AH=00h,AX=6F05h,AX=7000h/BX=0004h,AH=FFh"GO32"

Values for video mode:

text/ text   pixel pixel    colors disp scrn monitor  adapter

grph  resol  box   resoltn         page addr

00h = G                640x480   16              multsync EGA,VGA

01h = G                752x410   16              multsync EGA,VGA

02h = G                800x600   16              multsync EGA,VGA

03h = T   80x34                                  multsync EGA,VGA

04h = T   80x60                                  multsync EGA,VGA

05h = T   94x29                                  multsync EGA only

06h = T   94x51                                  multsync EGA only

07h = T  100x43  8x14            16                       VGA only

08h = T  100x75  8x8             16                       VGA only

09h = T   80x44                                  EGA      EGA only

0Ah = T  132x25                                  EGA      EGA,VGA

0Bh = T  132x44                                  EGA      EGA,VGA

0Ch = T  132x25                                  CGA      EGA only

0Dh = T   80x44                                  mono     EGA only

0Eh = T  132x25                                  mono

0Fh = T  132x44                                  mono

10h =	 reserved

11h = G                1280x350  4               EGA only

12h = G                1280x600  4               EGA only

13h = G                640x350   256             EGA only

14h = G                640x400   256

15h = G                512x480   256

16h = T   80x30  8x16            256             VGA only

18h = T  100x27  8x16            16              VGA only

20h = G                1024x768  16              Everex 629,678

21h = T  160x64  8x16  1280x1024 16              1MB VGA only

30h = G                640x480   256             Everex 629,678

31h = G                800x600   256             Everex 629,678

32h = G  128x48  8x16  1024x768  256             1MB VGA only

40h = T  132x30  8x16            16              VGA only

50h = T  132x32  8x16            mono            VGA only

62h = G   40x25  8x8   320x200   32K             Vwpt TC (EV629)

70h = G   64x30  8x16  512x480   32K             Viewpoint TC

71h = G   80x30  8x16  640x480   32K             Viewpoint TC

76h = G   64x30  8x16  512x480   16M             Viewpoint TC

77h = G   80x30  8x16  640x480   16M             Viewpoint TC

----------107000BX0000-----------------------

INT 10 - Everex Extended Video BIOS - RETURN EMULATION STATUS

AX = 7000h

BX = 0000h

Return: CL = monitor type

00h mono

01h CGA

02h EGA

03h digital multifrequency

04h IBM PS/2

05h IBM 8514

06h SuperVGA

07h analog multifrequency

08h super multifrequency

CH = feature bits

bits 7,6: 00 = 256K memory

01 = 512K memory

10 = 1024K

11 = 2048K memory

bit 5:    special oscillator present

bit 4:    VGA protect enabled

bit 0:    6845 emulation

DX = video board info

bits 4-15: board ID model

bits 0-3:  board ID revision

DI = BCD BIOS version number

----------107000BX0004-----------------------

INT 10 - Everex Extended Video BIOS - GET PAGING FUNCTION POINTER FOR CURR MODE

AX = 7000h

BX = 0004h

Return: ES:DI -> FAR paging function (call with DL = page to set)

Note:   the word preceding ES:DI is the length of the function in bytes, and

the last byte of the function is a FAR return instruction.

SeeAlso: AX=4F05h,AX=7000h/BX=0000h,AX=7000h/BX=0005h

----------107000BX0005-----------------------

INT 10 - Everex Extended Video BIOS - GET SUPPORTED MODE INFO

AX = 7000h

BX = 0005h

CL = maximum number of modes to get info for

CH = mode type to get info for (see below)

DL = monitor type to get info for

ES:DI -> buffer for mode info (see below)

Return: CL = total number of modes fitting criteria

CH = size of each info record

SeeAlso: AX=7000h/BX=0000h,AX=7000h/BX=0004h

Values for mode type:

00h all modes

01h monochrome text modes

02h color text modes

03h four-color CGA graphics modes

04h two-color CGA graphics modes

05h 16-color graphics modes

06h 256-color graphics modes

Format of mode information record:

Offset Size  Description

00h   BYTE  mode number (bit 7 set if extended mode)

01h   BYTE  mode type (see above)

02h   BYTE  info bits

bits 7,6 reserved

5 monochrome mode

4 interlaced display

3 requires special oscillator

2,1 memory required

00 = 256K

01 = 512K

10 = 1024K

11 = 2048K

0 reserved

03h   BYTE  font height

04h   BYTE  text columns on screen

05h   BYTE  text rows on screen

06h   WORD  number of scan lines

08h   BYTE  color information

bits 7-4 reserved

3-0 bits per pixel