ATI Technologies Super VGA Chip Sets.

Board               18810 Dot    ROM BIOS

Version   Chip      Clock Chip   Label

V3   18800 rev 1     no          V3M

V4   18800 rev 2     no          V4M    (18800-1)

V5   18800 rev 2    yes          V5M    (18800-1)

?   28800                              VGA Wonder+

ATI Prism Elite uses Trident 8800CS chips.

3C0h (R/W):  Palette index register

bit 4-5  Mode 67h Palette

Value   Pixel=0    1      2            3

0       Black  White  Grey      Bright White

1       Black  Green  Red       Yellow

2       Black  Cyan   Red       White

3       Black  Cyan   Magenta   White

1CEh index B0h (R/W): ATI Register 0

bit 0  Reserved

3  Enable 8 CRT accesses for each CPU access

6  Hercules 300 line emulation

7  Reserved

1,2,4,5  DRAM timing

1CEh index B1h (R/W): EGA Compatability and Double Scanning Enable

bit 0  Force all I/O addresses to be EGA compatible if set

1  Force all registers to be EGA compatible if set

2  General purpose read/write

3-5  Double scanning/3 of 4 scanning enable

1: Enable double scanning in graphics mode

2: Enable 3 of 4 scanning in graphics mode

5: Enable double scanning in text mode

6: Enable 3 of 4 scanning in text mode

6  Divide vertical timing parameters by 2 if set

7  Reserved

1CEh index B2h (R/W): Bank Register

bit 0  (rev 1) Enable interlace if set

1-4  (rev 1) Bank no. in 64 chunks

5  (rev 1) Enable internal DIP switch settings (EGA mode)

6  (rev 1) External clock select

7  (rev 1) Reserved

0  (rev 2+) Reserved

1-3  (rev 2+) Write/Single bank no

4  (rev 2+) Reserved

5-7  (rev 2+) Read Bank no

1CEh index B3h (R/W): ATI Register 3

bit 0  EEPROM data input

1  EEPROM clock source

2  Enable EEPROM interface

3  EEPROM chip select

4  (rev 1)  Enable PS/2 decoding

(rev 2+) Disable memory beyond 256K

5  Enable 16bit operation

6  (rev 2+)  Enable 1024x768 16 color planar pixel mode

7  (rev 2+)  Enable double scanning for 200 line modes

Note: This register should not be modified on revision 1 chips.

1CEh index B4h (R/W): ATI Register 4

bit 0  Enable CGA emulation

1  Enable Hercules emulation

2  Lock CR90-94,CR97

3  Lock vertical timing registers

4  Lock cursor start and end

5  Lock CR80-86 and CR140-144

6  Lock CR0-7 instead of CR117

7  Override locking of CR117

1CEh index B5h (R/W): ATI Register 5

bit 0  Select display enable as blanking signal

1  Invert blanking signal polarity

2  Enable display signal skew

3  Select Map 3 as programmable character generator

4  Enable 8 simultaneous fonts if set

Background is then always 0, and bit 4-7 of an attribute

selects the font.

5  Disable Cursor Blinking if set

6  Enable CGA Cursor Emulation if set

Adds 5 to the cursor start and end registers.

7  Reserved

1CEh index B6h (R/W): ATI Register 6

bit 0  Reserved

1  Enable 640x400 Hercules emulation

2  Reserved

3  Select 4 color high res modes

4  Select 16 color high res modes

5  Enable vertical interrupts

6  Select composite sync for output

7  Disable blanking screen blank in CGA and Hercules

emulation

1CEh index B7h (R/W): ATI Register 7

bit 0-7  Reserved

1CEh index B8h (R/W): ATI Register 8

bit 0  Lock Palette registers in Attribute Controller

1  Lock Overscan register in Attributre Controller

2  Lock All VGA registers except CRTC start and end

3  Lock write to 3C2h

4  Lock horizontal sync polarity

5  Lock vertical sync polarity

6-7  Clock divider

1CEh index B9h (R/W): ATI Register 9

bit 0  Clock select

1  Select input to clock chip

2-3  RAM address space

4-5  Wait cycles for 16 bit ROM access

6  Set horizontal total = register value +2 (vs +5)

7  Lock Line Compare register

1CEh index BAh (R/W): ATI Register A

bit 0-2  Delay chain timing compensation

3  Disable secondary Red output (for RGB monitors)

4  Enable EGA color simulation for RGB monitors

5  Enable monochrome gray scale circuit

6  reserved

7  Delay chain resolution compensation

1CEh index BBh (R/W): Input Status Register

bit 0-3  Monitor Type:

0: EGA

1: PS/2 Analog Monochrome

2: TTL Monochrome

3: PS/2 Color

4: RGB Color

5: MultiSync

7: PS/2 8514

8: Seiko 1430

9: NEC Multisync 2A

A: Crystalscan 860/Tatung 1439

B: NEC Multisync 3D

C: TVM 3M

D: NEC MultiSync XL

E: TVM 2A

F: TVM 3A

4  reserved

5  512Kbytes if set, 256K else

6-7  Reserved

1CEh index BCh (R/W): ATI Register C

bit 0-7  reserved

1CEh index BDh (R/W): ATI Register D

bit 0-3  Reserved

4-7  EGA switch settings

1CEh index BEh (R/W): ATI Register E        (rev 2+)

bit 0  Unlock Vertical Display End register of the

CRT Controller

1  Enable interlace mode

2  Select internal EGA DIP switch value

3  Dual bank mode if set, single else

4-5  Reserved

6  Enable 1024x768 16 color mode

7  Enable 1024x768 4 color mode

3C0h (R/W):  Palette index register

bit 4-5  Mode 67h Palette

Value   Pixel=0    1      2            3

0       Black  White  Grey      Bright White

1       Black  Green  Red       Yellow

2       Black  Cyan   Red       White

3       Black  Cyan   Magenta   White

Note:  The ATI chips handles the indexed registers sligtly differently

from other VGA chips, as the index register must be written before

each read or write of the associated data registyer.

Bank Switching

Bank switching can use either one single bank register or two

seperate read and write bank registers (18800-2 and 28800 Only).

Banks map to 64k boundries.

Reserved locations:

$C000:$10   2 bytes   ATI Register (usually $1CE).

$C000:$31   9 bytes   '761295520' ID's ATI product

$C000:$40   2 bytes   '31' ID's ATI Super VGA

'32' = ATI EGA Wonder 800+

'22' = ATI EGA Wonder

$C000:$42   1 byte    Bit 0  Can switch between 8 and 16 bit ROM if set

1  Mouseport present if set

2  Non-intelace if set

3  Microchannel if set, PC/AT else

4  Use clock chip if set

7  Use C000:0000 to D000:FFFF with 16 bit ROM if set

$C000:$43   1 byte    Gate revision. '1' = Version 1,

'2' = version 2, '3' = version 2 with VRAM

$C000:$4C   1 byte    Major Bios version

$C000:$4D   1 byte    Minor Bios version

ID ATI Super VGA Chip Set

s[0]:=#9;

move(mem[$c000:$31],s[1],9);

if s='761295520' then ATI card

if memw[$c000:$40]=$3331  then ATI_Super_VGA

ID Which ATI Super VGA Chip

rp.ah:=12h

rp.bx:=5506h

rp.al:=55h

rp.bp:=FFFFh

rp.si:=0h

intr($10,rp);

if rp.bp=FFFFh then ATI_rev1 else ATI_rev>=2

Video Modes:

23h  T   132   25  16  (8x14)

27h  T   132   25   2  (8x14)

33h  T   132   44  16  (8x8)

37h  T   132   44   2  (8x8)

54h  G   800  600  16  planar

55h  G  1024  768  16  planar   18800 rev 2 or later

58h  T    80   33      (8x8)

5Bh  T    80   30      (8x16)

61h  G   640  400 256  packed

62h  G   640  480 256  packed

63h  G   800  600 256  packed

64h  G  1024  768 256  packed   VGA Wonder +

65h  G  1024  768  16  packed   ****  See note

67h  G  1024  768   4  planar   ****  See note

6Ah  G   800  600  16           Undocumented ??

ATI enhanced Graphics modes do NOT support INT 10h with AH=

01h..0Eh or 11h or 13h.

Mode 65h  1024x768 16 color

4 bits per pixel packed mode

Even pixel is in bits 0-3 of the byte, odd in bits 4-7.

Mode 67h  1024x768  4 color

2 bits per pixel planar mode

Even pixels are in plane 2&3, odd pixels in plane 0&1.

Bios extensions:

----------1012--BH55-------------------------

INT 10 - VIDEO - ALTERNATE FUNC SELECT (ATI,Tatung,Taxan) - ENHANCED FEATURES

AH = 12h

BH = 55h

BL = subfunction

00h disabled enhanced features

01h enable enhanced features

02h get status

Return:           AL = status flags

bit    3  set if enhanced features enabled

bits 7-5  monitor type

000 PS/2 mono

001 PS/2 color

010 multi-sync

011 Taxan 650 25kHz

100 RGB

101 mono

110 EGA

111 Compaq internal

03h disable register trapping (CGA emulation)

04h enable register trapping

05h program mode described by table at ES:BP

06h get mode table

AL = video mode

BP = FFFFh  (Known illegal value).

SI = 0000h  (Known illegal value).

Return:           ES:BP -> table suitable for mode AL (and subfnc BL=05h)

BP = FFFFh on error

Format of ATI VGA Wonder video mode table:

Offset  Size     Description

00h    BYTE     number of columns

01h    BYTE     maximum row (number of rows - 1)

02h    BYTE     scan lines per row

03h    WORD     video buffer size in bytes

05h   4 BYTEs   values for Sequencer registers 1-4

09h    BYTE	     value for Miscellaneous Output register

0Ah   25 BYTEs  	values for CRTC registers 00h-18h

00h horizontal total size (chars)

01h horizontal displayed (chars)

02h horizontal sync position (chars)

03h horizontal sync width (chars)

04h vertical total size (char rows)

05h vertical total adjust (scan lines)

06h vertical displayed (char rows)

07h vertical sync position (char rows)

08h interlace mode

09h max scan line in row

0Ah cursor start scan line

0Bh cursor end scan line

0Ch screen memory start (high)

0Dh screen memory start (low)

0Eh cursor address (high)

0Fh cursor address (low)

10h light pen (high)

11h light pen (low)

23h   20 BYTEs	  default palette (values for Attribute Controller regs 00h-13h)

37h   9 BYTEs	   values for Graphics Controller registers 00h-08h