Modem Unit

presented by

KSH Software

Copyright (C) 1990  Version 1.0

All Rights Reserved

2

TABLE OF CONTENTS

1.  INTRODUCTION . . . . . . . . . . . . . . . . . . . . . 3

1.1.  Necessary Equipment  . . . . . . . . . . . . . . . . 3

1.2.  Restrictions . . . . . . . . . . . . . . . . . . . . 3

1.3.  Licensing Agreement  . . . . . . . . . . . . . . . . 3

2.  MODEM UNIT . . . . . . . . . . . . . . . . . . . . . . 4

2.1.  Interface  . . . . . . . . . . . . . . . . . . . . . 4

2.1.1.  Constants  . . . . . . . . . . . . . . . . . . . . 5

2.1.2.  Procedures . . . . . . . . . . . . . . . . . . . . 6

2.1.2.1.  DisableInterrupts  . . . . . . . . . . . . . . . 6

2.1.2.2.  EnableInterrupts . . . . . . . . . . . . . . . . 6

2.1.2.3.  Init_Port  . . . . . . . . . . . . . . . . . . . 6

2.1.2.4.  Port_Stat  . . . . . . . . . . . . . . . . . . . 7

2.1.2.5.  IntEna . . . . . . . . . . . . . . . . . . . . . 7

2.1.2.6.  IntDis . . . . . . . . . . . . . . . . . . . . . 7

2.1.2.7.  HookCom13  . . . . . . . . . . . . . . . . . . . 8

2.1.2.8.  HookCom24  . . . . . . . . . . . . . . . . . . . 8

2.1.2.9.  UnHook . . . . . . . . . . . . . . . . . . . . . 8

2.1.2.10. Send_Char  . . . . . . . . . . . . . . . . . . . 8

2.1.2.11. Send_String  . . . . . . . . . . . . . . . . . . 8

2.1.3.  Functions  . . . . . . . . . . . . . . . . . . . . 8

2.1.3.1.  Read_Char : Char . . . . . . . . . . . . . . . . 8

2.2.  Sample Program . . . . . . . . . . . . . . . . . . . 9

2.2.1.  Explanation  . . . . . . . . . . . . . . . . . . . 9

2.2.2.  Suggestions  . . . . . . . . . . . . . . . . . .  10

3.  REGISTRATION INFORMATION . . . . . . . . . . . . . .  11

4.  HOW TO CONTACT ME  . . . . . . . . . . . . . . . . .  12

APPENDIX A.  BIBLIOGRAPHY  . . . . . . . . . . . . . . .  13

3

1.  INTRODUCTION

The unit in this archive were designed using Turbo

Pascal 5.5.  They should work with TP V5.5 without any

problem.

1.1.  Necessary Equipment

This unit will require Turbo Pascal V5.5, MS-DOS 3.2 or

later, and a modem.

1.2.  Restrictions

The MODEM unit will not run on the PS/1 or PS/2 series

of computers in Interrupt Mode.  The Interrupts $0B and $0C

are reserved on these machines and should not be hooked into!

The unit should work fine in polling mode!

1.3.  Licensing Agreement

These Units are distributed as shareware.  These

programs can be freely distributed onto BBS systems and other

shareware distribution sites as long as all files in the

archive remain intack.  HOWEVER, these units cannot be used

in any other piece of software without being first

registered.  Any unregistered use of these programs will

prosecuted to the full extent of the law.  All files in this

archive are Copyright 1990 by Sean A.R. Haynes.  All Rights

Reserved.

Please support the shareware principal!  Send in your

payments and save a starving hacker.

4

2.  MODEM UNIT

The MODEM unit allows the user to program the modem.

Finding out how to do this was somewhat difficult and

required a lot of research.  To adequately support the modem

at high baud rates (above 1200 baud) a hardware Interrupt

Serviceing Routine (ISR) had to be used.  The following

sections contain information on how to use the Unit as well

as hints for operating at high speeds.

2.1.  Interface

The following is a listing of the Interface section of the

source code.  It is Copyright 1990 by Sean A.R. Haynes.  All

Rights Reserved.  For more indepth information, I suggest

that you get the book listed in the bibliography.

Unit Modem;

Interface

Uses DOS;

Type

ParType = 0..2;  {NONE,ODD,EVEN}

Const

Com13Int  {Com 1&3 SW Interrupt}

Com24Int  {Com 2&4 SW Interrupt}

ModemInt  {Modem Interrupt}

THR_RDR   {Transmit/Receive Data Port Offset}

BRDL      {Baud Rate Divisor Register}

IER       {Interrupt Enable Register}

BRDH      {Baud Rate Divisor Register}

IIR       {Interrupt Identification Register}

LCR       {Line Control Register}

MCR       {Modem Control Register}

LSR       {Line Status Register}

MSR       {Modem Status Register}

var

Com : Packed Array[1..4] of Word; {Store Values of Com

Ports}

Procedure DisableInterrupts; inline($FA);

Procedure EnableInterrupts; inline($FB);

Procedure Init_Port

(PortNum:Byte;Baud:Word;Parity:ParType;StopBit:Byte;

WordLen:Byte;var PortStat,ModStat:Byte);

Procedure Port_Stat(PortNum:Byte;var PortStat,ModStat:Byte);

Procedure IntEna(PortNum:Byte;IntMask:Byte);

Procedure IntDis(PortNum:Byte);

Procedure HookCom13(Address:Pointer);

5

Procedure HookCom24(Address:Pointer);

Procedure UnHook;

Procedure Send_Char(PortNum:Byte;Value:Char;

var PortStat,ModStat:Byte);

Function Read_Char(PortNum:Byte) : Char;

Procedure Send_String(PortNum:Byte;Value:String);

2.1.1.  Constants and Variables

The following constants and variables are defined in the

Interface section:

Com13Int:  This is the software interrupt that exists in

BIOS to service Interrupts on Com 1 or Com 3.

Com24Int:  This is the software interrupt that exists in

BIOS to service Interrupts on Com 2 or Com 4.

ModemInt:  This is the software interrupt that exists in

BIOS to access the Com ports.  This method is EXTREMELY slow

and not recommended except for initialization.

The following constants are offsets for the Port values

used to access the Com Ports.  To access any of these Ports,

use the following formats:

Output ->  Port[Com[#]+Register] := Value

Input  ->  Value := Port[Com[#]+Register]

THR_RDR & BRDL:  This is the offset value for the

Transmit Holding Register, Receive Data Register and Baud

Rate Divisor Low Byte Register.  To access the BRD Register,

bit 7 of the LCR must be set.  To access the THR_RDR Register

bit 7 of the LCR must be clear.  Use the BRD Register to

access non-standard baud rates.  BRD = 1843200/(16*Baud).

IER & BRDH:  This is the offset value for the Interrupt

Enable Register (see IntEna) and the Baud Rate Divisor High

Byte Register (see above).

IIR:  This is the offset for the Interrupt

Identification Register.  This will tell you exactly what has

happened to cause the Interrupt.  The values are as followed:

Bit 0 -> Interrupt Pending when Clear.

Bit 1&2 -> 00 -> Change in MSR

01 -> THR empty

10 -> Data Received

11 -> Data Error or Break.

LCR:  This is the offset for the Line Control Register.

Set Bit 7 to access the BRD or Clear Bit 7 to access the

THR/RDR/IER.

6

MCR:  This is the offset for the Modem Control Register.

LSR:  This is the offset for the Line Status Register.

This is the same as reading the PortStat.

MSR:  This is the offset for the Modem Status Register.

This is the same as reading ModStat.

Com:  This is an array from [1..4] containing the Word

addresses of the Com Ports.  This is set to default values

for normal PCs and Clones.  If your Com Ports access

different addresses, then you can change them.

2.1.2.  Procedures

The following procedures are defined in the Interface

Section:

2.1.2.1.  DisableInterrupts; inline($FA);

This procedure inserts a CLI into the code.  Once

executed no hardware interrupts will be performed.

2.1.2.2.  EnableInterrupts; inline($FB);

This procedure inserts a STI into the code.  Once

executed hardware interrupts will be performed.

2.1.2.3.  Init_Port

(PortNum:Byte;Baud:Word;Parity:ParType;StopBit:Byte;

WordLen:Byte;var PortStat,ModStat:Byte);

This procedure will initialize a port for use.  It uses

software interrupt $14 to configure the port, therefore it

will only handle baud rates to 9600 and standard parity/stop

bit settings.

PortNum -> Com Port Number being accessed ( 1 thru 4 ).

Baud    -> Baud Rate to use.  Available selections are

(110, 150, 300, 600, 1200, 2400, 4800, 9600).

Parity  -> Parity to use (0=None, 1=Odd, 2=Even).

StopBit -> Number of stop bits (1 or 2).

WordLen -> Word Length to use (7 or 8).

PortStat-> This is the Port Status Byte which is

returned upon completion of most port

functions.  The bit values have the following

meaning:

Bit 0 -> Data Ready

Bit 1 -> Overrun Error

Bit 2 -> Parity Error

7

Bit 3 -> Framing Error

Bit 4 -> Break Detected

Bit 5 -> Transmit Holding Register (THR)

empty.

Bit 6 -> Transmit Shift Register (TSR) empty.

Bit 7 -> Time Out

Monitoring this register you can tell when

data can be sent (THR Empty) or received

(Data Ready).

ModStat -> This is the Modem Status Byte which is

returned upon completion of most modem

functions.  The bit values have the following

meaning:

Bit 0 -> Change in Clear to Send (CTS)

Status.

Bit 1 -> Change in Data Set Ready (DSR)

Status.

Bit 2 -> Trailing Edge Ring Indicator.

Bit 3 -> Change in Receive Line Signal.

Bit 4 -> CTS

Bit 5 -> DSR

Bit 6 -> Ring Indicator

Bit 7 -> Receive Line Signal Detected

2.1.2.4. Port_Stat(PortNum:Byte;var PortStat,ModStat:Byte);

This procedure will update the values of PortStat and

ModStat for the PortNum supplied.

2.1.2.5. IntEna(PortNum:Byte;IntMask:Byte);

This procedure will enable hardware interrupting on the

PortNum listed.  By using the IntMask parameter, you can

control the conditions on which the Port will interrupt the

CPU.  The following table lists the conditions:

Bit   Interrupts on       Action to Clear Interrupt

0 -> Data Received       Read RDR

1 -> THR Empty           Write to THR

2 -> Data Error or Break Read Port Status

3 -> MSR Change          Read Modem Status

4-7-> Unused, always 0

Using interrupts is a tricky process.  I suggest that

you look at the sample file to see a good example of using an

interrupt.

2.1.2.6. IntDis(PortNum:Byte);

This procedure will disable interrupting on the PortNum

listed.

8

2.1.2.7.  HookCom13(Address:Pointer);

This procedure will hook into the Com 1 and 3 interrupt.

You must supply the address of your ISR as the only

parameter.  Your ISR must end with the line:

Port[$0020] := $0020;

2.1.2.8.  HookCom24(Address:Pointer);

This procedure will hook into the Com 2 and 4 interrupt.

You must supply the address of your ISR as the only

parameter.  Your ISR must end with the line:

Port[$0020] := $0020;

2.1.2.9.  UnHook;

This should be the last line of your program before

exiting.  This will return all the ports, registers, and

pointers to there original values.

2.1.2.10.  Send_Char(PortNum:Byte;Value:Char;

var PortStat,ModStat:Byte);

This procedure will send a the character in Value out

the THR for the PortNum given.  It will return the updated

values of PortStat and ModStat.  This routine does not check

to see if the THR is empty!  You must do this yourself,

otherwise you may lose data.

2.1.2.11.  Send_String(PortNum:Byte;Value:String);

This procedure will send the string in Value to the

PortNum listed.  No testing of the THR is necessary if this

method is used.

2.1.3.  Functions

The following function is defined in the interface

section:

2.1.3.1.  Read_Char(PortNum:Byte) : Char;

This function will read a character from the PortNum

listed.  This routine does not check to see if Data is Ready!

You must do this yourself otherwise the data received maybe

invalid.

HINT:  For speeds greater than 4800 it is suggested that you

read RDR directly.  For speeds greater than 1200 baud, a

buffering technique maybe neccessary (and the interrupt MUST

be used).

9

2.2.  Sample Program

The program SAMMODEM.PAS is included in the archive as

an example of simple modem usage.

2.2.1.  Explanation

This is a really simple program which will send the

display modem setup command to the modem.  It will

automatically set the port to COM 2 with 9600 Baud, 8 bits,

no parity, and 1 stop bit.  I suggest that you change these

to whatever you need to execute the program.

The procedure Com2ISR is an interrupt routine that will

be called whenever a character is received.  Because the

Pascal Write routine is too slow, I have used the Write Char

in TTY Mode interrupt service.  This will work fine unless

the screen scrolls.  For suggestions on handling this see

Section 2.2.2.

Const

VidInt = $10;

WriteChar = $0E;

var

PortStat,ModStat,IERVal : Byte;

Regs : Registers;

DisPage : Byte;

Choice : Char;

Extended : Boolean;

Procedure Com2ISR; Interrupt; {COM Port 2 ISR}

var

IIRVal : Byte;

Begin

DisableInterrupts;              {Disable HW Interrupts}

IIRVal := Port[Com[2]+IIR];     {Identify Interrupt}

While IIRVal and $01 = $00 do   {Is Interrupt Pending?}

Begin                     {Yes!}

Case IIRVal of

0 : Port_Stat(2,PortStat,ModStat);

{Change in MSR}

4 : Begin            {Character Received}

Regs.AH := WriteChar;

{Write Char in TTY Mode}

Regs.AL := Byte(Read_Char(2));

{Read Character}

Regs.BH := DisPage;

Intr(VidInt,Regs); {Display Character}

End;

6 : Port_Stat(2,PortStat,ModStat);

{Error or Break}

10

End;

IIRVal := Port[Com[2]+IIR]; {Identify Interrupt}

End;

Port[$0020] := $20;             {Clear HW Interrupt}

EnableInterrupts;               {Enable HW Interrupts}

End;

Begin

DisableInterrupts;              {Disable HW Interrupts}

ClrScr;

Regs.AH := $0F;

Intr(VidInt,Regs);

DisPage := Regs.BH;           {Get Current Display Page}

Init_Port(2,9600,0,1,8,PortStat,ModStat);

{Initial Port 9600,N,8,1}

HookCom24(@Com2ISR);          {Hook Com2&4}

IntEna(2,$01);                {Enable Int on Com 2}

EnableInterrupts;             {Enable HW Ints}

Send_String(2,'AT&V'#13);     {Send String to Modem}

Delay(5000);                  {Sit around for awhile}

ClrScr;

Extended := False;

Repeat                        {Echo typed character to}

Choice := ReadKey;      {modem until extended}

If Choice = #0 then     {character typed in}

Extended := True

Else

Send_Char(2,Choice,PortStat,ModStat);

Until Extended;

Choice := ReadKey;

UnHook;                       {Unhook Interrupt}

IntDis(2);                    {Disable HW Modem Intr.}

End.

2.2.2.  Suggestions

The only problem that you may have is that when the

screen scrolls, you may lose data from the modem.  The way I

suggest coding is to create a rotating buffer of any size.

By keeping track of the start, end, base, and top addresses

of your buffer, you can basically store the characters to

memory and display them when you get a chance.

11

3.  REGISTRATION INFORMATION

Please register your copies of the Modem Unit.  Below is

a copy of the Registration form:  (you can also print out

REGISTER.FRM).

------------------------------------------------------------

KSH SOFTWARE

MODEM VERSION 1.0

-------------------------------------------------------------

Name: ______________________________________________________

Company: __________________________ Title: _________________

Address: _____________________________________ Apt. #: _____

City: ____________________________ State: ____ ZIP: ________

------------------------------------------------------------

Comments:  _________________________________________________

____________________________________________________________

------------------------------------------------------------

ELECTRONIC ADDRESSES

[  ]  BBS: Telephone #: (____) ____-______ Name: ____________

[  ]  Internet Address:  ____________________________________

[  ]  Bitnet Address:  ______________________________________

[  ]  Other:  Service ____________________ Name: ____________

-------------------------------------------------------------

ORDER FORM

[  ]  Basic Registration      Amount: ______ x $25 = ________

[  ]  Source Code             Amount: ______ x $10 = ________

Total:         ________

Please State Disk Size:  3-1/2" [  ]   5-1/4" [  ]

------------------------------------------------------------

Mail this form to:    KSH Software Registration

P.O. Box 952

Naples, ME 04055

12

4.  HOW TO CONTACT ME

I can be reached at the following addresses:

USnail:    Sean A.R. Haynes

46 Udine St.

Arlington, MA 02174-5657

INTERNET:  PA_HAYNES@NUHUB.NORTHEASTERN.EDU

SHAYNES@LYNX.NORTHEASTERN.EDU

HAYNES@NUENG.COE.NORTHEASTERN.EDU

BITNET:    PA_HAYNES@NUHUB

BBS:       Xevious BBS   (617) 449-7322

Channel 1 BBS (617) 354-8873

Future Tech   (617) 720-1330

VOICE:     (617) 648-8390

OTHER:     PC-Link    Killer68

Prodigy    PCPN95A

Comments and suggestions are always welcome.....

13

APPENDIX A.  BILBLIOGRAPHY

The one book that I recommend purchasing for more information

is:

Dettmann, Terry & Kyle, Jim, DOS PROGRAMMERS REFERENCE (2nd

Edition), Que Corporation, 1989.