To:  All                        Message #:  7666

From:  Andy Vaught                Submitted:  13 Jul 90  9:41:00

Subject:  LOADALL, 1/4                  Status:  Public

Received:  No                             Group:  80XXX (30)

Article 90 of comp.sys.intel:

Path: bilver!rtmvax!peora!ge-dab!steinmetz!uunet!ccicpg!turnkey!conexch!root

From: root@conexch.UUCP (Larry Dighera)

Newsgroups: comp.sys.intel

Subject: Re: Returning the 80286 to Real Mode

Summary: Have you heard about the LOADALL instruction?

Message-ID: <11004@conexch.UUCP>

Date: 31 Oct 88 14:05:04 GMT

References: <1081@xenon.idec.stc.co.uk>

Reply-To: root@conexch.UUCP (Larry Dighera)

Organization: The Consultants' Exchange, Orange County, CA.  (714) 842-6348

In article <1081@xenon.idec.stc.co.uk> sanders@idec.stc.co.uk (Tom Sanders)

writes:

>

>   Rumour has it that it is possible to switch the 80286 back to real mode

>   once having gone protected.  Can anyone advise me how this is done?

>

Here is a copy of an article that recently appeared in a periodical magizine.

The informatin it contains and the opinions expressed in it are not mine.

I'm sure you will find it useful and informitive.

------------------------  LOADALL DOCUMENTATION  -------------------------

Secret 286 LOADALL instruction allows access to extended memory in real mode.

In last month's article on the P9, we described the method used to access

extended memory (memory beyond the lower 1 megabyte) from real-mode

programs running on a 286.  This method requires switching to protected

mode to perform access, and then resetting the processor to return to

real mode.  We have since learned that there is another way.  An undocumented

286 instruction, LOADALL, allows all of the processor's registers (including

protected mode registers and hidden internal registers) to be loaded, even

when operating in real mode.  By changing the value of the descriptor

cache base register, a program can select a segment beyond the lower 1Mbyte.

LOADALL also has other uses, as described later in this article.

----------------------------------------------------------------------

Physical Memory Address    CPU register

(hex)

======================================================================

800-805                    none

806-807                    MSW (Machine Status Word)

808-815                    None

816-817                    TR (Task Register)

818-819                    Flag Word

81A-81B                    IP (Instruction Pointer)

81C-81D                    LDT (Local Descriptor Table)

81E-81F                    DS (Data Segment Selector)

820-821                    SS (Stack Segment Selector)

822-823                    CS (Code Segment Selector)

824-825                    ES (Extra Segment Selector)

826-827                    DI (Destination Index)

818-829                    SI (Source Index)

82A-82B                    BP (Base Pointer)

82C-82D                    SP (Stack Pointer)

82E-82F                    BX (Data Register B)

830-831                    DX (Data Register D)

832-833                    CX (Data Register C)

834-835                    AX (Accumulator)

836-83B                    ES Descriptor Cache

83C-841                    CS Descriptor Cache

842-847                    SS Descriptor Cache

848-84D                    DS Descriptor Cache

84E-853                    GDTR (Global Descriptor Table Register)

854-859                    LDT Descriptor Cache

85A-85F                    IDTR (Interrupt Descriptor Table Register)

860-865                    TSS (Task State Segment) Descriptor Cache

=======================================================================

Table 1.  LOADALL data area format

-----------------------------------------------------------------------

Originally included by Intel for chip testing, Microsoft is now using this

instruction in their RAM Drive program and in OS/2's compatibility box.

While this instruction is probably appropriate only for use in operating

systems and system-level utilities, it is important because it provides a

set of capabilities that are not otherwise available in a 286-based system.

We have received a copy of a document that describes LOADALL.  No company

name is shown, but is almost surely written by Intel.  The 15-page

document describes in detail how to use the instruction, so calling it

"undocumented" is not quite correct -- it is documented, but not in the

data sheet.  By restricting access to this documentation, Intel gives unfair

advantage to Microsoft and other large customers.

----------------------------------------------------------------------

Descriptor Cache Format

Bytes 0-2      24-bit segment base address

Byte 3         Access rights byte.  Format is the same as the access

rights byte in a descriptor, except that the "present"

bit becomes a "valid" bit.  If a "valid" bit is not

set, any memory reference using the descriptor will

cause exception 13 with error code of 0.

Bytes 4-5       16-bit segment size

GDTR and IDTR Format

Bytes 0-2      24-bit base address

Byte 3         0

Bytes 4-5      16-bit segment limit

----------------------------------------------------------------------

Table 2.  Descriptor Cache Formats

Intel's position is that this instruction is not useful to most users,

and can easily lead to machine crashes if not used properly.  Intel says

that they will provide documentation for the instruction on a "need-to-know"

basis, presumably only after a non-disclosure agreement has been signed.

Most developers, of course, would not know to ask.  The information we are

presenting here should be enough for you to evaluate the usefulness of the

instruction; if you intend to use it in a product, you should contact

Intel for the full documentation.

The opcode for LOADALL is 0F05 hex.  No operands follow the instruction;

LOADALL gets its data from a 102-byte block of memory starting at the

fixed address 800 hex, as shown in Table 1.  Table 2 shows the format of

the descriptor cache entries.  These are hidden registers that cannot

otherwise be modified by the programmer; they are set automatically when

a descriptor is read from the descriptor table.

ACCESSING EXTENDED MEMORY FROM REAL MODE

The following sequence is required to access high memory from real mode

using LOADALL:

1. Disable interrupts

2. Save the 102 (decimal) bytes starting at 800 hex.  (MS-DOS uses this

area for system code.  OS/2 presumably leaves it free for LOADALL,

but depends on application programs not to change it.)

3. Set up the 102-byte register image at 800.  The base address in the

data segment descriptor cache is set to select the desired high-memory

area.

4. Execute the LOADALL instruction.  The data segment now points to the

new, high-memory segment.

5. Move data to or from high memory.

6. Restore the base address in the data segment descriptor cache in the

image at 800, and execute another LOADALL.

7. Enable interrupts.

<<< The next step is not in the article, but conspicuously absent. >>>

8. Restore the original 102 bytes to 800.

Thus while it is appealing not to have to go to protected mode and back to

access high memory from real mode, this isn't much better.  LOADALL requires

195 clock cycles on a machine with no wait states, or 19.5 us at 10Mhz.

LOADALL can also be used to allow programs to be executed in high memory,

even though the processor is in real mode.  A special paragraph ID (such as

FFFF) is used to indicate when a program is running in extended memory.

Interrupt service routines must check the segment register to see if any

contain the special ID, and if so, they must reload the registers using

LOADALL to restore the segment base addresses before returning from the

interrupt.

OTHER APPLICATIONS

LOADALL has other uses as well.  It can be used to switch back to a

protected mode task from real mode, serving as a fast intermode context

switch.  OS/2 presumably uses it to return to protected mode from a

real-mode task running in the compatibility box.

LOADALL can also be used to emulate real mode from protected mode.  By

setting the privilege level of all local and global descriptor table

entries lower than that of the current program, a protection violation

occurs whenever the program attempts to load a segment register.  (Actually,

loading the segment register with a value of 0000 to 0003 does not cause

an exception, and this case must be handled specially.)  The exception

handler then uses LOADALL to set the base address for the segment.

There are many other complexities to this emulation, as described in the

LOADALL document.  The emulation is imperfect, and Microsoft apparently

decided to use the "reset to real mode" technique (as described in our

article last month on the P9), rather than trying to emulate real mode

from within protected mode.

RESTRICTIONS

LOADALL performs no checking on the values loaded into the registers, so

no exception will occur even if an illegal value is loaded.  Thus, the

processor can potential be put into a strange state.  If an illegal

descriptor value is set, no exception occurs from the execution of LOADALL.

An exception will occur, however, when an access  using that descriptor

is attempted.

LOADALL can be executed in protected mode, but only at the most privileged

level (level 0).  Thus it does not violate the protection.  Unfortunately,

LOADALL cannot be used to switch back to real mode from protected mode.

Early versions of 286 (A1 and B1 steppings) have bugs which affect the use

of LOADALL.  Thus, the techniques described in this article may not work

properly on older systems.

LOADALL is not implemented on the 80386, so Microsoft has included code in

OS/2 and in the current version of RAM Drive that tests for the processor

type.  If it's a 286, LOADALL is used, and if it's a 386, the native 386

mechanisms are used.  If you use LOADALL, you should also perform this

check, or your software will not run on 386-based machines.

Using LOADALL is certainly fraught with peril, and requires careful

program design and knowledge of all the implications.  We have spoken with

a number of software vendors that were aware of the instruction, and all

had decided not to use it due to the risks and complexity involved.  We

don't recommend its use to most people, but we do feel that everyone should

have access to the information so they can make their own choice about

whether or not to use the instruction.

--

USPS: The Consultants' Exchange, PO Box 12100, Santa Ana, CA  92712

TELE: (714) 842-6348: BBS (N81); (714) 842-5851: Xenix guest account (E71)

UUCP: conexch Any ACU 2400 17148425851 ogin:-""-ogin:-""-ogin: nuucp

UUCP: ...!uunet!turnkey!conexch!root || ...!trwrb!ucla-an!conexch!root