UNLOAD86.COM is a program that converts or "unloads" a file into it's INTeL

hex representation.  Alternately, it will also output a BNPF, BHLF, OR B10F

type file for those users that require these formats.  This program was

written in response to the need to have INTeL hex files for downloading to

an EPROM programmer.  As MS-DOS programs don't support INTeL hex, the need

to convert binary files to hex arose.  Most development systems, INTeL, CP/M,

& others do support INTeL hex, and most EPROM programmers expect it.

Use of the program is simple:

UNLOAD86  <RETURN>

prints a brief help screen.

The main form of the unload86 invocation is:

UNLOAD86 FILENAME.TYP [/OPTIONS]

B  output a BNPF file instead of INTeL hex.  This option overrides any other

selections.  If the B option is chosen, no other options will work, and the

program will output a straight 8-bit BNPF file, uless it has been modified

for BHLF, or B10F.

L  Selects the new "extended linear address record".  See the discussion

at the end of this document for details.  If you don't know what this is,

you probably don't need it.  Interacts with /E, /O, & /P.  Automaticly

turns on /O, thus /O shouldn't be used with /L as /O will turn off /L.

/L also turns off /E & /P.

O Override extended addressing.  Any output file representative of 64K or

greater normally uses extended address records.  This is any file created

with /PC, /PD, /PE, or /PF, as well as any time the input file is longer

than 64K.  This inhibits the extended address records for the rare cases

where this is needed.  Interacts with /E.  Whichever is last is the effective

switch, normaly only one would be used at a time.  This option also turns

off /L

E forces the use of extended address records for file smaller than 64K.  Files

larger than 64K already automaticly select extended addressing.  Interacts

with /O and /L.  Whichever is last is the effective switch, normaly only one

would be used at a time.

P allows selection of an EPROM size so that all unused locations are filled

with the user specified data (defaults to FF)  This option interacts with

/L.  Whichever is last is the effective switch, normaly only one would be

used at a time.  This option is followed immediately by a parameter number

0-F.  This parameter is:

0  32 bytes.  Creates a file to fill a 32 byte ROM.  All unused locations

are filled with the user defined data.  As with all these options, if the

file is already longer than 32 bytes, an error message will be displayed

on the system console, and the operation will be aborted.  The file left in

this case is a useful INTeL hex file, but isn't padded up to size as it is

already too big for the device specified.  32 byte devices are generally

little bipolar ROMs often used as memory decoders

1  64 bytes.  This is also generally a bipolar device.

2  128 bytes.  Bipolar.

3  256 bytes.  1702A EPROM, some bipolars

4  512 bytes.  2704 EPROM.

5  1k bytes.  2708 EPROM.

6  2k bytes.  2716 EPROM.

7  4k bytes.  2732 EPROM.

8  8K bytes.  2764 EPROM.

9  16K bytes.  27128 EPROM.

A  32K bytes.  27256 EPROM.

B  64K bytes.  27512 EPROM.

C  *128K bytes.  27010 EPROM.

D  *256K bytes.

E  *512K bytes.

F  *1M bytes.

* These options force the generation of extended addressing and thus

do not require the use of the 'E' option.  This may be overridden

with the 'O' or 'L' options.

It was felt un-necessary to provide padding when the /L switch is used as

an extended linear address is normally used only for files larger than 1

megabyte.  If you are reading this in 1999 and you're still stuck with a

computer that can run MS-DOS programs, need to pad a 4 megabyte PROM, you're

getting what you deserve!  Get a real computer!

It should be noted that all INTeL hex output, regardless of options selected,

is padded up to the next 32 byte increment.

ERRORLEVEL support.  UNLOAD86 now supports ERRORLEVEL.  It exits with 0

for no errors, and 255 if there was some kind of error.

EXAMPLES:

UNLOAD86 FOOBAR.COM/B    <-- produces a BNPF file called FOOBAR.BNP

UNLOAD86 FOOBAR.COM/EB   <-- produces a BNPF file called FOOBAR.BNP

UNLOAD86 FOOBAR.COM/E    <-- produces a hex file called FOOBAR.HEX

w/extended addressing

UNLOAD86 FOOBAR.COM/L    <-- products a hex file called FOOBAR.HEX

w/extended linear addressing

UNLOAD86 FOOBAR.COM/LE   <-- produces a hex file called FOOBAR.HEX

w/extended addressing

UNLOAD86 FOOBAR.COM/EL   <-- products a hex file called FOOBAR.HEX

w/extended linear addressing

UNLOAD86 FOOBAR.COM      <-- produces a hex file called FOOBAR.HEX

w/extended addressing if input >64K

UNLOAD86 FOOBAR.COM/P0   <-- produces a hex file called FOOBAR.HEX

padded to 32 bytes.

UNLOAD86 FOOBAR.COM/PF   <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, w/ extended addressing

UNLOAD86 FOOBAR.COM/PFO  <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, WITHOUT extended addressing

UNLOAD86 FOOBAR.COM/PFE  <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, w/ extended addressing

UNLOAD86 FOOBAR.COM/EPF  <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, w/ extended addressing

UNLOAD86 FOOBAR.COM/EPFB <-- produces a BNPF file called FOOBAR.BNP

Notes, patches, etc.:

UNLOAD86 cannot realisticly handle binary input larger than a megabyte as

the extended addressing scheme runs out of addresses.  If you unload a file

larger than 1 megabyte, the extended addressing will wrap around if it has

been selected with the /E or the /Px options.  This limitation does not apply

if you use the /L switch to select the extended linear address records.

There are several user patchable options that can be patched with DEBUG.

They are as follows:

102:  FF   This is the data that is padded when using the /Px option.

Change to meet your requirements.

103:  'PN' Change to 'HL' for BHLF output instead of BNPF, or '10' for B10F.

Or any other two ASCII characters for other variations of this.

106:  'HEX' Output file type for INTeL hex.  Change to meet your needs

10B:  'BNP' Output file type for BNPF.  Change to 'BHL' for BHLF, or

'B10' for B10F, or to whatever filetype you prefer for this

format.

This program was written by Mark D. Pickerill on the Pro-Log ABL-2 CP/M

microcomputer. (what else?)  It bears no resemblance to UNLOAD 2.2 or

2.3 for CP/M by the same author.

UPDATES:

Ver. 1.3  It has come to my attention that Intel has finally patched

their specification to accomodate files larger than 1 Megabyte with the

addition of the "Extended Linear Address Record".  Support for this has

been added.  Also changed the help screen such that if the user patches

the file type for .HEX or .BNP, that the help screen will reflect this

change.  Added Intel hex discussion to the documentation.  04/23/91

Ver. 1.2  After receiving multiple complaints, I changed the command line

parser so that spaces are now allowed between the filename and the switch

character.  Added - and [ as alternate switch characters for UNIX and CP/M

types.  Input files larger than 64K now automaticly generate extended

address records unless overridden with the /O switch.  /O also overrides

forced extended addressing on the larger /Px options.  Added ERRORLEVEL

support. 01/18/91

Ver. 1.1  Corrects a bug that dis-allowed zeros in the output filename.

An input file of ROM0.BIN would be unloaded to ROM.HEX instead of

ROM0.HEX.  Spurious bugs possibly could have also appeared if the input

filename had no extension. 10/3/88

Ver. 1.0  Initial release. 8/10/88

DISCUSSION OF INTEL HEX:

Intel hex is a method of representing binary data as ASCII data.  This

has several advantages.  First, it can be viewed and changed with ordinary

text editors.  Second, the data has a checksum so that transmission errors

can be detected.  Third, it is an excellent way to transfer binary data through

a 7 bit port or with software that only has ASCII transfer capability.  Fourth,

and lastly, additional information such as load addresses and execution start

addresses are incoded in the file.  It has one drawback however.  The Intel

hex file is always at least slightly more than twice the size of the binary

file it represents.

Intel hex is represented in records.  All records have the same format.

They start with a colon, this is followed by a two digit byte count which tells

how many data bytes are in the data field.  This is followed by a four digit

load address which tells where the data loads in memory.  Then comes a two

digit record type specifier (discussed below) which tells what kind of record

this is.  Then comes 1 or more bytes (two or more digits) of actual data.  Last

comes a one byte (two digit) two's complement checksum (modulo-256) of

everything that came before, except of course, the colon.  Anything that comes

after this checksum, such as carriage return - line feed pairs, etc. is

ignored until the next colon.

:020000020000FC

This is a typical Intel hex record.  We will add spaces for clarity:

: 02 0000 02 0000 FC --Checksum

|  |   |  |    |

|  |   |  |    --Data bytes, two in this example

|  |   |  --Record type

|  |   --Load address for data

|  --Byte count of data record

-- Leading colon

RECORD TYPES:

There are currently 6 types of records.  The Intel hex specification has

undergone 3 different revisions over the years.  The original specification

only defined two record types, then the next revision added one more, and

the most recent has added three more to that.  These revisions were made such

that software that processes Intel hex need only write one routine to process

all different types.  Unfortunately people cannot seem to realize this and

use 3 different routines to process data that was intended to be backwards

compatable.  The exact same problem exists with the XMODEM file transfer

protocol.  XMODEM checksum, XMODEM CRC, and 1K XMODEM are all backwards

compatable, yet most file transfer software makes you select between the three.

00 - Data record

This record has a 00 in the record type field.  The byte count is typicaly

10 or 20 (hex), although it can be any number from 01 to FF.  The "wider"

the record (larger the byte count), the less overhead there is in the file,

but the checksum is less reliable.  Some software cannot handle byte counts

larger than 40.

01 - End record

This record has a 01 in the record type field.  This is the only record

type which can have (and always does have) a byte count of zero.  The most

common end record is :00000001FF.  This is a byte count of zero, a load

address of zero, a record type of 1, and the checksum of FF.  What most

people don't know is that the start execution address of the file can be

stored in the address field, with appropiate alterations to the checksum.

However, a lot of software will barf if it encounters a end record other than

the one shown here.

The above two types are the original Intel hex specification

02 - Extended address record

This record has a 02 in the record type field.  Its byte count is always 02.

It has a load address of zero, and the extended address is in the two data

bytes.  Why the address isn't stored in the address field for this, and all

succeeding record types is unknown.  This record type was added by Intel when

they introduced the 8086.  What Intel didn't make clear at the time was the

fact that this two byte extended address is actually a *segment* address.

It represents address bits 4 through 19.  So this address is the tired old

segment addition:

0000  -- "Extended address"

0000 -- "Load address"

00000 -- "Physical address"

Some software, especially software written to do Intel hex for other processors

such as the 68000 family, use the extended address record to represent address

bits 16-31.  This is incorrect!  When Intel (finaly) introducted a processor

capable of doing flat addressing, they added record type 4 for this purpose.

When UNLOAD86 uses this type of record, it starts at zero and increments it

in blocks of 64K which adds 1000 hex to this segment address.

The above three types are the "Rev 2" Intel hex specification

03 - Extended start address

This record has a 03 in the record type field.  Its byte count is always 02.

It is otherwise identical to record type 02, and is used to indicate the

segment start execution address of the file.  As UNLOAD86 has no way of knowing

this information, this record type is not supported.

04 - Extended linear address record

This record has a 04 in the record type field.  Its byte count is always 02.

The data field contains the extended linear load address, address bits 16-31.

This is the much more sensable extended address record.  When UNLOAD86 uses

this type of record, it starts at zero and increments it in blocks of 64K

which merely adds one to this address.  A lot of software cannot handle any

of these new record types.  It is recomended that this type of addressing be

used if your application and your software permit it, rather than record

type 02.  Although it is perfectly legal to use both types, UNLOAD86 only

uses one at a time as it is converting a contiguous binary file to Intel hex,

thus it doesn't make a lot of sense to use both.  Both types might be used

in the case of a compiler or assembler output where the data is non-contiguous.

05 - Extended linear start address

This record has a 05 in the record type field.  Its byte count is always 02.

It is otherwise identical to record type 04, and is used to indicate the

extended linear start execution address of the file.  As UNLOAD86 has no

way of knowing this information, this record type is not supported.

###############################################