[ i                   --   Exterior Ballistics   --            QBALL ]

=====================

D E D I C A T I O N

---------------------

QBALL was inspired by the many computer programs

published in The American Rifleman, and is

respectfully dedicated to their author:

William C. Davis, Jr.

====================

DISCLAIMER

----------

The application described in this documentation, this documentation

itself,  and all  programs and/or files written  or modified by the

application are supplied "as is."   Responsibility  for determining

its suitability for any purpose rests solely with the user. The au-

thor will  NOT be held responsible  for injury or damages resulting

from the use of the application;  all remedies rest solely with the

user.   The  application is  distributed  without  any  warranties,

express or implied, with these exceptions ONLY: copies of the  pro-

gram files distributed on author-supplied disks will be replaced if

the disk(s) prove defective; and, if your copy of the program isn't

operational because of an author error,  and you obtained your copy

from the author or an authorised distributor, a complete set of the

program files is available upon request.

+=================================================================+

|                         LIMITED LICENSE                         |

|                         ===============                         |

|      Individual users may make copies of the program files for  |

| backup and personal use.  Individual users are also encouraged  |

| to distribute copies to others, provided that all files are so  |

| distributed in their entirety and without modification.         |

|      Computer clubs may make copies for distribution, with the  |

| same provisions,  and also provided that no fee in excess of a  |

| reasonable copying/media fee is charged.  (Reasonable is taken  |

| to mean $10.00).  Commercial distribution by library companies  |

| or individuals wishing to act as distributors is permitted.  I  |

| would appreciate being asked, first!                            |

+=================================================================+

+=================================================================+

| QBALL is distributed as user-supported software, ("Shareware"), |

| and should be registered. See Appendix G for registration form. |

+=================================================================+

COPYRIGHT INFORMATION

---------------------

IBM, PC-DOS are trademarks of International Business Machines.

MS-DOS, QuickBASIC, Professional Development System

are copyright, Microsoft, Inc.

4DOS is copyright, JP Software.

---------------------

[ iii                 --   Exterior Ballistics   --            QBALL ]

INTRODUCTION

------------

QBALL is a menu-driven application for the rapid computation of

ballistic values for small arms projectiles.  QBALL will quickly com-

pute point-of-impact data, velocity and energy levels, trajectories

relative to line-of-sight, and times of flight.  Additional routines

permit the calculation of ballistic coefficients, point-blank range,

wind deflection corrections, sectional densities, impact variations

for non-horizontal fire, and free recoil energy.  Printing routines

provide for the production of hard copy where appropriate.  Starting

values to which the program will default may be modified using the

built-in configuration routines.

It is assumed that QBALL users will have some knowledge of exter-

ior ballistics.  QBALL is not intended as a means of learning the sub-

ject; several of the references in the Bibliography contain basic in-

troductions to ballistics.  Study of the references will amply repay

the effort involved.

QBALL uses the "G1" drag function for computation of trajectory

and other values.  Appendix D of this documentation compares values

from selected published tables with values calculated using QBALL.

Warning!  QBALL users are cautioned that data computed using the

routines contained herein should be used as guides to the determination

of ballistic performance!  Should the user be engaged in reloading

small arms ammunition, it is incumbent upon the user to verify the data

obtained using QBALL with a standard published reloading manual.  Sev-

eral such manuals are listed in Appendix C.

The application title, "QBALL" does not in any way refer to the

game of billiards; it's short for "Quick BALListics."  Seemed appro-

priate, somehow: QBALL was originally written in QuickBASIC.

If you find QBALL of use, but would like it to do something that

it doesn't do now, or have suggestions to improve what it does already,

you are invited to send in a copy of the Enhancement Form.  While no

promises are made that your suggestion will be incorporated, the first

person suggesting an improvement which is subsequently implemented will

receive a free copy of the first release incorporating that improvement.

The latest version of QBALL is available from the author for a

$25.00 registration fee.  Registration entitles the user to the latest

version with one free upgrade, and a 3-hole punched printed manual.  The

Professional Development System 7.1 source code is available to register-

ed users for $50.00; or in a registration/source code package for $60.00.

The most current version is also available on the EXEC-PC Bulletin

Board (Modem: 414 789-4210, 8N1, 1200/2400 baud), and on AMERICA ON LINE,

in the "Home and Hobbies" Conference, keyword "ballistics."  Downloads

from other sources may not include all the files -- caveat emptor.

[ 2                   --   Exterior Ballistics   --            QBALL ]

1.3  Copying the program files.

-------------------------

It is a good idea to make a copy of the original QBALL files, and

keep it in a safe place.  Run QBALL from copies of the original!  Make

sure that you copy both QBALL.FON and QBALL.EXE to the same directory,

or onto the same disk.  QBALL.EXE will look for its associated .FON

file in the current directory.  (If the *.FON file is not found, an

"Illegal Function Call" message is shown, and nothing else happens.)

QBALL no longer uses a separate *.INI file; the initialization data

is read from and written to the tail end of QBALL.EXE itself.

Put the documentation files wherever you can get at them.

1.4  Printing the documentation.

--------------------------

The QBALL???.DOC files are formatted to print with a wide-enough

left margin to allow for punching to fit a 3-ring binder.  There is no

need to set your printer to print the IBM graphics set as in earlier

editions; this version of the on-disk documentation uses only the low-

order characters.  You need not set your printer to skip perforations

in fanfold paper; the document files contain the appropriate form-feed

control characters.

You will be printing 55 pages, but using just 30 sheets; plan the

paper and time requirements accordingly!  First, copy "QBALLODD.DOC"

to your using the command

COPY QBALLODD.DOC PRN

When that's done, flip the paper over, and copy "QBALL.EVN" on the re-

verse side of the same sheets, using the command

COPY QBALLEVN.DOC PRN

Now go away and do something else.  Each pass takes about 25 min-

utes to print at 10 cpi, near-letter quality.  (If you're in a tearing

hurry, print in draft mode...)

[ 4                   --   Exterior Ballistics   --            QBALL ]

1.6  Starting QBALL

--------------

Log into the directory or onto the disk where you've put

QBALL.EXE and QBALL.FON, and type:

QBALL  (+ [Enter]).

The next thing you'll see is this screen, here somewhat modified

to permit printing it without using graphics characters:

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

+=====================================================================+

|                    +=======+  +            +  +                     |

|                    |       |  |            |  |                     |

|                    |       |  |     +===+  |  |                     |

|                    |       |  +===+ +===+  |  |                     |

|                    |    =+ |  |   | |   |  |  |                     |

|                    +=====|=+  +===+ +===+  +  +                     |

|                          +=         Reg #0208SW                     |

|           [ Exterior Ballistics for PC's & Compatibles ]            |

|           [ Using G1 Drag Functions   --   version 2.08]            |

|           [ Copyright (c) 1992  -  Benjamin W. Hartley ]            |

+=====================================================================+

Welcome to computer ballistics with QBALL!

QBALL provides small arms trajectory data quickly and accurately.

Users are encouraged to verify QBALL data with a standard reloading

handbook!  The author will NOT be responsible for the misuse or misapp-

lication of QBALL-generated data.

===> press any key <===

No Saved                                                  XXX

Graphics

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

If you have received an "Illegal Function Call" message, make sure

that QBALL.EXE and QBALL.FON are both in the current directory, and try

again.

QBALL reports tests for the presence of a graphics adapter/monitor,

and reports what it has found: none at all, CGA, or EGA.  (VGA "falls

through to "EGA."  "No Saved" is diplayed if: your computer has graphics

display capability, and you have not saved any graphics data.

Two pieces of information which you should note from the initial

screen: the Version, and the Registration Number.  When you register your

copy of QBALL, please include these numbers on your order.

Press any key to continue.

[ 6                   --   Exterior Ballistics   --            QBALL ]

The most frequent use for QBALL will be the production of ballis-

tics tables for a specific cartridge/load.  With the menu shown on the

screen, pressing "C" will open the menu for computation of ballistic

or gun data.  More information about using this option is contained in

Chapter 2.

If you wish to infer a ballistic coefficient, then press "B".

More information is to be found in Chapter 3.

"Use <U>tilities...", "U" provides a means to set display col-

ors, the status of the printer and noise switches, and the values to

which QBALL will default upon start-up.  You may also "shell to DOS"

from the utilities menu.  See Chapter 5 for more information about

the utilities menu, generally, and configuring QBALL.

"<N>ew or Changed..." goes to a secondary menu which allows entry

or modification of input data.  We'll try this one a bit later.

"Quit" -- press "Q" to return to DOS.  QBALL will ask you to

confirm that you indeed wish to quit.  Press "Y" to return to DOS or

press "N" to continue using QBALL.  (If you have saved data to disk

from within QBALL, you will also be prompted:

Retain Saved Data ? ([Y] or N).

The default is "Y" -- that is, keep the saved data.  It is STRONGLY

recommended that, in this case, you press "N," at least until you've

played with QBALL a bit.

Back to work...

For right now, press "C" to compute ballistic data and to do the

"tutorial" which follows.  Go on to Chapter 2, page 7.

[ 8                   --   Exterior Ballistics   --            QBALL ]

2.2  Computing and displaying a range table.

--------------------------------------

Assuming everything is going as it's supposed to go, the

screen will clear, and you'll be presented with this screen:

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

> Ballistic Computations: Typical Firearm <

+=================================+

|    *** Computations Menu ***    |

|  ------------------------------ |

|  <S>et/Reset Current Zero Range |

|  Range <T>able (0 MUST BE SET)  |

|  <O>ther Ballistics Procedures  |

|  <N>ew or Changed Input Data    |

|  ------------------------------ |

|  <ESC> to  M A I N   M E N U    |

+=================================+

---------------------------------------------------------------+

[ MV: 2300 ][ C1: .250 ][ Wt: 125 ][ Met: 59o 0' 29.53" 78%   ]|

|

(Note: Info line shown)            +---------------------+

(      "folded".      )            +---------------

[ H: 1.00 ][Yds]

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

AHA! All the spaces on the Info Line now have values in them,

some of which you just entered, and some of which are the QBALL de-

fault (or starting) values.

"<O>ther Ballistics Procedures" are described in Chapter 5.

As you can see, you can change the Input Data by using option "N"

on this menu; for now, continue with what you've got.

"ESC" will "back up" one menu; try it and see.  Then remember to

press "C" to get back here.

The other two options are described in the following section.

(From here on, the Screen Header line, which )

(tells you what cartridge/bullet you're deal-)

(ing with,  and the Info Line will be omitted)

(        from the printed examples.          )

[ 10                  --   Exterior Ballistics   --            QBALL ]

This table is interpreted as follows: if, for example, a bullet

having the given ballistic characteristics is fired at the given

muzzle velocity, then, at a range of, say, 200 yards, it will retain

1693 FPS velocity, 795 ft-lbs of energy, will have dropped 16.2 inches

below the line of departure, will have risen to a maximum of 1.8"

above the line of sight, will have been blown 7.7" sideways by a 10

MPH crosswind, and will have taken 0.305 seconds to travel 200 yards

downrange from the muzzle.  Additionally, the height of the bullet

above or below the sight line in both minutes of arc and inches is

given at each listed range.  Note that the tables displays "0.0" at

150 yards, the zero range.  In similar fashion, data may be read from

the table for each of the listed ranges.

Should you try this procedure with a larger range, or a smaller

range increment, QBALL will display as many lines as will fit on the

screen, and display a prompt like this:

----> More? ([Y] or N)  <C>ontinuous <----.

Press M, Y, or [enter] to display the next part of the table.  You can

press <C> to print the table without stopping, or N or <ESC> to go di-

rectly to the next menu.  When you have displayed the whole table, this

prompt is displayed:

======> Press any key <=====.

(The hardcopy print routines recompute the tabular data, so there

is no need to be concerned with information which has scrolled off the

screen.  Put a marker in this page, and return to it to experiment

later.)

Press any key to display this "Choose Next Output" menu:

+==================================+

|    *** Choose Next Output ***    |

|  ------------------------------  |

|  <R>edisplay Range Table         |

|  <P>rint Range Table             |

|  <S>ave Range Table to File      |

|  Display Trajectory <G>raphic    |

|  ------------------------------  |

|  <ESC>ape to Computations Menu   |

+==================================+

NOTE: if your computer is NOT equipped to display graphics, the

<D>isplay Trajectory Graphic option will not appear.  If this is the

case, please skip ahead to page 14.

Assuming that QBALL found a graphics card and monitor, press <G>

to display a graphic representation of the trajectory data.

[ 12                  --   Exterior Ballistics   --            QBALL ]

Now let's play "what if...?"  What would happen to the trajectory

if, say, the muzzle velocity is increased to 2500 FPS?  Let's find

out.  With the computations menu displayed, press "N" to access the

"New or Changed Input Data" menu, which looks like this:

+=================================+

|   *** New Input Data Menu ***   |

|  -----------------------------  |

|  <C>artridge/Bullet Identifier  |

|  Muzzle <V>elocity              |

|  <B>allistic Coefficient        |

|  Bullet <W>eight                |

|  <M>etro (dFnt 'Alt "Hg  %RH)   |

|  Sight <H>eight                 |

|  Yds/Mets <S>wap (Now Yards)    |

|  -----------------------------  |

|  <ESC> to Computations Menu     |

+=================================+

In this instance, press "V" to change the muzzle velocity, and

enter "2500" when prompted.  You'll be returned to this menu; it is

possible, but not recommended, to change more than one item of input

data at a time.  Note that the info line now reflects the new muzzle

velocity.  Press "ESC" to return to the computations menu.

Note that the range at which zeroed, 150 yards, is still set.  Step

through the process of developing a range table by entering a maximum

range (200), and the range increment (25).  Press "G" at the "Choose

Next Output" Menu.  In addition to the graph, this should be displayed:

+=========================+

> Current <    > Previous <      |     Graphics Menu       |

Max Ht:   1.5" @ 86                       +-------------------------+

Min Ht:  -3.5" @ 200                      | <G>rid Toggle           |

MuzVel:     2500 FPS                      | <S>ave Current Data     |

BCoeff:     .250                          | <R>ecall Saved Data     |

ZeroRg:   150 Yards                       +-------------------------+

Metro :   Standard                        | <ESC>ape to Choose Next |

+=========================+

Now press <R> and <0> (that's "zero") to retrieve the trajectory

data you saved previously.  Here's what you should get:

+=========================+

> Current <    > Previous 0 <    |     Graphics Menu       |

Max Ht:   1.5" @ 86        1.8" @ 86      +-------------------------+

Min Ht:  -3.5" @ 200      -4.5" @ 200     | <G>rid Toggle           |

MuzVel:     2500 FPS         2300         | <S>ave Current Data     |

BCoeff:     .250             .250         | <R>ecall Saved Data     |

ZeroRg:   150 Yards        150 Yards      +-------------------------+

Metro :   Standard         Standard       | <ESC>ape to Choose Next |

+=========================+

[ 14                  --   Exterior Ballistics   --            QBALL ]

If you're skipping ahead from page 10, try pressing "P" to obtain

hardcopy of your range table -- assuming, of course, that you have a

printer on and on-line.  Also try pressing "S" to save the range table

to a disk file.  I STRONGLY SUGGEST THAT YOU NOT ATTEMPT RANGE TABLE

TO A FLOPPY DISK -- at least until you've played with the program for

a while.   The disk file will be identical to the hardcopy.  Also try

"R" to redisplay the range table.  Let's see how to modify input data.

2.4  Changing Input Data.

-------------------

From "Choose Next Output," press "ESC" to get back to the Computa-

tions Menu, then "N" to access the "New or Changed Input" menu:

+=================================+

|   *** New Input Data Menu ***   |

|  -----------------------------  |

|  <C>artridge/Bullet Identifier  |

|  Muzzle <V>elocity              |

|  <B>allistic Coefficient        |

|  Bullet <W>eight                |

|  <M>etro (dFnt 'Alt "Hg  %RH)   |

|  Sight <H>eight                 |

|  Yds/Mets <S>wap (Now Yards)    |

|  -----------------------------  |

|  <ESC> to Computations Menu     |

+=================================+

Let's see what each option does.

Cartridge or Bullet Identifier.  This option allows you to change

the designation of the cartridge, bullet, or firearm with which you

are working.  Press "C" and enter a string of characters.  You may

edit your entry using the [backspace] key.

Muzzle Velocity.  This, as has been seen, changes the muzzle

velocity to be used.  QBALL accepts velocities from 300 FPS to 4500

FPS, inclusive.

Ballistic Coefficient.  This allows changing the ballistic coeff-

icient to be used, perhaps reflecting a different bullet or cartridge.

QBALL automatically adds a leading decimal point to your entry.

Bullet weight.  Allows changing the bullet weight.  Acceptable

values are any number greater than zero and less than 1000 grains.

Metro.  This one's got four parts; you may confirm or change each

of temperature, gun altitude, barometric pressure, and relative humid-

ity.  Standard metro values are shown on the prompt line in [brackets].

Acceptable metro values are as follows:

a.  Temperature         : -50 to +125 Fahrenheit.

b.  Gun Altitude        : -1312 to 29028 feet.

c.  Barometric Pressure : 25 to 35 inches of mercury.

d.  Relative Humidity   : 0 to 100 percent.

[ 16                  --   Exterior Ballistics   --            QBALL ]

Point-Blank Range.

-----------------

The "point-blank range" for any weapon / ammunition combination

may be defined as that range for which the trajectory deviates not

more than a specified amount from the line of sight, either above it

or below it.

Press "P" to access the point-blank range determination routine.

When prompted, enter maximum deviation = 3 (for 3 inches above and

below the line of sight).  The screen will look something like this:

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Determination of Point Blank Range

----------------------------------

Maximum Deviation                      3

----------------------------------

Trial Zero Range = 512 Yards

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

After a pause, during which the trial zero range will change several

times, the following will be displayed:

----------------------------------

Point Blank Zero Range = 179

Maximum Range = 207

----------------------------------

===> press any key <===

When you "press any key" you are returned to the computations menu,

where you will observe that the zero range has been set to 179.  You

may now go ahead and develop a range table for this weapon/ammunition

combination, using that computed zero range.

It is recommended that you use a range increment which will give

you trajectory heights at even multiples of 50 yards.  If you do so,

you'll know how high to sight in your weapon at, say, 100 yards so as

to be zeroed at 179, thus assuring a maximum deviation of 3 inches

from the line of sight out to 207 yards.  (The graphics routine, if

invoked, will draw a horizontal line on the screen at scale 3 inches

above and below the line of sight.  These lines will continue to be

displayed until you change the zero range.)

And what does all this do for you?  Well, with this "point-blank"

zero, you can hold "dead-on" out to slightly more than 200 yards, and

be assured that the bullet will strike no more than 3 inches high or

low at any point along its path.  Beats estimated holdover, don't you

think?

[ 18                  --   Exterior Ballistics   --            QBALL ]

Slant Angle of Fire.

-------------------

Virtually all firing tables and ballistics computations programs,

QBALL among them, assume that the angle of fire in use is relatively

close to horizontal.  It is known, however, that firing "upslope" or

"downslope" will effect the strike of the bullet: it will hit high

relative to point of aim in both cases.  How high?  Press "A," and

enter a maximum range of 400, a range increment of 100, and a vertical

angle of 25 degrees.  The following table will be displayed:

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Hits High for Slant Fire, 25d

Above or Below Horizontal

-----------------------------

Range      True      Hits

Yards      Drop      High

-----     -----     -----

0        0.0       0.0

100        3.6       0.3

200       16.2       1.5

300       41.0       3.8

400       82.9       7.8

-----------------------------

===> press any key <===

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

The table tells you that this weapon / ammunition combination,

fired at a vertical angle of 25 degrees above or below the horizontal,

will hit high by the specified distance, in inches, for each listed

range.  The effect is relatively small for small vertical angles, but

can become great enough to be of real concern, especially at relative-

ly long ranges and at large vertical angles.  Try several examples and

see.  (A clear exposition of the geometry of this problem may be found

in the LYMAN Manual, listed in the bibliography.)

Sectional Density.

-----------------

The sectional densities of two or more particular bullets may be

used to compare their relative penetrating power.  Sectional density

is defined as the weight of the bullet in pounds divided by the square

of its diameter in inches.  Press "S," and enter the bullet diameter

in either inches or millimeters.  QBALL assumes that any value you

enter which is greater than "1" is in millimeters, and makes the con-

version to inches.  The dimension may be taken as the bore diameter of

the weapon you are considering.

This routine reports the sectional density of the bullet under

consideration.  This value is included in the Range Table printed

header, but there is no specific routine to print just this value.

[ 20                  --   Exterior Ballistics   --            QBALL ]

Chapter 3

---------

Inferring ballistic coefficients

""""""""""""""""""""""""""""""""

From time to time, it may be desired to determine the ballistic

coefficient of a particular bullet, and the available tables do not

include that data.

Select "Infer <B>allistic Coefficient" at the main menu.  (You

will be encouraged to read this section of the manual, if you haven't

done so already.)  Press "Y" to continue with the routine.

QBALL will ask for several pieces of information: basically, two

ranges, and two velocities.  This diagram will help you visualize the

setup.

( + = target locations  )

|---[r1]----|              ( >>>>>>> = bullet path )

|-----------[r2]----------|

Gun >>>>>>>>+>>>>>>>>>>>>>+>>>>>>>>>>>>>>>>

v@1           v@2

r1  = range, muzzle to first chronograph

r2  = range, muzzle to second chronograph

v@1 = velocity at first range

v@2 = velocity at second range

It is possible for r1 to equal zero; in that case, of course, v@1

would equal the muzzle velocity.  (This is the only time that QBALL

will allow you to enter a range of zero; "zero" is the default value

for r1.)

The routine for inferring a ballistic coefficient allows use of

nonstandard metro, should you have developed your own velocity data

from live firing.  You may change the metro conditions to be used

before beginning to enter velocity data.  The ballistic coefficient

reported, however, is adjusted to standard metro conditions in all

cases.

You may enter up to twenty-five (25) individual range/velocity

pairs.  The average v@1, v@2, and ballistic coefficient are reported.

If r1 is greater than zero, i.e., you are not working with a muzzle

velocity, QBALL reports an calculated muzzle velocity.

Let's work through the process.  Refer to the table on page 9,

and either copy the range and velocity figures, or run the range table

routines and produce hard copy.  We'll do this twice: once, with r1

set to 0, and once more with r1 set to 25; r2 will be set at 100 in

both cases.

[ 22                  --   Exterior Ballistics   --            QBALL ]

3.2 Second Run: r1 = 25

-------------------

Enter "first range" = 25 and "second range" = 100.  When prompted

for the velocities, enter 2218 and 1982.  You should end up with the

following display:

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

For Round Number 1

First Velocity        (Ft/Sec)      2218

Second Velocity       (Ft/Sec)      1982

For Standard Atmosphere, C1 = .250

Calculate another Round? ([Y] or N)

[ Lower & Upper Ranges (yards)  25 & 100 ]

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Again, press "N;" you don't want to calculate another round.  The

center of the screen will clear, and, after a short time for computa-

tions, the following information is displayed:

-----------------------------------------

Average C1 for 1 round = .250

Between Mean Velocities 2218 and 1982

Calculated Muzzle Velocity = 2300

-----------------------------------------

Press "N" to skip printing the table, and return to the main

menu.  Again note that the information line shows the calculated

muzzle velocity and the inferred ballistic coefficient.  You may now

go on to calculate ballistic data using the values obtained using this

routine; QBALL will continue to use this data until changed.

This may have seemed like an exercise in futility: we did, after

all, know what the ballistic coefficient was in the first place.

Suppose, however, you have a ballistics table from, say, Winchester-

Western, which includes only velocity data?  Inferring a ballistic

coefficient based on the published velocity figures will allow you to

make your own "custom" firing table for a particular factory round.

(Most ammunition manufacturers are noticeably reticent when it comes

to some ballistics data.)  Additionally, should you be reloading your

own ammunition, and have access to two chronographs, you could make a

custom firing table for your own custom ammunition.

[ 24                  --   Exterior Ballistics   --            QBALL ]

4.5 Typical Printed Output.

----------------------

QBALL will print a "standard header" and a table title at the top

of each table each time you choose "<P>rint..." in a menu.  (This is a

change from previous releases of QBALL.)  The standard header looks

like this:

======================================================

>>> Ballistic Computations for the Typical Firearm <<<

------------------------------------------------------

Muzzle Velocity = 2300 FPS    Ballistic Coefficient = .250

Bullet Weight = 125 Gr        Sectional Density = n/a

Temperature = 59 deg F        Altitude = 0 Feet

Barometric Pressure = 29.53"  Relative Humidity = 78%

------------------------------------------------------

For printed data tables, QBALL recomputes each line of data which

appears or has appeared on the screen, so information which has

scrolled off the screen may be printed.  (This might occur, for exam-

ple, if you've used a large range, and a small range increment.)  In

most data table cases, the format of the printed output is virtually

identical to the screen display.

And here's the printed output for the "Infer Ballistic Coeffi-

cient" routine, using some of the same data we used in chapter 3:

============================================================

>>> Inferring Ballistic Coefficient for: Typical Firearm <<<

------------------------------------------------------------

Given the following meteorological conditions:

Temperature..........: 59 deg F Barometric Pressure....: 29.53"Hg

Relative Humidity....: 78 %     Gun Altitude...........: 0 Feet

Relative Air Density.: 1.000    Relative Sonic Velocity: 1.000

and the number of rounds shown, the ballistic

coefficient (C1) was computed using the veloci-

ties indicated.

----------------------------------------------

Round           V1            V2           C1

-----          ----          ----         ----

1            2300          1982         .250

----------------------------------------------

Average C1 for 1 round = .250

Between Mean Velocities 2300 and 1982

Calculated Muzzle Velocity = 2300

==============================================

[ 26                  --   Exterior Ballistics   --            QBALL ]

Press "ESC" to return to the Utilities menu.

5.3  Configuring display colors.

--------------------------

Press "C".  A special color configuration screen will appear.

Pressing the appropriate keys will "step" through the available colors

for background ("B"), main body text ("foreground," "F"), Highlighted

text ("highlight," "H"), and for the Title/Info line ("low intensity,"

"I"), and the help background ("L").  When you've found a combination

that you like, press "ESC" to return to the configuration menu.  Note

that the currently chosen and the default color values are displayed at

the bottom of the color configuration screen.  If you get in a muddle,

and want to start over again, press "D" to return to the default colors.

5.4  Shell to DOS.

------------

You may find it desirable (or necessary) to "drop to DOS" for

housekeeping purposes.  Press "D" to invoke a DOS shell; type "EXIT"

(+ [enter]) to return to QBALL.  (Upon EXITing a DOS shell, you'll be

returned to the Utilities menu.)

5.5  Using the values you've set.

---------------------------

Having set the display colors and toggles as you like them, and

<ESC>-ing to the main utilities menu, you may:

<W>rite QBALL.INI Data

If you're sure of the values you've entered, press "W" to save the

new initialization data.  (And remember, if you change your mind, there's

nothing to stop you from running Configuration again.)

If you'd like to use the values for this session only, perhaps to

see how they work out, press "ESC."  If you decide later to save your

"temporary" setup, choose "U" from the main menu, then choose "W" immed-

iately from the utilities menu.  QBALL doesn't care if there's a delay

between choosing configuration values and writing .INI data; it will

quite cheerfully write .INI data using whatever values are in effect

at the time the "W" option is chosen.

5.5  Some other considerations.

-------------------------

The initialization data includes a good bit more information than

just the display colors and toggle status.  It also controls the

starting values for the following:

a.  Cartridge/bullet Identifier

b.  Meteorological conditions

c.  Sight height.

[ 28                  --   Exterior Ballistics   --            QBALL ]

Some Afterthoughts and Some ~~~~Wrinkles~~~~

""""""""""""""""""""""""""""""""""""""""""""

Afterthoughts.

-------------

I'm not entirely happy with the file-handling routines, but

at least they work.  Without further bloating the source code, how-

ever, I don't see how to make them any more bullet-proof.  I should

very much like to include a way to reread the QBALLSAV.INI file; this

would be of value to those who use the DOS Shell routine to delete

saved trajectory data files.  At present, if you don't quit the cur-

rent session and start all over again, QBALL refuses to recognize that

you haven't run out of available QBALLSAV.0nn numbers.  (I'd call 'em

"handles" but for the fact that that's a bit too "techie" for me!)

~~~~Wrinkles~~~~.

----------------

QBALL will run from a floppy disk, albeit S-L-O-O-W-L-Y.

The major bottleneck, of course, is in disk read/write operations.

Two ways around the speed problem are, in order of improvement:

1.  Run QBALL from a directory -- preferably its own -- on your

hard disk.  You'll find the improvement in speed remarkable, and there

will be no need (generally) to be concerned with running out of room

for QBALLSAV files.

2.  If you can afford the room, run QBALL from a RAM disk.  At

least 350K is needed to hold a complete set of QBALL files: QBALL.EXE,

QBALL.FON, and 16 QBALLSAV.nnn files with their associated *.INI file.

The increase in speed, again, is in file save/retrieve operations, but

there's a side benefit (or hazard, if you prefer): you won't be trou-

bled with old data files.  So, here's a thought: use a batch file to

copy QBALL onto your RAM disk, and to save the data back to your source

disk or to a floppy.  It might go something like this (assuming that

QBALL is on a disk in drive "A:", and your RAM disk is drive "F:"):

COPY QBALL.EXE F:

COPY QBALL.CFG F:

F:

QBALL

DEL QBALLSAV.INI

COPY QBALLSAV.* A:QBALLOLD.***

A:

Call this batch file "DOQBALL.BAT" or some such.  This will work fine

in the absence of a hard drive for those who have but one or two 360K

floppy drives -- the lowest common denominator.  (Though come to think

of it, how many folks have an EGA and only floppy drives?)

[ 28.2                --   Exterior Ballistics   --            QBALL ]

[ 30                  --   Exterior Ballistics   --            QBALL ]

CALIBER (or CALIBRE) - Technically, the diameter of the barrel mea-

sured across the lands.  In practice, the diameter of the bullet.  A

measure of length equal to bore diameter; thus, a "fifty caliber"

barrel would be 50 bore diameters long.

CARTRIDGE - Fixed ammunition, including case, powder, primer, and

bullet, for small arms.  See ROUND.

CHAMBER - The breech, or rearmost portion of the barrel, which con-

tains and supports the cartridge prior to, during, and immediately

following firing.

COEFFICIENT OF FORM - A number relating the ballistic efficiency of a

given bullet shape to the shape of the projectile used to calculate

the ballistic table.

C1 - Formula symbol for ballistic coefficient.

CROSS WIND ANGLE - The angle between the wind and line of departure.

Typically tabulated as ninety degrees, or "pure" crosswind with no

range vector.  QBALL takes left-to-right wind as positive.

DRIFT - Technically, the distance a bullet will travel horizontally

due to its spin. This effect is usually quite small, amounting to

about 7 inches at 1000 yards for the military M2 Ball (.30-'06) car-

tridge.  Often confused with wind deflection.  See WIND DEFLECTION.

DEFLECTION - See wind deflection.

DOWNRANGE - Refers to the position of objects in space relative to the

direction of fire; point of aim and point of impact, for example, are

by definition downrange.

DROP - The distance a bullet will fall due to the influence of grav-

ity.  Measured from the line of departure, not the line of sight.

ELEVATION - Vertical sight adjustment.  Also, the (usually small)

angle between the axis of a firearm's bore and the horizontal.

ENERGY - Here, the  kinetic energy of a moving bullet, usually given

in ft-lbs.  Equal to one half the mass of the bullet in pounds multi-

plied by the square of the velocity in feet per second.

F.P.S. - Feet per second. A unit of velocity.  Also abbreviated as FPS

and Ft/Sec.

FT.-LBS. (Also FtLbs, Ft#) - Foot pounds.  A unit of energy equal to

the effort required to lift one pound, one foot.

FORM FACTOR - A multiplier which relates a bullet's shape to that of

the standard projectile used to prepare a particular ballistic table.

[ 32                  --   Exterior Ballistics   --            QBALL ]

MAXIMUM HEIGHT - The highest vertical distance above the line of sight

reached by a bullet between the muzzle and the point of aim.  Some-

times confused with midrange trajectory since it typically occurs

about halfway between the muzzle and the target.  Maximum height

is usually somewhat higher and occurs somewhat further downrange than

the midrange trajectory.  Also called MAXIMUM ORDINATE.

METALLIC SIGHTS - Sights on a firearm which contain no optical ele-

ments, i.e., glass or glasslike plastic lenses.

MET - An abbreviation for "meter"; also, a super-abbreviation of

"Meteorological Conditions."  See METER, METRO, STANDARD MET

METER  (also METRE) - A unit of length measure equal to 39.37 inches,

3.28083 feet, or 1.09361 yards.

METRIC - Having reference to length measurements based upon the meter,

its subdivisions, or its multiples.

METRO - Short for Meteorological Conditions; the air temperature, bar-

ometric pressure, gun altitude, and relative humidity existing at the

place and time of firing.  See STANDARD MET.

MIDRANGE TRAJECTORY - The height of the trajectory at a point halfway

between the firearm's muzzle and the point of aim.

M.O.A. - Minute of Arc.  Equal to 1/60 of an angular degree.  At 100

yards, 1 M.O.A. is approximately 1.047 inches.

MUZZLE - The foremost point of a firearm's barrel; that part of a

firearm closest to the target.

MUZZLE ENERGY - Foot-pounds of energy developed by a moving projectile

at the firearm's muzzle.

MUZZLE VELOCITY - The speed of a projectile at the firearm's muzzle,

generally expressed in FPS.

N.R.A - National Rifle Association. If you shoot, you SHOULD belong to

this organization.

OGIVE - The curve of a bullet's forward portion.  Also, the radius of

this curve, usually expressed in calibers.

POINT-BLANK RANGE - That range for which the bullet's path varies from

the line of sight by no more than a specified amount for the entire

trajectory from the muzzle to that range.

POINT OF AIM - That downrange point with which a firearm's sights are

aligned.

POINT OF IMPACT - That point which a bullet actually strikes.  In most

cases, considered in relation to point of aim.

[ 34                  --   Exterior Ballistics   --            QBALL ]

TARGETED RANGE - That distance furthest from the muzzle where the path

of the bullet crosses the line of sight. (The "sighting in" distance.)

TIME OF FLIGHT - The time necessary for a bullet to travel from the

muzzle of a firearm to a given range.

TRAJECTORY - The path of a moving projectile.  Often expressed as the

number of inches above or below the line of sight.

WINDAGE - Horizontal, lateral sight adjustment.  Also, the sight

adjustment required to compensate for crosswind effects at a given

range.

WIND DEFLECTION - The amount of a projectile's horizontal lateral

motion attributable to the action of the wind.  Often mistakenly

called drift.  See DRIFT, WINDAGE

YAW - Normally, a situation in which a bullet rotates on its axis at a

small angle to the line of flight.

ZERO - The sight setting at which point of impact and point of aim

coincide at a given range.   Also, the process of determining that

sight setting.  See also SIGHTING IN.

[ 36                  --   Exterior Ballistics   --            QBALL ]

Suppose you have determined a ballistic coefficient from pub-

lished data, say, the Ballistics Section of "Gun Digest."  Having the

ballistic coefficient, use that figure to calculate a table showing

the same data as the published table.  If, for example, the resulting

velocities are too high, then reduce the ballistic coefficient that

you're using to, say, 90% of the value QBALL has reported.  Recompute

the table.  If the resulting velocities are now low relative to the

published data, increase the second ballistic coefficient to 95% of

the value originally used.  In similar fashion, continue to refine

the value of ballistic coefficient until the QBALL results agree as

closely as possible with the published data.

In general, however, the process of successively refining a ball-

istic coefficient will prove unnecessary.  The very small variations

introduced by differences between successive bullets in a string,

variations in muzzle velocity, and, possibly, rapidly changing metro

render the process of successive refinement superfluous.

[ 38                  --   Exterior Ballistics   --            QBALL ]

H O U S E K E E P I N G

- - - - - - - - - - - -

It is STRONGLY recommended that you respond "N" to the "Retain

Saved Data Files" prompt, which is displayed after you confirm quit-

ing QBALL.  This is the cleanest technique, and doesn't require any

further action on your part.  If, however, you're a glutton for pun-

ishment, there are two ways of going about this, both from the DOS

prompt.

First, you could enter:

DEL QBALLSAV.*

This is the simplest way, and avoids confusing QBALL in a later ses-

sion.  This, however, is exactly what the "N" response does.

Second, and if you want to keep the trajectory data for later

examination, enter this (or use a batch file):

DEL QBALLSAV.INI

and

REN QBALLSAV.* QBALLOLD.*

This preserves the data and gets rid of QBALLSAV.INI, but the current

directory or the current disk may end up jammed with old data.

One further hint: EACH QBALLSAV.00n file uses 5600 bytes of disk

space, and the .INI file uses 5 bytes.  That's a LOT of disk sectors;

make sure you've got sufficient space in the current directory/on the

current disk for better than 90K worth of files!  QBALL will detect a

"disk full" error, but why take the chance?

[ 40                  --   Exterior Ballistics   --            QBALL ]

Appendix E

----------

Error analysis

""""""""""""""

How accurate are the results obtained with QBALL?

Results from several QBALL runs were compared with comparable

data from two sets of published tables: the Speer Reloading Manual #11

and the Hornady Handbook of Cartridge Reloading, 3d edition. Two ques-

tions were to be answered: first, how well do velocity figures compare

between the three sources?; second, are the trajectories found by the

different sources comparable?

Some difficulty was experienced in the choice of ballistic coeff-

icients and velocities due to the differences between the two pub-

lished sources.  Speer, for example, rounds all ballistic coefficients

to two decimal places, while Hornady carries three.  In addition, when

comparable values of ballistic coefficients could be found, the Horn-

ady tables quite often did not have the spread of velocities available

in the Speer manual.

The following table summarizes as-comparable-as-possible data

from all three sources.  While severely limited, the data shows that

the results obtained from QBALL are in substantial agreement with the

two published sources.

+----------------------------------------------------------------+

|            Remaining Velocity, Conditions as Shown             |

|            =======================================             |

|              [ Range = 200 yards, Standard Met ]               |

+----------------------------------------------------------------+

|           MuzVel = 2100     MuzVel = 2500     MuzVel = 3300    |

|              C1 = .12          C1 = .26          C1 = .50      |

|          ---------------   ---------------   ---------------   |

|                                                                |

|   QBALL        1112              1879               2899       |

|   SPEER        1102              1872               2895       |

|   HORNADY      1126              1875               2892       |

|                                                                |

+----------------------------------------------------------------+

(Key: MuzVel = Muzzle Velocity  C1 = Ballistic Coefficient)

Table D1.  Remaining Velocities With Given Inputs.  Note

should be made  that the  Hornady ballistic coefficients

were actually .122, .259, and .496 for the three sets of

data shown. This accounts for the apparent "high-on-low"

spread of values seen here.  QBALL was run with the bal-

listic coefficients as shown.

[ 42                  --   Exterior Ballistics   --            QBALL ]

A close examination of the data in Table D2 shows that there is

very close agreement between the QBALL results and the published data.

The greatest variation is at the greatest ranges; this is to be ex-

pected.  Since the ballistic coefficients used in obtaining data from

the Hornady tables are not exactly those chosen as typical, being

"high-on-low" respectively, it is also expected that the resulting

bullet strikes will be too small, "on", and too large.  There is, how-

ever, substantial agreement between the QBALL results and the data

from both published tables.  The actual bullet strike differences be-

tween QBALL's results and the published tabular data is two or three

tenths of an inch.  This indicates that QBALL is capable of excellent

practical accuracy.

(One and one half inches  at 400 yards)

(is equal to three-eighths minutes-of-)

(arc -- that's 22 1/2 SECONDS of arc!!)

(If your rifle  will group as tight as)

(this -- let alone tighter -- you have)

(an exceptional rifle, indeed!        )

[ 44                  --   Exterior Ballistics   --            QBALL ]

Error Messages (continued)

--------------

+--------------------------------------+

| Enter  0 <= Relative Humidity <= 100 |

+--------------------------------------+

Attempting to enter a relative humidity value which is out

of range.

+--------------------------------------+

| Enter  25 < Barometric Pressure < 35 |

+--------------------------------------+

Attempting to enter a barometric pressure value which is out

of range.

+-------------------------------------+

| Enter  -1312 < Gun Altitude < 29028 |

+-------------------------------------+

Attempting to enter a gun altitude value which is out of range.

+---------------------------------+

| Enter  0 < Bullet Weight < 1000 |

+---------------------------------+

You're attempting to enter a zero bullet weight, (Huh?), or a

weight better suited to artillery.  (1000 gr = 2.3 OUNCES)

+-------------------------------------+

| Enter  -2.0 <= Sight Height <= +5.0 |

+-------------------------------------+

You're attempting to enter a value which is out of range.

+------------------------------------------+

| Enter  Second Velocity  < First Velocity |

+------------------------------------------+

In inferring ballistic coefficients, attempting to enter a

downrange velocity which is larger than the velocity closer

to the muzzle.  (Rocket-assisted bullet, maybe?)

+-----------------------------------------------------------------+

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

|                                                                 |

+-----------------------------------------------------------------+

[ 46                  --   Exterior Ballistics   --            QBALL ]

Error Messages (continued)

--------------

+----------------------------+

| Enter  Range <= 1760 Yards |

+----------------------------+

+-----------------------------+

| Enter  Range <= 1609 Meters |

+-----------------------------+

In both cases, attempting to enter a range outside QBALL's

ability to use effectively.  (1760 yds = 1609 met = 1 MILE.

Contact the author if you really need greater ranges.)

+---------------------------------+

| Enter  Wind Velocity <= 231 MPH |

+---------------------------------+

In determining wind defelction corrections, attempting to

enter a wind velocity in excess of the world record wind

velocity (other than in tornadoes).

+-------------------------------+

| Enter  Wind Direction <= 360� |

+-------------------------------+

In determining wind deflection corrections, attempting to

enter a direction greater than a complete circle.

+--------------------------+

| Printer does not respond |

+--------------------------+

Your printer is off, or off-line.  This message does NOT

appear if you're simply out of paper.

+----------------+

| File not Found |

+----------------+

You're trying to retrieve a file which QBALL thinks exists,

but really doesn't.  The QBALLSAV.INI file is probably corrupt

or absent.  Quit QBALL, erasing the QBALLSAV.* files as you do

so and restart QBALL.

+-----------+

| Disk Full |

+-----------+

There is insufficient disk space on the default drive's disk

to write the desired QBALLSAV.nnn file.  Either: delete some

files, or copy QBALL to a fresh disk and start over.  (If this

error occurs, QBALL does a "fix-up" of QBALLSAV.INI, and dis-

ables "<S>ave Current Data.")

[ 48                  --   Exterior Ballistics   --            QBALL ]

---------

N O T E S

---------

[ 50                  --   Exterior Ballistics   --            QBALL ]

The following comments wouldn't fit on the other side!

------------------------------------------------------

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________