CHD20 is a unique TSR, permitting you to work with multiple fonts

using MDA/HERCULES video board with ANY text mode program, not only

specialized graphic mode word processors. CHD20 is especially useful

to those who work with non-latin alphabets.

Attention ! This program is designed to provide multifont text mode

=========   for MDA/HERCULES video adapter -- it will not work with

other adapters unless you switch them to HERCULES mode.

Monochrome video  display adapter(MDA)  is the  most inexpensive

video  adapter  providing  normal  resolution  for  full-scale  work.

However  it  does  not  support  multiple  fonts  in text mode -- the

feature  that  becomes  crucial  for  non-latin alphabets:  cyrillic,

hebrew,  greek,  arabic.  MDA  is  usually  accompanied  by  HERCULES

graphics.  CHD20  makes  use  of  this  fact.  It  user-transparently

emulates  mulitfont  text  mode  --  the  feature  provided  by  more

expensive EGA and VGA video adapters. This shareware version of CHD20

supports cyrillic font. The same algorithm enables using

different fonts,  switching  them  and  even  emulating  EGA  and VGA

character generator (CG) BIOS functions.

The  usual  method  of  changing   fonts  for  use  with  MDA   is

reprogramming  the  EPROM  containing   CG  as  a set of 256  16-byte

character  images  where  each  byte  corresponds  to a row in a 8x16

character matrix.   There are two  problems in reprogramming  CG. The

first problem is  purely technical. It  stems from vendor's  specific

structure of CG. Usually it  comes out as shuffling data  and address

lines on  the EPROM,  so there  is a  need in  a special  software to

recognize character images. The  second problem has hardware  origin.

The CG can be  physically located on the PROM, soldered to the  video

card or  even be  part of  the proprietary  VLSI chip. CHD20 software

solution to fonts  changing can be  the only possible  way of solving

this problem. CG's addressing is hardwired in the adapters logic,  so

the basic idea of    CHD20    is    emulation    of   text mode using

graphic HERCULES mode.  If systems  and   application programs  used

BIOS for text  screen  output  operations ,  it  would  be pretty

easy to intercept  10h  interrupt   and   change  the   video   mode

and the character to be sent to the  screen. But most of the programs

do not use  BIOS  for  speed  reasons,  so   this  approach has only

limited applicability.  The  basic  idea  of  my  approach  was to use

ticker interrupts (int 8h) to redraw the graphic screen  which depicts

text screen substituting needed  character images. The major problem I

had to deal  with, was  brushing up  the algorithms  of emulation  so

that there be no noticeable decrease in video performance.

The program is very easy to use. When Scroll Lock is on, F2 takes

you to multifont mode and F1 back to normal text mode. Caps Lock

toggles keyboard according to font layouts.

Current version of CHD20 supports only cyrillic font. It also

contains keyboard driver for the cyrillic typewriter layout. It does

not support DOS5 and Windows 3.x task switching, which means that you

have to start it in every DOS session. It is not recommended to use

CHD20 if you work in graphic mode -- you can easily turn it

off (Scroll Lock On + F1) when there is a need to do so.

Future versions of CHD20 will coexist with task switching

and also support fonts other than cirillic(hebrew, arabic,

greek.). You will be able to load any font, edit it and

switch between the loaded fonts. These features will be available to

registered users. The registration also includes free technical

support form the author. To register please send you check or money

order for $29.00 to:

David Kogosov

315 Avenue F, 1st Floor

Brooklyn NY 11218

If for any reasons you are still interested in the hardware

solution (reprogramming CG) I can provide you with my expertise or

consultation, having hundreds of different boards reprogrammed.

If you have any comments or questions please feel free to send me mail

at the above address or by the Compuserve mail at 70252,3356.