CLOCK

CLOCK consists of three programs.

CLOCK.SYS is a clock device driver (CLOCK$ device).  It replaces the DOS

internal clock driver.

CLK.EXE is a program that allows you to separately control your DOS

realtime clock and battery-protected calendar clock.  Most importantly,

CLK.EXE will handle time zones and will automatically switch between

standard and daylight savings time for you.

CLKDEMO.EXE is a small part of CLK.EXE that is provided in source code

(C language) form so that you can see how to add clock control to your

own applications.

CLOCK.SYS

Is a device driver for those whose clocks are not supported by the level

of DOS they are using or want to use.  CLOCK supports a wide variety of

different clock hardware and BIOS interfaces.  As far as possible, CLOCK will

automatically determine what clock interface to use.  If the automatic

determination does not work for your system, you may tell CLOCK specifically

what interface type to use.  CLOCK is also required by anyone who wants to

use CLK or CLKDEMO as it provides functions not provided by the normal DOS

CLOCK$ device driver.  CLOCK is fully compatible with the DOS device driver

on all modern PCs and many older ones.

If your PC clock is not currently supported but you would like to remove the

restrictions on what versions of DOS you can use, please contact me by mail or

on CompuServe.  I will be happy to add support for your clock.  I will need

some technical information and help from you in getting it to work so I will

give you a $0 usage license for your help.  Likewise, anyone finding a new bug

and helping me get the information to correct it will get a $0 usage license.

USING CLOCK

To install the device driver include the following line in CONFIG.SYS:

DEVICE=[d:][path\]CLOCK.SYS [type][,io_addr]

or

DEVICE=[d:][path\]CLOCK.SYS 3[,segment[,write0,write1,read]]

or

DEVICE=[d:][path\]CLOCK.SYS A[,base_year]

where "d:" and "path\" identify the drive and directory containing CLOCK.SYS.

"type" allows you to control which clock interface will be used.  It may be

important if the automatic determination does not work on your system.

See the information below for some help in determining the type number for

your system.

If "type" is an I/O clock (types 4, 6, 7, and 9), you may add "io_addr" to

specify the I/O address used by your clock.  Normally this is 240, 2C0, or 340

for types 4, 6, and 7 and 210 or 310 for type 9.  All numbers are in hex.  For

example, an AST Research clock might be specified as follows:

DEVICE=[d:][path\]CLOCK.SYS 4,2C0

If you do not specify the "io_addr", CLOCK.SYS will try to determine the

correct address by probing.

If you are sure that you have an I/O clock but you are not sure which one it

is, specify type 8 and supply the io_addr if possible.  Type 8 does not

represent a specific clock.  Rather it attempts to determine whether your

clock is a type 4, 6, 7, 9, or B using a sequence of operations also used by

some of the I/O clock vendors.

If you have a memory mapped clock, you may use the second format to specify

the addresses to be used.

"segment" is the base address used to access the clock.  Usually your

documentation will give the segment address as four hex digits such as FE00 or

F000.  Some addresses may be less than four digits (e.g., 70).  Other

documentation may show addresses in the form xxxx:yyyy.  In this case the

segment address is the xxxx value.

"write0" is the offset used to write 0 bits.  It must also be a hex address and

may be 0, F002, or anything else the vendor chose.  If the addresses are

given in the xxxx:yyyy form, use the yyyy.

"write1" is the offset used to write 1 bits.

"read" is the offset used to read bits.

If your documentation gives the segment address but isn't clear about the

offsets, just specify the segment address and CLOCK.SYS will try all offset

combinations that it knows about.

If you specify one of the offsets, you must specify them all.

"base_year" is used for the AT&T 6300 clocks (type=A).  Dates prior to that

year or more than 7 years after that year will not be handled correctly.  If

no base year is specified 1992 will be used.  Dates between the start of the

base year and the end of the seventh year later are handled (e.g., 1992-1-1

until 1999-12-31).

All DOS DATE and TIME commands and all DOS date and time function calls

will use the calendar clock as they did in earlier DOS versions released by

the vendor.

CLOCK.SYS supports all valid DOS dates from 1-1-1980 to 12-31-2079 and

handles leap years automatically for most clock types.

CLOCK.SYS in conjunction with CLOCK.EXE provide support for multiple time

zones and automatic conversion between daylight savings time and standard

time at the appropriate times of the year.  For this reason, CLOCK may be

of great use to you even if your version of DOS already supports your clock

correctly.

Normally, CLOCK.SYS keeps the calendar clock (battery-protected clock) and

the DOS real time clock in close synchronization.  At boot time it sets the

DOS clock from the calendar clock.  Any time you change the date or time it

normally writes to both clocks.  With CLK.EXE you may separate the handling

of the two clocks.

CLOCK TYPE

There are almost as many different implementations of PC clocks as there are

different types of PCs.  Until the PC/AT came along, there was no standard for

battery protected clocks and each PC and add-in was likely to implement them

differently.  With the PC/AT a standard and documented interface for such

clocks was provided.  Since that time, most PCs have followed the PC/AT

standard.  It is earlier PCs, especially 8088 PCs, that don't follow the

standard.  Since these are early PCs, there is also a high probability that

the vendor is no longer providing modified versions of DOS with built-in

support for their clocks.  This leaves you faced with the choice between

staying with DOS 3.2 or earlier or giving up your clock.

CLOCK.SYS is intended to free you from those limitations.  It will work with

all versions of DOS from 2.10 through 5.00 and undoubtedly beyond.  It also

attempts to automatically determine what kind of clock you have and install

the correct support for it.  However, there are often enough differences

between PCs that the automatic determination will not find anything or will

even make the wrong choice.  To give you more control of the situation, you

may specify the clock interface by giving an interface number.

It will not always be easy to decide what interface number to use.  You will

at least need some technical information on how your clock works or on the

BIOS calls used to control the battery-protected clock.  If you find a

description below that seems to match, give it a try.  The most that should go

wrong is that your time and date will be set incorrectly, but then they

probably weren't working anyhow.

If you aren't sure what interface to use or if you are sure that your clock is

not supported, LET ME KNOW.  I WOULD LIKE TO HELP.  You can help me make

CLOCK.SYS better for everyone.  I would even like to here from you if CLOCK.SYS

did work for you.  I would like to be able to add your PC manufacturer and

model number to the supported list so others know what to use.  If CLOCK.SYS

did not work for you, I am interested in making the changes so that it does

work.  If you can provide me with the necessary technical information, I will

make the changes.

Usually, a technical reference manual with hardware and BIOS interfaces is the

best source.  Even if you don't have one, your original hardware vendor may be

willing to send you enough information to allow you to program the clock.  If

all else fails, we may even be able to use an old copy of DOS or a clock

setting utility to figure out how it works.  Before we do that, check your

license to be sure that it is not forbidden to disassemble the software to see

how the clock works.

The following table shows the systems that have been determined to work with

existing clock types.  If your system is in the list, try the clock type

indicated.  If not, look through the appendix which tells you about the

characteristics of each type of clock and see if anything seems to match

your system type.

SYSTEM                  CLOCK TYPE

------                  ----------

Amstrad PC1521DD        0

AST Research            8 (4, 6, 7) See discussion under clock types

Six-Pack

I/O Plus, etc.

AT&T 6300               A

Hyundai 8088            B

IBM PC/AT               0

Leading Edge Model D    5

Mitsubishi 8088         5

Quadram Corp            9

QuadCard

QuadCard II

QuadCard 512+

SMT No-Slot Clock       3

Sperry PC-1             5

Sperry PC/HT            5

Tandy 1200HD            4

Vendex Turbo-888-XT     4

Zenith Supersport 20    1

Zenith Z-18x            1

_______________________________________________________________________________

CLK.EXE

CLK allows you to separately control your DOS realtime clock and your

calendar clock.  The most important reason to do this is to allow you to

shift your local time among multiple time zones including automatic

adjustment for daylight savings time.  Almost as important is your ability

to set just the DOS time without affecting your calendar clock.  This way you

can test end-of-month processing and then very simply restore your DOS time

to the correct time.  In fact, you can set any time and date that you want to

use to force an application to take an action that you want without the

normal trouble in reseting the clock to the correct time later.

The usage of CLK.EXE is:

CLK   [C] [D] [S=C] [S=D] [TZ=...] [/?] [T?] [TZ=:?]

Each of the arguments is itself a command.  You can supply any combination of

these commands on a single invocation of CLK and they will be processed in

the order you gave them.  If there is any error, none of the commands are

processed so you always know where you are.

If you execute CLK with no parameters or options, it displays the times from

both clocks and tells you whether they are Connected or Disconnected.  The

clocks are Connected when time and date changes affect both clocks.  Even if

the clocks are Connected, the times displayed may be slightly different.  The

two clocks rarely increment at exactly the same speeds (either one may be the

more accurate depending on your system).  Normally, the calendar clock is read

only at boot time and is used to set the DOS clock.  All further reads by DOS

use the DOS clock only.

All uses of CLK (except errors and /?) end by displaying the same information.

You always know the state of your two clocks after execution.

CLK   C

Connects the two clocks.  Neither clock is changed by doing this command.  It

only affects future DOS time and date functions.

CLK   D

Disconnects the two clocks.  Neither clock is changed.  Any future change of

the date and time will affect the DOS clock only.

CLK   S=C

Synchronizes both clocks to the current setting of the calendar clock.  You

can use this even if the clocks are not connected to make the DOS clock equal

to the calendar clock.

CLK   S=D

Synchronizes both clocks to the current setting of the DOS clock if they are

connected.  If they are not connected, this command displays a warning message

and has no effect on the clocks.  It would normally be used immediately after

a CLK C to set the calendar clock to the value in the DOS clock.

CLK   TZ=...

Sets the DOS clock to the correct local time based on the value of the

expression following the =.  This expression conforms exactly to the

POSIX 1003.1 standard for time zone handling.  CLK assumes that the calendar

clock currently contains UTC (also known as GMT, Greenwhich, ZULU, and Z

time).  UTC (Universal Time, Coordinated) is the name of the international

standard time that is provided by WWV, WWVH, the Naval Observatory, and NIST

as well as many other services around the world.  Most Unix (Unix is a

registered trademark of Unix Systems Laboratories.) systems also support TZ

although some only partially support the POSIX standard.

Normal usage is to set your date and time to UTC with the clocks connected.

Then set the DOS clock to local time using CLK TZ=...  You can put the

CLK TZ=... statement in your AUTOEXEC.BAT file.  You should put it prior to

any other statements that may create or update files or you run the risk of

having confusing times in your file directory (MAKE could get especially

confused.).  If you provide a full TZ specification, your system will

automatically adjust to the correct time zone every time you boot.  Note that

no attempt is made to change between daylight and standard time except when

you execute CLK TZ=...  Adjusting on the fly is very dangerous.  Many

programs can produce incorrect results or even corrupt your data if time

takes a big step forward and are even more likely to do so if time runs

backward.

CLK   /?

"/?" or any error will cause CLK to display the usage format.  If you have an

error in any field, CLK first displays that field.

CLK   T?

"T?" or any error in a TZ command will cause CLK to display the format of the

TZ command.

CLK    TZ=:?

"TZ=:?" or any error in a TZ command that refers to a pre-defined time zone

causes CLK to display all the current pre-defined time zone names and their

associated values.

The format of TZ=... can get quite complicated as the standard is trying to

deal with all time zones and daylight savings time rules around the world.

The full format is:

TZ=:zone

or

TZ=STD[offset][DAY[offset][,start[/time],end[/time]]]

The form TZ=:zone is the one you are most likely to need.  CLK knows the

offsets and daylight savings time rules for most of the U.S. and all you have

to supply is the name of your time zone.  The time zones currently known to

CLK are:

EST             (Eastern U.S. with automatic daylight adjustment)

CST             (Central U.S. with automatic daylight adjustment)

MST             (Mountain U.S. with automatic daylight adjustment)

PST             (Pacific U.S. with automatic daylight adjustment)

HST             (Hawaiian with no daylight savings time)

You can use CLK TZ=:? to display the list of

pre-defined zone names and their values.

If you would like your favorite time added, please

send me the abbreviations, offset from UTC, and any

daylight savings time rules that are different from

the U.S.

NOTE:  The abbreviations can be composed of any

characters except digits, plus, minus, and comma.

They can be up to 32 characters long.

You don't have to wait for me to modify CLK in order to use your local rules

or just try something out.  That's what the long form is for.

"STD" is any time zone name for standard time that you want to use.  See the

note above for the rules.

"offset" is the difference between your local time and UTC and is expressed

as hh[:mm[:ss]] with an optional leading + or -.  Times West of Greenwich

are positive and those East are negative.  A 24-hour clock is used with times

running from 00:00:00 to 23:59:59.  Mountain Standard time would be expressed

as:

TZ=MST+07:00:00    or

TZ=MST7

and that's what you would use if you live in Arizona and don't want any

daylight savings time adjustments.

"DAY" is the time zone name for daylight savings time.  If you don't use

daylight savings time in your location, you should not supply this or any of

the following fields.

The "offset" for daylight time is in the same form as for standard time.  If

you don't give this offset, it is assumed to be one hour later than standard

time.  Thus a more complete specification of the Eastern time zone would be:

TZ=EST+05:00:00EDT+04:00:00    or

TZ=EST5EDT4   or

TZ=EST5EDT   (since daylight is one hour later by default)

"start" is the day of the year on which daylight savings time starts.  There

are three formats that you can use to give this day.  They are given below.

"end" is the day of the year on which standard time starts again.  Note that

in the Southern hemisphere it is quite possible for "end" to be earlier in

the year than "start" as daylight savings time will include December and

January.  "end" is in the same format as "start".  Note from the syntax above

that you must provide "end" if you provide "start".

"time" is the time of day that the change takes place.  It is in exactly the

same format as "offset" except that leading + and - signs are not allowed.

If you do not specify the "time", 02:00:00 is used as the default.

The day of the year form for "start" and "end" may be any of:

Jn      where n is a Julian day of the year from 1 to 365.  This form

does not allow you to refer to February 29.  Day 59 is always

February 28 and Day 60 is always March 1.  This is most useful

in locations where a particular calendar date is used every

year.

n       where n is a Julian day of the year from 0 to 365.  This form

does allow you to refer to February 29.  January 1 is day 0,

February 28 is day 58, and either February 29 or March 1 is

day 59 depending on whether it is a leap year or not.  Not

the most useful of forms but it does deal with locations that

might specify February 29 as the date that time changes.

Mm.w.d  is used in places like the U.S. that do not specify a date,

but rather specify a rule.  "m" is the month (1 to 12).

"d" is the day of the week (0=Sunday, 1=Monday, ...

6=Saturday).  "w" is the week of the month (1 to 5).  Week

1 indicates the first time that day "d" occurs in that month.

Week 5 indicates the last time that day "d" occurs in that

month whether it is the 4th or 5th time.  Thus the U.S. rule

is M4.1.0/02:00:00,M10.5.0/02:00:00 or the first Sunday in

April at 2AM until the last Sunday in October at 2AM.

The long form for Central U.S. time is:

TZ=CST+06:00:00CDT+05:00:00,M4.1.0/02:00:00,M10.5.0/02:00:00

or

TZ=CST6CDT5,M4.1.0/2,M10.5.0/2 (just leaving out unnecessary zeros)

or

TZ=CST6CDT,M4.1.0,M10.5.0    (since 2AM is the default)

or

TZ=CST6CDT                   (since the U.S. rule is the default)

USING CLK

The most common use of CLK will be to handle your time zone and daylight

savings time changes.  You need only do the following:

1.  Set your clocks (use CLK C if necessary to make sure the calendar clock

gets set) to UTC.

2.  Include the following line in your AUTOEXEC.BAT file.

CLK D TZ=:zone  (where you use one of the pre-defined zones)  or

CLK D TZ=...    (where you define your own rules)

That's it.  Everything else will take care of itself.  The only other time

you will need to do anything is if you decide to correct your calendar clock

to a more accurate time.  You will probably use TIMESET or one of the other

programs that call the Naval Observatory or NIST.  Before you do, use:

CLK C S=C

Then run the program requesting that it set your time to UTC (GMT).  Then do

CLK D TZ=...

again.

If you don't want to be bothered with UTC, CLK will quite happily work with

your calendar clock set to its current value.  For example, lets assume that

you currently have both clocks set to Pacific Standard Time and you don't

really want to be bothered changing to UTC, but you would like future changes

between standard and daylight time to be handled automatically.  Just use the

following statement in AUTOEXEC.BAT:

CLK D TZ=PST0PDT

CLK believes whatever you tell it and will quite happily operate assuming

that PST is equal to UTC.  You could even use CLK with your calendar clock

set to Pacific Daylight Time by using:

CLK D TZ=PST1PDT

_______________________________________________________________________________

CLKDEMO.EXE

A simple version of the CLK program, called CLKDEMO, is provided in both

executable and source code form.  CLKDEMO only provides support for

Connecting, Disconnecting, and displaying both sets of times.  It is included

to show you how to add clock handling to your own applications.  CLOCK.SYS

has a couple of additional APIs (Application Programming Interfaces) that are

not present in the normal DOS CLOCK$ device driver.  These interfaces are

provided by the DOS IOCTL (Input/Output of ConTroL data) functions.  The

Output Control Data to Character Device function is used to Connect and

Disconnect.  The Input Control Data from Character Device function is used to

get the current connection status and the date and time from the calendar

clock.  CLKDEMO is written in C.

The CLKDEMO command line has the format:

CLKDEMO [C] [D]

If CLKDEMO is used without any commands it displays the connect status and

the current times.

CLKDEMO C

Connects the two clocks.

CLKDEMO D

Disconnects the clocks.

Since CLKDEMO.EXE is merely a subset of CLK.EXE, it isn't terribly useful as

a utility in its own right.  Its value is that the source code is provided.

There are three source code files provided.  CLKDEMO.C is the main program

and performs command interpretation and execution.  IOCTL.C is a set of

functions that read and write the CLOCK$ device driver using the DOS IOCTL

functions.  CLOCK.H is a set of definitions, only some of which are used by

CLKDEMO.

There are two additional APIs that CLOCK.SYS provides.  In order to use them

you must open a handle to the CLOCK$ device driver.  You do this using the

DOS INT 21H Open File with Handle (3DH) function specifying read-write

access.  You then use the handle to invoke the other two functions.

To change the "connected" status, you use the Send Control Data to Character

Device (4403H) function.  You send one byte containing a 1 to connect the

clocks or a 0 to disconnect the clocks.

To get the current status and time, you use the Receive Control Data from

Character Device (4402H) function.  You may read 1 byte or 9 bytes.  In

either case the first byte read is the "connected" status.  If you read 9

bytes, the following bytes are (in order):

YEAR            two bytes in integer (word) format (1980-2099)

DAY             one byte (1-31)

MONTH           one byte (1-12)

MINUTE          one byte (0-59)

HOUR            one byte (0-23)

SECONDS/100     one byte (0-99) 100ths of seconds

SECOND          one byte (0-59)

Yes, the order of information is funny.  That's the DOS standard for the

CLOCK$ device driver and is an effect of the Intel architecture byte order.

If you write in assembly language, you will notice that you can do whole word

moves between that form and the registers for the DOS time and date calls

since the bytes will be reversed in moving.

_______________________________________________________________________________

APPENDIX:               CLOCK TYPES

TYPE                INTERFACE DESCRIPTION AND PCs THAT USE THIS TYPE

0              IBM PC/AT and all fully compatible PCs.  Interface 0 is

included for completeness.  Most versions of DOS fully support

this clock interface without a separate driver.  Use

CLOCK.SYS to provide support for CLK.EXE and CLKDEMO.EXE.

Amstrad PC1521DD systems use this type of clock.

Automatic determination checks location F000:FFFE for the hex

value FC.

BIOS calls are used to read and write the clock values which

are usually stored in the CMOS RAM along with other

configuration information.  The primary interfaces are:

Get Date:       MOV   AH,4

INT   1AH

Returns the year in CX and month and day in DX

in BCD format (e.g., a hex display would show:

CX=1991, DX=1009 for October 9, 1991).

Set Date:       MOV   AH,5

Load CX and DX as above for Get Date

INT   1AH

Get Time:       MOV   AH,2

INT   1AH

Returns hours and minutes in CX and seconds in

DH in BCD format.  May also return 100ths of

seconds in DL (e.g., CX=2359, DX=5900 for 1

second before midnight).

Set Time:       MOV   AH,3

Load CX and DX as for Get Time

INT   1AH

_______________________________________________________________________________

1              Zenith Z-18x 8088 portables, Zenith Supersport 20.

Automatic determination checks location F000:FFFA for the two-

byte hex value 4D32 ("M2").

BIOS calls are used to read and write the clock

Get Date:       MOV   AH,2

INT   1AH

Returns CX=year (1980 - 2079) in binary and

DH=month, DL=day.  All values are binary.

A hex display of the above date would look

like CX=7C7, DX=A09.

Set Date:       MOV   AH,3

Load CX and DX as for Get Date.

INT   1AH

Get Time:       MOV   AH,4

INT   1AH

Returns CH=hours, CL=minutes, DH=seconds.

A hex display would show: CX=173B, DX=3B00.

Set Time:       MOV   AH,5

Load CX and DX as for Get Time.

INT   1AH

______________________________________________________________________________

2              Various Zenith Data System PCs.

Automatic determination looks at location F000:800A for the

pattern 00,F0,"ZDS".

Get Date:       MOV   AH,2AH

INT   1AH

Returns CX and DX as for type 1.

Set Date:       MOV   AH,2BH

Load CX and DX as for type 1.

INT   1AH

Get Time:       MOV   AH,2CH

INT   1AH

Returns CX and DX as for type 1.

Set Time:       MOV   AH,2DH

Load CX and DX as for type 1.

INT   1AH

______________________________________________________________________________

3              Memory-mapped clock used on some Zenith Data Systems PCs and

other similar clocks including many slotless or no-slot

clocks.  The SMT (Systems Manufacturing Technology) No-Slot

clocks are supported.

Automatic determination is to attempt to read the clock.  If

values other than all 0FFH or 0 are obtained, this clock

type is chosen.  All known segments and offsets are tried.

Several different memory mapping approaches are used and

many different address possibilities are supported.  In

general a segment address in the ROM region (C800H through

FFFFH) is used.  Offsets from that segment address are used

to read bits, write zero bits, and write one bits.

Documentation may specify those segments and addresses in

several forms.  For example, segment F000 with offset F002

is the same as segment FF00 with offset 2.

Each memory reference reads or writes a single bit.  Read

operations are used for all references with separate offsets

for reading a bit, writing a zero bit, and writing a one bit.

Clock segments and addresses currently probed include:

SEGMENT   WRITE0   WRITE1   READ  DECR  LAST      VENDOR

F000      F002     F003     F004  0000  F000      (Zenith)

FE00      2        3        5     0080  C800      SMT

FE00      0        2        8     0080  C800      SMT

DECR indicates that the segment has that value subtracted from

it for another attempt at probing until the value falls below

LAST.  Note that like the segment address, DECR and LAST are

both in units of paragraphs (16-bytes).  Thus DECR of 80 is

equivalent to 800H or 2048 bytes.

When the addresses used by a memory-mapped clock are determined

by probing, the segment and read address is displayed.  As the

read address varies the most it is a good indication of the

type of clock found.

Data is stored in BCD in bytes that you construct by shifting

in the bits.  The most important data bytes are:

0 = Hundredths of seconds, 1 = Seconds, 2=Minutes, 3=hours,

5=days, 6=months, 7=years mod 1980.

_______________________________________________________________________________

4              I/O bus clock used on some PCs.  Many add-in clocks on

expansion boards are also accessed similarly.  This clock was

used on many AST Research I/O boards for 8088 PCs such as the

Six-Pack Plus, IO Plus, etc.  Also used on some Tandy systems

(e.g., 1200HD) and the Vendex Turbo-888-XT.

Automatic determination is to attempt to read, complement,

write, re-read, and re-complement one of the registers.  If the

first value read and the re-complemented second value are

equal, the register exists and is presumed to be the clock.

See also the description for types 6 and 7.  If you aren't sure

which variant you have, specify type 8 and automatic probing

will determine if it is really a type 4, 6, or 7.  For the

greatest chance of detecting the correct clock type, supply the

base I/O address if at all possible.

NOTE:  If the clock is not correctly initialized (e.g., you

have just replaced the battery), CLOCK.SYS must initialize it

so that the counters will start working.  CLOCK.SYS determines

that the clock has been previously initialized by looking at

register 14H (see 2D4H below).  If register 14H contains the

value 0DEH, the clock is presumed to be initialized.  If not,

CLOCK.SYS will clear all the registers to zero and start the

counters running.  While this form of checking for initial-

ization was commonly used, your previous software may have used

a different method.  If so, the first time you run CLOCK.SYS

your date and time may be set to zero (1980-1-1 00:00:00).  If

that happens, just enter the date and time.  It should work OK

after that without resetting to zero on further boots.

Each clock register is implemented as a separate I/O register

address.  Clock base I/O addresses of 240H, 2C0H, 300H, and

340H are supported by automatic probing.  Similar clocks at

other base I/O addresses are supported by specifying type 4 and

the base address. The address range used is the base plus 0

through 1FH (e.g., 2C0H through 2DFH).

Important registers are (using 2C0 as the example):

2C1  =  100ths of seconds

2C2  =  seconds

2C3  =  minutes

2C4  =  hours

2C6  =  days

2C7  =  months

2C9  =  month last time clock was read/set

2CA  =  years

2CB  =  initialized flag (0DEH if initialized)

2D4  =  Status

All values are in BCD.  That means that each register consists

of two 4-bit nybbles.  Each nybble contains one decimal digit

of the value.  Years are mod 1980 (i.e., 0 means 1980, 99 means

2079).

Since these clocks do not keep the year and do not handle leap

years for you, CLOCK.SYS does the necessary calculations to

keep the year correctly maintained and to adjust the date for

leap days.  The clocks do provide a couple of memory locations

in which CLOCK.SYS maintains the year and previous month.  As

long as you boot your system before a full year has passed,

CLOCK.SYS will correctly maintain the date.

_______________________________________________________________________________

5              Mitsubishi 8088 PCs.  These were sold by Sperry as the PC-1

and PC/HT, by Leading Edge as the Model D, by Mitsubishi, and

others.

Automatic determination checks location F000:FFB3 for the

string "MITSUBISHI".  Later Mitsubishi PCs had it at a

different location.  We hope all early versions had it at this

location.

Get Date:       MOV   AH,4

INT   1AH

Returns CL=year mod 1980, DH=month, and

DL=day all in binary.

Also sets AL=0FFH if the clock is busy and the

request should be retried.

Set Date:       MOV   AH,5

Load CL, DX as for Get Date.

INT   1AH

Returns AL=0FFH if busy.

Get Time:       MOV   AH,2

INT   1AH

Returns CH=hour, CL=minute, DH=second, and

AL=status.  AL=0FFH means busy.  AL=0 means

24-hour mode.  AL=1 means AM and AL=3 means

PM.

Set Time:       MOV   AH,3

Load AL, CX, DH as for Get Time.

INT   1AH

Returns AL=0FFH if busy.

NOTE:  Some early Mitsubishi PCs came with fixed disk

controllers that were not DOS compatible.  In addition to the

clock modifications the DOS for those systems also handled the

disk controller variants.  If you have such a PC, CLOCK.SYS is

not sufficient to enable you to go above DOS level 3.2.  You

will also need to replace your fixed disk controller.  I

strongly suggest that you create a boot floppy for your current

DOS level before you try to install your new DOS level.  Then

boot your new DOS from a floppy disk, do SYS C: and copy

COMMAND.COM to C:, then try a disk boot.  If the disk boot

fails, boot the floppy you just created and do a SYS C: and

copy COMMAND.COM to C: to restore your old DOS version.  At

that point, your best bet is to simply give up and stay with

DOS 3.2. If you want to proceed, you must replace your fixed

disk controller.

If SYS C: gives you an error status that indicates that there

isn't room for the new DOS version (very possible when moving

from 3.2 to 3.3 or above), you must backup your entire fixed

disk and use FORMAT C:/S to place the new version of DOS on the

disk.  While this is a hassle, at least if your PC won't boot

from the disk, the fall-back approach is identical.  There will

be room to put DOS 3.2 or earlier back on the fixed disk.

_______________________________________________________________________________

6              I/O bus clock.  Used on AST Research boards and others.

Typically uses the same I/O base address as type 4.  However

the clock registers are selected by sending the register

address to the base I/O address and then reading or writing

data at the base I/O address + 1.  The internal register

numbers and their contents are the same as for type 4.

Automatic determination by probing the address register.  If

the low-order bit is one, it may be this type of clock or

type 7.  We then look further to determine if it is type 7.

If you aren't sure which type of clock you have but believe it

is type 4, 6 or 7, specify type 8 and automatic probing will

detect which variant it is.  If at all possible specify the

base I/O address as that will increase the likelihood that

the correct determination is made.

I/O addresses used are (again using 2C0 as the example base

address):

2C0  =  Address register.  Writing a value to this address

selects the internal register to read or write.

2C1  =  Data register.  Reading or writing this address,

reads or writes the internal register previously

selected by writing 2C0.

The internal register numbers are (the values to write to 2C0):

1  =  100ths of seconds

2  =  seconds

3  =  minutes

4  =  hours

6  =  days

7  =  months

9  =  month last time date was read or written

A  =  year

B  =  initialization flag

14  =  status

Other processing is the same as type 4.

_______________________________________________________________________________

7              I/O bus clock.  Used on AST Research boards and others.

Typically uses the same base I/O address as types 4 and 6.

Uses a more complex register access sequence than type 6.

Can be detected by looking to see if the value in register D

has bit 1 (value=2) set.  If you aren't sure which type of

clock you have but believe it is type 4, 6 or 7, specify type

8 and automatic probing will detect which variant it is.  If

at all possible specify the base I/O address as that will

increase the likelihood that the correct determination is made.

The base I/O address is used as an address register and the

base + 1 as a data register.  This is the same as for clock

type 6.  However, the internal registers and the form of the

commands is quite different.  The primary difference is that

the internal registers only hold four bits of data each.  Thus

each internal register represents a single decimal digit.

The I/O addresses are used as follows (using 2C0 as an

example):

2C0  =  Address register.  The number of an internal

register plus 80H is written to this I/O address to

select an internal register.

2C1  =  Data register.  Four bits of data are read or

using this I/O address from or to the internal

register previously selected by writing to 2C0.

The internal registers are:

0  =  Units digit of seconds

1  =  Tens digit of seconds

2  =  Units digit of minutes

3  =  Tens digit of minutes

4  =  Units digit of hours

5  =  Tens digit of hours

6  =  Day of week

7  =  Units digit of day of month

8  =  Tens digit of day of month

9  =  Units digit of month

A  =  Tens digit of month

B  =  Units digit of year

C  =  Tens digit of year

D  =  Function register

E  =  Function register

F  =  Function register

_______________________________________________________________________________

8              I/O bus clock.  Type unknown.  Probe only for types 4, 6, 7,

9, and B.  Use type 8 if you are sure that you have an I/O bus

(i.e., the clock is on an add-on card) clock but aren't sure

whether it is type 4, 6, 7, 9, or B.  If at all possible,

supply the base I/O address of the clock as this will greatly

increase the likelihood of the correct handling method being

chosen.

_______________________________________________________________________________

9              I/O bus clock.  This type was used by Quadram Corporation on

the QuadCard, QuadCard II, and QuadCard 512+.  It may have been

used on others as well.

Automatic determination checks to see that I/O registers exist

at 310H or 210H.

This clock used four I/O control registers and twelve internal

registers.  The I/O registers (using 310H as the example) are:

310  =  Data register (used to move data values)

311  =  Address register (used to select internal register)

312  =  Function register (Read, write, hold counter)

313  =  Control register (Enables read or write function)

The internal registers are:

0  =  Units digit of seconds

1  =  Tens digit of seconds

2  =  Units digit of minutes

3  =  Tens digit of minutes

4  =  Units digit of hours

5  =  Tens digit of hours - Also uses bit 2 (=4) to

indicate PM if in 12-hour mode and bit 3 (=8) to

select 24-hour mode

6  =  Day of week (we don't use this)

7  =  Units digit of days

8  =  Tens digit of days - Also uses bit 3 (=8) to

indicate that this is a leap year.  This bit must

be set by software whenever there is less than 366

days until the next leap day.  If you do not boot

within that period, leap day will not be taken.

9  =  Units digit of months

10  =  Tens digit of months

11  =  Units digit of year mod 1980

12  =  Tens digit of year mod 1980

NOTE:  If you have this type of clock, you can only set the

time to the nearest minute.  Whenever the time is set, the

seconds are always set to zero.  You should set the time as

the minute changes.

_______________________________________________________________________________

A              AT&T 6300, 6300 PLUS, 6300 WGS

Automatic probing is accomplished by trying to read the clock.

If the read attempt using the BIOS is successful (carry flag

not set), the clock is assumed to be present.

NOTE:  AT&T clocks do not advance the year on January 1.  It

is necessary to change the date at the beginning of each

year.  Also, AT&T clocks always clear the seconds to zero

when the time is changed.  For this reason you should always

set the date and time at the beginning of a minute.

A base year should be specified on the DEVICE= line.  If it

is not, 1992 will be used.  This base will be good for all

dates from 1992-1-1 till 1999-12-31.

Get Date:       MOV   AH,0FEH

INT   1AH

Returns BX = Days since base,

CH = Hours, CL = Minutes, DH = Seconds,

DL = 100ths of seconds

Set Date:       MOV   AH,0FFH

MOV   BX,Days since base

MOV   CH,Hours

MOV   CL,Minutes

MOV   DH,0

MOV   DL,0

INT   1AH

_______________________________________________________________________________

B              Hyundai 8088 clock.

Automatic determination probes the control registers.  If they,

appear to be present, CLOCK tries to read the clock.  Legal

time and date values will select this clock.

This clock uses I/O addresses E0H through EFH as follows:

E0      Units digit of seconds

E1      Tens digit of seconds

E2      Units digit of minutes

E3      Tens digit of minutes

E4      Units digit of hours

E5      Tens digit of hours

E6      Day of week

E7      Units digit of days

E8      Tens digit of days

E9      Units digit of month

EA      Tens digit of month

EB      Units digit of year

EC      Tens digit of year

ED      Control

EE      Control

EF      Control

_______________________________________________________________________________

DEFINITION OF SHAREWARE

Shareware distribution gives users a chance to try software before buying

it. If you try a Shareware program and continue using it, you are expected

to register. Individual programs differ on details -- some request

registration while others require it, some specify a maximum trial period.

With registration, you get anything from the simple right to continue using

the software to an updated program with printed manual.

Copyright laws apply to both Shareware and commercial software, and the

copyright holder retains all rights, with a few specific exceptions as

stated below. Shareware authors are accomplished programmers, just like

commercial authors, and the programs are of comparable quality. (In both

cases, there are good programs and bad ones!) The main difference is in the

method of distribution. The author specifically grants the right to copy

and distribute the software, either to all and sundry or to a specific

group. For example, some authors require written permission before a

commercial disk vendor may copy their Shareware.

Shareware is a distribution method, not a type of software. You should find

software that suits your needs and pocketbook, whether it's commercial or

Shareware. The Shareware system makes fitting your needs easier, because

you can try before you buy. And because the overhead is low, prices are low

also. Shareware has the ultimate money-back guarantee -- if you don't use

the product, you don't pay for it.

DISCLAIMER - AGREEMENT

Users of CLOCK must accept this disclaimer of warranty:  "CLOCK is

supplied as is.  The author disclaims all warranties, expressed or implied,

including, without limitation, the warranties of merchantability and of

fitness for any purpose.  The author assumes no liability for damages,

direct or consequential, which may result from the use of CLOCK."

CLOCK is a "shareware program" and is provided at no charge to the user

for evaluation.  Feel free to share it with your friends, but please do not

give it away altered or as part of another system.  The essence of

"user-supported" software is to provide personal computer users with

quality software without high prices, and yet to provide incentive for

programmers to continue to develop new products.  If you find this program

useful and find that you are using CLOCK and continue to use CLOCK after

a reasonable trial period, you must make a registration payment of $10 to

Ronald Q. Smith.  The $10 registration fee will license one copy for use on

any one computer at any one time.  For a registration fee of $25, I will

immediately send you the latest version of CLOCK including the source

code.  You may use the source code for your own maintenance of CLOCK or as

a learning tool for any software that you develop.  You may not use all or

part of the source code in any software that you develop or release to

others without the permission of Ronald Q. Smith.

You must treat this software just like a book.  An example is that this

software may be used by any number of people and may be freely moved from

one computer location to another, so long as there is no possibility of it

being used at one location while it's being used at another.  Just as a

book cannot be read by two different persons at the same time.

Commercial users of CLOCK must register and pay for their copies of CLOCK

within 30 days of first use or their license is withdrawn.  Site-License

arrangements may be made by contacting Ronald Q. Smith.

Anyone distributing CLOCK for any kind of remuneration must first contact

Ronald Q. Smith at the address below for authorization.

You are encouraged to pass a copy of CLOCK along to your friends for

evaluation.  Please encourage them to register their copy if they find that

they can use it.  All registered users will receive a copy of the latest

version of the CLOCK system.

Send the fees and any inquiries to:

Ronald Q. Smith

11 Black Oak Road

North Oaks, Mn. 55127-6204

You may also contact me via CompuServ mail at userid 71620,514.  I will be

happy to respond to any problems and suggestions for future capabilities.

You may register CLOCK through the CompuServe Shareware Registration program.

CLOCK.SYS is found under registration ID 104.  The fee for registering

through CompuServe is $12 (to cover their charge) and will be added directly

to your CompuServe bill.

_______________________________________________________________________________

REVISION HISTORY:

921221 -        Version 3.01: Fixed a couple of bugs in beta test version

3.00 with non-PC/AT clocks.  Added logic to CLK to display

the pre-defined time zones on errors or request.

921219 -        Version 3.00: Added support for automatic time zone and

daylight savings time adjustment.  CLOCK.SYS now has the

ability to manage the DOS real time clock and the battery-

protected clock separately.  CLK.EXE is used to convert

a base value in the battery-protected clock to a correct

local time in the DOS clock.

920614 -        Version 2.24: Fixed bug in probing of AT&T 6300 clocks.

Some of the clocks do not set AL on successful operation.

920604 -        Version 2.23: Fixed bug causing fall through into memory

probing when only I/O probing was wanted.

920411 -        Version 2.22: Changed order of probing to probe memory-

mapped clocks last as they are being spuriously found on

some Tandy systems.

920401 -        Version 2.21:  Changed pattern of I/O address probing to

probe all possible I/O addresses for one type (e.g., 4)

before trying any for another type.  Try to avoid finding

the wrong type.  Added the type B clock to the type 8 probe.

920328 -        Version 2.20:  Changed order of I/O address probing for clock

types 4, 6, and 7 to avoid, where possible, accessing other

boards.  Most commonly used I/O clock addresses, I hope, are

now probed first.

920322 -        Version 2.19:  Improved probing for I/O clocks to check that

legal values are found in all the data registers.  Several of

the I/O clocks use operation sequences so much like each other

that it is possible to choose the wrong one if we don't

actually read the time and date and see if the values are

legal.  This approach to probing may possibly result in not

finding a clock that is present but that has been totally

reset.  Such a clock should be found on a second try to boot

with CLOCK.SYS as the counters will have advanced by then.

920320 -        Version 2.18:  Added probing for generic PC/AT type clock on

systems that don't have the PC/AT signature in the BIOS.

920307 -        Version 2.17:  Added Hyundai 8088 clock.

920112 -        Version 2.16:  Added AT&T 6300 clock.

920111 -        Version 2.15:  Fixed memory address used by one type of SMT

No-Slot clock.

920105 -        Version 2.14:  Added support for a wider variety of memory

mapped clocks including the SMT (Systems Manufacturing

Technology) No-Slot clocks.

911208 -        Version 2.13:  Add probing for Vendex Headstart Turbo-888-XT

system and any other I/O clock at 300H.  Change I/O clock

probing logic to make it less likely to find the wrong type

by accident.

911129 -        Version 2.12:  Fix handling of Quadram clocks (type 9).

They can only set seconds to zero so generate an error

if anyone tries to set the seconds to any other value.

911128 -        Version 2.11:  Fix bug in handling clock type 9, Quadram

clocks that left clock turned off after operations.  Also

fix bug in busy determination for clock types 4 and 6 that

could result in a decision that the clock was not operational.

911121 -        Version 2.10:  Add clock types 6 ,7 and 9.  Add type 8 to do

specific probing for the various I/O bus clock types.

911109 -        Version 2.05:  Fix bug in leap day handling for I/O clocks.

Fix bug that caused hung systems when setting the date or

time on Z-18x portables.

911026 -        Version 2.04:  Add support for I/O clocks at addresses 240H

and 340H.  Reduce resident memory required by clock driver to

only that required for specific clock type.

911021 -        Version 2.03:  Change handling of new day flag from BIOS to

accommodate BIOSs that indicate multiple days have passed.

911020 -        Version 2.02:  Fix bug in returning date on clock read

functions.  Added message in automatic determination to

indicate type found.

911010  -       Version 2.01:  Added automatic determination of Mitsubishi

8088 PC clocks.  Fixed handler to keep the BIOS counter set

properly for programs that read it directly.  Also returns a

finer granularity value (100ths of seconds) on reads.

911009  -       Version 2.00:  Modified clock driver to support additional

clock types.  Thanks to Eric Smith (no relation) for help in

finding out how the Zenith PC clock worked.

911005  -       Version 1.20:  Changed prologue and epilogue to conform to

DOS 5.00 documentation.  Seems to work OK with DOS 5.0

without the changes, but we might as well be consistent to

save problems later.

910903  -       Version 1.10:  Added display of current date and time when

initializing.

910518  -       CLOCK version 1.00.  Initial version developed at the request

of a friend with a Sperry PC/HT who wanted to move to DOS 4.01.

1984    -       Write clock device driver for AST clock for IBM PC-1.  This

serves as the introduction to what device drivers and

especially the CLOCK$ driver are all about.