==============================

KXP-1180 UPGRADE

==============================

by Ken Schmidt

According to the Tenex Computer catalog all 1100 series Panasonic printers use

the same buffer upgrade kit. So after reading the file MOD1124.ZIP, a download

file on this BBS, I decided that since the KX-P1180 appeared to have the same

case as the KX-P1124 that I would have no trouble adding a 32K buffer to my

printer. WRONG! The only similarity between the two is that you have to remove

the front faceplate first to disassemble the rest of the case. Thanks to a

helpful service technician at a local computer store I was able to get a look

at the correct disassembly instructions for the KX-P1180. You will need to

acquire a 62256LP-12, 28 pin static ram chip to make this modification.

Computer Shopper has several companies that sell them for around $6.00 + 3.50

S&H. If you can get it locally you may be able to save some because of no S&H.

DO NOT ATTEMPT THIS MODIFICATION UNLESS YOU HAVE EXPERIENCE INSTALLING COMPUTER

CHIPS. IF YOU HAVE A FRIEND THAT COULD INSTALL THE CHIP YOU COULD GET THE

PRINTER READY FOR HIM BY DISASSEMBLING THE CASE FIRST. The procedure I used is

as follows:

1) Disconnect printer from AC power and remove RS232 cable, plastic paper guide

panel and plastic cover above print head. Turn printer upside down and observe

two holes at front of printer about 5/8 x 3/8. One on each side. Shine a flash

light in the hole and look for a small plastic latch at the end of the hole.

Using a flat tip screwdriver, gently pry this latch away from the case with one

hand. When it releases the front panel, put another flat tip screw driver in

the crack and pry gently on the front panel. This will release several

male/female catches along the top and the bottom of the front panel. I believe

there were 3 or 4 on both the top and bottom of the front panel. Lastly pry the

plastic latch open through the other hole. If your work table is big enough the

front panel need not be disconnected from its electrical cable, otherwise

disconnect the front panel and lay it safely aside.

2) Behind the front you will find two more plastic latches holding the top of

the case to the base. Gently pry open these latches and swing the front of the

top backwards. It may then be unhooked from the base at the back. Lay the top

safely aside.

3) On the rear of the pc board will be a 28 pin chip with an empty 28 pin

socket next to it. This is were the 62256LP-12 chip goes. It is the only empty

socket on the pc board. If you are installing a brand new chip the pins will

have to be bent perpendicular to the chip to fit into the socket. This can be

done by laying the chip on its side on a hard flat surface. If it don't come

out right the first time gently bent the pins with a needle nose pliers. BE

SURE TO TAKE PRECAUTIONS FOR STATIC ELECTRICITY WHILE WORKING WITH THE CHIP.

When all the pins are OK insert the chip with the notch on the same side as the

adjacent chip. Make sure all pins are in the socket before proceeding with the

reassembly.

4) Put the plastic top on by attaching the rear first, then swing it down to

the front. Gently pry the two front latches open until the top snaps in place.

If you disconnected the front control panel now is the time to reconnect it.

Next attach the front panel by sliding the male/female catches together on the

top first. Then snap closed a latch on one side. Next mate the male/female

catches on the bottom. It may take a little prying with a screw driver to do

this. Last snap the other side latch closed.

Now for the fun. The new 32K buffer can be used for a text buffer, to  download

a user designed character set or a combination of both. After a new character

set is downloaded the printer will automatically configure the rest of the

buffer for text. The full 32K will store about 7 pages of text. This can be

tested by loading a large text file into any program and then pressing ESC

after it has been printing for several minutes. The printer will continue

printing for about 5 minutes. I will also be uploading a program to the Mahoney

collection that will allow designing new character sets, downloading them to

the printer and print text files with the new character set. The program will

be called FXMATRIX.ZIP, this is for 9 pin printers and only supports draft

style fonts. The same author has a program for 24 pin printers called

LQMATRIX.ZIP that is available in the Mahoney collection.