Syntax:-

PRTEST [n]

Where [n] can be 1, 2 or 3 for LPT1, LPT2 or LPT3. If [n] is omitted then

LPT1 is assumed.

The following information is displayed on the screen when you run the program:-

IBM PC PARALLEL PORT TEST PROGRAM

---------------------------------

The program works by looping the outputs onto the inputs, therefore

if a fault is found it can only be isolated as far as one or both of

two things not functioning. Since any fault is serious, and printer

ports are hardly repairable, this level of fault finding is quite

adequate. Also, since all the inputs are tested with both plugs, it

will usually be possible to tell exactly where the fault is.

The final test with a printer is not really necessary.

You need two 25 pin male 'D' connectors connected as loop back plugs.

The connections below are between pins on the same plug, there are no

connections between the two plugs:-

PLUG1                                 PLUG 2

-----                                 ------

pin 2 to pin 10                       pin 6 to pin 10

pin 3 to pin 11                       pin 7 to pin 11

pin 4 to pin 12                       pin 8 to pin 12

pin 5 to pin 13                       pin 9 to pin 13

pin 6 to pin 15                       pin 1 to pin 15\n

It's simple, but it works!

Roger Barker, G4IDE

6th November 1992