PREFACE

FCC COMPLIANCE

This equipment generates and uses radio frequency energy and if not installed

and used properly, that is, in strict accordance with the manufacturer's

instructions, may cause interference to radio and television reception. It has

been type tested and found to comply with the limits for a Class B computing

device in accordance with the specifications of Subpart J of Part 15 of FCC

Rules, which are designed to provide reasonable protection against such

interference in residential installation. However, there is no guarantee that

interference will not occur in a particular installation. If this equipment

does cause interference to radio or television equipment reception, which can

be determined by turning the equipment off and on, the user is encouraged to

try to correct the interference by one or more of the following measures:

Reorient the receiving antenna

Relocate the computer with respect to the receiver

Move the computer away from the receiver

Plug the computer into a different outlet so that computer and receiver are

on different branch circuits

If necessary, the user should consult the dealer or an experienced

radio/television technician for additional suggestions. The user may find the

following booklet prepared by the Federal Communications Commission helpful:

"How to Identify and Solve Radio-TV Interference Problems" This booklet is

available from the U.S. Government Printing Office, Washington, D.C. 20402,

Stock No. 004-000-00345-4.

Warning: This equipment has been certified to comply with the limits for Class

B computing device, pursuant to Subpart J of Part 15 of FCC Rules. Only

peripherals (computer input/output devices, terminals, printers, etc.)

certified to comply with the Class B limits may be attached to this computer.

Operation with non-certified peripherals is likely to result in interference

to radio and TV reception. Also this equipment must be used with shielded

power cables and shielded I/O cables.

COPYRIGHT

Copyright 1988 Adaptec, Inc. All Rights Reserved.

No part of this publication may be reproduced, stored in a retrieval system,

or transmitted, in any form or by any means, electronic, mechanical,

photocopying, recording or otherwise, without the prior written consent of

Adaptec Inc., 691 S. Milpitas Blvd., Milpitas, California 95035

TRADEMARKS

The trademarks referred to in this manual are identified and acknowledged as

follows: IBM PC AT, PC-DOS, Micro Channel, Personal System/2, PS/2, Operating

System/2, OS/2 are registered trademarks of International Business Machines

Corporation. MS-DOS and XENIX are registered trademarks of Microsoft

Corporation. UNIX is a registered of AT&T. Netware is a registered trademark

of Novell, Inc. SCO is a trademark of the Santa Cruz Operation.

CHANGES

The material in this manual is for information only and is subject to change

without notice.

Adaptec reserves the right to make changes in the product design without

reservation and without notification to its users.

Additional information may be obtained from:

Adaptec, Inc

Literature Department

691 South Milpitas Blvd.

Milpitas, CA 95035

Table of Contents

1.0 Introduction                                                1-1

1.1 Document Scope                                           1-1

1.2 Purpose                                                  1-1

1.3 AHA-1640 Product Features                                1-1

1.4 Product Specifications                                   1-2

1.5 Reference Documents                                      1-3

2.0 Host Adapter Architecture                                   2-1

2.1 Hardware                                                 2-1

2.1.1 First Party DMA                                     2-1

2.1.2 SCSI Protocol Chip (ACI-6250)                       2-1

2.1.3 Host Adapter DMA Chip                               2-1

2.1.4 High Performance Control Processor                  2-1

2.1.5 Hardware Block Diagram                              2-2

2.2 Firmware                                                 2-3

2.2.1 Multithreaded Operation                             2-3

2.2.1.1 Mailboxes                                         2-3

2.2.1.2 Command Control Block                             2-6

2.2.1.3 Command Descriptor Block                          2-6

2.2.1.4 Principles of Operation                           2-7

2.2.1.5 Task Queueing                                     2-7

2.2.2 Single-Threaded Operation through the BIOS          2-7

3.0 Installation                                                3-1

3.1 Unpacking and Inspecting                                 3-1

3.2 Installation                                             3-1

3.2.1 System Requirements                                 3-1

3.2.2 Hardware Installation                               3-1

3.2.3 Software Installation                               3-2

3.2.4 Common Problems during Installation                 3-5

4.0 Multithreaded Software Interface                            4-1

4.1 Hardware Overview                                        4-1

4.2 POS Register Interface                                   4-2

4.2.1 POS Register 0                                      4-4

4.2.2 POS Register 1                                      4-5

4.2.3 POS Register 2                                      4-5

4.2.4 POS Register 3                                      4-5

4.2.5 POS Register 4                                      4-6

4.2.6 POS Register 5                                      4-7

4.3 Preparation of Micro Channel DMA Hardware                4-7

4.3.1 8237 Compatibility Mode                             4-8

4.3.2 DMA Extended Mode                                   4-9

4.4 I/O Port Interface                                       4-9

4.4.1 Control and Status Port                             4-11

4.4.2 Command/Data Out and Data In Port                   4-13

4.4.3 Interrupt Flag Port                                 4-13

4.5 Reset Overview                                           4-15

4.5.1 Hard Reset Operations                               4-15

4.5.2 SCSI Reset Operations                               4-16

4.5.2.1 SCSI "Soft" Reset Option                          4-16

4.5.2.2 SCSI "Hard" Reset Option                          4-17

i

Table of Contents

4.6 I/O Command Port (IOCP) Command Set                      4-17

4.6.1 No Operation                                        4-18

4.6.2 Mailbox Initialization                              4-19

4.6.3 Start SCSI Command                                  4-19

4.6.4 Start BIOS Command                                  4-19

4.6.5 Adapter Inquiry                                     4-20

4.6.6 Enable Mailbox Out Available Interrupt              4-21

4.6.7 Set SCSI Selection Time Out                         4-21

4.6.8 Set Bus On Time                                     4-22

4.6.9 Set Bus Off Time                                    4-22

4.6.10 Set Transfer Speed                                 4-22

4.6.11 Return Installed Devices                           4-22

4.6.12 Return Configuration Data                          4-22

4.6.13 Enable Target Mode                                 4-25

4.6.14 Return Setup Data                                  4-25

4.6.15 Write Channel 2 Buffer                             4-27

4.6.16 Read Channel 2 Buffer                              4-27

4.6.17 Write Adapter FIFO Buffer                          4-27

4.6.18 Read Adapter FIFO Buffer                           4-28

4.6.19 Echo Command Data                                  4-28

4.7 Mailbox Description                                      4-28

4.7.1 Mailbox Out Definition                              4-29

4.7.2 Mailbox In Definition                               4-30

4.8 Command Block Definition                                 4-34

4.9 Scatter/Gather List Definition                           4-40

4.10 Description of Operation                                4-41

4.10.1 Initialization Procedure                           4-41

4.10.2 Execution of Initiator Mode Operations             4-41

4.10.3 Execution of Target Mode Operations                4-42

4.10.3.1 Target Mode with a Prepared CCB                  4-42

4.10.3.2 Target Mode without a Prepared CCB               4-42

4.10.4 CCB Queueing and Scan Order                        4-43

5.0 Single Threaded Operation                                     5-1

5.1 Single Threaded BIOS Command Interface                   5-1

5.2 BIOS Command Execution using Interrupt 13                5-1

5.2.1 Physical to Logical Address Translation             5-2

5.2.2 BIOS Command Return Codes                           5-2

5.3 BIOS Disk Commands                                       5-4

5.3.1 Reset Disk System                                   5-5

5.3.2 Read Status of Last Operation                       5-5

5.3.3 Read Desired Sectors from Memory                    5-5

5.3.4 Write Desired Sectors from Memory                   5-6

5.3.5 Verify Desired Sectors                              5-6

5.3.6 Identify SCSI Devices                               5-6

5.3.7 Read Drive Parameters                               5-7

5.3.8 Initialize Drive Pair Characteristics               5-7

5.3.9 Seek                                                5-7

5.3.10 Alternate Disk Reset                               5-8

5.3.11 Test Drive Ready                                   5-8

5.3.12 Recalibrate                                        5-8

5.3.13 Read DASD Type                                     5-9

ii

Table of Contents

6.0 Device Drivers                                              6-1

6.1 PS/2_ Compatible POS Initialization                      6-1

6.2 PS/2_ Compatible BIOS                                    6-1

6.2.1 Initialization                                      6-1

6.2.2 System Boot                                         6-2

6.2.3 Hard Disk I/O                                       6-2

6.3 Unix/Xenix                                               6-2

6.4 DOS Driver                                               6-3

6.5 OS/2_ Driver Availability                                6-3

6.6 Virtual Memory Driver                                    6-3

6.7 AHA-1640 On-board Utilities                              6-4

6.8 Third Party Software Suppliers for AHA-1640              6-5

7.0 SCSI Features                                                7-1

7.1 SCSI Description, Initiator Mode                         7-1

7.1.1 Linked SCSI Commands                                7-2

7.1.2 Zero Latency Read Operation                         7-2

7.2 SCSI Description, Target Mode                            7-3

7.2.1 Initiator Conformance Level Requirements            7-3

7.2.2 Support of Synchronous Transfer                     7-3

7.2.3 SCSI Target Operation In Processor Target Mode      7-3

7.2.3.1 TEST UNIT READY                                   7-4

7.2.3.2 REQUEST SENSE                                     7-5

7.2.3.3 INQUIRY                                           7-7

7.2.3.4 SEND DIAGNOSTIC                                   7-8

7.2.3.5 SEND                                              7-8

7.2.3.6 RECEIVE                                           7-9

7.2.4 Incorrect Length Management                         7-9

8.0 Problem Determination                                       8-1

8.1 Self Diagnostics                                         8-1

8.2 Indicators                                               8-2

8.3 Problem Correction                                       8-2

8.3.1 Common Problems during Installation                 8-2

8.3.2 IOCP Command Errors and Corrective Actions          8-2

8.3.3 HA Status Error Indications and Corrective Actions  8-3

8.3.4 SCSI Error Indications and Corrective Actions       8-5

9.0 Glossary                                                    9-1

iii

Section One                                                       Introduction

------------------------------------------------------------------------------

1.1 DOCUMENT SCOPE

This manual provides  the information  required to  install and

program the AHA-1640 intelligent host adapter in a Micro Channel

compatible computer system.

1.2 PURPOSE

The Adaptec AHA-1640 provides a powerful multitasking interface

between the Micro Channel and the Small Computer System Interface

(SCSI) bus. The AHA-1640 is a high-performance intelligent host

adapter supporting  a  maximum asynchronous  SCSI  rate  of 2.0

Mbytes/second and a synchronous transfer rate of 5 Mbytes/second.

The AHA-1640  supports multithreaded  I/O  operations, allowing

simultaneous  operations  on  multiple  targets/Logical  Units.

Disconnect/Reconnect  support  maximizes  bus  utilization  for

multiple target  systems.  Target  Mode  operation  allows  the

AHA-1640 to  receive  information  from  other  host  adapters.

Scatter/gather capability allows a continuous stream of data from

the SCSI bus to be distributed  across many segments of memory.

Zero Latency Read (ZLR) speeds delivery of data from peripherals

that support the function.

The Adaptec AHA-1640 meets  system application requirements for

very high performance,  configuration flexibility, multitasking

capability, and system  redundancy. The  AHA-1640 also provides

standard hard disk controller services through BIOS calls.

1.3 AHA-1640 PRODUCT FEATURES

� High-performance First Party DMA with  a burst data rate of 8

Mbytes/second

� 16- and 8-bit transfers

� Even or odd starting address transfers

� Maximum synchronous SCSI transfer rate of 5.0 Mbytes/second

� Asynchronous and synchronous peripherals supported simultaneously

� Full arbitration support with fairness

� Programmable Option Select (POS) to set arbitration level

� POS to set interrupt level

� POS to select port addresses

� POS to select BIOS addresses

� POS to obtain the card id (0F1FH)

� True  multithreaded operation supporting up to 255 tasks simultaneously

� Programmable mailbox architecture

� Adaptec BIOS for standard hard disk emulation

� Internal and external SCSI connectors

� Host memory scatter/gather capability

� Processor type target functions supported  for 1 to 8 logical threads

� Host adapter status and diagnostic results displayed by LED

1-1

Section One                                                       Introduction

------------------------------------------------------------------------------

1.4 PRODUCT SPECIFICATIONS

� Approximate Physical Dimensions

Length: 323 mm (12.7")

Width: 100 mm (3.93")

Height: 21 mm (0.82")

Standard 16-bit Micro Channel form-factor.

Electrical interface and interface timing as specified for

16-bit Micro Channel.

� Power Requirements

+5.0 +/- 0.25 Volts at 1.6 Amps maximum.

� Environmental Requirements

Operating temperature: 0-60� C

Storage temperature:    0-60� C

� Reliability Information

Mean Time Between Failures: 15000 hours

(calculated per Mil Handbook 217E, ground benign, 40�C)

Mean Time Between Failures: TBD hours

(experimental:)

Mean Time To Repair: 30 minutes

� SCSI Interface

Electrical Interface:

As specified by ANSI X3.131-1986 for single-ended operation.

Minimum driver output: 48 milliamps, sinking @ 0.5 volts

Signal true:          min:   0   volts     max:   0.8  volts

Signal false:         min:   2.0 volts     max:   5.25 volts

Input hysteresis:     min:   0.2 volts

Terminator Power voltage:  min: 4.25 volts max:   5.25 volts

Terminator Power current:  min: 800 ma.    max:   1.5 amp (fused)

1-2

Section One                                                       Introduction

------------------------------------------------------------------------------

Internal connector:

Unshrouded 50-pin header,  compatible with unshielded

alternative  1   connector  as   specified   in  ANSI

X3.131-1986 Figure 4-1

Partial list of compatible connector plugs:

Manufacturer          Model           Part Number

-------------------------------------------------

3-M                   N.A.            3425-6000

T&B Ansley            N.A.            609-5000M

Molex                 N.A.            15-29-8505

External connector:

Shielded 50-pin  connector as  specified  in proposed

SCSI-2 standard  X3T9.2/86-109  Revision  5, Appendix

D, Figure D-3

Partial list of compatible connector plugs:

Manufacturer          Model           Part Number

-------------------------------------------------

Amp                   Amplimite

Fujitsu

Honda

Meets radiation limits specified for a Class B computing device

in accordance with the specifications in Subpart J of Part 15 of

FCC Rules.  See  FCC Compliance  notes  and  recommendations in

preface of this document.

1.5 REFERENCE DOCUMENTS

� Small Computer Systems  Interface, ANSI X3.131-1986, American

National Standards Institute, N.Y., N.Y., 1986

� IBM Personal  System/2 and  Personal Computer  BIOS Interface

Technical Reference,  Document 84X1514, April, 1987

� IBM Personal  System/2 Model  50 and  60 Technical Reference, April, 1987

� IBM Personal  System/2 Model  80 Technical  Reference, April, 1987

� Enhanced Small  Computer Systems  Interface (SCSI-2), working draft,

X3T9.2/87-109  Revision 5, August 8, 1988.

� Adaptec AHA-1640 Host Adapter, Installation Guide

� Adaptec Host Adapter Programming Guide

1-3

Section Two                                         Host Adapter Architecture

------------------------------------------------------------------------------

2.1 HARDWARE

2.1.1 First Party DMA

The AHA-1640 controls the  host Micro Channel  as a Bus Master,

transferring data directly to and  from main system memory. The

host adapter provides its own  DMA services, using the standard

arbitration process  to  gain  control  of  the  Micro Channel.

Operation as a Bus Master reduces  system overhead in two ways.

First, the two memory cycle transfer required by an ordinary DMA

Slave is reduced by operation as a  Bus Master to one cycle. In

addition, host CPU  overhead is reduced  because the  CPU is no

longer required to maintain the  DMA channel's address pointers

and word  counts, manage  multitasking sub-channels,  or handle

extra interrupts.

Adaptec's implementation of Bus Master operation achieves a 6.7

Mbyte/second burst data rate.  This speed is especially valuable

in multitasking systems where the tasks execute on a time-shared

basis. Appendix A shows  a diagram of  the Micro Channel timing

requirements.

The AHA-1640  DMA hardware  will handle  transfers of  odd data

length and transfers beginning  on odd byte  boundaries with no

performance degradation.

2.1.2 SCSI Protocol Chip (AIC-6250)

The AHA-1640 utilizes the AIC-6250 SCSI protocol chip to maximize

the SCSI bus utilization. The AIC-6250 is an Adaptec VLSI device

which allows the AHA-1640 to achieve 2.0 Mbyte/Sec asynchronous

SCSI data transfer rates, and 5.0 MB/second synchronous transfer

rates. The AIC-6250 will also enable the AHA-1640 to operate as

both an initiator and as a target device.

The  AIC-6250   has  separate   data   busses  for   the  local

microprocessor and  for  the  system  data  bus.  This  further

increases the  performance  of  the  AHA-1640  by  reducing the

overhead associated with SCSI commands.

2.1.3 Host Adapter DMA Chip

The AHA-1640  uses  a  highly integrated  DMA  control  chip to

implement  the  Bus   Master  functions.  The   high  level  of

integration assures high performance and high reliability in the

small form factor of the Micro Channel standard card.

2-1

Section Two                                         Host Adapter Architecture

------------------------------------------------------------------------------

2.1.4 High Performance Control Processor

The   AHA-1640   uses   a   high-performance   16-bit   control

microprocessor to reduce system  overhead and provide extremely

fast response time. Total SCSI bus overhead associated with the

host  adapter  is  typically  300   to  500  microseconds.  The

sophisticated multitasking and queueing capability together with

the very low firmware overheads assures superior performance in

any environment.

The  microprocessor   performs   diagnostic   and  verification

operations for installation  and troubleshooting.  An LED under

control of the microprocessor indicates that normal operation is

occurring.

2.1.5 Hardware Block Diagram

[Picture]

FIGURE 2-1. HARDWARE BLOCK DIAGRAM

2-2

Section Two                                         Host Adapter Architecture

------------------------------------------------------------------------------

2.2 FIRMWARE

The AHA-1640  supports multithreaded  SCSI  initiator operation

through a simple mailbox protocol. The firmware accepts up to 112

Command Control Blocks (CCB's) and executes them from its local

RAM. The firmware controls all of the SCSI activity that a task

may require. This includes Arbitration, Selection, Disconnection,

Reconnection, and Command completion.

Using the same mailbox protocol, the  AHA-1640 can operate as a

processor type device serving as a multitasking target to other

initiators. This  feature allows  high  bandwidth communication

between multiple hosts. The communication capability can be used

to  create   high  availability   computer   systems,  parallel

processing systems, or  systems that can  use shared peripheral

devices.

The Bus Master  mode of  operation allows  the host  adapter to

scatter an incoming  data stream  into as  many as  16 separate

memory segments,  or to  collect  segments into  a  single data

stream.  This  scatter/gather   capability  provides  important

performance enhancements  for many  operating systems  that use

virtual memory or memory paging. Zero latency read and queueing

provide the capability of further increasing system performance.

In addition,  the AHA-1640  firmware  cooperates with  the BIOS

installed on the host adapter to emulate the PS/2 hard disk BIOS

interface. This  allows booting  operations from  attached SCSI

hard drives. In addition, the emulation provides support for up

to 2 SCSI hard  drives under DOS without  any requirement for a

device driver.

The firmware also supplies diagnostic capability.

2.2.1 Multithreaded Operation

A multiuser, multitasking operating system issues a large number

of I/O  tasks  in a  rapid  sequence. The  architecture  of the

AHA-1640 makes  management  of  this  activity  very  easy  and

straightforward for the operating system and its associated I/O

drivers. This section briefly  explains the interaction between

the system and the AHA-1640 required to accomplish an I/O task.

2.2.1.1 Mailboxes

The AHA-1640 uses a mailbox architecture for task communication

between the host and host adapter. This allows the host adapter

to perform  multithreaded  operations with  a  minimum  of host

intervention.The mailboxes are  located in  main system memory.

Each mailbox entry is four bytes long. After power up sequencing,

host initialization  procedures,  and  the  boot  procedure are

completed, the host issues an  initialization command to inform

the host adapter of  the mailbox location.  There are always an

equal number of Outgoing Mailboxes (MBO) and Incoming Mailboxes

(MBI). MBIs are located  immediately after the  MBOs. A typical

mailbox structure is shown in Figure 2-2.

2-3

Section Two                                         Host Adapter Architecture

------------------------------------------------------------------------------

[Picture]

FIGURE 2-2. TYPICAL MAILBOX STRUCTURE

In this example, there  are four MBOs and  four MBIs. The first

byte of each MBO contains the command byte. The remaining three

bytes point to a Command Control  Block (CCB). The CCB provides

additional task information. AMBO is free  if the first byte is

zero. The  first byte  of  each MBI  contains the  status  of a

completed task. The remaining three bytes point to the CCB of the

task. An MBI is free if the  Status byte is zero. Mailboxes may

point to CCBs that control initiator tasks, that service target

tasks or that perform a Bus Device Reset.

2.2.1.2 Command Control Block

A Command Control  Block provides  the information  required to

control a SCSI command sequence when the AHA-1640 is performing

initiator functions. The  block contains  pointers to  the data

area to be used by the command. It contains areas for presenting

status of both the host adapter  and the addressed SCSI device.

In addition,  it  contains the  SCSI  Command  Descriptor Block

defining the action to be taken by the addressed SCSI device. An

error information buffer area is also provided.

The AHA-1640 can also serve as a target, providing or accepting

data blocks at the command of some other initiator. The AHA-1640

acts like a standard  SCSI processor device.  In this case too,

Command Control  Blocks  are  used  to  perform  the operations

requested by other initiators to the AHA-1640.

The CCB is defined completely in Section 4.8.

A typical initiator CCB is shown in Figure 2-3.

2-4

Section Two                                         Host Adapter Architecture

------------------------------------------------------------------------------

[Picture]

FIGURE 2-3. TYPICAL INITIATOR CCB

2-5

Section Two                                         Host Adapter Architecture

------------------------------------------------------------------------------

2.2.1.3 Command Descriptor Block

The Command Descriptor Block (CDB), a part of the Command Control

Block, is a standard format command packet that is transmitted to

the addressed  SCSI  device.  It contains  all  of  the command

information required by the SCSI  device to perform the desired

operation. The  Command Descriptor  Block contains  the command

Operation Code followed by a Logical Unit Number (LUN), command

parameters if required and a control byte. A typical Group 0 six

byte CDB is shown in Figure 2-4.

[Picture]

FIGURE 2-4. TYPICAL GROUP 0 SIX BYTE COB

Please refer to the SCSI  specification ANSI X3.131, the Common

Command Set  (CCS)  revision  4B,  and  the  SCSI  2 draft  for

additional information on Command Descriptor Blocks.

2.2.1.4 Principles of Operation

At power-up, the host informs the  host adapter of the location

and number of mailboxes. To start a task, the host builds a CCB

and stores the memory address of the CCB into a free mailbox. A

non-zero Mailbox Out command  byte is then  written to indicate

that the mailbox entry is full and valid. The host then writes to

an I/O port  (defined in Section  4) to indicate  that the host

adapter should scan the MBO area. When a full MBO is found, the

host adapter copies the mailbox's CCB pointer into its internal

RAM and clears the Mailbox  entry by writing a  zero to the MBO

command byte. This frees the MBO so that it can be used to start

another task.  If  the  host  software  prefers,  an  MBO Empty

interrupt can be generated every time that an MBO is cleared by

the host adapter.

After completing a task, the host adapter scans the MBI area for

a free mailbox. When one is found, it is updated with the task's

completion status and  CCB pointer. The  CCB pointer identifies

the completed task. AMBI Full  interrupt is generated to notify

the host that a task has been completed. The host scans the MBI

area searching for a non-zero Status byte. When one is located,

the host obtains the CCB pointer and frees the MBI by writing a

zero into the Status byte. The host then examines the contents of

the  CCB  to  determine  that   the  command  was  successfully

completed.The freed  MBI  can  now  be  used  to  indicate  the

completion of another task. The host adapter fills the MBI area

in a round-robin fashion. If the host saves the position of the

last active MBI entry, it can determine  the MBI of a new entry

immediately without scanning.

2-6

Section Two                                         Host Adapter Architecture

------------------------------------------------------------------------------

The host adapter  never transmits a  new interrupt  to the host

until the previous interrupt has been cleared and serviced by the

host. The host should analyze the  interrupts and clear them as

soon as  possible so  that the  host adapter  can post  any new

interrupts quickly.  The host  adapter will  not wait  until an

interrupt can be transmitted to the host before processing an MBO

entry or creating a new MBI entry. Thus, in processing a single

MBI interrupt, the host may find several MBI entries waiting by

the time  the interrupt  processing  is finished.  Similarly, a

later MBI interrupt for  the last of the  later MBI entries may

find nothing to service because the  MBI entry was examined and

processed as a result  of the first MBI  Full interrupt. If the

interrupts are reset quickly by the host, the probability of an

interrupt occurring when no MBI entry is available is much lower,

providing an  important  performance  improvement.  If  the MBI

entries are emptied by the host in a round-robin order, the scan

for the next full entry is very  simple, since it is always the

next MBI entry in the mailbox area.

2.2.1.5 Task Queueing

Multiple tasks may be  started against a  target/LUN or against

multiple targets/Logical Units. Since only one task can be active

against any one given LUN at a time, all other tasks for the same

LUN are queued.  Other LUNs may  have active tasks  at the same

time.

The host adapter executes commands  on a first-found, first-out

basis for each  target/LUN combination. The  host adapter scans

the MBO entries in  a round robin manner  starting at the first

entry after the location  that was last  identified by the host

adapter as full. If new  MBO entries are made by  the host in a

similar round robin manner, then all entries will be queued and

executed in  a  first  in,  first  out  (FIFO)  order for  each

target/LUN combination. There  is no guarantee  of a particular

order of execution between different target/LUN combinations.

2.2.2  Single threaded operation through the BIOS

A host adapter BIOS is provided to emulate the standard PS/2 hard

disk BIOS and boot functions. With  this BIOS, the host adapter

can be used in lieu of a standard hard disk controller on a PS/2

compatible system.

The BIOS is compatible  with the standard  PS/2 hard disk BIOS.

This allows DOS to access up to two hard disk devices on the SCSI

bus without a  driver. All  normal I/O  functions are supported

including  system  booting.   Single  threaded   operation  and

multithreaded operation do  not operate  simultaneously. Single

threaded operation cannot be  requested until all multithreaded

operations  are   completed.   Similarly,   all   multithreaded

operations must be complete before  a single threaded operation

can be requested. For most multitasking operating systems, such

as Xenix and OS/2,  single threaded operation  is normally used

only for  the early  part of  the  boot operation,  after which

multithreaded  drivers  take  over  all  control  of  the  SCSI

operations.

2-7

Section Three                                                    Installation

------------------------------------------------------------------------------

3.1 UNPACKING AND INSPECTING

The carrier is responsible for damage incurred during shipment.

In case of damage, have the carrier note the damage on both the

delivery receipt and the freight bill, then notify your freight

company representative so that the necessary insurance claims can

be initiated.

After opening the shipping  container, use the  packing slip to

verify receipt of the individual items listed on the slip. Retain

the shipping container and packing  material for possible later

reuse  should  return  of  the  equipment  to  the  factory  be

necessary.

CAUTION: The  AHA-1640  is  carefully  designed  to  resist the

effects of  static  electricity. However,  like  all electronic

equipment, it can  be damaged or  its life can  be shortened by

unusual static discharges.  Please take  the proper precautions

when handling  the  board.  Keep the  board  in  its conductive

wrapping until it is  ready to be installed  in your system. Be

sure that  the host  computer  and the  personnel  handling the

board are properly grounded while installing the board.

3.2 INSTALLATION

The following section details the installation procedure for the

Adaptec AHA-1640  Micro  Channel  to  SCSI  host  adapter.  The

installation of the board requires that the board be installed in

a 16-bit Micro Channel  slot. The appropriate  SCSI devices and

subsystems must then be  installed using a  shielded cable with

shielded connectors for external devices and a ribbon cable for

internal devices. Using the POS capability of the Micro Channel,

the proper installation parameters are set in the host computer

for transfer to the  host adapter during  the boot process. The

proper software must then be installed. From then on, full SCSI

operation is available.

CAUTION: The system must be  turned off during the installation

procedure.

3.2.1 System Requirements

The  host  adapter  must  be  installed  in  a  Micro  Channel

compatible host computer.  The proper  levels of  software and

software drivers  must  be  available.  The  System  Reference

Diskette for the  system must  have the  proper initialization

file present to allow host adapter initialization. The file is

named  @0F1F.ADF  and  is  available   from  Adaptec  for  OEM

customization. Using the  information provided in  Section 4.2

and knowing the proper  Adapter Description File  (ADF) format

described in the IBM Technical Reference manuals, a customized

@0F1F.ADF may also be prepared to meet special requirements.

3-1

Section Three                                                    Installation

------------------------------------------------------------------------------

3.2.2  Hardware Installation

The installation procedure of the SCSI Host Adapter/MC requires

configuring of  the host  adapter  terminators, inserting  the

board into a 16  bit connector, and connecting  the SCSI cable

from the  internal  or external  SCSI  connector  to the  SCSI

devices or subsystems.

To install the host adapter, you should follow the installation

steps in order.

1) Turn OFF the power to the computer system.

2) Configure  the  host  adapter  termination.  The  SCSI  bus

requires that there be only two terminators installed and that

the terminators must be placed at the farthest ends of the SCSI

bus. If the  host adapter  is neither  the first nor  the last

peripheral on the SCSI bus, the terminator resistor networks at

locations RN2 and RN3  must be removed from  the host adapter.

The terminator resistor networks  at RN2 and RN3  must also be

removed if the host adapter is the first or last peripheral on

the SCSI bus  and if  inline terminator modules  are connected

directly to the external SCSI connector of the host adapter.

3) Open the  computer system  according to  the manufacturer's

directions to expose the  adapter slots in  which the AHA-1640

will be placed.

4) If only an external SCSI subsystem is to be used, no internal

cabling is required. If  an internal SCSI peripheral  is to be

used, install  the supplied  50-pin ribbon  cable to  the host

adapter. This cable must  be oriented correctly. Pin  1 of the

SCSI cable is designated  by a red  stripe. Pin 1  on the host

adapter 50-pin SCSI header  is located on the  left hand side,

farthest from the installation bracket and is designated by the

words PIN  1  on  the  board  adjacent  to the  header.  After

locating pin  1 on  the host  adapter and  on the  SCSI cable,

carefully insert the connector located at the  end of the long

end of  the cable  in  to the  host  adapter connector.  After

ensuring that  all  pins  are lined  up  and  that  the pin  1

orientation is correct, firmly seat the connector to the board.

If it is ever necessary to remove the cable, gentle prying with

a small thin-bladed  screwdriver may  be required.  Be careful

not to bend or scar the header pins.

5) After installing  the SCSI cable,  the host adapter  can be

installed in any one of the 16-bit slots in the computer.

6) If internal SCSI peripherals are to be used, they should be

installed in the drive bays in  accordance with the directions

on the  peripherals.  The  50-pin  ribbon  cable  can  now  be

attached  to  the  SCSI  peripherals.   Please  refer  to  the

installation instructions to ensure proper pin 1 orientation of

the SCSI  peripherals. Pin  1 orientation  must  be consistent

throughout the system.

7) After installing the  host adapter and cabling  to the SCSI

peripheral, review the installation steps to  be sure that all

operations are correctly executed. Take  special care to check

that the side of the  SCSI cable with the solid  red stripe is

always  attached  to  pin  1  of  the  host  adapter  and  the

peripherals and that  the connector  is mated with  the header

correctly. Take special care  that the peripheral  devices are

properly fastened  into  the computer  and  that all  required

cabling is  completed. Be  sure  that the  system  is properly

terminated.

8) Carefully  reinstall  the  cover  on  the  computer  system

according to the manufacturer's instructions.

9) If an external SCSI subsystem is to be installed, it can now

be cabled to the  external SCSI connector  projecting from the

shielding bracket on the back of the SCSI Host Adapter/MC. The

proper shielded  SCSI  cable  must  be  used.  The  subsystem,

cables, and SCSI terminators  must be installed  in accordance

with the directions provided with the external SCSI subsystem.

3-2

Section Three                                                    Installation

------------------------------------------------------------------------------

3.2.3 Software Installation

The SCSI Host Adapter/MC requires initialization parameters to

be loaded when the host adapter is first installed or after the

system standby  battery has  been changed.  The  host adapter,

after the initialization parameters have once been loaded, will

automatically be initialized  with these parameters  each time

the system  performs a  boot operation.  The  System Reference

Diskette is restricted from write operations, so a copy must be

prepared if the parameters are to be backed up to a diskette.

The initialization parameters  are installed by  executing the

following steps.

A)     Prepare the back-up Reference Diskette.

1) Install the System Reference Diskette in floppy drive A.

2) Turn on the host computer system. The system will boot using

the  reference  diskette's  operating  system   and  the  host

system's BIOS.

3) After the boot is complete, the system Main Menu will appear.

Select the 'Back up  the reference diskette'  option and press

the ENTER key. The Back Up menu will appear.

4) Follow the instructions on the Back Up menu to complete the

diskette back up process. The instructions will require you to

select the  drive  in  which  the  original  System  Reference

Diskette will  be read  and  the drive  in which  the  back up

diskette will be written. If the same  drive is used for both,

you will be  given instructions to  swap the diskettes  at the

appropriate times. To  be sure that  there is enough  space to

include all the reference information on the back up diskette,

the back up diskette  should have no other  information on it.

If the  diskette  is unformatted,  the  Back  Up program  will

automatically format the diskette.

5) Remove both the  System Reference Diskette and  the back up

reference diskette and return to the Main Menu.

B) Set the initial parameters, save them in RAM, and back them

up to the back up reference diskette.

1) Install the back-up copy of the System Reference Diskette in

floppy drive A.

2) Boot by  pressing the  CONTROL, ALTERNATE, and  DELETE keys

simultaneously.  The  system  will  boot   using  the  back-up

reference diskette's  operating system  and the  host computer

system's BIOS.

3) After the boot is complete, the system Main Menu will appear.

Select the 'SET CONFIGURATION' option and press the ENTER key.

The Set Configuration menu will appear.

3-3

Section Three                                                    Installation

------------------------------------------------------------------------------

4) Press  the  proper  index  number  to  request the  'CHANGE

CONFIGURATION' option.  The  Change  Configuration  menu  will

appear, displaying the installed or  the default configuration

for each installed function or  adapter. The following options

may be changed:

TABLE 3-1

OPTION                        DEFAULT VALUE

-------------------------------------------

Port I/O Address                       330H

Arbitration Level                         1

Interrupt Level                          15

Adapter BIOS                DC000H, ENABLED

SCSI Device Identifier                    7

SCSI Parity Checking On/Off              ON

Synchronous Transfer Initiated by Host   NO

MCA Fairness On/Off                      ON

Each option may be  changed by stepping  through the available

choices  using  the  specified  function  key.  Help  text  is

available for each option to indicate how the choice should be

made.  The  default  values  should  be  used  unless  address

conflicts have been  identified by the  configuration program.

Detailed information about each option and setting is contained

in Section 4.2.

5) After the  desired choices  have been selected,  invoke the

'SAVE CHANGE' function by pressing  the required function key.

Those choices have now been assigned for the new host adapter.

6) Exit the CHANGE CONFIGURATION menu.

7) Using the  SET CONFIGURATION  menu, press the  proper index

number to  execute the  BACK UP  CONFIGURATION  function. This

stores the selected configuration on the floppy for future use.

8) Exit the SET CONFIGURATION menu to the Main Menu and remove

the copy of the System Reference Diskette.

9) The SCSI Host Adapter/MC is now ready for normal operation.

The selected operating system can now  be installed and booted

in accordance with the installation and operating instructions

for the operating system.

3-4

Section Three                                                    Installation

------------------------------------------------------------------------------

3.2.4 Common Problems during Installation

Problem                         Resolution

Host adapter cannot be          Review installation steps. Host adapter may be

found by configuration          inserted in slot incorrectly.

program.

Host adapter finds solid SCSI   Review SCSI cable installation. Pin 1 is

reset.                          likely to be incorrectly oriented. Cable may

be displaced in the SCSI connector.

If inline terminators are installed, terminator

fuse F1 may need replacement.

Host adapter cannot find        Review peripheralinstallation. Peripheral is

attached peripheral device.     likely to be powered off or to have

incorrect SCSI device ID.

Host adapter operates           Review SCSI cable installation and

inconsistently                  configuration. Likely to

have triple or missing terminators.

May have overlapping SCSI device ID.

Other problems.                 Refer to Section 8, Problem Determination

Contact technical assistance personnel where

the AHA-1640 was purchased.

3-5

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

This section  provides  detailed descriptions  of  the AHA-1640

software interface to the Micro Channel host software operating

in multithreaded mode.

4.1 HARDWARE OVERVIEW

There  are  three  groups  of   addressable  hardware  and  one

unaddressable set of  hardware that  are used  to implement the

software interface between  the AHA-1640 and  the Micro Channel

host.

[Picture]

Figure 4-1. Software Interface for Multithreaded Operation

The unaddressable hardware is the DMA control logic that allows

the host adapter access to the host system's memory. The memory

contains the critical software objects used by the host adapter,

including the Mailboxes, the Command Control Blocks, and the Data

areas. The definition of all these software objects is intended

to be upward  compatible with the  AHA-1540 PC AT  to SCSI host

adapter, so  that all  driver  programs that  operate  with the

AHA-1540A/1542A will also  operate with  the AHA-1640.  The DMA

control logic  on the  host adapter  manages the  Micro Channel

arbitration and data  transfer handshaking  necessary to access

these areas of the host memory. All memory addresses used by the

DMA control logic are real physical memory addresses, not virtual

addresses. Any conversions  between real  and virtual addresses

must be done by  the operating system  before the addresses are

provided to the host adapter. During DMA transfers, the AHA-1640

becomes a bus master. The DMA logic supports odd or even starting

addresses and odd  or even data  lengths. All  DMA transfers to

16-bit MCA addresses are  16-bit unless a  single first or last

byte  must  be  transferred  to  meet  the  odd  byte  boundary

requirements.

4-1

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

The POS registers are a set of addressable registers that can be

managed by the host CPU when the CDSETUP signal is active. These

registers are  normally  set up  by  the  system initialization

programming. The register addresses and register bit definitions

are described in Section 4.2.

The BIOS  EPROM is  available as  a  second set  of addressable

information. The BIOS  address range is  established by setting

the correct values in the POS registers at initialization time.

Once the  system has  been  properly initialized,  the programs

resident in the BIOS address space will automatically be executed

when required.  Normally,  they  are  used  only  during single

threaded operation and in initialization utilities.

The third set of addressable registers defined by the AHA-1640 is

the I/O  Port interface.  The registers  in this  address space

provide  a   simple   set  of   sense   information,  controls,

initialization parameters, and  interrupt information necessary

for the host  system to manage  the host  adapter. The register

assignments and functions are defined  in Section 4.3. They are

intended to be completely compatible with the AHA-1540A/1542A, so

that all driver programming  designed to run on  the PC AT host

adapter will operate correctly with the AHA-1640.

In addition to  these addressable  and non-addressable software

communication paths to the host adapter, the host adapter has an

interrupt path to the  host to request  services from the host.

The interrupt uses one of the standard interrupt channels of the

Micro  Channel.  The  interrupt  is  assigned  by  setting  the

appropriate bits in the POS Registers.

The SCSI port  is controlled by  the AIC-6250,  an Adaptec SCSI

protocol device  which  supports  arbitration,  selection,  and

reselection with a minimum of processor intervention. This VLSI

device also supports target mode and synchronous SCSI transfers.

4.2 POS REGISTER INTERFACE

The Programmable Option Select registers provide the functions to

replace the jumpers required by previous host adapters. The POS

registers provide the information necessary to define the address

ranges of the BIOS  EPROM and the I/O  Ports. The POS registers

also establish  certain hardware  states  required by  the host

adapter during  its  initialization. These  states  include the

enabling of the host adapter's hardware interface, the enabling

of arbitration fairness and the enabling of certain SCSI options.

In addition, a unique ID can be  read from the POS registers to

identify the installed card as an AHA-1640.

4-2

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

POS ADDRESS = 0100H

WRITE: Reserved, Do Not Write  READ: Adapter Identification, LSB = 1FH

Bit 7                        Bit 7   ID   Bit 7 = 0

Bit 6                        Bit 6    "   Bit 6 = 0

Bit 5                        Bit 5    "   Bit 5 = 0

Bit 4                        Bit 4    "   Bit 4 = 1

Bit 3                        Bit 3    "   Bit 3 = 1

Bit 2                        Bit 2    "   Bit 2 = 1

Bit 1                        Bit 1    "   Bit 1 = 1

Bit 0                        Bit 0    "   Bit 0 = 1

POS ADDRESS = 0101H

WRITE: Reserved, Do Not Write     READ: Adapter Identification, MSB= 0FH

Bit 7                        Bit 7   ID   Bit 7 = 0

Bit 6                        Bit 6    "   Bit 6 = 0

Bit 5                        Bit 5    "   Bit 5 = 0

Bit 4                        Bit 4    "   Bit 4 = 0

Bit 3                        Bit 3    "   Bit 3 = 1

Bit 2                        Bit 2    "   Bit 2 = 1

Bit 1                        Bit 1    "   Bit 1 = 1

Bit 0                        Bit 0    "   Bit 0 = 1

POS ADDRESS = 0102H

WRITE: POS Register 2        READ:  POS Register 2

Bit 7  BIOS Disable         Bit 7    BIOS Disable

Bit 6  Reserved (0)         Bit 6    Reserved (0)

Bit 5  Reserved (0)         Bit 5    Reserved (0)

Bit 4  Reserved (0)         Bit 4    Reserved (0)

Bit 3  Reserved (0)         Bit 3    Reserved (0)

Bit 2  Reserved (0)         Bit 2    Reserved (0)

Bit 1  Reserved (0)         Bit 1    Reserved (0)

Bit 0  AHA-1640 Card Enable Bit 0    AHA-1640 Card Enable

4-3

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

POS ADDRESS = 0103H

WRITE:  POS Register 3                 READ:  POS Register 3

Bit 7   I/O Port Address Bit 3        Bit 7  I/O Port Address Bit 3

Bit 6   I/O Port Address Bit 2        Bit 6  I/O Port Address Bit 2

Bit 5   BIOS Address Bit 16           Bit 5    BIOS Address Bit 16

Bit 4   BIOS Address Bit 15           Bit 4    BIOS Address Bit 15

Bit 3   BIOS Address Bit 14           Bit 3    BIOS Address Bit 14

Bit 2   I/O Port Address Bit 10       Bit 2  I/O Port Address Bit 10

Bit 1   I/O Port Address Bit 9        Bit 1  I/O Port Address Bit 9

Bit 0   I/O Port Address Bit 8        Bit 0  I/O Port Address Bit 8

POS ADDRESS = 0104H

WRITE:  POS Register 4                        READ:  POS Register 4

Bit 7     SCSI Device ID 22                  Bit 7 SCSI Device ID 22

Bit 6     SCSI Device ID 21                  Bit 6 SCSI Device ID 21

Bit 5     SCSI Device ID 20                  Bit 5 SCSI Device ID 20

Bit 4     SCSI Parity Enable                 Bit 4 SCSI Parity Enable

Bit 3     SCSI Sync Negotiation              Bit 3 SCSI Sync Negotiation

Bit 2     Interrupt Select 22                Bit 2 Interrupt Select 22

Bit 1     Interrupt Select 21                Bit 1 Interrupt Select 21

Bit 0     Interrupt Select 20                Bit 0 Interrupt Select 20

POS ADDRESS = 0105H

WRITE:  POS Register 5              READ:  POS Register 5

Bit 7     Channel Check Active     Bit 7      Channel Check Active

Bit 6     Reserved                 Bit 6      Reserved

Bit 5     Sync Ready (=1)          Bit 5      Sync Ready (=1)

Bit 4     Fairness Enabled         Bit 4      Fairness Enabled

Bit 3     Arbitration Level 23     Bit 3      Arbitration Level 23

Bit 2     Arbitration Level 22     Bit 2      Arbitration Level 22

Bit 1     Arbitration Level 21     Bit 1      Arbitration Level 21

Bit 0     Arbitration Level 20     Bit 0      Arbitration Level 20

4.2.1 POS Register 0

POS Register 0  contains the  least significant  byte of  the host adapter's

Adapter Identifier. Since  the identifier is  0F1F, the low  order byte will

contain 1FH. This register is a read only register.

4-4

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.2.2 POS Register 1

POS Register  1 contains  the most  significant byte  of the  host adapter's

Adapter Identifier. The high order byte is 0FH. This register is a read only

register.

4.2.3 POS Register 2

POS Register 2 is a read/write register.

BIOS Disable, bit 7  of the register, is  set to zero  by the reset process.

When zero, the Micro Channel can access the BIOS EPROM using the address space

specified by POS Register 3. If  the BIOS Disable bit is  1, accesses to the

EPROM address space will not provide data to the Micro Channel.

Card Enable, bit 0 of the register, is set to zero by the reset process. When

zero, the Micro Channel can only access  the POS registers of the card using

the CDSETUP signal and  the appropriate bus protocol.  Any other attempts to

access information  which would  normally  be in  the  address space  of the

AHA-1640 will  be ignored  and no  lines will  be driven  back to  the Micro

Channel. When one,  the Card Enable  bit allows the  AHA-1640 to participate

normally in Micro Channel interchanges.

4.2.4 POS Register 3

POS Register 3 is a read/write register. The bits in the register define I/O

Command Port address space and the BIOS address space in the following manner.

TABLE 4-1. I/O COMMAND PORT BASE ADDRESS

POS Register Bit   I/O Command Port Base Address

2 1 0 7 6

0 0 0 X X          Reserved

0 0 1 0 0          Port Address = 0130H (Allowed for BIOS)

0 0 1 0 1          Port Address = 0134H

0 0 1 1 X          Reserved

0 1 0 0 0          Port Address = 0230H (Allowed for BIOS)

0 1 0 0 1          Port Address = 0234H

0 1 0 1 X          Reserved

0 1 1 0 0          Port Address = 0330H (Allowed for BIOS)

0 1 1 0 1          Port Address = 0334H

0 1 1 1 X          Reserved

1 X X X X          Reserved

4-5

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Port Addresses 0130, 0230, and  0330 are recognized by  the BIOS as possible

host adapters which may have DOS system disks attached. The BIOS examines the

installed adapters in order from slot 0 on to the maximum installed slot. The

first host adapter encountered that has one  of the allowable port addresses

will be selected as the host adapter which may have DOS system disks attached.

All other host  adapters must have  appropriate device  drivers installed to

access their attached devices.

TABLE 4-2. BASE ADDRESS OF BIOS ADDRESS SPACE

POS Register Bit       BIOS Address Space Base Address

5 4 3

0 0 0                  Unavailable, Reserved for Video Memory

0 0 1                  BIOS Base Address = 0C4000H

0 1 0                  BIOS Base Address = 0C8000H

0 1 1                  BIOS Base Address = 0CC000H

1 0 0                  BIOS Base Address = 0D0000H

1 0 1                  BIOS Base Address = 0D4000H

1 1 0                  BIOS Base Address = 0D8000H

1 1 1                  BIOS Base Address = 0DC000H

4.2.5 POS Register 4

This POS Register is a read/write register.

The SCSI Device Identifier which will be used by the AHA-1640 for both Target

and Initiator Mode operation is defined by bits 7-5 of the register. The SCSI

Device Identifier  may take  on any  value from  7 to  0. Every  SCSI device

attached to the SCSI Bus must have a different identifier.

The SCSI Parity Enable bit is in bit 4  of the register. If the bit is zero,

SCSI parity is not checked. This is  required for certain systems containing

older SCSI devices that did not  support the Data Parity bit.  If the bit is

one, SCSI parity is verified  by the AHA-1640. This is  the normal state for

modern SCSI systems.

The SCSI Synchronous Negotiation bit is in bit 3 of the register. If the bit

is zero, the AHA-1640 will not initiate synchronous data transfer negotiation,

but will respond if an attached SCSI device initiates the negotiation. If the

bit is  one, the  AHA-1640  will both  initiate  negotiation and  respond to

negotiation. The AHA-1640 will initiate synchronous  data transfer if it has

been forced  to abandon  the result  of  previous synchronous  data transfer

negotiations. Events that will  force the AHA-1640  to abandon those results

include the host setting the HRST or  SRST bits in I/O Port Control Register

(Section 4.4.1), another SCSI device transmitting a  Bus Device Reset to the

host adapter, or a power on reset.

The Interrupt Selection bits define the interrupt  line that will be used to

communicate with the host microprocessor across  the Micro Channel. The high

order bit of the interrupt channel selection is not specified by the register

and is always one. The  result is that the  following selection algorithm is

used to choose the interrupt line to be used.

4-6

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

TABLE 4-3. INTERRUPT CHANNEL SELECTION

POS Register Bit       IRQ Channel to be used

2  1  0

0  0  0                Reserved

0  0  1                IRQ 9

0  1  0                IRQ 10

0  1  1                IRQ 11

1  0  0                IRQ 12

1  0  1                Reserved

1  1  0                IRQ 14

1  1  1                IRQ 15

4.2.6 POS Register 5

This POS Register is a read/write register.

The Channel Check Active bit of POS Register  5 is used to indicate that the

AHA-1640 has detected a hardware failure. All the processor detectable errors

are presented through  the I/O  port interface  or through  the multitasking

interface. Channel Check  Active indicates that  the processor  is no longer

providing inputs to the watch dog timer and is inoperable.

The Channel Check Status bit is not implemented in the AHA-1640.

The Sync Ready bit should always be set to one.

The Fairness  bit  controls  the Micro  Channel  access  protocol.  When the

Fairness bit is set to one, all other attached adapters are granted access to

the Micro Channel  after the  AHA-1640 has  released the  Micro Channel. The

AHA-1640 can only regain control of the Micro Channel after no other adapters

request it. When the Fairness bit is  set to zero, the AHA-1640 will release

the Micro Channel only when a higher priority adapter preempts it.

The Arbitration Level bits indicate the arbitration level that will be used to

gain access to the  Micro Channel by the  AHA-1640's internal DMA circuitry.

The host adapter will arbitrate as a Bus Master, and once the arbitration is

successful, it will manage the required data  transfers until preempted by a

higher priority Micro Channel function  or until the burst  of data has been

transferred. Arbitration levels  from 1  to 15  can be  selected by  the POS

Register.

4.3 PREPARATION OF MICRO CHANNEL DMA HARDWARE

The Micro Channel architecture provides a central DMA controller with one DMA

channel for each arbitration  level. The BIOS normally  sets up the required

DMA channels as each operation is requested. The AHA-1640 provides DMA Master

operation to achieve higher data rates with less contention for Micro Channel

bandwidth. To allow DMA Master operation, the central DMA controller channel

corresponding to the AHA-1640's  selected arbitration level  must be masked.

This is normally  done automatically  by the  on-board BIOS  during the boot

process so that the host adapter can begin normal Bus Master operation with no

additional  program  initialization.  If  other  programs  have  incorrectly

modified the DMA setup, the DMA functions can be restored to the proper masked

condition using either  the 8237  compatibility mode  or using  DMA extended

operations. Both mechanisms are shown in the following sections.

4-7

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.3.1 8237 Compatibility Mode

The DMA controller is  normally initialized to  the correct masked condition

during the boot process. If the DMA controller must be restored to the masked

condition, the programmer  can set  the proper  control bits  using the 8237

compatibility mode.

The DMA controller can be masked using  the Mask Register (Set/Clear) if the

program has only information about the single DMA channel. If the program has

information about all the DMA channels, the  Write Mask Register can be used

to set four of the DMA channels to the correct masked or unmasked state at the

same time. All  system microprocessor access  to the DMA  controller must be

with 8-bit I/O instructions.

I/O Register Address   Mask Register

000A                Channel 0-3, Mask Register (Set/Clear)

000F                Channel 0-3, Write Mask Register

00D4                Channel 4-7, Mask Register (Set/Clear)

00DE                Channel 4-7, Write Mask Register

Definition of bits for Mask Register (Set/Reset):

Register Bit              Bit Definition

7-3                  Reserved = 0

2                   if 0, Clear Mask Bit

if 1, Set Mask Bit

1,0                  if 00, Set/Clear Channel 0 or 4

if 01, Set/Clear Channel 1 or 5

if 10, Set/Clear Channel 2 or 6

if 11, Set/Clear Channel 3 or 7

4-8

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Definition of bits for Write Mask Register:

Register Bit              Bit Definition

7-4                  Reserved = 0

3                   if 0, Unmask Channel 3 or 7

if 1, Mask Channel 3 or 7

2                   if 0, Unmask Channel 2 or 6

if 1, Mask Channel 2 or 6

1                   if 0, Unmask Channel 1 or 5

if 1, Mask Channel 1 or 5

0                   if 0, Unmask Channel 0 or 4

if 1, Mask Channel 0 or 4

The following operation  is an example  demonstrating how to  mask Channel 2

using the Mask Register (Set/Clear) Addresses:

Set I/O Address 000A to the value 06H

The following operation is an example demonstrating how to mask Channel 2 and

unmask Channels 0, 1, and 3:

Set I/O Address 000F to the value 02H

4.3.2 DMA Extended Mode

The DMA controller  channels can  be masked using  the extended  mode of DMA

control. In this case,  I/O Address 0018  is used as a  function register to

specify the operation to be  performed. I/O Address 001A  is used to provide

additional parameters for some specified functions. None of the Mask Register

control functions require the use of I/O Address 001A.

Definition of bits for mask control functions executed by setting I/O Address

0018:

Register Bit             Bit Definition

7-4                 Program Command

if 9H, Set Specified Mask Bit

if AH, Clear Specified Mask Bit

if DH, Clear All Mask Bits

3                   Reserved = 0

2-0                 Channel number to which Program Command applies

4-9

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

The following operation is an example of setting the mask bit for channel 2 to

1 using the Extended DMA Mode:

Set I/O Address 0018 to 92H

4.4 I/O PORT INTERFACE

Three port addresses are decoded  in the Micro Channel  I/O address space to

provide access to the I/O  Port registers. The architecture  of the I/O Port

registers is fully compatible  with the AHA-1540A/1542A.  The port registers

form the primary communications channel between the host and the host adapter.

The I/O ports are 8 bits wide. The  base port is for control and status. The

second port  passes command  and data  parameters.  The third  port presents

interrupt information. The port structure is summarized in the following table

and described in detail in the following sections.

The base  port address  is  established by  the  proper setting  of  the POS

registers. The base port address may be set to 130H, 134H, 230H, 234H, 330H,

or 334H.

I/O PORT INTERFACE BIT DEFINITION

PORT ADDRESS = BASE + 0

WRITE: Control Register              READ:  Status Register

Bit 7  Hard Reset (HRST)       Bit 7   Self Test in Progress (STST)

Bit 6  Soft Reset (SRST)       Bit 6   Internal Diagnostic Failure (DIAGF)

Bit 5  Interrupt Reset (IRST)  Bit 5   Mailbox  Initialization Required(INIT)

Bit 4  SCSI Bus Reset (SCRST)  Bit 4   SCSI Host Adapter Idle (IDLE)

Bit 3  Reserved (0)            Bit 3   Command/Data Out Port Full (CDF)

Bit 2  Reserved (0)            Bit 2   Data In Port Full (DF)

Bit 1  Reserved (0)            Bit 1   Reserved ( Undefined )

Bit 0  Reserved (0)            Bit 0   Invalid H A Command (INVDCMD)

PORT ADDRESS = BASE + 1

WRITE: Command / Data Out              READ: Data In

Bit 7     Command/Data Out  Bit 7   Bit 7   Data In Bit 7

Bit 6            "          Bit 6   Bit 6      "    Bit 6

Bit 5            "          Bit 5   Bit 5      "    Bit 5

Bit 4            "          Bit 4   Bit 4      "    Bit 4

Bit 3            "          Bit 3   Bit 3      "    Bit 3

Bit 2            "          Bit 2   Bit 2      "    Bit 2

Bit 1            "          Bit 1   Bit 1      "    Bit 1

Bit 0            "          Bit 0   Bit 0      "    Bit 0

4-10

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

PORT ADDRESS = BASE + 2

WRITE: Reserved, do not write        READ: Interrupt Flags

Bit 7                              Bit 7   Any Interrupt

Bit 6                              Bit 6   Reserved

Bit 5                              Bit 5   Reserved

Bit 4                              Bit 4   Reserved

Bit 3                              Bit 3   SCSI Reset Detected (SCRD)

Bit 2                              Bit 2   HA Command Complete (HACC)

Bit 1                              Bit 1   MBO Empty (MBOE)

Bit 0                              Bit 0   MBI Full (MBIF)

4.4.1 Control and Status Port

Writing a one to the bits of the Control Port initiates certain special host

adapter operations. There is no  requirement to return the  bits to the zero

state, since they are  reset automatically after  the requested operation is

initiated. Read operations  to the Status  port address  return host adapter

status information.

Base+0 Port, Write: Host Adapter Control Port.

Bit 7 Hard  Reset (HRST):  The setting  of the  Hard Reset bit  to one

forces the host adapter into a state identical to a normal power

on state. Diagnostic functions  are executed and  all status for

ongoing SCSI operations is lost.  A Reset Condition is generated

on the SCSI  bus. While the  reset is being  processed, the Self

Testing in Progress bit (Host Adapter Status Port bit 7) is set.

When the reset  is complete, that  bit is reset  and the Mailbox

Initialization Required bit (Host Adapter Status Port bit 5) and

the SCSI Host Adapter Idle bit (Host  Adapter Status Port bit 4)

are set, indicating that the  AHA-1640 mailbox structure must be

re-initialized and that no other operations are active on the host

adapter. See Section 4.5, describing the overall reset structure.

Bit 6 Soft Reset  (SRST): The Soft  Reset bit clears  all ongoing SCSI

and host  adapter  commands.  All  Command  Control  Blocks  are

abandoned, all queued commands are abandoned, and all Mailbox In

and Mailbox Out entries are cleared. No diagnostic functions are

executed. No Reset Condition  is generated on  the SCSI bus. The

SCSI Host  Adapter Idle  bit (Host  Adapter  Status Port  bit 4)

remains off until the reset processing is completed, at which time

it is  turned  on  to  indicate  that  normal  operation  may be

restarted. All  parameters  established  by  I/O  Port Commands,

including transfer speed and selection time-out, remain unchanged

by the Soft Reset  processing. See Section  4.5, which describes

the overall reset structure.

Bit 5  Interrupt Reset  (IRST):  The setting  of  this bit  clears the

interrupt port of  all bits  that have  been set  and resets the

interrupt line. The host adapter  prevents interrupts from being

lost by setting new interrupts only after previous interrupts have

been reset. The host adapter will not raise new interrupts until

the previous interrupt information has been accepted and cleared.

If an  interrupt needs  to be  presented  while an  interrupt is

already active, the host adapter will save the interrupt for later

presentation.

4-11

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Bit 4 SCSI Bus  Reset (SCRST): The  setting of this bit  causes a SCSI

Bus Reset to be generated on the SCSI bus. The SCSI Bus Reset is

triggered at the time the SCRST bit is set to one and raises the

RST line  on the  SCSI Bus  for  the architected  25 microsecond

period. The reset is managed as a SCSI Soft Reset and will allow

partially completed operations to continue after the reset occurs.

See Section 4.5, describing the overall reset structure.

Bits 0-3  Reserved:  Reserved  bits  must  be  set  to  zero to  avoid

compatibility problems  with  future extensions  of  the Control

Register.

Base+0 Port, Read: Host Adapter Status Port

Bit 7 Self Testing  in progress (STST): This  bit, when one, indicates

that the  host  adapter  is  performing  self-initialization and

internal diagnostics. The  bit is  asserted after  a power-on or

hard reset  (Control  Port  Bit 7  HRST  =  1).  When diagnostic

operation is complete, the STST bit is  set to zero and bit 5 or

bit 6 is set to indicate the successful or unsuccessful completion

of the diagnostics. If  bit 7 remains on,  it indicates that the

initialization or diagnostic  could not be  completed. The error

condition can be determined  as described in  Section 8. In most

cases, bit 6  (DIAGF) will be  set to indicate  that an internal

diagnostic failure occurred.

Bit 6  Internal  Diagnostic  Failure  (DIAGF):  This  bit,  when  one,

indicates that the self-testing process has completed and that an

error was detected. The host adapter must be reset by setting the

Hard Reset bit (bit 7 of the Control Port) to clear the error. If

the AHA-1640 again detects an error, diagnostic utilities must be

performed to identify  the error  condition. The  diagnostic LED

will present  a  flash code  that  indicates the  nature  of the

failure. Section 8 describes the corrective procedures.

Bit 5  Mailbox Initialization  Required  (INIT): This  bit,  when one,

indicates that the self-testing process has completed successfully

and that the AHA-1640 is ready  for mailbox initialization to be

performed. The base memory  address of the  Mailbox area must be

established by execution of  the Mailbox Initialization command.

After execution of  the Mailbox  Initialization command  and any

other desired initialization  operations, the  AHA-1640 is ready

for full operation.

Bit 4 SCSI  Host Adapter  Idle (IDLE):  This bit,  when one, indicates

that the host adapter is in the idle state. The host adapter has

no outstanding I/O Command Port (IOCP) commands or SCSI commands.

The host processor must wait for the idle state before executing

any IOCP command except the Start SCSI (02) and Enable Mailbox Out

Interrupt (05) commands.

Bit 3 Command /  Data Out Port Full  (CDF): The host  uses the CDF bit

to synchronize command and data transfers to the host adapter. An

IOCP command byte or an outbound parameter byte can be placed in

the Command / Data Out Port when the port is empty, indicated by

the CDF bit being zero.  When a byte is placed  in the Command /

Data Out Port, the CDF bit is set to one and remains one until the

host adapter has obtained  and processed the  byte. When the CDF

bit returns to zero,  the next Command or  parameter byte can be

placed in the port.

4-12

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Bit 2 Data In Port Full (DF): The  host uses the DF bit to synchronize

transfers of data from the host adapter to the host. When the DF

bit is set to one, the host adapter has placed a byte in the Data

In Port  for  the host  to  remove  and process.  When  the host

performs a read to the  Data In Port address, the  DF bit is set

back to zero automatically and not set  to one again until a new

data byte has been placed in the Data In Port by the host adapter

for the host.

Bit 1   Reserved:  This bit is zero.

Bit 0  Invalid  Host  Adapter  Command  (INVDCMD):  The  Invalid  Host

Adapter Command  bit is  set  to one  if  an invalid  command or

parameter byte was  received in  the Command  / Data  Out Port .

After sending a  command byte  or data  byte, the  host software

determines that the next byte is ready to send by waiting for the

CDF bit to be  reset. If the  command byte or  parameter byte is

not valid, the command sequence will instead be terminated by the

host adapter. The  host adapter  always terminates  a command by

raising the Host  Adapter Command Complete  (HACC) interrupt. If

the HACC interrupt is set to one and the INVDCMD bit is not set,

the command terminated normally. If the  INVDCMD bit is also set

to one, the  command or  parameter bytes  were determined  to be

invalid and the command terminated abnormally. The INVDCMD bit is

only valid from the time the HACC interrupt is set until the HACC

interrupt is reset. The  bit's value is  not predictable until a

new HACC interrupt is set for a new IOCP command.

4.4.2 Command / Data Out and Data In Port

The second I/O port address is used by  the host to write IOCP command bytes

and accompanying parameter data bytes to the host adapter. It is also used by

the host  adapter  to send  parameters  back to  the  host to  be  read. The

Command/Data Out Port is used by the host to send host adapter initialization

and management commands and  parameters that cannot be  sent by the standard

Mailbox protocol. Information requested  by the IOCP  commands placed in the

Command/Data Out Port is returned through the  Data In Port. The host should

understand the format and number of bytes to be transmitted for each command

so that extra invalid  bytes are not  passed across the  interface. Bytes in

addition to  those  required by  the  particular  command are  likely  to be

interpreted as invalid, although they may instead cause the execution of valid

commands that were not supposed to be performed.

The host should only write to the Command/Data Out Port when CDF is zero. This

allows time for the host adapter to  process a previously written command or

parameter byte. CDF is automatically set to  one when the host writes to the

port and is reset to zero  after the host adapter removes  the byte from the

port.

If an IOCP command needs additional data  bytes, the host waits until CDF is

zero before writing the additional bytes to the Command/Data Out Port. Just as

in the command transfer case, each parameter  byte written will set the CDF.

The host can write additional  data bytes only after CDF  is again zero. The

HACC interrupt will  indicate when the  command has  terminated, normally or

abnormally. If INVDCMD is also set, the host adapter found either the command

or data bytes to be invalid and terminated abnormally.

If an IOCP command  requires data transfer  from the host  adapter, the host

adapter will place the  data bytes in  the Data In  Port and set  the DF bit

(Status Port bit 2) to indicate that the requested parameter is ready for the

host to read. When the host reads  the Data port, DF is automatically reset.

The host should wait until DF is again set before attempting to transfer the

next parameter byte. After the last data byte has been transferred, the HACC

interrupt bit will be set indicating command completion. If the IOCP command

was invalid, the HACC interrupt  will occur before all  data bytes have been

transmitted and the INVDCMD bit will be set.

4-13

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.4.3 Interrupt Flag Port

The Interrupt  Flag Port  contains  bits that  indicate  the reason  that an

interrupt was provided to the  host from the host  adapter. The host adapter

uses the interrupt  to notify the  host that  the host adapter  is ready for

immediate service from the host. The Interrupt Flag Port is a read-only port.

When an interrupt bit is set  by the host adapter to  indicate that the host

should respond, the Any Interrupt  bit and the interrupt  line are both also

set. When the host begins to examine the returned registers and mailboxes to

determine the cause of the interrupt and to perform the operations needed to

service the interrupt, the host  will first read the  Interrupt Flag Port to

record which  interrupts must  be  serviced. The  host  will then  clear the

interrupts by setting the IRST  bit (Host Adapter Control  Port bit 5). Once

the host adapter has set the Interrupt Flag Port to indicate that one or more

interrupts must be processed, it will not set the interrupt bits again until

after the Interrupt Flag  Port has been  reset. This prevents  the host from

accidentally resetting any interrupts that have been set after it has read the

Interrupt Flag Port.  Other reset operations  will also  reset the Interrupt

Flag Port and the interrupt  line, including the Hard  Reset bit (HRST), the

Soft Reset bit (SRST), and the power on reset.

Base+2 Port, Read only:  Interrupt Flag Port

Bit  7  Any  Interrupt:  This  bit,   when  one,  indicates  that  the

interrupt to the host has  been established. The actual identity

of the interrupting condition is provided in bits 0 to 3.

Bit 6   Reserved:     Returned as zero.

Bit 5   Reserved:     Returned as zero.

Bit 4   Reserved:     Returned as zero.

Bit 3  SCSI  Reset  Detected (SCRD):  This  bit,  when  one, indicates

that a SCSI  Reset has  been received on  the SCSI  bus. The Any

Interrupt bit and the Micro Channel  interrupt will also be set.

The host adapter supports SCSI Soft Reset (see Section 4.5). Any

ongoing target  or initiator  activities will  continue normally

after first  clearing the  SCSI bus.  In  some rare  cases, host

intervention will be required to restart a SCSI command that was

aborted by the  reset operation. The  host can  convert the SCSI

Soft Reset to a SCSI Hard Reset  by setting the Control Register

Soft Reset (Bit 6) to one, clearing all ongoing operations in the

host adapter. In  this case,  the host  must recognize  that any

operations not yet  completed will  never be  completed and must

perform appropriate error recovery  operations. See Section 4.5,

which describes the overall reset protocol.  The SCRD bit is not

set for host initiated SCSI Reset conditions caused by the setting

of the HRST bit or the SCRST bit, since the host is already aware

of the actions it has  requested and does not  need to have them

indicated again. If any interrupt flags have not yet been cleared

by the  host, no  new interrupts,  including  the SCRD,  will be

generated until the interrupts are reset.

Bit 2  Host  Adapter  Command Complete  (HACC):  This  bit,  when one,

indicates that an IOCP  command has been  completed, normally or

abnormally. The Any Interrupt bit and the Micro Channel interrupt

will also be set. If the command was completed normally, only the

HACC bit will be on. If the  command was completed abnormally or

was aborted before it was completed, the HACC bit will be one and

the Invalid Command Bit (Status Register bit 5) will also be one.

4-14

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

During parameter transfers to or from the host adapter, the HACC

bit should be examined to verify that the command is still being

processed and has  not been  ended abnormally.  If any interrupt

flags have not yet been cleared by  the host, no new interrupts,

including the HACC, will  be generated until  the interrupts are

reset.

Bit 1  Mailbox Out  Available (MBOA):  This  bit, when  one, indicates

that an outbound Mailbox  entry is now available  for use by the

host. The Any Interrupt bit and the Micro Channel interrupt will

also be set.  Most operating systems  will choose  to leave this

interrupt disabled to avoid the  generation of extra interrupts.

The host adapter will normally empty  Mailbox Out entries to its

local RAM so rapidly that round-robin filling of the Mailbox Out

entries will assure that a Mailbox Out entry will already be empty

by the time that the host is ready to fill it again.

If the host finds that all Mailbox Out entries are full, it can

enable the Mailbox Out Available interrupt by executing an Enable

Mailbox Out Interrupt command through  the I/O Command Port. The

Enable Mailbox Out Interrupt command is  one of the two commands

that can be executed  without waiting for the  IDLE state of the

host adapter. As soon as any Mailbox Out entry is cleared by the

host adapter, an MBOA interrupt will be generated to indicate to

the host that  an MBO entry  is available. An  MBOA interrupt is

generated after that each time a Mailbox Out entry is cleared by

the host adapter until an Enable Mailbox Out Interrupt command is

executed to force the reporting of MBOA interrupts to be disabled.

If any interrupt flags have not yet been cleared by the host, no

new interrupts, including the MBOA  interrupt, will be generated

until the interrupts are reset.

Bit 0 Mailbox In  Full (MBIF): This  bit, when one,  indicates that an

entry has been placed by the host adapter in the Mailbox In. The

interrupt should  be  reset  as soon  as  possible  so  that any

subsequent interrupts  can  be detected.  The  host  adapter may

return information in other Mailbox In entries, so the host should

check the  next  entry to  determine  if  more than  one  set of

information has  been provided.  The MBI  entries are  filled in

round-robin order, so the host should  simply check the next MBI

entry after the last one that was found when a new MBIF interrupt

occurs.

Once an interrupt of any  kind has been  set, a new interrupt

will not be set until the  original interrupt flags are cleared.

If an MBIF interrupt is set and other Mailbox In entries are made

before the interrupt  is cleared,  then all  the entries  can be

scanned as found. The new MBIF interrupt will not be generated if

the old  interrupts  have  not  yet  been  cleared.  Thus  it is

important to clear the MBIF interrupt  promptly so that the host

can complete the servicing of MBI entries and the MBIF interrupt

flag can then be  re-examined to determine  if other MBI entries

have been generated after those that have already been serviced.

4.5 RESET OVERVIEW

The reset  functions provided  by the  AHA-1640 are  extensive to  allow the

fullest flexibility and architectural consistency, both with SCSI and with the

Micro Channel.

4-15

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.5.1 Hard Reset operations

Resets may be generated by the hardware  through two mechanisms. The CHRESET

signal from the Micro Channel  is generated by the system  board to reset or

initialize all installed adapters upon power on or during a low line voltage

condition. The  system  can also  activate  this signal  under  host program

control. In addition to the CHRESET signal, the setting of the HRST bit (bit

7 of  the Host  Adapter Control  Port) will  force a  hardware reset  to the

AHA-1640. Regardless of the source, a Hard Reset forces the following actions

on the AHA-1640.

� All  internal  registers  of  the AHA-1640  are  returned  to  their reset

condition.

� The  host adapter's  microprocessor returns  all internal  information and

parameters to their initial state.

� The host adapter's microprocessor reloads the microprocessor program RAM.

� The host adapter performs all required internal diagnostics.

� A standard SCSI  Reset condition is  generated to all  other attached SCSI

devices.

During the Hard Reset process, the AHA-1640 will indicate to the attached host

that self-testing is in process by raising  the STST bit in the Host Adapter

Status Port. After  the Hard  Reset process  is complete,  the AHA-1640 will

indicate that initialization parameters are required from the host by raising

the INIT  bit in  the Host  Adapter  Status Port.  Since the  host adapter's

microprocessor must initialize the read only POS  registers, the read of the

host adapter's card ID cannot be executed until several microseconds after the

Hard Reset condition  has ended and  firmware initialization  has begun. The

system initialization functions take long enough so that the host adapter has

plenty of time to initialize the host adapter card ID.

4.5.2 SCSI Reset Operations

The SCSI Reset condition is defined in the SCSI Standard, X3.131-1986, Section

5.2.2. A SCSI Reset condition may  be forced by any SCSI  device on the bus.

The condition is forced by the assertion of the SCSI Reset signal.

The AHA-1640 has three mechanisms which may force a SCSI Reset condition. The

SCSI Reset condition may be  invoked from the host  software if the software

sets the SCRST bit (Bit 4  of the Host Adapter Control  Port). In this case,

the normal SCSI Reset operations will be performed by the Host Adapter and the

SCSI Reset signal will be asserted  on the SCSI bus. In  this case, the SCRD

bit (Interrupt Flag Port bit 3) will not be set, since the host itself caused

the reset.

The SCSI  Reset condition  may be  invoked by  the AHA-1640  as part  of the

recovery mechanism  for a  bus  phase error.  Bus  phase errors  may include

detection of  an  invalid  information transfer  phase  or  detection  of an

impossible phase  sequence (Command  Phase after  a Data  Phase in  the same

command). In this case,  the normal SCSI Reset  operations will be performed

and the SCSI Reset signal will be asserted on the SCSI bus. In addition, the

SCRD bit (Interrupt  Flag Port bit  3) will be  set to indicate  to the host

computer that a SCSI Reset  condition occurred. The setting  of the SCRD bit

also causes  the Any  Interrupt  bit (Interrupt  Flag  Port bit  7)  and the

appropriate Micro Channel interrupt to be presented.

The SCSI Reset condition may be invoked by another attached SCSI device to the

AHA-1640 as part of the other  device's recovery mechanism or initialization

procedure. In this case, the normal SCSI Reset operations will be performed.

In addition, the SCRD bit (Interrupt Flag Port bit 3) will be set to indicate

to the host computer that a SCSI Reset condition occurred. The setting of the

SCRD bit also causes the Any Interrupt bit (Interrupt Flag Port bit 7) and the

appropriate Micro Channel interrupt to be presented.

4-16

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.5.2.1 SCSI "Soft" Reset Option

The SCSI standard indicates two optional methods of handling the normal SCSI

Reset operations. The AHA-1640 implements the "Soft" Reset Option, described

in Section 5.2.2.2  of the  SCSI specification.  The "Soft"  Reset Option is

designed to allow a SCSI Reset signal to correctly clear the SCSI bus, but to

allow ongoing system operations to continue without any major interruptions.

When the SCSI Reset occurs, any activity on the SCSI bus is immediately halted

and all bus lines are cleared from  the bus. After the Reset condition ends,

any operations in progress are again allowed to start up in the normal manner.

No status or pointer information is destroyed. All disconnected commands are

allowed to reselect  and continue  in the  normal manner.  This "Soft" Reset

function allows a  multiple initiator system  to use reset  to clear certain

types of bus failures without damaging ongoing tasks from any initiator.

The SCSI "Soft" Reset  option is useful in  multitasking systems that cannot

tolerate the overhead of a complex reconfiguration and reinitialization after

a normal  reset  operation. For  the  SCSI  "Soft" Reset  option  to operate

correctly, all SCSI devices that communicate on the SCSI bus must support the

"Soft" Reset option. If any SCSI devices support the "Hard" Reset option, it

is likely that operations will be terminated without warning, and the system

will have to time out and monitor the requirement to restart some activities.

4.5.2.2 SCSI "Hard" Reset Option

The "Hard" Reset  Option is designed  to restore all  attached SCSI devices,

including both hosts and peripheral devices, to  their power on reset state.

All system activities that  have not been recorded  on a non-volatile memory

device or through another SCSI path are completely lost and must be restarted.

The system must be  completely re-initialized. For  certain types of systems

that  frequently  do  back-up  or   check  point  their  transactions,  that

re-initialize quickly and easily, or that infrequently do resets, the "Hard"

Reset Option is appropriate. The AHA-1640 responds to a SCSI Reset Condition

by executing only the "Soft" Reset Option, but it notifies the host whenever a

SCSI Reset condition has been established by causing an interrupt to the host.

The host then has the option of converting the "Soft" Reset to a "Hard" Reset

by forcing the host adapter to clear all the ongoing operations and return to

its initial state. The host requests this by  raising the SRST bit (bit 6 of

the Host Adapter Control Port). The host  must raise the SRST bit within 300

microseconds to disable the restarting of operations according to the rules of

"Soft" Reset. The raising of the SRST bit causes the host adapter to abandon

all CCB's and  prepare itself to  begin accepting new  instructions from the

host.  No  secondary   SCSI  Reset   signals  are   activated.  The  Mailbox

initialization and all normal  SCSI initial conditions are  not reset by the

SRST bit, so that system rebooting can be completed changing only that minimum

set of parameters necessary to restart the system. Of course the system still

has the right at any time that the IDLE bit (bit 4 of the Host Adapter Status

Port) is on to execute any of the IOCP commands and modify the Mailbox Address

or the SCSI initial conditions.

If the host requires that the SCSI be reset according to the SCSI "Hard" Reset

option, the host  raises both the  SRST and  the SCSIRST bits  with the same

instruction. The host adapter  will then set  a SCSI Reset  Condition on the

SCSI Bus and clear all its CCB and status information, thus performing a SCSI

"Hard" Reset with a single load to the Host Adapter Control Port.

4-17

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.6 I/O COMMAND PORT (IOCP) COMMAND SET

The AHA-1640 supports  low-level types of  commands through  the I/O Command

Port. The commands support host adapter initialization, performance parameter

adjustment, and special commands used by the BIOS while emulating the standard

hard disk controller. The IOCP command structure is fully compatible with the

AHA-1540. IOCP commands are executed by placing the appropriate command byte

in the Command/Data Out port. Parameters are then transferred out to the host

adapter or into the host to provide the information necessary to complete the

command. When the  command is completed,  either normally  or abnormally, an

HACC interrupt is  generated in  most cases. Most  IOCP commands  can not be

issued unless the host adapter is IDLE.

Below is a table of the IOCP command operation codes.

TABLE 4-4. I/O Command Port (IOCP) command Operation Codes

Operation Code               Operation

00               No Operation

01               Mailbox Initialization

02               Start SCSI Command *

03               Start Micro Channel BIOS Command

04               Adapter Inquiry

05               Enable Mailbox Out Available Interrupt *

06               Set SCSI Selection Time out

07               Set Bus-On Time

08               Set Bus-Off Time

09               Set Transfer Speed

0A               Return Installed Devices

0B               Return Configuration Data

0C               Enable Target Mode

1A               Write Channel 2 Buffer

1B               Read Channel 2 Buffer

1C               Write Adapter FIFO Buffer

1D               Read Adapter FIFO Buffer

1F               ECHO Command Data

All IOCP commands  except Start SCSI  (02) and  Enable Mailbox-Out Available

Interrupt (05) must be executed only when the IDLE bit (Status bit 4) is one.

Many commands require additional parameter bytes which are then written to the

Command / Data Out I/O port  (base + 1). Before each  byte is written by the

host to the host adapter, the host must verify that the CDF bit (Status bit 3)

is zero,  indicating that  the command  port  is ready  for another  byte of

information. Some commands require information bytes to be returned from the

host adapter to the host. In this case, the host monitors the DF bit (Status

bit 2) to determine when the host adapter has placed a byte in the Data In I/O

port for the host to read.  The DF bit is reset  automatically when the host

reads the byte.

4-18

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.6.1 No Operation (Operation Code 00)

No host adapter action  is taken, but  the HACC interrupt  is set indicating

command completion. No additional information bytes are exchanged.

4.6.2 Mailbox Initialization (Operation Code 01)

This command is used to specify the  number of mailbox locations used by the

host adapter and to specify the base memory location of the mailbox area. The

host adapter requires  that four bytes  of outbound data  follow the command

byte. The definition of those four bytes is shown below.

00         Mailbox count    (>0)

01      Mailbox address     (MSB)

02      Mailbox address

03      Mailbox address     (LSB)

Byte  0      Mailbox Count:  The Mailbox count must be greater than zero.

Byte 1,2,3   Mailbox Address:  The  Mailbox address  specifies the first

byte of the Mailbox area. Byte 1 is

the most significant byte.

When the Mailbox Initialization command and parameters are received, the host

adapter will then assume that a number of Mailbox Out entries and a number of

Mailbox In entries  will be assigned  beginning at the  Mailbox address. The

Mailbox count gives the number of each kind of mailbox entry to be used. The

total number of bytes reserved for the  Mailbox area will be eight times the

Mailbox count.

At command completion, HACC will be set to one and INIT will be reset to zero.

HACC will be reset as specified by Section 4.3.3.

4.6.3 Start SCSI Command (Operation Code 02)

This command indicates that the host has made at least one Mailbox Out entry

and that the host adapter should begin  to scan for active MBO entries. Once

scanning has  been started,  it continues  until all  MBO entries  have been

serviced, either by  beginning the requested  operations or  by queueing the

activities for  later  execution. Since  it  is  not easy  for  the  host to

determine that scanning is still taking place, the host should normally issue

this command  every time  a Mailbox  Out  is filled.  This command  does not

require additional data bytes. To avoid  unnecessary interrupts, HACC is NOT

set after command completion. HACC will be set with INVDCMD if the mailbox was

not yet initialized.

4.6.4 Start BIOS Command (Operation Code 03)

This command is used by the Adaptec BIOS to communicate with the host adapter

firmware. The command is  not available for  use by programs  other than the

Adaptec BIOS.

4-19

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

[This page intentionally left blank]

4-20

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.6.5 Adapter Inquiry (Operation Code 04)

After receiving this  command, the host  adapter returns four  bytes of data

describing the host  adapter firmware  revision level.  After completing the

transfer of  the four  bytes  of inbound  data,  the HACC  interrupt  is set

indicating normal  command  completion.  The  bytes  contain  the  following

information:

00      Board Identification

01      Special Options Identification

02      Hardware Revision Level

03      Firmware Revision (ASCII)

Byte 0  Board ID:             The value in this byte allows software

supported on both the PC AT and on the

Micro Channel to distinguish the type of

supporting host adapter.

Value                 Meaning

00H                   Board is an AHA-1540

41H ('A' ASCII)       Board is an AHA-1540A

42H ('B' ASCII)       Board is an AHA-1640

All Others            Reserved

Byte 1  Special Options ID:   The value in this byte indicates what

special options are supported on the

AHA-1640 host adapter. Other host

adapters use other values in this byte.

Value                 Meaning

41H ('A' ASCII)       Board is standard model

All Others            Reserved

Byte 2  Hardware Revision:    The value in this byte indicates what

hardware level of the AHA-1640 board is

provided. Other host adapters use other

values in this byte.

Value                 Meaning

31H ('1' ASCII)       Engineering Prototype

32H ('2' ASCII)       Early Production / Pilot Production

41H ('A' ASCII)       Initial Standard Production Level

All Others            Reserved for future levels

Byte 3  Firmware Revision:    This value indicates an ASCII value from

0-9,A-Z that indicates the firmware

revision installed in the AHA-1640.

After completing this command, HACC will be asserted, indicating normal

completion.

4-21

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.6.6 Enable Mailbox-Out Available Interrupt (Operation Code 05)

The Enable Mailbox Out Available Interrupt  command specifies that a Mailbox

Out Available Interrupt should be generated whenever a Mailbox Out entry has

been cleared by the host adapter. One byte of outbound data is transmitted to

indicate whether the interrupt should be enabled or disabled.

Byte 0 Enable/Disable Parameter:  The Enable/Disable parameter byte must be

either 00H or 01H. If the Enable/Disable

parameter byte is 00H, the Mailbox Out

Available interrupt is not returned. If

the Enable/Disable parameter byte is 01H,

the Mailbox Out Available interrupt is

returned as soon as a Mailbox Out has been

cleared by the host adapter.

After completing this command, HACC will NOT be asserted to avoid generating

additional interrupts. If the data byte is neither 00 nor 01, INVDCMD will be

asserted indicating  an invalid  command. In  this case,  HACC will  also be

asserted.

The Mailbox Out Available interrupt  is normally intended to  be used by the

host as an indicator that a Mailbox Out entry is available after all entries

have been found full by the  host. If used in other  ways, the interrupt may

generate a large number of relatively useless interrupt handling activities.

4.6.7 Set SCSI Selection Time-out (Operation Code 06)

This command sets the SCSI selection time-out value. The SCSI Selection

time-out value is used to determine whether or not a SCSI selection was

successful. If the SCSI BSY signal is not returned within the specified

time-out value, the  selection will  be terminated  and an appropriate

error message posted with the returned  CCB. This command expects four

outbound data bytes to be provided as defined below:

00         Enable/Disable Selection Time-Out

01      Reserved (00)

02      Time Out Value      (MSB)

03      Time Out Value      (LSB)

Byte 0  Enable/Disable Selection Time-Out: This parameter byte specifies

whether or not the Selection Time-out will

be used. If the byte is set to 00H, no

time-out will be performed. If the byte is

set to 01H, the time specified in bytes 02

and 03 will be used as the Selection Time-

Out by the SCSI. The default value

established by the reset process is 01H,

indicating the time out is enabled.

Byte 1     Reserved:          This byte must be zero.

Byte 2,3   Time Out Value:    This two-byte value specifies the time in

milliseconds that will be used for the

Selection Time-Out. The default value is

250 milliseconds.

4-22

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

After completing  this command,  HACC  will be  asserted,  indicating normal

completion. INVDCMD will be asserted if the data byte 0 is neither 00 nor 01

or if byte 1 is not zero, indicating an invalid command.

4.6.8 Set Bus-On Time (Operation Code 07)

Data Byte 0 -- Bus-On Time (not examined by AHA-1640)

This command performs no operation  and is accepted by  the AHA-1640 only to

provide software compatibility with the AHA-1540A/1542A. After completing this

command, HACC will  be asserted  indicating normal  completion. This command

controlled the data transfer  burst time on the  AHA-1540A/1542A, but is not

used on the AHA-1640 because the actual bus activity on the Micro Channel is

controlled by the FIFO  management protocol of  the AHA-1640 and  by the bus

arbitration and fairness protocols.

4.6.9 Set Bus-Off Time (Operation Code 08)

Data Byte 0 -- Bus-Off Time (not examined by AHA-1640)

This command performs no operation  and is accepted by  the AHA-1640 only to

provide software compatibility with the AHA-1540A/1542A. After completing this

command, HACC will  be asserted  indicating normal  completion. This command

controlled the period between data transfer burst on the AHA-1540A/1542A, but

is not used  on the AHA-1640  because the  actual bus activity  on the Micro

Channel is controlled by the FIFO management protocol of the AHA-1640 and by

the bus arbitration and fairness protocols.

4.6.10 Set Transfer Speed (Operation Code 09)

The Micro Channel timing  is fully specified.  It is expected  that all well

behaved Micro  Channel  attached  devices  will  provide  appropriate timing

signals, including CHRDY. The Set Transfer Speed is not required for standard

Micro Channel  implementations  and  is included  only  to  provide software

compatibility with the AHA-1540A/1542A. It may also be required to establish

special timings for special high speed versions of the Micro Channel.

One data byte follows this command. It performs no function.

Data Byte 0   -- Not Examined

After completing this command HACC is set indicating normal completion.

4-23

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.6.11 Return Installed Devices (Operation Code 0A)

This command returns information about which  SCSI Targets and Logical Units

(LUs) are installed on the  SCSI bus. The host adapter  issues the SCSI Test

Unit Ready command  to each target/LUN  combination and  reports the results

using eight bytes of data returned through the Data In Register. This command

can be executed once after power-up or any reset. A bit having a value of one

indicates that  the  associated LU  is  installed  and ready.  Each  byte is

associated with the corresponding  target. Each bit within  a target byte is

associated with a particular Logical Unit, bit 7 indicating the status of LU 7

and so forth.

After receiving  this  command,  the host  adapter  returns  eight  bytes of

information which specify the installed configuration as shown in Table 4-5.

TABLE 4-5.

Byte 0     Target 0 Configuration:

Bit 7              LU 7 Installed

Bit 6              LU 6 Installed

Bit 5              LU 5 Installed

Bit 4              LU 4 Installed

Bit 3              LU 3 Installed

Bit 2              LU 2 Installed

Bit 1              LU 1 Installed

Bit 0              LU 0 Installed

Byte 1     Target 1 Configuration

Byte 2     Target 2 Configuration

Byte 3     Target 3 Configuration

Byte 4     Target 4 Configuration

Byte 5     Target 5 Configuration

Byte 6     Target 6 Configuration

Byte 7     Target 7 Configuration

The byte associated with the SCSI Device Identifier of the host adapter will

always be zero. After completing the information transfer, the HACC bit will

be set to indicate normal completion.

4-24

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.6.12 Return Configuration Data (Operation Code 0B)

The DMA arbitration priority, the Interrupt channel,  and the SCSI ID of the

adapter are returned by this command. All three of these values are normally

set through the POS registers, but for compatibility with the AHA-1540A/1542A,

they can also be read back using this command. The three bytes of information

are returned in the following format:

Byte 0     DMA Arbitration Priority:

Byte is reserved. Value returned is 00h.

Byte 1     Interrupt Channel:

Bit 7              Reserved (0)

Bit 6              Interrupt channel 15

Bit 5              Interrupt channel 14

Bit 4              Reserved (0)

Bit 3              Interrupt channel 12

Bit 2              Interrupt channel 11

Bit 1              Interrupt channel 10

Bit 0              Interrupt channel 9

Byte 2     SCSI Identifier

Bits 7-3           Reserved (0)

Bits 2-0           Binary value of SCSI Identifier

After completing this command, HACC will be set indicating normal completion.

Note  that  the  DMA   channel  information  previously   available  in  the

AHA-1540A/1542A in  information byte  0 is  now available  to the  host only

through the POS registers.

4-25

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.6.13 Enable Target Mode Command

A special host  adapter command (0C)  enables and disables  target mode. The

host adapter  requires that  two bytes  of  outbound information  follow the

command byte. The information bytes contain the following information:

Byte 0 Enable/Disable Target Mode: This parameter specifies which operating

modes the host adapter will use. If the

byte is set to 00H, the AHA-1640 will

operate only in Initiator Mode. If the

byte is set to 01H, the host adapter will

operate both as an initiator and as a

Processor type Target SCSI Device. Any

other value is invalid. The default value

after a Hard Reset, Soft Reset, or Power

On Reset is 00H (Initiator only).

Byte 1 Logical Unit Mask:          This parameter byte contains an 8-bit

bit-significant mask indicating the

Logical Units which will respond in Target

Mode. Bit 7 corresponds to LU 7, and so

forth. If the bit is one, the

corresponding LU will be treated as

installed. If the bit is zero, the LU

will be treated as not installed.

If the AHA-1640 does  not have the  Target Mode feature  installed, the host

adapter will indicate an invalid host adapter command. If an attempt is made

to change from  target mode  while there  are still  target mode  CCBs being

processed by the host  adapter, the host  adapter will post  an invalid host

adapter command indication.

If the command  disables Target  Mode, the  Logical Unit  Mask byte  will be

ignored. If the command enables Target Mode, the Logical Unit Mask byte must

contain at least one  bit, indicating the  presence of at  least one Logical

Unit.

The SCSI INQUIRY command will provide an indication to other initiators that

the Logical Unit  is installed or  not installed  in byte 1  of the returned

inquiry data.

If target mode  is not  enabled, the  host adapter  will behave on  the SCSI

interface as if it were an ordinary SCSI initiator. Any attempt to select the

host adapter will result in a SCSI selection time- out. Any Reset Operation,

including Soft Reset, Hard Reset, and Power-On Reset will return the AHA-1640

Target Mode to the disabled state. SCSI Resets, generated either by the host

or by other SCSI Devices, will not change the previously established enabled

or disabled state of Target Mode.

4.6.14 Return Setup Data (Operation Code 0D)

This command returns information  describing the setup  of the host adapter.

The information returned reflects either the values supplied by previous host

adapter commands or default  values. The command is  followed by an outbound

data transfer and an inbound  data transfer. The outbound  transfer is a one

byte parameter indicating the length of  the required inbound data transfer.

The inbound data  transfer contains  from zero  to 255  bytes of information

describing the setup of the host adapter.

4-26

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Outbound Data Byte:

Byte 0     Data In Length:    The number of data bytes requested can be

from 0 to 255. A value of zero will be

accepted and no data will be returned.

Inbound Data Summary:

00      SDT and Parity Status

01      Reserved (00)

02      Reserved (00)

03      Reserved (00)

04      Number of Mailboxes

05      Mailbox Address (MSB)

06      Mailbox Address

07      Mailbox Address (LSB)

08-FF   Reserved (00)

Inbound Data Bit Assignments:

Byte 0     SDT and Parity Status:

Bit                Meaning

0                If this bit is zero, Synchronous Data

Transfer negotiation will not be initiated

by the host adapter, but will be responded

to if requested by an attached SCSI

Device. If the bit is one, Synchronous

Data Transfer negotiation will be

initiated by the host adapter under those

conditions which require such negotiation.

The state is set from POS Register 104,

bit 3.

1                If this bit is zero, parity checking on

inbound SCSI transfers has been disabled.

If this bit is one, parity checking on

inbound SCSI transfers is enabled. The

state is set from POS Register 104, bit 4.

Byte 1     Reserved:          This byte will always be zero.

Byte 2     Bus On Time:       This byte will always be set to 00H. The

AHA-1540A/1542A would normally find 0FH as

the default in this location.

Byte 3     Bus Off Time:      This byte will always be set to 00H for

compatibility with the AHA-1540A/1542A.

Byte 4     Number of          The number of Mailboxes established by

previous Mailboxes:

Mailbox Initialization command will be

returned in this byte. This number will be

00H if the Mailbox Initialization command

has not yet been successfully completed.

4-27

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Byte 5-7   Mailbox Address:   The base address of the Mailbox area

established by a previous Mailbox

Initialization command will be returned in

this byte. The Most Significant byte is

byte 5. These bytes have no meaning if

Mailbox Initialization has not yet been

successfully completed.

If the command completes normally, the HACC interrupt will be set to one. If

the Mailbox area has  not been properly  initialized, all 8  bytes are still

requested by the host. If byte 4, the Number of Mailboxes, is zero, bytes 5-7

must be ignored.

4.6.15 Write Channel 2 Buffer (Operation Code 1A)

After receiving  this  command,  the  host  adapter  expects  three outbound

information bytes to be  transferred which point  to an area of  64 bytes in

system RAM. The area  pointed to will  be transferred to  the host adapter's

Channel 2 Buffer  using the host  adapter's DMA  circuitry. After completing

the transfer of  the 64  bytes from  the indicated  buffer area to  the host

adapter, the HACC interrupt  will be set  indicating normal completion. This

command is used in conjunction with the Read Channel 2 Buffer command for host

adapter diagnostics.

Byte 0     Buffer area address, Most significant byte

Byte 1     Buffer area address

Byte 2     Buffer area address, Least significant byte

4.6.16 Read Channel 2 Buffer (Operation Code 1B)

After receiving  this  command,  the  host  adapter  expects  three outbound

information bytes to be  transferred which point  to an area of  64 bytes in

system RAM. The area pointed to will be used as a buffer to receive 64 bytes

of information transferred from  the host adapter's Channel  2 Buffer to the

host's memory using the  host adapter's DMA  circuitry. After completing the

transfer of the  64 bytes from  the Channel  2 Buffer to  the indicated host

memory area, the  HACC interrupt will  be set  indicating normal completion.

This command is used in conjunction with  the Write Channel 2 Buffer command

for host adapter diagnostics.

Byte 0     Buffer area address, Most significant byte

Byte 1     Buffer area address

Byte 2     Buffer area address, Least significant byte

4.6.17 Write Adapter FIFO Buffer (Operation Code 1C)

After receiving  this  command,  the  host  adapter  expects  three outbound

information bytes to be  transferred which point  to an area of  54 bytes in

system RAM. The area pointed to will be transferred to the host adapter's FIFO

buffer using the host adapter's DMA circuitry. After completing the transfer

of the 54 bytes from the indicated buffer area to the host adapter, the HACC

interrupt will be set indicating normal completion.  This command is used in

conjunction with  the  Read Adapter  FIFO  Buffer command  for  host adapter

diagnostics.

4-28

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Byte 0     Buffer area address, Most significant byte

Byte 1     Buffer area address

Byte 2     Buffer area address, Least significant byte

4.6.18 Read Adapter FIFO Buffer (Operation Code 1D)

After receiving  this  command,  the  host  adapter  expects  three outbound

information bytes to be  transferred which point  to an area of  54 bytes in

system RAM. The area pointed to will be used as a buffer to receive 54 bytes

of information transferred from the host adapter's FIFO to the host's memory

using the host adapter's DMA circuitry. After completing the transfer of the

64 bytes from the FIFO to the indicated buffer area, the HACC interrupt will

be set indicating normal completion. This command is used in conjunction with

the Write Adapter FIFO Buffer command for host adapter diagnostics.

Byte 0     Buffer area address, Most significant byte

Byte 1     Buffer area address

Byte 2     Buffer area address, Least significant byte

4.6.19 Echo Command Data (Operation Code 1F)

This command is used to test the Command  / Data Out Port, the Data In Port,

and the associated control bits in the other I/O Ports. After receiving this

command, the host  adapter expects  one byte  of outbound  information to be

transferred through the Command / Data Out Port. The host adapter then sends

(Echos) the same data value back to the host through the Data In Port. After

the host has  read the  data value provided  on the  Data In Port,  the host

adapter  will  generate  the  HACC  interrupt  to  indicate  normal  command

completion.

Byte 0, Out Outbound Echo Value

Byte 0, In Returned Echo Value

4.7 MAILBOX DESCRIPTION

The AHA-1640 uses a mailbox architecture  for task communication between the

host and the host adapter when executing SCSI commands as described in Section

2. This allows the  host adapter to perform  multithreaded operations with a

minimum of host intervention. The mailboxes are located in main system memory.

Each mailbox  entry is  four bytes  long. At  power up,  the host  issues an

initialization command to inform the host of the mailbox location. There are

always an equal  number of Outgoing  Mailboxes (MBO)  and Incoming Mailboxes

(MBI). The MBOs pass pointers  from the host to the  host adapter, while the

MBIs pass pointers or activity  requests from the host  adapter to the host.

The MBIs are  located immediately after  the MBOs.  Initiator operations and

target  operations  use  the  same  mailboxes.  Both  initiator  and  target

operations may be in process at the same time. A typical mailbox structure is

shown in Figure 4-2.

4-29

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

[Picture]

FIGURE 4-2. TYPICAL MAILBOX STRUCTURE

In this example, there are four MBOs and four MBIs. The Mailbox Count in the

Mailbox Initialization  command was  set to  four. The  base address  is the

address specified by Mailbox Address field.

4.7.1 Mailbox Out Definition

The first byte  of each  MBO entry  contains the  Mailbox Command  byte. The

remaining three bytes  contain an  address pointer  to the  first byte  of a

Command Control  Block  (CCB).  The  CCB  provides  additional  task control

information. AMBO entry is free if the first byte is zero. The host can make

an entry in any free MBO and indicate  that it is filled out or completed by

placing the proper MBO Command in  the first byte of the  MBO. After the MBO

has been examined by the host adapter and all relevant information obtained by

the host adapter, the host adapter sets the MBO Command byte back to zero to

allow the host to fill it again.  For a multitasking operating system, it is

desirable that the number of  mailboxes be sufficient to  allow at least one

mailbox for each active independent task or activity.

The MBO format is described in detail below:

Byte 0     MBO Command        This byte specifies the state of the MBO

entry.

Value              Definition

00H                Mailbox Out is free

01H                SCSI or Host Adapter command is to be

started. CCB pointer indicates location

of CCB to be processed.

02H                SCSI or Host Adapter command is to be

aborted. CCB pointer indicates location

of CCB to be terminated.

4-30

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Byte 1     CCB Pointer (MSB)

Byte 2     CCB Pointer

Byte 3     CCB Pointer (LSB)

The use of Mailbox Out (MBO) entries to pass pointers to CCBs is identical for

target and initiator modes. The appropriate target  mode CCB may be prepared

early and posted to the host adapter in preparation for an operation that is

expected to happen. If a  SCSI operation occurs before  the CCB is prepared,

the host  adapter processes  as much  of the  transaction as  possible, then

requests a CCB from the host through  the MBI. The host is fully responsible

for preparing the correct CCB from the information provided through the MBI.

One target CCB may be presented for each unique combination of LUN, Initiator,

and direction. If a second CCB  to the same LUN and  initiator with the same

direction bit is sent to the AHA-1640, the  CCB will be returned with a host

status of 19H, Duplicate CCB Received.

Initiator type CCBs  may be queued  for an  LUN and Target,  as described in

Section 4.10.2. The host adapter will always search for new MBO entries in a

round-robin order, beginning with the entry after the last MBO entry that was

processed. By always placing the MBO entries  in the MBO area consecutively,

the host can assure that the SCSI commands will be executed in FIFO order with

a minimum scan overhead.

If the Mailbox Out Available Interrupt is enabled by execution of IOCP command

05 (Enable Mailbox Out Available Interrupt), the host adapter will take one of

the following actions after each MBO entry is freed by the host adapter:

1) If  the Any  Interrupt  bit in  the  Interrupt Flag  Port  is zero,

indicating that there are no interrupts pending, the host adapter

will set the Mailbox Out Available Interrupt and the Any Interrupt

bit and  indicate a  hardware interrupt  to  the host.  The host

adapter will then continue scanning for other stored MBO entries.

2) If  the Mailbox  Out Available  Interrupt  has been  set to  one by

the previous clearing of a Mailbox Out entry and the interrupt has

not yet been cleared by the host, the host adapter will take not

post any new interrupt and will continue to scan for other stored

MBO entries.

3) If  interrupts  other  than  the  Mailbox  Out  Available Interrupt

are pending, the host adapter will wait for the pending interrupts

to be cleared before setting the Mailbox Out Available Interrupt.

This guarantees that the MBO Empty Interrupt will not be cleared

accidentally by clearing another interrupt.

4.7.2 Mailbox In Definition

The Mailbox In entries are used to  pass completion information concerning a

task from the host adapter to the host. In addition, requests for Target Mode

CCBs are passed to  the host using MBI  entries. The first  byte of each MBI

contains the MBI Status byte, summarizing the type of information being passed

from the  host  adapter  to  the host.  The  remaining  three  bytes contain

specialized information that provides more  detail about the information. In

the case of a CCB Completed MBI, the bytes contain a pointer to the completed

CCB. In the case  of a CCB  Required MBI, the bytes  contain the information

necessary for the host to prepare an appropriate CCB. Only those MBIs with a

MBI Status that  is non-zero  have information for  the host.  When the host

returns the MBI Status byte to zero, the MBI Free state, the host adapter is

allowed to place a new set of information in the MBI entry.

4-31

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

When a SCSI command completes, or if a new CCB is required, the host adapter

scans the first byte of an MBI entry to find a free mailbox. The scan will be

performed in  a round-robin  order so  that MBI  entries will  be made  in a

round-robin order.  If a  free MBI  entry  is found,  the host  adapter will

update the MBI's status byte with a  non zero value and update the following

bytes with the  appropriate required pointers  or parameters.  Since the MBI

entries are usually filled in a round-robin order, the host can usually find

the full MBI immediately  by performing its  scan in a  round-robin order as

well. The valid MBI formats are defined in Table 4-6.

TABLE 4-6. Mailbox In Format for CCB Complete

Byte 0     MBI Status         This byte specifies the state of the MBI

entry.

Value              Definition

00H                Mailbox In is free

01H                CCB completed without error. CCB pointer

indicates location of successfully

completed CCB.

02H                CCB aborted by host. CCB pointer

indicates location of CCB that was aborted.

03H                Aborted CCB not found. CCB pointer

indicates the supposed location of the CCB

that was to have been aborted. It is

likely that the CCB was already presented

to the host before the Abort CCB MBO

entry was completed.

04H                CCB Completed with Error. The CCB fields

indicate the details of the error

condition. This code allows normal CCB

completion to be processed without

bothering to examine the completion codes

in the CCB.

Byte 1     CCB Pointer (MSB)

Byte 2     CCB Pointer

Byte 3     CCB Pointer (LSB)

4-32

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Mailbox In Format for CCB Required

Byte 0     MBI Status         This byte specifies the state of the MBI

entry.

Value              Definition

10H                SCSI Target Command received with no CCB

available. The host must prepare an

appropriate CCB and place it in the MBO to

complete the SCSI command. The remaining

three bytes of the MBI specify the

information necessary to prepare a SCSI

command.

Byte 1     Initiator and LUN

Bit 7-5            Binary address of initiator that selected

the host adapter in Target Mode.

Bit 4              The SCSI Target Command received was a

RECEIVE command. A CCB must be prepared

to transmit data to the Initiator.

Bit 3              The SCSI Target Command received was a

SEND command. A CCB must be prepared to

transmit data from the Initiator.

Bits 2-0           Binary address of the logical unit of the

Target Mode host adapter that was

addressed by the Initiator.

Byte 2-3   Data Length        The high order two bytes of the data

length specified in the SEND or the

RECEIVE command. A CCB must be prepared

to transmit this amount of data plus up to

256 more bytes. It is assumed that the

data fields are self-defining or of a

known length in such protocols.

For the CCB Required Mailbox In entries, byte 0 takes on a distinctive value

of 10H, indicating both that the MBI entry is filled and that the MBI entry is

being presented to request a CCB appropriate to service a SCSI transaction for

which no CCB was available.  The host adapter will  have disconnected on the

SCSI after receiving the initiator and LU addresses and after having received

either a SCSI SEND or a RECEIVE command. If the LU address is enabled by the

Logical Unit Mask, the MBI will provide the addresses in bits 7-5 and bits 2-0

and will indicate by setting either bit 3 or bit 4 that a SCSI SEND or RECEIVE

command was accepted from the initiator. The host system is then expected to

prepare a  CCB suitable  for  processing the  Send  or Receive  command. The

information in bytes 2 and 3 is a copy of bytes 2 and 3 of the Send or Receive

command CDB. This provides  the host an  approximation of the  length of the

requested data transfer.  This length  is used by  the host  to allocate the

correct amount of buffer to accept data in  a SEND command or to control the

maximum amount of data that can be transferred in a RECEIVE command.

If the LU address received is disabled, no MBI will be presented and a check

condition will be posted to the Initiator. A subsequent REQUEST SENSE command

will recover the error  indication. The error  will be an  Invalid LUN error

with a sense key of 05h and an additional sense code of 25h.

4-33

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

After each Mailbox In entry is stored  by the host adapter, it will indicate

that Mailbox In  scanning must  be performed by  the host  by presenting the

Mailbox In Full interrupt in the following manner:

1) If the Any  Interrupt bit is not  set to one in  the Interrupt Flag port,

indicating that no interrupts are pending, the host adapter will set the

Mailbox In Full interrupt bit, the Any Interrupt bit, and raise the hardware

interrupt line to indicate that there is at least one Mailbox In entry to be

examined by the host system.

2) If the  Mailbox In Full  interrupt bit  has already been  set to indicate

that an entry  was made in the Mailbox In area and if that interrupt has not

yet  been cleared by the  host, no further notification is provided by the

host adapter.

3) If interrupts other than the Mailbox In Full interrupt are pending, the

host adapter will wait for all pending interrupts to be cleared  before

setting the Mailbox In Full interrupt.

Note that careful host  Target Mode software design  is necessary to prevent

ambiguity in the notification process. If a CCB was just prepared but the MBO

has not yet been searched by  the host adapter at the  time the MBI entry is

made to the host, the host may choose to examine the MBI entry and not act on

it, having already provided the needed CCB. The host should also examine the

entire MBI space to be sure that a previously supplied CCB which should have

been able to service the  notification MBI entry was not  already used for a

previous operation that  had been  posted by  the host  adapter but  not yet

serviced by the host. Proper  care in host system  design will prevent these

overlapping operations from becoming a problem.

4-34

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.8 COMMAND BLOCK DEFINITION

The CCB specifies detailed information about a SCSI command. The format of a

CCB is shown below. Each of the fields is separately explained following the

table.

COMMAND CONTROL BLOCK FORMAT

Byte 0     Command Control Block Operation Code

Value              Definition

00H                SCSI Initiator Command

01H                Target Mode Command

02H                SCSI Initiator Command with Scatter/Gather

Byte 1     Address and Direction Control

Bits 7-5           CCB Op Code = 00, 02:  SCSI Target Id

CCB Op Code = 01:      SCSI Initiator Id

Bit  4             Outbound data transfer, length is checked.

Bit  3             Inbound data transfer, length is checked.

Bit  2-0           Logical Unit Number

Byte 2     SCSI Command Length

Byte 3     Returned Sense Information Length

Byte 4-7   Data Length (Byte 4 most significant)

Byte 7-9   Data Pointer (Byte 7 most significant)

Byte 10-12 Link Pointer (Byte 10 most significant)

Byte 13    Command Linking Identifier

Byte 14    Host Adapter Status (HASTAT)

Byte 15    Target Device Status (TARSTAT)

Byte 16-17 Reserved (00)

Byte 18-n  SCSI Command Descriptor Block

(Length specified by byte 2)

Byte n-m   Reserved for REQUEST SENSE information bytes

(Length of reserved space in byte 3)

The standard format of the Command Control Block is shown in Figure 4-3.

4-35

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

[Picture]

FIGURE 4-3. COMMAND CONTROL BLOCK

The bytes of the CCB are defined further below:

Byte 0: Command Control Block Operation Codes

The AHA-1640 supports four CCB Operation Codes. The valid command values are

shown below:

00   SCSI Initiator Command Control Block

01   SCSI Target Command Control Block

02   SCSI Initiator Command Control Block with Scatter/Gather

81   SCSI Bus Device Reset

If the Operation Code value is 00H, the SCSI command specified in the Command

Descriptor Block  field  of  the  CCB  is  executed  against  the  addressed

Target/LUN. The  other  fields of  the  CCB support  the  required Initiator

functions of the AHA-1640.

If the Operation Code value is 01H, the CCB is intended to service a SEND or

RECEIVE command sent to the host adapter as a target from another initiator.

The values in the other fields are used to service the Target Mode operation.

If an Operation Code of 01H is specified  to a host adapter that has not had

its Target Mode enabled, the host adapter returns a Host Status indication of

18H, Invalid CCB Parameter.

If the Operation Code value is 02H, the SCSI command specified in the Command

Descriptor Block field of the CCB is executed against the addressed Target /

LUN. The definition of the  Data Length and the Data  Pointer is modified to

support the Scatter/Gather function.

4-36

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

If the value  is 81(hex)  a Bus  Device Reset  message will  be sent  to the

addressed target.  This  command  forces  the  host  adapter  to  abort  all

outstanding tasks against the  selected target. All  remaining CCB Bytes are

ignored.

Any other command value generates a Host Adapter Detected Error (Host Status

byte of 16H).

Byte 1:  Address and Control Byte

This byte identifies the  address of the  devices that will  be serviced and

provides information about the expected direction of data flow.

Bit Field     Definition if Initiator CCB     Definition if Target CCB

Bits 7-5      Target SCSI Device ID           Initiator SCSI Device ID

Bits 4 and 3  Direction of Data Xfer          Service Send/Receive

00           Direction not checked          Invalid

01           Data In expected               SEND Command

10           Data Out expected              RECEIVE Command

11           No Data Xfer expected          No Data Xfer expected

Bits 2-0      Target Logical Unit Number      HA Logical Unit Number

If the CCB is an Initiator CCB, this byte identifies the target SCSI device in

bits 7,  6 and  5. If  the  CCB is  a Target  CCB,  the byte  identifies the

initiator which the CCB will serve.

Bits 4 and 3 are optionally set to determine the direction of data transfer.

For an Initiator CCB, if neither bit  is set, the direction of data transfer

will be established by the SCSI command being executed. For a Target CCB, if

neither bit is set, the CCB will be returned with an indication of Invalid CCB

Parameters (18H) in  the Host  Status field, since  each Target  CCB must be

identified as to whether it will service a SEND or a RECEIVE command. If both

bits are  set for  either type  of  CCB, the  command must  perform  no data

transfer. If only  bit 3 is  active, the data  transfer will be  to the host

adapter and from the external SCSI  device. If the CCB is  a target CCB, the

data transfer will be required to be a Data Out phase, while if the CCB is an

initiator CCB, the data transfer will be required  to be a Data In phase. If

only bit 4 is active, the data transfer will be from the host adapter and to

the external SCSI device. If the CCB is a target CCB, the data transfer must

be a Data In phase, while if the  CCB is an initiator CCB, the data transfer

will be a Data Out phase.

If bits 3 or 4 are set for an Initiator CCB, the data length will be checked.

If the amount of data transferred exceeds the specified amount, the CCB Host

Status field will contain an indication of  Data Over/Under Run (12H). For a

Processor Target mode CCB, the handling of incorrect lengths is described in

Section 7.2.4.

If the CCB is an initiator CCB, bits 2, 1 and 0 define the target Logical Unit

which will be addressed. If the target  accepts an IDENTIFY message out, the

value in bits 2,1,  and 0 will be  provided in the LUN  field of the message

byte. The LUN field in the SCSI Command Descriptor Block (CDB) is expected to

be zero. If the target does not accept an IDENTIFY message out, the LUN field

in the SCSI CDB must contain the  correct Logical Unit address. SCSI devices

with conformance level 2, including Common Command Set (CCS) disk drives and

4-37

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

all SCSI-2 devices will always accept the IDENTIFY message out. The few SCSI

devices not meeting  those requirements must  be examined  on a case-by-case

basis to determine whether the Logical Unit  Address should be placed in CCB

Byte 1 or in the CDB.

If the CCB is a target CCB, bits 2, 1 and 0 define the Processor type Logical

Unit within the host adapter which will use the CCB. When an initiator sends

a SCSI command to the host adapter, the host adapter expects the initiator to

require an IDENTIFY  message out.  The LUN  in the  message will be  used to

properly select the target CCB that has  the same LUN and the same initiator

device ID.

Byte 2: SCSI Command Length

This byte establishes the length, in bytes, of the space reserved for the SCSI

Command Descriptor Block beginning at byte 18.  The actual length of the CDB

may be shorter than the reserved space. The number of bytes transferred will

be the number requested by the target as  long as the number does not exceed

the reserved space.

Byte 3:  Request Sense Allocation Length

This byte indicates the length, in bytes, of the area reserved for information

that may be obtained  by a REQUEST  SENSE command. The  reserved area begins

immediately after the last  byte allocated by  the CCB for  the SCSI Command

Descriptor Block. A value of 00H defaults to 14 bytes, sufficient to capture

the sense key and  error code of  all normal extended sense  type devices. A

REQUEST SENSE command is automatically generated by the host adapter if Check

Condition status is presented at the end of a SCSI command. The host adapter

uses the indicated Request Sense Allocation Length  as its byte count in the

REQUEST SENSE Command Descriptor Block that it generates to obtain the sense

information.

If an operation that treats  the AHA-1640 as a Target  Mode device fails and

presents a Check Condition status byte, the  Initiator is expected to return

automatically with  a  REQUEST  SENSE  command.  The  AHA-1640  will  return

appropriate sense information in response to the command. If the command that

originally failed was a  SEND or a  RECEIVE command, the  same REQUEST SENSE

information bytes that will later be sent  to the Initiator are also sent to

the host when the  CCB is returned.  The sense information is  placed in the

specified Request  Sense Allocation  area with  a  length not  exceeding the

Request Sense Allocation Length. The sense information is placed in the area

in the same format that will be sent to the initiator when it is requested.

Bytes 4, 5, 6: Data Length

These bytes determine the length,  in bytes, of the  data transfer. CCB host

adapter error 12h  (Data Over run  / Under  run) is posted  if the addressed

target attempts to transfer more than the indicated number of bytes.

If the CCB  specifies a  Scatter / Gather  operation, the  Data Length field

contains the length  of the  Data Segment List.  The operation  of Scatter /

Gather is described in Section 4.9.

Bytes 7, 8, 9: Data Pointer

These bytes specify the real address of the first byte of the host memory data

area to be used during the data phase of the SCSI command.

If the  CCB specifies  a Scatter/Gather  operation,  the Data  Pointer field

contains the pointer to the first byte of the Data Segment List.

4-38

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Bytes 10, 11, 12: Link Pointer

These bytes are used when a SCSI command contains a LINK or LINK WITH TAG bit

in the command. When a Linked command is completed, the host adapter will use

the contents of this field as  a pointer to the next  CCB to execute. If the

Linked Tag bit is set, an interrupt will be generated before the next command

is started. A completed CCB is always reported back in an MBI.

Target Mode does not support the linking function.

Byte 13: Command Link ID

This byte is used in conjunction with linked commands. It is set by the host

to identify commands in a command chain.

Linking is not supported in Target Mode.

Byte 14: Host Adapter Status

This byte is used to report the host adapter status (HASTAT).

HASTAT has the following definitions:

00 H                   No host adapter detected error

The CCB was completed normally.

0A H                   Linked command complete without error

The SCSI command completed and linked

normally.

0B H                   Linked command complete without error,

interrupt generated.

The SCSI command completed and linked with

a LINKED COMMAND COMPLETE WITH FLAG message.

11 H                  Selection time out

The initiator selection or target

reselection was not complete within the

Set SCSI Selection Timeout period.

12 H                  Data over run/Under run

The target attempted to transfer more data

than was allocated by the Data Length

field or the sum of the Scatter / Gather

Data Length fields.

13 H                  Unexpected bus free

The target dropped the SCSI BSY at an

unexpected time.

14 H                  Target bus phase sequence failure

An invalid bus phase or bus phase sequence

was requested by the target. The host

adapter will generate a SCSI Reset

Condition, notifying the host with a SCRD

interrupt.

15 H                  MBO command was not 00, 01, or 02

The first byte of the MBO command was

invalid. This usually indicates a

software failure.

16 H                  Invalid CCB Operation Code

The first byte of the CCB was invalid.

This usually indicates a software failure.

4-39

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

17 H                  Linked CCB does not have the same LUN

A subsequent CCB of a set of linked CCB's

does not specify the same logical unit

number as the first.

18 H                  Invalid CCB Parameter Received from Host

A parameter of the CCB was invalid. The

CCB direction was invalid.

19 H                  Duplicate CCB Received in Target Mode

More than once CCB was received to service

data transfer between the same target LUN

and initiator SCSI ID in the same

direction.

1A H                  Invalid Segment List Received

A segment list with a zero length segment

or invalid segment list boundaries was

received.

1B H                  Request Sense Command Failed

Non zero status was received by the host

adapter in response to a REQUEST SENSE

command.

Byte 15: Target Status

For an initiator CCB, this byte is used to return the SCSI status byte sent to

the host adapter  from the initiator.  For a  target mode CCB,  this byte is

used to indicate to  the host what  status the host adapter  returned to the

initiator.

Target Status may have the following values:

02 -- Check Status (See REQUEST SENSE byte area)

08 -- LUN Busy

18 -- Reservation Conflict

If a SCSI command returns with BUSY status, the normal recovery process is to

execute the command again. The  AHA-1640 takes that burden  off the host and

periodically restarts the command automatically until the command is executed

successfully.

Byte 16: Reserved (must be 0)

Byte 17: Reserved (must be 0)

Byte 18 - n: SCSI Command Descriptor Block

For an initiator CCB, this field holds the SCSI Command Descriptor Block (CDB)

to  be  transmitted  to  the  selected  target  as  described  in  the  SCSI

specification. The length of this command is defined in byte 02.

For a target mode CCB, this field is not prepared by the host, but instead is

returned by the host adapter to the host. The CCB must have a minimum of six

bytes allocated for the CDB.  The field contains the  actual SEND or RECEIVE

CDB transmitted from the initiator. The SEND  command's control bytes can be

examined to determine if the information presented is a normal SEND command or

an Asynchronous Even Notification.

4-40

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Byte n.+1 - n+m:  Allocated for Sense Data

If a Check Status condition  is detected by the AHA-1640  as it completes an

operation on the SCSI bus, the host adapter automatically executes a REQUEST

SENSE command with  the data  length specified  by Request  Sense Allocation

Length. The actual bytes  returned, up the maximum  indicated by the Request

Sense Allocation Length, are placed in the area Allocated for Sense Data.

If a check condition  is detected by  the AHA-1640 while it  is operating in

Processor Target Mode, the same information that  will later be recovered by

the initiator that  received the  Check Status  is also  placed in  the area

Allocated for Sense  Data so that  the host  processor is also  aware of the

failure.

4.9 SCATTER/GATHER LIST DEFINITION

Using the normal CCB Operation  Codes of 00 and 01  (SCSI Initiator and SCSI

Target CCBs),  the CCB  itself contains  a pointer  to the  first byte  of a

contiguous area of data of a specified length. The direction of transfer with

respect to the data area and the checking of the length of the data transfer

to the  data  area  are  both  managed  by  the  AHA-1640  according  to the

requirements of the particular mode and control bit set-up.

Using the scatter/gather  Operation Code of  02, the CCB  instead contains a

pointer to a list of data segments and an indication of the length of the list

of data  segments. The  data segment  list contains  pointers to  the actual

location in host memory of the data segments  to be transferred as well as a

precise indication of the length of each data segment. Each data segment list

entry contains a three-byte  pointer to the  location of a  data segment and

three-byte length indicator  telling how  long that  particular data segment

shall be. The data segment list is arranged  in the order in which data will

be gathered or distributed, the first pointer in the list being used first.

A typical data  segment list is  shown in  table 4-7. The  data segment list

describes the  distribution  of  4096  bytes  distributed  in  four separate

locations within  the memory.  The  beginning of  the  data segment  list is

indicated by the data segment  list pointer. The length  of the data segment

list (48 bytes) will be contained in the data segment list length field of the

CCB.

TABLE 4-7.

[Picture]

A data segment  list can have  from one to  sixteen segments. A  list with 0

segments or a list with more than 16 segments causes an Invalid Segment List

error to be posted in the host status field.

4-41

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

The AHA-1640 has certain constraints in  the data segment address boundaries

and lengths that  can be implemented.  If these boundaries  are violated, an

Invalid Segment List error will be posted in the host status field of the CCB.

The simplest way  to assure  that these  boundaries are  not violated  is to

require all boundaries between segments to be  one even word boundaries. The

first segment may begin on any boundary and  the last segment may end on any

boundary. All segments but the first and last should have even byte counts.

The actual boundary limitation  is somewhat looser. If  the binary values of

the starting address of a  segment, the byte count of  that segment, and the

starting address of the next segment are all exclusive OR'd together, and the

result of that exclusive OR is an even number, the boundary between those two

segments is valid. Another way to express  the limitation is to require that

the ending boundary of one segment must be the same as the beginning boundary

of the next segment. If a segment ends  on a word boundary, the next segment

must begin on a word  boundary. Similarly, if a segment  ends on an odd-byte

boundary, the next segment must begin on an odd-byte boundary.

4.10 DESCRIPTION OF OPERATION

This section describes the interface operations required to invoke the desired

SCSI behavior. The AHA-1640 must be properly initialized before any of these

operations can be performed.  The initialization steps  are described in the

first section.

4.10.1 Initialization Procedure

The POS registers must be properly initialized by the starting code. The POS

registers allow the selection and assignment of the I/O Port Addresses, of the

BIOS PROM  addresses,  and of  the  required Micro  Channel  arbitration and

initialization parameters.

After POS initialization is done, the I/O Command Port initialization commands

must be executed.  The Mailbox  Initialization command  must be  executed to

assign the Mailbox  area. The Enable  Target Mode command  may optionally be

executed if Target Mode is to be allowed.

4.10.2 Execution of Initiator Mode operations

To begin an  Initiator Mode  SCSI command, the  host first  allocates a data

buffer area. A CCB is  then created to perform the  desired operation to the

correct peripheral device and a pointer to the data buffer area is placed in

the CCB. Once the CCB is completely defined, the host places a pointer to the

CCB in an empty MBO location, places a Start SCSI Command in the MBO Command

Byte and transmits a Start SCSI Command to the I/O Command Port. If it is not

already scanning the MBO for an active  MBO entry, the Start SCSI Command to

the I/O Port causes the host adapter to begin scanning for such an entry.

After finding an active MBO entry, the AHA-1640 copies the MBO Command field

contents and CCB pointer  into its internal  RAM and clears  the MBO Command

byte, freeing the mailbox. Up to 112 CCBs can be stored in the host adapter's

internal RAM concurrently. The AHA-1640 maximizes the SCSI bus utilization by

starting the next available CCB as soon as the bus is free. Disconnection and

reconnection on the bus are automatically taken  care of by the AHA-1640. As

the host  adapter  prepares  to  start the  SCSI  operation,  it  will first

determine if a CCB addressed to that target/LUN is already being executed. If

so, then the command will be placed in the task queue instead of being started

immediately (See Section 4.10.4).

4-42

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

Commands are removed from  the queue in  the order that  they were received,

establishing a first-in first-out (FIFO) command initiation order. Of course,

the order of task completion  may vary due to the  different amounts of time

required to  process  and complete  different  commands. If  BUSY  status is

received by  the AHA-1640,  the CCB  is  returned to  the FIFO  queue  to be

restarted later.

If a CHECK CONDITION status  is received from the  target, the AHA-1640 will

issue a REQUEST SENSE command to get the sense data. The sense data is stored

in the CCB after the SCSI command data. The driver software must reserve the

allocated number of bytes at the end of the CCB to hold the sense data which

is returned as a result of a receiving a check status.

If the MBO Command is to abort a SCSI command, the host adapter first searches

the active and queued CCBs. If that CCB is found, the task is aborted at the

earliest possible moment and an MBI entry is made to indicate that the CCB's

execution was successfully  terminated. If  the CCB  is not  found among the

active or queued CCBs,  the AHA-1640 completes the  command and reports that

fact in  the MBI  Status  byte. The  CCB  may not  be found  because  it has

previously been aborted, because an intervening reset occurred, or because the

CCB was already finished normally and returned.

When executing a START SCSI command, the AHA-1640 does not verify that the new

CCB resides in an unused memory area. Therefore, the host must not reuse a CCB

location until it has been returned through  a Mailbox In entry and the host

has examined all  the pertinent  information in  the CCB.  CCB addresses are

used by the AHA-1640 and by the host as task identifiers.

4.10.3 Execution of Target Mode Operations

4.10.3.1 Target Mode with a Prepared CCB

Typical target operation is managed in the following way by the host software.

After power-on initialization is  complete, the host  sends an Enable Target

Mode command to the host adapter to enable Processor Target mode. After that,

the host, from previous configuration knowledge, generates a pair of CCBs for

each likely initiator and for each LUN supported by the host. One CCB is for

outbound data from the initiator using the  SEND command, while the other is

for inbound data from the target to the initiator using the RECEIVE command.

The SEND command  CCB defines  a data buffer  for the  expected network type

command from the initiator. The RECEIVE command CCB, if any, defines a set of

data which is known  and expected by  the initiator, typically  a welcome or

configuration type message packet.

As  the  initiator  finishes  its   initialization  procedure,  it  requests

availability and configuration information using the TEST UNIT READY, REQUEST

SENSE, and  INQUIRY commands.  Finally, it  may choose  to address  SEND and

RECEIVE commands  to  the  Processor  type  targets  it  has  located during

initialization.

When the initiator executes a SEND or  a RECEIVE command, the target accepts

the selection on the SCSI  bus, accepts the command  from the initiator, and

executes the proper data transfer to or from the area specified by the proper

CCB. When all SCSI activity is finished successfully, the CCB is posted back

to the host program by an entry in  the MBI. The host is notified that there

is an entry in the MBI by an  interrupt from the host adapter. The host then

prepares a new CCB to  control the target's next  activity expected from the

initiator.

4-43

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

4.10.3.2 Target Mode without a Prepared CCB

Alternatively, the processor type device may be addressed by an initiator when

no CCB has yet been prepared for use by the host adapter. If the host adapter

command enabling target mode has not yet been executed, the host adapter will

act like any initiator.  It will not  accept any selection  sequence and any

attempts to select it by its target ID  will cause the initiator to detect a

selection time-out.

If the host adapter has been enabled as a Processor Target by a host adapter

command, any selection to the host adapter's target address will be accepted.

The availability  commands  and identification  commands  (TEST  UNIT READY,

REQUEST SENSE, and INQUIRY)  will be executed completely  and normally. If a

SEND or RECEIVE command is received and if a CCB is not active, either because

the host has not completed its program initialization or because the host has

not finished analyzing  the results of  previous data transfers  and has not

generated a new CCB, the Processor Target  will accept the selection, obtain

the IDENTIFY Message Out  to determine what Logical  Unit has been selected,

request the CDB from the SCSI, and then disconnect. The CDB will be partially

interpreted so that the proper MBI can be generated to notify the host that a

new CCB with a certain address, direction, and data length must be generated.

When the new CCB is passed  to the host adapter by  an MBO entry, the CCB is

filled up with  the command  information, the  SCSI is  reconnected, and the

command is completed as described above.

4.10.4 CCB Queuing and Scan Order

The MBO is scanned by the host adapter in a round-robin order. When a new MBO

entry has been placed in the  MBO, the host adapter first  looks in the next

entry higher than the  location where an  MBO entry was last  found. If that

entry is clear, the host adapter looks in consecutively higher entries until

the end of the MBO list is reached, then begins again at the first MBO entry,

scanning until the location where the last active MBO entry was encountered in

that scan cycle. If no  active MBO entry was found,  the scan terminates. If

any active MBO entries  are found, the  CCB pointer and CCB  for each active

entry are transferred to the host adapter in the scanned order until the scan

is complete. If the CCB can begin execution, it is started as soon as the CCB

pointer and CCB have been moved to the host adapter. To minimize the overhead

of the host adapter in the scanning process, the host should place new active

MBO entries in the MBO in the same round-robin order in which scanning occurs.

When each  CCB has  been  moved to  the  host adapter,  it is  placed  in an

appropriate queue. The  target mode CCBs  are placed  in a queue  that has a

maximum length of one to await an initiator action that requires the initiator

address, LUN, and data direction provided by the particular CCB. If a second

target mode CCB is presented with the  same addresses and data direction, it

will be rejected with a host status of Duplicate CCB Received in Target Mode.

Each initiator CCB is placed in a queue with other CCBs having the same target

address and logical unit address as each CCB is found during the scan process.

There is no  ordering relationship enforced  by the host  adapter among CCBs

requesting service from different targets and logical units. For CCBs having

the same target address and logical unit address, the CCBs are executed in the

precise order that they were  found during the scan  process. By placing the

CCBs in the MBO in round-robin order, the host can guarantee FIFO service for

all CCBs directed  to a  particular LUN.  The queueing  of CCBs in  the host

adapter allows the host adapter to begin the execution of a new operation on a

LUN which has just finished an operation in the shortest time possible. It is

not necessary for the host to service the MBI completion of the command before

the new operation is  started. If errors occur  during queued operation, the

4-44

Section Four                                  Multithreaded Software Interface

------------------------------------------------------------------------------

REQUEST SENSE command is  automatically executed between  CCBs and the sense

information is transmitted  automatically to the  host in  the returned CCB.

CCBs that must wait  for successful completion of  a previous SCSI operation

must not be  placed in the  MBO until  the previous SCSI  operation has been

returned successfully in an MBI entry.

The MBI entries are made in the MBI using the same type of scan order. A new

active MBI entry is made by the host  adapter in the next MBI entry location

after the one used by the previous MBI entry. The scan process for empty MBI

entry locations is performed in the same round-robin manner. The host can use

this to facilitate its scan for active MBI entries by performing a round-robin

scan beginning at the first MBI entry location after the last MBI entry that

was cleared. More than  one MBI entry may  be active at a  time, so the scan

should continue around the MBI until all entries have been examined.

The MBI Full interrupt should be cleared by the host as soon as possible after

it begins MBI processing so that, when a scan has been completed by the host,

a second scan will only be required if a new MBI Full interrupt has been set.

4-45

Section Five                                Single-Threaded Software Interface

------------------------------------------------------------------------------

5.1 SINGLE-THREADED BIOS COMMAND INTERFACE

The AHA-1640 BIOS resides on the host adapter board. It provides support for

up to two  SCSI Common Command  Set disk  drives under DOS.  If SCSI devices

other than CCS disk  drives or if support  for more than  two disk drives is

required, a DOS  device driver must  be used. Support  for operating systems

other than DOS usually requires appropriate drivers. The BIOS provides a very

simple single-threaded  capability that  does not  make  use of  the Mailbox

Interface used by more  sophisticated programs and  drivers. This capability

allows the host system to boot from a SCSI disk drive and to support hard disk

calls from any standard DOS program.

The BIOS communicates with the host adapter through a special set of commands

passed under the IOCP  command 02H (Start Bios  Command). These commands are

not available  to any  programs except  the Adaptec  BIOS. The  Adaptec BIOS

provides a standard BIOS interface that meets  the requirements of the PS/2.

The DOS interface to the standard BIOS is described in this section.

On a PS/2 compatible machine, up to two hard disks are supported by the BIOS

resident on the  motherboard. Any other  drives must be  managed through the

Mailbox Interface using an appropriate driver. The operation of the AHA-1640

BIOS on attached SCSI devices  depends on the number  of standard hard disks

installed directly on the system.

No PS/2 hard disks directly attached:

In this case, two  SCSI drives can  be supported by the  AHA-1640 BIOS. SCSI

Device 0 Logical Unit Number  0 is designated as drive  0 (Drive C:). Either

SCSI Device 0 LUN 1 or SCSI Device 1 LUN 0 may be designated as drive 1 (Drive

D:). This set of addresses allows the use of two drives with either embedded

controllers or bridge controllers.  Booting is performed  from SCSI Device 0

LUN 0. When no  hard disks are  directly attached to the  Micro Channel, the

AHA-1640 BIOS is operating in non-concurrent mode.

One PS/2 hard disk directly attached:

In this case, the  internally installed hard  disk is designated  as drive 0

(Drive C:). SCSI Device 0 LUN 0 is designated as the second drive (Drive D:).

Booting is performed from  the internally installed hard  disk. This mode of

operation is  called concurrent  operation,  indicating that  the internally

installed drive and one SCSI drive can operate together under the BIOS.

Two PS/2 internal hard disks attached:

In this case, all the disks  that can be supported by  the BIOS are directly

installed. The AHA-1640 BIOS will not support any SCSI drives.

5-1

Section Five                                Single-Threaded Software Interface

------------------------------------------------------------------------------

5.2  BIOS COMMAND EXECUTION USING INTERRUPT 13

On IBM compatible systems,  disk I/O requests are  passed from the operating

system to the BIOS through software interrupt 13H. CPU registers are used to

pass information about the desired  operation and the associated parameters.

This process is described in Section 1 of the IBM Personal Computer and PS/2

BIOS Interface Technical Reference manual.

The AHA-1640 BIOS provides  SCSI support by  intercepting each interrupt and

managing the request according to the drive address. If the drive is a floppy

or internally installed fixed  disk, the call is  managed by the motherboard

BIOS with no changes. If the selected drive is a SCSI disk drive, the request

is handled  by the  Adaptec BIOS  resident on  the AHA-1640.  For concurrent

operation, only calls to drive 81H (D:) are handled by the AHA-1640 BIOS. For

nonconcurrent operation, calls to both drive 80H (C:) and 81H (D:) are managed

by the AHA-1640 BIOS.

On each Interrupt 13 call, the correct host microprocessor registers are set

by the operating system program to provide the required parameters to both the

Adaptec BIOS and  the motherboard BIOS.  Most commands use  the registers as

indicated in Table 5-1.

TABLE 5-1

Register      Function

AH         BIOS Function Code

AL         Number of Sectors

CH         Low Order Byte of Cylinder Number

CL         Cylinder and Sector Numbers

Bits 7,6      High Order Cylinder Bits

Bits 5-0      Sector Number

DH         Head Number

DL         Drive Number

ES:BX      Data Buffer Address

5.2.1 Physical to Logical Address Translation

As shown  above, the  starting  sector address  for  read, write  and verify

requests is passed as a physical address including a 10-bit Cylinder Number,

an 8-bit Head Number, and a 6-bit  Sector Number. SCSI devices are addressed

by logical block address without regard to the physical layout of the drive.

For this reason, all SCSI devices are defined as drives with 64 heads and 32

sectors per track and with the appropriate number of cylinders to provide the

full capacity of the  drive using this physical  to logical translation. The

SCSI logical address is formed by the BIOS by concatenating the values as show

below:

Cylinder(10 bits) || Head(6 bits) || Sector(5 bits) = SCSI Logical Block Address

5-2

Section Five                                Single-Threaded Software Interface

------------------------------------------------------------------------------

5.2.2 BIOS Command Return Codes

The BIOS command, when complete, returns control to the requesting program at

the next instruction after the  software interrupt. A return  code is set in

the Carry Flag (CF) and a status code is placed in register AH for examination

by the requesting  program. The  expected return  codes placed  in the Carry

Flag follow.

CF Contents at Return      Return Code

---------------------------------------

CF = 1 Status is non-zero. Unusual condition or error detected.

CF = 0 Status is zero.     Normal command completion occurred.

AH Contents at Return Status of Operation

-----------------------------------------

AH = 00H              No error: The operation completed successfully.

AH = 01H              Invalid Function Request: The operation

code or an associated parameter provided

by the  Interrupt 13 operation  was not

valid.

AH = 02H              Unable to Read Address  Mark: One  of the

following additional  sense  codes (SCSI

ASC)  was   presented   in   the   sense

information returned by the REQUEST SENSE

command:

12H (No AM Found on Data Field)

21H (Illegal Logical Block Address)

AH = 03H              Write Protect Error: Returned SCSI ASC was:

27H (Data Protect)

AH = 04H              Read Error: Returned SCSI ASC during a

read operation was:

14H (No Record Found)

16H (Data Sync Error)

AH = 10H              Uncorrectable ECC Error in Data: Returned

SCSI ASC during a read operation was:

10H (ID ECC Error)

11H (Unrecovered Read Error)

AH = 11H              ECC Corrected Data Error: Returned SCSI

ASC during a read operation was:

17H (Recovered Read Error w/o ECC)

18H (Recovered Read Error w/ ECC)

AH = 20H              General Controller Failure: A host

adapter detected failure occurred or one

of the following SCSI ASC's was returned:

5-3

Section Five                                Single-Threaded Software Interface

------------------------------------------------------------------------------

01H   08H    40H     46H

03H   09H    41H     47H

05H   1BH    42H     48H

06H   1CH    43H     49H

07H   1DH    44H

AH = 40H              Seek Operation Failed: Returned SCSI ASC was:

15H  (Seek Positioning Error)

02H  (No Seek Complete)

AH = 80H              Selection Time-out: A host adapter does not

respond to BIOS.

AH = AAH              Device Not Ready: Returned SCSI ASC was:

04H  (LUN Not Ready)

28H  (Unit Attention, Ready)

29H  (Unit Attention, Power On)

2AH  (Unit Attention, Mode Select Change)

AH   =   BBH          Unknown Target Sense Error Occurred: A

SCSI ASC other than those specified above

was  returned from the selected SCSI device.

AH = CCH              Write Error:  Not Returned

AH = FFH              Sense Operation Failed: BIOS unable to

retrieve sense information from target.

5.3 BIOS DISK COMMANDS

The commands that  can be  accepted from  the operating  system by  the BIOS

include all those commands required for normal DOS operation, for booting of

the DOS operating system, for the booting of other operating systems and for

basic maintenance and verification of normal disk operation. The command set

is summarized in table 5-2 and each command is described in more detail in the

sections following.

AH Register            Meaning

00 H                Reset Disk System

01 H                Read Status of Last Operation

02 H                Read Desired Sectors into Memory

03 H                Write Desired Sectors from Memory

04 H                Verify Desired Sectors

06 H                Identify SCSI Devices

08 H                Read Drive Parameters

09 H                Initialize Drive Pair Characteristics

0C H                Seek

0D H                Alternate Disk Reset

10 H                Test Drive Ready

11 H                Recalibrate

15 H                Read DASD Type

5-4

Section Five                                Single-Threaded Software Interface

------------------------------------------------------------------------------

TABLE 5-2

The Format commands  (AH =  05, 06, 07,  and 1AH)  are not supported  by the

AHA-1640 BIOS.  The format  operation is  performed  under a  special format

utility installed in ROM and initiated through the DEBUG program.

The Diagnostic Reserved commands (AH = 0A, 0B,  0E, 0F, 12, and 14H) are not

supported by the AHA-1640 BIOS.

The Park Heads command (AH = 19H) is not supported by the AHA-1640 BIOS.

The Reserved for Diskette commands (AH = 16, 17, 18) and the commands from 1BH

up to FFH are not supported by the AHA-1640 BIOS. Any attempt to execute any

of these unsupported commands will be ended by the BIOS and Carry Flag will be

set to one. An error code of 01H (Invalid Function Request) will be returned

in the AH register.

5.3.1 Reset Disk System

The BIOS is requested to reset the disk subsystem. A SCSI Reset operation is

performed on the SCSI bus if the bus is busy at the time the Reset Disk System

command is  received by  the host  adapter.  No other  SCSI or  host adapter

activity occurs. The BIOS command is then  passed on to the standard BIOS so

that any internally installed hard disks and floppy disk devices can also be

reset. If the  BIOS is operating  concurrently with  an internally installed

hard disk, the drive number is temporarily decremented by the AHA-1640 BIOS so

that the  standard BIOS  does not  attempt to  reset the  drive that  is not

installed.

Input Parameters:       AH = 00H

DL = Drive Number (80H or 81H)

Output Parameters:      AH = Status of Operation

CF = Return Code

5.3.2 Read Status of Last Operation

The status of the last operation performed to the specified disk is returned.

No SCSI activity occurs. The disk status is reset to zero.

Input Parameters:      AH = 01H

DL = Drive Number (80H or 81H)

Output Parameters:     AH = Status of Operation

CF = Return Code

5.3.3 Read Desired Sectors Into Memory

The sectors requested by the parameters are read from the disk to the system

memory. The  SCSI command  is executed  as a  READ (EXTENDED)  command (SCSI

5-5

Section Five                                Single-Threaded Software Interface

------------------------------------------------------------------------------

Operation Code 28H). If the operation fails and an error is reported through

the BIOS status byte, the operation should  be retried one time. If a status

of ECC Corrected Data Error (AH=11H) is returned, the data returned has been

corrected and may be used with confidence.

Input Parameters:      AH = 02H

DL = Drive Number (80H or 81H)

DH = Head

CH = Cylinder

CL = High Cylinder and Sector

AL = Number of Sectors to Read

ES:BX = Address of buffer area

Output Parameters:     AH = Status of Operation

CF = Return Code

5.3.4 Write Desired Sectors from Memory

The sectors requested by the parameters are written from the system memory to

the indicated  disk. The  SCSI  command is  executed  as a  WRITE (EXTENDED)

command (SCSI Operation Code  2AH). If the  operation fails and  an error is

reported through the BIOS  status byte, the operation  should be retried one

time.

Input Parameters:      AH = 03H

DL = Drive Number (80H or 81H)

DH = Head

CH = Cylinder

CL = High Cylinder and Sector

AL = Number of Sectors to Write

ES:BX = Address of buffer area

Output Parameters:     AH = Status of Operation

CF = Return Code

5.3.5 Verify Desired Sectors

The sectors defined by the parameters are verified to be correctly written on

the SCSI  disk. The  SCSI  command is  executed  as a  VERIFY  command (SCSI

Operation 2F) with the Byte Check bit set  to zero. If the VERIFY command is

not supported by the selected disk, the Verify Desired Sectors will perform a

SCSI READ command (SCSI Operation Code 28H) and throw away the received data.

If the operation fails and an error is reported through the BIOS status byte,

the operation should be retried one time.

Input Parameters:      AH = 04H

DL = Drive Number (80H or 81H)

DH = Head

CH = Cylinder

CL = High Cylinder and Sector

AL = Number of Sectors to Verify

Output Parameters:     AH = Status of Operation

CF = Return Code

5-6

Section Five                                Single-Threaded Software Interface

------------------------------------------------------------------------------

5.3.6 Identify SCSI Devices

This is a special AHA-1640 BIOS call that is used to return the number of the

first supported SCSI drive. The BIOS call returns information obtained during

initialization of the BIOS  to determine what SCSI  drives are available. If

the Carry Flag is set, the 1640 BIOS  is not installed or no SCSI drives are

available under the  BIOS. In  the nonconcurrent  case (2  SCSI drives), the

returned value will be 80H. If only one SCSI drive is installed, the returned

value will be 81H.

Input Parameters:      AH = 06H

Output Parameters:     AH = Status of Operation

not set

BX = First Drive Supported

80H           Nonconcurrent operation

81H           Concurrent operation

CF = Return Code

0             BX information is valid

1             BIOS not installed or no BIOS SCSI drives

5.3.7 Read Drive Parameters

A SCSI READ CAPACITY command is used to determine the maximum logical block of

the selected drive.  This information is  then used to  calculate the proper

number of cylinders to be  returned to the host system.  The number of heads

returned will always be 64 and the number of sectors per track will always be

32. The number  of drives  returned will  include both  internally installed

drives and SCSI drives.

Input Parameters:      AH = 08H

DL = Drive Number (80H or 81H)

Output Parameters:     AH = Status of Operation

DL = Number of BIOS accessible drives attached (1 or 2)

DH = Maximum value for head number (3FH)

CH = Maximum value for Cylinder Range (Low Byte)

CL = Maximum value for Sector, High Cylinder Bits

Bits 7,6         High Order Cylinder Bits

Bits 5-0         Sector Number Maximum Value ( 20H)

CF = Return Code

5.3.8 Initialize Drive Pair Characteristics

This command performs no operation, since SCSI CCS drives are self configuring.

Input Parameters:      AH = 09H

DL = Drive Number (80H or 81H)

Output Parameters:     AH = Status of Operation

CF = Return Code

5-7

Section Five                                Single-Threaded Software Interface

------------------------------------------------------------------------------

5.3.9 Seek

A seek to the  logical block address  defined by the  physical parameters is

performed. The seek  is performed  using the  SEEK (EXTENDED)  command (SCSI

Operation 2BH). The SEEK commands are not mandatory CCS commands and are not

required for proper functioning of SCSI devices. If the addressed disk drive

reports an error indicating that the SEEK (EXTENDED) command is not supported,

the BIOS command will complete normally anyway.

Input Parameters:      AH = 0CH

DL = Drive Number (80H or 81H)

DH = Head

CH = Cylinder

CL = High Cylinder. (Sector bits = 0)

Output Parameters:     AH = Status of Operation

CF = Return Code

5.3.10 Alternate Disk Reset

The BIOS is requested to reset the disk subsystem. A SCSI Reset operation is

performed on the SCSI bus if the bus is busy at the time the Reset Disk System

command is  received by  the host  adapter.  No other  SCSI or  host adapter

activity occurs. The BIOS command is then  passed on to the standard BIOS so

that any internally installed hard disks and floppy disk devices can also be

reset. If the  BIOS is operating  concurrently with  an internally installed

hard disk, the drive number is decremented  by the AHA-1640 BIOS so that the

standard BIOS does not attempt to reset the drive that is not installed.

Input Parameters:      AH = 00H

DL = Drive Number (80H or 81H)

Output Parameters:     AH = Status of Operation

CF = Return Code

5.3.11 Test Drive Ready

The BIOS command determines that the specified drive is available and ready by

executing a TEST UNIT  READY command (SCSI  Operation Code 00H)  to the SCSI

device. The command may have to be executed a second time if a Unit Attention

condition was detected in the first execution of the TEST UNIT READY.

Input Parameters:      AH = 10H

DL = Drive Number (80H or 81H)

Output Parameters:     AH = Status of Operation

CF = Return Code

5-8

Section Five                                Single-Threaded Software Interface

------------------------------------------------------------------------------

5.3.12 Recalibrate

The BIOS command transmits a REZER0 UNIT command (SCSI Operation Code 01H) to

the specified drive. Since the REZERO UNIT command is not a mandatory command

and is not  required for  proper functioning  of the  SCSI device,  the BIOS

command will complete  without indicating an  error even if  the SCSI device

indicates that the command is invalid.

Input Parameters:      AH = 11H

DL = Drive Number (80H or 81H)

Output Parameters:     AH = Status of Operation

CF = Return Code

5.3.13 Read DASD Type

The AHA-1640  BIOS  executes this  command  by performing  an  INQUIRY (SCSI

Operation Code 12H)  and a  READ CAPACITY (SCSI  Operation Code  25H) to the

selected SCSI drive. The INQUIRY command is used to verify that the device is

a Direct Access Storage Device.  The READ CAPACITY is  used to determine the

number of logical blocks available. A special return format is used to obtain

the required information.

Input Parameters:      AH = 15H

DL = Drive Number (80H or 81H)

Output Parameters:     AH = Status of Operation (Special Format)

00H      Drive not present or DL invalid

01H      Reserved

02H      Reserved

03H      Fixed Disk installed

CX,DX    Number of 512 byte blocks available on disk

CF = Return Code

5-9

Section Six                                                    Device Drivers

------------------------------------------------------------------------------

6.1 PS/2 COMPATIBLE POS INITIALIZATION

Section 3 describes the initialization procedure required to enable the host

adapter and to program the address spaces, interrupts, and hardware controls

associated with the Programmable Option Select. The Adapter Description File

(ADF) required for  the initialization procedure  is named  @0F1F.ADF and is

normally included on  the System  Reference Diskette  for the  host computer

system. If desired, a specialized ADF can be prepared using the POS register

information specified in  Section 4.2  and the  format specified  by the IBM

Technical Reference manuals.

Once the installation process is complete, the initialization program in the

system BIOS automatically obtains the required POS information from the CMOS

Ram non-volatile memory in the host system and transfers it to the correct POS

registers whenever the  system is  powered up.  No operator  intervention is

required to initialize the adapter at power-up time.

6.2 PS/2 COMPATIBLE BIOS

The PS/2 Compatible BIOS is provided to allow the user to use the AHA-1640 in

lieu of, or in addition  to, a standard Micro  Channel hard disk controller.

This compatibility includes booting from a SCSI device. Hard disk requests are

sent to the  BIOS via  an INT 13  software interrupt.  CPU register contents

specify the desired I/O function and associated parameters. The BIOS then uses

host adapter IOCP commands to emulate  Micro Channel functions. Again, since

the mailbox architecture is circumvented, Micro Channel BIOS functions run in

a single-threaded mode.

The BIOS consists of three major functional components. These are:

1. Initialization Routine

2. System Boot Routine

3. Hard Disk I/O Routine

6.2.1 Initialization

The Initialization routine is called at power-up. This routine performs all of

the necessary initialization functions and will display the following message

if the system successfully recognizes the AHA-1640 BIOS.

Adaptec AHA-1640 BIOS

Copyright 1988 Adaptec, Inc.

All Rights Reserved.

If any subsequent initialization of the AHA-1640 fails, the following message

is displayed:

HOST ADAPTER NOT FOUND AT PORT [address specified in POS registers].

6-1

Section Six                                                    Device Drivers

------------------------------------------------------------------------------

Otherwise, the AHA-1640 BIOS chooses a course of action based upon the number

and types of hard disks already installed.  In each case, the number of hard

disks already installed is displayed  along with the ID  numbers of any SCSI

devices that are successfully used by the BIOS. No error message is displayed

if a second SCSI drive is not available.

Possible Messages:

HARD DISK C: ALREADY INSTALLED.

HARD DISK D: ALREADY INSTALLED.

HOST ADAPTER DIAGNOSTIC FAILURE.

SCSI DEVICE 0 : 0 NOT FOUND.

USING SCSI DEVICE 0 : 0 AS HARD DISK C:

USING SCSI DEVICE 0 : 0 AS HARD DISK D:

USING SCSI DEVICE 0 : 1 AS HARD DISK D:

USING SCSI DEVICE 1 : 0 AS HARD DISK D:

If the system reports an  initial number of hard drives  greater than 2, the

following message is displayed:

SYSTEM CONFIGURATION ERROR.

The AHA-1640 BIOS allows the use of zero, one, or two hard disks on the SCSI

bus under DOS without a device driver.

If two hard disks are attached to  an internal hard disk controller, and are

already recognized by the system, the BIOS does nothing. A device driver will

be needed to access SCSI devices.

If one hard  disk is attached  to an  internal hard disk  controller, and is

already recognized by the system, the BIOS attempts to assign the SCSI device

(Target : LUN) 0 : 0 as the second drive. (drive D:) If this is unsuccessful,

an initialization error is reported to the system.

If there are zero  hard disks attached  to an internal  controller, the BIOS

attempts to assign the first SCSI  device (Target : LUN) 0  : 0 as the first

drive (drive C:). If this is unsuccessful, an initialization error is reported

to the system. If successful, the BIOS attempts to assign another SCSI device

as the second drive.  (drive D:) Device  0 : 1  is tried first,  and if that

fails, device 1 : 0 is tried. There is no initialization error reported if a

second drive is not available.

6.2.2 System Boot

System boot is invoked by  execution of an Interrupt  19. No interception of

this interrupt is necessary, as the PS/2 BIOS manages the boot process. When

the boot procedure attempts to read the boot track from drive C:, the AHA-1640

BIOS will respond if drive  C: is a SCSI Drive.  Motherboard BIOS's that use

interrupt 13 to perform booting should boot  normally from SCSI drives using

the AHA-1640.

6.2.3 Hard Disk I/O

The BIOS enables up  to two SCSI drives  to be accessed  via interrupt 13 as

described in Section Five.

6-2

Section Six                                                    Device Drivers

------------------------------------------------------------------------------

6.3 XENIX/UNIX DRIVERS

SCO  XENIX  support  will  be   available  at  production  release.  Generic

(non-bootable) Xenix and Unix drivers are available for OEM integration.

6.4 DOS DRIVER

An optional  DOS Driver  is available  that enables  the AHA-1640  to access

additional SCSI devices on the SCSI bus.  When operating under DOS, a device

driver is needed to access  more than two hard disks,  tape devices or other

types of SCSI peripherals. If all DOS operations will be confined to two SCSI

disks or one internal and one SCSI disk, a device driver is not needed because

of the on-board BIOS.

The DOS driver is installed by creating a DOS configuration (CONFIG.SYS) file

that calls the device driver at boot time. Please refer to your DOS manual for

information on creating  a CONFIG.SYS file.  At initial  system loading, the

AHA-1640 device driver will determine the number of hard disks attached to the

system. The device driver is normally only installed if one of the following

cases is true.

1) There are two internal hard disks and one or more SCSI disks.

2) There is one internal hard disk and two or more SCSI disks.

3) There are no internal hard disks and more than two SCSI disks.

Table 6-1 shows how hard disks in a system are addressed:

TABLE 6-1. DISK CONFIGURATIONS FOR DOS WITHOUT DRIVERS

[Picture]

The SCSI disk drives that are accessed by the device driver are

divided into  32 Mbyte  logical  disks. There  can  be multiple

logical disks on one physical SCSI disk drive. A utility program

is also provided that performs the functions of the DOS FDISK and

FORMAT utilities. This utility will perform FDISK and FORMAT to

SCSI disks that can only be accessed through the device driver.

These utility  programs  partition the  disks  and  create file

directories so that DOS  files may be  copied and accessed from

SCSI drives. Please refer to your DOS manual for details of FDISK

and FORMAT.

SCSI II compatible and QIC 104 command set 1/4" tape drives are

also be supported by the DOS device driver. The user accesses a

SCSI tape drive by building a CCB as described in Section Five.

The device driver passes the CCB  to the AHA-1640 and instructs

the AHA-1640 to complete the command.

6-3

Section Six                                                    Device Drivers

------------------------------------------------------------------------------

6.5 OS/2 DRIVER AVAILABILITY

Contact Adaptec for information on OS/2 support.

6.6 VIRTUAL MEMORY DRIVER

Programs that make  use of the  V86 mode (Virtual  Mode) of the

80386 while using the  interrupt 13 interface  of the BIOS will

require a  special software  driver.  The driver  functions are

already included in the Adaptec DOS  driver. If the system does

not make use of the Adaptec DOS driver, a special software driver

is available that  will allow this  type of  application to run

under DOS.  Installation  instructions  are  provided  with the driver.

6.7 AHA-1640 ON-BOARD UTILITIES

The AHA-1640 BIOS contains a utility program that allows the user

to perform several important functions. The utility programs are

accessed using a self-explanatory menu.

The utilities  are accessed  using the  DOS Debug  function. To

access the utilities, boot using a  standard DOS system disk or

diskette. When the  > prompt symbol  is appears  on the screen,

enter the debug program by typing debug and pressing Enter. The

debug program will load and present a new cursor, the - symbol.

Enter the following line and press enter to invoke the On-board

Utility program.

G=dc00:6

If a BIOS base address  other than DC000H is  being used on the

adapter to be tested, replace the dc00 parameter with the actual

BIOS base segment. The segment  is the upper 16  bits of the 20

bit BIOS base address.

The menu options include the following items which are selected

by pressing the index number of the menu function.

(1) A list of all devices installed on the SCSI bus may be requested.

This information is retrieved via a RETURN INSTALLED DEVICES host

IOCP command, followed by INQUIRY commands to all available

targets.

(2) A device may be selected for execution of the following two

functions. Pressing the '2' key  toggles the selection from one

installed device to the next.

(3) A low-level, or primary, format of the selected Target/LUN can be

requested. A MODE SELECT is issued first to set the block size at

512 bytes per sector. Then a FORMAT command is issued which will

format the drive with factory as well as grown defects.  Only

one SCSI drive can be formatted at a time.

6-4

Section Six                                                    Device Drivers

------------------------------------------------------------------------------

(4) Integrity of a target can be checked by verifying an entire

drive. Any correctable errors found will automatically be handled

through the SCSI Reassign Blocks Command. If any uncorrectable

errors are found the user will first be asked via a screen prompt

if reassignment of the effected block should be performed.

(5) The user can leave  the AHA-1640 On-board Utility functions

and return to normal DOS operation.

6.8 THIRD PARTY SOFTWARE SUPPLIERS FOR AHA-1640

This is a partial list of  the third party software and support

suppliers for the AHA-1640.

SCO Xenix

The Santa Crux Operation, Inc.

400 Encinal Street

P.O. Box 1900

Santa Cruz, CA 95061

(408) 425-7222

Unix

Interactive Systems Corporation

2401 Colorado Avenue

Santa Monica, CA 90404

(213) 453-8649 (Contact Sales Department)

Unix

Microport Systems, Inc.

10 Victor Square

Scotts Valley, CA 95066

(800) 722-8649

SYTOS (SYTRON Tape Operating System)

Sytron Corporation

135 Maple Street

Marlboro, MA 01752

East Coast : (617) 460-0106  (Contact William Mason)

West Coast: (415) 558-8533  (Michael Evans)

Novell 2.1

Ontrack Computer Systems

6200 Bury Drive

Eden Prairie, MN 55346

(612) 937-11-7

6-5

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

7.1 SCSI DESCRIPTION, INITIATOR MODE

The AHA-1640 provides  a very  high performance  SCSI interface

connection.  The   host   adapter  meets   the   ANSI  Standard

X3.131-1986, describing the SCSI. The host adapter additionally

meets conformance level 2 of  the specification by implementing

the following SCSI options:

Accepts or manages the following messages:

Command Complete

Disconnect

Message Reject

Identify

Save Data Pointer

Performs arbitration

The following alternatives  are selected for  the AHA-1640 SCSI

interface connection from those described by X3.131-1986.

Single-ended SCSI driver/receivers are used.

The Termination Power is supplied through a removable fuse,

allowing the user to select whether or not termination power

is provided at installation time.

Parity is always generated. Parity checking can be disabled

through the POS registers.

The "Soft" Reset option is always performed by the AHA-1640.

If the user desires the "Hard" Reset, an interrupt servicing

program that detects the SCSI  Reset Detected interrupt in

the Interrupt Flags Port must set the Soft Reset bit in the

Control Port. This bit forces the host adapter to clear all

SCSI related operations, but does not require execution of

the diagnostic functions.

Synchronous data transfer is supported. Negotiation for the

synchronous transfer initiated by another SCSI device will

be accepted  by  the  AHA-1640 at  any  time.  If  the POS

registers have been initialized to  allow it, the AHA-1640

will also initiate  synchronous data  transfer negotiation

when it has detected that such negotiation may be required.

Multitasking is fully supported.

Modify Data Pointers is supported to allow Zero Latency Read

operations.

In addition to these SCSI functions, the SCSI Common Command Set

at level  4B  is  supported  in  the  BIOS  for single-threaded

7-1

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

operation. While the CCS document was never made a standard, it

describes a widely available set  of disk drive functions which

are supported  by  the  host  adapter.  The  Host  Adapter BIOS

commands are all  mapped in to  SCSI CCS commands  to allow the

proper support of all the most common SCSI disk drives. The IOCP

command "Return Installed  Devices" also  uses CCS  commands to

determine which devices are available. The CCS commands used are

compatible with the SCSI-2 draft documents.

The draft ANSI standard for SCSI-2 has been used as a reference

for the implementation of all SCSI functions with the expectation

that the host adapter  will be fully  compatible with the final

version of the  SCSI-2 standard.  In particular,  the Processor

type device command set has been selected from the SCSI-2 manual

for Target Mode operation.

7.1.1 Linked SCSI Commands

The AHA-1640 supports linking of SCSI commands. When the link bit

in the SCSI command control byte is set, the target will present

either a 0A or  0B message at  command completion. The AHA-1640

uses the the link pointer in a CCB to fetch another CCB. At the

same time, the completed CCB status and address are stored in an

MBI. If the target returns the 0B status, the AHA-1640 generates

an interrupt to inform the host of the full MBI. The linked CCBs

must address the  same target and  LUN since the  target is not

re-selected.

7.1.2 Zero Latency Read Operation

The AHA-1640 implements Zero Latency operation through the use of

modify  data  pointer  messages.  Zero  Latency  can  eliminate

rotational latency, depending on the length of the data transfer,

by supporting out of order data transfers. This advanced feature

is currently  only implemented  on the  ACB-4525Z SCSI  to ESDI

controller, and is supported transparently to the user.

Since the Adaptec host  adapter family is  the first to support

Zero Latency Read operation, this section is intended to briefly

explain the required target support. Please refer to the Adaptec

ACB-4525Z SCSI-to-ESDI controller manual for complete details of

Zero Latency Read operation.

After seeking to the target track,  the ACB-4525Z or other disk

device having the Zero Latency  Read feature will begin reading

block IDs. If the  first block ID  is within range  of the data

transfer, but is not the last block of the the data transfer, the

ACB-4525Z will  begin reading  the  subsequent blocks  into its

buffer. Before transferring  data, the  ACB-4525Z will  issue a

MODIFY DATA POINTER  message to  the AHA-1640.  This supplies a

positive number that is added to  the value of the current data

pointer. The  ACB-4525Z  will now  send  the data  to  the host

beginning at this point in the data blocks to be transferred. The

ACB-4525Z will resume reading data into its buffer as soon as the

first block of the data blocks to be transferred is found. Before

sending this data to the host, the ACB-4525Z will issue a second

MODIFY DATA POINTER message to the AHA-1640 which adds a negative

number to the value  of the current data  pointer to return the

data pointer  to  the  beginning  of  the  data  blocks  to  be

transferred. This guarantees that a data transfer of one track or

less will never require more than a single revolution since data

can now be transferred out of order.

7-2

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

7.2 SCSI DESCRIPTION, TARGET MODE

7.2.1Initiator Conformance Level Requirements

Initiators that execute commands  against an AHA-1640 operating

in Target Mode are  required to have  the following conformance

levels, as described in  Appendix E of  the SCSI Specification,

ANSI X3.131-1986. Conformance  must be present  with respect to

each of the following items:

The initiator must use single-ended drivers.

Termination power may optionally be  provided by the initiator,

but must meet the  SCSI specification in  both its over-current

protection  and  its  reverse  current  diode  protection.  The

terminators may be installed on the AHA-1640 board or installed

as in-line terminators at the cable connectors.

The implementation of parity is optional, but desirable.

The initiator may support either  "hard" reset or "soft" reset.

All attached devices must support the same type of reset.

The initiator must  meet the requirements  of conformance level

two. In particular, all LUN addressing must be performed by the

IDENTIFY message, not by the LUN field in the CDB.

Disconnection and reconnection must be supported.

The initiator  and  target  functions have  the  same  SCSI ID,

selected by the POS Registers.

7.2.2 Support of Synchronous Transfer

Synchronous transfer is supported by the AHA-1640 in target mode

without any instruction or support from the system processor. If

an initiator  invokes a  synchronous transfer  negotiation, the

AHA-1640 will complete the negotiation of the required transfer

offset and period. If the proper POS register bits have been set,

the AHA-1640 will also attempt to negotiate synchronous transfer

during the initial selection period of the first command after an

initialization or after a SCSI reset.

7.2.3 SCSI Target Operation in Processor Target Mode

When the AHA-1640 has  been set to  respond in Processor Target

Mode, the  host adapter  appears on  the SCSI  bus as  a normal

processor type device as defined by the SCSI specification. From

one to eight LUNs  may be supported, depending  on the LUN mask

byte in the Enable  Target Mode command.  Six SCSI commands are

accepted:

TEST UNIT READY

REQUEST SENSE

INQUIRY

SEND

RECEIVE

SEND DIAGNOSTIC

7-3

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

All other commands are rejected with CHECK CONDITION status. The

sense information  will indicate  a Sense  Key of  05H (Illegal

Request) with a  Sense Code  of 20H  (Invalid Command Operation Code).

The commands that do  not perform data transfer  to or from the

host are  handled  completely  by  the  AHA-1640  with  no  CCB

communication with the host system. Those commands are the TEST

UNIT READY, REQUEST SENSE, SEND DIAGNOSTIC and INQUIRY commands.

The SEND  and  RECEIVE  commands must  either  already  have or

successfully request  a  CCB  from  the  host  with  the proper

direction bits, the proper initiator address, and the proper LUN

number to complete the SCSI operation.

Each time an initiator activates a  command to the AHA-1640, an

internal sub-channel is  activated to  manage the  command. The

sub-channel  is  dedicated  to  that  particular  LUN-Initiator

transaction until all operations associated with the command are

completed. Such operations include disconnection to obtain a CCB

and pending error conditions.  If all sub-channels  are busy, a

selection to the AHA-1640 will result in the AHA-1640 accepting

the command and then generating BUSY status immediately. If this

occurs, the initiator  must re-issue  the command  later. There

are 112 sub-channels available, so it is very unusual for a busy

indication to be required.

7.2.3.1 Test Unit Ready

[Picture]

FIGURE 7-1. COMMAND DESCRIPTOR BLOCK FOR TEST UNIT READY

The TEST UNIT READY command  follows the SCSI specification. If

the AHA-1640  has been  initialized by  the Enable  Target Mode

command to the Processor  Target Mode, the  command will finish

normally with GOOD status  and a COMMAND  COMPLETE message. The

host adapter supports the normal definition of UNIT ATTENTION on

the first operation after power on, after a SCSI Reset, or after

a BUS DEVICE RESET.

If a TEST UNIT READY command is executed against an LUN which was

not allowed  by  the  Enable Target  Mode  command,  then CHECK

CONDITION status will be presented with sense data of Sense Key 5

(Invalid Request) and an Error Code of 25H (Invalid LUN).

7-4

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

7.2.3.2 Request Sense

[Picture]

FIGURE 7-2. COMMAND DESCRIPTOR BLOCK FOR REQUEST SENSE

If the AHA-1640 has returned CHECK CONDITION status to a previous

command, the  REQUEST  SENSE  command  will  obtain  the  sense

information associated  with the  error. The  sense information

will be sent in the extended sense format according to the SCSI

standard. The data format is given in the following Figure:

[Picture]

FIGURE 7-3. REQUEST SENSE DATA

7-5

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

The following errors are detected and presented by the AHA-1640

while operating in Processor Target Mode:

Error                               Sense       Sense

Key         Code

No Sense Data                        00          00

(Incorrect Length Indication may be present)

Invalid Command Operation Code       00          20

Invalid LUN                          05          25

Invalid Command Parameter            05          26

Power Up Attention                   06          29

Reset Attention                      06          29

Interface Parity Error               0B          47

Initiator Detected Error             0B          48

One set of error data  may be buffered for  each initiator - LU

association possible, up to a total of 56 sets of sense data. No

Contingent Allegiance or Extended Contingent Allegiance state is

established.

7-6

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

7.2.3.3 Inquiry

[Picture]

FIGURE 7-4. COMMAND DESCRIPTOR BLOCK FOR INQUIRY

The INQUIRY  command  provides  the  information  necessary  to

uniquely identify  the  Adaptec  AHA-1640  as  a Processor-type

device. The following information is  returned to any selecting

initiator from any selected AHA-1640 Logical Unit:

[Picture]

FIGURE 7-5. INQUIRY DATA

7-7

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

If a length shorter than the  required 36 bytes is specified by

the INQUIRY command, the number of bytes specified by the command

is transferred. If a length longer  than 36 bytes is specified,

the command will only transmit 36 bytes.

7.2.3.4 Send Diagnostic

[Picture]

FIGURE 7-6. COMMAND DESCRIPTOR BLOCK FOR SEND DIAGNOSTIC

The SEND  DIAGNOSTIC  command  invokes  a  subset  of  the host

adapter's initial power-on diagnostics to  verify that the host

adapter is fully functional. The  only diagnostic function that

may be requested is  the Self Test function.  A request for any

other function will cause an error of Invalid Command Parameters

to be posted. Normal  completion of the  diagnostic function is

indicated by GOOD status. Detection of a failure is indicated by

CHECK CONDITION status and the posting of the appropriate error

condition.

7.2.3.5 Send

[Picture]

FIGURE 7-7. COMMAND DESCRIPTOR BLOCK FOR SEND

The SEND command has the format specified by the SCSI standard.

7-8

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

The SEND command transfers data from the initiator to the target.

The  information  is  placed  in  the  area  specified  by  the

appropriate CCB.  If an  appropriate CCB  has not  already been

provided to the host adapter by the host software, an MBI entry

requesting the appropriate CCB is sent to the host from the host

adapter. In this case, the target host adapter disconnects from

the SCSI until  the CCB is  made available to  the host adapter

through the MBO protocol. An appropriate CCB must have the same

initiator address, target LUN, and  direction as is required to

complete the command.

The transfer length in the SEND command specifies the length in

bytes of data that is sent during the DATA OUT phase. A transfer

length of zero  indicates that no  data is  sent. Management of

incorrect length transfers is described in Section 7.2.4.

The AHA-1640 supports the SEND command  with or without the AEN

bit set. The CDB information is included in the returned CCB so

that the receiving host programming can determine whether or not

the information transmitted by the SEND command was application

data or asynchronous event notification data.

7.2.3.6 Receive

[Picture]

FIGURE 7-8. COMMAND DESCRIPTOR BLOCK FOR RECEIVE

The RECEIVE command has the format specified by the SCSI standard

The RECEIVE  command  transfers  data from  the  target  to the

initiator. The information is taken  from the area specified by

the appropriate CCB. If an appropriate CCB has not already been

provided to  the  host  adapter  host  software,  an  MBI entry

requesting the appropriate CCB is sent to the host from the host

adapter. In this case, the target host adapter disconnects from

the SCSI until  the CCB is  made available to  the host adapter

through the MBO protocol. An appropriate CCB must have the same

initiator address, target LUN, and  direction as is required to

complete the command.

The transfer length in the RECEIVE command specifies the length

in bytes  of data  that is  sent  during the  DATA IN  phase. A

transfer length  of  zero  indicates  that  no  data  is  sent.

Management of incorrect length transfers is described in Section

8.3.2.6.

7-9

Section Seven                                                    SCSI Features

------------------------------------------------------------------------------

7.2.4 Incorrect Length Management

The messages transmitted using the SEND and RECEIVE commands are

normally expected  to have  a length  that has  previously been

agreed to by  the initiator system  software and  by the target

system software. If the transfer length specified by the command

is equal to  the transfer length  specified by  the CCB, normal

operation takes place and no errors are posted.

If the transfer length specified  by the initiator's command is

shorter than  the  space defined  by  the CCB,  all  data bytes

expected and required by the initiator will be transmitted. The

AHA-1640 will indicate GOOD status at  the end of the transfer.

The target system's software however  must be notified that the

entire area of data  defined by the CCB  was not transmitted. A

Target Status of GOOD will be presented in the returned CCB. At

the same time,  the Incorrect Length  Indication bit  (bit 5 of

byte 2) will be set in the CCB Request Sense data area. Bytes 3

through 6 of the Request Sense data area will contain the residue

in two's  complement notation  of the  length requested  in the

initiator command minus the length of  the data area defined by

the CCB. In this  case, the residue will  be a negative number,

since the requested length was less  than the area specified by

the CCB. The MBI Status  Byte will be set  to 04 (CCB Completed

with Error) to indicate that the CCB was completed with an error.

If the transfer length specified  by the initiator's command is

longer than  the space  defined by  the  CCB, only  those bytes

contained  within  the   CCB's  data  transfer   area  will  be

transmitted. The AHA-1640 must indicate with an error condition

to the  initiator that  not all  the  requested bytes  could be

transferred. The AHA-1640 indicates a CHECK CONDITION at the end

of the data transfer. The Request Sense information transmitted

to the initiator as a result of an immediately following REQUEST

SENSE command indicates that an  Incorrect Length Indication is

present by setting bit  5 of byte  2. Bytes 3  through 6 of the

Request Sense  information  transmitted to  the  initiator will

contain the residue in two's  complement notation of the length

requested in the initiator command minus the length of the data

area defined by the  CCB. In this  case, the residue  will be a

positive number, since the requested length was greater than the

available area.  The  target  system's  software  must  also be

notified that the  transfer length  requested by  the initiator

exceeded the  assigned buffer  area. A  Target Status  of CHECK

CONDITION will be  presented in the  returned CCB.  At the same

time, the information that will later be posted to the initiator

by the REQUEST SENSE command will be posted to the target system

in the  CCB Request  Sense data  area.  This includes  both the

Incorrect Length Indicator and the Residue. The MBI Status Byte

will be set to 04 to indicate that the CCB was completed with an

error.

7-10

Section Eight                                           Problem Determination

------------------------------------------------------------------------------

8.1 SELF DIAGNOSTICS

The AHA-1640 executes self diagnostics upon power-up or after a

hard reset. These  diagnostics test  the CPU  operation, verify

that the control program information is correctly recovered from

the firmware EPROM,  and check the  data transfer  paths on the

board. If the self diagnostics are not successful, the AHA-1640

will not identify itself during  the Programmable Option Select

process and will therefore not be enabled.

The red light emitting diode (LED) on the host adapter indicates

the result of the self diagnostic  process. When power is first

applied to  the  board,  the  LED turns  on.  If  the  board is

operating normally, the light will soon go off and stay off until

SCSI activity is  requested by  the host.  If the board  is not

operating correctly, a flash code number  is flashed on the LED

to indicate  which  test  failed.  The  flash  code  number  is

indicated by a series of 1 to 6 closely spaced flashes followed

by a longer pause. The flash code is repeated continuously until

the board is powered off, reset or repaired. The SCSI interface

should be disconnected if these  diagnostic tests are being run

for fault  isolation purposes.  The  flash code  and associated

failure modes are indicated in table 8-1.

TABLE 8-1. TABLE OF FLASH CODES

FLASH CODE        POSSIBLE MEANINGS OF FLASH CODE

LED Remains On        Host Adapter Control Processor inoperative.

1 Flash           Firmware EPROM check sum failed.

RAM download failed.

2 Flashes         RAM test failed.

3 Flashes         AIC-6250 SCSI protocol chip verification failed

4 Flashes         FIFO read data path test failed.

5 Flashes         FIFO write data path test failed.

6 Flashes         Channel 2 test failed.

If any  of these  failure  conditions is  identified,  the host

computer should be powered down, the host adapter removed and the

host adapter  inspected for  physical  damage. Such  damage can

include EPROMs that are  not correctly installed  or not firmly

seated,  broken  wires,  missing   or  damaged  components,  or

conductive debris on the  board. If no  such physical damage is

found, the AHA-1640 should be returned for repair.

8-1

Section Eight                                           Problem Determination

------------------------------------------------------------------------------

All boards are fully  tested, burned in,  cleaned and inspected

before they are shipped. Care should be taken to keep the board

in its protective  conductive wrapping  until it  is installed.

With these simple precautions, mechanical damage can normally be

avoided.

8.2 INDICATORS

The red  LED  on the  AHA-1640  is  used to  provide  the fault

isolation information  described  above. In  addition,  the LED

indicates when the host adapter is performing activities on the

host interface and the SCSI interface. The LED is such a useful

activity indicator that a connector is made available on the host

adapter to allow an externally visible LED to be attached.

As an activity indicator, the LED is turned on from the time that

an IOCP command is transmitted to the host adapter until the HACC

interrupt is  generated to  indicate that  the IOCP  command is

complete. In addition, the LED will  be turned on whenever SCSI

bus activity is occurring.  This is roughly  the same period of

time that the SCSI BSY signal is present on the SCSI bus. If the

light stays on when no activity is expected to be present on the

bus, it  is  possible that  the  bus is  hung,  that unexpected

activity is occurring or  that the SCSI  cables are incorrectly

installed.

8.3 PROBLEM CORRECTION

The AHA-1640, other Micro Channel compatible components, and the

attached peripheral devices are very reliable devices because of

the high level of circuit integration in their design. Failures

are so rare that most such devices use symptoms to identify the

failing unit and use replacement of the failing unit to correct

the failure. The  AHA-1640 has only  one replaceable component.

If no  installation error  or mechanical  damage is  present, a

failing AHA-1640  must be  replaced. The  AHA-1640 can  only be

repaired at a qualified repair facility. This section presents a

list of possible error symptoms together  with a list of likely

corrective actions.

8.3.1 Common Problems during Installation

The description of installation problems and probably corrections

is given in Section 3.2.4.

8.3.2 IOCP Command Errors and Corrective Actions

Operation of the I/O Port interface is controlled and monitored

by host system software. Two bits in the Status Port (Base + 0),

described in Section  4.4.1, are  provided to  indicate unusual

conditions in the host adapter.

Internal Diagnostic Failure (Status Port, bit 6):

This bit indicates that the self testing process after either a

hard reset or a power on operation was terminated by an error

condition. The error indicates that a critical failure was found

in the AHA-1640 control circuitry or data paths.   The error

should be presented to the host video display to indicate that

diagnostic actions are required. The diagnostic action uses the

host adapter diagnostic flash codes to determine if the AHA-1640

is failing. The host system should be powered off, then opened

so that the LED can be observed. All SCSI cables, both external

and internal, should be removed from the host adapter. The

system should then be powered on. If the LED flashes once and

turns off, the host adapter has passed its diagnostics and should

be accessible again. If the LED flashes one of the error flash

codes, the AHA-1640 should be replaced. If the LED remains on

8-2

Section Eight                                           Problem Determination

------------------------------------------------------------------------------

and does not turn off, the host adapter is not able to begin

operation at all. The host system may be holding the AHA-1640 in

the reset state or the host adapter may have failed. To

distinguish these two cases, the AHA-1640 should be replaced and

the new host adapter operation should be verified. Be sure to

power down the host system when removing or replacing the host

adapter. If the diagnostics indicate that the AHA-1640 must be

replaced, it should be sent to an authorized service facility for

repair.

Invalid Host Adapter Command (Status Port, bit 0):  This bit is

set to indicate that a command  or parameter transmitted to the

host adapter was invalid.  This is an  indication that the host

software, usually  an  operating  system,  special  application

program, or device driver, has generated an invalid request. The

responsible programming organization should be notified so that

the invalid sequence or parameter can be corrected.

8.3.3 Host  Adapter  Status  Error  Indications  and Corrective

Actions

The Host Adapter Status indications,  in addition to indicating

several normal completion states, indicate three general groups

of errors. One  set describes  software errors  in the requests

made to the host adapter. Even  though most of these errors are

caused by software design problems, the AHA-1640 may need to be

replaced as an isolation step, since there exists a small chance

that hardware failures in the AHA-1640 cause the appearance of a

software error.

A second  set describes  errors  detected by  the  host adapter

firmware in the host adapter  hardware. The most likely failing

component in this case is the AHA-1640, although certain system

or cable failures may generate the appearance of a host adapter

hardware error.

A third set describes the appearance of unexpected or incorrect

sequences executed by the attached  SCSI devices. In this case,

the cables are the most likely failure point, with the peripheral

device second and the  AHA-1640 third. The  host adapter status

code should be returned to the host video display to indicate to

the operator what errors have occurred and under what conditions

the errors occurred. The following section describes those Host

Adapter Status indications that indicate an error.

NUMBER  TYPE   ERROR    DESCRIPTION AND CORRECTIVE ACTION

11      SCSI    NO    Selection Time Out:

The SCSI attempted to select a

device that was not installed.

Verify correct address values set

to SCSI devices. Verify that SCSI

cable routing includes the required

devices. Verify SCSI cable

integrity by replacement of cables.

Verify that SCSI Selection Time Out

value has been correctly

established by IOCP command.

12      HOST    NO    Data Over Run/Under Run:   Data

length or direction specified by

CCB did not agree with the data

length actually provided by the

attached peripheral device. Often

a normal error or accompanied by a

check condition indicating transfer

truncation. Verify program

requested correct length or

direction. Verify peripheral

provided expected data length,

number of blocks, or block length.

13      SCSI    YES   Unexpected Bus Free:

The target dropped BSY without

executing the proper message first.

8-3

Section Eight                                           Problem Determination

------------------------------------------------------------------------------

This normally indicates

that the BSY and/or other portions

of the SCSI bus failed or that the

target encountered such an invalid

sequence that no recovery was

possible. Some targets may have

sense information available to

qualify the error condition.

Verify SCSI cable integrity.

Verify SCSI cables are all properly

connected. Replace SCSI cables.

Replace peripheral. Replace AHA-1640.

NUMBER  TYPE  ERROR   DESCRIPTION AND CORRECTIVE ACTION

14      SCSI    YES   Target bus phase sequence failure:

The initiator detected an invalid

phase or an invalid phase sequence.

If this occurs, it is likely that

the host adapter forces a SCSI

reset on the bus as the first phase

of the recovery process. Verify

SCSI cable integrity. Verify SCSI

cables are all properly connected.

Replace SCSI cables. Replace

peripheral. Replace AHA-1640

15      HOST    YES   MBO Command Byte invalid:

This indicates a software failure

or bug in the host's development of

the MBO entry. Generally, this

indication will only occur during

initial debug processes of new

operating systems. Replace the

AHA-1640. Replace the host

computer system. Contact the

software developer for design

support.

16      HOST    YES   Invalid CCB Operation Code:

This indicates a software failure

or bug in the host's development of

the CCB. Generally, this

indication will only occur during

initial debug processes of new

operating systems. Replace the

AHA-1640. Replace the host

computer system. Contact the

software developer for design

support.

17      HOST    YES   Linked CCB does not have same LUN:

This indicates that the host

software generated an invalid

combination of link commands.

Generally, this indication will

only occur during initial debug

processes of new operating systems.

Replace the AHA-1640. Replace the

host computer system. Contact the

software developer for design support.

18      HOST    YES   Invalid CCB Parameter Received from

Host: This indicates that the host

software generated an invalid CCB.

Generally, this indication will

only occur during initial debug

processes of new operating systems.

Replace the AHA-1640. Replace the

host computer system. Contact the

software developer for design support.

8-4

Section Eight                                           Problem Determination

------------------------------------------------------------------------------

19      HOST    YES   Duplicate CCB Received in Target

Mode: This indicates that the host

system was not keeping correct

management information for target

mode operation and incorrectly

generated a second CCB identical to

one already active.    Generally,

this indication will only occur

during initial debug processes of

new operating systems. Replace the

AHA-1640. Replace the host

computer system. Contact the

software developer for design support.

NUMBER  TYPE    ERROR DESCRIPTION AND CORRECTIVE ACTION

1A      HOST    YES   Invalid Segment List:

A segment list was presented to the

host adapter with a zero length

segment or invalid segment

boundaries. Generally, this

indication will only occur during

initial debug processes of new

operating systems. Replace the

AHA-1640. Replace the host

computer system. Contact the

software developer for design support.

1B      SCSI    YES   Request Sense Command Failed:

A REQUEST SENSE command generated

automatically by the host adapter

did not complete correctly.

Non-zero status was returned.

Verify SCSI cable integrity.

Verify SCSI cables are all properly

connected. Replace SCSI cables.

Replace peripheral. Replace AHA-1640.

8.3.4 SCSI Error Indications and Corrective Actions

Error conditions detected  by SCSI peripherals  usually cause a

CHECK CONDITION  Status  byte  to be  presented.  When  this is

presented, the host  adapter automatically  retrieves the sense

information from the SCSI peripheral by executing a REQUEST SENSE

command according to the SCSI standard. The information returned

is mapped for single-threaded BIOS operation into the categories

of error conditions described in Section 5.2.2. For multitasking

(mailbox) operation, the information returned is made available

in the  area  allocated  for Sense  Data.  In  either  case, an

intelligent attempt is made by  most operating systems to retry

the operation  at least  one  time. Such  retry  operations may

require the management  of queued operations  that have already

started execution. This attempt to retry the operation is rarely

successful, since SCSI  peripheral devices  have very extensive

automatic retry and correction mechanisms designed to be executed

before the error information is returned in the first place.

Since it is rare that such an operation is successful, the host

software and operating systems should make every attempt to make

available key information about the error which can be analyzed

by the system operator or  customer engineer. In small systems,

it may be sufficient to present the important Sense Data together

with the command that  created the error.  Some simpler systems

may perform a preliminary analysis of the error information and

generate a code or descriptive text that describes the error and

indicates the  corrective action.  In  very complex  systems, a

logging process may take place, allowing a customer engineer to

analyze the  data at  some future  time.  In all  systems, such

information is very important for host program verification and

for system integrity verification, even if the designers choose

not to make the information available to any system users other

than the design and maintenance engineers.

8-5

Section Eight                                           Problem Determination

------------------------------------------------------------------------------

The SCSI Sense  Data is generally  self-descriptive. The errors

can be mapped in to four major categories, each category with its

own diagnostic procedure.

The first category  includes those  errors caused  by incorrect

command or parameter bytes or by incorrect sequences of commands.

These errors  typically  are presented  only  during  the early

development stages of an operating system or device driver, since

a properly operating program will not generate invalid commands.

The second category includes those  sense codes associated with

peripheral status presentation. Such sense  codes are not truly

errors, in that they inform the  host program of an unusual but

not unexpected condition. Such  sense codes include indications

that a device  is not  ready, that  the device has  just become

ready, that a device has  reached the end of  its media (End of

Tape or  Blank Check),  or that  the  expected data  length and

actual data length differed. These sense codes are normally used

by the device driver to execute the correct operation in response

to the condition. Such sense codes are not normally presented to

the system user except as text that requests some action. As one

example, an End Of Tape condition may require the system user to

replace the tape cartridge.

The third category  of errors  points to  a particular hardware

failure in the peripheral device or its supporting electronics.

Such error conditions usually require the adjustment, repair, or

replacement of the peripheral device or some of its components.

In some cases, the  error condition may  also indicate possible

cable or host adapter failures. These errors must be exposed to

the system user so that the proper actions can be taken.

The fourth category of errors points  to a media failure in the

peripheral device.  Such  errors include  bits  that  cannot be

recovered from magnetic media and imperfections in the surface of

optical  media.  Most  such  errors  are  recovered  using  the

extensive retry and  correction algorithms  programmed into the

peripheral device.  In  some technologies,  the  errors  may be

caused by noise conditions or by marginal electronic failures in

the read  or  write  path. Those  rare  errors  that  cannot be

recovered usually indicate that some information important to the

operating system or application has  been lost. The system must

make this information available to the  system user so that the

system user can replace the media or recover the data from a back

up copy as required.  In addition, logging  of errors that were

successfully recovered by the peripheral device is often useful

as an indicator of the overall  reliability of the device or of

the requirement for periodic maintenance.

8-6

Section Nine                                                         Glossary

------------------------------------------------------------------------------

AEN

See Asynchronous Event Notification

Asynchronous Event Notification

A process by which a target can send unsolicited sense information to

an initiator using the  SEND command in order  to inform the initiator

about the occurrence of an important unusual occurrence.

Asynchronous Data Transfer

Data  transfer  performed  by   the  SCSI  interface  involving  the

interlocking of a signal  to the initiator  (REQ) and a  signal to the

target (ACK) such  that each step  of the data  transfer protocol must

occur before the next step can begin. Characterized by a low data rate

and independence of external timing constraints, including cable length

and circuit response times.

Bus Device Reset

A SCSI message that clears all activity  in the target to which it is

addressed.

Byte

An 8  bit unit  of data. An  octet. A  byte is normally  the smallest

addressable unit of a memory and the unit of transfer on the SCSI.

CCB

See Command Control Block

CCS

See Common Command Set

CDB

See Command Descriptor Block

Command Control Block

A software object prepared by the host microcomputer software for the

host adapter to  provide it  all the  control information  it needs to

execute a SCSI command. Abbreviated: CCB.

9-1

Section Nine                                                         Glossary

------------------------------------------------------------------------------

Command Descriptor Block

A  block of  information  passed across  the  SCSI bus  to provide  the

command, parameter, and address information necessary for the target to

execute the desired functions. Prepared by the host software and placed in

the CCB to be passed to the target by the host adapter. Abbreviated: CDB.

Common Command Set

A defacto standard SCSI command set  for communication with hard disk

drives. The  Common Command  Set  (CCS) is  the  basis for  the SCSI-2

command set for all types of peripheral devices.

Control Microprocessor

An  integrated circuit  computer used  to  execute the  software that

controls the host adapter's operation.

Device Driver

A program that  is linked with  or attached to  an operating system  to

map the software interface of the operating system to the requirements

of attached peripheral devices and host adapters.

DMA

See Direct Memory Access

Direct Memory Access

A mechanism  that allows hardware  control of  the transfer of  streams

of data to or from the main memory of a computing system. The mechanism

may require  set  up  the  host  software.  After  initialization,  it

automatically sequences the  required data  transfer and  provides the

necessary address information.

EPROM

Erasable Programmable  Read Only  Memory. An  integrated circuit used

to store the host adapter firmware and the host adapter BIOS.

9-2

Section Nine                                                         Glossary

------------------------------------------------------------------------------

FIFO

First In / First Out.

A queueing order in which items are removed from the queue for execution

in the same order in which they are placed in the queue.

An integrated circuit that buffers data in such a manner that the each

byte placed in the buffer is removed from the buffer in the same order.

Firmware

The  software  that controls  and  manages  the host  adapter.  It is

'firm' as opposed  to 'soft'  because it  is designed  in to  the host

adapter and cannot be modified by the user.

I/O Command Port Command

A command transmitted to the host  adapter using the Command/Data Out

Port and the Data In Port. The commands are sequenced using the Control

Port, the Status Port, and the Interrupt Flag Port. Abbreviated as IOCP

command.

Initiator

A SCSI device that  requests an operation to  be performed by another

SCSI device  (the  target).  The initiator  provides  all  the command

information and parameters required to  perform the operation, but the

details of the operation are actually sequenced by the target.

IOCP Command

See I/O Command Port Command

Host

A microcomputer  in which a  host adapter is  installed. The host  uses

software to request the  services of the  host adapter in transferring

information to and  from peripheral devices  attached to  the SCSI bus

connector of the host adapter.

Host adapter

A  hardware  printed  circuit  board  that  installs  in  a  standard

microcomputer backplane and provides a SCSI bus connection so that SCSI

devices can  be  connected to  the  microcomputer. A  host  adapter is

"intelligent" if it has a simple  high-level software interface to the

microcomputer. A  host adapter  is  "dumb" if  the  microcomputer must

directly manage the SCSI protocol using the microcomputer processor.

9-3

Section Nine                                                         Glossary

------------------------------------------------------------------------------

Logical Unit

A physical or virtual device addressed through a target.

Logical Unit Number

An encoded three-bit identifier for a logical unit.

LU

See Logical Unit

LUN

See Logical Unit Number

Mailbox In

An area  in main memory  assigned by the  host microcomputer software

for communication with the host adapter. The host adapter places status

and pointer information in entries in the Mailbox In (MBI) to indicate

to the host microcomputer what operations  have been completed or what

information must be obtained from the host microcomputer.

Mailbox Out

An area  in main memory  assigned by the  host microcomputer software

for communication with  the host  adapter. The  microcomputer software

places commands and pointer information in  entries in the Mailbox Out

(MBO) to indicate what operations should be started by the host adapter.

MBI

See Mailbox In.

MBO

See Mailbox Out.

Micro Channel

A  microcomputer backplane  created  by IBM  for  the high  performance

PS/2_ series of small computers. The name is trademarked by IBM.

9-4

Section Nine                                                         Glossary

------------------------------------------------------------------------------

Multi-tasking Operation

The execution of commands in such a way that more than one command is

in progress at the same time, allowing the system to take advantage of

overlapping activities  by using  resources  that are  temporarily not

required for other operations. More than one  program or more than one

portion of a program may be operating in parallel.

POS

See Programmable Option Select

Programmable Option Select

A feature  of the  Micro Channel  that allows  the setting of  hardware

related switches  through  the  microcomputer  software.  Programmable

Option Select (POS)  allows the  use of  geographic addressing  of the

backplane card slots  and pre-established  ID values  assigned to each

adapter card to properly verify and configure the microcomputer.

PS/2

A  family of  small computers  sold by  IBM, also  called the  Personal

System / 2 family of computers. The name is trademarked by IBM.

RAM

Random  Access  Memory. Memory  of  which  any byte  can  be accessed

directly in a  single memory cycle.  Information can be  read from and

written to the memory.

SCSI

The Small Computer System Interface.

SCSI ASC

SCSI  Additional  Sense   Code.  Byte  12   of  the  extended  sense

information. Provides  a  standardized  description  of  the condition

described by the sense information.

SCSI Device

A  device attached  to a  Small  Computer System  Interface bus  cable.

The device may be an initiator, a  target, or capable of both types of

operation. The device may be a peripheral  device, a host device, or a

device mixing both roles.

9-5

Section Nine                                                         Glossary

------------------------------------------------------------------------------

Single Threaded Operation

Operation of the computing  system such that only  one program can be

operating or active at a time. The computing system must wait until all

resources are available before starting  an operation and cannot start

another operation until the first one  is completed. No overlapping of

latencies or program operation occurs.

Synchronous Data Transfer

A method of data transfer on  the SCSI bus involving clocking data on

to the  bus with  a  fixed-length fixed-frequency  strobe  pulses. The

acknowledgements may be  delayed several  clock periods  from the data

requests.  Synchronous  Data  Transfer  can  be  used  only  for  data

transmission on the SCSI  bus. It is  prohibited for command, message,

and status transmission.

Synchronous Data Transfer Negotiation

The  message  exchange  between the  initiator  and  the  target that

allows the negotiation of the data transfer frequency and delay between

requests and acknowledgements required  for synchronous data transfer.

Once negotiated, synchronous data transfer parameters remain unchanged

until certain reinitialization activities occur.

Target

A SCSI  device that performs  an operation  requested by an  initiator.

The target  may be  a peripheral  device performing  a service  for an

initiator. The target may also be a host adapter performing a Processor

type device service for an initiator.

Word

A two byte (16 bit) unit of data.

Writable Control Store

RAM  containing  the  host adapter's  firmware  while  the firmware  is

being executed. The RAM is loaded from EPROM.

9-6

8/25/89

********************************************************************

ADAPTEC AHA-1640 PS/2 TO SCSI HOST ADAPTER EXTERNAL CONNECTOR

********************************************************************

NOTICE:  THIS IS PRELIMINARY AND ADAPTEC IS NOT RESPONSIBLE FOR

ANY CHANGES, ERRORS, OR OMISSIONS TO THIS INFORMATION.

THIS PUBLIC INFORMATION SHOULD NOT BE CONSTRUED AS A

RECOMMENDATION BY ADAPTEC FOR ANY MANUFACTURER OR PART.

********************************************************************

********************************************************************

I)  LIST OF MANUFACTURERS AND PART NUMBERS OF CABLE CONNECTORS

THAT MATE WITH THE EXTERNAL SCSI CONNECTOR

ON THE ADAPTEC AHA-1640 PS/2 TO SCSI HOST ADAPTER

********************************************************************

AMP           Connector Part No:      749111-4

Back Shell Part No:     749193-1

FUJITSU       Connector Part No:      FCN-237R050-G/F

Back Shell Part No:     FCN-230C050-D/E (or -C/E)

HONDA         Connector Part No:      PCS-XE50MA

Back Shell Part No:     PCS- E50LA

********************************************************************

II)  INFORMATION ON THE RECOMMENDED EXTERNAL SCSI CABLE MATERIAL

********************************************************************

Partial description of cable:  100% shielded

25 twisted pairs

90 - 135 ohm -> intrinsic impedence

C&T           Cable Part No:          16035

Madison       Cable Part No:          4099

(Both these cable materials are IPI type cables.  Approximately 120

ohms measured differential TDR.)

********************************************************************

III)  INFORMATION ON COMPANIES THAT MAKE COMPLETE CABLE ASSEMBLIES

********************************************************************

Some companies that manufacture complete cable assemblies are:

Amphenol Interconnect Products, Endicott, New York

contact Bill Sopchak at (607) 786-4370  or (607) 754-4444

Quintec Interconnect Systems, San Jose, California

contact Lon Hudson    at (408) 272-8000