Preface

This release of the Technical Reference Manual for the AHA-1740/1744

EISA-to-Fast SCSI Host Adapter is made available to customer technical

personnel needing detailed information on the functioning of the board

at a register and command protocol level. Documentation of the board

schematics, integrated circuits, microcode and BIOS routines is not

provided.

Programmers writing drivers for specific peripherals are strongly

advised to use the Advanced SCSI Programming Interface (ASPI)

specification appropriate to the operating system chosen. This will

allow flexibility across all boards complying with ASPI through their

ASPI Manager modules and prevent obsolescence. Please contact Adaptec

Corporate Communications for copies of ASPI specifications. Software

managers are documented and sold separately.

Every effort has been made to ensure the technical accuracy of the

contents of this reference manual. Should any reader believe that

inaccuracies exist, they should be communicated in writing to your

local field applications support or with the product manager directly.

Revision 1.0,

March, 1991

COPYRIGHT

_ Copyright 1991 Adaptec, Inc. All rights reserved.

No part of this publication may be reproduced, stored in a retrieval

system, or transmitted, in any form or by any means, electronic,

mechanical, photocopying, recording, or otherwise, without the prior

written consent of Adaptec, Inc. 691 South Milpitas Blvd., Milpitas,

CA 95035.

TRADEMARKS

IBM is a registered trademark of International Business Machines

Corporation. PS/2, PC AT, DOS, MS-DOS, OS/2 and Micro Channel are

trademarks of International Business Machines Corporation. SCO and SCO

XENIX are trademarks of The Santa Cruz Operation. Unix is a registered

trademark of AT & T Bell Laboratories. EISA is a registered trademark

of BCPR Services Inc. NetWare is a registered trademark of Novell.

CHANGES

While reasonable efforts have been made in the preparation of this

manual to assure its accuracy, Adaptec, Inc. assumes no liability

resulting from any errors or omissions in this manual, or from the use

of the information contained herein.

Adaptec reserves the right to make changes in the product design

without reservation and without notification to its users.

Additional information may be obtained from:

adaptec

Literature Department

691 South Milpitas Blvd.

Milpitas, CA 95035

TABLE OF CONTENTS

Section 1 Introduction                                                  1-1

1.1      DOCUMENT SCOPE                                               1-1

1.2      PURPOSE                                                      1-1

1.3      AHA-1740/1744 PRODUCT FEATURES                               1-1

1.4      PRODUCT SPECIFICATIONS                                       1-3

1.5      REFERENCE DOCUMENTS                                          1-6

Section 2 Architecture                                                  2-1

2.1      HARDWARE                                                     2-1

2.1.1   Bus Master DMA                                            2-1

2.1.2   SCSI Interface and Protocol Chip (AIC-6251)               2-2

2.1.3   16- and 8-Bit Memory                                      2-2

2.1.4   Bus Auxiliary Interface Chip (AIC-565)                    2-3

2.1.5   EISA Configuration Chip (AIC-575)                         2-3

2.1.6   BMIC Bus Interface Chip (82355)                           2-3

2.1.7   Hardware Block Diagram                                    2-4

2.2      COMPATIBLE MODE FIRMWARE                                     2-4

2.2.1       Mailboxes                                             2-5

2.2.2       Command Control Block                                 2-5

2.2.3       Command Descriptor Block                              2-6

2.2.4       Principles of Operation                               2-7

2.2.5       Task Queuing                                          2-7

2.3     ENHANCED MODE FIRMWARE                                        2-8

2.4     ONBOARD BIOS OPERATION                                        2-9

2.5     POWER-UP DIAGNOSTICS                                          2-9

2.6     ASW-C174 CONFIGURATION UTILITY                                2-10

Section 3 Installation                                                  3-1

3.1     UNPACKING AND INSPECTION                                      3-1

3.2     INSTALLATION                                                  3-1

3.2.1       System Configuration Background for Compatible Mode   3-2

3.2.2       Preparation                                           3-2

3.2.3       Termination                                           3-2

3.2.4       Addressing                                            3-3

3.2.5       Installation Procedure                                3-3

3.2.7       Terminators                                           3-5

3.3 SYSTEM CONFIGURATION                                              3-6

3.3.1       MCS Configuration                                     3-6

3.3.2       Standard Mode SCSI Configuration                      3-8

3.3.3       Enhanced Mode SCSI Configuration                      3-9

3.3.4       Phoenix Configuration                                 3-11

3.4     FIRMWARE DOWNLOAD                                             3-12

3.5     CONFIGURATION OPERATION                                       3-13

3.5.1       List of Adapters                                      3-13

3.5.2       Main Menu                                             3-13

3.5.3       Download Firmware                                     3-13

3.5.4       Firmware Information                                  3-14

3.6     LOW-LEVEL FORMAT UTILITY                                      3-14

i

TABLE OF CONTENTS (Continued)

Section 4  Hardware Functional Description                              4-1

4.1     HARDWARE OVERVIEW                                             4-1

4.2     COMPATIBLE MODE I/O PORT INTERFACE                            4-1

4.2.1       Control and Status Port                               4-2

4.2.2       Command/Data Out and Data In Port                     4-5

4.2.3       Interrupt Flag Port                                   4-6

4.3     RESET FUNCTIONS                                               4-8

4.3.1       Hard Reset Operations                                 4-8

4.3.2       SCSI Reset Operations                                 4-8

4.3.3       SCSI "Soft" Reset Option                              4-9

4.3.4       SCSI "Hard" Reset Option                              4-9

4.4     EISA EXPANSION IDENTIFIERS                                    4-10

4.4.1       Expansion Board ID Registers                          4-11

4.4.1.1 Host ID 0 (HID0, zC80, R)                            4-11

4.4.1.2 Host ID 1 (HID1, zC81, R)                            4-11

4.4.1.3 Host ID 2 (HID2, zC82, R)                            4-12

4.4.1.4 Host ID 3 (HID3, zC83, R)                            4-12

4.4.2     Board Configuration Registers                           4-12

4.4.2.1 Exp. Board Control Register (EBCTRL, zC84, W/R)      4-12

4.4.2.2 I/O Port Address (PORTADDR, zCC0, W/R)               4-12

4.4.2.3 BIOS Control                                         4-13

4.4.2.4 Interrupt Definition (INTDEF, zCC2, W/R)             4-15

4.4.2.5 SCSI Definition (SCSIDEF, zCC3, W/R)                 4-16

4.4.2.6 Bus Definition (BUSDEF, zCC4, W/R)                   4-16

4.4.2.7 Reserved Registers                                   4-17

4.4.3     I/O Port Register Standard Mode (Group 1)               4-17

4.4.3.1 Control Port (CNTRL, Base address + 0, W)            4-18

4.4.3.2 Status Port (STATUS, Base address + 0, R)            4-18

4.4.3.3 Command/Data (CMD/DAT, Base address + 1, W/R)        4-19

4.4.3.4 System Interrupt (INTRPT, Base + 2, R)               4-19

4.4.4     I/O Port Register Enhanced Mode (Group 2)               4-20

4.4.4.1 Mailbox Registers                                    4-21

4.4.4.2 Attention (ATTN, zCD4, W/R)                          4-22

4.4.4.3 Enhanced Mode (Group 2) Control (G2CNTRL, zCD5, R)   4-23

4.4.4.4 Enhanced Mode(Group 2)Interrupt Status(G2INTST, zCD6,R)4-23

4.4.4.5 Enhanced Mode (Group 2) Status (G2STAT, zCD7, R)     4-23

4.4.4.6 Enhanced Mode (Group 2) Status 2 (G2STAT2, zCDC, R)  4-24

4.4.5     82355 BMIC Registers (zC88 - z(9F)                      4-24

Section 5 Compatible Mode Command Set                                   5-1

5.1     HOST ADAPTER COMMAND OVERVIEW                                 5-1

5.1.1  No Operation (Operation Code 00)                           5-2

5.1.2  Mailbox Initialization (Operation Code 01)                 5-2

5.1.3  Start SCSI Command (Operation Code 02)                     5-3

5.1.4  Start PC AT BIOS Command (Operation Code 03)               5-3

5.1.5  Adapter Inquiry (Operation Code 04)                        5-3

5.1.6  Enable Mailbox-Out Available Interrupt (Operation Code 05) 5-4

5.1.7       Set Selection Time-out (Operation Code 06)            5-5

5.1.8       Set Bus-On Time (Operation Code 07)                   5-5

5.1.9       Set Bus-Off Time (Operation Code 08)                  5-6

5.1.10      Set Transfer Speed (Operation Code 09)                5-6

5.1.11      Return Installed Devices (Operation Code 0A)          5-7

5.1.12      Return Configuration Data (Operation Code 0B)         5-8

ii

TABLE OF CONTENTS (Continued)

5.1.13      Enable Target Mode Command (Operation Code 0C)        5-9

5.1.14      Return Setup Data (Operation Code 0D)                 5-9

5.1.15      Initialize SCSI Subsystem (Operation Code 10)         5-12

5.1.16      Return Firmware Checksum (Operation Code 11)          5-12

5.1.17      Write Adapter Channel 2 Buffer (Operation Code 1A)    5-12

5.1.18      Read Adapter Channel 2 Buffer (Operation Code 1B)     5-13

5.1.19      Write Adapter FIFO Buffer (Operation Code 1C)         5-13

5.1.20      Read Adapter FIFO Buffer (Operation Code 1D)          5-13

5.1.21      ECHO Command Data (Operation Code 1F)                 5-13

5.1.22      Adapter Diagnostic (Operation Code 20)                5-14

5.1.23      Set Host Adapter Options (Operation Code 21)          5-14

5.2      MAILBOX OVERVIEW                                             5-14

5.2.1       Mailbox Out Definition                                5-15

5.2.2       Mailbox In Definition                                 5-16

5.3      COMMAND BLOCK DEFINITION                                     5-19

5.4      SCATTER/GATHER LIST DEFINITION                               5-26

5.5      DESCRIPTION OF OPERATION                                     5-27

5.5.1  Execution of Initiator Mode Operations                     5-27

5.5.2  Execution of Target Mode Operations With a Prepared CCB    5-28

5.5.3  Execution of Target Mode Operations Without a Prepared CCB 5-29

5.6     DMA CHANNEL INITIALIZATION (with HA BIOS Disabled)            5-29

5.7     INTERRUPT INITIALIZATION                                      5-30

Section 6 Enhanced Mode Interface                                       6-1

6.1     CONTROL BLOCK                                                 6-2

6.1.1       Command Word                                          6-3

6.1.2       Flag Word 1                                           6-3

6.1.3       Flag Word 2                                           6-5

6.1.4       Data or Scatter/Gather List Pointer                   6-9

6.1.5       Data or Scatter/Gather List Length                    6-10

6.1.6       Status Block Pointer                                  6-10

6.1.7       Chain Address                                         6-10

6.1.8       Sense Information Pointer                             6-10

6.1.9       Sense Length                                          6-11

6.1.10      CDB Length                                            6-11

6.1.11      Checksum of Data                                      6-11

6.1.12      SCSI Command Descriptor Block                         6-11

6.2      STATUS BLOCK                                                 6-11

6.2.1       Status Word                                           6-12

6.2.2       Host Adapter Status                                   6-15

6.2.3       Target Status                                         6-16

6.2.4       Residual Byte Count                                   6-17

6.2.5       Residual Buffer Address                               6-17

6.2.6       Additional Status Length                              6-17

6.2.7       Sense Length                                          6-18

6.2.8       Target Mode Status Word and CDB                       6-18

6.3      CONTROL BLOCK COMMANDS                                       6-18

6.3.1       Download Firmware                                     6-19

6.3.2       Initialize SCSI Subsystem                             6-20

6.3.3       Initiator SCSI Command                                6-22

6.3.4       INo Operation                                         6-23

6.3.5 Read Host Adapter Inquiry Data                              6-24

6.3.6       Read Sense Information                                6-27

6.3.7       Run Diagnostic Test                                   6-28

6.3.8       Target SCSI Command                                   6-29

iii

TABLE OF CONTENTS (Continued)

6.4      ABORTING CONTROL BLOCK COMMANDS                              6-30

6.5      ASYNCHRONOUS EVENT NOTIFICATION                              6-32

6.6      IMMEDIATE COMMANDS                                           6-34

6.6.1 Reset                                                       6-34

6.6.2 Reset Device                                                6-34

6.6.3 Reset Host Adapter                                          6-36

6.6.4 Resume                                                      6-37

6.7 SCSI BUS RESET HANDLING                                           6-37

6.7.1       Reset Initiated by the Host                           6-37

6.7.2       Reset Initiated by the Host Adapter                   6-38

6.7.3     Reset Initiated by Another SCSI Device                   6-38

Section 7 Onboard BIOS Interface                                        7-1

7.1     OPERATION WITH THE STANDARD INTERFACE                         7-1

7.2     OPERATION WITH THE ENHANCED INTERFACE                         7-2

7.2.1       Hardware                                              7-2

7.2.2       Initialization                                        7-2

7.2.3       Boot Issues                                           7-4

7.3     INTERRUPT 13H INTERFACE FUNCTIONALITY                         7-4

7.3.1       Physical to Logical Block Address Translation         7-5

7.3.2       Virtual to Physical Buffer Address Translation        7-5

7.3.3       BIOS Command Return codes                             7-5

7.3.4       Hardware BIOS Commands                                7-6

7.4     INT 15H FUNCTIONALITY                                         7-9

7.5     DIFFERENCES BETWEEN OPERATING MODES                           7-9

7.6     MULTIPLE ADAPTER SUPPORT                                      7-10

Section 8 Software Managers                                             8-1

8.1     DOS OPERATION WITHOUT DRIVERS                                 8-1

8.2     SYSTEM CONFIGURATION                                          8-1

8.3     LOW-LEVEL FORMAT                                              8-2

8.4     INSTALLATION AND INITIALIZATION UNDER DOS                     8-2

8.5     MANAGERS                                                      8-2

8.6     DOS MANAGER                                                   8-3

8.7     WINDOWS 386                                                   8-4

8.8     OS/2 MANAGER                                                  8-4

8.9     NOVELL NETWARE MANAGER                                        8-4

8.10    UNIX/XENIX                                                    8-5

Section 9 SCSI Features                                                 9-1

9.1     SCSI DESCRIPTION, INITIATOR MODE                              9-2

9.1.1       Linked SCSI Commands                                  9-2

9.1.2       Zero Latency Read Operation                           9-2

9.1.3       SCSI Messages                                         9-3

9.2     SCSI DESCRIPTION, TARGET MODE                                 9-3

9.2.1       Initiator Conformance Level Requirements              9-3

9.2.2       Support of Synchronous Transfer                       9-4

9.2.3       SCSI Target Operation in Processor Target Mode        9-4

9.2.3.1  Test Unit Ready                                     9-5

9.2.3.2  Request Sense                                       9-5

9.2.3.3  Inquiry                                             9-7

9.2.3.4  Send                                                9-8

9.2.3.5  Receive                                             9-8

iv

TABLE OF CONTENTS (Continued)

9.2.4       Incorrect Length Management for Target Mode Operation 9-9

9.2.5       Aborting Target Mode Commands                         9-10

Section 10 Problem Determination                                       10-1

10.1      SELF DIAGNOSTIC CAPABILITY                                 10-1

10.2      INDICATORS                                                 10-2

10.3 PROBLEMS DETECTED DURING OPERATION                              10-2

10.3.1  HA Status Error Indications and Corrective Actions      10-3

10.3.2  SCSI Error Indications and Corrective Actions           10-6

10.4      PROBLEMS DETECTED DURING INSTALLATION                        10-7

Section 11 Glossary                                                    11-1

Appendix A AHA-1740/1744 Memory Cycle Timing                            A-1

Appendix B Connector Pinout                                             B-1

Appendix C Register Reference                                           C-1

Appendix D EISA Free-Form Data                                          D-1

v

Section One                                                     Introduction

------------------------------------------------------------------------------

1.1 DOCUMENT SCOPE

This manual provides information on installation and defines the

program interfaces of the AHA-1740 and the AHA-1744 intelligent host

adapters in EISA bus-based systems. Programming peripheral drivers

directly to the hardware interface of the board is not recommended.

The ASPI specifications provide a simpler and more flexible interface

which is protected against changes, upgrades and obsolescence of the

boards.

1.2 PURPOSE

The Adaptec AHA-1740/1744 provides a powerful multitasking interface

between the Extended Industry Standard Architecture (EISA) bus and the

Small Computer System Interface (SCSI) bus. The AHA-1740/1744 is a

high-performance intelligent host adapter which supports a maximum

asynchronous SCSI rate of 2.0 MBytes/second and a synchronous transfer

rate of 10.0 MBytes/second. The AHA-1740/1744 supports multithreaded

I/O operations, allowing simultaneous operations on multiple

targets/LUNs. Disconnect/ Reconnect support maximizes bus utilization

for multiple target systems. Target mode operation allows the

AHA-1740/1744 to receive information from other host adapters.

Scatter/Gather allows high performance even in systems with fragmented

memory buffers.

The Adaptec AHA-1740/1744 provides a solution for system applications

requiring very high performance, configuration flexibility and

multithreaded I/O capability, and system redundancy. The Adaptec BIOS

also allows the AHA-1740/1744 to be used in place of a standard hard

disk controller. The AHA-1740 host adapter provides high-performance

host adapter circuitry connected to the more common single-ended SCSI

bus capable of running up to 18 feet at normal SCSI speeds of up to 5

MBytes/second. The AHA-1744 host adapter provides identical host

adapter circuitry, but connects to a differential SCSI bus capable of

running fast (10 MBytes/second) SCSI up to 18 feet.

1.3 AHA-1740/1744 PRODUCT FEATURES

EISA Features

The AHA-1740/1744 is a full-performance EISA board offering the

highest performance available across the bus. It takes advantage of

first-party DMA operations, also known as bus mastering, which allow

data transfers to proceed at high data rates without system processor

intervention. The main EISA functions which come standard on the board

include:

� Supports Auto Configuration

� High-performance Bus Master transfers with Burst cycles which

provide a data rate of 33 MBytes/second

� 32-bit data path

� 32-bit address space

1-1

Section One                                                     Introduction

------------------------------------------------------------------------------

� Even or odd starting address and byte count handling

� 1K FIFO for efficient data transfer

� Interrupts enabled/disabled through I/O port write

� Interrupts programmable as shared or independent, high or low active

� Dual I/O paths, AHA-1540 family-compatible or EISA-SCB compatible

SCSI Features

Adaptec has long been a supplier of leading edge SCSI products and the

AHA-1740/1744 is no exception. The board uses industry-standard protocol ICs

which fully comply with the 1990 ANSI SCSI specification, including:

�    Maximum synchronous SCSI transfer rate of 10.0 MBytes/second

�    Asynchronous and synchronous peripherals supported simultaneously

�    Disable synchronous negotiation independently on each target as a

configuration option

�    Disable disconnection independently on each target as a

configuration option

�    Supports Modify Data Pointer message (Zero Latency Reads)

�    High-density SCSI-2 connector

�    Non-destructive current limit on Terminator Power supply

�    Internal and external SCSI connectors

�    Initiator and Target modes of operation fully supported

�    Differential stuffing option

Board Features

In addition to extensive functions on the two main interfaces of the

board, the intelligence built into the board microcode and BIOS

software allows a number of additional functions to be offered,

including:

�    Scatter/Gather operation

�    Boot from any target, set as configuration option

�    Compatible with existing AHA-1540 family driver software

(ASW-1400 series managers)

�    True multithreaded operation supporting up to 255 tasks

simultaneously

�    Programmable mailbox architecture

�    Downloadable microcode for flexible operation and upgrades

�    Bootable BIOS for standard hard disk emulation

The microcode with which the AHA-1740/1744 is equipped is available in

two separate versions which coexist simultaneously in the onboard

EEPROM, but which must be configured into one of the two available modes:

_ Compatible Mode

_ Enhanced Mode

1-2

Section One                                                     Introduction

------------------------------------------------------------------------------

The modes cannot operate simultaneously _ the configuration utility

must be run to select between them. The mode dictates the software

manager revision which can support the board, but has no effect on the

SCSI or host bus hardware connections. The firmware download should be

used only to upgrade the board to a later revision of firmware for a

particular mode. It is not required to configure the board for one of

the two main modes.

The Compatible mode allows software drivers written for the AHA-1540

or AHA-1640 families to run unaltered on the AHA-1740 family. There is

no performance penalty for this _the AHA-1740/1744 performs 32-bit

transfers at speeds up to 33 MBytes/second and can operate 10

MByte/second Fast SCSI transfers.

The Enhanced mode allows the board to take advantage of a number of

features which were not available on earlier host adapters. These include:

� 32-bit addressing capability

� Ability to access all EISA board registers

� Single fast mailbox handling

� Contingent allegiance (SCSI)

� Tagged queuing (SCSI)

Configuration Utility (ASW-C174)

As with other EISA borads, the AHA-1740/1744 is configured by software

in an EISA system. This utility is available as a software package,

the ASW-C174. Along with EISA configuration files, this includes

additional files for configuring the SCSI bus and downloading

microcode to the board, plus a low-level format utility.

Microcode Utility (ASWM174)

Periodically, Adaptec may make available a utility which takes

advantage of the AHA-1740/1744's ability to reconfigure its microcode.

This ASW-M174 package provides in-circuit upgrade ability through a

microcode download utility and versions of the Standard and Enhanced

microcode.

1.4 PRODUCT SPECIFICATIONS

� Physical Dimensions:

Length : 13 3/8" Width : 5/8" Height : 5"

Standard EISA-compatible form factor

� Power Requirements:

+5.0 �0.25 Volts at 2.9 Amps maximum

�

Environmental Requirements:

Temperature 0-55� C (operating or storage)

1-3

Section One                                                     Introduction

------------------------------------------------------------------------------

� Reliability Information

Mean Time Between Failures: 100,000 hours

(calculated per Mil Handbook 217E, ground benign, 40 �C)

Mean Time Between Failures: 1740: 60,462 hours

(calculated)               1744: 54,856 hours

Mean Time to Repair: 30 minutes

� Extended Industry-Standard Architecture Interface

Standard EISA bus electronic and physical interface.

Driver output:

VOL: min: 0 volts     max. 0.4 volts   IOL:  24 mA

VOH: min: 2.4 volts   max. 5.25 volts  IOH:   8 mA

Receiver Input:

VIL: 0.8 volts

VIH: 2.0 volts

Connector configuration as specified by manual of Extended

Industry Standard Architecture host computer.

� SCSI Interface:

Electrical Interface:

As specified by ANSI X3.131-1986 for single-ended operation.

Minimum driver output:  48 milliamps, sinking @ 0.5 volts

Signal true:            min: 0 volts    max:    0.8 volts

Signal false:           min: 2.0 volts  max:    5.25 volts

Input hysteresis:       min: 0.2 volts

Terminator Power voltage:  min: 4.25 volts  max: 5.25 volts

Terminator Power current:  min: 800 mA      max: 1.5 amp (fused)

Internal connector:

Unshrouded 50-pin header, compatible with unshielded alternative 1

connector as specified in ANSI X3.131-1986 (Figure 4-1).

For connector pin out and drawing of the connector, see Appendix B.

1-4

Section One                                                     Introduction

------------------------------------------------------------------------------

Partial list of compatible connector plugs (for reference only):

Manufacturer    Model                Part Number

3-M             N.A.                 3425-6000

T&B Ansley      N.A.                 609-5000M

Molex           N.A.                 15-29-8505

Cable for internal SCSI connector should be good quality

50-conductor flat cable with 26 or 28 gauge conductors

and a characteristic impedance (Z0) of 100 �10�. Cable

shielding is necessary if extremely noisy circuitry or

extremely noise-sensitive circuitry is present inside the

host computer frame.

External connector:

Shielded 50-pin connector as specified in proposed ANSI

standard X3T9.2/86-109 Revision 10c, Section 4, Figure 4-5.

For connector pin out and drawing of the connector, see Appendix B.

Partial list of compatible connector plugs or cable assemblies

(for reference only):

Manufacturer    Model                Part Number

AMP             Connector            749111-4

Back Shell           749193-1

Fujitsu         Connector            FCN-237R050-G/F

Back Shell           FCN-230C050-D/E or

-C/E

Honda           Connector            PCS-XE50MA

Back Shell           PCS-E50LA

Cable for external SCSI connector should be good quality 100%

shielded round cable with 25 twisted pairs. Each pair should have a

characteristic impedance (Z0) between 90� and 135�. Wire gauge may

be 26 or 28 AWG. All pairs should have the same impedance and should

have the same delay per length of cable. Cables meeting these

requirements will normally operate correctly in any SCSI

configuration and should normally meet all FCC requirements.

1-5

Section One                                                     Introduction

------------------------------------------------------------------------------

Cable material which meets this specification is available from

a number of vendors, including:

Manufacturer    Part Number          Phone

C&T             16035

Madison         4099                 (508) 752-7320

Complete cable assemblies are available from a number of

manufacturers. Among them are:

Manufacturer                          Phone

Amphenol Interconnect Products        (607) 786-4370

Lynn Products Inc.                    (800) 634-5093

Quitec Interconnect Systems           (408) 272-8000

Icontec                               (408) 945-7766

Enhance Cable Technology              (408) 293-2425

1.5 REFERENCE DOCUMENTS

� EISA Specification, version 3.0 from BCPR Services

� AHA-1640 User's Manual

� AIC-6250 Data Sheet

� Intel 82355 (BMIC) Functional data sheet Rev 2.5 or later

� Small Computer System Interface, ANSI X3T9.2/86-109 Revision 10c,

American National Standards Institute

� Adaptec AHA-1740/AHA-1744 Host Adapter Installation Guide Using the ASW-C174

� Adaptec AHA-1740/AHA-1744 Host Adapter User's Manual

1-6

Section Two                                                     Architecture

------------------------------------------------------------------------------

2.1 HARDWARE

The hardware of the AHA-1740/1744 products is based on the latest VLSI

technology for maximum performance in a minimum of board space. Where

commercial products with sufficient performance and functionality were

not available, Adaptec has developed custom circuits using its long

experience in high-performance silicon design for peripheral control

applications. These parts are assembled on a multilayer printed

circuit board in Adaptec's volume manufacturing plant and subjected to

a number of functional and mechanical inspections and tests.

The AHA-1740/1744 uses the same PCB design and have over 85% of the

circuitry in common, including the processor, bus interface, memory

and SCSI protocol control. Only the AHA-1744 is equipped with

differential driver circuits for the SCSI bus.

The general architecture of the board is shown in Figure 2-1.

2.1.1 Bus Master DMA

The AHA-1740/1744 controls the host EISA bus as a master and transfers

data directly to and from main system memory. This implementation is

known as Bus Master DMA. Bus Master DMA greatly reduces the host

software overhead because the host CPU is no longer required to

maintain the DMA channel's address pointers and word counts. Bus

Master DMA also reduces the number of interrupts generated per I/O

command. The Intel BMIC includes the functions of the DMA controller.

Adaptec's implementation of Bus Master DMA can achieve a 33

MByte/second burst data rate. This speed is especially valuable in

multitasking systems where the tasks execute on a time shared basis.

Appendix A shows a diagram of the timing required to achieve the DMA

rates that are supported by the AHA-1740/1744.

The host adapter uses Burst Cycles on the EISA bus if the memory

supports the transfer by asserting SLBURST*. If not, the host adapter

will use 32-bit wide data transfers with the normal 2 cycle timing.

The adapter relies on system translation logic when reading or writing

16-bit expansion board memory in nonburst mode.

The AHA-1740/1744 DMA hardware will handle both odd-byte and

odd-memory address data transfers with no performance degradation. The

adapter has the ability to align bytes when the starting address is

not a multiple of four or the byte count is an odd value. It will

transfer 1, 2, or 3, bytes at the beginning or end of the transfer so

that 32-bit burst cycles may be used.

2-1

Section Two                                                     Architecture

------------------------------------------------------------------------------

The adapter will also be an 8-bit I/O slave with registers for use

during setup and operation. Two modes of operation are defined which

are mutually exclusive. The two modes are the AHA-1540-compatible

mode and the AHA-1740 mode. The current AHA-1540 ISA register set is

implemented for software compatibility. The I/O address is selected

by programming a configuration register. AHA-1740 mode is implemented

to give extended addressing ability as well as additional SCSI-2

features not available in Compatible mode. Several configuration

registers will be implemented in EISA I/O space to allow

autoconfiguration.

The user may program the adapter to use interrupts 9, 10, 11, 12, 14,

and 15. The interrupt may also be programmed to a high or low level.

When the high level is used, the board will be compatible with the ISA

implementation, and current drivers. When the low level is used, the

interrupt may also be shared and EISA drivers may be written to use

multiple boards on the same interrupt.

2.1.2 SCSI Interface and Protocol Chip (AIC-6251)

The host adapter supports SCSI functions that are a superset of

Adaptec's AT (AHA-1540 family) and Micro Channel (AHA-1640) host

adapters. The AHA-1740 will support new SCSI-2 features such as

tagged queuing and 10 MByte data transfers. It is, however, not

recommended that this data rate be implemented on a single-ended

cable, although short cables may support it. In particular, the

adapter will support synchronous negotiation to 10 MBytes, up to an

offset of seven, and it will support the Modify Data Pointers extended

message, Tagged Queuing, and Contingent Allegiance. Note that fast,

synchronous and asynchronous peripherals can be freely mixed on a

cable connected to the AHA-1740/1744.

In addition, the host adapter will have the ability to select

particular targets during configuration to initiate synchronous

negotiation and to allow disconnection. The current limiting fuse on

other host adapters has been replaced with a thermistor to allow

nondestructive current limiting.

The AHA-1740/1744 utilizes the Adaptec AIC-6251 Fast SCSI protocol

chip to maximize the SCSI bus utilization. The AIC-6251 is an Adaptec

VLSI development of the popular AIC-6250 device which allows the

AHA-1740/1744 to achieve greater than 2.0 MBytes/second asynchronous

SCSI data transfer rates, and up to 5.0 MBytes/second synchronous

transfer rates. The AIC-6251 will also enable the AHA-1740/1744 to

operate simultaneously as both an initiator and as a target device.

Through a 16-bit interface internal to the board, the AIC-6251 reduces

bus busy time during data transfer by allowing a bursting data across

the EISA bus at up to 33 MBytes/second. The AIC-6251 has separate data

buses for the local microprocessor and for the system data bus. This

further increases the performance of the AHA-1740/1744 by reducing the

overhead associated with SCSI commands.

2.1.3 16- and 8-Bit Memory

During normal DMA operations, nearly all transfers to and from memory

are 32-bit transfers. At the very end, or the very beginning of an odd

address boundary, an 8-bit or 16-bit transfer will occur.

2-2

Section Two                                                     Architecture

------------------------------------------------------------------------------

2.1.4 Bus Auxiliary Interface Chip (AIC-565)

This device replaces approximately 18 ICs in the bus interface. It

implements the following main functional blocks:

� Bus control interface for Compatible mode

� Host adapter microprocessor interface

� BIOS decode

The bus control interface section decodes the possible base addresses

for the board I/O port address, including a select external to the

board. It also provides all registers used to communicate between the

host adapter and the motherboard which are accessed through the bus in

Compatible mode. This ensures full compatibility with software written

for the AHA-1540/1542 family of AT Bus Master Host Adapters.

2.1.5 EISA Configuration Chip (AIC-575)

The host adapter provides a number of options which may be programmed

at configuration time. They are summarized below.

� AHA-1540 Base Port address

� Bus on time after preempt

� Interrupt definition and selection

� Synchronous negotiation enable/disable per target

� Disconnection enable/disable per target

� Boot Drive identification for DOS

� BIOS enable, location, and options

Current versions of the ASW-1400 family of software managers are not

capable of supporting these features. Revisions of the managers which

will are:

� ASW-1410, version 3.0 and later

� ASW-1420, version 2.0 and later

� ASW-1440, version 3.0 and later

� ASW-1450, version 1.0 and later

2.1.6 BMIC Bus Interface Chip (82355)

This device replaces approximately 50 ICs in the EISA bus interface.

It implements the following main functional blocks:

� 32-bit bus interface

� Adapter FIFO interface

� EISA bus protocol logic

2-3

Section Two                                                     Architecture

------------------------------------------------------------------------------

The local processor must load the I/O decode registers before loading

the ID in the ID registers. There are two decode groups, 0 and 1,

which are used for I/O address decode. The Group 0 registers decode

the address of the configuration group to CD0-CDF. In order to do

this, one must load values into the I/O Control Register and the I/O

Base Register . These registers are accessed by first loading the

index associated with that register in and then writing the data. The

EISA IDs are written in the same way.

2.1.7 Hardware Block Diagram

[Picture]

FIGURE 2.1. AHA-1740/1744 BLOCK DIAGRAM

2.2 COMPATIBLE MODE FIRMWARE

The AHA-1740/1744 support multithreaded SCSI initiator operation

through a simple mailbox protocol. The firmware accepts as many

Command Control Blocks (CCBs) as required and executes them from its

local RAM. The firmware controls all of the SCSI activity that a task

may require. This includes: Arbitration, Selection, Disconnection,

Reconnection, and Command completion.

Using the same mailbox protocol, the AHA-1740/1744 can operate as a

processor-type device serving as a multitasking target to other

initiators. This feature allows high bandwidth communication between

multiple hosts.

In addition, the AHA-1740/1744 firmware cooperates with the BIOS

installed on the host adapter to emulate the standard DOS BIOS calls.

This allows booting operations and the execution of standard DOS

operations from attached SCSI disks, allowing the SCSI subsystem to

completely replace the usual internal disk functions.

A multiuser, multitasking operating system issues a large number of

I/O tasks in a rapid sequence. The architecture of the AHA-1740/1744

makes management of this activity very easy and straightforward for

the operating system and its associated I/O drivers. This section

briefly explains the interaction between the system and the

AHA-1740/1744 required to accomplish an I/O task in compatible mode.

2-4

Section Two                                                     Architecture

------------------------------------------------------------------------------

2.2.1 Mailboxes

The AHA-1740/1744 Compatible Mode uses a mailbox architecture for task

communication between the host and host adapter. This allows the host

adapter to perform multithreaded operations with a minimum of host

intervention. The mailboxes are located in main system memory. Each

mailbox entry is four bytes long. After power-up sequencing, host

initialization procedures, and the boot procedure are completed, the

host issues an initialization command to inform the host adapter of

the mailbox location. There is always an equal number of Outgoing

Mailboxes (MBO) and Incoming Mailboxes (MBI). MBIs are located

immediately after the MBOs. A typical mailbox structure is:

[Picture]

In this example there are four MBOs and four MBIs. The first byte of

each MBO contains the MBO Command byte. The remaining three bytes

point to a Command Control Block (CCB). The CCB provides all the rest

of the information needed to complete a task. An MBO is available to

accept a new entry if the first byte is zero.

The first byte of each MBI contains the status of a completed task.

The remaining three bytes point to the CCB of the completed task. An

MBI is free if the Status byte is zero. Mailboxes may point to CCBs

controlling initiator tasks, controlling target tasks, or controlling

error recovery tasks.

2.2.2 Command Control Block

A Command Control Block provides the information required to control a

SCSI command sequence. The block contains pointers to the data area to

be used by the command. It contains areas for presenting status of

both the host adapter and the addressed SCSI device. In addition, it

contains the SCSI Command Descriptor Block defining the action to be

taken by the addressed SCSI device. An error information buffer area

is also provided.

A Command Control Block is also used to service an operation requested

by another initiator when the AHA-1740/1744 is being addressed as a

SCSI Processor device.

2-5

Section Two                                                     Architecture

------------------------------------------------------------------------------

The CCB is defined completely in Section 5.

A typical CCB is shown below:

[Picture]

2.2.3 Command Descriptor Block

The Command Descriptor Block (CDB), a part of the Command Control

Block, is a standard format command packet that is transmitted to the

addressed SCSI device. It contains all the command information

required by the SCSI device to perform the desired operation. The

Command Descriptor Block contains the command Operation Code followed

by a Logical Unit Number (LUN), command parameters if required, and a

control byte. A typical Group 0 6-byte CDB is shown below:

[Picture]

Please refer to the SCSI specification ANSI X3.131, the Common Command

Set (CCS) revision 4B, and the SCSI-2 draft for additional information

on Command Descriptor Blocks.

2-6

Section Two                                                     Architecture

------------------------------------------------------------------------------

2.2.4 Principles of Operation

At power-up, the host must inform the host adapter of the location and

number of mailboxes. To start a task, the host builds a CCB and stores

its memory address into a free mailbox. A nonzero Mailbox Out command

byte is then written to indicate that the mailbox entry is full and

valid. The host then writes to an I/O port (defined in Section 4) to

indicate that the host adapter should scan the MBO area. When a full

MBO is found, the host adapter copies the mailbox's CCB pointer into

its internal RAM and clears the Mailbox entry by writing a zero to the

MBO command byte. This frees the MBO so that it can be used to start

another task.

After completing a task, the host adapter scans the MBI area for a

free mailbox. When one is found, it is updated with the task's

completion status and CCB pointer. The CCB pointer identifies the

completed task. An MBI Stored interrupt is generated to notify the

host that a task has been completed. The host scans the MBI area

searching for a nonzero Status byte. When one is located, the host

obtains the CCB pointer and frees the MBI by writing a zero into the

Status byte. The host then examines the contents of the CCB to

determine that the command was successfully completed. The freed MBI

can now be used to indicate the completion of another task. The host

adapter fills the MBI area and scans the MBO area in a round-robin

fashion. If the host saves the position of the last active MBI entry,

it can determine the MBI of a new entry immediately without searching,

since a new entry will be in the next MBI location.

The host adapter transmits a new MBO Available or MBI Full interrupt

to the host whenever all non-Mailbox interrupts have been cleared and

serviced by the host. The host should analyze the interrupts and clear

them as soon as possible so that the host adapter can post any new

interrupts quickly. The host adapter will not wait until an interrupt

can be transmitted to the host before processing an MBO entry or

creating a new MBI entry. Thus, in processing a single MBI interrupt,

the host may find several MBI entries waiting by the time the

interrupt processing is finished. Similarly, a later MBI interrupt for

the last of the later MBI entries may find nothing to service because

the MBI entry was examined and processed as a result of the first MBI

Full interrupt. If the interrupts are reset quickly by the host, the

probability of an interrupt occurring when no MBI entry is available

is much lower, providing an important performance improvement. If the

MBI entries are emptied by the host in a round-robin order, the scan

for the next full entry is very simple, since it is always the next

MBI entry in the Mailbox area.

2.2.5 Task Queuing

Multiple tasks may be started against a target/LUN or against multiple

targets/Logical Units. Since only one task can be active against any

one LUN at a time, all other tasks for the same LUN are queued. Other

LUNs may have active tasks at the same time.

The host adapter dequeues on a first in, first out (FIFO) basis for

each target/LUN combination. However, due to the optimization

algorithm used by the host adapter, a task may sometimes be started

earlier in spite of its late entrance in the queue on multiple

target/LUN systems. Task queuing should not be used where changes in

the order of command execution may cause data integrity failures.

2-7

Section Two                                                     Architecture

------------------------------------------------------------------------------

2.3 ENHANCED MODE FIRMWARE

The Enhanced mode is an interface architecture which allows the

AHA-1740/1744 to take full advantage of the EISA bus facilities. It

utilizes a distinct hardware interface control logic, implemented in

the AIC-575 device. This permits features such as full 32-bit

addressing and the entire EISA register set to be used. This mode is

not compatible with older versions of the ASW-1400 series of software

managers. The following levels of manager revision are capable of

operating with the AHA-1740/1744 in Enhanced mode:

� ASW-1410 version 2.2 and later (ASPI for DOS)

� ASW-1420 version 2.0 and later (ASPI for OS/2)

� ASW-1440 version 2.1 and later (ASPI for NetWare)

In order to use the Enhanced mode, the AHA-1740/1744 must be correctly

reconfigured by using the configuration utility. The AHA-1740/1744 are

not capable of simultaneous operation in Compatible and Enhanced mode.

The firmware in this mode uses a different architecture to the

Compatible mode. The firmware is interrupt driven by events on the

host and SCSI side. It uses only a single mailbox, unlike the

Compatible mode which uses up to 255. This does not result in any

bottleneck or performance impairment, due to the fast processor used

on the AHA-1740/1744 and the servicing algorithm used.

The Control Block is a 48-byte structure created and maintained in

shared memory by software in the system unit. It is used to convey

requests to the host adapter.

|--------------------------------------------------------------|

|                                                      Byte    |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0     Offset  |

|--------------------------------------------------------------|

|   Command Word                                         00    |

|--------------------------------------------------------------|

|   Flag Word 1                                          02    |

|--------------------------------------------------------------|

|   Flag Word 2                                          04    |

|--------------------------------------------------------------|

|                                                        06    |

|--------------------------------------------------------------|

|   Data or Scatter/Gather List Pointer                  08    |

|                                                        0A    |

|--------------------------------------------------------------|

|   Data or Scatter/Gather List Length                   0C    |

|                                                        0E    |

|--------------------------------------------------------------|

|   Status Block Pointer                                 10    |

|                                                        12    |

|--------------------------------------------------------------|

|   Chain Address                                        14    |

|                                                        16    |

|--------------------------------------------------------------|

|                                                        18    |

|--------------------------------------------------------------|

|                                                        1A    |

|--------------------------------------------------------------|

|   Sense Information Pointer                            1C    |

|                                                        1E    |

|--------------------------------------------------------------|

|   CDB Length               I   Sense Length           20     |

|--------------------------------------------------------------|

|   Data Checksum                                        22    |

|--------------------------------------------------------------|

|   CDB Bytes 1              I   CDB Byte 0             24     |

|--------------------------------------------------------------|

|   CDB Bytes 3              I   CDB Byte 2             26     |

|--------------------------------------------------------------|

|   CDB Bytes 5              I   CDB Byte 4             28     |

|--------------------------------------------------------------|

|   CDB Bytes 7              I   CDB Byte 6             2A     |

|--------------------------------------------------------------|

|   CDB Bytes 9              I   CDB Byte 8             2C     |

|--------------------------------------------------------------|

|   CDB Bytes 11             I   CDB Byte 10            2E     |

|--------------------------------------------------------------|

A full description of the control block and its operation is given in

Section 6.

2-8

Section Two                                                     Architecture

------------------------------------------------------------------------------

2.4 ONBOARD BIOS OPERATION

A host adapter BIOS is provided to emulate the standard hard disk BIOS

and boot functions. With this BIOS, the host adapter can be used in

lieu of a standard hard disk controller on any ISA-compatible system.

The BIOS is compatible with the standard hard disk BIOS. This allows

DOS to access up to two hard disk devices on the SCSI bus without a

driver. All normal I/O functions are supported including system

booting. Single-threaded operation and multithreaded operation do not

operate simultaneously. Single-threaded operation cannot be requested

until all multithreaded operations are completed. Similarly, all

multithreaded operations must be complete before a single-threaded

operation can be requested. For most multitasking operating systems,

such as Xenix and OS/2, single-threaded operation is normally used

only for the early part of the boot operation, after which

multithreaded drivers take over all control of the SCSI operations.

During system boot, the BIOS will scan for the availability of

configuration information in the Free-form data area of system RAM. If

it finds the correct data format, it will use this data to configure

both the board parameters and parameters associated with the

peripherals on the SCSI bus. If data cannot be found or correctly

recognized, the BIOS will configure the board to a default set of

parameters. Refer to Section 3, "Installation" for details.

.c9.2.5 POWER-UP DIAGNOSTICS

After power on, the host adapter initializes the firmware to 16-bit

RAM for execution. After the firmware is running, the Extended ID is

written to the registers. Several onboard diagnostics are then

performed.

� RAM will be verified.

� Write/Read registers will be checked for proper operation.

� The Data path will be checked for correct internal operation.

The red light-emitting diode (LED) on the host adapter indicates the

result of the self diagnostic process. When power is first applied to

the board, the LED turns on. If the board is operating normally, the

light will soon go off and stay off until SCSI or I/O port activity is

requested by the host. If the board is not operating correctly, a

flash code number is flashed on the LED to indicate which test failed.

The flash code number is indicated by a series of one to three closely

spaced flashes, followed by a longer pause, or a continuous series of

flashes. The flash code is repeated continuously until the board is

powered off or reset. The SCSI interface should be disconnected if

these diagnostic tests are being run for fault isolation purposes. At

least one set of terminators must remain installed or the LED will

stay on, indicating that the AHA-1740/1744 is receiving an active RST

signal. The flash code and associated failure modes are indicated in

the following table.

2-9

Section Two                                                     Architecture

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Flash Code          Possible Meanings of Flash Code

------------------------------------------------------------------------------

LED Remains On      Host Adapter Control Processor

inoperative/terminators missing or not

powered or card enable has not been

asserted after reset.

1 Flash             RAM test failed.

2 Flashes           AIC-6251 SCSI protocol chip verification

failed.

3 Flashes           FIFO write/read data path test failed.

Continuous Flashes  EEPROM has not been programmed.

------------------------------------------------------------------------------

2.6 ASW-C174 CONFIGURATION UTILITY

The AHA-1740/1744 is normally supplied with configuration software,

product number ASW-C174. This includes a 5 1/4" and a 3 1/2"

high-density diskette and an installation guide. The disks contain

identical information. The functions of the files on disk may be

supplied as part of the system utilities for the host system in which

the AHA-1740/1744 is installed. The utilities disk contains the

following files:

�        !ADP0000.CFG configuration file for the AHA-1740

�        !ADP0400.CFG configuration file for the AHA-1744

At the time of writing, these utilities were still in development and

earlier versions of ASW-C174 may not contain all features listed.

Operation of the utilities is described in Section 3 on Installation

and Section 10 Problem Determination. If the board has no ASW-C174

accompanying it, or the utility is incomplete and the system utilities

do not include the function needed, either contact the board vendor

for a copy or query the Adaptec bulletin board for a download copy of

the files. The bulletin board number is (408) 945-7727     (8 data

bits, 1 stop bit, no parity, 1200 or 2400 baud).

The utility disk does not include copies of the microcode files to be

downloaded by the download utility. These are normally not required,

except in the case of microcode upgrade. They are available separately

from Adaptec.

The freeform data area is used to configure the SCSI bus and is not

specified by the EISA specification. Adaptec uses a data structure for

each device (ID #n) consisting of two bytes, shown in the following

table.

2-10

Section Two                                                     Architecture

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Byte 2n+1                       Byte 2n

------------------------------------------------------------------------------

bit 7-3 reserved                bit 7   reserved

bit 2-0 maximum sync Xfer rate  bit 6   More than 1 LUN supported

000   10.0 MBytes/second    bit 5 Parity check enable

001    6.67 MBytes/second   bit 4 Send start command

010    5.0 MBytes/second    bit 3 Sync negotiation enabled

011    4.0 MBytes/second    bit 2 Disconnection enabled

100    3.33 MBytes/second   bit 1-0 Power-on information

00 not installed

01 installed, report error

10 reserved

11 installed, do not report error

------------------------------------------------------------------------------

� !ADP0000.ON Configuration overlay file for the SCSI bus.

� ADL.EXE Microcode download utility, including low-level format utility.

� STANDARD.HEX Current production release of standard mode microcode.

� ENHANCED.HEX Current production release of enhanced mode microcode.

2-11

Section Three                                                   Installation

------------------------------------------------------------------------------

3.1 UNPACKING AND INSPECTION

The carrier is responsible for damage incurred during shipment. In

case of damage, have the carrier note the damage on both the delivery

receipt and the freight bill, then notify your freight company

representative so that the necessary insurance claims can be initiated.

After opening the shipping container, use the packing slip to verify

receipt of the individual items listed on the slip. Retain the

shipping container and packing material for possible later reuse

should return of the equipment to the factory be necessary.

CAUTION: The AHA-1740/1744 is carefully designed to resist the effects

of static electricity. However, like all electronic equipment, it can

be damaged or its life can be shortened by unusual static discharges.

Please take the proper precautions when handling the board. Keep the

board in its conductive wrapping until it is ready to be installed in

your system. Be sure that the host computer and the personnel handling

the board are properly grounded while installing the board.

3.2 INSTALLATION

The following section details the installation procedure for the

Adaptec AHA-1740/1744 AT-to-SCSI host adapter. The installation of the

board consists of unpacking the board, preparing the SCSI devices,

installing the correct terminations, inserting the board into a

full-length EISA-compatible connector, and connecting a SCSI cable

from the onboard connector to a SCSI target. The system is then

powered up, the microcode downloaded and the board configured for operation.

The Adaptec AHA-1740/1744 EISA-to-Fast SCSI host adapter has been

designed to operate as shipped in standard EISA class computers. The

board (or the system in which it is installed) is normally shipped

with a configuration disk which permits the board to be configured to

the actual slot location in which it is installed. Unlike AT/ISA

boards, but like Micro Channel boards, EISA boards do not normally

require hardware jumpers.

Ensure that you have the correct version of the AHA-1740/1744 for your

system. The AHA-1740 supports the more common single-ended SCSI

interface. The AHA-1744 supports the differential interface, more

common on SCSI peripherals on minicomputers. While the two use the

same signal protocol, they are not electrically compatible.

3-1

Section Three                                                   Installation

------------------------------------------------------------------------------

3.2.1 System Configuration Background for Compatible Mode

The DOS operating system and standard BIOS supports two hard disk

drives; drive C: and drive D:. If two standard hard disk drives are

installed, they are the only hard disk drives accessible from the

operating system. If one standard hard disk drive is installed, the

AHA-1740/1744 BIOS allows DOS to access the SCSI drive with the

Target:LUN address of 0:0 as the second of the two supported drives

(drive D:). If no standard hard disk drives are installed, the

AHA-1740/1744 BIOS allows DOS to access the SCSI drive with the

address of 0:0 as the first of two supported drives (drive C:) and the

SCSI drive with the address of 1:0 as the second of the drives (drive

D:). System booting is performed from the floppy drive if a floppy

diskette is installed. If no floppy is installed, system booting is

attempted from the drive chosen as drive C: through the above process,

whether the drive is a standard hard disk or a SCSI hard disk. The

AHA-1740/1744 BIOS fully supports the extended partitioning

capabilities of DOS for up to two drives. Adaptec supplies a range of

products based on the Advanced SCSI Programming Interface (ASPI)

architecture that allow the support of more than two physical or

logical devices under DOS. Many other operating systems, including SCO

Xenix and Unix, also have this feature and will allow the access of

any number of attached SCSI devices. Refer to Section 8 for more

details. Peripheral devices such as SCSI tape, DAT, CD-ROM and others

require device driver software to be installed.

The term "standard hard disk" refers to the disks attached to the

system by a standard ISA disk controller. These standard hard disks

can be set to the "Installed" or "Not Installed" state by the SETUP

program that is supplied with each ISA host computer. The SETUP

program allows the user to select the number of standard hard disks

that are recognized by the system regardless of whether they are

physically installed.

3.2.2 Preparation

Few preparatory steps need to be taken to install the host adapter in

the host computer. There are no initialization jumpers to be placed,

but the SCSI Bus terminators must be installed in the correct SCSI

devices and the correct SCSI addresses must be assigned to each

peripheral device.

Each EISA system is shipped with a configuration utility to assign

parameters such as port addresses and interrupt priorities to boards

installed in the system. The details vary from vendor to vendor. Refer

to the system documentation for details.

The Adaptec AHA-1740/1744 32-bit EISA bus-to-SCSI bus host adapter has

been designed to operate as shipped in the majority of EISA class

computers. It is normally shipped with the ASW-C174 Configuration and

Download Utilities. These utilities permit parameters associated with

the board and the SCSI bus to be set up through the system keyboard

and CRT.

3-2

Section Three                                                   Installation

------------------------------------------------------------------------------

3.2.3 Termination

The SCSI bus must be terminated correctly to assure proper

operation. The first and last physical SCSI devices on the SCSI

cable must have terminators installed. All other SCSI devices must

have terminators removed. The host adapter is shipped with

terminators already installed at locations RN3, RN4, and RN5. The

internal and external connectors connect to the same SCSI bus, so

both internal and external cabling must be considered in determining

where terminators are installed.

If only one cable (either internal or external) is connected to the

host adapter, the terminators must remain installed in the host

adapter. Terminators must also be installed on the device at the

farthest end of the cable from the host adapter. Terminators must be

removed from all other attached SCSI devices.

If both an internal and an external cable are connected to the host

adapter, remove the terminators on the host adapter and install

terminators on the devices at the farthest end of each cable.

Terminators must be removed from all devices except the device at the

end of each cable. The instruction manuals for each SCSI device will

indicate how the terminators can be removed or replaced.

3.2.4 Addressing

The SCSI target address for each SCSI device to be attached must be

selected by setting the proper jumpers or switches on the device. If

the SCSI device is a hard disk that is to be used as the boot disk, it

must have the Target Identifier (SCSI Address) set to zero and the

Logical Unit Number set to zero. SCSI Addresses 0 and 1 should be

reserved for SCSI hard disk drives. SCSI Address 2 is often used for

tape devices. Each installed peripheral must have a different Target

Identifier. The host adapter's default address is 7. Duplicate SCSI

Addresses will cause errors that are extremely difficult to identify.

Any jumpers that control operating modes must also be properly set. If

there is a jumper that enables synchronous transfer negotiation, the

jumper should be set to enable negotiation. Check the SCSI drive or

controller to ensure that Parity Checking is enabled. If Parity

Checking on the device is disabled or not supported, the Parity

Checking must be disabled for that device by the configuration

utility. Parity checking should only be enabled if all SCSI devices

support it.

3.2.5 Installation Procedure

Now that the host adapter and the devices to be installed have

been properly prepared, the devices should be installed in the

following manner:

1)  Turn OFF the power to the computer system.

2)  Remove the cover of your EISA personal computer to expose

the EISA bus slots on the motherboard.

3)  Locate an unused EISA slot in your system.

4)  Remove the corresponding system expansion slot cover by

turning the screw that secures it from the top counterclockwise.

3-3

Section Three                                                   Installation

------------------------------------------------------------------------------

5)  Align the EISA Bus Connector on the bottom of the

AHA-1740/1744 to the open chassis slot with the slot cover

removed. Ensure the external connector and bracket pass

cleanly through the cutout in the rear wall.

6) Firmly plug the board into the slot. Use the screw from the corresponding

expansion slot cover to secure the board bracket to the system frame.

7) If an internal SCSI peripheral is to be used, install a

50-pin SCSI ribbon cable to the host adapter. This cable must

be oriented correctly. Pin 1 of the SCSI cable is designated

by a red stripe. Multicolor 50-pin ribbon cables signify pin

1 with a brown color. Pin 1 on the host adapter 50-pin SCSI

header is located on the lefthand side, farthest from the

installation bracket and is designated by the words PIN 1 on

the board adjacent to the header. After locating pin 1 on the

host adapter and on the SCSI cable, carefully insert the

connector located at the end of the long end of the cable

into the host adapter connector. After ensuring that all pins

are lined up and that the pin 1 orientation is correct,

firmly seat the connector to the board such that the locking

ears snap in place to hold the cable firmly. If it is ever

necessary to remove the cable, gently push the two locking

ears horizontally outwards along the axis of the board until

the cable connector is pushed upward and free. Ensure that

the external device drive types all correspond with the

Single-Ended/Differential marking on the bracket of the

AHA-1740/1744, respectively.

WARNING: The AHA-1740 must be used only with Single-Ended Drive

SCSI Devices. The AHA-1744 must be used only with Differential

Drive SCSI Devices. Failure to match drive types may result in

electrical damage to the board and the peripherals.

8) If an internal SCSI device is also being installed, it should be

installed in the drive bays in accordance with the directions on

the peripheral at this time. The proper power supply connection

also must be made to the SCSI peripheral device.

9) The 50-pin SCSI ribbon cable can now be attached to each internal

SCSI device. Refer to the device's installation instructions to

ensure proper pin 1 orientation. Pin 1 orientation must be

consistent throughout the system. Keep the ribbon cable neatly

dressed away from the ventilation slots in the computer system.

Keep the ribbon cable dressed away from possible electrical noise

sources or noise sensitive components, particularly large

microprocessors, memory boards, switching power supplies, and

analog data acquisition boards. If the internal configuration

requires the cable to come near noise sensitive circuits, make sure

that the cable crosses the boards at right angles and is near the

noise sensitive circuits for the shortest distance possible.

10) Carefully reinstall the cover of the computer.

11) If an external SCSI subsystem is being installed, it can now be

cabled to the External SCSI Connector projecting from the shielding

bracket on the back of the AHA-1740/ 1744 host adapter. The

external connector on the AHA-1740/1744 is a small form factor SCSI

alternate-2 D shell connector that ensures correct pin 1

orientation on the host adapter. The proper shielded SCSI cable

must be used for proper operation. Ensure that the external device

drive types all correspond with the Single-Ended/Differential

marking on the bracket of the AHA-1740/1744 respectively.

3-4

Section Three                                                   Installation

------------------------------------------------------------------------------

WARNING: The AHA-1740 must be used only with Single-Ended Drive

SCSI Devices. The AHA-1744 must be used only with Differential

Drive SCSI Devices. Failure to match drive types may result in

electrical damage to the board and the peripherals.

The subsystem, cables, and SCSI terminators must be installed in

accordance with the directions provided with the external SCSI

subsystem. The addresses selected for external SCSI devices must

not overlap with the addresses of the host adapter or any other

SCSI devices attached internally.

3.2.6  Checklist

Before applying power to your system, the following items should be

completed and checked:

1. The 50-pin SCSI ribbon cable is connected to the host adapter with

proper pin 1 orientation.

2. The host adapter is firmly seated in the host computer's adapter

slot.

3. The correct SCSI addresses are selected on all attached SCSI

devices. Address 0 is reserved for the boot hard disk and address 1

is used for a second hard disk.

4. The correct operating modes are selected on all attached SCSI

devices.

5. Terminators are installed or removed on the drives and host adapter

as required.

6. External SCSI devices are properly installed and cabled.

3.2.7  Terminators

The SCSI bus must also be terminated correctly to ensure proper

operation. The first and last physical SCSI devices on the SCSI cable

must have terminators installed. All other SCSI devices must have

terminators removed. The AHA-1740/1744 host adapter is usually the

first device on the SCSI Bus and has terminators installed at the factory.

3.3 SYSTEM CONFIGURATION

When it is being installed in an EISA system, the AHA-1740/1744

requires an EISA system configuration file (file name: !ADP0000.CFG

for the AHA-1740 and !ADP0400.CFG for the AHA-1744, with an additional

overlay, ADP0000.OVL, for configuring SCSI peripherals on the bus).

This may be part of the system software or may be supplied on a

separate diskette. If it is separate, installation is easier if the

contents of the separate diskette are copied onto the main bootable

system configuration disk.

To perform system configuration:

Place the bootable configuration diskette in an operative drive and

reset the system to boot from this diskette.

3-5

Section Three                                                   Installation

------------------------------------------------------------------------------

Ignore any error which indicates that an unknown board has been

detected in the system. Selection of board configuration varies with

the configuration utility supplied with the EISA system. There are two

main types, supplied by MCS and Phoenix. The type may be obscured by

the screen banner used by the system vendor. Follow the procedure

which is closest to your environment. In the case of the MCS

configuration, type "CF". For Phoenix configuration type "PTLECU".

Select board configuration and press <RTN> .

If the main configuration disk does not contain files for the

AHA-1740/1744, place the ASW-C174 diskette with the AHA-1740/1744

configuration files, copy the contents to the boot floppy and run the

configuration program (file name: !ADP0000.CFG for the AHA-1740 and

!ADP0400.CFG for the AHA-1744, plus ADP0000.OVL for either board).

The program will autoconfigure the system and display a diagram of the

motherboard showing which boards have been configured into which

slots.

3.3.1  MCS Configuration

For configuration under MCS, select the slot in which the

AHA-1740/1744 is installed and press <RTN>. The screen will display

the System Configuration Detailed View. These settings should normally

not be changed for standard mode operation. Use the cursor arrows to

make selections and the <RTN> key to enter the selection. When

installing multiple AHA-1740/1744s in Standard Mode, ensure that none

share the same configuration parameters for port address. The

configuration utility normally automatically ensures that there is no

conflict. SCSI ID can be the same provided that the boards do not

share the same SCSI bus. Normally, the host adapter is given address

7. A typical selection detailed view is shown in Figure 3-1. Use the

arrow keys to move around in a selection and the Enter or Return key

to choose a highlighted selection. If a mouse is installed, it may

also be used.

[Picture]

FIGURE 3-1. SYSTEM CONFIGURATION DETAILED VIEW

3-6

Section Three                                                   Installation

------------------------------------------------------------------------------

Normally, it is not necessary to alter any settings for configuration.

Selection of the appropriate interrupt request will also select

between Compatible and Enhanced Mode. If selecting Enhanced Mode, it

is not necessary to alter the system resource selection, but if

resources are left selected, they will be allocated, even if not used.

Select the AHA-1740/1744 installation configuration as follows:

1) Select the host adapter interface. There are two available. Standard

allows software written for the AHA-1540 or AHA-1640 families to run

the AHA-1740/1744. Enhanced Mode allows a higher-performance

interface to be used. If Standard Mode is selected, the host adapter

interrupt level must also be selected (see Figure 3-2).

[Picture]

FIGURE 3-2. INTERFACE MODE AND INTERRUPT SELECTION DISPLAY

If the Enhanced Mode is selected for a board which has Compatible Mode

firmware loaded, the LED on the board will blink repeatedly and the

only operation the board will perform is a Microcode Download.

2) If Standard Mode is selected, choose both the host adapter port

address and DMA channel. For Enhanced Mode, both should be disabled.

3) Select the base address of the bootstrap BIOS on the host adapter.

4) Select the device address for the host adapter on the SCSI bus.

5) Select whether the host adapter generates a reset on the SCSI bus at

power-on or reset.

6) If required, select the device configuration to select parameters on

the SCSI bus. A pop-up window appears. Select the host adapter

interface mode chosen in Step 1.

3-7

Section Three                                                   Installation

------------------------------------------------------------------------------

3.3.2  Standard Mode SCSI Configuration

The view for Standard Mode is a simple selection for those variables

which apply to all SCSI peripherals connected. This is shown in Figure

3-3. Normally, the default of one device at SCSI ID#0 with normal

default parameters is selected.

[Picture]

FIGURE 3-3. STANDARD MODE SCSI CONFIGURATION

Select "Enable Parity Checking" to allow the host adapter to generate

parity on data sent and check parity on data received on the SCSI bus.

Select "Initiate Synch Negotiation" to allow the host adapter to

attempt to use the faster synchronous protocol for data transfers on

the SCSI bus. If deselected, the transfers will be done

asynchronously, unless a SCSI device itself requests synchronous

negotiation.

Select "Enable Disconnection" to allow a peripheral to go "off-line"

while it completes a lengthy operation like a seek. This allows the

host adapter to perform other operations on the SCSI bus while the

device is temporarily disconnected.

3-8

Section Three                                                   Installation

------------------------------------------------------------------------------

3.3.3  Enhanced Mode SCSI Configuration

The view for Enhanced Mode allows more flexible selection and is shown

in Figure 3-4.

[Picture]

FIGURE 3-4. ENHANCED MODE SCSI CONFIGURATION SCREEN

The enhanced mode offers a greater selection of SCSI configuration

options. Use the Tab or Cursor keys or a mouse to highlight a

particular selection and press <Return> to toggle between "no" and

"yes." Each device can have a number of parameters selected

individually.

"Error if Device Not Found" selects whether the operator will be

notified if the BIOS is unable to find the device during the boot

procedure.

"BIOS Support Option" allows up to two devices attached to the SCSI

bus to be recognized by the host adapter BIOS and installed as devices

on the system without the need for driver software. Only fixed and

removable hard disks are supported under the BIOS, with default for

hard disks only. Select this option to allow removable support or to

remove any disk support for each SCSI address. Note that support for

removable disks does not allow the disk media to be removed during

operation _ only use of a removable disk as if it were a fixed disk.

3-9

Section Three                                                   Installation

------------------------------------------------------------------------------

When this option is selected, use the cursor keys to move among the

options shown in Figure 3-5.

[Picture]

FIGURE 3-5. BIOS SUPPORT SELECTION SCREEN

"Send Start Command" is used for devices which require a command to

start up after power on. Most devices do not require this.

"Enable Parity Check" is similar to Standard Mode where the host

adapter will generate and check for parity on SCSI bus data.

"Initiate Synch Negotiation" is similar to Standard Mode and allows

the host adapter to attempt to communicate with the device using the

faster synchronous SCSI protocol. Most disk drives support synchronous

protocol.

"Enable Disconnection" is similar to Standard Mode. It allows the

device to go off-line while performing an operation, freeing up the

SCSI bus and host adapter for other operations in parallel.

"Maximum Synch Transfer Rate" allows selection of the highest data

transfer speed that the host adapter will attempt with that

peripheral. The AHA-1740/1744 supports rates up to the Fast SCSI

maximum of 10 MBytes/second.

3-10

Section Three                                                   Installation

------------------------------------------------------------------------------

When the selection is completed, press <ESC> to reach the SCSI

configuration exit menu shown in Figure 3-6.

[Picture]

FIGURE 3-6. ENHANCED MODE CONFIGURATION EXIT SCREEN

Use the cursor keys or mouse to select whether to make the changes or

abandon all changes made on this attempt. When the correct line is

highlighted, press <ESC> to select.

Exit the configuration utility by hitting CTRL + X.

Select the Save Configuration and Exit option using the cursor arrows

and press <RTN>.

Reboot from the original boot diskette.

3.3.4  Phoenix Configuration

Configuration using the Phoenix utility is similar to that under MCS.

Use the arrow keys to select the board to configure after entering the

Configuration menu, selecting function choices and pressing <RTN>.

Press <RTN> when the appropriate board slot is highlighted.

Selection among the available system options is made by using the

up/down arrows. Press the spacebar at the option to alter for a

toggling among options available. There is no facility for selection

of peripheral options under Phoenix. To exit, skip to the bottom of

the page and press <RTN> when "OK" is highlighted.

3-11

Section Three                                                   Installation

------------------------------------------------------------------------------

3.4  FIRMWARE DOWNLOAD

The AHA-1740/1744 has the unique flexibility of allowing firmware to

be altered or upgraded while the board is installed in a system. This

is done by using a utility on the ASW-C174 and either the firmware

files supplied or new files supplied with ASW-M174. Note that it is

not necessary to perform a download when installing a system for the

first time or to select between Standard and Enhanced Mode.

The microcode with which the AHA-1740/1744 is equipped is available in

two main versions:

_ Standard Mode

_ Enhanced Mode

Selection between these modes affects the software manager required to

support the board. It has no effect on the SCSI or host bus hardware.

The firmware download can either be used to configure the board for

one of the two main modes or to upgrade the board to a later revision

of firmware for a particular mode.

The Standard Mode allows software drivers written for the AHA-1540 or

AHA-1640 families to run unaltered on the AHA-1740 family. There is no

performance penalty for this _ the AHA-1740/1744 performs 32-bit

transfers at speeds up to 33 MBytes/second and can operate 10

MByte/second Fast SCSI transfers.

The Enhanced mode allows the board to take advantage of a number of

features which were not available on earlier host adapters. These

include:

� 32-bit addresssing capability

� Single fast mailbox handling

� Contingent allegiance (SCSI)

� Tagged queuing (SCSI)

Adaptec's AHA-1740/1744 Download Utility should be used to perform a

firmware download or to upgrade the installed microcode to a new

revision to retrieve firmware information and also format attached

drives. This utility contains three main features:

� Download Firmware

� Firmware Information

� Low-level disk format

Firmware download causes the host adapter to overwrite its program

memory with microcode supplied by the user from an external source

like the ASW-C174. After the data is written to the host adapter's

internal RAM, and a successful checksum test done by the host adapter,

the host adapter will then reprogram its EEPROM with the new firmware.

In order to change the host adapter current interface mode, the user

must reconfigure the system memory by running the Configuration

Utility (described above).

3-12

Section Three                                                   Installation

------------------------------------------------------------------------------

Firmware Information is used to obtain release information and

revision level of the host adapter's firmware. Use this command to

ensure that proper firmware is loaded or to confirm the mode in which

the board is currently operating.

3.5  CONFIGURATION OPERATION

Insert the utility floppy into Floppy Drive A. At the DOS prompt, type

"ADL" to bring up Adaptec's AHA-1740/1744 Download Utility. The

following will describe the procedures and the screen display. During

the operation, <F1> can be pressed to display help messages and <ESC>

to exit the current message box and the utility. If using a monochrome

or gray-scale monitor, type "ADL -M" to improve viewing contrast.

3.5.1  List of Adapters

When the utility is first loaded, a list of installed host adapter(s)

is displayed at the upper left corner of the screen. Along with each

installed host adapter is its configured mode. (Note that the user

must run the System Configuration Utility in order to change the mode

in which the host adapter is selected to operate). If more than one

host adapter is installed, use the up and down arrow key to choose the

host adapter. When the host adapter to be selected is highlighted,

press <ENTER> to select.

The <ESC> key can be used to exit the utility. When using other parts

of the utility, return to this screen to exit. If either the Download

Firmware or Low-level Format utility has been used, the system will

reboot automatically to allow correct configuration.

3.5.2  Main Menu

After selecting the host adapter, a main menu will appear. Three

operations are provided. They are:

� Download Firmware

� Firmware Information

� Low-level Format

Use the up and down arrow key to choose the operation. Press <ENTER>

to start an operation. Each operation is described below.

3.5.3  Download Firmware

Have the new firmware ready on a floppy diskette or copy it onto the

internal hard disk drive. The new firmware will come with its checksum

value on the floppy diskette label.

First, an edit form will pop up when this operation is chosen. Use the

arrow keys to move from field to field, or within the field. Type in

the firmware file name (example : A:\phase1.hex) that is going to load

into the host memory, and choose the mode (Standard or Enhanced) that

is going to download by toggling with the <+/_> key. Press <ESC> to

exit the form and continue.

3-13

Section Three                                                   Installation

------------------------------------------------------------------------------

After exiting from this screen the new firmware file will be

automatically loaded into the host memory. The checksum will be

calculated and displayed if the firmware file is loaded successfully.

At this point, verify the displayed checksum with the checksum on the

floppy label. Press <ESC> to cancel the operation. Press <Y> to

continue. This will issue the DOWNLOAD FIRMWARE command to the host

adapter. The whole process may take up to 45 seconds. A message will

indicate the success or failure of downloading the new firmware.

Subsequently, a confirm message window will be displayed to allow

verification of the download operation. If the displayed information

is valid, press <ESC> to continue and return to the main menu.

3.5.4  Firmware Information

This operation queries the board for the release, revision, and

checksum information of microcode currently on the host adapter. If

the command is successfully completed, all information that is

retrieved from the host adapter will be displayed. Otherwise, an error

message will be displayed. After reading the host adapter information,

press <ESC> to continue.

If multiple host adapters are installed, the <ESC> key can be used to

select another host adapter from the List of Host Adapters menu.

3.6  LOW-LEVEL FORMAT UTILITY

The Low-Level Format Utility is used to reformat a hard disk and erase

all previous information stored on the disk. This may be necessary

with disks formatted to operate with a different system or SCSI

controller.

For format utility is entered as a part of the Down Utility described

above. Run the ADL and select "Low-Level Format" from the menu

displayed.

Choosing the option will display a "List of Device(s)," which contains

the list of all attached SCSI device(s) on the previously selected

host adapter. Press <ESC> to return to the main menu or <ENTER> to

select the SCSI drive to be formatted. Note that the only device type

that is currently supported is Direct Access Device.

After choosing a SCSI device, a menu will be displayed to allow the

user to choose either the format and verify option, or the wipe boot

sector option. Press <ENTER> to select. A warning message will be

displayed to alert the user that this particular operation will result

in losing the data content that is stored on the drive. The user can

either press <Y> to continue the operation or <ESC> to abort the

operation.

Appropriate messages for successful or unsuccessful completion of the

operation are displayed. Simply press <ESC> to continue and return to

the main menu.

3-14

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

4.1 HARDWARE OVERVIEW

This section provides a description of the AHA-1740/1744 hardware

functional interface to the EISA host software.

The hardware consists of a custom SCSI protocol chip, the AIC-6251,

the EISA Interface Controller, the 82355, a FIFO buffer, a set of I/O

ports, a controlling microprocessor and both the AIC-565 Compatible

mode control interface and the AIC-TBD AHA-1740 mode interface.

The DMA control logic in the 82355 controls the bus arbitration and

data transfer handshaking. During DMA data transfers, the

AHA-1740/1744 becomes a bus master. The DMA logic supports odd- and

even-byte and odd- and even-word starting addresses. For odd-byte

starting addresses, the first transfer will be an 8-bit transfer. For

odd-word starting address, the first transfer will be a 16-bit

transfer.

The SCSI port is controlled by the AIC-6251, an Adaptec SCSI protocol

device for fast (10- MByte/second transfers) which supports

arbitration, selection, and reselection with a minimum of processor

intervention. This VSLI device also supports target mode (simultaneous

to initiator mode) and synchronous SCSI transfers.

4.2 COMPATIBLE MODE I/O PORT INTERFACE

The I/O port interface consists of three address locations. These

three port addresses are decoded in the AT I/O address space. They

form the primary communications channel between the host and the host

adapter. The I/O ports are eight bits wide. The base port is for

control and status, the second port for command and data transfer, and

the third port for interrupt flags.

4-1

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

I/O PORT INTERFACE BIT DEFINITION

PORT ADDRESS = BASE + 0

WRITE: Control Register           READ:  Status Register

Bit 7  Hard Reset (HRST)      Bit 7   Self Test in Progress (STST)

Bit 6  Soft Reset (SRST)      Bit 6   Internal Diagnostic Failure (DIAGF)

Bit 5  Interrupt Reset (IRST) Bit 5   Mailbox Initialization Required(INIT)

Bit 4  SCSI Bus Reset (SCRST) Bit 4   SCSI Host Adapter Idle (IDLE)

Bit 3  Reserved (0)           Bit 3   Command/Data Out Port Full (CDF)

Bit 2  Reserved (0)           Bit 2   Data In Port Full (DF)

Bit 1  Reserved (0)           Bit 1   Reserved (Undefined)

Bit 0  Reserved (0)           Bit 0   Invalid H A Command (INVDCMD)

PORT ADDRESS = BASE + 1

WRITE: Command/Data Out        READ: Data In

Bit 7   Command/Data Out Bit 7      Bit 7  Data In  Bit 7

Bit 6           "        Bit 6      Bit 6      "    Bit 6

Bit 5           "        Bit 5      Bit 5      "    Bit 5

Bit 4           "        Bit 4      Bit 4      "    Bit 4

Bit 3           "        Bit 3      Bit 3      "    Bit 3

Bit 2           "        Bit 2      Bit 2      "    Bit 2

Bit 1           "        Bit 1      Bit 1      "    Bit 1

Bit 0           "        Bit 0      Bit 0      "    Bit 0

PORT ADDRESS = BASE + 2

WRITE: Reserved, do not write        READ: Interrupt Flags

Bit 7                          Bit 7    Any Interrupt

Bit 6                          Bit 6    Reserved

Bit 5                          Bit 5    Reserved

Bit 4                          Bit 4    Reserved

Bit 3                          Bit 3    SCSI Reset Detected (SCRD)

Bit 2                          Bit 2    HA Command Complete (HACC)

Bit 1                          Bit 1    MBO Empty (MBOA)

Bit 0                          Bit 0    MBI Full (MBIF)

4.2.1 Control and Status Port

Writing a one to the bits of the Control Port initiates certain

special host adapter operations.  There is no requirement to return

the bits to the zero state, since they are reset automatically after

the requested operation is initiated. Read operations to the Status

port address return host adapter status information.

4-2

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

Base+0 Port, Write: Host Adapter Control Port.

Bit 7   Hard Reset (HRST): The setting of the Hard Reset bit to

one forces the host adapter into a state identical to a

normal power-on state. Diagnostic functions are executed and

all status for ongoing SCSI operations is lost. A Reset

Condition is generated on the SCSI bus. While the reset is

being processed, the Self Testing in Progress bit (Host

Adapter Status Port bit 7) is set. When the reset is

complete, that bit is reset and the Mailbox Initialization

Required bit (Host Adapter Status Port bit 5) and the SCSI

Host Adapter Idle bit (Host Adapter Status Port bit 4) are

set, indicating that the AHA-1740/1744 mailbox structure

must be reinitialized and that no other operations are

active on the host adapter. See Section 4.3, describing the

overall reset structure.

Bit 6   Soft Reset (SRST): The Soft Reset bit clears all ongoing

SCSI and host adapter commands. All Command Control Blocks

are abandoned and all queued commands are abandoned. Mailbox

In and Mailbox Out entries must be cleared by the host. No

diagnostic functions are executed. No Reset Condition is

generated on the SCSI bus. The Mailbox Initialization

Required bit (Host Adapter Status Port bit 5) and the SCSI

Host Adapter Idle bit (Host Adapter Status Port bit 4) are

set when the reset processing is completed. This indicates

that the AHA-1740/1744 Mailbox structure must be

reinitialized and that no other operations are active on the

host adapter. See Section 4.3, describing the overall reset

structure.

Bit 5   Interrupt Reset (IRST): The setting of this bit clears

the interrupt port of all bits that have been set and resets

the interrupt line. The host adapter manages the interrupt

presentation to minimize the possibility of incorrectly

resetting an interrupt. MBOE and MBIF interrupts are

presented immediately unless an SCRD or HACC interrupt has

not yet been cleared. An SCRD or HACC will only be presented

after any interrupt bit has been cleared and DF is zero,

indicating an operation is fully completed. The prompt

resetting of MBOE and MBIF interrupts minimizes the chance

of a reset of one also resetting the other. Host programs

should, however, be aware that there is a small chance of

falsely resetting a new MBIF reset while clearing an MBOE

interrupt. This can be resolved by periodically scanning the

MBI entries when activity is expected on the host adapter or

by not enabling the MBOE interrupt.

Bit 4   SCSI Bus Reset (SCRST): The setting of this bit causes a

SCSI Bus Reset to be generated on the SCSI bus. The SCSI Bus

Reset is triggered at the time the SCRST bit is set to one

and raises the RST line on the SCSI Bus for the architected

25 microsecond period. The reset is managed as a SCSI Soft

Reset and will allow partially completed operations to

continue after the reset occurs. See Section 4.3, describing

the overall reset structure.

Bits 0-3 Reserved: Reserved bits must be set to zero to avoid compatibility

problems with future extensions of the Control Register.

4-3

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

Base+0 Port, Read: Host Adapter Status Port

Bit 7   Self Testing in progress (STST): This bit, when one,

indicates that the host adapter is performing

self-initialization and internal diagnostics. The bit is

asserted after a power-on or hard reset (Control Port Bit 7

HRST = 1). When diagnostic operation is complete, the STST

bit is set to zero and bit 5 or bit 6 is set to indicate the

successful or unsuccessful completion of the diagnostics. If

bit 7 remains on, it indicates that the initialization or

diagnostic could not be completed. The error condition can

be determined as described in Section 10. In most cases, bit

6 (DIAGF) will be set to indicate that an internal

diagnostic failure occurred.

Bit 6   Internal Diagnostic Failure (DIAGF): This bit, when one,

indicates that the self-testing process has completed and

that an error was detected. The host adapter must be reset

by setting the Hard Reset bit (bit 7 of the Control Port) to

clear the error. If the AHA-1740/1744 again detects an

error, troubleshooting procedures must be performed to

identify and correct the error condition. The diagnostic LED

will usually present a flash code that indicates the nature

of the failure. Section 10 describes the corrective

procedures.

Bit 5   Mailbox Initialization Required (INIT): This bit, when

one, indicates that the self-testing process has completed

successfully and that the AHA-1740/1744 is ready for mailbox

initialization to be performed. The base memory address of

the Mailbox area must be established by execution of the

Mailbox Initialization command. After execution of the

Mailbox Initialization command and any other desired

initialization operations, the AHA-1740/1744 is ready for

full operation.

Bit 4   SCSI Host Adapter Idle (IDLE): This bit, when one,

indicates that the host adapter is in the idle state. The

host adapter has no outstanding Adapter Commands or SCSI

commands. The host processor must wait for the idle state

before executing any Adapter Command except the Start SCSI

(02) and Enable Mailbox Out Interrupt (05) commands.

Bit 3   Command/Data Out Port Full (CDF): The host uses the CDF

bit to synchronize command and data transfers to the host

adapter. An Adapter Command byte or an outbound parameter

byte can be placed in the Command/Data Out Port when the

port is empty, indicated by the CDF bit being zero. When a

byte is placed in the Command/Data Out Port, the CDF bit is

set to one and remains one until the host adapter has

obtained and processed the byte. When the CDF bit returns to

zero, the next Command or parameter byte can be placed in

the port.

Bit 2   Data In Port Full (DF): The host uses the DF bit to

synchronize transfers of data from the host adapter to the

host. When the DF bit is set to one, the host adapter has

placed a byte in the Data In Port for the host to remove and

process. When the host performs a read to the Data In Port

address, the DF bit is set back to zero automatically and

not set to one again until a new data byte has been placed

in the Data In Port by the host adapter for the host.

4-4

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

Bit 1   Reserved: This bit is zero.

Bit 0   Invalid Host Adapter Command (INVDCMD): The Invalid Host

Adapter Command bit is set to one if an invalid command or

parameter byte was received in the Command/Data Out Port.

After sending a command byte or data byte, the host software

determines that the next byte is ready to send by waiting

for the CDF bit to be reset. If the command byte or

parameter byte is not valid, the command sequence will

instead be terminated by the host adapter. The host adapter

always terminates a command by raising the Host Adapter

Command Complete (HACC) interrupt. If the HACC interrupt is

set to one and the INVDCMD bit is not set, the command

terminated normally. If the INVDCMD bit is also set to one,

the command or parameter bytes were determined to be invalid

and the command terminated abnormally. The INVDCMD bit is

only valid from the time the HACC interrupt is set until the

HACC interrupt is reset. The bit's value is not predictable

until a new HACC interrupt is set for a new Adapter Command.

4.2.2 Command/Data Out and Data In Port

The second I/O port address is used by the host to write Adapter

Command bytes and accompanying parameter data bytes to the host

adapter. It is also used by the host adapter to send parameters back

to the host to be read. The Command/Data Out Port is used by the host

to send host adapter initialization and management commands and

parameters that cannot be sent by the standard Mailbox protocol.

Information requested by the Adapter Commands placed in the

Command/Data Out Port is returned through the Data In Port. The host

should understand the format and number of bytes to be transmitted for

each command so that extra invalid bytes are not passed across the

interface. Bytes in addition to those required by the particular

command are likely to be interpreted as invalid, although they may

instead cause the execution of valid commands that were not supposed

to be performed.

The host should only write to the Command/Data Out Port when CDF is

zero. This allows time for the host adapter to process a previously

written command or parameter byte. CDF is automatically set to one

when the host writes to the port and is reset to zero after the host

adapter removes the byte from the port.

If an Adapter Command needs additional data bytes, the host waits

until CDF is zero before writing the additional bytes to the

Command/Data Out Port. Just as in the command transfer case, each

parameter byte written will set the CDF. The host can write additional

data bytes only after CDF is again zero. The HACC interrupt will

indicate when the command has terminated, normally or abnormally. If

INVDCMD is also set, the host adapter found either the command or data

bytes to be invalid and terminated abnormally. The use of CDF as a

handshaking bit is required to prevent the transfer of invalid data.

If an Adapter Command requires data transfer from the host adapter,

the host adapter will place the data bytes in the Data In Port and set

the DF bit (Status Port bit 2) to indicate that the requested

parameter is ready for the host to read. When the host reads the Data

port, DF is automatically reset. The host should wait until DF is

again set before attempting to transfer the next parameter byte. The

use of the DF bit to control the handshaking process is required to

prevent the transfer of invalid data. After the last data byte has

been transferred, the HACC interrupt bit will be set indicating

command completion. If the Adapter Command was invalid, the HACC

interrupt will occur before all data bytes have been transmitted and

the INVDCMD bit will be set.

4.5

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

4.2.3 Interrupt Flag Port

The Interrupt Flag Port contains bits that indicate the reason that an

interrupt was provided to the host from the host adapter. The host

adapter uses the interrupt to notify the host that the host adapter is

ready for immediate service from the host. The Interrupt Flag Port is

a read-only port. When an interrupt bit is set by the host adapter to

indicate that the host should respond, the Any Interrupt bit and the

interrupt line are both also set. When the host begins to examine the

returned registers and mailboxes to determine the cause of the

interrupt and to perform the operations needed to service the

interrupt, the host will first read the Interrupt Flag Port to record

which interrupts must be serviced. The host will then clear the

interrupts by setting the IRST bit (Host Adapter Control Port bit 5).

The host adapter presents MBOA and MBIF interrupts immediately unless

there is already an SCRD or HACC interrupt present. If the SCRD or

HACC is present, the MBOA and/or MBIF interrupt will be posted after

the SCRD or HACC interrupt is cleared. An SCRD or HACC interrupt will

only be presented if the Any Interrupt signal is zero and the DF

signal is zero, indicating the completion of all pending interrupt

presentation. It is recommended that the MBOA interrupt be enabled

only when required by the host. This prevents the possible

presentation and resetting of MBIF interrupts before they are

processed. Other reset operations will also reset the Interrupt Flag

Port and the interrupt line, including the Hard Reset bit (HRST), the

Soft Reset bit (SRST), and the power-on reset issued by the motherboard.

Base+2 Port, Read only: Interrupt Flag Port

Bit 7   Any Interrupt: This bit, when one, indicates that the

interrupt to the host has been established. The interrupting

condition is identified in bits 0 to 3.

Bit 6   Reserved: Returned as zero.

Bit 5   Reserved: Returned as zero.

Bit 4   Reserved: Returned as zero.

Bit 3   SCSI Reset Detected (SCRD): This bit, when one, indicates

that a SCSI Reset has been received on the SCSI bus. The Any

Interrupt bit and the AT interrupt signal will also be set.

The host adapter supports SCSI Soft Reset (see Section 4.3).

Any ongoing target or initiator activities will continue

normally after first clearing the SCSI bus. In some rare

cases, host intervention will be required to restart a SCSI

command that was aborted by the reset operation. The host

can convert the SCSI Soft Reset to a SCSI Hard Reset by

setting the Control Register Soft Reset (Bit 6) to one,

clearing all ongoing operations in the host adapter. In this

case, the host must recognize that any operations not yet

completed will never be completed and must perform

appropriate error recovery operations. See Section 4.3,

describing the overall reset protocol. The SCRD bit is not

set for host-initiated SCSI Reset conditions caused by the

setting of the HRST bit or the SCRST bit, since the host is

already aware of the actions it has requested. If the Any

Interrupt signal or DF signal is present, the SCRD interrupt

will not be presented until the interrupts already present

are cleared.

4-6

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

Bit 2   Host Adapter Command Complete (HACC): This bit, when one,

indicates that an adapter command has been completed,

normally or abnormally. The Any Interrupt bit and the AT

interrupt signal will also be set. If the command was

completed normally, only the HACC bit will be on. If the

command was completed abnormally or was aborted before it

was completed, the HACC bit will be one and the Invalid

Command Bit (Status Register bit 5) will also be one. During

parameter transfers to or from the host adapter, the HACC

bit should be examined to verify that the command is still

being processed and has not been ended abnormally. If the

Any Interrupt signal or DF signal is set, the HACC interrupt

will not be presented until the interrupts already presented

are cleared.

Bit 1   Mailbox Out Available (MBOA): This bit, when one,

indicates that an outbound Mailbox entry is now available

for use by the host. The Any Interrupt bit and the AT

interrupt signal will also be set. Most operating systems

will choose to leave this interrupt disabled to avoid the

generation of extra interrupts. The host adapter will

normally empty Mailbox Out entries to its local RAM so

rapidly that round-robin filling of the Mailbox Out entries

will assure that a Mailbox Out entry will already be empty

by the time the host is ready to fill it again.

If the host finds that all Mailbox Out entries are full,

it can enable the Mailbox Out Available interrupt by

executing an Enable Mailbox Out Interrupt command through

the I/O Command Port. The Enable Mailbox Out Interrupt

command is one of the two commands that can be executed

without waiting for the IDLE state of the host adapter. As

soon as any Mailbox Out entry is cleared by the host

adapter, an MBOA interrupt will be generated to indicate to

the host that an MBO entry is available. An MBOA interrupt

is generated after that each time a Mailbox Out entry is

cleared by the host adapter until an Enable Mailbox Out

Interrupt command is executed to force the reporting of MBOA

interrupts to be disabled. If the SCRD or HACC interrupts

are present, the MBOA interrupt is not presented until they

are cleared. At all other times, MBOA is presented immediately.

Bit 0   Mailbox In Full (MBIF): This bit, when one, indicates

that an entry has been placed by the host adapter in the

Mailbox In. The interrupt should be reset as soon as

possible so that any subsequent interrupts can be detected.

The host adapter may return information in other Mailbox In

entries, so the host should check the next entry to

determine if more than one set of information has been

provided. The MBI entries are filled in round-robin order,

so the host should simply check the next MBI entry after the

last one that was found when a new MBIF interrupt occurs.

4-7

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

If an MBIF interrupt is set and other Mailbox In entries

are made before the interrupt is cleared, then all the

entries can be scanned as found. The new MBIF interrupt will

be presented if the SCRD and HACC interrupts are cleared. A

new MBIF interrupt will be presented regardless of the state

of the MBOA interrupt bit. It is important to clear and

record each interrupt as soon as possible to avoid the

possible accidental resetting of a valid interrupt. In

addition, it is desirable to enable the MBOA interrupt as

rarely as possible. The host system software must be ready

to scan for MBI entries even if no MBIF interrupt occurred.

4.3 RESET FUNCTIONS

The reset functions provided by the AHA-1740/1744 are extensive to

allow the fullest flexibility and architectural consistency, both with

SCSI and with the Industry Standard Architecture.

4.3.1 Hard Reset Operations

Resets may be generated by the hardware through two mechanisms. The

RESET signal from the ISA socket is generated by the system board to

reset or initialize all installed adapters upon power-on or during a

low line voltage condition. The system can also activate this signal

under host program control. In addition to the RESET signal, the

setting of the HRST bit (bit 7 of the Host Adapter Control Port) will

force a hardware reset to the AHA-1740/1744. Regardless of the source,

a Hard Reset forces the following actions on the AHA-1740/1744.

�   All internal registers of the AHA-1740/1744 are returned to their

reset condition.

�   The host adapter's microprocessor returns all internal information

and parameters to their initial state.

�   The host adapter performs all required internal diagnostics.

�   A standard SCSI Reset condition is generated to all other attached

SCSI devices.

During the Hard Reset process, the AHA-1740/1744 will indicate to the

attached host that self-testing is in process by raising the STST bit

in the Host Adapter Status Port. After the Hard Reset process is

complete, the AHA-1740/1744 will indicate that initialization

parameters are required from the host by raising the INIT bit in the

Host Adapter Status Port.

4.3.2 SCSI Reset Operations

The SCSI Reset condition is defined in the SCSI Standard, X3.131-1986,

Section 5.2.2. A SCSI Reset condition may be forced by any SCSI device

on the bus. The condition is forced by the assertion of the SCSI Reset

signal.

The AHA-1740/1744 has four mechanisms which may force a SCSI Reset

condition. The SCSI Reset condition may be invoked from the host

software if the software sets the SCRST bit (Bit 4 of the Host Adapter

Control Port). In this case, the normal SCSI Reset operations will be

performed by the host adapter and the SCSI Reset signal will be

asserted on the SCSI bus. In addition, the SCRD bit (Interrupt Flag

Port bit 3) will not be set, since the host itself caused the reset.

4-8

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

The SCSI Reset is invoked if a hard reset is generated, either by the

system board RESET signal or by the setting of the HRST bit. This

reset is described in Section 4.3.1.

The SCSI Reset condition may be invoked by the AHA-1740/1744 as part

of the recovery mechanism for a bus phase error. Bus phase errors may

include detection of an invalid information transfer phase or

detection of an impossible phase sequence (Command Phase after a Data

Phase in the same command). In this case, the normal SCSI Reset

operations will be performed and the SCSI Reset signal will be

asserted on the SCSI bus. In addition, the SCRD bit (Interrupt Flag

Port bit 3) will be set to indicate to the host computer that a SCSI

Reset condition occurred. The setting of the SCRD bit also causes the

Any Interrupt bit (Interrupt Flag Port bit 7) and the appropriate

interrupt signal to be presented.

The SCSI Reset condition may be invoked by another SCSI device

attached to the AHA-1740/1744 as part of the other device's recovery

mechanism or initialization procedure. In this case, the normal SCSI

Reset operations will be performed. In addition, the SCRD bit

(Interrupt Flag Port bit 3) will be set to indicate to the host

computer that a SCSI Reset condition occurred. The setting of the SCRD

bit also causes the Any Interrupt bit (Interrupt Flag Port bit 7) and

the appropriate ISA interrupt signal to be presented.

4.3.3 SCSI "Soft" Reset Option

The SCSI standard indicates two optional methods of handling the

normal SCSI Reset operations. The AHA-1740/1744 implements the "Soft"

Reset Option, described in Section 5.2.2.2 of the SCSI specification.

The "Soft" Reset Option is designed to allow a SCSI Reset signal to

correctly clear the SCSI bus, but to allow ongoing system operations

to continue without any major interruptions. When the SCSI Reset

occurs, any activity on the SCSI bus is immediately halted and all bus

lines are cleared from the bus. After the Reset condition ends, any

operations in progress are again allowed to start up in the normal

manner. No status or pointer information is destroyed. All

disconnected commands are allowed to reselect and continue in the

normal manner. This "Soft" Reset function allows a multiple initiator

system to use reset to clear certain types of bus failures without

damaging ongoing tasks from any initiator.

The SCSI "Soft" Reset option is useful in multitasking systems that

cannot tolerate the overhead of a complex reconfiguration and

reinitialization after a normal reset operation. For the SCSI "Soft"

Reset option to operate correctly, all SCSI devices that communicate

on the SCSI bus must support the "Soft" Reset option. If any SCSI

devices support the "Hard" Reset option, it is likely that operations

will be terminated without warning and the system will have to time

out and monitor the requirement to restart some activities.

4.3.4 SCSI "Hard" Reset Option

The "Hard" Reset Option is designed to restore all attached SCSI

devices, including both hosts and peripheral devices, to their

power-on reset state. All system activities that have not been

recorded on a nonvolatile memory device or through another SCSI path

are completely lost and must be restarted. The system must be

completely reinitialized. For certain types of systems that frequently

do back-up or check point their transactions, that reinitialize

quickly and easily, or that infrequently do resets, the "Hard" Reset

Option is appropriate. The AHA-1740/1744 responds to a SCSI Reset

Condition by executing only the "Soft" Reset Option, but it notifies

4-9

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

the host whenever a SCSI Reset condition has been established by

causing an interrupt to the host. The host then has the option of

converting the "Soft" Reset to a "Hard" Reset by forcing the host

adapter to clear all the ongoing operations and return to its initial

state. The host requests this by raising the SRST bit (bit 6 of the

Host Adapter Control Port). The host must raise the SRST bit within

300 microseconds to disable the restarting of operations according to

the rules of "Soft" Reset. The raising of the SRST bit causes the host

adapter to abandon all CCBs and prepare itself to begin accepting new

instructions from the host. No secondary SCSI Reset signals are

activated. The Mailbox initialization and all normal SCSI initial

conditions are reset by the SRST bit, so that reinitialization is

required to restart the system. Of course the system still has the

right at any time that the IDLE bit (bit 4 of the Host Adapter Status

Port) is on to execute any of the Adapter Commands and modify the

Mailbox Address or the SCSI initial conditions.

If the host requires that the SCSI be reset according to the SCSI

"Hard" Reset option, the host raises the HRST bit. The host adapter

will then set a SCSI Reset Condition on the SCSI Bus and clear all its

CCB and status information, thus performing a SCSI "Hard" Reset with a

single load to the Host Adapter Control Port. Reinitialization will be

required.

4.4 EISA EXPANSION IDENTIFIERS

"Expansion Board Identifiers" is a term used in the EISA specification

for the parameters used to integrate an add-in card into an EISA host

system. These identifiers are used by the system ROM during

configuration to recognize the information provided in the

configuration file. The configuration file is provided as a part of

the system software or in the separate ASW-C174 Configuration software

package.

This automatic configuration eliminates the need for switches or

jumpers and replaces them with programmable registers. It also

resolves conflicts between system resources such as interrupt levels

and DMA channels when they are assigned. Refer to the EISA

Specification, Section 2.6 and Section 3.3 for more information.

There are five groups of addressable hardware used to interface the

host adapter with the host system across the EISA bus;

1. Board ID Registers

2. Configuration registers

3. BIOS EPROM

4. Standard Mode (Group 1) I/O ports (used in Standard Mode)

5. Enhanced Mode (Group 2) I/O ports (used in Enhanced Mode)

Only the Board ID and Configuration registers are accessible after a

power on reset (RESDRV). The I/O port Standard Mode (Group 1)

registers are accessible when the CDEN bit on the board is set. The

BIOS EPROM is accessible when both the CDEN and BIOSEN bits are set.

The board IDs are required for EISA board configuration. The ID is

used by the system ROM at power up initialization to locate the

adapter and determine the slot in which it is installed. After the

RESDRV signal indicates or power on, HID0 is set to 7X hex to indicate

not ready. The IDs are loaded by the local processor. The interrupt

level, I/O port base address, BIOS base address, and SCSI

initialization parameters are programmed in the Configuration

registers. All of these registers may be written or read at any time.

4-10

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

4.4.1 Expansion Board ID Registers

The expansion board identifier registers are always available for

access on the bus by the host system. They are used to transfer basic

information on the identity of the board and its revision level from

the AHA-1740/1744 to the host.

4.4.1.1 Host ID 0 (HID0, zC80, R)

HID0 is the first byte of the Expansion board ID. It is written by the

local processor and read by the system processor. HID0 is initialized

to 7Xh on RESDRV. HARDST or SOFTRST has no effect.

------------------------------------------------------------------------------

Bit     Definition

------------------------------------------------------------------------------

7      Reserved (0).

6      First character of manufacturer's ID ( msb ). (1)

5      First character of manufacturer's ID. (1)

4      First character of manufacturer's ID. (1)

3      First character of manufacturer's ID.

2      First character of manufacturer's ID ( lsb ).

1      Second character of manufacturer's ID ( msb ).

0      Second character of manufacturer's ID.

------------------------------------------------------------------------------

4.4.1.2 Host ID 1 (HID1, zC81, R)

HID1 is the second byte of the Expansion board ID. It is written by

the local processor and read by the system processor. RESDRV, HARDST

or SOFTRST has no effect.

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7-5      Second character of manufacturer's ID.

4-0      Third character of manufacturer's ID.

------------------------------------------------------------------------------

4-11

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

4.4.1.3 Host ID 2 (HID2, zC82, R)

HID2 contains the adapter's product number. It is written by the local

processor and read by the system processor. RESDRV, HARDST or SOFTRST

has no effect.

4.4.1.4 Host ID 3 (HID3, zC83, R)

HID3 contains the revision level of the firmware resident on the

board. This is used to identify the capabilities of the board to the

manager and other software. The revision level may be changed during

the firmware download process described in Section 3.4. It is written

by the local processor and read by the system processor. RESDRV,

HARDST or SOFTRST has no effect.

4.4.2 Board Configuration Registers

These registers are used by the host process to define parameters on

the board necessary for normal operation. This function was normally

provided by jumpers or onboard DIP switches in ISA boards. These

registers are accessible at any time.

4.4.2.1 Expansion Board Control Register (EBCTRL, zC84, W/R)

EBCTRL is used by the system software to control the host adapter

hardware. Bit 2 is write only, 1 is read only and bit 0 is

write/read.

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7-3     Unused. (0)

2      ERRST _ When this bit is set, HAERR and CDEN are reset to 0. (0)

1      HAERR _ When this bit is one, the host adapter has detected a

serious error. Further operation will produce unreliable results.

This bit is set by the local processor. IOCHK* will also be

asserted on the bus. (0)

0      CDEN _ When this bit is cleared, the host adapter is held

disabled. During this time only Expansion Board Registers can be

accessed. (0)

------------------------------------------------------------------------------

4.4.2.2 I/O Port Address (PORTADDR, zCC0, W/R)

PORTADDR selects the base address for the I/O port interface. It is

compared with the system address every cycle. If a match occurs, data

will either be driven or sampled on the bus. PORTADDR is initialized

with a value provided in the configuration file during configuration.

4-12

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

Bit     Definition

7      ENHANCED INTERFACE _ When set, the host adapter will power up

in Enhanced mode. It will use the EISA I/O address space for

command execution. When cleared, the host adapter will power up

in Standard Mode. When in this mode, the host adapter will use

ISA space for command execution.

6      CONFIGURE _ When set, the host adapter is instructed to

program the local EEPROM with the data to be defined in the next CCB.

5-3     Reserved

2-0     ADR 2-0. Port address selection bits. These bits select the base

address of the host adapter I/O register and should be written

according to the chart below.

I/O Port Address Table

----------------------------

ADR bits

2 1 0           Port Address

----------------------------

0 0 0           Disable

0 0 1           N/A

0 1 0           130

0 1 1           134

1 0 0           230

1 0 1           234

1 1 0           330

1 1 1           334

------------------------------------------------------------------------------

4.4.2.3 BIOS Control

The onboard BIOS EPROM is available as the third set of addressable

information. It is a system processor read access only and is

independent of the local processor. After RESDRV system processor

access is not allowed. CDEN and BIOSEN must be set to 1 (see Section 4.4.2.1).

The BIOS is provided to allow the user to use the host adapter in lieu

of, or in addition to a standard hard disk controller. The BIOS has

at its disposal 2K of 8-bit wide system RAM which overlays the last 2K

of the BIOS. This RAM is writable when WRTPRT (BIOSADDR bit 7) is

zero, and may only be read when RAMEN (BIOSADDR bit 6) is set.

The BIOS will take on the personality of the firmware that is

activated. If the Compatible mode is selected at configuration time,

the Compatible mode BIOS will run, and conversely for the AHA-1740

mode, the AHA-1740 BIOS will run.

4-13

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

BIOS Address Register (BIOSADDR, zCC1, W/R)

The BIOSADDR register maps the BIOS to one of 12 locations between

C0000 and EFFFF hex at 16K boundaries. In addition, the BIOS has the

last 2K bytes of address space as overlaid RAM. The RAM may be

written to when enabled by setting the RAMEN bit and clearing the

WRTPRT bit. The RAM may be read by setting the RAMEN bit. On

power-up, the RAM is disabled.

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7      WRTPRT _ When set, write protects the RAM.

6      BIOSEN _ Enables the BIOS on this board to respond to system

reads in the selected address space.

5      RAMEN _ When set, enables the BIOS RAM overlay. When cleared,

disables the RAM overlay writing or reading.

4      Reserved

3-0     BIOSSEL _ These bits select the starting address of the system

BIOS according to the following table.

BIOS Address Table

----------------------------------

BIOSSEL

3 2 1 0         BIOS Address (hex)

----------------------------------

0 0 0 0         C0000

0 0 0 1         C4000

0 0 1 0         C8000

0 0 1 1         CC000

0 1 0 0         D0000

0 1 0 1         D4000

0 1 1 0         D8000

0 1 1 1         DC000

1 0 0 0         E0000

1 0 0 1         E4000

1 0 1 0         E8000

1 0 1 1         EC000

1 1 0 0 - 1111  Not used

------------------------------------------------------------------------------

4-14

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

4.4.2.4 Interrupt Definition (INTDEF, zCC2, W/R)

An interrupt channel may be selected from one of several on the EISA

bus. The interrupt may be programmed as high true or low true. When

high true, the interrupt is a two state signal and acts like an edge

triggered interrupt, and may not be shared with another board. If the

interrupt is on and disabled, it will go to the low state, and back

again to the high state when enabled again.

When programmed as low true, interrupt is an open collector signal and

may be shared with another board that also has an open collector

interrupt. If the interrupt is on and disabled, it will turn off, and

then to an on low state when enabled again.

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7-5     Reserved

4       INTEN _ When set, enables interrupts to occur on the EISA bus.

3       INTHIGH _ When set, the interrupt is programmed to high true state.

2-0     INTSEL _ These bits select the interrupt channel to be driven on the

EISA bus according to the following table.

Interrupt Channel Table

---------------------------

INTSEL bits

2 1 0     Interrupt Channel

---------------------------

0 0 0           9

0 0 1           10

0 1 0           11

0 1 1           12

1 0 0           Not used

1 0 1           14

1 1 0           15

1 1 1           Not used

---------------------------

4-15

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

4.4.2.5 SCSI definition (SCSIDEF, zCC3, W/R)

Programming this byte sets up basic SCSI parameters.

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7-5     Reserved (0)

4      RSTPWR _ When set, the host adapter will generate a SCSI Reset

on Power up and on a hard reset. When cleared, the host adapter

will not generate a SCSI Reset on Power up or on a hard reset to

the board.

3-0     HSCSIID3-0 _ These bits define the SCSI ID of the host

adapter. Bit 3 is the MSB.

SCSI ID Table

-----------------------

HSCSIID

3 2 1 0   SCSI Address

----------------------

0 0 0 0         0

0 0 0 1         1

0 0 1 0         2

0 0 1 1         3

0 1 0 0         4

0 1 0 1         5

0 1 1 0         6

0 1 1 1         7

1000-1111       Reserved

------------------------------------------------------------------------------

4.4.2.6 Bus Definition (BUSDEF, zCC4, W/R)

This register allows the configuration of certain EISA bus features

supported by the AHA-1740/1744.

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7-4  Reserved

3-2  DMA Channel for Standard Mode

-----------------------------

Bit 3      Bit 2  DMA Channel

Returned

-----------------------------

0       0             0

0       1             5

1       0             6

1       1             7

4-16

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

1-0     BUSON _ These bits are written in order to set the time that

the AHA-1740 stays on the bus when preempted by another device.

------------------------------

Bit 1   Bit 2      �s before

ending transfer

------------------------------

0       0          0

0       1          4

1       0          8

1       1      Not used

4.4.2.7 Reserved Registers

The following registers are reserved for future use.

Reserved 0 (RESV0, zCC5, W/R)

Reserved 1 (RESV1, zCC6, W/R)

Reserved 2 (RESV2, zCC7, W/R)

4.4.3 I/O Port Register Standard Mode (Group 1)

The I/O Control Standard Mode (Group 1) registers form the primary

communication channel between the system and the adapter for

Compatible Mode operation.  When power is initially applied, these

registers will not respond to system accesses. EBCTRL, bit 0, must be

set before access is allowed.

These registers are not writable or readable until the CDEN bit in the

control register is set. Refer to Section 4.4.2.1 The eight-bit port

registers form the primary communications channel and are addressed

with reference to a base address, plus offset. There are three I/O

ports which reside in ISA I/O address space. The base address is

selected by writing to the PORTADDR register.

Control and status _ base +0

Command and data _ base+1

Interrupt information _ base+2

The Control/Status port controls the host adapter. The bits can

initiate hardware or firmware operation directly. The Control port

allows the system to control the host adapter hardware, such as

executing a hardware reset. The Status port provides status about the

state of the host adapter firmware and hardware.

The host adapter supports commands sent directly to I/O address space

through CMD/DAT. LAVAIL indicates that the write port is full and

should be sampled for zero before writing to CMD/DAT.

4-17

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

HAVAIL indicates that the read port is full and should be sampled for one

before reading from CMD/DAT. The low level commands are executed by

placing the appropriate command byte followed by any additional

parameters into CMD/DAT. Parameters are then transferred from the host

adapter through port I/O or into the system through Bus Master

transfers to provide the information necessary to complete the command.

In response to a reset, if the host adapter passes diagnostics, then

the local processor will set POCIP (LSTAT bit 7) to zero and INITRQD

(LSTAT bit 5) to one. These bits are available in the system STATUS

register and this state means that mailbox initialization may proceed.

Additional parameters are passed via the CMDDAT register.

If the host adapter fails diagnostics, then the local processor will

set POCIP (LSTAT bit 7) to zero and POCFAIL (LSTAT bit 6) to one.

These bits are available in the system STATUS register and this state

means that a hardware error has occurred on the board.

4.4.3.1 Control Port (CNTRL, Base address + 0, W)

The Control Port is written by the system at the Base address. It is

used to control the hardware directly.

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7   HARDRST _ When set, the host adapter will be put in a power on

state. The firmware will be loaded from EEPROM to RAM and power

on diagnostics will be run. This bit is self-clearing and need

not be reset by the system. (0)

6   SOFRST _ When set, the soft reset interrupt will be generated

and presented to the local processor. POC diagnostics will not

be run, but the host adapter hardware will be initialized and all

commands will be cleared from the internal queue. This bit is

cleared by a local processor write to CLRINT. (0)

5   CLRINT _ When set, the EISA interrupt line IRQx and the

Interrupt Status register are reset. This bit is self-clearing

and need not be reset by the system. (0)

4   SCSIRST _ When set, the SCSI Reset interrupt will be generated

and presented to the local processor. This bit is cleared by a

local processor write to CLRINT. (0)

3-0     Reserved. (0)

------------------------------------------------------------------------------

4.4.3.2 Status Port (STATUS, Base address + 0, R)

Data read on the Status Port does not reflect data that was written to

Control port; although, they share the same address space. The Status

Port is written by the local processor to present host adapter

information. Bits 3 and 2 are set by hardware, and bit 7 is set by

hardware on power up only.

4-18

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7   POCIP _ Power On Confidence in Progress. When this bit is one,

it indicates that the host adapter is performing internal

diagnostics. It is cleared by the firmware when POC diagnostics

are finished. This bit is set by the local processor when it

writes a one to bit 7 of HASTAT. (1)

6   POCFAIL _ Power on Diagnostics Failed. When this bit is one,

it indicates that the power on diagnostics failed. This bit is

valid when bit 7 is zero. This bit is set by the local processor

when it writes a one to bit 6 of HASTAT.  (0)

5   INITRQD _ Initialization Required. When this bit is one, it

indicates that the host adapter has successfully completed

self-diagnostics and initialization data is required.

Initialization data will be passed via the Command/Data, Base + 1

register. This bit is set by the local processor when it writes

a one to bit 5 of HASTAT.  (0)

4   HAIDLE _ Host Adapter Idle. When this bit is one, it indicates

that the host adapter is in the idle state. This bit is set by

the local processor when it writes a one to bit 4 of HASTAT. (0)

3   LAVAIL _ Command/Data Available to local processor. When

this bit is one, the system has loaded CMD/DAT (Base + 1) with a

command or parameter.  This bit is polled by the local processor

to determine if a byte is ready. It is cleared when the local

processor reads the LCMDDAT register.  (0)

2   HAVAIL _ Information is Available to the system. When this bit

is one, the local processor has loaded CMD/DAT (Base + 1) with

data. It is cleared when the system reads CMD/DAT.  This bit

is also read by the local processor.  (0)

1   Reserved. (0)

0   INVCMD _Invalid Host Command. When this bit is one it

indicates that the command or parameter received from the Local

Command/Data register is illegal. This bit is set by the local

processor when it writes a one to bit of LSTAT. (0)

4.4.3.3 Command/Data (CMD/DAT, Base address + 1, W/R)

This register gives the system a path by which it can communicate with

the host adapter. Commands and parameters are sent by the system and

status is received through this port. HAVAIL and LAVAIL provide the

handshake for accessing this register.

4.4.3.4 System Interrupt (INTRPT, Base + 2, R)

The interrupt register provides further status regarding why an

interrupt was generated. The interrupt status is written by the local

processor via Local Interrupt Register (LINTRG). It is reset by the

system by setting CLRINT (bit 5 in CNTRL) to one. Bit 7 must be set

along with the interrupt status bit for the interrupt to be generated.

When bit 7 is set, IRQx is asserted on the EISA bus.

4-19

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7   ANYINT _ When this bit is one, a host interrupt is generated.

It is set when local register LINTRG bit 7 is set. (0)

6-4  Reserved (0).

3   SCSIRD _ SCSI Reset Detected. When this bit is one, it

indicates that the host adapter received a reset from the SCSI

BUS. Set when local register LINTRG bit 3 is set. (0)

2   HACC _ Host Adapter Command Complete. When this bit is one, it

indicates that the host adapter has completed the command that

was sent through the CMD/DAT register. Set when local register

LINTRG bit 2 is set. (0)

1   MBOAVAIL _ Mailbox Out Available. When this bit is one, it

indicates that at least one mailbox out area is available for the

host to use. Set when local register LINTRG bit 1 is set. (0)

0   MBISTRD _ Mailbox In Stored. When this bit is one, it

indicates that the host adapter has completed a command from the

Mailbox Out area and has placed the completion status to the

Mailbox In area. (0)

------------------------------------------------------------------------------

4.4.4 I/O Port Register Enhanced Mode (Group 2)

These registers are used for operating in Enhanced Mode and may be

used at any time. They consist of mailbox out, mailbox in, control,

status, attention, interrupt 1 status, and interrupt 2 status. The

mailbox out and in are used for general data transfer and represent

either an address or Immediate command parameters. Control, status,

attention and the interrupt registers are used for general operation.

The host should start operation by testing for the Mailbox Out Empty

status bit to be set and the Busy bit to be cleared to see if the

Mailbox Out registers are available for a CCB address or Immediate

command. If Mailbox Out Empty is set and Busy is cleared, the host

must write to the address which will clear the Mailbox Out Empty

status bit and write the appropriate code into the Attention register.

Writing to the Attention register will set the Busy bit, and will set

the local processor interrupt. The local processor will then read the

Attention register in response to the interrupt which will clear the

Busy bit. The OP Code in the Attention register must indicate that a

CCB address is present in the mailbox out. The local processor will

then read the mailbox out address and set the Mailbox Out Empty bit.

The host processor may then load another address into the mailbox out.

When the adapter has completed the command, it will sample the EISA

interrupt status line. If cleared, then the mailbox in will be loaded

with the CCB pointer of the command which has just finished. The

adapter will then load the appropriate interrupt status code which

will set the EISA interrupt.

4-20

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

The host responds to the interrupt by reading the interrupt status 1

register to determine if the command completed with or without error,

and reads the Mailbox In register to get the pointer of the command

which has just finished. The host then clears the interrupt by

writing to G2CNTRL register with bit 6 set. When the EISA interrupt

is cleared, the EOI local interrupt is generated which tells the local

processor that the Mailbox In is free and another CCB may be posted.

When Immediate commands are to be executed, the OP Code of the command

is written to the LSB of the Mailbox Out register. Additional data

will be placed in the rest of the Mailbox Out registers in order of

incrementing address. The host will set the Host Ready bit and write

the Attention register with the Immediate command code. The adapter

reads the Mailbox Out registers and determines the Immediate command

and the amount of data to follow, and then clears the Host Ready bit.

If more data is needed, the host monitors the Host Ready bit and when

it is cleared, loads the next group of data into the Mailbox Out. The

host then sets the Host Ready bit. The adapter monitors the Host

Ready bit, reads the data from the Mailbox Out registers and clears

the Host Ready bit. This process repeats until all data is received.

The adapter then executes the command and posts status in the Mailbox

In registers. The adapter clears the Host Ready bit, loads the

appropriate status in the Interrupt 1 Status register and interrupts

the host. The host reads the interrupt status and reads the

information from the Mailbox In registers. If more than four bytes

are to be returned, the host sets the Host Ready bit and the adapter

loads the next data group in the Mailbox In registers. The adapter

clears the Host Ready bit, and the host may then read the mailbox in

data. This process repeats until the transfer is complete.

In response to a reset, if the host adapter passes diagnostics, then a

value of 00H will be loaded in MBOXIN0. ATTN will also be read to

clear the Busy bit. If the host adapter fails diagnostics, then the

local processor will load the value appropriate for the failure in

MBOXIN0, according to the following list, and read ATTN to clear the

busy bit.

Reset codes

00 No Error

01 �P ROM Test failure

02 RAM test failure

03 Power Protection device error

04 �P internal peripheral test failure

05 Buffer control chip failure

06   Reserved

08   SCSI interface chip failure

Soft reset status

07   Hardware failure

4.4.4.1 Mailbox Registers

Mailbox out byte 0 (MBOXOUT0, zCD0, W/R)

This is the least significant byte of a 32-bit address pointer to a

CCB or the OP Code of an immediate command.

Mailbox out byte 1 (MBOXOUT1, zCD1, W/R)

4-21

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

This is the least middle significant byte of a 32-bit address pointer

to a CCB or additional data for an immediate command.

Mailbox out byte 2 (MBOXOUT2, zCD2, W/R)

This is the most middle significant byte of a 32-bit address pointer

to a CCB or additional data for an immediate command.

Mailbox out byte 3 (MBOXOUT3, zCD3, W/R)

This is the most significant byte of a 32-bit address pointer to a CCB

or additional data for an immediate command.

Mailbox in 0 (MBOXIN0, zCD8, R)

This is the least significant byte of a 32-bit address pointer to a

CCB or additional data for an immediate command.

Mailbox in 1 (MBOXIN1, zCD9, R)

This is the least middle significant byte of a 32-bit address pointer

to a CCB or additional data for an immediate command.

Mailbox in 2 (MBOXIN2, zCDA, R)

This is the most middle significant byte of a 32-bit address pointer

to a CCB or additional data for an immediate command.

Mailbox in 3 (MBOXIN3, zCDB, R)

This is the most significant byte of a 32-bit address pointer to a CCB

or additional data for an immediate command.

4.4.4.2 Attention (ATTN, zCD4, W/R)

------------------------------------------------------------------------------

Bit   Definition

------------------------------------------------------------------------------

7-4  OP Code - Operation code. Defines the contents and usage of

the mailbox out registers. All codes not named will be ignored.

1 = Immediate command

3 = Reserved

4 = Start CCB

5 = Abort CCB

E = Reserved

F = Reserved

3-0  Target ID - Contains the SCSI ID of the device or host adapter for which

this command pertains.

------------------------------------------------------------------------------

4-22

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

4.4.4.3 Enhanced Mode (Group 2) Control (G2CNTRL, zCD5, W/R)

------------------------------------------------------------------------------

Bit  Definition

------------------------------------------------------------------------------

7   Hard Reset - Resets hardware. When set, holds hardware reset

and sets the Busy bit in the status regsiter. This bit should be

set for a minimum of 10 microseconds. When cleared, the reset

line is released.

6   Clear EISA interrupt - When set, this bit clears a pending

EISA interrupt. At the same time, it sets the EOI interrupt to

the local processor, to indicate the end of interrupt processing.

It is self-clearing.

5   Set Host Ready - This bit sets the Host Ready status bit in

G2STAT2 register.

4-0  Unused

------------------------------------------------------------------------------

4.4.4.4 Enhanced Mode (Group 2) Interrupt Status (G2INTST, zCD6, R)

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7-4  Interrupt Status - 4 bits

1 = CCB completed with success

5 = CCB completed with success after retry

7 = Adapter hardware failure

A = Immediate command completed with success

C = CCB completed with error

D = Asynchronous Event Notification

E = Immediate command completed with error

3-0      Target ID _ SCSI ID of command

------------------------------------------------------------------------------

4.4.4.5 Enhanced Mode (Group 2) Status (G2STAT, zCD7, R)

------------------------------------------------------------------------------

Bit    Definition

------------------------------------------------------------------------------

7-3  Reserved

2   Mailbox out empty _ Set to one by firmware when it is done

with the mailbox out. Cleared by software writing to any

mailbox out location.

1   Interrupt pending _ Reflects the state of the EISA interrupt

line from the Enhanced Mode (Group 2) interrupt before the

enable/disable logic.

0   Busy _ Set by a write to the Attention register or a hard

reset. Cleared when the firmware reads the Attention register.

------------------------------------------------------------------------------

4-23

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

4.4.4.6 Enhanced Mode (Group 2) Status 2 (G2STAT2, zCDC, R)

------------------------------------------------------------------------------

Bit  Definition

------------------------------------------------------------------------------

7-1  Unused

0   Host Ready _ This bit is set by the host (G2CNTRL bit 5) to

indicate that it has loaded the next data group into the mailbox

out or that it has read the data group from the mailbox in. This

bit is used during Immediate command execution and need not be

set during normal operation. It is cleared by the adapter when

all pending information is processed.

------------------------------------------------------------------------------

4.4.5  82355 BMIC Registers (zC88 - z(9F)

The local processor must load the I/O decode registers before loading

the ID in the ID registers. There are two decode groups, 0 and 1,

which are used for I/O address decode. The Group 0 registers decode

the address of the configuration group to CD0-CDF. In order to do

this, one must load a value of 47h in the I/O Control Register

(register 3A), and a value of F0h in the I/O Base Register (register

39). These registers are accessed by first loading the index

associated with that register in LINDX (4001) and then writing the

data to LDAT (4000). The EISA IDs are written in the same way.

The Group 1 decode logic is used for the address of C84. In order to

decode this address, a value of 40h must be loaded into the I/O

Control Register (register 3C) and a value of E1h in the I/O Base

Register (register 3B).

4-24

Section Four                                Hardware Functional Description

------------------------------------------------------------------------------

4.5 FIRMWARE DOWNLOAD

The Local Processor will download the firmware from EEPROM and will

run from RAM. In order to promote maximum data integrity, the

firmware calculates checksums for itself while not executing commands.

The firmware is interrupt driven by events on the host and SCSI side.

The firmware is programmed into the EEPROM by setting the Enable Prom

Write bit in the PORTPH Register.  When this is done, reads will come

from the RAM and writes will go to the EEPROM. The firmware is

enabled to reprogram when the CONFIG bit in PORTADDR is set and the

firmware download command is given. The AHA-1740 INTERFACE bit in

PORTADDR indicates which code to program. Provision is also made for

loading of new firmware without programming. This definition is in

the AHA-1740 Configuration document.

Once the code is loaded in the RAM, and the firmware is enabled to

program the EEPROM, then the following steps must be followed.

1. Write AA to 5555

2. Write 55 to 2AAA

3. Write A0 to 5555

4. Write data to addresses, 64-byte blocks

5. Repeat 1 through 4 until done

4-25

Section 5                                    Standard Mode Firmware Description

------------------------------------------------------------------------------

5.1 HOST ADAPTER COMMAND OVERVIEW

Standard Mode is the mode in which the AHA-1740/1744 can operate in

order to run existing software for earlier Bus Master Host Adapters.

In Standard Mode, the AHA-1740/1744 supports two types of commands:

SCSI and Adapter Commands. SCSI commands are issued using the mailbox

protocol and a Command Control Block. When SCSI commands are used, the

AHA-1740/1744 is operating in true multithreading mode. In this mode

of operation, the AHA-1740/1744 is capable of executing multiple

commands for multiple targets concurrently. The AHA-1740/1744

maximizes the I/O transaction throughput by managing SCSI

disconnection and reconnection.

Adapter Commands are issued by writing to the Command/Data Out port.

Most Adapter Commands cannot be issued when there are outstanding SCSI

commands. Adapter      Commands are used to initialize the host

adapter and to establish control conditions within the host adapter.

Adapter Commands are also used to transmit the special parameters for

communication between the BIOS and the host adapter for the execution

Interrupt 13 operations.

Following is a list of the Adapter Command operation codes.

00 _ No Operation

01 _ Mailbox Initialization

02 _ Start SCSI Command*

03 _ Start PC AT BIOS Command

04 _ Adapter Inquiry

05 _ Enable Mailbox Out Available Interrupt*

06 _ Set Selection Time Out

07 _ Set Bus-On Time

08 _ Set Bus-Off Time

09 _ Set Transfer Speed

0A _ Return Installed Devices

0B _ Return Configuration Data

0C _ Enable Target Mode_

0D _ Return Setup Data_

10 _ Initialize SCSI Subsystem

11 _ Return Firmware Checksum

1A _ Write Adapter Channel 2 Buffer_

1B _ Read Adapter Channel 2 Buffer_

1C _ Write Adapter FIFO Buffer

1D _ Read Adapter FIFO Buffer

1F _ ECHO Command Data

20 _ Adapter Diagnostic

21 _ Set Host Adapter Options

*This command can be issued when the host adapter is executing a SCSI command.

5-1

Section 5                                    Standard Mode Firmware Description

------------------------------------------------------------------------------

All Adapter Commands except Start SCSI (02) and Enable Mailbox Out

Available Interrupt (05) must be executed only when the IDLE bit

(Status bit 4) is one. Many commands require additional parameter

bytes which are then written to the Command/Data Out I/O port (base + 1).

Before each byte is written by the host to the host adapter, the

host must verify that the CDF bit (Status bit 3) is zero, indicating

that the command port is ready for another byte of information. The

host adapter usually clears the Command/Data Out Port within 100

microseconds. Some commands require information bytes to be returned

from the host adapter to the host. In this case, the host monitors the

DF bit (Status bit 2) to determine when the host adapter has placed a

byte in the Data In I/O port for the host to read. The DF bit is reset

automatically when the host reads the byte. The format of each Adapter

Command is strictly defined, so the host adapter and host system can

always agree upon the correct number of parameter bytes to be

transferred during a command.

All Adapter Commands except Return Installed devices, Start SCSI, and

Start PC AT BIOS typically require less than 200 microseconds to

complete. Return Installed Devices will typically complete in less

than three seconds. Start SCSI and Start PC AT commands completion

times will vary with the SCSI device and the command issued.

5.1.1 No Operation (Operation Code 00)

No host adapter action is taken, but HACC is set indicating command

completion. No additional information bytes are exchanged.

5.1.2 Mailbox Initialization (Operation Code 01)

This command is used to specify the number of mailbox locations used

by the host adapter and to specify the base memory location of the

mailbox area. The host adapter requires that four bytes of outbound

data follow the command byte. The definition of those four bytes is

shown below.

00   Mailbox count      (>0)

01   Mailbox address    (MSB)

02   Mailbox address

03   Mailbox address    (LSB)

Byte 0    Mailbox Count:  The Mailbox count must be greater than zero.

Byte 1,2,3 Mailbox Address: The Mailbox address specifies the

location of the first byte of the

Mailbox area. Byte 1 is the most

significant byte.

When the Mailbox Initialization command and parameters are received,

the host adapter will then assume that the specified number of Mailbox

Out entries and the same number of Mailbox In entries will be assigned

beginning at the Mailbox address. The total number of bytes reserved

for the Mailbox area will be eight times the Mailbox Count. If the

Mailbox Count is zero, the INVDCMD bit will be set with HACC to

indicate that the parameter is invalid.

5-2

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

At command completion, HACC will be set to one and INIT will be reset

to zero. HACC will be reset as specified by Section 4.2.3.

5.1.3 Start SCSI Command (Operation Code 02)

This command indicates that the host has made at least one Mailbox Out

entry and that the host adapter should begin to scan for active MBO

entries. Once scanning has been started, it continues until all MBO

entries have been serviced, either by beginning the requested

operations or by queuing the activities for later execution. Since it

is not easy for the host to determine that scanning is still taking

place, the host should normally issue this command every time a

Mailbox Out is filled. This command does not require additional data

bytes. To avoid unnecessary interrupts, HACC is NOT set after command

completion. HACC will be set with INVDCMD if the mailbox was not yet

initialized.

5.1.4 Start PC AT BIOS Command (Operation Code 03)

This command is used by the Adaptec BIOS to communicate with the host

adapter firmware. The command is not available for use by programs

other than the Adaptec BIOS.

5.1.5 Adapter Inquiry (Operation Code 04)

After receiving this command, the host adapter returns four bytes of

data describing the host adapter firmware revision level. After

completing the transfer of the four bytes of inbound data, the HACC

interrupt is set indicating normal command completion. The bytes

contain the following information:

00      Board Identification

01      Special Options Identification

02      Firmware Revision Level (First byte)

03      Firmware Revision Level (Second byte)

Byte 0  Board ID:          The value in this byte allows software

supported on both the PC AT and on the

Micro Channel to distinguish the type

of supporting host adapter.

Value

Meaning

00H

Board is an AHA-1540 with 16-Head BIOS

30H ('0' ASCII)

Board is an AHA-1540 with 64-Head BIOS

41H ('A' ASCII)

Board is an AHA-1540/1542, 64-Head

BIOS

42H ('B' ASCII)

Board is an AHA-1640, 64-Head BIOS

43H ('C' ASCII)

Board is an AHA-1740/1744

All Others

Reserved

5-3

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Byte 1  Special Options ID:  The value in this byte indicates what

special options are supported on the

AHA-1740/1744 host adapter. Other host

adapters use other values in this

byte.

Value              Meaning

41H ('0' ASCII)    Board is standard model

All Others         Reserved

Byte 2  Firmware Revision: This value indicates an ASCII value

from 0-9, that indicates the first

digit of the firmware base code

revision installed in the

AHA-1740/1744.

Byte 3  Firmware Revision: This value indicates an ASCII value

from 0-9, that indicates the second

digit of the firmware base code

revision installed in the

AHA-1740/1744.

After completing this command, HACC will be asserted, indicating normal

completion.

5.1.6 Enable Mailbox Out Available Interrupt (Operation Code 05)

The Enable Mailbox Out Available Interrupt command specifies that a

Mailbox Out Available Interrupt should be generated whenever a Mailbox

Out entry has been cleared by the host adapter. One byte of outbound

data is transmitted to indicate whether the interrupt should be

enabled or disabled.

Byte 0 Enable/Disable Parameter:  The Enable/Disable parameter byte

must be either 00H or 01H. If the

Enable/Disable parameter byte is

00H, the Mailbox Out Available

interrupt is not returned. If the

Enable/Disable parameter byte is

01H, the Mailbox Out Available

interrupt is returned as soon as a

Mailbox Out has been cleared by the

host adapter.

After completing this command, HACC will NOT be asserted to avoid

generating additional interrupts. If the data byte is neither 00 nor

01, INVDCMD will be asserted indicating an invalid command. In this

case, HACC will also be asserted.

The Mailbox Out Available interrupt is normally intended to be used by

the host as an indicator that a Mailbox Out entry is available. This

function should only be enabled if all Mailbox Out entries have been

found full by the host. It should be disabled soon afterward. If used

in other ways, the interrupt may generate a large number of relatively

useless interrupt handling activities.

5-4

Section 5                                    Standard Mode Firmware Description

------------------------------------------------------------------------------

5.1.7 Set Selection Time out (Operation Code 06)

This command sets the SCSI selection time out value. The SCSI

Selection time out value is used to determine whether a SCSI selection

was successful. If the SCSI BSY signal is not returned within the

specified time out value, the selection will be terminated and an

appropriate error message posted with the returned CCB. This command

expects four outbound data bytes to be provided as defined below:

00       Enable/Disable Selection Time Out

01       Reserved (00)

02       Time Out Value      (MSB)

03       Time Out Value   (LSB)

Byte 0    Enable/Disable      This parameter byte specifies whether the

Selection Time Out: Selection Time Out will be used. If

the byte is set to 00H, no time out

will be performed. If the byte is

set to 01H, the time specified in

bytes 02 and 03 will be used as the

Selection Time Out by the SCSI. The

default value established by the

reset process is 01H, indicating

the time out is enabled.

Byte 1    Reserved:           This byte must be zero.

Byte 2,3  Time Out Value:     This two-byte value specifies the

time in milliseconds that will be

used for the Selection Time Out.

The default value is 250

milliseconds.

After completing this command, HACC will be asserted, indicating

normal completion. INVDCMD will be asserted if the data byte 0 is

neither 00 nor 01 or if byte 1 is not zero, indicating an invalid

command.

5.1.8 Set Bus-On Time (Operation Code 07)

This command specifies the time that the host adapter spends on the

bus when transferring data. The Bus-On duration is adjustable from 2

to 15 microseconds. The default setting is 11 microseconds. One data

byte is passed out to the host adapter to indicate the Bus-On duration

in microseconds.

Data Byte 0 -- Bus On Time (2 to 15 �s)

After completing this command, HACC will be asserted indicating normal

completion. INVDCMD will be asserted, indicating an invalid command,

if the data byte is greater than 15. The valid range is 2 to 15

decimal. Due to the internal differences between the AHA-1740/1744 and

earlier host adapters, the data not actually used is saved and

returned in the "Return Setup Data" command. The command is accepted

only for compatibility.

5-5

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

5.1.9 Set Bus Off Time (Operation Code 08)

This command sets the time that the host adapter will spend off the

bus during a data transfer. The Bus-On duration is adjustable from one

to 64 microseconds. The default setting is four microseconds. After

receiving this command, the host adapter expects one byte of data

which specifies the bus off time in microseconds.

Data Byte 0 - Bus Off Time (1 to 64 microseconds)

After completing this command, HACC will be asserted indicating normal

completion. INVDCMD will be asserted, indicating an invalid command,

if the data byte is greater than 64. The valid range is four to 64

decimal. The actual time implemented by the host adapter is rounded

down to the next 4-microsecond step at or below the specified value.

The minimum value is approximately one microsecond. Due to the

internal differences between the AHA-1740/1744 and earlier host

adapters, the data not actually used is saved and returned in the

"Return Setup Data" command. The command is accepted only for compatibility.

5.1.10 Set Transfer Speed (Operation Code 09)

This command adjusts the Bus Master DMA circuitry to a specified

maximum AT bus transfer speed to and from the host memory. Several

speeds may be selected by setting the jumper configuration as

explained in Section 3.3. The selected jumper speeds are overridden if

the Set Transfer Speed command is executed. The single data byte

transmitted after the command byte either sets the read and write

speed together, or establishes separate values for each. Appendix A

gives the timings that result from the selected values. The default

setting is the value set by the jumpers. The I/O Channel Ready signal

automatically slows the system further if required by the host memory.

Data Byte  0  _ AT Bus Transfer Speed

00  _ 5.0 MBytes/second

01  _ 6.7 MBytes/second

02  _ 8.0 MBytes/second

03  _ 10 MBytes/second

04  _ 5.7 MBytes/second

80-FF  _ See Table

Due to the internal differences between the AHA-1740/1744 and earlier

host adapters, the data not actually used is saved and returned in the

"Return Setup Data" command. The command is accepted only for compatibility.

5-6

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

[Picture]

After completing this command, HACC is set indicating normal

completion.

5.1.11 Return Installed Devices (Operation Code 0A)

This command returns information about which SCSI Targets and Logical

Units (LUs) are installed on the SCSI bus. The host adapter issues the

SCSI Test Unit Ready command to each target/LUN combination and

reports the results using eight bytes of data returned through the

Data In Register. A bit having a value of one indicates that the

associated LU is installed. Each byte is associated with the

corresponding target. Each bit within a target byte is associated with

a particular Logical Unit, bit 7 indicating the presence of LU 7 and

so forth. The state of the target is analyzed using the SCSI TEST UNIT

READY command, analyzing the returned data to determine if the

addressed LU is available.

After receiving this command, the host adapter returns eight bytes of

information which specify the installed configuration as shown below:

Byte 0    Target 0 Configuration:

Bit 7       LU 7 Installed

Bit 6       LU 6 Installed

Bit 5       LU 5 Installed

Bit 4       LU 4 Installed

Bit 3       LU 3 Installed

Bit 2       LU 2 Installed

Bit 1       LU 1 Installed

Bit 0       LU 0 Installed

5-7

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Byte 1    Target 1 Configuration

Byte 2    Target 2 Configuration

Byte 3    Target 3 Configuration

Byte 4    Target 4 Configuration

Byte 5    Target 5 Configuration

Byte 6    Target 6 Configuration

Byte 7    Target 7 Configuration

If, during the execution of this command, a target reports a status of

Busy, the host adapter will continue reissuing the command until

either the drive reports not Busy, or one minute of time elapses.

Commands will be reissued at .25-second intervals. The byte associated

with the SCSI Device Identifier of the host adapter will always be

zero. After completing the information transfer, the HACC bit will be

set to indicate normal completion.

5.1.12 Return Configuration Data (Operation Code 0B)

The DMA arbitration priority, the Interrupt channel, and the SCSI ID

of the adapter are returned by this command. The three bytes of

information are returned in the following format:

Byte 0    DMA Arbitration Priority:

Bit 7       Channel 7

Bit 6       Channel 6

Bit 5       Channel 5

Bits 4-1    Reserved (0)

Bit 0       Channel 0

Byte 1    Interrupt Channel:

Bit 7       Reserved (0)

Bit 6       Interrupt channel 15

Bit 5       Interrupt channel 14

Bit 4       Reserved (0)

Bit 3       Interrupt channel 12

Bit 2       Interrupt channel 11

Bit 1       Interrupt channel 10

Bit 0       Interrupt channel 9

Byte 2    SCSI Identifier

Bits 7-3    Reserved (0)

Bits 2-0    Binary value of SCSI Identifier

After completing this command, HACC will be set indicating normal completion.

5-8

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

5.1.13 Enable Target Mode Command (Operation Code 0C)

A special host adapter command (0C) enables and disables target mode.

The host adapter requires that two bytes of outbound information follow the

command byte. The information bytes contain the following information:

Byte 0 Enable/Disable Target Mode:     This parameter specifies which

operating modes the host adapter will

use. If the byte is set to 00H, the

AHA-1740/1744 will operate only in

Initiator Mode. If the byte is set to

01H, the host adapter will operate

both as an initiator and as a

Processor-type Target SCSI Device. Any

other value is invalid. The default

value after a Hard Reset, Soft Reset,

or Power-On Reset is 00H (Initiator

only).

Byte 1       Logical Unit Mask: This parameter byte contains an 8-bit

bit-significant mask indicating the

Logical Units which will respond in

Target Mode. Bit 7 corresponds to LU

7, and so forth. If the bit is one,

the corresponding LU will be treated

as installed. If the bit is zero, the

LU will be treated as not installed.

If the AHA-1740/1744 does not have the Target Mode feature installed,

the host adapter will indicate an invalid host adapter command. If an

attempt is made to change from target mode while there are still

target mode CCBs being processed by the host adapter, the host adapter

will post an invalid host adapter command indication.

If the command disables Target Mode, the Logical Unit Mask byte will

be ignored. If the command enables Target Mode, the Logical Unit Mask

byte must contain at least one bit, indicating the presence of at

least one Logical Unit.

The SCSI INQUIRY command will provide an indication to other

initiators that the Logical Unit is installed or not installed in byte

1 of the returned inquiry data.

If target mode is not enabled, the host adapter will behave on the

SCSI interface as if it were an ordinary SCSI initiator. Any attempt

to select the host adapter will result in a SCSI selection time out.

Most Reset Operations, including Soft Reset, Hard Reset, and Power-On

Reset will return the AHA-1740/1744 Target Mode to the disabled state.

SCSI Resets, generated either by the host or by other SCSI Devices,

will not change the previously established enabled or disabled state

of Target Mode.

5.1.14 Return Setup Data (Operation Code 0D)

This command returns information describing the setup of the host

adapter. The information returned reflects either the values supplied

by previous host adapter commands or default values. The command is

followed by an outbound data transfer and an inbound data transfer.

The outbound transfer is a 1-byte parameter indicating the length of

the required inbound data transfer. The inbound data transfer contains

from zero to 255 bytes of information describing the setup of the host adapter.

5-9

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

The inbound information normally transferred will be truncated or padded with

zeros as necessary to transfer the requested number of bytes. The number of

bytes normally transferred by the AHA-1740/1744 is 17.

Outbound Data Byte:

Byte 0    Data In Length:  The number of data bytes requested can

be from 0 to 255. A value of zero will

be accepted and 256 bytes will be returned.

Inbound Data Summary:

00      SDT and Parity Status

01      Transfer Speed

02      Bus On Time

03      Bus Off Time

04      Number of Mailboxes

05      Mailbox Address (MSB)

06      Mailbox Address

07      Mailbox Address (LSB)

08-0F   Synchronous Transfer Agreements

10      Disconnection Option

11-FF   Reserved (00)

Inbound Data Bit Assignments:

Byte 0    SDT and Parity Status:

Bit              Meaning

0              If this bit is zero, Synchronous Data

Transfer negotiation will not be

initiated by the host adapter, but

will be responded to if requested by

an attached SCSI Device. If the bit is

one, Synchronous Data Transfer

negotiation will be initiated by the

host adapter under those conditions

which require such negotiation. The

state is set from jumper J1.

1              If this bit is zero, parity checking

on inbound SCSI transfers has been

disabled. If this bit is one, parity

checking on inbound SCSI transfers is

enabled. The state is set from jumper J1.

2-7              Reserved (0)

Byte 1    Transfer Speed   This byte returns the value passed in

to the host adapter by the Set

Transfer Speed command (see Section 5.1.10)

5-10

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Byte 2   Bus On Time:         Indicates the Bus On Time in

microseconds (see Section 5.1.8)

Byte 3   Bus Off Time:        Indicates the Bus Off Time specified

by the Bus Off Time Value in

microseconds (see Section 5.1.9)

Byte 4  Number of Mailboxes:  The number of Mailboxes established by

a previous Mailbox Initialization

command will be returned in this byte.

This number will be 00H if the Mailbox

Initialization command has not yet

been successfully completed.

Byte 5-7 Mailbox Address:     The base address of the Mailbox area

established by a previous Mailbox

Initialization command will be

returned in these bytes. The Most

Significant byte is byte 5. These

bytes have no meaning if Mailbox

Initialization has not yet been

successfully completed.

Byte 8  Sync Neg, Target 0    Returns information about the

synchronous negotiation with Target 0.

The byte will be 00H for the address

of the host adapter.

Bit 7                 Set to one if synchronous transfer has

been negotiated. Set to zero

negotiation has not occurred.

Bits 6-4              These bits contain a value between 0

and 7 that defines the synchronous

transfer period according to the

following equation.

Period = 200 + 50 (value) nanoseconds

Bits 3-0              These bits contain the negotiated

offset value. The value will normally

be between 1 and 7. A value of 0

indicates asynchronous transfer.

5-11

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Byte 9  Sync Neg, Target 1  Same as byte 8, for target 1

Byte 0A Sync Neg, Target 2  Same as byte 8, for target 2

Byte 0B Sync Neg, Target 3  Same as byte 8, for target 3

Byte 0C Sync Neg, Target 4  Same as byte 8, for target 4

Byte 0D Sync Neg, Target 5  Same as byte 8, for target 5

Byte 0E Sync Neg, Target 6  Same as byte 8, for target 6

Byte 0F Sync Neg, Target 7  Same as byte 8, for target 7

Byte 10                     Disconnection Option

Each bit corresponds to a SCSI device,

e.g., bit 0 corresponds to the device

at SCSI address 0. When set to one,

the host adapter will prevent the SCSI

device from disconnecting.

If the command completes normally, the HACC interrupt will be set to

one. If the Mailbox area has not been properly initialized, all 17

bytes are still requested by the host. If byte 4, the Number of

Mailboxes, is zero, bytes 5-7 must be ignored.

5.1.15 Initialize SCSI Subsystem (Operation Code 10)

After receiving this command, the host adapter expects one byte from

the host to configure the SCSI bus.

Enable/Disable Disconnect/Reconnect

Enable/Disable Synchronous Negotiation

Enable/Disable SCSI Bus Parity

5.1.16 Return Firmware Checksum (Operation Code 11)

After receiving this command, the host adapter passes two bytes to the

host which reflect the checksum of the Standard Mode micromode. Note

that this checksum is not marked on the board. The checksum on the

EEPROM reflects an aggregate of the Enhanced Mode, Standard Mode and

Download Utility microcodes.

5.1.17 Write Adapter Channel 2 Buffer (Operation Code 1A)

After receiving this command, the host adapter expects three outbound

information bytes to be transferred which point to an area of 64 bytes

in system RAM. The area pointed to will be transferred to the host

adapter's Channel 2 Buffer using the host adapter's DMA circuitry.

After completing the transfer of the 64 bytes from the indicated

buffer area to the host adapter, the HACC interrupt will be set

indicating normal completion. This command is used in conjunction with

the Read Channel 2 Buffer command for host adapter diagnostics. The

Channel 2 Buffer is used for transmission of all information except

the actual data field between the host adapter and the host system.

Byte 0    Buffer area address, Most significant byte

Byte 1    Buffer area address

Byte 2    Buffer area address, Least significant byte

5-12

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

5.1.18 Read Adapter Channel 2 Buffer (Operation Code 1B)

After receiving this command, the host adapter expects three outbound

information bytes to be transferred which point to an area of 64 bytes

in system RAM. The area pointed to will be used as a buffer to

receive 64 bytes of information transferred from the host adapter's

Channel 2 Buffer to the host's memory using the host adapter's DMA

circuitry. After completing the transfer of the 64 bytes from the

Channel 2 Buffer to the indicated host memory area, the HACC interrupt

will be set indicating normal completion. This command is used in conjunction

with the Write Channel 2 Buffer command for host adapter diagnostics.

Byte 0 Buffer area address, Most significant byte

Byte 1 Buffer area address

Byte 2 Buffer area address, Least significant byte

5.1.19 Write Adapter FIFO Buffer (Operation Code 1C)

After receiving this command, the host adapter expects three outbound

information bytes to be transferred which point to an area of 54 bytes

in system RAM. The area pointed to will be transferred to the host

adapter's FIFO buffer using the host adapter's DMA circuitry. After

completing the transfer of the 54 bytes from the indicated buffer area

to the host adapter, the HACC interrupt will be set indicating normal

completion. This command is used in conjunction with the Read Adapter

FIFO Buffer command for host adapter diagnostics.

Byte 0 Buffer area address, Most significant byte

Byte 1 Buffer area address

Byte 2 Buffer area address, Least significant byte

5.1.20 Read Adapter FIFO Buffer (Operation Code 1D)

After receiving this command, the host adapter expects three outbound

information bytes to be transferred which point to an area of 54 bytes

in system RAM. The area pointed to will be used as a buffer to

receive 54 bytes of information transferred from the host adapter's

FIFO to the host's memory using the host adapter's DMA circuitry.

After completing the transfer of the 54 bytes from the FIFO to the

indicated buffer area, the HACC interrupt will be set indicating

normal completion. This command is used in conjunction with the Write

Adapter FIFO Buffer command for host adapter diagnostics.

Byte 0 Buffer area address, Most significant byte

Byte 1 Buffer area address

Byte 2 Buffer area address, Least significant byte

5.1.21 ECHO Command Data (Operation Code 1F)

This command is used to test the Command/Data Out Port, the Data In

Port, and the associated control bits in the other I/O Ports. After

receiving this command, the host adapter expects one byte of outbound

information to be transferred through the Command/Data Out Port.

5-13

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

The host adapter then sends (Echos) the same data value back to the host

through the Data In Port. After the host has read the data value

provided on the Data In Port, the host adapter will generate the HACC

interrupt to indicate normal command completion.

Byte 0, Out       Outbound Echo Value

Byte 0, In         Returned Echo Value

5.1.22 Adapter Diagnostic (Operation Code 20)

Unlike other Adaptec host adapters, this command does not cause the

host adapter to perform its internal self-diagnostic. A host adapter

Hard Reset is not executed, nor is a SCSI bus reset. The host adapter

need not be reinitialized after this command before resuming normal

operation. No actual diagnostics are performed, although internal

flags are reset to the default state.

5.1.23 Set Host Adapter Options (Operation Code 21)

Unlike other Adaptec host adapters, this command does not provide the

host a mechanism for specifying certain host adapter configuration

options. Configuration parameters sent to the host adapter via a

parameter list following the command OP Code are ignored.

5.2 MAILBOX OVERVIEW

The AHA-1740/1744 uses a mailbox architecture for task communication

between the host and the host adapter when executing SCSI commands.

This allows the host adapter to perform multithreaded operations with

a minimum of host intervention. The mailboxes are located in main

system memory. Each mailbox entry is four bytes long. At power-up, the

host issues an initialization command to inform the host of the

mailbox location. There are always an equal number of Outgoing

Mailboxes (MBO) and Incoming Mailboxes (MBI). The MBIs are located

immediately after the MBOs. Initiator operations and target operations

use the same mailboxes. Both initiator and target operations may be in

process at the same time. A typical mailbox structure follows.

This command is provided only for compatibility. Refer to Section

5.1.15 for a discussion of operation code 10, which provides these functions.

[Picture]

5-14

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

In this example, there are four MBOs and four MBIs. The Mailbox Count

in the Mailbox Initialization command was set to four. The base

address is the address specified by the Mailbox Address field.

Note: The Adaptec BIOS initializes the AT DMA controller to

accommodate Bus Master DMA. If the host adapter BIOS is removed or

disabled, the host DMA controller must be initialized, via software,

for Bus Master DMA operation. See Section 5.5.

5.2.1 Mailbox Out Definition

The first byte of each MBO contains the Mailbox status byte. The

remaining three bytes contain an address pointer to the first byte of

a Command Control Block (CCB). The CCB provides additional task

control information. An MBO is free if the first byte is zero. The

host can make an entry in any free MBO and indicate that it is filled

out or completed by placing the proper MBO command in the first byte

of the MBO. After the MBO has been examined by the host adapter and

all relevant information obtained by the host adapter, the host

adapter sets the MBO command byte back to zero to allow the host to

fill it again. For a multitasking operating system, it is desirable

that the number of mailboxes be sufficient to allow at least one

mailbox for each active independent task or activity.

The MBO format is described in detail below:

Byte 0    MBO Command      This byte specifies the state of the MBO entry.

Value            Definition

00H              Mailbox Out is free

01H              SCSI or Host Adapter command is to be

started. CCB pointer indicates

location of CCB to be processed.

02H              SCSI or Host Adapter command is to be

aborted. CCB pointer indicates

location of CCB to be terminated.

Byte 1    CCB Pointer (MSB)

Byte 2    CCB Pointer

Byte 3    CCB Pointer (LSB)

For values of byte 0 other than those listed above, the host adapter

will simply set byte 0 to 00H and ignore the Mailbox Out (MBO) entry.

No corresponding Mailbox In will be generated.

The use of Mailbox Out (MBO) entries to pass pointers to CCBs is identical for

target and initiator modes. The appropriate target mode CCB may be prepared

early and posted to the host adapter in preparation for an operation that is

expected to happen, If a SCSI operation occurs before the CCB is prepared, the

host adapter processes as much of the transaction as possible, then requests a

CCB from the host through the MBI. The host is fully responsible for preparing

the correct CCB from the information provided through the MBI.

5-15

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

In target mode, one CCB may be presented for each unique combination

of LUN, Initiator, and direction. If a second CCB to the same LUN and

initiator with the same direction bit is sent to the AHA-1740/1744,

the CCB will be returned with a host status of 19H, Duplicate CCB Received.

Initiator-type CCBs may be queued for a LUN and Target. The host

adapter will always search for new MBO entries in a round-robin order,

beginning with the entry after the last MBO entry that was processed.

By always placing the MBO entries in the MBO area consecutively, the

host can assure that the SCSI commands will be started with a minimum

scan overhead. Initiator CCB queuing must be used with caution, since

under some circumstances it is possible for CCBs to be executed out of order.

If the Mailbox Out Available Interrupt is enabled by execution of

Adapter Command 05 (Enable Mailbox Out Available Interrupt), the host

adapter will take one of the following actions after each MBO entry is

freed by the host adapter:

1) If the Any Interrupt bit in the Interrupt Flag Port is zero,

indicating that there are no interrupts pending, the host adapter

will set the Mailbox Out Available Interrupt and the Any Interrupt

bit and indicate a hardware interrupt to the host.

2) If the Mailbox Out Available Interrupt has been set to one by the

previous clearing of a Mailbox Out entry and the interrupt has not

yet been cleared by the host, the host adapter will not change the

MBOA interrupt bit and will continue to scan for other stored MBO entries.

3) If interrupts other than the Mailbox Out Available Interrupt are

pending, the host adapter will wait for the pending interrupts to be

cleared before setting the Mailbox Out Available Interrupt. This

guarantees that the MBO Available Interrupt will not be cleared

accidentally by clearing another interrupt.

5.2.2 Mailbox In Definition

The Mailbox In entries are used to pass completion information

concerning a task from the host adapter to the host. In addition,

requests for Target Mode CCBs are passed to the host using MBI

entries. The first byte of each MBI contains the MBI Status byte,

summarizing the type of information being passed from the host adapter

to the host. The remaining three bytes contain specialized information

that provides more detail about the information. In the case of a CCB

Completed MBI, the bytes contain a pointer to the completed CCB. In

the case of a CCB Required MBI, the bytes contain the information

necessary for the host to prepare an appropriate CCB. Only those MBIs

with an MBI Status that is nonzero have information for the host. When

the host returns the MBI Status byte to zero, the MBI Free state, the

host adapter is allowed to place a new set of information in the MBI entry.

5-16

Section 5                                   Standard Mode Firmware Decription

------------------------------------------------------------------------------

When a SCSI command completes or if a new CCB is required, the host

adapter scans the first byte of an MBI entry to find a free mailbox.

If one is found, the host adapter will update the MBI's status byte

with a nonzero value and update the following bytes with the

appropriate required pointers or parameters. The valid MBI formats are

defined below:

Mailbox In Format for CCB Complete

Byte 0    MBI Status    This byte specifies the state of the MBI entry.

Value         Definition

00H           Mailbox In is free

01H           CCB completed without error. CCB pointer

indicates location of successfully completed CCB.

02H           CCB aborted by host. CCB pointer indicates

location of CCB that was aborted.

03H           Aborted CCB not found. CCB pointer

indicates the supposed location of the

CCB that was to have been aborted. It

is likely that the CCB was already

presented to the host before the Abort

CCB MBO entry was completed.

04H           CCB Completed with Error. The CCB

fields indicate the details of the

error condition. This code allows

normal CCB completion to be processed

without bothering to examine the

completion codes in the CCB.

Byte 1    CCB Pointer (MSB)

Byte 2    CCB Pointer

Byte 3    CCB Pointer (LSB)

Mailbox In Format for CCB Required

Byte 0    MBI Status       This byte specifies the state of the MBI entry.

Value            Definition

10H              SCSI Target Command received with no

CCB available. The host must prepare

an appropriate CCB and place it in the

MBO to complete the SCSI command. The

remaining three bytes of the MBI

specify the information necessary to

prepare a SCSI command.

5-17

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Byte 1    Initiator and LUN

Bit 7-5          Binary address of initiator that

selected the host adapter in Target Mode.

Bit 4            The SCSI Target Command received was a

RECEIVE command. A CCB must be

prepared to transmit data to the Initiator.

Bit 3            The SCSI Target Command received was a

SEND command. A CCB must be prepared

to transmit data from the Initiator.

Bits 2-0         Binary address of the logical unit of

the Target Mode host adapter that was

addressed by the Initiator.

Byte 2-3  Data Length      The high-order two bytes of the data

length specified in the SEND or the

RECEIVE command. A CCB must be

prepared to transmit this amount of

data plus up to 256 more bytes. It is

assumed that the data fields are

self-defining or of a known length in

such protocols.

The CCB Required Mailbox In entries are only generated when a command

has been received by the host adapter in target mode. For the CCB

Required Mailbox In entries, byte 0 takes on a distinctive value of

10H, indicating the MBI entry is filled and the MBI entry is being

presented to request a CCB appropriate to service a SCSI transaction

for which no CCB was available. The host adapter will have

disconnected on the SCSI after receiving the initiator and LU

addresses and after having received either a SCSI SEND or a RECEIVE

command. If the LU address is enabled by the Logical Unit Mask, the

MBI will provide the addresses in bits 7-5 and bits 2-0 and will

indicate by setting either bit 3 or bit 4 that a SCSI SEND or RECEIVE

command was accepted from the initiator. The host system is then

expected to prepare a CCB suitable for processing the Send or Receive

command. The information in bytes 2 and 3 is a copy of bytes 2 and 3

of the Send or Receive command CDB. This provides the host an

approximation of the length of the requested data transfer. This

length is used by the host to allocate the correct amount of buffer to

accept data in a SEND command or to control the maximum amount of data

that can be transferred in a RECEIVE command. If the LU address

received is disabled, no MBI will be presented and a check condition

will be posted to the Initiator. A subsequent REQUEST SENSE command

will recover an Invalid LUN error indication.

After each Mailbox In entry is stored by the host adapter, it will

indicate that Mailbox In scanning must be performed in the following

manner:

1)If the Any Interrupt bit is not set to one in the Interrupt Flag

port, indicating that no interrupts are pending, the host adapter

will set the Mailbox In Full interrupt bit, the Any Interrupt bit,

and raise the hardware interrupt line to indicate that there is at

least one Mailbox In entry to be examined by the host system.

5-18

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

2)If the Mailbox In Full interrupt bit has already been set to

indicate that an entry was made in the Mailbox In area and if that

interrupt has not yet been cleared by the host, no further

notification is provided by the host adapter.

3)If interrupts other than the Mailbox In Full interrupt are

pending, the host adapter will wait for all pending interrupts to be

cleared before setting the Mailbox In Full interrupt.

Note: Careful host Target Mode software design is necessary to prevent

ambiguity in the notification process. If a CCB was just prepared but

the MBO has not yet been searched by the host adapter at the time the

MBI entry is made to the host, the host may choose to examine the MBI

entry and not act on it, having already provided the needed CCB. The

host should also examine the entire MBI space to be sure that a

previously supplied CCB which should have been able to service the

notification MBI entry was not already used for a previous operation

that had been posted by the host adapter but not yet serviced by the

host. Proper care in host system design will prevent these overlapping

operations from becoming a problem.

5.3 COMMAND BLOCK DEFINITION

The CCB specifies detailed information about a SCSI command. The

format of a CCB is shown below. Each of the fields is separately

explained following the table.

COMMAND CONTROL BLOCK FORMAT

Byte 0    Command Control Block Operation Code

Value         Definition

00H           SCSI Initiator Command

01H           Target Mode Command

02H           SCSI Initiator Command with Scatter/Gather

03H           SCSI Initiator Command, residual data

length returned

04H           SCSI Initiator command with

Scatter/Gather, residual data length returned

81H           Bus Device Reset

Byte 1    Address and Direction Control

Bits 7-5         CCB OP Code = 00, 02, 03, 04: SCSI Target Id

CCB OP Code = 01: SCSI Initiator Id

Bit  4           Outbound data transfer, length is checked.

Bit  3           Inbound data transfer, length is checked.

Bit  2-0         Logical Unit Number

Byte 2    SCSI Command Length

Byte 3    Request Sense Allocation Length/Disable Auto Sense

5-19

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Byte 4-6   Data Length (Byte 4 most significant)

Byte 7-9   Data Pointer (Byte 7 most significant)

Byte 10-12 Link Pointer (Byte 10 most significant)

Byte 13    Command Linking Identifier

Byte 14    Host Adapter Status (HASTAT)

Byte 15    Target Device Status (TARSTAT)

Byte 16-17 Reserved (00)

Byte 18-n  SCSI Command Descriptor Block

(Length specified by byte 2)

Byte n-m   Reserved for REQUEST SENSE information bytes

(Length of reserved space in byte 3)

The standard format of the Command Control Block is shown in the

following diagram:

[Picture]

5-20

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

The bytes of the CCB are defined further below:

Byte 0: Command Control Block Operation Codes

The AHA-1740/1744 supports six CCB Operation Codes. The valid command

values are shown below:

00   SCSI Initiator Command Control Block

01   SCSI Target Command Control Block

02   SCSI Initiator Command Control Block with Scatter/Gather

03H  SCSI Initiator Command, residual data length returned

04H  SCSI Initiator Command with Scatter/Gather, residual data length returned

81   SCSI Bus Device Reset

If the Operation Code value is 00H or 03H, the SCSI command specified

in the Command Descriptor Block field of the CCB is executed against

the addressed Target /LUN. The other fields of the CCB support the

required Initiator functions of the AHA-1740/1744. Operation codes 00H

and 03H differ only in the updating of Bytes 4, 5, and 6, following

completion of the command.

If the Operation Code value is 01H, the CCB is intended to service a

SEND or RECEIVE command sent to the host adapter as a target from

another initiator. The values in the other fields are used to service

the Target Mode operation. If an Operation Code of 01H is specified to

a host adapter that has not had its Target Mode enabled, the host

adapter returns a Host Status indication of 18H, Invalid CCB

Parameter.

If the Operation Code value is 02H or 04H, the SCSI command specified

in the Command Descriptor Block field of the CCB is executed against

the addressed Target/LUN. The definition of the Data Length and the

Data Pointer is modified to support the Scatter/Gather function. Refer

to Section 5.2.4. Operation codes 02H and 04H differ only in the

updating of Bytes 4, 5, and 6 following the completion of the command.

If the value is 81(hex) a Bus Device Reset message will be sent to the

addressed target. This command forces the host adapter to abort all

outstanding tasks against the selected target. All remaining CCB Bytes

are ignored. The host adapter will generate a Bus Device Reset message

out to the specified target.

Any other command value generates a Host Adapter Detected Error (Host

Status byte of 16H).

Byte 1: Address and Control Byte

This byte identifies the address of the devices that will be serviced

and provides information about the expected direction of data flow.

If the CCB is an Initiator CCB, this byte identifies the target SCSI

device in bits 7, 6, and 5. If the CCB is a Target CCB, the byte

identifies the initiator which the CCB will serve.

5-21

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Bits 4 and 3 are set to determine the direction of data transfer. For

an Initiator CCB, the direction of data transfer will be established

by the SCSI command being executed independent of the value of bits 3

and 4. For a Target CCB, if neither bit is set or both bits are set,

the CCB will be returned with an indication of Invalid CCB Parameters

(18H) in the Host Status field, since each Target CCB must be

identified as to whether it will service a SEND or a RECEIVE command.

If both bits are set for an initiator CCB, the command must perform no

data transfer. If only bit 3 is active, the data transfer will be to

the host adapter and from the external SCSI device. If the CCB is a

target CCB, the data transfer will be required to be a Data Out phase,

while if the CCB is an initiator CCB, the data transfer will be

required to be a Data In phase. If only bit 4 is active, the data

transfer will be from the host adapter and to the external SCSI

device. If the CCB is a target CCB, the data transfer must be a Data

In phase, while if the CCB is an initiator CCB, the data transfer will

be a Data Out phase.

If bits 3 or 4 are set for an Initiator CCB, the data length will be

checked. If the amount of data transferred exceeds the specified

amount, the CCB Host Status field will contain an indication of Data

Over/Under Run (12H). For a Processor Target mode CCB, the handling of

incorrect lengths is described in Section 8.2.4.

If a data underrun/overrun condition occurs for an operation that

accesses the drive's media (READ/WRITE, EXTENDED READ/WRITE, WRITE AND

VERIFY) and the direction bits are set to zero, the host adapter will

complete the operation without error. However, some or all of the data

specified by the host may not be transferred.

CAUTION: THE APPROPRIATE DIRECTION BIT SHOULD BE SET FOR ALL

OPERATIONS THAT ACCESS THE DRIVE'S MEDIA.

This enables the host adapter to check the length of the data transfer

and if a data underrun/overrun condition occurs, the CCB will be

returned with an indication of Data Underrun/Overrun (12H) in the Host

Status field. For operations that do not access the drive's media, the

direction bits should be set to zero unless transfer length checking

is desired. Setting both direction bits to one should be used only

when no data transfer is expected or suppression of data transfer is

desired for READ operations.

If the CCB is an initiator CCB, bits 2, 1, and 0 define the target

Logical Unit which will be addressed. If the target accepts an

IDENTIFY message out, the value in bits 2, 1, and 0 will be provided

in the LUN field of the message byte. The LUN field in the SCSI

Command Descriptor Block (CDB) is expected to be zero. If the target

does not accept an IDENTIFY message out, the LUN field in the SCSI CDB

must contain the correct Logical Unit address. SCSI devices with

conformance level 2, including Common Command Set (CCS) disk drives

and all SCSI-2 devices will always accept the IDENTIFY message out.

The few SCSI devices not meeting those requirements must be examined

on a case-by-case basis to determine whether the Logical Unit Address

should be placed in CCB Byte 1 or in the CDB.

Byte 2: SCSI Command Length

This byte establishes the length, in bytes, of the SCSI Command Descriptor

Block.

5-22

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Byte 3: Request Sense Allocation Length

When a SCSI device terminates an operation with CHECK CONDITION

status, it means that the device has error or status information as a

result of execution of the operation. The SCSI specification indicates

that a REQUEST SENSE command must be executed before any other command

is executed in order for the host initiator to be sure of obtaining

the error information. Since the AHA-1740/1744 host adapter has the

capability of queuing commands for execution, the host adapter itself

must take charge of generating the Request Sense command. The

automatic generation of Request Sense is inconvenient for some

specialized operating systems, so the function can be disabled by

installation of a jumper in J9 (see Section 3.3.11). If the jumper is

not installed, the function can also be disabled by specifying a value

of 01H for the Request Sense Allocation Length.

This byte indicates the length, in bytes, of the area reserved for

information that may be obtained by a REQUEST SENSE command. A value

of 00H indicates that an allocation length of 14 bytes is to be used,

sufficient to capture the sense key and error code of all normal

extended sense type devices. A value of 01H requests that no automatic

REQUEST SENSE be executed. The values from 02H to 07H are reserved.

Values from 08H to FFH are valid allocation lengths. The value is used

to notify the host adapter that the specified number of bytes have

been reserved at the end of the CCB to receive possible Request Sense

data bytes. The REQUEST SENSE also uses the indicated allocation

length as its byte count in the Command Descriptor Block generated by

the host adapter.

If an operation that treats the AHA-1740/1744 as a Target Mode device

fails and presents a Check Condition status byte, the Initiator should

return a REQUEST SENSE command. The AHA-1740/1744 will return

appropriate sense information in response to the command. If the

command that originally failed was a SEND or a RECEIVE command, the

same REQUEST SENSE information bytes that will later be sent to the

Initiator are also sent to the host when the CCB is returned. The

sense information is placed in the specified Request Sense Allocation

area with a length not exceeding the Request Sense Allocation Length.

Bytes 4, 5, 6: Data Length

These bytes determine the length, in bytes, of the data transfer. CCB

host adapter error 12(hex) is posted if a data overrun occurs.

If the CCB specifies a Scatter/Gather operation, the Data Length field

contains the total number of bytes in the Data Segment List.

If the Operation Code value is 00 or 02, these bytes remain unchanged

upon completion of the command. If the Operation Code value is 01,

these bytes will be set by the host adapter upon completion of the

command to the number of bytes actually transferred. If the Operation

Code value is 03H or 04H, these bytes will be set by the host adapter

upon completion of the command to the difference between the original

Data Length, as specified by the host, and the actual number of bytes

transferred across the SCSI bus. In the event of data overrun, these

bytes will be set to zero. For Operation Code 04H, the original Data

Length is the sum of all segment Data Lengths.

5-23

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Bytes 7, 8, 9 : Data Pointer

These bytes specify the real address of the first byte of the data

area to be used during the data phase of the SCSI command.

If the CCB specifies a Scatter/Gather operation, the Data Pointer

field contains the pointer to the first byte of the Data Segment List.

Bytes 10, 11, 12: Link Pointer

These bytes are used when a SCSI command contains a LINK or LINK WITH

TAG bit in the command. When a Linked command is completed, the host

adapter will use the contents of this field as a pointer to the next

CCB to execute. If the Linked Flag bit is set, an interrupt will be

generated before the next command is started. A completed CCB is

always reported back in an MBI, but MBIF interrupts are only reported

if the linked set of commands is finished or if a Link with Flag

message is presented. There must be enough MBI entries to receive the

entire set of linked commands.

Target Mode does not support the linking function.

Byte 13 : Command Link ID

This byte is used in conjunction with linked commands. It is set by

the host to identify commands in a command chain.

Linking is not supported in Target Mode.

Byte 14 : Host Status

This byte is used to report the host adapter status (HASTAT).

HASTAT has the following definitions:

00 H          No host adapter detected error

The CCB was completed normally.

11 H          Selection time out

The initiator selection or target

reselection was not complete within the Set

SCSI Selection Time out period.

12 H          Data Overrun/Underrun

The target attempted to transfer more data

than was allocated by the Data Length field

or the sum of the Scatter/Gather Data

Length fields.

13 H          Unexpected bus free

The target dropped the SCSI BSY at an unexpected time.

5-24

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

14 H          Target bus phase sequence failure

An invalid bus phase or bus phase sequence

was requested by the target. The host

adapter will generate a SCSI Reset

Condition, notifying the host with a SCRD

interrupt.

16 H          Invalid CCB Operation Code

The first byte of the CCB was invalid. This

usually indicates a software failure.

17 H          Linked CCB does not have the same LUN

A subsequent CCB of a set of linked CCBs

does not specify the same logical unit

number as the first.

18 H          Invalid Target Direction received from Host

The direction of a Target Mode CCB was invalid.

19 H          Duplicate CCB Received in Target Mode

More than one CCB was received to service

data transfer between the same target LUN

and initiator SCSI ID in the same direction.

1A H          Invalid CCB or Segment List Parameter

A segment list with a zero length segment

or invalid segment list boundaries was

received. A CCB parameter was invalid.

Byte 15: Target Status

For an initiator CCB, this byte is used to return the SCSI status byte

sent to the host adapter from the initiator. If a SCSI command returns

with BUSY status, the normal recovery process is to execute the

command again. The AHA-1740/1744 takes that burden off the host and

periodically restarts the command automatically until the command

completes with a status other than Busy.

For a target mode CCB, this byte is used to indicate to the host what

status the host adapter returned to the initiator.

Target Status may have the following values in target mode

00 - Good Status

02 - Check Status (See REQUEST SENSE byte area)

08 - LUN Busy

Byte 16: Reserved (must be 0)

Byte 17: Reserved (must be 0)

5-25

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Byte 18: n: SCSI Command Descriptor Block

This field holds the SCSI Command Descriptor Block (CDB) as described

in the SCSI specification. The length of this command is described in

byte 02. For initiator mode CCBs, the CDB provided by the host is

transmitted to the target. For target mode CCBs, the CDB provided from

the initiator is returned to the host in this space.

Byte 18+n - 18+n+m : Allocated for Sense Data

If a Check Status condition is detected by the AHA-1740/1744 as it

completes an operation on the SCSI bus, the host adapter

automatically executes a REQUEST SENSE command with the data length

specified by Request Sense Allocation Length. The actual bytes

returned, up to the maximum indicated by the Request Sense Allocation

Length, are placed in the area Allocated for Sense Data. If the

Request Sense Allocation Length was 01H, no REQUEST SENSE command is

executed.

If the REQUEST SENSE command completed without a Check Status

condition, the Target Status will be set to Check Status, and the Host

Status will be set to 00H. If the REQUEST SENSE command completed with

Check Status, the Target Status will be set to Check Status, and the

Host Status will be set to 14H.

If a check condition is detected by the AHA-1740/1744 while it is

operating in Processor Target Mode, the same information that will

later be recovered by the initiator that received the Check Status is

also placed in the area Allocated for Sense Data so that the host

processor is also aware of the failure.

5.4 SCATTER/GATHER LIST DEFINITION

Using the normal CCB Operation Codes of 00 and 01 (SCSI Initiator and

SCSI Target CCBs), the CCB itself contains a pointer to the first byte

of a contiguous area of data of a specified length. The direction of

transfer with respect to the data area and the checking of the length

of the data transfer to the data area are both managed by the

AHA-1740/1744 according to the requirements of the particular mode and

control bit set-up.

Using the Scatter/Gather Operation Code of 02 or 04H, the CCB instead

contains a pointer to a list of data segments and an indication of the

length of the list of data segments. The data segment list contains

pointers to the actual location in host memory of the data segments to

be transferred as well as a precise indication of the length of each

data segment. Each data segment list entry contains a 3-byte pointer

to the location of a data segment and 3-byte length indicator telling

how long that particular data segment shall be. The data segment list

is arranged in the order in which data will be gathered or

distributed, the first pointer in the list being used first.

A typical data segment list is shown in the following table. The data

segment list describes the distribution of 4096 bytes distributed in

four separate locations within the memory. The beginning of the data

segment list is indicated by the data segment list pointer. The length

of the data segment list (24 bytes) will be contained in the data

segment list length field of the CCB.

5-26

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

[Picture]

A data segment list can have from one to 17 segments. A list with zero

segments or a list with more than 17 segments causes an Invalid

Segment List error to be posted in the host status field.

The AHA-1740/1744 has certain constraints in the data segment address

boundaries and lengths that are allowed. If these boundaries are

violated, an Invalid Segment List error will be posted in the host

status field of the CCB. The simplest way to assure that these

boundaries are not violated is to require all boundaries between

segments to be on even word boundaries. The first segment may begin on

any boundary and the last segment may end on any boundary. All

segments but the first and last should have even byte counts.

The actual boundary limitation is somewhat less restrictive. If the

binary values of the starting address of a segment, the byte count of

that segment, and the starting address of the next segment are all

exclusive OR'd together, and the result of that exclusive OR is an

even number, the boundary between those two segments is valid. Another

way to express the limitation is to require that the ending boundary

of one segment must be the same as the beginning boundary of the next

segment. If a segment ends on a word boundary, the next segment must

begin on a word boundary. Similarly, if a segment ends on an odd-byte

boundary, the next segment must begin on an odd-byte boundary.

5.5 DESCRIPTION OF OPERATION

This section describes the interface operations required to invoke the

desired SCSI behavior. The AHA-1740/1744 must be properly initialized

before any of these operations can be performed.

After system initialization is done, the I/O Command Port

initialization commands must be executed. The Mailbox Initialization

command must be executed to assign the Mailbox area. The Enable Target

Mode command may optionally be executed if Target Mode is to be allowed.

5.5.1 Execution of Initiator Mode Operations

To begin an Initiator Mode SCSI command, the host first allocates a

data buffer area. A CCB is then created to perform the desired

operation to the correct peripheral device and a pointer to the data

buffer area is placed in the CCB. Once the CCB is completely defined,

the host places a pointer to the CCB in an empty MBO location, places

an MBO full status in the MBO status byte, and transmits a Start SCSI

Command to the I/O Command Port. If it is not already scanning the MBO

for an active MBO entry, the Start SCSI Command to the I/O Port causes

the host adapter to begin scanning for such an entry.

5-27

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

After finding an active MBO entry, the AHA-1740/1744 copies the MBO

Command field contents and CCB pointer into its internal RAM and

clears the MBO Command byte, freeing the mailbox. Up to 16 initiator

CCBs and 16 target CCBs can be stored in the host adapter's internal

RAM concurrently. The AHA-1740/1744 maximizes the SCSI bus utilization

by starting the next available CCB as soon as the bus is free.

Disconnection and reconnection on the bus are automatically taken care

of by the AHA-1740/1744. As the host adapter starts the SCSI

operation, it will first determine if the addressed target/LUN is

busy. If the target/LUN is busy, the command will be placed in the

task queue to be tried again later.

Commands are removed from the queue in the order they were received,

establishing a first in first out (FIFO) command execution order. Of

course, the order of task completion may vary due to the different

amounts of time required to process and complete different commands.

If the target/LUN is not busy, the new command is started at the next

bus free phase. If BUSY status is received by the AHA-1740/1744, the

CCB is placed at the end of the FIFO queue to be restarted later.

If a CHECK CONDITION status is received from the target, the

AHA-1740/1744 will issue a Request Sense command to get the sense

data. The sense data is stored in the CCB after the SCSI command data.

The driver software must reserve the allocated number of bytes at the

end of the CCB to hold the sense data which is returned as result of a

receiving a check status. Automatic Request Sense can be optionally

disabled by jumpers or by using the CCB.

If the MBO Command is to abort a SCSI command, the host adapter first

searches the active and queued CCBs. If the CCB is found, the task is

aborted at the earliest possible moment and an MBI entry is made to

indicate that the CCBs execution was successfully terminated. If the

CCB is not found among the active or queued CCBs, the AHA-1740/1744

completes the command and reports that fact in the MBI Status byte.

The CCB may not be found because it has previously been aborted,

because an intervening reset occurred, or because the CCB was already

finished normally and returned.

The AHA-1740/1744 scans the MBOs in a round-robin fashion. This is to

ensure that all of the mailboxes will be scanned with equal

probability. A host can minimize the AHA-1740/1744 MBO scan time by

using MBOs sequentially.

When executing a START SCSI command, the AHA-1740/1744 does not verify

that the new CCB resides in an unused memory area. Therefore, the host

must not reuse a CCB location until it has been returned through a

Mailbox In entry and the host has examined all the pertinent

information in the CCB. CCB addresses are used by the AHA-1740/1744

as task identifiers.

5.5.2 Execution of Target Mode Operations With a Prepared CCB

Typical target operation is managed in the following way by the host

software. After power-on initialization is complete, the host sends an

Enable Target Mode command to the host adapter to enable Processor

Target mode. After that, the host, from previous configuration

knowledge, generates a pair of CCBs for each likely initiator and for

each LUN supported by the host. One CCB is for outbound data from the

initiator using the SEND command, while the other is for inbound data

from the target to the initiator using the RECEIVE command.

5-28

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

The SEND command CCB defines a data buffer for the expected network-type

command from the initiator. The RECEIVE command CCB defines a set of

data which is known and expected by the initiator, typically a welcome

or configuration-type message packet. The host adapter tests to be

sure that duplicate CCBs are not provided by the host.

As the initiator finishes its initialization procedure, it requests

availability and configuration information using the TEST UNIT READY,

REQUEST SENSE, and INQUIRY commands. Finally, it may choose to address

SEND and RECEIVE commands to the targets it has located to transmit or

request information packets.

When the initiator executes a SEND or a RECEIVE command, the target

accepts the selection on the SCSI bus, accepts the command from the

initiator, and executes the proper data transfer to or from the area

specified by the proper CCB. When all SCSI activity is finished

successfully, the CCB is posted back to the host program by an entry

in the MBI. The host adapter updates the CCB byte count and CDB field

contents to correctly reflect the operation performed. The host is

notified that there is an entry in the MBI by an interrupt from the

host adapter. The host then prepares a new CCB to control the

target's next activity expected from the initiator.

5.5.3 Execution of Target Mode Operations Without a Prepared CCB

Alternatively, the processor-type device may be addressed by an

initiator when no CCB has yet been prepared for use by the host

adapter. If the host adapter command enabling target mode has not yet

been executed, the host adapter will act like any initiator. It will

not accept any selection sequence and any attempts to select it by its

target ID will cause the initiator to detect a selection time out.

If the host adapter has been enabled as a Processor Target by a host

adapter command, any selection to the host adapter's target address

will be accepted. The availability commands and identification

commands (TEST UNIT READY, REQUEST SENSE, and INQUIRY) will be

executed completely and normally. If a SEND or RECEIVE command is

received and if a CCB is not active, either because the host has not

completed its program initialization or because the host has not

finished analyzing the results of previous data transfers and has not

generated a new CCB, the Processor Target will accept the selection,

obtain the IDENTIFY Message Out to determine what Logical Unit has

been selected, input the CDB from the SCSI, and then disconnect. The

CDB will be partially interpreted so that the proper MBI can be

generated to notify the host that a new CCB with a certain address,

direction, and data length must be generated. When the new CCB is

passed to the host adapter by an MBO entry, the CCB is filled up with

the command information, the SCSI is reconnected, and the command is

completed as described above.

5.6 DMA CHANNEL INITIALIZATION (with Host Adapter BIOS Disabled)

The DMA circuitry must be set to a special state to allow the Bus

Master operation of the AHA-1740/1744 to operate correctly. This state

is normally established by the BIOS during initialization so that no

other activity is required. If a DMA channel other than the default

channel is being used or if the DMA channel is manipulated by other

programs, the circuitry must be initialized in the following way

before the AHA-1740/1744 can be used.

5-29

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

For the DMA channel being used, two bytes of data must be written to

the DMA controller port specified to initialize the Host DMA

controller for Bus Master DMA operation. The following table specifies

these values in HEX.

_____________________________________________

DMA Channel     DMA Controller Port      Data

---------------------------------------------

0                 0B                0C

0A                00

5                 D6                C1

D4                01

6                 D6                C2

D4                02

7                 D6                C3

D4                03

______________________________________________

5.7 INTERRUPT INITIALIZATION

This setup procedure is normally completed by the BIOS during

initialization so that no other activity is required. If modifications

to the interrupt handler are required, this information together with

the programming information provided by the host system should be

sufficient to properly set up the interrupt vectors.

The host adapter will drive one of several interrupts in the AT

system. The particular interrupt used must be set up on power-up

initialization and be properly managed during usage. A summary of the

AT interrupts of interest to the host adapter driver along with their

corresponding vector locations follow. All of these interrupts are

handled by a slave interrupt controller. The master controller handles

all system interrupts such as keyboard, timer, etc. and is assumed to

be correctly initialized to allow interrupts by the slave controller.

Upon receiving an interrupt, the processor will be vectored to the

contents of the corresponding vector location.

Hardware     Software interrupt

interrupt    vector location (hex)

int 9    -     INT71

int 10   -     INT72

int 11   -     INT73

int 12   -     INT74

int 13   -     INT76

int 14   -     INT77

int 15   -     INT78

The interrupt is initialized by clearing the corresponding interrupt

mask bit in the slave controller. The mask register is a read/write

register, and only the bit of interest should be cleared. The port

address is A1 hex, and bit definitions follow.

5-30

Section 5                                   Standard Mode Firmware Description

------------------------------------------------------------------------------

Interrupt mask bit definition (port address A1 hex)

bit 0 --  int 8

bit 1 --  int 9

bit 2 --  int 10

bit 3 --  int 11

bit 4 --  int 12

bit 5 --  int 13

bit 6 --  int 14

bit 7 --  int 15

5-30

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

The Enhanced mode is an interface architecture which allows the

AHA-1740/1744 to take full advantage of the EISA bus facilities. It

utilizes a distinct hardware interface control logic, known as the

Configuration chip. This permits features such as full 32-bit

addressing and the entire EISA register set to be used. This mode is

not compatible with older versions of the ASW-1400 series of software

managers. In order to use the AHA-1740 mode , the AHA-1740/ 1744 must

be correctly reconfigured by downloading the appropriate microcode

from the host. The AHA-1740 is not capable of simultaneous operation

in Standard and Enhanced mode.

Operations using the Enhanced Mode utilize a mailbox interface which

is substantially different from the Standard Mode. The principal

difference is the use of a single mailbox structure which is serviced

at a rate that prevents overwriting. The mailbox structure is

described in detail in Section 4.4. The components of the mailbox are:

XCDO  _  Mailbox Out Pointer

XCD1  _  Mailbox Out Pointer

XCD2  _  Mailbox Out Pointer

XCD3  _  Mailbox Out Pointer

XCD4  _  Attention Register

XCD5  _  Enhanced Mode Control Register

XCD6  _  Interrupt Status Register

XCD7  _  Enhanced Mode Status Register

XCD8  _  Mailbox In Pointer

XCD9  _  Mailbox In Pointer

XCDA  _  Mailbox In Pointer

XCDB  _  Mailbox In Pointer

Operations using the mailbox begins with the host assembling an

Enhanced Control Block (ECB) in memory. The host then stores the

pointer to the ECB in the Mailbox Out, provided that bit 2 of the SCD7

Status register (MBO empty) is true. Due to the speed with which the

AHA-1740/1744 services the mailbox, the MBO will almost always be

empty.

The host then checks the busy bit (Bit 0) in XCD7 to ensure that the

AHA-1740/1744 has serviced any previous requests placed in the

Attention Register. If the bit is false, which will almost always be

the case, the host writes a byte into the attention register with the

format

_____________________

|Command  |  SCSI ID|

--------------------

The command will be either a Start command (4), Immediate command (1),

or Abort (5). Writing to this register causes an internal interrupt to

the host adapter, which will then examine the command and the SCSI ID

for which it is destined and then use the MBO to

6-1

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

access the ECB associated with the command. Both MBO Empty and Busy

are cleared in XCD7 to allow other commands to be sent.

While the board is busy executing the command, the host may monitor

XCDG Interrupt Status register to look for completion of the command.

When any command completes, this register will be loaded with

|-------------------------|

|  Completion |           |

|  Code       |   SCSI ID |

|-------------------------|

and the MBI (XCD8-B) loaded with pointers to any host adapter data in

the ECB area of memory. At the same time, the Interrupt Pending bit is

set in XCD7. This is reset when the host writes the Clear EISA

Interrupt bit in the Control Register XCD5.

Multitasking is achieved by setting up multiple commands through the

mailbox before the first has been completed. The microcode maintains

control of up to 64 commands executing simultaneously in the host adapter.

6.1 CONTROL BLOCK

The Control Block is a 48-byte structure created and maintained in

shared memory by software in the system unit. It is used to convey

requests to the host adapter.

|------------------------------------------------------------|

|  F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0 Byte Offset|

|------------------------------------------------------------|

| Command Word                                             00|

|------------------------------------------------------------|

| Flag Word 1                                              02|

|------------------------------------------------------------|

| Flag Word 2                                              04|

|------------------------------------------------------------|

|                                                          06|

|------------------------------------------------------------|

| Data or Scatter/Gather List Pointer                      08|

|                                                          0A|

|------------------------------------------------------------|

| Data or Scatter/Gather List Length                       0C|

|                                                          0E|

|------------------------------------------------------------|

| Status Block Pointer                                     10|

|                                                          12|

|------------------------------------------------------------|

| Chain Address                                            14|

|                                                          16|

|------------------------------------------------------------|

|                                                          18|

|------------------------------------------------------------|

|                                                          1A|

|------------------------------------------------------------|

| Sense Information Pointer                                1C|

|                                                          1E|

|------------------------------------------------------------|

| CDB Length                 Sense Length                  20|

|------------------------------------------------------------|

| Data Checksum                                            22|

|------------------------------------------------------------|

| CDB Byte 1                CDB Byte 0                     24|

|------------------------------------------------------------|

|  CDB Byte 3                CDB Byte 2                    26|

|------------------------------------------------------------|

|  CDB Byte 5                CDB Byte 4                    28|

|------------------------------------------------------------|

|  CDB Byte 7                CDB Byte 6                    2A|

|------------------------------------------------------------|

|  CDB Byte 9                CDB Byte 8                    2C|

|------------------------------------------------------------|

|  CDB Byte 11               CDB Byte 10                   2E|

|------------------------------------------------------------|

6-2

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

6.1.1 Command Word

The Command Word is a 16-bit structured field used to identify the

operation to be performed by the host adapter.

Command Word Structure

F E D C B A 9 8 7 6 5 4 3 2 1 0

____________________________________________

I -  I Options I -  - - - I Operations Code I

--------------------------------------------

Operation Code

00   No Operation

01   Initiator SCSI Command

05   Run Diagnostic Test

06   Initialize SCSI Subsystem

08   Read Sense Information

09   Download Firmware

0A   Read Host Adapter Inquiry Data

10   Target SCSI Command

Option Flags

The Option Flags (bits 14-12) are used to modify the meaning of the

Operation Code and/or the control block fields associated with the

Operation Code. The meaning of these flags are dependent upon the

Operation Code that uses them.

6.1.2 Flag Word 1

Flag Word 1 is a 16-bit structured field used to specify execution

parameters for this control block.

F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0

-------------------------------------------------

A  D     S     S        D                    C

R  S     /     E        I                    N

S  B     G     S                             E

Fields that are empty are reserved and should be set to zero.

Flag Name and Function

CNE  Chain No Error

The Chain No Error flag (bit 0) indicates whether control

block chaining is enabled.

If this bit is set to one, chaining is enabled. Upon

successful completion of the control block, the host adapter

will fetch the next control pointed to by the Chain Address.

In the event of an error, chaining is halted.

When set to zero, chaining is disabled and the Chain Address

field in the control block is ignored by the host adapter.

6-3

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Note: This option is available only for Initiator SCSI and No

Operation commands. For all other commands, setting this flag to

one will result in a Specification Check.

DI  Disable Interrupt

The Disable Interrupt flag (bit 7) indicates whether the

host is to be interrupted when the control block is completed.

If this bit is set to one, the host is NOT interrupted upon

successful completion of the control block. In the event of

an error, the host adapter ignores this bit and interrupts

the host.

If this bit is set to zero, the host is interrupted when the

control block is completed.

Note: The use of this flag is recommended only when control

block chaining is enabled.

SES  Suppress Error on Underrun

This flag (bit 10) indicates whether the host adapter should

consider a data underrun as an error. A data underrun occurs when

the amount transferred is less than the amount specified by the

Data Length field in the control block or Scatter/Gather list.

If this bit is set to one, the host adapter will not report a

data underrun as an error. Information relating to the data

underrun (i.e., residual byte count) will still be stored in the

Status Block if storing of the Status Block is enabled (bit 14

set to zero).

If this bit is set to zero, the host adapter will treat the Data

underrun as an error condition. Information relating to the data

underrun will be stored in the Status Block, control block

chaining (if enabled) will be halted, and the host will be

interrupted.

S/G Scatter/Gather

This flag (bit 12) indicates whether Scatter/Gather should

be implemented for this control block.

If this bit is set to one, then the Data Pointer field

contains the address of a list of data pointers and lengths.

The Data Length field contains the length of the list in

bytes.

If this bit is set to zero, then the Data Length field

contains the number of bytes to be transferred to or from

memory starting at the address in the Data Pointer field.

Note: This option is available only for Initiator and Target

SCSI commands. For all other commands, setting this flag to one

will result in a Specification Check.

6-4

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

DSB  Disable Status Block

The Disable Status Block flag (bit 14) indicates whether the

Status Block is to be stored.

If this bit is set to one, then the host adapter does not

store the Status Block if the control block is completed

without error. In the event of an error, the Status Block

will be stored.

If this bit is set to zero, then the Status Block is stored

unconditionally upon completion of the control block.

Note: This flag should be set to one for most operations.

ARS Automatic Request Sense

This flag (bit 15) indicates whether the host adapter is to

automatically issue a Request Sense command to the target if

the target responds to the control block operation with a

Check Condition.

If this bit is set to one, then the host adapter will

automatically issue a Request Sense command to the target

immediately following a Check Condition (Target Status =

02), and prior to interrupting the host. The number of bytes

of sense information the host adapter requests is specified

in the Sense Length field. The host adapter stores the bytes

to the memory address pointed to by the Sense Information

Pointer.

If this bit is set to zero, then the host adapter ignores

the Sense Length and Sense Information Pointer fields.

Note: This option is available only for Initiator SCSI commands.

This flag will be ignored for Target SCSI commands. For all other

commands, setting this flag to one will result in a Specification Check.

6.1.3 Flag Word 2

Flag Word 2 is a 16-bit structured field used to specify execution

parameters for this control block.

F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0

----------------------------------------------

R        C  S  D  D     N  T  T  T  L  L  L

E        H  T  I  A     D  T  T  A  U  U  U

C        K     R  T              G  N  N  N

Fields that are empty are reserved and should be set to zero.

6-5

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Flag Name and Function

LUN  Logical Unit Number

This field (bits 2-0) specify the Logical Unit Number of the

target for which the command is to be executed.

TAG  Tagged Queuing

This flag (bit 3) indicates whether tagged queuing should be

implemented for this control block.

If this bit is set to one, the host adapter will send a

2-byte Tag Queue message to the target immediately following

the Identify message. The first message byte contains the

Tag Code and specifies the type of ordering that is desired

for this command. The contents of this byte is determined by

the Tag Type flags (bits 4 and 5). Table 1 gives a

description of the Tag Queuing and Tag Type flags. The

second message byte is a unique Tag ID assigned by the host adapter.

If this bit is set to zero, tagged queuing is not

implemented for this control block.

TT  Tag Type

These flags (bits 4 and 5) are used by the host adapter to

determine which Tag Code (Tag Queue message byte 1) to send

to the target. These flags are ignored by the host adapter

if the Tag Queuing flag (bit 3) is set to zero. Table 6-1

gives a description of the Tag Queuing and Tag Type flags.

Table 6-1. Description of Tag Queuing and Tag Type Flags

--------------------------------------------------------------------

TT       tag   Description of Tag Queuing option flags

--------------------------------------------------------------------

X     X     0   Tagged Queuing not implemented for this command.

0     0     1   A Simple Queue Tag (Tag Code = 20h) followed by a

unique Tag ID will be sent to the target

immediately after the Identify message.

0     1     1   A Head of Queue Tag (Tag Code = 21h) followed by

a unique Tag ID will be sent to the target

immediately after the Identify message.

1     0     1   An Ordered Queue Tag (Tag Code = 22h) followed by

a unique Tag ID will be sent to the target

immediately after the Identify message.

1     1     1   INVALID. Specification Error.

-----------------------------------------------------------------------

Note: X means "don't care"

6-6

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

ND  No Disconnect

The setting of this flag (bit 6) prevents the target from

disconnecting during command execution.

If this bit is set to one, the host adapter overrides the

target's configuration setting and instructs the target,

via the Identify message, not to disconnect during command

execution.

If this bit is set to zero, the host adapter uses the

target's configuration setting to determine whether

disconnection should be allowed.

DAT Data Transfer - Check Direction

This flag (bit 8) determines whether the host adapter should

verify the direction of the expected data phase for this

command.

If this bit is set to one, the host adapter expects the

target to enter a data phase during command operation. The

host adapter will check the Direction flag to determine the

direction of the expected data phase (Data In or Data Out).

It will also check the Suppress Transfer flag to determine

whether to suppress the data transfer. Table 6-2 shows the

relationship between this flag and the Direction and

Suppress Transfer flags. Although a data phase is expected,

no Invalid Phase error will be reported if the target does

not enter a data phase. (This situation, however, does not

preclude a Data Underrun error.)

If this bit is set to zero, the host adapter ignores the

Direction and Suppress Transfer flags. If a data phase

occurs, the host adapter will not check the direction and

proceed with the transfer. This bit should be set to zero if

no data transfer is expected.

Note: Although not necessary, this bit should be set to one for

commands that access the target's media (i.e., Read, Write, Write

with Verify).

DIR Direction of Transfer

This flag (bit 9) indicates the expected direction of the

data transfer. This flag will be ignored if the Data

Transfer flag (bit 8) is set to zero. Table 6-2 shows the

relationship between this and the other data option flags.

If this bit is set to one, the host adapter expects the

target to enter Data In phase during the command (Read

operation). If a Data Out phase is detected, the host

adapter will abort the command and report an Invalid Phase

Error. The Invalid Phase Error will not be reported if no

data phase is detected.

If this bit is set to zero, the host adapter expects the

target to enter Data Out phase during the command (Write

operation). If a Data In phase is detected, the host adapter

will abort the command and report an Invalid Phase Error.

The Invalid Phase Error will not be reported if no data

phase is detected.

6-7

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

ST  Suppress Transfer to Host Memory

This flag (bit 10) indicates whether an incoming data

transfer is to be suppressed. This flag will be ignored if

the Data Transfer flag (bit 8) is set to zero. Table 6-2

shows the relationship between this flag and the other

direction flags.

If this bit is set to one, the host adapter will accept data

from the target, but will not transfer it to host memory.

If this bit is set to zero, data transfer is not suppressed.

Table 6-2. Relationship between Data Transfer Option flags.

------------------------------------------------------------------------------

st    dir   dat   Description of data transfer option flags

------------------------------------------------------------------------------

X     X     0    No data transfer expected or direction of

transfer not checked. If a data phase is

detected, the direction is determined by the

phase and not the Direction flag.

0     0     1    Data Out phase expected. If Data In phase is

detected, transfer will be suppressed and Invalid

Phase Error reported. Invalid Phase Error is not

reported if no data phase is detected.

0     1     1    Data In phase expected. If Data Out phase is

detected, transfer will be suppressed and Invalid

Phase Error reported. Invalid Phase Error is not

reported if no data phase is detected.

1     0     1    INVALID. Specification Error.

1     1     1    Data In phase expected and suppress transfer.

Same as above except data received from target is

not transferred to host memory. (See Note.)

------------------------------------------------------------------------------

Note: X means "don't care".

Note: Setting the Suppress Transfer bit differs from setting the

Data Length field to 0. If the Data Length equals 0 and a data

phase is detected, the host adapter will automatically suppress data

transfer and report a Data Overrun Error. If the Suppress Transfer

bit is set, the host adapter will accept data from the target but

not transfer it to host memory. The amount of data received will be

compared to the value stored in the Data Length field.

CHK Calculate Checksum on Data

This flag (bit 11) is used to verify the integrity of the

data being transferred.

If this bit is set to one, the host adapter performs a

checksum of the data as it is transferred. The checksum

algorithm is a simple 2-byte sum of the data. If the

checksum is not equal to the value supplied in the Checksum

field in the control block, an error will be reported.

If this bit is set to zero, no checksum will be performed on

the data and the Checksum field in the control block will be

ignored by the host adapter.

6-8

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Note: This option is currently available only for the Download

Firmware command. For all other commands, setting this flag to

one will result in a Specification Check.

REC Error Recovery

This flag (bit 14) is used by the host to perform error

recovery procedures while the associated target is in a

state of Extended Contingent Allegiance. A state of Extended

Contingent Allegiance exists as long as the host adapter

returns this status (Status Word 1, bit 14) in the Status

Block. The condition is cleared by issuing a Resume

immediate command.

If this bit is set to one, and the target is in a state of

Extended Contingent Allegiance, the host adapter will

execute the command. If the target is not in a state of

Extended Contingent Allegiance, the host adapter will reject

the control block and report a Specification Check. The

control blocks will also be rejected with a Specification

Check if the Tagged Queuing flag (bit 3) is set.

If this bit is set to zero, and the target is in a state of

Extended Contingent Allegiance, the host adapter will accept

the command but not execute it until the condition is

cleared. If the target is not in a state of Extended

Contingent Allegiance, the control block will be executed

normally.

Note: This option has been defined for future versions of SCSI-2

subsystems. For current versions, this flag should always be set

to zero.

6.1.4 Data or Scatter/Gather List Pointer

This 32-bit field contains the physical memory address (least

significant byte first) of the first byte of the host memory data area

to be used during the data phase of the SCSI command.

If the Scatter/Gather Flag (Flag Word 1, bit 12) is set to one, this

field contains the physical address of the first byte of list of data

pointers and lengths to be used during the data phase of the SCSI

command. The number of 8-byte segments is limited to 128 (one pointer

and one length), but the host adapter will only store a maximum of 17

segments in its internal memory at a time. The diagram below describes

the structure of the Scatter/Gather list.

_____________________________________________________

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0  |

|___________________________________________________|

|   Data Pointer 1                                  |

|___________________________________________________|

|  Data Length 1                                    |

|___________________________________________________|

|  Data Pointer 2                                   |

|___________________________________________________|

|  Data Length 2                                    |

|   .                                               |

|   ,                                               |

|   .                                               |

|                                                   |

|___________________________________________________|

|  Data Pointer n                                   |

|___________________________________________________|

|  Data Length n                                    |

|___________________________________________________|

6-9

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

The data lengths must be less than 4 MBytes and the total length of

the transfer must be less than 16 MBytes.

6.1.5 Data or Scatter/Gather List Length

This 32-bit field contains the length in bytes (least significant byte

first) of the data transfer. The host should always store a value in

this field (even if data transfer is to be suppressed), if a data

phase is expected during the SCSI command. Otherwise, a Data Overrun

error will occur. If no data transfer is expected, this field should

be set to 0. The data lengths must be less than 4 MBytes.

If the Scatter/Gather Flag (Flag Word 1, bit 12) is set to one, this

field contains the length in bytes of the Scatter/Gather list. This

value must be nonzero and a multiple of eight (8), otherwise, a

specification error will be reported by the host adapter. The length

of the Scatter/Gather list is limited to 1K.

6.1.6 Status Block Pointer

This 32-bit field contains the physical memory address (least

significant byte first) of the Status Block. The Status Block is

written to by the host adapter and used by the host to determine the

completion status of a control block operation.

The length of the Status Block is always 32 bytes, though the number

of bytes stored by the host adapter may be less.

This field is required for all control blocks. However, the host

adapter will bypass writing to the Status Block if the control block

completes without error and the Disable Status Block flag (Flag Word

1, bit 14) is set to one.

6.1.7 Chain Address

This 32-bit field contains the physical memory address (least

significant byte first) of the next control block to be processed by

the host adapter.

This field is optional and used only when the Chain No Error flag

(Flag Word 1, bit 0) is set to one. Control block chaining will only

occur when the current control block operation completes without error.

6.1.8 Sense Information Pointer

This 32-bit field contains the physical memory address (least

significant byte first) of the first byte of the host memory data area

where the target's Sense Information is to be stored.

This field, along with Sense length, must be supported if the

Automatic Request Sense flag (Flag Word 1, bit 15) is set to one, or

if the Operation Code in the Command Word specifies a Read Sense

Information command. The number of bytes transferred will not exceed

the value specified in the Sense Length field.

6-10

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

6.1.9 Sense Length

This 8-bit field specifies the maximum number of Sense Information

bytes the host adapter will transfer. The actual number of bytes

transferred may be less. The supported values are from 0 to 256 bytes.

If a value of zero is stored in this field, no bytes will be

transferred and the Sense Information will be lost. The actual number

of bytes transferred will be stored in the Status Block (byte 14).

6.1.10 CDB Length

This 8-bit field specifies the length in bytes of the SCSI CDB to be

sent to the target. The supported values for this field is six, 10 and

12 bytes.

6.1.11 Checksum of Data

This 16-bit field contains the checksum (least significant byte first)

of the data to be transferred by the host adapter.

This field is optional and used only when the Calculate Checksum on

Data flag (Flag Word 2, bit 11) is set to one. The host adapter will

transfer the number of bytes specified in the Data Length field and

add the bytes together resulting in a 16-bit checksum. The host

adapter will then verify the result with the value specified in this

field. If the two values do not compare, an error will be reported.

Note: This option is currently available only for the Download

Firmware command.

6.1.12 SCSI Command Descriptor Block

This 6-, 10-, or 12-byte field contains the actual CDB to be sent to

the target during Command phase.

6.2 STATUS BLOCK

The Status Block is a 32-byte structure created in shared memory by

software in the system unit. Information is placed into this structure

by the host adapter to report completion status of a control block.

6-11

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

|-------------------------------------------------------------|

|                                               Byte          |

|F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0 Offset        |

|-------------------------------------------------------------|

|   Status Word                                          02   |

|-------------------------------------------------------------|

|   Target Status             I   HA Status              02   |

|-------------------------------------------------------------|

|   Residual Byte Count                                  04   |

|                                                        06   |

|-------------------------------------------------------------|

|   Residual Buffer Address                              08   |

|                                                        0A   |

|-------------------------------------------------------------|

|   Additional Status Length                             0C   |

|-------------------------------------------------------------|

|                             I   Sense Length           0E   |

|-------------------------------------------------------------|

|                                                        10   |

|-------------------------------------------------------------|

|                                                        12   |

|-------------------------------------------------------------|

|                                                        14   |

|-------------------------------------------------------------|

|                                                        16   |

|-------------------------------------------------------------|

|    CDB Byte 1               I   CDB Byte 0             1A   |

|-------------------------------------------------------------|

|    CDB Byte 3               I   CDB Byte 2             1C   |

|-------------------------------------------------------------|

|    CDB Byte 5               I   CDB Byte 4             1E   |

|-------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

6.2.1 Status Word

The Status Word is a 16-bit structured field used to hold flags

indicating how the command for the associated control block is

completed.

F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0

----------------------------------------------

E     M  I     S  A  I  C  D  C  Q     D  D

C     E  N     N  S  N  H  O  C  F     U  O

A        I     S  A  T                    N

Fields that are empty are reserved and will be set to zero.

Flag Name and Function

DON  Command Done -_ No Error

The Command Done flag (bit 0) indicates whether the control

block completed successfully.

If this bit is one, the control block completed without error.

If this bit is zero, the other flags in this word as well as

the Host Adapter and Target Status bytes should be analyzed

to determine the completion status of the control block.

Note: When this flag is zero, the Status Block is stored

unconditionally by the host adapter and an interrupt is

generated, regardless of the settings of the Disable Interrupt

and Disable Status Block flags (Flag Word 1, bits 7 and 14, respectively).

6-12

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

DU Data Underrun

The Data Underrun flag (bit 1) indicates whether the amount

of data transferred was less than the amount specified by

the Data Length field in the control block.

If this bit is one, a data underrun occurred. The Residual

Byte Count and Residual Buffer Address fields are valid. If

the Suppress Error on Underrun flag (Flag Word 1, bit 10)

was set to one, then the Command Done flag will also be set

to one, indicating that the underrun condition was not

considered an error. If the Suppress Error on Underrun flag

was set to zero, then the Command Done flag will be set to

zero and the Major Error/Exception flag will be set to one.

If this bit is zero, no underrun condition was detected.

QF Host Adapter Queue Full

The Queue Full flag (bit 3) indicates that the host adapter

was not able to accept the control block.

If this bit is set to one, the host adapter has rejected the

control block because its internal storage area is full. No

action is taken on the control block. The host should

reissue the control block after an outstanding control block

completes or after an appropriate time delay.

SC   Specification Check

The Specification Check flag (bit 4) indicates that the host

adapter detected an error in the control block.

If this bit is set to one, the host adapter has rejected the

control block due to an error in one of the control block's

fields (i.e., unsupported Operation Code). No action is

taken on the control block. The Host Adapter Status byte

contains additional information regarding the error.

DO Data Overrun

The Data Overrun flag (bit 5) indicates whether the target

attempted to transfer more data than was specified by the

Data Length field in the control block.

If this bit is one, a data overrun occurred. The Residual

Byte Count and Residual Buffer Address fields are valid, and

the Major Error/Exception flag will be set to one.

If this bit is zero, no overrun condition was detected.

CH Chaining Halted

The Chaining Halted flag (bit 6) indicates that a chain of

control blocks was terminated prematurely due to an error.

This bit is set to one when the Command Done flag is zero

(control block terminated with error) and the Chain No Error

flag (Flag Word 1, bit 0) is one.

6-13

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

INT Interrupt Issued for SCB

The Interrupt flag (bit 7) indicates whether an interrupt

has been issued for the associated control block.

This bit is always set to one if the control block

terminated with an error (Command Done flag is zero) or if

the target is in a state of Extended Contingent Allegiance

(Extended Contingent Allegiance flag is one). This bit is

also set to one if the control block completed without error

(Command Done flag is one) and the Disable interrupt flag

(Flag Word 1, bit 7) is set to zero.

If this bit is set to zero, no interrupt was issued for the

associated control block.

ASA Additional Status Available

This flag (bit 9) is used to indicate that additional status

in bytes 14 through 31 are available.

If this bit is set to one, then the Additional Status Length

field contains the number of additional status bytes that

are available, starting at byte 14 of the Status Block.

If this bit is set to zero, then additional status is not

available and the Additional Status Length field is invalid.

SNS Sense Information Stored

The Sense Information Stored flag (bit 9) indicates whether

the target's Sense Information has been stored in host

memory pointed to by the address in the Sense Information

Pointer field.

If this bit is set to one, the Sense Information data area

for the associated control block contains the target's Sense

data. The Additional Status Length field will contain a

value of at least one, and the Sense Length field will

contain the actual number of Sense Information bytes

transferred.

If this bit is set to zero, no data has been stored in the

Sense Information data area.

INI Initialization Required

This flag (bit 11) indicates that the host adapter needs to

be initialized by the host before it can perform any normal

control block operation.

If this bit is set to one, the host adapter has rejected the

control block because some sort of initialization is needed

to complete the command; either firmware needs to be

downloaded or configuration data needs to be supplied to the

host adapter. The Host Adapter Status byte contains

additional information regarding what type of initialization

is required.

6-14

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

ME  Major Error or Exception Occurred

This flag (bit 12) indicates whether a major error or

exception occurred on the SCSI subsystem. A major error is

one that prevents the host adapter from initiating or

completing a control block operation. For instance, a host

adapter hardware failure, a selection timeout, or an invalid

bus phase change would be considered major errors. A major

exception is one that changes the configuration of the

subsystem, such as a SCSI bus reset.

If this bit is set to one, the host adapter has detected a

major error or exception. The Host Adapter Status byte

contains additional information regarding the error.

If this bit is set to zero, the implication is that any

error associated with this control block does not impact the

host adapter's ability to communicate with the target.

Note: A target returning a Check Condition Status (Target Status

= 02) is generally not considered a Major Error or Exception.

ECA  Extended Contingent Allegiance

This flag (bit 14) indicates whether the associated target

is in a state of Extended Contingent Allegiance.

If this bit is set to one, all outstanding operations for

the associated target queued in the host adapter and/or the

target is suspended. While the Extended Contingent

Allegiance condition exists, any control blocks issued for

the associated target that have the Recovery flag (Flag

Word 2, bit 14) set to one will be executed. While the

Extended Contingent Allegiance condition exists, any control

blocks issued for the associated target that do not have the

Recovery flag set to one will be accepted and queued by the

host adapter, but not executed until the condition is

cleared. The condition is cleared by issuing a Resume

command for the associated target. It is also cleared by

resetting the host adapter or by issuing a Bus Device Reset

to the associated target.

Note: This flag is set to one for every control block completed

while the Extended Contingent Allegiance condition exists.

6.2.2 Host Adapter Status

This 8-bit field contains additional error information pertaining to

the host adapter or SCSI subsystem in general. This field is valid

when the Command Done flag is set to zero and one of the following

flags is set to one: Specification Check, Initialization Required, or

Major Error/Exception.

6-15

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Code    Description                           Associated Status Flag

------------------------------------------------------------------------------

00      No Host Adapter Status Available

04      Command Aborted by Host                     Major Error/Exception

05      Command Aborted by Host Adapter             Major Error/Exception

08      Firmware Not Downloaded                     Initialization Required

0A      Target Not Assigned to SCSI Subsystem       Specification Check

11      Selection Timeout                           Major Error/Exception

12      Data Overrun or Underrun Occurred           Major Error/Exception

13      Unexpected Bus Free Occurred                Major Error/Exception

14      Invalid Bus Phase Detected (SCSI Reset)     Major Error/Exception

16      Invalid Operation Code                      Specification Check

17      Invalid SCSI Linking Operation              Major Error/Exception

18      Invalid Control Block Parameter             Specification Check

19      Duplicate Target Control Block Received     Specification Check

1A      Invalid Scatter/Gather List                 Specification Check

1B      Request Sense Command Failed                Major Error/Exception

1C      Tagged Queuing Message Rejected by Target   Major Error/Exception

20      Host Adapter Hardware Error                 Major Error/Exception

21      Target Did Not Respond to Attn (SCSI Reset) Major Error/Exception

22      SCSI Bus Reset by Host Adapter              Major Error/Exception

23      SCSI Bus Reset by Other Device              Major Error/Exception

80      Program Checksum Failure                    Major Error/Exception

------------------------------------------------------------------------------

6.2.3 Target Status

This 8-bit field contains the data byte returned by the target during

Status Phase. If the command operation terminated before the target

was able to transfer a status byte, this field will be set to zero and

the Host Adapter Status will be nonzero.

------------------------------------------------------------------------------

Code    Description

------------------------------------------------------------------------------

00      Good Status or No Target Status Available

02      Check Condition

08      Target Busy

04      Condition Met (For Search Data and Pre-Fetch SCSI Commands)

10      Intermediate (For Linked SCSI Commands)

14      Intermediate Condition Met

18      Reservation Conflict

------------------------------------------------------------------------------

When this field contains 02 (Check Condition) and the Sense

Information Stored flag (Status Word, bit 9) is set to zero, a state

of Contingent Allegiance exists for the target. All outstanding

operations for the associated target queued in the host adapter and/or

the target is suspended. Any control blocks for the associated target

received after Contingent Allegiance occurred and before the Read

Sense Information command will be accepted and queued by the host

adapter, but not executed until the condition is cleared. The

Contingent Allegiance condition is cleared by sending a Read Sense

Information control block for the associated target. It is also

cleared by resetting the host adapter or by issuing a Bus Device Reset

to the associated target.

6-16

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

If the target returns Busy Status (Code = 08h), the host adapter will

retry the command indefinitely. This status will be reported only when

the command was aborted by the host while the host adapter was

attempting retries.

If the target returns Queue Full Status (Code = 28h), the host adapter

will queue the command internally, therefore, this status will never

be reported to the host. If the host adapter's internal memory is

full, the control block will be rejected with Host Adapter Queue Full

status (Status Word, bit 3).

6.2.4 Residual Byte Count

The Residual Byte Count is a 32-bit field (least significant byte

first) that contains the number of bytes that remain to be written

from, or read into, the host memory buffer most recently used to

transfer data. This field is valid only if the Data Underrun or Data

Overrun flags (Status Word, bits 1 and 5, respectively) are set to

one. If the Data Overrun flag is set to one, this field will be set to zero.

If Scatter/Gather was enabled for the associated control block, then

this field contains the number of remaining bytes for the most

recently used segment in the Scatter/Gather list.

If Scatter/Gather was not enabled for the associated control block,

then this field contains the number of remaining bytes for the entire

transfer count specified in the Data Length field of the control

block.

6.2.5 Residual Buffer Address

The Residual Buffer Address is a 32-bit field (least significant byte

first) that contains the physical host memory address identifying the

buffer last read from or written to. This field is valid only if the

Data Underrun or Data Overrun flag (Status Word, bits 1 and 5,

respectively) is set to one.

If Scatter/Gather was enabled for the associated control block, then

this field contains the address of the Data Pointer in the

Scatter/Gather list that was most recently used to transfer data.

If Scatter/Gather was not enabled for the associated control block,

then this field contains the address specified in the Data Pointer

field of the control block.

6.2.6 Additional Status Length

This 16-bit field specifies the number of additional bytes of status

that is available. This field is valid if the Additional Status

Available flag (Status Word, bit 8) is set to one. The value of this

field will range from 0 to 18 (decimal) bytes. Byte 14 supplies the

number of bytes transferred during a Request Sense command. Bytes 15

through 23 of the Status Block are reserved for future use. Bytes 24

through 31 supply information pertaining to a target mode command

(either Receive or Send) received by the host adapter from another

SCSI initiator.

6-17

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

If this field is zero, then no additional status is available and the

the values in bytes 14 through 31 are invalid.

6.2.7 Sense Length

This 8-bit field contains the number of Sense Information bytes

actually transferred. This number can be used to determine which bytes

in the Sense Information data area are valid.

This field is valid only when the Sense Information Stored flag

(Status Word, bit 9) is set to one.

6.2.8 Target Mode CDB

These six bytes are stored by the host adapter following the

completion of a Target SCSI Command control block. They provide to the

host the actual SCSI Command Descriptor Block received by the host

adapter from another initiator device on the SCSI bus.

Byte 26 contains the command opcode (08 = Receive, 0A = Send). Bytes

28 through 30 contain the number of bytes (most significant byte

first) the initiator wanted to be transferred.

6.3 CONTROL BLOCK COMMANDS

Control Block Commands utilize a fully multitasking delivery

interface. The address of the control block is stored into the Mailbox

Out registers, least significant byte first. The Attention Register is

then stored with the following value:

7 6 5 4    3 2 1 0

-------

0 1 0 0    SCSI ID

The SCSI ID specifies either the SCSI address of a particular target

or initiator, or the SCSI address of the host adapter. The host

adapter's ID can be found in bits 3-0 of the SCSI Definition Register.

Multiple control blocks or chains of control blocks can be issued for

a single SCSI ID.

When the Attention Register is written to, the host adapter is

interrupted. When the command is completed, the host adapter (if

enabled) will interrupt the host by writing to the Enhanced Mode

(Group 2) Interrupt Status Register. The Mailbox In Registers will

contain the address of the completed control block.

Following are the Control Block Commands described in alphabetical order.

6-18

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

6.3.1 Download Firmware

|-----------------------------------------------------------------------------|

|                                                      Byte                   |

|    F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0    Offset                 |

|-----------------------------------------------------------------------------|

|    0  Options  0  0  0  0  0  0  0  0  1  0  0  1      00                   |

|-----------------------------------------------------------------------------|

|   Flag Word 1                                          02                   |

|-----------------------------------------------------------------------------|

|   Flag Word 2                                          04                   |

|-----------------------------------------------------------------------------|

|                                                        06                   |

|-----------------------------------------------------------------------------|

|   Pointer to firmware                                  08                   |

|                                                        0A                   |

|-----------------------------------------------------------------------------|

|    Length of firmware                                  0C                   |

|                                                        0E                   |

|-----------------------------------------------------------------------------|

|   Status Block Pointer                                 10                   |

|                                                        12                   |

|-----------------------------------------------------------------------------|

|                                                        14                   |

|-----------------------------------------------------------------------------|

|                                                        16                   |

|-----------------------------------------------------------------------------|

|                                                        18                   |

|-----------------------------------------------------------------------------|

|                                                        1A                   |

|-----------------------------------------------------------------------------|

|                                                        1C                   |

|-----------------------------------------------------------------------------|

|                                                        1E                   |

|-----------------------------------------------------------------------------|

|                                                        20                   |

|-----------------------------------------------------------------------------|

|   Checksum of Data                                     22                   |

|-----------------------------------------------------------------------------|

|                                                        24                   |

|-----------------------------------------------------------------------------|

|                                                        26                   |

|-----------------------------------------------------------------------------|

|                                                        28                   |

|-----------------------------------------------------------------------------|

|                                                        2A                   |

|-----------------------------------------------------------------------------|

|                                                        2C                   |

|-----------------------------------------------------------------------------|

|                                                        2E                   |

|-----------------------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

This command causes the host adapter to overwrite its program memory

with data supplied by the host. The host adapter will, if enabled,

perform a checksum on the data after it is written to its internal

RAM, and compare the checksum to the value stored in the control

block. If the two values miscompare, the host adapter will report an error and

will reject all subsequent commands except a Download Firmware command.

After a hard reset, the host adapter will default to the firmware

stored in its EEPROM.

If the CONFIGURE Flag (PORTADDR Register, bit 6) is set to one, the

host adapter will reject all commands (control block and immediate)

other than a Download Firmware command. After receiving this command,

the host adapter will download the firmware into its internal RAM as

described above. After successfully overwriting its program memory,

the host adapter will then reprogram its EEPROM with the new firmware

and perform a checksum on the new data in the EEPROM. If this checksum

fails, the host adapter will report an error.

Supported Options

Flag Word 1 - Disable Interrupt (bit 7) and Disable Status Block (bit

14) are supported. All other flags should be zero.

6-19

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Flag Word 2 - The Calculate Checksum option (bit 11) should be used.

All other flags should be zero.

6.3.2 Initialize SCSI Subsystem

|------------------------------------------------------------------|

|                                                        Byte      |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0       Offset    |

|------------------------------------------------------------------|

|   0  0  0  0  0  0  0  0  0  0  0  0  0  1  1  0       00        |

|------------------------------------------------------------------|

|   Flag Word 1                                          02        |

|------------------------------------------------------------------|

|                                                        04        |

|------------------------------------------------------------------|

|                                                        06        |

|------------------------------------------------------------------|

|   Pointer to Initialization Configuration Data         08        |

|                                                        0A        |

|------------------------------------------------------------------|

|   Length of Initialization Configuration Data          0C        |

|                                                        0E        |

|------------------------------------------------------------------|

|   Status Block Pointer                                 10        |

|                                                        12        |

|------------------------------------------------------------------|

|                                                        14        |

|------------------------------------------------------------------|

|                                                        16        |

|------------------------------------------------------------------|

|                                                        18        |

|------------------------------------------------------------------|

|                                                        1A        |

|------------------------------------------------------------------|

|                                                        1C        |

|------------------------------------------------------------------|

|                                                        1E        |

|------------------------------------------------------------------|

|                                                        20        |

|------------------------------------------------------------------|

|                                                        22        |

|------------------------------------------------------------------|

|                                                        24        |

|------------------------------------------------------------------|

|                                                        26        |

|------------------------------------------------------------------|

|                                                        28        |

|------------------------------------------------------------------|

|                                                        2A        |

|------------------------------------------------------------------|

|                                                        2C        |

|------------------------------------------------------------------|

|                                                        2E        |

|------------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

This command allows the host to place a copy of the configuration

information pertaining to the SCSI subsystem (stored in system CMOS

RAM) into the host adapter's internal RAM.

The Data Pointer field contains the physical memory address (least

significant byte first) from which the host adapter will read the

configuration data. The data length field contains the number of bytes

available to the host adapter. The maximum number of bytes read by the

host adapter will be 32.

If this command is not sent, the host adapter will use its default

settings. After a hard reset, any configuration data sent prior to the

reset will be lost and the settings will revert to their default

state.

The configuration data consists of two bytes per possible SCSI device

(up to 16 devices) and is read from host memory in the following format.

6-20

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

|----------------------------------------------------------------|

|                                                         Byte   |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0        Offset |

|----------------------------------------------------------------|

| SCSI Device 0 (MSB)            SCSI Device 0 (LSB)       00    |

| bits 2-0: Maximum synchronous  bit 5: Parity Check enable      |

|          transfer rate         bit 3: Synchronous Negotiation  |

|   000  _  10.0 MB/S                    enable                  |

|   001  _   6.7 MB/S            bit 2: Disconnection enable     |

|   010  _   5.0 MB/S                                            |

|   011  _   4.0 MB/S            (For these bits, 1 means        |

|   100  _   3.3 MB/S             enabled and 0 means disabled)  |

|   101  _   2.9 MB/S                                            |

|   110  _   2.5 MB/S                                            |

|   111  _   2.2 MB/S                                            |

|----------------------------------------------------------------|

|  SCSI Device 1 (MSB)*     SCSI Device 1 (LSB)**           02   |

|----------------------------------------------------------------|

|  SCSI Device 2 (MSB)*     SCSI Device 2 (LSB)**           04   |

|----------------------------------------------------------------|

|  SCSI Device 3 (MSB)*     SCSI Device 3 (LSB)**           06   |

|----------------------------------------------------------------|

|  SCSI Device 4 (MSB)*     SCSI Device 4 (LSB)**           08   |

|----------------------------------------------------------------|

|  SCSI Device 5 (MSB)*     SCSI Device 5 (LSB)**           0A   |

|----------------------------------------------------------------|

|  SCSI Device 6 (MSB)*     SCSI Device 6 (LSB)**           0C   |

|----------------------------------------------------------------|

|  SCSI Device 7 (MSB)*     SCSI Device 7 (LSB)**           0E   |

|----------------------------------------------------------------|

|  SCSI Device 8 (MSB)      SCSI Device 8 (LSB)             10   |

|----------------------------------------------------------------|

|  SCSI Device 9 (MSB)      SCSI Device 9 (LSB)             12   |

|----------------------------------------------------------------|

|  SCSI Device 10 (MSB)     SCSI Device 10 (LSB)            14   |

|----------------------------------------------------------------|

|  SCSI Device 11 (MSB)     SCSI Device 11 (LSB)            16   |

|----------------------------------------------------------------|

|  SCSI Device 12 (MSB)     SCSI Device 12 (LSB)            18   |

|----------------------------------------------------------------|

|  SCSI Device 13 (MSB)     SCSI Device 13 (LSB)            1A   |

|----------------------------------------------------------------|

|  SCSI Device 14 (MSB)     SCSI Device 14 (LSB)            1C   |

|----------------------------------------------------------------|

|  SCSI Device 15 (MSB)     SCSI Device 15 (LSB)            1E   |

|----------------------------------------------------------------|

* Same as byte 1, ** Same as byte 0

All bits not defined above are ignored by the host adapter and may be

set to either 0 or 1.

The disconnection enable bit (least significant byte, bit 2)

determines whether the host adapter will allow the specified target to

disconnect during a SCSI operation. If the enable bit is one, then

disconnection is allowed for that device. The default is disconnection

enabled for all devices.

The Synchronous Negotiation enable bit (least significant byte, bit 3)

determines whether the host adapter will initiate synchronous

negotiation with the specified target. If the enable bit is one, the

host adapter will initiate synchronous negotiation for that device. If

the enable bit is zero, the host adapter will not initiate

negotiation, but will respond if the target initiates synchronous

negotiation. The default is synchronous negotiation enabled for all devices.

The Parity Check enable bit (least significant byte, bit 5) determines

whether the host adapter will check parity on incoming (SCSI to host)

data transfers for the specified target. If the enable bit is one,

then the parity will be checked for that device. The default is parity

checking enabled for all devices.

The maximum synchronous transfer (most significant byte, bits 2-0)

determines the maximum rate at which the host adapter will negotiate

with the target for synchronous transfers. The default is 10

MBytes/second for all devices.

6-21

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Supported Options

Flag Word 1 - Disable Interrupt (bit 7) and Disable Status Block (bit

14) are supported. All other flags should be zero.

None of the options in Flag Word 2 are supported by this command.

6.3.3 Initiator SCSI Command

|---------------------------------------------------------------|

|                                                        Byte   |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0       Offset |

|---------------------------------------------------------------|

|   0  Options  0  0  0  0  0  0  0  0  1  0  1  0       00     |

|---------------------------------------------------------------|

|   Flag Word 1                                          02     |

|---------------------------------------------------------------|

|   Flag Word 2                                          04     |

|---------------------------------------------------------------|

|                                                        06     |

|---------------------------------------------------------------|

|   Data or Scatter/Gather List Pointer                  08     |

|                                                        0A     |

|---------------------------------------------------------------|

|   Data or Scatter/Gather List Length                   0C     |

|                                                        0E     |

|---------------------------------------------------------------|

|   Status Block Pointer                                 10     |

|                                                        12     |

|---------------------------------------------------------------|

|   Chain Address                                        14     |

|---------------------------------------------------------------|

|                                                        16     |

|---------------------------------------------------------------|

|                                                        18     |

|---------------------------------------------------------------|

|                                                        1A     |

|---------------------------------------------------------------|

|   Sense Information Pointer                            1C     |

|                                                        1E     |

|---------------------------------------------------------------|

|   CDB Length               I   Sense Length           20      |

|---------------------------------------------------------------|

|                                                       22      |

|---------------------------------------------------------------|

|   CDB Bytes 1              I   CDB Byte 0             24      |

|---------------------------------------------------------------|

|   CDB Bytes 3              I   CDB Byte 2             26      |

|---------------------------------------------------------------|

|   CDB Bytes 5              I   CDB Byte 4             28      |

|---------------------------------------------------------------|

|   CDB Bytes 7              I   CDB Byte 6             2A      |

|---------------------------------------------------------------|

|   CDB Bytes 9              I   CDB Byte 8             2C      |

|---------------------------------------------------------------|

|   CDB Bytes 11             I   CDB Byte 10            2E      |

|---------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

This command causes the host adapter to initiate a SCSI Command to a

target. The SCSI address of the target is supplied in bits 3-0 of the

Attention Register. The Logical Unit Number of the target is supplied

in bits 2-0 of Flag Word 2.

Supported Options

All options in Flag Words 1 and 2 are supported except for the

Calculate Checksum option (Flag Word 2, bit 11).

6-22

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

6.3.4 No Operation

|----------------------------------------------------------------|

|                                                        Byte    |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0       Offset  |

|----------------------------------------------------------------|

|   0  Options  0  0  0  0  0  0  0  0  0  0  0  0       00      |

|----------------------------------------------------------------|

|   Flag Word 1                                          02      |

|----------------------------------------------------------------|

|   Flag Word 2                                          04      |

|----------------------------------------------------------------|

|                                                        06      |

|----------------------------------------------------------------|

|                                                        08      |

|----------------------------------------------------------------|

|                                                        0A      |

|----------------------------------------------------------------|

|                                                        0C      |

|----------------------------------------------------------------|

|                                                        0E      |

|----------------------------------------------------------------|

|   Status Block Pointer                                 10      |

|                                                        12      |

|----------------------------------------------------------------|

|   Chain Address                                        14      |

|                                                        16      |

|----------------------------------------------------------------|

|                                                        18      |

|----------------------------------------------------------------|

|                                                        1A      |

|----------------------------------------------------------------|

|                                                        1C      |

|----------------------------------------------------------------|

|                                                        1E      |

|----------------------------------------------------------------|

|                                                        20      |

|----------------------------------------------------------------|

|                                                        22      |

|----------------------------------------------------------------|

|                                                        24      |

|----------------------------------------------------------------|

|                                                        26      |

|----------------------------------------------------------------|

|                                                        28      |

|----------------------------------------------------------------|

|                                                        2A      |

|----------------------------------------------------------------|

|                                                        2C      |

|----------------------------------------------------------------|

|                                                        2E      |

|----------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

If the SCSI ID specified in the Attention Register specifies the SCSI

address of the host adapter, then this command is intended for

diagnostic purposes to test whether the transfer of control blocks and

their associated interrupts are working properly. It can also be used

to test whether control block chaining is working properly. The

Logical Unit Number (Flag Word 2, bits 2-0) are ignored.

If the SCSI ID specified in the Attention Register specifies the SCSI

address of a particular target, then this command is intended to queue

an interrupt to signal a progress point in a chain of control blocks.

Supported Options

Flag Word 1 - Chain No Error (bit 0), Disable Interrupt (bit 7) and

Disable Status Block (bit 14) are supported. All other flags should be

zero.

Flag Word 2 - Logical Unit Number (bits 2-0) are supported. All other

flags should be zero.

6-23

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

6.3.5 Read Host Adapter Inquiry Data

|----------------------------------------------------------------|

|                                                        Byte    |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0       Offset  |

|----------------------------------------------------------------|

|   0  Options  0  0  0  0  0  0  0  0  1  0  1  0       00      |

|----------------------------------------------------------------|

|   Flag Word 1                                          02      |

|----------------------------------------------------------------|

|                                                        04      |

|----------------------------------------------------------------|

|                                                        06      |

|----------------------------------------------------------------|

|   Destination Pointer of inquiry data                  08      |

|                                                        0A      |

|----------------------------------------------------------------|

|   Length of inquiry data                               0C      |

|                                                        0E      |

|----------------------------------------------------------------|

|   Status Block Pointer                                 10      |

|                                                        12      |

|----------------------------------------------------------------|

|                                                        14      |

|----------------------------------------------------------------|

|                                                        16      |

|----------------------------------------------------------------|

|                                                        18      |

|----------------------------------------------------------------|

|                                                        1A      |

|----------------------------------------------------------------|

|                                                        1C      |

|----------------------------------------------------------------|

|                                                        1E      |

|----------------------------------------------------------------|

|                                                        20      |

|----------------------------------------------------------------|

|                                                        22      |

|----------------------------------------------------------------|

|                                                        24      |

|----------------------------------------------------------------|

|                                                        26      |

|----------------------------------------------------------------|

|                                                        28      |

|----------------------------------------------------------------|

|                                                        2A      |

|----------------------------------------------------------------|

|                                                        2C      |

|----------------------------------------------------------------|

|                                                        2E      |

|----------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

This Command can be used to obtain release information and revision

level of the host adapter's firmware. It also provides information

regarding parameters of the host adapter.

The Data Pointer field contains the physical memory address (least

significant byte first) to which the host adapter will store the

Configuration. The Data Length field contains the maximum number of

bytes allocated for the Status Block. The maximum number of bytes this

command supports is 256. The host adapter may not transfer the total

number of bytes allocated, therefore, the Suppress Error on Underrun

flag (Flag Word 1, bit 10) should be set to one.

Supported Options

Flag Word 1 - Disable Interrupt (bit 7), Suppress Error on Underrun

(bit 10) and Disable Status Block (bit 14) are supported. All other

flags should be zero.

Flag Word 2 - None of the options in Flag Word 2 are supported by this command.

6-24

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

The information is written into host memory in the following format.

|------------------------------------------------------------|

|                                                      Byte  |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0     Offset|

|------------------------------------------------------------|

|   SCSI Device Type                                     00  |

|------------------------------------------------------------|

|   SCSI Support Level                                   02  |

|------------------------------------------------------------|

|   No. of LUNs               I   Additional Lng         04  |

|------------------------------------------------------------|

|    Flags                    I   No. of CBs             06  |

|------------------------------------------------------------|

|   Vendor Verification                                  08  |

|                                                        0A  |

|                                                        0C  |

|                                                        0E  |

|------------------------------------------------------------|

|   Product Identification                               10  |

|                                                        12  |

|                                                        14  |

|                                                        16  |

|------------------------------------------------------------|

|   Firmware Type                                        18  |

|                                                        1A  |

|                                                        1C  |

|                                                        1E  |

|------------------------------------------------------------|

|   Firmware Revision Level                              20  |

|                                                        22  |

|------------------------------------------------------------|

|   Release Date                                         24  |

|                                                        26  |

|                                                        28  |

|                                                        2A  |

|------------------------------------------------------------|

|   Release Time                                         2C  |

|                                                        2E  |

|                                                        30  |

|                                                        32  |

|------------------------------------------------------------|

|   Firmware Checksum                                    34  |

|------------------------------------------------------------|

|   Reserved                                             36  |

|------------------------------------------------------------|

|   Reserved for Information on SSCI Peripherals         30  |

|   ,                                                        |

|   .                                                        |

|   .                                                        |

|                                                        5E  |

|------------------------------------------------------------|

|   Reserved                                             60  |

|   ,                                                        |

|   .                                                        |

|   .                                                        |

|                                                        FE  |

|------------------------------------------------------------|

The SCSI Device Type Word contains the following information.

F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0

-----------------------------------------------

0     RESERVED          0  T  T  0  0  0  1  1

M  M

S  D

Bits 4-0 will be set to a value of 03 indicating that the host adapter

is a processor type. Bit 5 indicates whether the host adapter is

enabled for Target Mode. If this bit is zero, then Target Mode is

enabled. Byte 5 specifies the number of LUNs the host adapter has

enabled for Target Mode. If this bit is one, then target mode is

disabled and byte 5 will be zero. Bit 6 indicates whether Target Mode

is supported by this version of firmware. If this bit is zero, then

Target Mode is supported. If this bit is one, then Target Mode is not

supported. Bit 5 will always be set to one if bit 6 is one. Bits 14-8

are reserved for flags indicating support for special features the

firmware may incorporate. All other bits will be set to zero.

6-25

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

The SCSI Support Level Word contains the following information.

F E D C B A 9 8 7 6 5 4 3 2 1 0

-------------------------------

A 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0

E

N

Bits 2-0 indicate that this firmware version supports SCSI-2. Bit 15

indicates whether this firmware version can accept Asynchronous Event

Notifications from a target. If this bit is one, the host adapter

supports this feature. If this bit is zero, then the host adapter will

reject any Asynchronous Event Notification messages. All other bits

will be set to zero.

The Additional Length byte indicates the number of bytes of additional

information the host adapter has available. This value will not be

truncated if the Data Length field is smaller than the amount of valid

information.

Byte 5 indicates the number of LUNs the host adapter has enabled for

Target Mode. The actual LUNs supported start at LUN 0 (e.g., if this

byte contains a 3, then LUN 0, 1 and 2 are enabled.)

Byte 6 indicates the number of control blocks the host adapter can

store internally before responding with a Queue Full status.

The Flags Byte contains the following information.

7  6   5   4   3   2   1   0

-------------------------------

0  0   W   S   L   D   0   0

I   Y   N   I

D   N   K   F

Bit 2 indicates whether the board SCSI electrical interface is

single-ended/AHA-1740 (bit reset) or differential/AHA-1744 (bit set).

Bit 3 indicates that the host adapter supports SCSI-linked commands

and will always be set to one. Bit 4 indicates that the host adapter

supports synchronous transfers and will always be set to one. Bit 5

indicates whether the SCSI bus supports 8-bit or 16-bit data

transfers. If this bit is one, then the SCSI data bus is 16 bits wide.

If this bit is zero, then the SCSI data bus is 8 bits wide. All other

bits will be set to zero.

The Vendor Identification field contains eight bytes of ASCII data

identifying the vendor of the host adapter (e.g.,"ADAPTEC ").

The Product Identification field contains eight bytes of ASCII data

identifying the product name of the host adapter (e.g.,"AHA-1740").

The Firmware Type field contains eight bytes of ASCII data indicating

whether the firmware is a standard product or a special version (e.g.,

"ENHANCED").

6-26

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

The Firmware Revision Level contains four bytes of ASCII data

identifying the release level. The release level starts at "A" for

each Firmware Type.

The Release Data contains eight bytes of ASCII data identifying the

date that the firmware was released.

The Release Time contains eight bytes of ASCII data identifying the

time of day that the firmware was released.

6.3.6 Read Sense Information

|-------------------------------------------------------------|

|                                                       Byte  |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0      Offset|

|-------------------------------------------------------------|

|   0  0  0  0  0  0  0  0  0  0  0  0  1  0  0  0       00   |

|-------------------------------------------------------------|

|   Flag Word 1                                          02   |

|-------------------------------------------------------------|

|   Flag Word 2                                          04   |

|-------------------------------------------------------------|

|                                                        06   |

|-------------------------------------------------------------|

|                                                        08   |

|-------------------------------------------------------------|

|                                                        0A   |

|-------------------------------------------------------------|

|                                                        0C   |

|-------------------------------------------------------------|

|                                                        0E   |

|-------------------------------------------------------------|

|   Status Block Pointer                                 10   |

|-------------------------------------------------------------|

|                                                        12   |

|-------------------------------------------------------------|

|                                                        14   |

|-------------------------------------------------------------|

|                                                        16   |

|-------------------------------------------------------------|

|                                                        18   |

|-------------------------------------------------------------|

|                                                        1A   |

|-------------------------------------------------------------|

|   Sense Information Pointer                            1C   |

|-------------------------------------------------------------|

|                                                        1E   |

|-------------------------------------------------------------|

|   Sense Length                                         20   |

|-------------------------------------------------------------|

|                                                        22   |

|-------------------------------------------------------------|

|                                                        24   |

|-------------------------------------------------------------|

|                                                        26   |

|-------------------------------------------------------------|

|                                                        28   |

|-------------------------------------------------------------|

|                                                        2A   |

|-------------------------------------------------------------|

|                                                        2C   |

|-------------------------------------------------------------|

|                                                        2E   |

|-------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

This Command causes the host adapter to issue a Request Sense command

to the target. The SCSI address of the target is supplied in bits 3-0

of the Attention Register. The Logical Unit Number of the target is

supplied in bits 2-0 of Flag Word 2. This command must be issued when

an Initiator SCSI command with Automatic Request Sense disabled (ARS =

0) returns a Check Condition (Target Status = 02) in the Status Block.

After completion of this command, the host adapter and the target will

release Contingent Allegiance and resume operation for any queued

commands. If Extended Contingent Allegiance was reported in the Status

Block of the Initiator SCSI command, this command must be followed by

a Resume command to release Extended Contingent Allegiance.

6-27

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Supported Options

Flag Word 1 - Suppress Error on Underrun (bit 10) and Disable Status

Block (bit 14) are supported. All other flags should be zero.

Flag Word 2 - Logical Unit Number (bits 2-0) and No Disconnect (bit 6)

are supported. All other flags should be zero.

6.3.7 Run Diagnostic Test

|--------------------------------------------------------------|

|                                                        Byte  |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0       Offset|

|--------------------------------------------------------------|

|--------------------------------------------------------------|

|   0  Options  0  0  0  0  0  0  0  0  0  1  0  1       00    |

|--------------------------------------------------------------|

|   Flag Word 1                                          02    |

|--------------------------------------------------------------|

|   Write FIFO Data Pointer                              04    |

|                                                        06    |

|--------------------------------------------------------------|

|   Read FIFO Data Pointer                               08    |

|                                                        0A    |

|--------------------------------------------------------------|

|   Test FIFO Data Length                                0C    |

|                                                        0E    |

|--------------------------------------------------------------|

|   Status Block Pointer                                 10    |

|                                                        12    |

|--------------------------------------------------------------|

|                                                        14    |

|--------------------------------------------------------------|

|                                                        16    |

|--------------------------------------------------------------|

|                                                        18    |

|--------------------------------------------------------------|

|                                                        1A    |

|--------------------------------------------------------------|

|                                                        1C    |

|--------------------------------------------------------------|

|                                                        1E    |

|--------------------------------------------------------------|

|                                                        20    |

|--------------------------------------------------------------|

|                                                        22    |

|--------------------------------------------------------------|

|                                                        24    |

|--------------------------------------------------------------|

|                                                        26    |

|--------------------------------------------------------------|

|                                                        28    |

|--------------------------------------------------------------|

|                                                        2A    |

|--------------------------------------------------------------|

|                                                        2C    |

|--------------------------------------------------------------|

|                                                        2E    |

|--------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

This Command causes the host adapter to perform certain diagnostic

routines. The actual tests performed are dependent upon the setting of

the Option Flags in the Command Word, as described below. Any value of

these bits not defined below may result in a Specification Check.

6-28

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Value Meaning

000  Self-Test. The host adapter performs a subset to its

self-diagnostic routines that it normally performs at power-up.

The two Data Pointer fields and the Data Length field is ignored.

001  Test FIFO. The host adapter will transfer data from the

address in the Write FIFO Data Pointer field (least significant

byte first) to its internal FIFO. The host adapter will transfer

the number of bytes specified in the Test FIFO Data Length field

(up to 1024 bytes). After writing to the FIFO, the host adapter

will then transfer the contents of the FIFO to the address in the

Read FIFO Data Pointer field (least significant byte first),

whereupon the host may compare the data.

Supported Options

Flag Word 1 - Disable Interrupt (bit 7) and Disable Status Block (bit

14) are supported. All other flags should be zero.

Flag Word 2 is replaced by the Write FIFO Data Pointer for this command.

6.3.8 Target SCSI Command

|---------------------------------------------------------------|

|                                                        Byte   |

|   F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0       Offset |

|---------------------------------------------------------------|

|   0  0  0  0  0  0  0  0  0  0  0  1  0  0  0  0       00     |

|---------------------------------------------------------------|

|   Flag Word 1                                          02     |

|---------------------------------------------------------------|

|   Flag Word 2                                          04     |

|---------------------------------------------------------------|

|                                                        06     |

|---------------------------------------------------------------|

|   Data or Scatter/Gather List Pointer                  08     |

|                                                        0A     |

|---------------------------------------------------------------|

|   Data or Scatter/Gather List Length                   0C     |

|                                                        0E     |

|---------------------------------------------------------------|

|   Status Block Pointer                                 10     |

|                                                        12     |

|---------------------------------------------------------------|

|                                                        14     |

|---------------------------------------------------------------|

|                                                        16     |

|---------------------------------------------------------------|

|                                                        18     |

|---------------------------------------------------------------|

|                                                        1A     |

|---------------------------------------------------------------|

|   Sense Information Pointer                            1C     |

|                                                        1E     |

|---------------------------------------------------------------|

|   Sense Length                                         20     |

|                                                        22     |

|---------------------------------------------------------------|

|                                                        24     |

|---------------------------------------------------------------|

|                                                        26     |

|---------------------------------------------------------------|

|                                                        28     |

|---------------------------------------------------------------|

|                                                        2A     |

|---------------------------------------------------------------|

|                                                        2C     |

|---------------------------------------------------------------|

|                                                        2E     |

|---------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

6-29

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

This command enables the host adapter to complete a SCSI Command

initiated by another SCSI device. The SCSI address of the initiator is

supplied in bits 3-0 of the Attention Register. The Logical Unit

Number of the host adapter is supplied in bits 2-0 of Flag Word 2.

Supported Options

Flag Word 1 - Scatter/Gather (bit 12) is supported. Suppress Error on

Underrun (bit 10) should be used with care since the exact length of

the transfer may not be known. Disable Status Block (bit 14) should

also be used with care since the Status Block will return information

regarding the actual command received. The Automatic Request Sense

flag (bit 15) is ignored - if the host adapter returns the initiator a

Check Condition, the Sense information will automatically be stored in

host memory pointed to by the Sense Information Pointer. If the host

has no need for the Sense information, a zero should be stored in the

Sense Length field. All other flags should be zero.

Flag Word 2 - Logical Unit Number (bits 2-0), Data Transfer (bit 8)

and Direction of Transfer (bit 9) are supported. All other flags

should be zero. Table 6-4 shows the relationship between the Data

Transfer and Direction flags.

Table 6-4.

Relationship between Data

Transfer Option flags for Target Mode.

------------------------------------------------------------------------------

dir     dat  Description of data transfer option flags

------------------------------------------------------------------------------

X       0   INVALID. Specification Error.

0       1   RECEIVE Command expected. The host adapter will enter

a Data In phase. (Data Transfer from host memory to

initiator.)

1       1   SEND Command expected. The host adapter will enter a

Data Out phase. (Data Transfer from initiator to host memory.)

------------------------------------------------------------------------------

Note:  X means "don't care"

6.4 ABORTING CONTROL BLOCK COMMANDS

The Attention Register provides a means for aborting an outstanding

control block operation. The address of the control block to be

aborted is stored into the Mailbox Out registers, least significant

byte first. The Attention Register is then stored with the following value:

7  6  5  4   3  2  1  0

----------

0  1  0  1   SCSI ID

The SCSI ID specifies the SCSI address of a particular target or

initiator.

When the Attention Register is written to, the host adapter is

interrupted. The host adapter then scans its internal control block

queue for a control block that has an address that matches the one

stored in the Mailbox Out registers. If the associated control block

is not found, the abort is ignored and no interrupt will be posted

back to the host. If the associated control block is found, the host

adapter will take the following action based on the current progress

of the operation.

6-30

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Control Block is Queued for Operation

A control block is queued for operation when it has not yet

initiated any activity on the SCSI bus. The host adapter will

terminate the control block and place it on the interrupt queue.

When the host is interrupted for this control block, the Enhanced

Mode (Group 2) Interrupt Status Register will report Control

Block Complete With Error. The Mailbox In Registers will contain

the address of the aborted control block. The Status Block will

report a Major Error/Exception and the Host Adapter Status will

indicate that the control block was aborted by the host. If

chaining was enabled for this control block, the chain will be

halted.

Associated Target for the Control Block is Disconnected

If the command had been initiated, but the target had

disconnected, the host adapter will terminate the control block

as described above. When the target reconnects, the host adapter

will issue an Abort (or Abort Tag) message.

Note: Even though the control block will be terminated and

reported back to the host, the host adapter must maintain a

history of the control block in its internal RAM in order to send

the Abort message when the target reconnects. It is possible that

the target may never reconnect. If this happens, the host adapter

will continue to operate normally, but that portion of internal

RAM will be lost as a resource until a Device Reset or Host

Adapter Reset is issued.

Control Block is Currently Active on the SCSI Bus

If the command had been initiated and the target was currently

active on the SCSI bus, the host adapter will assert the

Attention signal. If the target responds to Attention with

Message Out phase, the host adapter will issue an Abort (or Abort

Tag) message which forces the target to go bus free. If the

target goes bus free either from the Abort message,

disconnection, or command completion, the host adapter will

terminate the control block as described above.

If the target does not go bus free within 250 milliseconds, the

host adapter will reset the SCSI bus to force the target off the

bus. The control block will be terminated as described above

except that the Host Adapter Status byte in the Status Block of

the control block will reflect that the target did not respond to

Attention.

6-31

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Control Block in the Interrupt Queue

If the command had already been completed and an interrupt was

pending to the host for the associated control block, the host

adapter will report the completion of the control block normally.

The abort operation will have no affect.

Note: There is no interrupt associated with the Abort Operation.

Its sole purpose is to prematurely terminate any control block

queued or in progress.

6.5 ASYNCHRONOUS EVENT NOTIFICATION

An Asynchronous Event occurs when the host adapter detects an

unexpected condition on the SCSI subsystem.

Asynchronous Event Conditions

� A SCSI bus reset has occurred

� The host adapter was selected by another initiator

The host adapter notifies the host of an Asynchronous Event via an

interrupt. When the host detects the interrupt, the Interrupt Status

Register will contain the following value:

7  6  5  4  3  2  1  0

|-----------------------|

|1  1  0  1  |  SCSI ID |

|-----------------------|

The SCSI ID specifies either the SCSI address of a particular device

on the subsystem, or the SCSI address of the host adapter. The host

adapter's ID can be found in bits 3-0 of the SCSI Definition Register.

Prior to interrupting the host, the host adapter will store additionl

information in the Mailbox In Registers.

SCSI Bus Reset

If a SCSI bus reset has occurred, the SCSI ID in the Interrupt Status

Register will be the host adapter's ID. The Mailbox In Registers will

contain the following information:

F E D C B A 9 8 7 6 5 4 3 3 2 1 0 F E D C B A 9 8 7 6 5 4 3 2 1 0

|-----------------------------------------------------------------------|

|                                                    |     Reset Code   |

|-----------------------------------------------------------------------|

The least significant byte contains the reset error code. The three

most significant bytes will be zero.

6-32

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Possible Reset Error Codes:

-------------------------------

Code    Description

-------------------------------

22      SCSI Bus Reset by Host Adapter

23      SCSI Bus Reset by Other Device

For more information on how the host adapter handles SCSI resets,

refer to the section on SCSI Bus Reset Handling later in this

document.

Host Adapter Selected by Another Initiator

If the host adapter is selected by another initiator on the SCSI

subsystem, the host adapter will respond as a processor-type target.

If the command received by the initiator requests a data transfer

(SEND or RECEIVE command) and the host adapter does not already have a

prepared Target SCSI command from the host, the host adapter will

disconnect from the selecting initiator and interrupt the host with an

Asynchronous Event Notification. The SCSI ID in the Interrupt Status

Register will be the ID of the selecting initiator. The Mailbox In

Registers will contain the following information:

F E D C B A 9 8 7 6 5 4 3 3 2 1 0 F E D C B A 9 8 7 6 5 4 3 2 1 0

--------------------------------------------------------------------

D         LUN       CDB BYTE 2        CDB BYTE 3     CDB BYTE 4

--------------------------------------------------------------------

The first three bytes of the Mailbox In Registers contain the number

of bytes (least significant byte first) that was requested to be

transferred by the selecting initiator. These bytes reflect the values

of bytes 2-4 of the Command Descriptor Block that was sent by the

selecting initiator.

The most significant byte of the Mailbox In Registers, bits 2-0,

contains the Logical Unit Number of the host adapter that was

specified in the Identify Message received from the selecting initiator.

The most significant byte of the Mailbox In Registers, bit 7, contains

the direction of the requested data transfer. If this bit is set to

one, then a SEND command was issued by the selecting initiator (data

transfer from initiator to host memory). If this bit is set to zero,

then a RECEIVE command was issued by the selecting initiator (data

transfer from host memory to initiator).

The remaining bits in the most significant byte of the Mailbox In

registers will be set to zero.

Upon detecting the interrupt, the host can use the information in the

Interrupt Status and Mailbox In Registers to create a Target SCSI

Command Control Block that will enable the host adapter to complete

the data transfer.

6-33

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

6.6 IMMEDIATE COMMANDS

Immediate Commands are non-control block operations that will be

processed by the host adapter immediately. They use the same delivery

interface as control block commands. Instead of a control block

address, a 32-bit immediate command is stored into the Mailbox Out

registers, least significant byte first. The Attention Register is

then stored with the following value:

7  6  5  4  3  2  1  0

----------

0  0  0  1    SCSI ID

The SCSI ID specifies either the SCSI address of a particular target,

or the SCSI address of the host adapter. The host adapter's ID can be

found in bits 3-0 of the SCSI Definition Register.

When the Attention Register is written to, the host adapter is

interrupted. When the command is completed, the host adapter (if

enabled) will interrupt the host by writing to the Enhanced Mode

(Group 2) Interrupt Status Register. If the immediate command

completed with an error, the first byte of the Mailbox In Registers

will contain the appropriate error code.

Supported Immediate Commands

80      Reset

90      Resume

The Control Block Commands are described below in alphabetical order.

6.6.1 Reset

|-------------------------------------------------------------|

| F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0       Offset |

|-------------------------------------------------------------|

|                         1  0  0  0  0  0  0  0       00     |

|-------------------------------------------------------------|

|                                     A  D             02     |

|-------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

This command allows the host, via software control, to reset a

particular target or the host adapter.

This command will always interrupt the host upon completion. The host

should wait until the interrupt is posted in order to determine if the

reset was completed successfully.

6.6.2 Reset Device

If the Attention Register contains the ID of a SCSI target, then this

command is intended to reset the associated device to a known state

with minimum impact to other devices on the bus.

6-34

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

On receiving this command, the host adapter will clear its internal

memory of all outstanding control blocks for the associated target. It

will also clear any queued interrupts for the associated target. A

queued interrupt occurs when a control block completes, with or

without error, and the host adapter has not yet issued the interrupt

to the host. The action taken on the target depends on the setting of

the Device Reset Option Flag (byte 3, bit 2).

If the Device Reset Option Flag is set to zero, the host adapter

will attempt to issue a Bus Device Reset to the target. The host

adapter will attempt to select the target with Attention and send

a Bus Device Reset message. If the target does not respond to

selection, an error will be reported. If the target is currently

active on the bus, the host adapter will assert the Attention

signal. If the target does not respond to the Attention signal

within 250 milliseconds, the host adapter will reset the bus and

report an error.

If the Device Reset Option Flag is set to one, the host adapter

will attempt to abort all outstanding operations for the target

without resetting the device. If the target is currently active,

the host adapter will assert the Attention signal and send an

Abort message. If the target does not respond to the Attention

signal within 250 milliseconds, the host adapter will reset the

SCSI bus and report an error. If the target reconnects with an

operation that was cleared by this command, the host adapter will

respond with an Abort (or Abort Tag) message.

The Adapter Reset Option flag (byte 3, bit 3) is ignored by the host

adapter for Device Reset operations.

Possible Errors:

------------------------------------------------------------------------------

Code    Description

------------------------------------------------------------------------------

08      Firmware Not Downloaded

11      Selection Timeout

20      Host Adapter Hardware Failure

21      Target Did Not Respond to Attention: SCSI Bus Reset by Host Adapter

------------------------------------------------------------------------------

If the host adapter resets the SCSI bus because the device did

not respond to Attention, the host adapter will perform the

following sequence:

Step 1. Report Immediate Command Complete with Error.

Step 2. Flush Interrupt Queue for all Other Devices. The host

adapter will post back any commands for devices other than

the one reset that completed prior to the SCSI bus reset.

Step 3. Interrupt the Host With Asynchronous Event Notification.

The host adapter will interrupt the host with Asynchronous

Event status. The SCSI ID in bits 3-0 of the Interrupt

Status Register will contain the SCSI ID of the host

adapter. The least significant byte of the Mailbox In

Registers will contain a value of 22 (hex) signifying that

the SCSI bus was reset by the host adapter.

6-35

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Step 4: Post Completion Status of all Outstanding Commands

Received Prior to the Host Acknowledgment of the

Asynchronous Event Notification interrupt. Any commands

that were outstanding when the SCSI bus reset occurred and

any commands received between the reset and the host

acknowledgment of the Asynchronous Event Notification

interrupt will be returned with error status. The status

returned will reflect that the command was aborted by the

host adapter (error code = 05). Any commands received

after the acknowledgment of the Asynchronous Event

Notification interrupt will be operated on normally.

6.6.3 Reset Host Adapter

If the Attention Register contains the SCSI ID of the host adapter,

then this command is intended to reset the SCSI subsystem to a known state.

On receiving this command, the host adapter will clear its internal

memory of all outstanding control blocks and queued interrupts for all

targets on the SCSI bus. The action taken on the SCSI bus depends on

the setting of the Adapter Reset Option Flag (byte 3, bit 3).

If the Adapter Reset Option Flag is set to zero, the host adapter

will assert the Reset signal on the SCSI bus, thus initializing

all devices on the bus.

If the Adapter Reset Option Flag is set to one, the host adapter

will attempt to abort all outstanding operations for all targets

without resetting the SCSI bus. If a target is currently active,

the host adapter will assert the Attention signal and send an

Abort message. If the target does not respond to the Attention

signal within 250 milliseconds, the host adapter will reset the

SCSI bus and report an error. If a target reconnects with an

operation that was cleared by this command, the host adapter will

respond with an Abort (or Abort Tag) message.

The Device Reset Option flag (byte 3, bit 2) is ignored by the host

adapter for Host Adapter Reset operations.

Possible Errors:

------------------------------------------------------------------------------

Code    Description

08      Firmware Not Downloaded

20      Host Adapter Hardware Failure

21      Target Did Not Respond to Attention: SCSI Bus Reset by Host Adapter

------------------------------------------------------------------------------

If the host adapter resets the SCSI bus because the device did not

respond to Attention, the result of the command will be as if the Host

Adapter Reset was issued with Adapter Reset Option Flag set to zero.

6-36

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

6.6.4 Resume

|------------------------------------------------------------|

|                                                    Byte    |

| F  E  D  C  B  A  9  8  7  6  5  4  3  2  1  0     Offset  |

|------------------------------------------------------------|

|                         1  0  0  1  0  0  0  0     00      |

|------------------------------------------------------------|

|                               i                    02      |

|------------------------------------------------------------|

Fields that are empty are reserved and should be set to zero.

This command is used to clear an Extended Contingent Allegiance

condition. The host adapter notifies the host that a target is in a

state of Extended Contingent Allegiance by setting bit 14 in the

Status Word. After issuing this command, all queued operations for the

target specified in the Attention Register will no longer be suspended

and normal operation is resumed.

If the host adapter receives this command for a target that is not in

a state of Extended Contingent Allegiance, this situation is not

considered an error, and the command is treated as a Noop.

Bit 5 in byte 3 provides the option of whether an interrupt should be

issued upon completion of this command. Setting this bit to one,

disables the interrupt. If this bit is set to zero, an interrupt will

be reported.

Possible Errors:

------------------------------------------------------------------------------

Code    Description

------------------------------------------------------------------------------

08      Firmware Not Downloaded

20      Host Adapter Hardware Failure

------------------------------------------------------------------------------

6.7 SCSI BUS RESET HANDLING

A SCSI bus reset can occur in several ways. The following describes

what can cause the reset and how the host adapter will respond to the

condition.

6.7.1 Reset Initiated by the Host

Hard Reset. When the host adapter is reset via hardware control and

the RSTPWR flag (SCSIDEF Register, bit 4) is set to one, the SCSI bus

will be reset by the host adapter as part of its power-on sequence.

There will be no notification of the SCSI reset since it is implied by

the hard reset.

Soft Reset. When the host adapter is reset via software control

(Immediate Reset command) and the Adapter Reset Option Flag is set to

zero, the SCSI bus will be reset by the host adapter as part of its

reinitialization. There will be no notification of the SCSI reset

since it is implied by the soft reset.

6-37

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

6.7.2 Reset Initiated by the Host Adapter

The host adapter will reset the bus if it encounters an error on a

SCSI operation that it cannot recover from. Some examples include: the

target goes to an invalid bus phase, the host adapter cannot force the

target to go bus free when attempting to abort a command, or the host

adapter cannot send a Bus Device Reset message. After resetting the

bus, the host adapter will perform the following sequence:

Step 1. Flush Interrupt Queue. The host adapter will post back all

commands that completed prior to the SCSI bus reset.

Step 2. Interrupt the Host With Asynchronous Event Notification. The

host adapter will interrupt the host withAsynchronous Event

status. The SCSI ID in bits 3-0 of the Interrupt Status

Register will contain the SCSI ID of the host adapter. The

least significant byte of the Mailbox In registers will

contain a value of 22 (hex).

Step 3. Interrupt the Host With Completion Status of the Command That

Caused the SCSI Bus Reset.

Step 4. Post Completion Status of all Outstanding Commands Received

Prior to the Host Acknowledgment of the Asynchronous Event

Notification interrupt. Any commands that were outstanding

when the SCSI bus reset occurred and any commands received

between the reset and the host acknowledgment of the

Asynchronous Event Notification interrupt will be returned

with error status. The status returned will reflect that the

command was aborted by the host adapter (error code: 05). Any

commands received after the acknowledgment of the Asynchronous

Event Notification interrupt will be operated on normally.

Note: If the host adapter resets the SCSI bus when attempting

a Device or Host Adapter Reset (via an Immediate Command) the

action taken by the host adapter differs from that described

above. Refer to the Immediate Command descriptions for more detail.

6.7.3 Reset Initiated by Another SCSI Device

If the host adapter detects a SCSI bus reset, it will perform the

following sequence:

Step 1. Flush Interrupt Queue. The host adapter will post back all

commands that completed prior to the detection of the SCSI bus reset.

Step 2. Interrupt the Host With Asynchronous Event Notification. The

host adapter will interrupt the host with Asynchronous Event

status. The SCSI ID in bits 3-0 of the Interrupt Status

Register will contain the SCSI ID of the host adapter. The

least significant byte of the Mailbox In Registers will

contain a value of 23 (hex).

6-38

Section 6                                   Enhanced Mode Firmware Description

------------------------------------------------------------------------------

Step 3. Post Completion Status of all Outstanding Commands Received

Prior to the Host Acknowledgment of the Asynchronous Event

Notification Interrupt. Any commands that were outstanding

when the SCSI bus reset was detected and any commands received

between the reset and the host acknowledgment of the

Asynchronous Event Notification Interrupt will be returned

with error status. The status returned will reflect that the

command was aborted by the host adapter (error code: 05). Any

commands received after the acknowledgment of the Asynchronous

Event Notification will be will be operated on normally.

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

The AHA-1740/1744 BIOS operates in two different modes which

correspond to the two possible firmware configurations. The two modes

supported by the BIOS are Standard Mode (communicates with the adapter

via the Standard compatible interface) and the Enhanced (EISA) Mode

(communicates with the adapter via the Enhanced interface). In each

mode the responsibilities of the BIOS are the same:

� Perform any power-up initialization needed for either the

AHA-1740/1744 or SCSI devices connected to it.

� Support the standard Interrupt 13H interface for SCSI disk drives on

the SCSI Bus. This support enables use of drives attached to the

adapter under real mode operating systems such as DOS without a

device driver. The limit for Int 13H supported drives will be set

at 16 (80H-8fh). As far as the system drive count and Drive Tables

are concerned the drive limit is two.

� Provide capability for booting from a fixed disk installed on the

AHA-1740/1744.

7.1 OPERATION WITH THE STANDARD INTERFACE

The AHA-1740/1744 BIOS resides on the host adapter board. It provides

support for up to two SCSI Common Command Set disk drives under DOS.

If SCSI devices other than CCS disk drives, or if support for more

than two disk drives is required under DOS, the Adaptec DOS Driver,

ASW-1410 must be used. The BIOS provides a very simple single-threaded

capability that does not make use of the Mailbox Interface used by

more sophisticated programs. This capability allows the host system to

boot from a SCSI disk drive and to support standard DOS calls from any

standard program.

The BIOS communicates with the host adapter through a special set of

commands passed under the I/O Command Port command 03 (Start PC AT

BIOS Command). These commands are not available to any programs except

the Adaptec BIOS. The Adaptec BIOS provides a standard BIOS interface.

The DOS interface to the standard BIOS is described in this section.

In many machines, up to two hard disks are supported by the BIOS

resident on the motherboard. Any other drives must be managed through

the Mailbox Interface using an appropriate driver. The operation of

the AHA-1740/1744 BIOS on attached SCSI devices depends on the number

of standard hard disks installed directly on the system.

7-1

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

No standard hard disks installed:

In this case, two SCSI drives can be supported by the AHA-1740/1744

BIOS. SCSI Target 0 Logical Unit Number 0 is designated as drive 0

(Drive C:). Either SCSI Target 0 LUN 1 or SCSI Target 1 LUN 0 may be

designated as drive 1 (Drive D:). This set of addresses allows the use

of two drives with either embedded controllers or bridge controllers.

Booting is only performed from SCSI Target 0 LUN 0. When no standard

hard disks are installed, the AHA-1740/1744 BIOS is operating in

nonconcurrent mode.

One standard hard disk installed:

In this case, the internally installed hard disk is designated as

drive 0 (Drive C:). SCSI Device 0 LUN 0 is designated as drive 1

(Drive D:). Booting is performed from the internally installed hard

disk. This mode of operation is called concurrent operation,

indicating that the internally installed drive and one SCSI drive can

operate together under the BIOS.

Two standard internal hard disks installed:

In this case, all the disks that can be supported by the BIOS are

directly installed. The AHA-1740/1744 BIOS will not support any SCSI

drives. Therefore a device driver will need to be installed to access

the SCSI drives.

7.2 OPERATION WITH THE ENHANCED INTERFACE

7.2.1 Hardware

The BIOS occupies 16K of system ROM space at one of a number of

optional physical locations. These locations are CC000, D0000, D4000,

D8000, DC000, E0000, E4000, E8000 and EC000. 2K of this space is R/W

shadow RAM, enabling the BIOS to store critical information without

risking compatibility by using system data areas. The BIOS is coded

using the 386 instruction set.

7.2.2 Initialization

The system BIOS recognizes the AHA-1740 BIOS by the standard ROM

signature information in the first four bytes of the EPROM. This

signature includes an offset to an initialization routine which is

subsequently called by the system BIOS. When the AHA-1740 BIOS gains

control, the following steps will be taken:

� The BIOS will search the EISA slots for the adapter onto which it is

installed. The correct adapter can be identified by matching the

current Code segment with the BIOS base address in the boards setup

registers. In the case where the board is not functioning properly,

the BIOS will display the message 'Unable to initialize SCSI host

adapter!' and the BIOS will fail initialization. The failure will

be flagged to the system via the BP register so that the system BIOS

can halt system initialization until the user strikes a key (F1 on

most systems).

7-2

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

� The BIOS will normally reset the adapter. Using the Configuration

utility, the BIOS may be configured not to reset the adapter. This

adapter reset may also result in a Reset of the SCSI bus. The BIOS

will wait two seconds before proceeding with initialization, as many

SCSI devices are unable to communicate properly shortly after a SCSI

Reset.

�  Having found and Reset the board, the BIOS will proceed to setup

and test the onboard shadow RAM. If diagnostics on this RAM fail,

the message 'SCSI host adapter shadow RAM diagnostic failure!' will

be displayed, and Initialization will fail. If the RAM is OK, the

BIOS will then perform a diagnostic check on the adapter hardware to

make sure that it is functioning properly. If not the message 'SCSI

host adapter diagnostic failure!' will be displayed. If there is

any failure, it will be flagged to the system.

� The BIOS will pull adapter configuration information out of system

CMOS RAM via Int 15H and will download this configuration

information to the host adapter.

� The BIOS will determine configuration information regarding

installed SCSI devices from the information acquired from CMOS RAM.

A BIOS support option set to "fr" indicates that the BIOS should

attempt to support the device under Int 13H. If the device is not

installed or not SCSI device type 0 or 7 (byte 0 of Inquiry info), the

drive will not be supported under Int 13H. A BIOS support option of

"hd" is the same as "fr" except that removable hard drives are not

considered hard drives and are not eligible for Int 13H support.

A BIOS support option of "no" disables Int 13H support for the device.

The default setting will be "hd" for all devices. The net effect of

the defaults is to cause the BIOS to scan the entire SCSI bus looking

for FIXED hard drives to support via Int 13H. If the user ever changes

a device to "fr" a warning screen should appear informing the user of

the issues involved with BIOS support for removable media.

Other Changes

The BIOS is interrupt driven in Enhanced Mode and fully supports Int

15H allowing operation with various write caching programs. The BIOS

supports drives 80H on one board and 81H on the next board with both

boards sharing the same interrupt.

The BIOS also supports a new Interrupt 13H call to identify which Int

13H drives (80H and 81H) correspond to which host adapter and SCSI ID.

Note that the ASPIEDOS.SYS, AFDISK.EXE and ASPIDISK.SYS files in the

appropriate manager software must be changed to support this function.

This eliminates the requirement on previous Adaptec host adapters that

Int 13H devices may only be at targets 0 and 1.

Note: In Standard Mode, the BIOS will stop scanning for devices as

soon as a device is indicated as not present. This is done to be

compatible with the AHA-1540 which requires the first SCSI drive to be

at Target 0, the second at Target 1, etc.

The AHA-1740 BIOS will perform the steps necessary to support Int 13H

on the SCSI drives which were assigned Int 13H IDs. This includes the

building of Drive Tables in shadow RAM for drives 80H and 81H and

7-3

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

updating Drive table pointers at interrupt vector locations 41H and

46H, respectively. The drive tables are not actually used by the

AHA-1740 BIOS during normal operation, but are merely provided for

compatibility. The AHA-1740 BIOS will revector Interrupt 13H so that

all calls for DISK I/O can be filtered by the AHA-1740 BIOS. Requests

for non-SCSI drives are passed on to the original Int 13H vector which

is stored in Shadow RAM. The system drive count (40:75) is updated to

reflect the new system drive count. See Section 2.4 for more

information on Int 13H functionality.

Once initialization is complete and all appropriate data structures

have been setup in Shadow RAM, the BIOS will Write protect the shadow

RAM so that further access is prohibited. Because an outside agent

can always move the BIOS into write protected area, no write access is

made to this area after initialization. Systems which move the BIOS

into such an area before initialization will not work with the

AHA-1740 BIOS.

7.2.3 Boot Issues

Traditionally, systems have booted from Drive 80H (often referred to

as Drive C:). For ISA systems, Adaptec SCSI BIOS ROMs were forced to

intercept Interrupt 19H in order to provide booting capability from a

SCSI drive. This was due mainly to the fact that many motherboard

BIOS products did not properly issue Int 13H calls to perform the

boot. It is currently not the case with EISA systems. Therefore, the

AHA-1740 BIOS will not contain any booting code and Int 19H will be

left alone. The system BIOS is free to implement any boot algorithm

it chooses, and SCSI drives will be supported by such schemes as long

as Int 13H calls are used to read the boot sector(s). On systems

which will only boot from drive 80H (this is very typical) then

booting from SCSI will be limited to a SCSI drive assigned as Drive

80H. Note that this excludes booting from SCSI if there is already a

standard controller in the system. The user can control the boot

drive by mapping out drives via the AHA-1740 CMOS configuration so the

desired boot drive is the first drive found by the system.

7.3 INTERRUPT 13H INTERFACE FUNCTIONALITY

Disk I/O requests are passed from the operating system to the BIOS

through software interrupt 13H. CPU registers are used to pass a

function code as well as the associated parameters for the function.

The AHA-1740 BIOS supports SCSI drives under Int 13H by intercepting

each request and routing the request based on the Drive number. If

the Drive number corresponds to a SCSI drive, the AHA-1740 services

the request through the AHA-1740. If the Drive number specifies a

floppy or non-SCSI hard drive, the request is passed to the original

Int 13H vector for processing. Most Int 13H requests use the

following registers for parameter passing:

_____________________________________

Register     Function

-------------------------------------

AH           BIOS Function Code

AL           Sector Count

CH           Low byte of cylinder #

CL (Bits 7,6) High bits of cylinder #

CL (Bits 5-0) Sector Number (1-based)

DH           Head Number

DL           Drive Number

ES:BX        Data Buffer Address

-------------------------------------

7-4

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

7.3.1 Physical to Logical Block Address Translation

As shown above, the starting sector addresses for read, write and

verify requests is passed as a physical address including a 10-bit

cylinder number, an 8-bit head number, and a 6-bit sector number

(which is 1-based so the maximum sector number is 63 instead of 64).

SCSI devices are addressed by logical block address without regard to

the physical geometry of the drive. For this reason, all SCSI devices

are defined as drives with 64 heads and 32 sectors/track. An

appropriate number of cylinders is used to provide the full capacity

of the drive using this physical to logical translation. Each

cylinder is exactly one megabyte in size using this scheme and the

physical to logical translation can be described by the following

simple algorithm:

LBA = Cylinder (10 bits) || Head (6 bits) || (Sector-1) (5 bits)

This provides 21 bits of addressing, which is equivalent to one

gigabyte given that the Int 13H interface assumes a block size of 512

bytes/sector.

7.3.2 Virtual to Physical Buffer Address Translation

A data buffer address is passed to the BIOS via ES:BX. This virtual

address can be easily translated into the physical address needed by a

BUS Master device when the system processor is in real mode. Because

this is the case at system boot time and under normal DOS operation,

simple segment arithmetic is normally used to convert ES:BX to a

physical address usable by the AHA-1740:

Physical address = (Segment * 16) + Offset

The 386 processor, however has given rise to a large number of

software products which use the 386 memory management feature to

provide virtual 8086 operation. The address passed in ES:BX still

appears valid to the BIOS, but the processor is not in fact in real

mode, and normal segment arithmetic will not yield the correct

physical location of the data buffer. This issue has been addressed

by IBM and Microsoft in a specification known as Virtual DMA Services

(VDS). This specification provides a method to the ROM BIOS for

determining the physical location of a buffer given the

selector:offset as well as its layout (it may not be contiguous). The

AHA-1740 BIOS will fully support the VDS specification so that 386

memory management programs will be able to run without a driver as

long as they are VDS compliant. In ISA Standard Mode, however, VDS

will not be supported and a driver will be necessary to support VDS.

7.3.3 BIOS Command Return codes

Any Int 13H request, upon competion, returns control to the requestor

with a return code set in the carry flag and a status code placed in AH.

The carry flag is set as follows:

CF=1         Error or unusual condition. Status is nonzero.

CF=0         Command completed normally. Status is zero.

7-5

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

The definition of the status code, as well as the mapping from SCSI

error conditions, is provided in the table below:

AH=00H       No Error: Operation completed successfully

AH=01H       Invalid Function Request. The Int 13H function code provided

was not valid or the Drive Number was out of range.

AH=02H       Unable to Read Address mark: One of the following additional

sense codes (SCSI ASC) was presented in the sense information

returned by the Target:

12H - No AM Found on Data Field

21H - Illegal Logical Block Address

AH=03H       Write protect error: Returned SCSI ASC:

27H - Data Protect

AH=04H       Read Error: Returned SCSI ASC:

14H - No Record Found

16H - Data Sync Error

AH=10H       Uncorrectable ECC error: Returned SCSI ASC:

10H - ID ECC Error

11H _ Unrecovered Read Error

AH=11H       ECC Corrected Data Error: Returned SCSI ASC:

17H _ Recovered Read Error w/o ECC

18H _ Recovered Read Error w/ECC

AH=20H       General Controller Failure: Returned SCSI ASC:

01H 03H 05H 06H 07H 08H 09H 1BH 1CH 1DH 40H 41H 42H

43H 44H 46H 47H 48H 49H

AH=40H       Seek Operation Failed: Returned SCSI ASC:

15H _ Seek positioning error

02H _ No seek complete

AH=80H       Time-out: Host adapter not responding to BIOS

AH=AAH       Device Not Ready: Returned SCSI ASC:

04H _ LUN not ready

28H _ Unit Attention, Ready

29H _ Unit Attention, Power on

2AH _ Unit Attention, Mode Select Change

AH=BBH       Undefined error occurred: A SCSI ASC other than those

listed was returned by the target.

AH=CCH       Write Fault: Not returned by AHA-1740 BIOS

AH=FFH       Sense Operation Failed: An error occurred issuing the SCSI

request sense command to the target.

7.3.4 Hardware BIOS Commands

(AH) = 00H - Reset Disk System

This function does nothing to SCSI drives. Regardless of Drive

number, however, this request is passed to the original Int 13H vector

so that floppy and non-SCSI drives may be reset.

7-6

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

(AH) = 01H - Read Status of Last Operation

The status of the last operation performed is returned. The status is

reset to zero.

(AH) = 02H - Read Desired Sectors into Memory

The sectors requested are read from the disk to system memory. A READ

(EXTENDED) command (SCSI Operation Code 28H) is used to execute this

function.

(AH) = 03H - Write Desired Sectors from Memory

The sectors requested are written from system memory to the disk. A

WRITE (EXTENDED) command (SCSI Operation Code 2AH) is used to execute

this function.

(AH) = 04H - Verify Desired Sectors

The sectors requested are verified to be correctly written on the SCSI

disk. A VERIFY command (SCSI Operation Code 2FH) with the Byte check

bit set to zero is used to execute this function. If the VERIFY

function is not supported by the selected disk, a READ (EXTENDED)

command is used and the data is thrown away.

(AH) = 06H - Identify SCSI devices

This is a special call that is used to return the first supported SCSI

drive. If there are only SCSI drives, then the return value will be

80H. If there is a SCSI and a standard controller, then the return

value will be 81H.

The ID of the first supported drive is returned in BL. If an error

code is returned (C is set) on this call, it indicates that the SCSI

adapter BIOS is not in the Int 13H chain at all, hence no SCSI drives

are supported by the BIOS.

(AH) = 08H - Read Drive Parameters

A SCSI READ CAPACITY command is used to determine the maximum logical

block of the selected SCSI drive. This information is then used to

calculate the proper number of cylinders to be returned to the host

system. The number of heads returned will always be 64 and the number

of sectors per track will always be 32. The number of drives will

reflect the value in 40:75 (system hard drive count). The capacity is

rounded to the nearest full megabyte, and the cylinder count is

returned as the number of megabytes (as each cylinder is a megabyte).

7-7

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

The output of this function is defined as follows:

AH           Status of Operation

DL           Number of Hard Drives supported by Interrupt 13H.

DH           Maximum value of head number = 63

CH           Maximum value for cylinder number(Low byte)

CL (Bits 7,6) Maximum value for cylinder (High bits)

CL (Bits 5-0) Maximum value for sector number (1-based) = 32

CF           Return Code

(AH) = 09H - Initialize Drive Pair Characteristics

This command does nothing to SCSI drives, since they are self-configuring.

(AH) = 0CH - Seek

A SEEK (EXTENDED) command (SCSI Operation Code 2BH) is used to perform

this function. As the seek command is not mandatory for SCSI, an

Invalid command error will not be reported back to the system, but

instead the command will complete normally. Any other error will be

reported as usual.

(AH) = 0DH - Alternate Disk Reset

This function does nothing to SCSI drives. Regardless of Drive

number, however, this request is passed to the original Int 13H vector

so that non-SCSI hard drives may be reset.

(AH) = 10H - Test Drive Ready

A TEST UNIT READY command (SCSI Operation Code 00H) is used to execute

this function.

(AH) = 11H - Recalibrate

A REZERO command (SCSI Operation Code 01H) is used to execute this

function. As the REZERO command is not mandatory for SCSI compliance,

an Invalid command error will not be reported back to the system, but

instead the command will complete normally. Any other error will be

reported as usual.

(AH) = 15H - Read DASD Type

A SCSI READ CAPACITY command is used to determine the maximum logical

block of the selected SCSI drive. This information is returned to the

host along with the DASD type of the target which will always be 03H

for fixed disk.

7-8

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

The output of this function is defined as follows:

AH   Status of Operation

DL      00H _ Drive Not Present or Invalid DL

03H _ Fixed Disk Present

01H,02H _Reserved

CX      Number of Blocks on Disk

CF      Return Code

The following function codes are not supported by the AHA-1740 BIOS:

(AH) = 05H _       Format Desired Cylinder

(AH) = 06H _       Format Desired Cylinder and Set Bad Sector Flags

(AH) = 07H _       Format Drive Starting at Desired Cylinder

(AH) = 0AH _       Reserved

(AH) = 0BH _       Reserved

(AH) = 0EH _       Reserved

(AH) = 0FH _       Reserved

(AH) = 12H _       Reserved

(AH) = 13H _       Reserved

(AH) = 14H _       Reserved

(AH) = 16H _       Reserved

(AH) = 17H _       Reserved

(AH) = 18H _       Reserved

(AH) = 19H _       Park Heads

(AH) = 1AH _       Format Unit

(AH) = 1BH _ FFH   Reserved

An Invalid command error is returned for any of these function codes.

7.4 INT 15H FUNCTIONALITY

After issuing an I/O command to the host adapter, the AHA-1740 BIOS

will make use of the Int 15 Device Busy (function 90H) function to

inform the operating system that the BIOS is about to wait for a

device.

When the AHA-1740 interrupts indicating that the I/O is complete, an

Interrupt Complete (function 91H) will be issued by the BIOS

indicating that the wait is complete.

7.5 DIFFERENCES BETWEEN OPERATING MODES

� VDS supported in Enhanced Mode only.

� In EISA Mode, BIOS will scan for Int 13H devices. In Standard Mode,

disk devices must be installed sequentially starting at Target 0.

7-9

Section 7                                              Onboard BIOS Interface

------------------------------------------------------------------------------

7.6 MULTIPLE ADAPTER SUPPORT

When multiple SCSI adapters are installed, the BIOS should still be

enabled on all boards and each BIOS is responsible for initializing

the board on which it resides. This has the following advantages:

_ Full 2K of shadow RAM is available for each board.

_ Facilitates mixing of different versions of adapter in same system because

each board is initialized by BIOS belonging to that board.

7-9

Section 8                                                       Device Drivers

------------------------------------------------------------------------------

8.1 DOS OPERATION WITHOUT DRIVERS

The SCSI host adapter operates with very high performance

characteristics using drivers that directly access its multitasking

interface. In addition, the AHA-1540B and its BIOS support the

Interrupt 13 interface used by MS-DOS. Only two hard disk drives can

be accessed by MS-DOS without the use of special driver programs. If

no internal hard disk drives are installed in the host computer

system, the SCSI drive at SCSI Address 0, and LUN 0 (0:0) is used as

drive C:, the boot hard drive. If a drive is installed at SCSI Address

1, LUN 0 (1:0), it will be used as drive D:, the second hard drive. If

one internal hard disk drive is installed in the computer system, that

drive is assigned as drive C:. In that case, the SCSI disk drive at

address 0:0 will be assigned as drive D:. If two internal hard disk

drives are installed in the computer system, any SCSI disk drives can

be accessed only by a special software driver.

8.2 SYSTEM CONFIGURATION

Standard AT motherboard BIOS supports up to two floppy disk drives and

two hard disk drives under DOS. DOS uses Interrupt 13 to access the

floppy and hard disk services. Interrupt 13 refers to the first and

second floppy disk as 0 and 1, respectively, and refers to the first

and second hard disks as 80 and 81, respectively. Floppy disk drive 0

is assigned drive letter A: and floppy disk drive 1 is assigned drive

letter B:. The first partition on hard disk drive 0 is assigned drive

letter C: and the first partition on hard disk drive 1 is assigned

drive letter D:. If there is only one physical hard disk, D: refers to

the second partition on that drive.

To permit the use of more than two hard disks in your system, Adaptec

provides a software driver that is loaded via the CONFIG.SYS file.

(Refer to Section 8.5.)

AT computers or compatibles come with a SETUP utility either on floppy

diskette or included in the motherboard BIOS. This SETUP utility is

used to store the configuration of your system in low-power CMOS

memory powered by a battery. It is, therefore, saved when your

computer is turned off. The presence of "standard hard disks" should

be reflected in this SETUP utility. The presence of SCSI hard disks is

NOT entered in SETUP.

A "standard hard disk" refers to a disk that uses an ST-506/412 (MFM

or RLL) or an ESDI interface. A "standard hard disk controller" refers

to an adapter card that controls one or two "standard hard disks" and

uses the standard AT hard disk register set 1F0 _>1F7 (i.e., WD 1003

or Adaptec ACB-2300 series).

To boot from a SCSI hard disk, the SETUP utility must indicate that no

hard disks are installed, must have a bootable SCSI hard disk at SCSI

ID 0, and there must not be a floppy diskette in floppy disk drive A:.

8-1

Section 8                                                       Device Drivers

------------------------------------------------------------------------------

8.3 LOW-LEVEL FORMAT

Most SCSI drives are shipped from the factory with a complete

low-level format. A low-level format for use with the AHA-1740/1744

is in development.

8.4 INSTALLATION AND INITIALIZATION UNDER DOS

Hard disks must be initialized with the proper DOS structures by

executing the DOS initialization programs FDISK and FORMAT.

Run the DOS FDISK program to partition the disk for the number of

cylinders to be used by DOS. When using DOS 3.2 or under, it is

generally a good idea to select one less cylinder than the maximum

allowable per partition according to DOS. This eliminates the

possibility of exceeding the 32 MByte limit. Activate the first DOS

partition before leaving the FDISK program if it is to be the boot

partition. If you are not sure if a DOS partition exists, use the menu

entry in FDISK to display partition data.

Caution: If the drive you are using was previously formatted or

partitioned with a different host adapter or disk controller, a DOS

partition may already exist. If such a partition does exist it should

be deleted and recreated using this host adapter. If this precaution

is not followed, erratic system operation may result.

After the DOS partition has been created and activated, the drive is

ready for a DOS format. Refer to your DOS manual for the format

options that are available. If the system is going to boot from a SCSI

disk, it is usually easiest to transfer the hidden system files to

that disk during the DOS format by using the /S option..

The host adapter and SCSI disks are now ready for normal DOS

operation.

8.5 MANAGERS

The AHA-1740/1744 will operate under DOS with up to two hard disk

drives without a dedicated software driver through the onboard code

embedded in the BIOS EPROM.

However, for optimal use of the features and benefits of this

high-performance board, the following cases suggest that a driver is required:

� Use of a DOS application that runs in a protected mode (such as

the Virtual Memory Mode used by Windows 386)

� Use of an operating system other than DOS

� Use of more than two hard disks

� Use of SCSI peripherals other than disk

� Any combination of the above

8-2

Section 8                                                       Device Drivers

------------------------------------------------------------------------------

Adaptec has developed an architecture called Advanced SCSI Programming

Interface (ASPI), which is available as a standard to all companies.

ASPI divides the driver problem into two levels with a pass-through

ASPI layer interface between. The driver component above the layer is

called a module and is peripheral-specific. The driver component below

the layer is called a manager and is host adapter-specific. Since the

ASPI layer is a standard, different modules can work with different

managers in a plug-and-play manner. Note that ASPI details are

specific to a given operating system.

Only an overview of the managers available is given here. For more

complete information, refer to Adaptec's ASPI and ASW software

documentation. Drivers available at time of printing were:

ASW-1140  Non-ASPI Novell NetWare Driver

ASW-1410  ASPI Manager and ASPI Disk Module for DOS

ASW-210   ASPI Tape Module and Sytron SY-TOS for tape for DOS

ASW-310   ASPI Tape Module and Sytron Sytos Plus for tape for DOS

ASW-410   ASPI CD-ROM Module for DOS.

ASW-1420  ASPI Manager and Disk Module for OS/2

ASW-220   ASPI Tape Module and Sytron SY-TOS for tape for OS/2

ASW-1440  ASPI Manager and Disk Module for NetWare 286/386

SCO Unix driver support in kernel

SCO Xenix driver support in kernel

Interactive Unix driver support in kernel

Everex ESIX support in kernel

Banyon Vines support

8.6 DOS MANAGER

A DOS Manager is available that enables the AHA-1740/1744 in

Compatible mode to access additional SCSI devices on the SCSI bus.

When operating under DOS, a device driver is needed to access more

than two hard disks or to access tape devices and other types of SCSI

peripherals. If all DOS operations will be confined to two SCSI disks

or to one internal and one SCSI disk, a device driver is not needed.

The onboard BIOS will successfully manage the SCSI disk drives in

those cases.

See the documentation supplied with the DOS Driver for instructions on

the procedure for installing and enabling the driver program.

The following table shows how hard disks in a system are addressed:

[Picture]

8-3

Section 8                                                       Device Drivers

------------------------------------------------------------------------------

The Adaptec DOS Manager supplies a number of services, including

support for a special version of the FDISK program called AFDISK.

AFDISK is needed to partition and high-level format the third, fourth,

etc., hard disk in your system. Use DOS FDISK and FORMAT for your

first two hard disks. AFDISK performs both the FDISK and the FORMAT

function for disk drives beyond the first two. In addition, the DOS

Driver supplies an interface suitable for the SY-TOS tape utility

programs provided by Sytron.

Two versions of the DOS driver exist in the field. The older ASW-1110

does not comply with ASPI and is no longer being shipped. It provides

only disk and tape support. The newer ASW-1440, written to the ASPI

standard, is recommended for future compatibility.

8.7 WINDOWS 386

The use of high-performance 16-bit first-party (or Bus Master) Direct

Memory Access (DMA) on the AHA-1740/1744 SCSI host adapters may

require a driver to be compatible with software applications using the

protected mode of the Intel 80386 microprocessor. A common example of

this is the Virtual Memory mode used by Microsoft's Windows 386. The

driver is used to convert the logical addresses used by the software

application to the physical addresses used by the Bus Master

AHA-1740/1744. There are three drivers available that support Windows

386. They are SCSIHA.SYS, AHA1540.SYS (which are non-ASPI) and the new

ASPI4DOS.SYS. In CONFIG.SYS, type in only one of the following lines,

depending upon which driver you have:

DEVICE=SCSIHA.SYS /V386 /B=xx (where xx can be 16, 32, or 64)

or DEVICE=AHA1540.SYS /V386 /Bixx (included in ASW-110)

or DEVICE=ASP14DOS.SYS /W (included in ASW-1410)

Refer to documentation for the ASW-1110 or ASW-1410 for more

information.

For versions of the manager earlier than 2.0 which did not provide

this support, Adaptec has made available drivers which will circumvent

these restrictions through use of a double-buffering technique.

The first driver listed, SCSIHA.SYS, is available from Adaptec free of

charge. SCSIHA.SYS is available direct from the factory or may be

downloaded from Adaptec's hotline bulletin board at (408) 945-7727.

8.8 OS/2 MANAGER

The ASW-1420 OS/2 manager for the AHA-1740/1744 allows the board to

achieve its full multitasking potential under OS/2. Earlier versions

of the manager were compatible with OS/2 version 1.1. Versions will be

available which will run under Version 1.21 and 2.0 and support the

LADDR specification. Tape support is also available via the ASW-220.

8.9 NOVELL NETWARE MANAGER

The ASW-1440 Novell NetWare manager provides a vehicle for running the

AHA-1740/1744 in the latest industry-standard network operating system

- NetWare 386, as well as the earlier NetWare 286.

8-4

Section 8                                                       Device Drivers

------------------------------------------------------------------------------

8.10 UNIX/XENIX

By working closely with the software vendors, Adaptec has arranged for

full driver support of the AHA-1740/1744 in the kernel of SCO

Unix/Xenix and Interactive System's Unix. No other software drivers

are necessary. Refer to the relevant vendor documentation.

SCO Unix driver support in kernel

SCO Xenix driver support in kernel

Interactive Unix driver support in kernel

Everex ESIX support in kernel

Banyan Vines support

8-5

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

9.1 SCSI DESCRIPTION, INITIATOR MODE

The AHA-1740/1744 provides a very high performance SCSI interface

connection. The host adapter meets the ANSI Standard X3T9.2/86-109

Revision 10c, describing the SCSI. The host adapter additionally meets

conformance level 2 of the specification by implementing the following

SCSI options:

� Accepts or manages the following messages:

Command Complete

Disconnect

Message Reject

Identify

Save Data Pointer

� Performs arbitration

The following alternatives are selected for the AHA-1740/1744 SCSI

interface connection from those described by X3.131-1986.

� Single-ended SCSI driver/receivers are used.

� The Termination Power is supplied through a removable fuse,

allowing the user to select whether termination power is

provided by the host adapter at installation time.

� Parity is always generated. Parity checking can be disabled

through the jumper selection.

� The "Soft" Reset option is always performed by the

AHA-1740/1744. If the user desires the "Hard" Reset, an

interrupt servicing program that detects the SCSI Reset

Detected interrupt in the Interrupt Flags Port must set the

Soft Reset bit in the Control Port. This bit forces the host

adapter to clear all SCSI-related operations, but does not

require execution of the diagnostic functions. If the Soft

Reset bit has been set, the host software must reinitialize

the AHA-1740/1744.

� Synchronous data transfer is supported. Negotiation for the

synchronous transfer initiated by another SCSI device will be

accepted by the AHA-1740/1744 at any time. If the jumper has

been set to allow it, the AHA-1740/1744 will also initiate

synchronous data transfer negotiation when it has detected

that such negotiation may be required.

9-1

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

� Multitasking is fully supported.

� Modify Data Pointers is supported to allow Zero Latency Read operations.

In addition to these SCSI functions, the SCSI Common Command Set at

level 4B is also supported. While this document was never made a

standard, it describes a widely available set of disk drive functions

which are supported by the host adapter. The host adapter BIOS

commands are all mapped into SCSI CCS commands to allow the proper

support of all the most common SCSI disk drives. The Adapter Command

"Return Installed Devices" also uses CCS commands to determine which

devices are available.

The draft ANSI standard for SCSI-2 has been used as a reference for

the implementation of all SCSI functions with the expectation that the

host adapter will be fully compatible with the final version of the

SCSI-2 standard. In particular, the Processor-type device command set

has been selected from the SCSI-2 manual for Target Mode operation.

9.1.1 Linked SCSI Commands

The AHA-1740/1744 supports linking of SCSI commands in initiator mode.

When the link bit in the SCSI command control byte is set, the target

will present either a 0A or 0B message at command completion. The

AHA-1740/1744 uses the link pointer in a CCB to fetch another CCB. At

the same time, the completed CCB status and address are stored in an

MBI. If the target returns a 0B or 00 message, the AHA-1740/1744

generates an interrupt to inform the host of the full MBI. If the

target returns a 0A message, the MBIF interrupt is not posted until

all linked commands are completed. The linked CCBs must address the

same target and LUN since the target is not reselected.

9.1.2 Zero Latency Read Operation

The AHA-1740/1744 implements Zero Latency operation through the use of

modify data pointer messages. Zero Latency can eliminate rotational

latency, depending on the length of the data transfer, by supporting

out of order data transfers. This advanced feature is currently only

implemented on the ACB-4525Z SCSI-to-ESDI controller, and is supported

transparently to the user.

After seeking to the target track, the drive will begin reading block

IDs. If the first block ID is within range of the data transfer, but

not the last block of the the data transfer, the drive will begin

reading the subsequent blocks into its buffer. Before transferring

data the drive will issue a MODIFY DATA POINTER message to the

AHA-1740/1744. This supplies a positive argument that is added to the

value of the current data pointer. The drive will now send this

portion of the data transfer to the host. The drive will resume

reading data into its buffer as soon as the first block of the data

transfer is detected. Before sending this data to the host, the drive

will issue a second MODIFY DATA POINTER message to the AHA-1740/1744

which supplies a negative argument. This returns the data pointer to

its original position. This guarantees that a data transfer of one

track or less will never require more than a single revolution since

data can now be transferred out of order.

9-2

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

9.1.3 SCSI Messages

The AHA-1740/1744 host adapter supports a number of special messages

in addition to the messages required by meeting conformance level 2.

Those messages are described in detail in the SCSI specification,

X3.131-1986, and in this Section where they are used. The messages are

summarized in this table:

------------------------------------------------------------------------------

Function               Message                  Caused By

------------------------------------------------------------------------------

Standard Messages      Command Complete         Normal Sequencing

Error Management       Message Reject           Invalid Messages

Bus Device Reset         Special CCB

Abort                    Special MBO

Disconnect/Reconnect   Identify                 Normal Sequencing

Disconnect               Normal Sequencing

Save Data Pointer        Normal Sequencing

Restore Pointers         Special Sequencing/ZLR

Synchronous Transfer   Synchronous Data Transfer Initialization Sequencing

Request

Zero Latency Operation Modify Data Pointers     ZLR Sequencing

Linked Commands        Linked Command Complete  Command Linking

Linked Command Complete  Command Linking

With Flag

------------------------------------------------------------------------------

9.2 SCSI DESCRIPTION, TARGET MODE

9.2.1 Initiator Conformance Level Requirements

Initiators that execute commands against an AHA-1740/1744 operating in

Target Mode are required to have the following conformance levels, as

described in Appendix E of the SCSI Specification, ANSI X3.131-1986.

Conformance must be present with respect to each of the following items:

� The initiator must use single-ended drivers.

� Termination power may optionally be provided by the initiator,

but must meet the SCSI specification in both its over-current

protection and its reverse current diode protection. The

terminators may be installed on the AHA-1740/1744 board or

installed as in-line terminators at the cable connectors.

� The implementation of parity is optional, but desirable.

� The initiator may support either "hard" reset or "soft" reset.

All attached devices must support the same type of reset.

9-3

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

� The initiator must meet the requirements of conformance level

two. In particular, all LUN addressing must be performed by the

IDENTIFY message, not by the LUN field in the CDB. Disconnection

and reconnection must be supported.

� The initiator and target functions have the same SCSI ID.

9.2.2 Support of Synchronous Transfer

Synchronous transfer is supported by the AHA-1740/1744 in target mode

without any instruction or support from the system processor. If an

initiator invokes a synchronous transfer negotiation, the

AHA-1740/1744 will complete the negotiation of the required transfer

offset and period. If the proper jumper has been set, the

AHA-1740/1744 will also attempt to negotiate synchronous transfer

during the initial selection period of the first command after an

initialization or after a SCSI reset.

9.2.3 SCSI Target Operation in Processor Target Mode

When the AHA-1740/1744 has been set to respond in Processor Target

Mode, the host adapter appears on the SCSI bus as a normal

processor-type device as defined by the SCSI specification. From one

to eight LUNs may be supported, depending on the LUN mask byte in the

Enable Target Mode command. Five SCSI commands are accepted:

TEST UNIT READY

REQUEST SENSE

INQUIRY

SEND

RECEIVE

All other commands are rejected with CHECK CONDITION status. The sense

information will indicate a Sense Key of 05H (Illegal Request) with a

Sense Code of 20H (Invalid Command Operation Code).

The commands that do not perform data transfer to or from the host are

handled completely by the AHA-1740/1744 with no CCB communication with

the host system. Those commands are the TEST UNIT READY, REQUEST

SENSE, and INQUIRY commands. The SEND and RECEIVE commands must have a

CCB from the host with the proper direction bits, the proper initiator

address, and the proper LUN number to complete the SCSI operation. The

SEND and RECEIVE CCBs may be provided to the host adapter before a

command is received on the SCSI or may be requested after the command

is received.

Each time an initiator activates a command to the AHA-1740/1744, an

internal subchannel is activated to manage the command. The subchannel

is dedicated to that particular LUN-Initiator transaction until all

operations associated with the command are completed. Such operations

include disconnection to obtain a CCB, pending error conditions, and

linked operations. If all subchannels are busy, a selection to the

AHA-1740/1744 will result in the AHA-1740/1744 accepting the command

and then generating BUSY status immediately. If this occurs, the

initiator must reissue the command later.

9-4

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

9.2.3.1 Test Unit Ready

[Picture]

The TEST UNIT READY command follows the SCSI specification in all

respects. If the AHA-1740/1744 has been initialized by the Enable

Target Mode command to the Processor Target Mode, the command will

finish normally with GOOD status and a COMMAND COMPLETE message. The

host adapter supports the normal definition of UNIT ATTENTION on the

first operation after power-on, after a SCSI Reset, or after a BUS

DEVICE RESET.

If a TEST UNIT READY command is executed against an LUN which was not

allowed by the Enable Target Mode command, then CHECK CONDITION status

will be presented with sense data of Sense Key 5 (Invalid Request) and

an Error Code of 25H (Invalid LUN).

In order to minimize interference with peripheral devices. TEST UNIT

READY commands are issued only every 250 microseconds when waiting for

a device to come ready.

9.2.3.2 Request Sense

[Picture]

9-5

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

If the AHA-1740/1744 has returned CHECK CONDITION status to a previous

command, the REQUEST SENSE command will obtain the sense information

associated with the error. The sense information will be sent in the

extended sense format according to the SCSI standard. The data format

is given in the following figure:

[Picture]

The following errors are detected and presented by the AHA-1740/1744

while operating in Processor Target Mode:

------------------------------------------------------------------------------

Sense    Additional Sense

Error                      Key          Code

------------------------------------------------------------------------------

No Sense Data                           00           00

Invalid Command Operation Code          00           20

Invalid LUN                             05           25

Invalid Command Parameter               05           26

Power-Up Attention                      06           29

Reset Attention                         06           29

Interface Parity Error                  0B           47

Initiator Detected Error                0B           48

Dumb Initiator                          05           2B

One set of error data may be buffered for each initiator_LU

association possible, up to a total of 56 sets of sense data. No

Contingent Allegiance or Extended Contingent Allegiance state is

established.

The Error Code (Byte 0) will be F0H if the residue field is valid and

70H if the residue field has no information.

The Incorrect Length Indicator (ILI) will be set if an incorrect data

transfer length is executed as described in Section 9.2.4.

The residue is set equal to the transfer length requested in the

initiator CDB minus the target host's specified data length specified

as a 4-byte two's complement number.

9-6

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

9.2.3.3 Inquiry

[Picture]

The INQUIRY command provides the information necessary to uniquely

identify the Adaptec AHA-1740/1744 as a Processor-type device. The

information is returned in the SCSI-2 format. The following

information is returned to any selecting initiator from any selected

AHA-1740/1744 Logical Unit:

[Picture]

If the INQUIRY command is attempted against a Logical Unit that has

not been enabled as a target, byte 0 is returned as 23H, indicating

that the LUN is not installed, but would be a Processor Device if it

were installed. The remaining bytes are returned normally.

If a length shorter than the required 36 bytes is specified by the

INQUIRY command, the number of bytes specified by the command is

transferred. If a length longer than 36 bytes is specified, the

command will only transmit 36 bytes.

9-7

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

9.2.3.4 Send

[Picture]

The SEND command has the format specified by the SCSI standard.

The SEND command transfers data from the initiator to the target. The

information is placed in the area specified by the appropriate CCB. If

an appropriate CCB has not already been provided to the host adapter

by the host software, an MBI entry requesting the appropriate CCB is

sent to the host from the host adapter. In this case, the target host

adapter disconnects from the SCSI until the CCB is made available to

the host adapter through the MBO protocol. An appropriate CCB must

have the same initiator address, target LUN, and direction as is

required to complete the command.

The transfer length in the SEND command specifies the length in bytes

of data that is sent during the DATA OUT phase. A transfer length of

zero indicates that no data is sent. Management of incorrect length

transfers is described in Section 9.2.4.

The CDB information is included in the returned CCB so that the

receiving host programming can determine whether the information

transmitted by the SEND command was application data or asynchronous

event notification data.

9.2.3.5 Receive

[Picture]

The RECEIVE command has the format specified by the SCSI standard

9-8

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

The RECEIVE command transfers data from the target to the initiator.

The information is taken from the area specified by the appropriate

CCB. If an appropriate CCB has not already been provided to the host

adapter host software, an MBI entry requesting the appropriate CCB is

sent to the host from the host adapter. In this case, the target host

adapter disconnects from the SCSI bus until the CCB is made available

to the host adapter through the MBO protocol. An appropriate CCB must

have the same initiator address, target LUN, and direction as is

required to complete the command.

The transfer length in the RECEIVE command specifies the length in

bytes of data that is sent during the DATA IN phase. A transfer length

of zero indicates that no data is sent. Management of incorrect length

transfers is described in Section 9.2.4.

9.2.4 Incorrect Length Management for Target Mode Operation

The messages transmitted using the SEND and RECEIVE commands are

normally expected to have a length that has previously been agreed to

by the initiator system software and by the target system software. If

the transfer length specified by the command is equal to the transfer

length specified by the CCB, normal operation takes place and no

errors are posted.

If the transfer length specified by the initiator's command is shorter

than the space defined by the target CCB, all data bytes expected and

required by the initiator will be transmitted. The target

AHA-1740/1744 will indicate GOOD status on the SCSI at the end of the

transfer. The target system's software, however, must be notified that

the entire area of data defined by the CCB was not transmitted. A

Target Status of GOOD will be presented in the returned CCB. At the

same time, the Incorrect Length Indication bit (bit 5 of byte 2) will

be set in the CCB Request Sense data area. Bytes 3 through 6 of the

Request Sense data area will contain the residue in two's complement

notation of the length requested in the initiator command minus the

length of the data area defined by the CCB. In this case, the residue

will be a negative number, since the requested length was less than

the area specified by the CCB. The HA status stored in the CCB will be

12h, indicating a Data Over/Under Run. The MBI Status Byte will be set

to 04 to indicate that the CCB was completed with an error.

If the transfer length specified by the initiator's command is longer

than the space defined by the target CCB, only those bytes contained

within the CCB's data transfer area will be transmitted. The target

AHA-1740/1744 must indicate, with an error condition to the initiator,

that not all the requested bytes could be transferred. The target

AHA-1740/1744 presents a SCSI status of CHECK CONDITION at the end of

the data transfer. The Request Sense information transmitted to the

initiator as a result of an immediately following REQUEST SENSE

command indicates that an Incorrect Length Indication is present by

setting bit 5 of byte 2. Bytes 3 through 6 of the Request Sense

information transmitted to the initiator will contain the residue in

two's complement notation of the length requested in the initiator

command minus the length of the data area defined by the target CCB.

In this case, the residue will be a positive number, since the

requested length was greater than the available area. The target

system's software must also be notified that the transfer length

requested by the initiator exceeded the assigned buffer area. A Target

Status of CHECK CONDITION will be presented in the returned target

CCB. A Host Status of '12'H will be returned indicating a Data

Over/Under Run. At the same time, the information that will later be

posted to the initiator by the REQUEST SENSE command will be posted to

the target system in the CCB Request Sense data area. This includes

both the Incorrect Length Indicator and the Residue. The MBI Status

Byte will be set to 04 to indicate that the CCB was completed with an

error.

9-9

Section 9                                                        SCSI Features

------------------------------------------------------------------------------

9.2.5 Aborting Target Mode Commands

The target host may abort a command by sending an MBO Command of 02H

to the host adapter. If execution of the command has not already been

completed, the CCB will be aborted and a MBI Status of 02H will be

returned to the host. Otherwise, an MBI Status of 03H will be

returned.

The initiator may also abort commands by sending an abort message to

the target. All CCBs, either received or requested from the target

host for the current initiator and LUN, and corresponding to commands

received from that initiator, will be aborted. An MBI Status of 03H

will be aborted. Any CCB provided by the target host for which a

corresponding initiator command has not been received will not be

aborted.

9-10

Section 10                                              Problem Determination

------------------------------------------------------------------------------

10.1 SELF DIAGNOSTIC CAPABILITY

The AHA-1740/1744 executes self diagnostics upon power-up or after a

hard reset. These diagnostics test the CPU operation, perform a sum

check on the EPROM, and check the data transfer paths on the board.

The host system has the option of exercising more extensive

diagnostics involving reads and writes to memory.

The red light emitting diode (LED) on the host adapter indicates the

result of the self diagnostic process. When power is first applied to

the board, the LED turns on. If the board is operating normally, the

light will soon go off and stay off until SCSI or I/O port activity is

requested by the host. If the board is not operating correctly, a

flash code number is flashed on the LED to indicate which test failed.

The flash code number is indicated by a series of one to three closely

spaced flashes followed by a longer pause. The flash code is repeated

continuously until the board is powered off, reset, or repaired. The

SCSI interface should be disconnected if these diagnostic tests are

being run for fault isolation purposes. At least one set of

terminators must remain installed or the LED will stay on, indicating

that the AHA-1740/1744 is receiving an active RST signal. Continuous

execution of the diagnostics can also be forced by inserting the

diagnostic jumper (pin pair 2 in jumper J1). The flash code and

associated failure modes are indicated in the following table.

FLASH CODES

---------------------------------------------------------------------

Flash Code          Possible Meanings of Flash Code

---------------------------------------------------------------------

LED Remains On      Host Adapter Control Processor

inoperative/terminators missing or not

powered or card enable has not been

asserted after reset.

1 Flash             RAM test failed.

2 Flashes           AIC-6251 SCSI protocol chip verification

failed

3 Flashes           FIFO write/read data path test failed.

Continuous Flashes  EEPROM has not been programmed.

---------------------------------------------------------------------

If any of these failure conditions is identified, the host computer

should be powered-down, the host adapter removed, and the host adapter

inspected for physical damage. Such damage can include EPROMs that are

not correctly installed or not firmly seated, broken wires, missing or

damaged components, or conductive debris on the board. If no such

physical damage is found, the AHA-1740/1744 should be returned for repair.

10-1

Section 10                                              Problem Determination

------------------------------------------------------------------------------

All boards are fully tested, burned in, cleaned, and inspected before

they are shipped. Care should be taken to keep the board in its

protective conductive wrapping until it is installed. With these

simple precautions, mechanical damage can normally be avoided.

10.2 INDICATORS

The red LED on the AHA-1740/1744 is used to provide the fault

isolation information described above. In addition, the LED indicates

when the host adapter is performing activities on the host interface

and the SCSI interface. The LED is such a useful activity indicator

that a connector is made available on the host adapter to allow the

cable to an externally visible LED to be attached. (See Section 3.3.9.)

As an activity indicator the LED is turned on from the time an Adapter

Command is transmitted to the host adapter until the HACC interrupt is

generated to indicate that the Adapter Command is complete. In

addition, the LED will be turned on whenever SCSI bus activity is

occurring. This is roughly the same period of time the SCSI BSY signal

is present on the SCSI bus. If the light stays on when no activity is

expected to be present on the bus, it is possible that the bus is

hung, the processor has failed, unexpected activity is occurring, or

the SCSI cables are incorrectly installed. If the SCSI cables are

installed reversed, the host adapter is forced into a solid SCSI reset

state which halts normal operation.

10.3 PROBLEMS DETECTED DURING OPERATION

The information in Section 10.3 is not intended for routine users of

the AHA-1502B/1542B. It is intended to provide a reference for

programmers preparing device drivers, error recovery procedures, and

error information presentation programs.

Operation of the I/O Port interface is controlled and monitored by

host system software. Two bits in the Status Port (Base + 0),

described in Section 4.2.1, are provided to indicate unusual

conditions in the host adapter.

Internal Diagnostic Failure (Status Port, bit 6): This bit indicates

that the self-testing process after either a hard reset or a power-on

operation was terminated by an error condition. The error indicates

that a critical failure was found in the AHA-1740/1744 control

circuitry or data paths. The error should be presented to the host

video display to indicate that diagnostic actions are required. The

diagnostic action uses the host adapter diagnostic flash codes to

determine if the AHA-1740/1744 is failing. The host system should be

powered off, then opened so the LED can be observed. All SCSI cables,

both external and internal, should be removed from the host adapter.

The system should then be powered on. If the LED flashes once and

turns off, the host adapter has passed its diagnostics and should be

accessible again. If the LED flashes one of the error flash codes, the

AHA-1740/1744 should be replaced. If the LED remains on and does not

turn off, the host adapter is not able to begin operation at all. The

host system may be holding the AHA-1740/1744 in the reset state or the

host adapter may have failed or the card may not have been enabled. To

distinguish these two cases, the AHA-1740/1744 should be replaced and

the new host adapter operation should be verified. Be sure to

power-down the host system when removing or replacing the host

adapter. If the diagnostics indicate that the AHA-1740/1744 must be

replaced, it should be sent to an authorized service facility for repair.

10-2

Section 10                                              Problem Determination

------------------------------------------------------------------------------

Invalid Host Adapter Command (Status Port, bit 0):  This bit is set to

indicate that a command or parameter transmitted to the host adapter

was invalid. This is an indication that the host software, usually an

operating system, special application program, or device driver, has

generated an invalid request. The responsible programming organization

should be notified so that the invalid sequence or parameter can be

corrected.

10.3.1 HA Status Error Indications and Corrective Actions

The Host Adapter Status indications, in addition to indicating several

normal completion states, indicate three general groups of errors. One

set describes software errors in the requests made to the host

adapter. Even though most of these errors are caused by software

design problems, the AHA-1740/1744 may need to be replaced as an

isolation step, since there exists a small chance that hardware

failures in the AHA-1740/1744 cause the appearance of a software

error.

A second set describes errors detected by the host adapter firmware in

the host adapter hardware. The most likely failing component in this

case is the AHA-1740/1744, although certain system or cable failures

may generate the appearance of a host adapter hardware error.

A third set describes the appearance of unexpected or incorrect

sequences executed by the attached SCSI devices. In this case, the

cables are the most likely failure point, with the peripheral device

second and the AHA-1740/1744 third. The host adapter status code

should be returned to the host video display to indicate to the

operator what errors have occurred and under what conditions the

errors occurred. The following section describes those Host Adapter

Status indications that indicate an error.

10-3

Section 10                                              Problem Determination

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Number  Type     Error  Description and Corrective Action

------------------------------------------------------------------------------

11      SCSI     NO     Selection Time Out: The SCSI attempted

to select a device that was not

installed or that did not respond to

selection due to a power, parity or

addressing failure. Verify correct

address values set to SCSI devices.

Verify that SCSI cable routing

includes the required devices. Verify

SCSI cable integrity by replacement of

cables. Verify that SCSI Selection

Time-Out value has been correctly

established by Adapter Command.

12      HOST     NO     Data Over Run/Under Run: Data length

or direction specified by CCB did not

agree with the data length actually

provided by the attached peripheral

device. Often a normal error or

accompanied by a check condition

indicating transfer truncation. Verify

program requested correct length or

direction. Verify peripheral provided

expected data length, number of

blocks, or block length.

13      SCSI     YES    Unexpected Bus Free: The target

dropped BSY without executing the

proper messages first. This normally

indicates that the BSY and/or other

portions of the SCSI bus failed or

that the target encountered such an

invalid sequence that no recovery was

possible. Some targets may have sense

information available to qualify the

error condition. Verify SCSI cable

integrity. Verify SCSI cables are all

properly connected. Replace SCSI

cables. Replace peripheral. Replace

AHA-1740/1744.

14      SCSI     YES    Target bus phase sequence failure:

The initiator detected an invalid

phase or an invalid phase sequence. If

this occurs, it is likely that the

host adapter forces a SCSI reset on

the bus as the first phase of the

recovery process. Verify SCSI cable

integrity. Verify SCSI cables are all

properly connected. Replace SCSI

cables. Replace peripheral. Replace

AHA-1740/1744.

10-4

Section 10                                              Problem Determination

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Number  Type     Error  Description and Corrective Action

------------------------------------------------------------------------------

15      HOST     YES    MBO Command Byte invalid:

This indicates a software failure or

bug in the host's development of the

MBO entry. Generally, this indication

will only occur during initial debug

processes of new operating systems.

Replace the AHA-1740/1744. Replace the

host computer system. Contact the

software developer for design support.

16      HOST     YES    Invalid CCB Operation Code: This

indicates a software failure or bug in

the host's development of the CCB.

Generally, this indication will only

occur during initial debug processes

of new operating systems. Replace the

AHA-1740/1744. Replace the host

computer system. Contact the software

developer for design support.

17      HOST     YES    Linked CCB does not have same LUN:

This indicates that the host software

generated an invalid combination of

link commands. Generally, this

indication will only occur during

initial debug processes of new

operating systems. Replace the

AHA-1740/1744. Replace the host

computer system. Contact the software

developer for design support.

18      HOST     YES    Invalid Target Direction Parameters

received from Host: This indicates

that the host software generated an

invalid Target Direction Parameter.

Generally, this indication will only

occur during initial debug processes

of new operating systems. Replace the

AHA-1740/1744. Replace the host

computer system. Contact the software

developer for design support.

19      HOST     YES    Duplicate CCB Received in Target Mode:

This indicates that the host system

was not keeping correct management

information for target mode operation

and incorrectly generated a second CCB

identical to one already active.

Generally, this indication will only

occur during initial debug processes

of new operating systems. Replace the

AHA-1740/1744. Replace the host

computer system. Contact the software

developer for design support.

10-5

Section 10                                              Problem Determination

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Number  Type     Error  Description and Corrective Action

------------------------------------------------------------------------------

1A      HOST     YES    Invalid CCB Parameter or Segment List:

A segment list was presented to the

host adapter with a zero length

segment or invalid segment boundaries.

An invalid CCB parameter was presented

to the host adapter. Generally, this

indication will only occur during

initial debug processes of new

operating systems. Replace the

AHA-1740/1744. Replace the host

computer system. Contact the software

developer for design support.

------------------------------------------------------------------------------

10.3.2 SCSI Error Indications and Corrective Actions

Error conditions detected by SCSI peripherals usually cause a CHECK

CONDITION Status byte to be presented. When this is presented, the

host adapter automatically retrieves the sense information from the

SCSI peripheral by executing a REQUEST SENSE command according to the

SCSI standard. The information returned is mapped for single-threaded

BIOS operation into the categories of error conditions described in

Section 5.2.2. For multitasking (mailbox) operation, the information

returned is made available in the area allocated for Sense Data. In

either case, an intelligent attempt is made by most operating systems

to retry the operation at least one time. Such retry operations may

require the management of queued operations that have already started

execution. This attempt to retry the operation is rarely successful,

since SCSI peripheral devices have very extensive automatic retry and

correction mechanisms designed to be executed before the error

information is returned in the first place.

Since it is rare that such an operation is successful, the host

software and operating systems should make every attempt to make

available key information about the error which can be analyzed by the

system operator or customer engineer. In small systems, it may be

sufficient to present the important Sense Data together with the

command that created the error. Some simpler systems may perform a

preliminary analysis of the error information and generate a code or

descriptive text that describes the error and indicates the corrective

action. In very complex systems, a logging process may take place,

allowing a customer engineer to analyze the data at some future time.

In all systems, such information is very important for host program

verification and for system integrity verification, even if the

designers choose not to make the information available to any system

users other than the design and maintenance engineers.

The SCSI Sense Data is generally self descriptive. The errors can be mapped

into four major categories, each category with its own diagnostic procedure.

10-6

Section 10                                              Problem Determination

------------------------------------------------------------------------------

The first category includes those errors caused by incorrect command

or parameter bytes or by incorrect sequences of commands. These errors

typically are presented only during the early development stages of an

operating system or device driver, since a properly operating program

will not generate invalid commands.

The second category includes those sense codes associated with

peripheral status presentation. Such sense codes are not truly errors,

in that they inform the host program of an unusual but not unexpected

condition. Such sense codes include indications that a device is not

ready, that the device has just become ready, that a device has

reached the end of its media (End of Tape or Blank Check), or that the

expected data length and actual data length differed. These sense

codes are normally used by the device driver to execute the correct

operation in response to the condition. Such sense codes are not

normally presented to the system user except as text that requests

some action. As one example, an End Of Tape condition may require the

system user to replace the tape cartridge.

The third category of errors points to a particular hardware failure

in the peripheral device or its supporting electronics. Such error

conditions usually require the adjustment, repair, or replacement of

the peripheral device or some of its components. In some cases, the

error condition may also indicate possible cable or host adapter

failures. These errors must be exposed to the system user so that the

proper actions can be taken.

The fourth category of errors points to a media failure in the

peripheral device. Such errors include bits that cannot be recovered

from magnetic media and imperfections in the surface of optical media.

Most such errors are recovered using the extensive retry and

correction algorithms programmed into the peripheral device. In some

peripheral device technologies, the errors may be caused by noise

conditions or by marginal electronic failures in the read or write

path. Those rare errors that cannot be recovered usually indicate that

some data important to the operating system or application has been

lost. The system must make this error information available to the

system user so that the system user can replace the media or recover

the data from a backup copy as required. In addition, logging of

errors that were successfully recovered by the peripheral device is

often useful as an indicator of the overall reliability of the device

or of the requirement for periodic maintenance.

10.4 PROBLEMS DETECTED DURING INSTALLATION

This section may be useful to correct problems related to

installation. The information in this section is included in the Host

Adapter Installation Guide.

If the system will not boot from the flexible diskette drive after

initial hardware installation the following items should be checked:

�  AHA-1740/1744 internal diagnostics: The LED on the

AHA-1740/1744 should come on briefly at system power-up. If the

LED begins to blink at regular intervals then the host adapter

has detected an internal failure and should be returned for

repair or replacement to the place of purchase. A message may

also be posted to the screen.

�  If the AHA-1740/1744 LED and the SCSI drive LED are always on,

the SCSI cable's pin 1 orientation has probably been reversed

between the host adapter and the drive. See Section 3.2.

10-7

Section 10                                              Problem Determination

------------------------------------------------------------------------------

�  Is the AHA-1740/1744 BIOS message displayed on the screen? If

not, the AHA-1740/1744 BIOS is not being recognized by the

system.

a)    Check for BIOS address conflicts between the

AHA-1740/1744 and other option boards.

b)    Try a different BIOS address. See Section 3.3.

c)    Change the BIOS wait state jumper. See Section 3.3.

�  If the HOST ADAPTER NOT FOUND AT PORT 330H message is

displayed check the port address setting. Also verify that the

SCSI cable is correctly installed. A forced SCSI reset caused by

an inverted or displaced internal SCSI cable may cause the

problem. See Section 3.3.

Problems Booting the System from a SCSI drive:

�  Make sure that both standard hard disks are mapped out of the

system.

�  Make sure that the SCSI boot drive address is set to SCSI ID

0:0. Check the drive installation manual for information about

setting the SCSI ID for that device. The Return Installed Devices

utility in the Onboard Utilities can also be used to determine

the SCSI addresses of peripherals on the SCSI bus.

�  Make sure that SCSI parity is consistently enabled or disabled

on all devices on the SCSI bus.

�  Verify that the host adapter and the SCSI devices are properly

configured and installed by referencing Section 3.

�  Power should be cycled off and on after changing any values on

an HA, Setup program, or SCSI device to be sure that a DOS format

operation has been successfully completed.

�  Make sure that the SCSI bus is properly terminated.

�  Make sure that the intended boot disk has an active DOS

partition and a DOS format. See Section 3.0 and the DOS manual

for more information.

�  Check the cabling.

Problems using a SCSI drive as drive D: with a standard hard disk as drive C:

�  Make sure that the second hard disk is mapped out of the system.

�  Make sure that the SCSI drive to be used as drive D: is set

to SCSI ID 0:0. Check the drive manual for information on setting

the SCSI ID for that device. The Return Installed Devices utility

in the Onboard Utilities can also be used to determine the SCSI

addresses of peripherals on the SCSI bus.

10-8

Section 10                                              Problem Determination

------------------------------------------------------------------------------

�  Make sure that SCSI parity is consistently enabled or disabled

on all devices on the SCSI bus.

�  Verify that the host adapter and the SCSI devices are properly

configured and installed by referencing Section 3.

�  Power should be cycled off and on after changing any values on

an HA, Setup program or SCSI device to be sure that the new

initial values are loaded.

�  Make sure that the SCSI bus is properly terminated. See Section 3.

�  Make sure that the disk has a DOS partition and a DOS format.

See Section 5 and the system DOS manual for more information.

�  Check the cabling.

Problems using a SCSI drive as drive D: with another SCSI drive as drive C:

�  Make sure that both standard hard disks are mapped out of the

system with the AT SETUP program.

�  Make sure that the SCSI drive to be used as drive D: is set

to SCSI ID 0:1 or 1:0. Check the drive manual for information on

setting the SCSI ID for that device. The Return Installed Devices

utility in the Onboard Utilities can also be used to determine

the SCSI addresses of peripherals on the SCSI bus.

�  Power should be cycled off and on after changing any values on

an HA, Setup program or SCSI device to be sure that the new

initial values are loaded.

�  Make sure that SCSI parity is consistently enabled or disabled

on all devices on the SCSI bus.

�  Verify that the host adapter and the SCSI devices are properly

configured and installed by referencing Section 3.

�  Make sure that the SCSI bus is properly terminated. See Section 3.

�  Make sure that the disk has a DOS partition and a DOS format.

See Section 3 and the DOS manual for more information.

System works erratically. Hangs or the host adapter can't always find

the drives.

�  Check SCSI parity for consistency.

�  Check Termination.

�  Check cable length and integrity.

10-9

Section 10                                              Problem Determination

------------------------------------------------------------------------------

�  If host adapter and drive light remain on during a hang

condition, make sure that the SCSI drive conforms to the Common

Command Set revision 4B (CCS 4B).

�  If only the host adapter LED remains on during a hang, it is

probably a host adapter, system interface problem. The system may

not be capable of First-Party DMA transfers. Check with the

System manufacturer for information.

10-10

Section 11                                                            Glossary

------------------------------------------------------------------------------

Adapter Command              A command transmitted to the host

adapter using the Command/Data Out Port

and the Data In Port. The commands are

sequenced using the Control Port, the

Status Port, and the Interrupt Flag

Port. Abbreviated as IOCP command.

AEN                          See Asynchronous Event Notification

AHA-1540                     Adaptec host adapter for connecting

SCSI devices to the PC AT backplane bus.

The AHA-1740/1744 is an enhancement of

the AHA-1540 (see Appendix C)

AHA-1540B                    The enhanced high-performance Adaptec

host adapter for connecting SCSI devices

to the PC AT backplane bus.

AHA-1542B                    The enhanced high-performance Adaptec

host adapter for connecting SCSI devices

and standard IBM-compatible floppy disk

devices to the PC AT backplane bus.

AHA-1740/1744                Either the AHA-1740 or the AHA-1744 or both.

ASPI                         Advanced SCSI Programming Interface. A

software architecture which permits

device modules to migrate across

different hardware by communicating with

the hardware through a passthrough

interface to a manager written to the

specific hardware.

Asynchronous Event Notification A process by which a target can send

unsolicited sense information to an

initiator using the SEND command in

order to inform the initiator about the

occurrence of an important unusual

occurrence.

11-1

Section 11                                                            Glossary

------------------------------------------------------------------------------

Asynchronous Data Transfer   Data transfer performed by the SCSI

interface involving the interlocking of

a signal to the initiator (REQ) and a

signal to the target (ACK) such that

each step of the data transfer protocol

must occur before the next step can

begin. Characterized by a low data rate

and independence of external timing

constraints, including cable length and

circuit response times.

AT Bus                       The Industry Standard Architecture bus.

Bus Device Reset             A SCSI message that clears all activity in the

target to which it is addressed.

Byte                         An eight-bit unit of data. An octet. A

byte is normally the smallest

addressable unit of a memory and the

unit of transfer on the SCSI.

CCB                          See Command Control Block

CCS                          See Common Command Set

CDB                          See Command Descriptor Block

Command Control Block        A software object prepared by the host

microcomputer software for the host

adapter to provide it all the control

information it needs to execute a SCSI

command. Abbreviated CCB.

Command Descriptor Block     A block of information passed across

the SCSI bus to provide the command,

parameter, and address information

necessary for the target to execute the

desired functions. Prepared by the host

software and placed in the CCB to be

passed to the target by the host

adapter. Abbreviated CDB.

Common Command Set           A defacto standard SCSI command set

for communication with hard disk drives.

The Common Command Set (CCS) is the

basis for the SCSI-2 command set for all

types of peripheral devices.

Compatible Mode              The operation mode of the

AHA-1740/1744 to allow software drivers

written for the AHA-1540/1640 family to

operate fully on the board. This has no

performance limitations but does not

allow addressing beyond 16 MBytes nor

access to several EISA registers.

11-2

Section 11                                                            Glossary

------------------------------------------------------------------------------

Configuration                The operation of configuring a device

on the EISA bus through access of

registers in the device by the host. It

replaces the method of using jumpers

common on ISA bus devices.

Control Microprocessor       An integrated circuit computer used to

execute the software that controls the

host adapter's operation.

Device Driver                A program that is linked with, or

attached to, an operating system to map

the software interface of the operating

system to the requirements of attached

peripheral devices and host adapters.

Under APSI, the main component

associated with the board is known as a

Manager.

DMA                          See Direct Memory Access

Differential                 A term referring to the electrical

characteristics of the signals used on

the SCSI Bus interface. Differential

signals occupy two conductors with a

positive (+) and negative (_) polarity

component of the signal. This minimizes

the effect of common mode signal noise

and allow the SCSI bus to operate

reliably over greater distances at a

higher speed.

Direct Memory Access         A mechanism that allows hardware

control of the transfer of streams of

data to or from the main memory of a

computing system. The mechanism may

require setup the host software. After

initialization, it automatically

sequences the required data transfer and

provides the necessary address

information.

EISA                         Extended Industry Standard

Architecture. A superset standard of the

16- and 8-bit ISA standard which allows

32 bits of data to be transferred across

the bus at up to 33 MBytes/second.

Enhanced Mode                The operation mode of the

AHA-1740/1744 to take full advantage of

the addressing range and register set

available under EISA. It is not

compatible with earlier revisions of

ASPI Managers and drivers.

EEPROM                       Electrically-Erasable Programmable

Read Only Memory. An integrated circuit

used to store the host adapter firmware,

which allows both mode download and

firmware upgrade while in-circuit.

11-3

Section 11                                                            Glossary

------------------------------------------------------------------------------

EPROM                        Erasable Programmable Read Only

Memory. An integrated circuit used to

store the host adapter BIOS.

FIFO                         First In/First Out.

A queuing order in which items are

removed from the queue for execution in

the same order in which they are placed

in the queue.

An integrated circuit that buffers

data in such a manner that each byte

placed in the buffer is removed from the

buffer in the same order.

Firmware                     The software that controls and manages

the host adapter. It is 'firm' as

opposed to 'soft' because it is designed

into the host adapter and cannot be

modified by the user.

IBM PC AT Compatible         Any computer system that emulates

exactly the IBM PC AT and that uses an

ISA backplane bus.

Industry Standard Architecture  The IBM PC AT functions have been

duplicated by a number of manufacturers.

All the IBM PC AT compatible machines

use a backplane bus that very closely

emulates the function of the backplane

bus of the PC AT. Because of the broad

usage of this bus structure, it has

become known as the Industry Standard

Architecture bus, even though there is

no presently accepted standard for the bus.

Initiator                    A SCSI device that requests an

operation to be performed by another

SCSI device (the target). The initiator

provides all the command information and

parameters required to perform the

operation, but the details of the

operation are actually sequenced by the

target.

ISA                          See Industry Standard Architecture

Host                         A microcomputer in which a host

adapter is installed. The host uses

software to request the services of the

host adapter in transferring information

to and from peripheral devices attached

to the SCSI bus connector of the host

adapter.

11-4

Section 11                                                            Glossary

------------------------------------------------------------------------------

Host adapter                 A hardware printed circuit board that

installs in a standard microcomputer

backplane and provides a SCSI bus

connection so that SCSI devices can be

connected to the microcomputer. A host

adapter is "intelligent" if it has a

simple high-level software interface to

the microcomputer. A host adapter is

"dumb" if the microcomputer must

directly manage the SCSI protocol using

the microcomputer processor.

Logical Unit                 A physical or virtual device addressed

through a target.

Logical Unit Number          An encoded three-bit identifier for a

logical unit.

LU                           See Logical Unit

LUN                          See Logical Unit Number

Mailbox In                   An area in main memory assigned by the

host microcomputer software for

communication with the host adapter. The

host adapter places status and pointer

information in entries in the Mailbox In

(MBI) to indicate to the host

microcomputer what operations have been

completed or what information must be

obtained from the host microcomputer.

Mailbox Out                  An area in main memory assigned by the

host microcomputer software for

communication with the host adapter. The

microcomputer software places commands

and pointer information in entries in

the Mailbox Out (MBO) to indicate what

operations should be started by the host

adapter.

Manager                      The component of a driver which is

specific to a particular board

architecture and presents a standard

ASPI pass-through interface for use by

the peripheral-specific component of the

driver, known as a module.

MBI                          See Mailbox In.

MBO                          See Mailbox Out.

11-5

Section 11                                                            Glossary

------------------------------------------------------------------------------

Multitasking Operation       The execution of commands in such a

way that more than one command is in

progress at the same time, allowing the

system to take advantage of overlapping

activities by using resources that are

temporarily not required for other

operations. More than one program or

more than one portion of a program may

be operating in parallel.

PC AT                        A family of small computers sold by

IBM, also called the Personal

Computer/AT family of computers. The

name is trademarked by IBM.

RAM                          Random Access Memory. Memory of which

any byte can be accessed directly in a

single memory cycle. Information can be

read from and written to the memory.

SCB                          SCSI Control Block. The mechanism used

in Emulation mode to transfer control

information to and from the board. This

is equivalent to the mailbox in/out

system used in Compatible mode.

SCSI                         Small Computer System Interface.

SCSI ASC                     SCSI Additional Sense Code. Byte 12 of

the extended sense information. Provides

a standardized description of the

condition described by the sense

information.

SCSI Device                  A device attached to a Small Computer

System Interface bus cable. The device

may be an initiator, a target, or

capable of both types of operation. The

device may be a peripheral device, a

host device, or a device mixing both

roles.

Single-Ended                 A term referring to the electrical

characteristics of the signals used on

the SCSI Bus interface. Single-ended

signals occupy a single conductor and

are references to a common ground

carried on the cable between the SCSI

components attached.

Single-Threaded Operation    Operation of the computing system such

that only one program can be operating

or active at a time. The computing

system must wait until all resources are

available before starting an operation

and cannot start another operation until

the first one is completed. No

overlapping of latencies or program

operation occurs.

11-6

Section 11                                                            Glossary

------------------------------------------------------------------------------

Synchronous Data Transfer    A method of data transfer on the SCSI

bus involving clocking data on to the

bus with a fixed-length fixed-frequency

strobe pulses. The acknowledgements may

be delayed several clock periods from

the data requests. Synchronous Data

Transfer can be used only for data

transmission on the SCSI bus. It is

prohibited for command, message, and

status transmission.

Sync Data Transfer Negotiation  The message exchange between the

initiator and the target that allows the

negotiation of the data transfer

frequency and delay between requests and

acknowledgements required for

synchronous data transfer. Once

negotiated, synchronous data transfer

parameters remain unchanged until

certain reinitialization activities

occur.

Target                       A SCSI device that performs an

operation requested by an initiator. The

target may be a peripheral device

performing a service for an initiator.

The target may also be a host adapter

performing a Processor-type device

service for an initiator.

Word                         A 2-byte (16-bit) unit of data.

11-6

This page intentionally left blank.

Appendix B                                                    Connector Pinout

------------------------------------------------------------------------------

INTERNAL CONNECTOR PIN ASSIGNMENTS

AHA-1740

(Single-Ended)

---------------------------------------------------------------

Signal Name      Pin       Pin           Signal Name

---------------------------------------------------------------

Ground           1         2             -DB(0)

Ground           3         4             -DB(1)

Ground           5         6             -DB(2)

Ground           7         8             -DB(3)

Ground           9         10            -DB(4)

Ground           11        12            -DB(5)

Ground           13        14            -DB(6)

Ground           15        16            -DB(7)

Ground           17        18            -DB(P)

Ground           19        20            Ground

Ground           21        22            Ground

Ground           23        24            Ground

Open             25        26            Term Power (Fused)

Ground           27        28            Ground

Ground           29        30            Ground

Ground           31        32            -ATN

Ground           33        34            Ground

Ground           35        36            -BSY

Ground           37        38            -ACK

Ground           39        40            -RST

Ground           41        42            -MSG

Ground           43        44            -SEL

Ground           45        46            -C/D

Ground           47        48            -REQ

Ground           49        50            -I/O

---------------------------------------------------------------

B-1

Appendix B                                                    Connector Pinout

------------------------------------------------------------------------------

INTERNAL CONNECTOR PIN ASSIGNMENTS

AHA-1744

(Differential)

---------------------------------------------------------------

Signal Name      Pin       Pin           Signal Name

---------------------------------------------------------------

Ground           1         2             Ground

+DB(0)           3         4             -DB(0)

+DB(1)           5         6             -DB(1)

+DB(2)           7         8             -DB(2)

+DB(3)           9         10            -DB(3)

+DB(4)           11        12            -DB(4)

+DB(5)           13        14            -DB(5)

+DB(6)           15        16            -DB(6)

+DB(7)           17        18            -DB(7)

+DB(P)           19        20            -DB(P)

DIFFSENS         21        22            Ground

RESERVED         23        24            RESERVED

Term Power (Fused)         25        26            Term Power (Fused)

RESERVED         27        28            RESERVED

+ATN             29        30            -ATN

Ground           31        32            Ground

+BSY             33        34            -BSY

+ACK             35        36            -ACK

+RST             37        38            -RST

+MSG             39        40            -MSG

+SEL             41        42            -SEL

+C/D             43        44            -C/D

+REQ             45        46            -REQ

+I/O             47        48            -I/O

Ground           49        50            Ground

---------------------------------------------------------------

B-2

Appendix B                                                    Connector Pinout

------------------------------------------------------------------------------

EXTERNAL CONNECTOR PIN ASSIGMENTS

AHA-1740

(Single-Ended)

---------------------------------------------------------------

Signal Name      Pin       Pin           Signal Name

---------------------------------------------------------------

Ground           1         26            -DB(0)

Ground           2         27            -DB(1)

Ground           3         28            -DB(2)

Ground           4         29            -DB(3)

Ground           5         30            -DB(4)

Ground           6         31            -DB(5)

Ground           7         32            -DB(6)

Ground           8         33            -DB(7)

Ground           9         34            -DB(P)

Ground           10        35            Ground

Ground           11        36            Ground

Ground           12        37            RESERVED

Open             13        38            Term Power (Fused)

Ground           14        39            RESERVED

Ground           15        40            Ground

Ground           16        41            -ATN

Ground           17        42            Ground

Ground           18        43            -BSY

Ground           19        44            -ACK

Ground           20        45            -RST

Ground           21        46            -MSG

Ground           22        47            -SEL

Ground           23        48            -C/D

Ground           24        49            -REQ

Ground           25        50            -I/O

---------------------------------------------------------------

B-3

Appendix B                                                    Connector Pinout

------------------------------------------------------------------------------

EXTERNAL CONNECTOR PIN ASSIGMENTS

AHA-1744

(Differential)

---------------------------------------------------------------

Signal Name      Pin       Pin           Signal Name

---------------------------------------------------------------

Ground           1         26            Ground

+DB(0)           2         27            -DB(0)

+DB(1)           3         28            -DB(1)

+DB(2)           4         29            -DB(2)

+DB(3)           5         30            -DB(3)

+DB(4)           6         31            -DB(4)

+DB(5)           7         32            -DB(5)

+DB(6)           8         33            -DB(6)

+DB(7)           9         34            -DB(7)

+DB(P)           10        35            -DB(P)

DIFFSENS         11        36            Ground

RESERVED         12        37            RESERVED

Term Power (Fused)         13        38            Term Power (Fused)

RESERVED         14        39            RESERVED

+ATN             15        40            -ATN

Ground           16        41            Ground

+BSY             17        42            -BSY

+ACK             18        43            -ACK

+RST             19        44            -RST

+MSG             20        45            -MSG

+SEL             21        46            -SEL

+C/D             22        47            -C/D

+REQ             23        48            -REQ

+I/O             24        49            -I/O

Ground           25        50            Ground

---------------------------------------------------------------

B-4

Appendix C                                                 Register Reference

------------------------------------------------------------------------------

System Register Reference

Expansion Board ID's

Host ID 0            HID0         zC80      R

Host ID 1            HID1         zC81      R

Host ID 2            HID2         zC82      R

Exp. Board Control   EBCTRL       zC84      W/R

BMIC registers                    zC88 - zC9F

Group 1

I/O Port Address     PORTADDR     zCC0      W/R

BIOS Address         BIOSADDR     zCC1      W/R

Interrupt definition INTDEF       zCC2      W/R

SCSI definition      SCSIDEF      zCC3      W/R

Bus Definition       BUSDEF       zCC4      W/R

Reserved 0           RESV0        zCC5      W/R

Reserved 1           RESV1        zCC6      W/R

Reserved 2           RESV2        zCC7      W/R

Mailbox out byte 0   MBOXOUT0     zCD0      W/R

Mailbox out byte 1   MBOXOUT1     zCD1      W/R

Mailbox out byte 2   MBOXOUT2     zCD2      W/R

Mailbox out byte 3   MBOXOUT3     zCD3      W/R

Attention            ATTN         zCD4      W/R

Group 2 Control      G2CNTRL      zCD5      W/R

Group 2 INT. Status  G2INTST      zCD6      R

Group 2 Status       G2STAT       zCD7      R

Mailbox in 0         MBOXIN0      zCD8      R

Mailbox in 1         MBOXIN1      zCD9      R

Mailbox in 2         MBOXIN2      zCDA      R

Mailbox in 3         MBOXIN3      zCDB      R

Group 2 Status 2     G2STAT2      zCDC      R

Control Group 1 (Programmable location)

Control Port         CNTRL        Base address + 0   W

Status Port          STATUS       Base address + 0   R

Command/Data         CMD/DAT      Base address + 1   W/R

System Interrupt     INTRPT       Base + 2           R

C-1

Appendix C                                                 Register Reference

------------------------------------------------------------------------------

Local Control Registers

PORTB                                   00E2,00E3 W/R

PORTD                                   0104      R

PORTPL                                  0152      W

PORTPH                                  0153      W

Local Status Port          LSTAT        2000      W

Local Comd/Data Reg        LCMDDAT      2001      W/R

Local Interrupt Register   LINTRG       2002      W

Local Aux. Status Reg.     AUXSTAT      2003      R

Clear Reset                CLRRST       2003      W

Local Port Address         LPORTADDR    3000      R

Local BIOS address         LBIOSADDR    3001      R

Local Interrupt defn       LINTDEF      3002      R

Local SCSI definition      LSCSIDEF     3003      R

Local Bus definition       LBUSDEF      3004      R

Local Reserved 0           LRESV0       3005      R

Local Reserved 1           LRESV1       3006      R

Local Reserved 2           LRESV2       3007      R

Local mailbox out 0        LMBOXOUT0    3010      R

Local mailbox out 1        LMBOXOUT1    3011      R

Local mailbox out 2        LMBOXOUT2    3012      R

Local mailbox out 3        LMBOXOUT3    3013      R

Local mailbox in 0         LMBOXIN0     3014      W

Local mailbox in 1         LMBOXIN1     3015      W

Local mailbox in 2         LMBOXIN2     3016      W

Local mailbox in 3         LMBOXIN3     3017      W

Local Group 2 Control      LG2CNTRL     3018      W

Local Group 2 Status       LG2STAT      3019      R

Local Group 2 Int. Status  LG2INTST     301A      W

Local Group 2 Attention    LATN         301C      R

Local Exp. Board Ctrl      LEXBCTL      3020      W

Local Exp. Board Ctrl      LEXBCTL      3020      R

Local data register        LDAT         4000      W/R

Local Index register       LINDX        4001      W/R

Local control/status       LCONSTAT     4002      W/R

SCSI Control               SCSICNTRL    4004      W

EISA Word Count LSB        ECOUNTL      4005      W

EISA Word Count MSB        ECOUNTH      4006      W

SCSI Registers                     5000-500F      W/R

On Board RAM                       8000 - FFFF

C-2

Appendix D                                                 EISA Free-Form Data

------------------------------------------------------------------------------

SCSI Subsystem, Data Structure

|----------------------------------------------------------------------------|

|Byte 1 _ SCSI Device 0            Byte 0 _ SCSI Device 0                    |

|bits 7-3: Reserved                                                          |

|                                  bit 7:   Reserved                         |

|bit 2-0: Max synch transfer rate  bit 6:   More than 1 LUN supported        |

|    000 _ 10.0 MB/S               bit 5:   Parity Check enable              |

|    001 _  6.67 MB/S              bit 4:   Send Start command               |

|    010 _  5.0 MB/S               bit 3:   Synchronous negotiation enable   |

|    011 _  4.0 MB/S               bit 2:   Disconection enable              |

|    100 _ 3.33 MB/S               bit 1-0 Power on information              |

|                                  00   Not installed                        |

|                                  01   Installed, report error if not       |

|                                       found at power on                    |

|                                  10   Reserved                             |

|                                  11   Installed, do not report error if not|

|                                       found at power on                    |

|----------------------------------------------------------------------------|

|  Byte 3 - SCSI device 1          Byte 2 - SCSI device 1                    |

|    bit 7-0   Same as Byte 1        bit 7-0   Same as Byte 0                |

|----------------------------------------------------------------------------|

|  Byte 5 - SCSI device 2          Byte 4 - SCSI device 2                    |

|   bit 7-0   Same as Byte 1         bit 7-0   Same as Byte 0                |

|----------------------------------------------------------------------------|

|  Byte 7 - SCSI device 3          Byte 6 - SCSI device 3                    |

|   bit 7-0   Same as Byte 1         bit 7-0   Same as Byte 0                |

|----------------------------------------------------------------------------|

|  Byte 9 - SCSI device 4          Byte 8 - SCSI device 4                    |

|   bit 7-0   Same as Byte 1         bit 7-0   Same as Byte 0                |

|----------------------------------------------------------------------------|

|  Byte 11 - SCSI device 5     Byte 10 - SCSI device 5                       |

|   bit 7-0   Same as Byte 1        bit 7-0   Same as Byte 0                 |

|----------------------------------------------------------------------------|

|  Byte 13 - SCSI device 6     Byte 12 - SCSI device 6                       |

|   bit 7-0   Same as Byte 1        bit 7-0   Same as Byte 0                 |

|----------------------------------------------------------------------------|

|  Byte 15 - SCSI device 7     Byte 14 - SCSI device 7                       |

|   bit 7-0   Same as Byte 1        bit 7-0   Same as Byte 0                 |

|----------------------------------------------------------------------------|

|  Byte 17 - Reserved for future use Byte 16 - Reserved for future use       |

|----------------------------------------------------------------------------|

|  Byte 19 - Reserved for future use Byte 18 - Reserved for future use       |

|----------------------------------------------------------------------------|

|  Byte 21 - Reserved for future use Byte 20 - Reserved for future use       |

|----------------------------------------------------------------------------|

|  Byte 23 - Reserved for future use Byte 22 - Reserved for future use       |

|----------------------------------------------------------------------------|

|  Byte 25 - Reserved for future use Byte 24 - Reserved for future use       |

|----------------------------------------------------------------------------|

|  Byte 27 - Reserved for future use Byte 26 - Reserved for future use       |

|----------------------------------------------------------------------------|

|  Byte 29 - Reserved for future use Byte 28 - Reserved for future use       |

|----------------------------------------------------------------------------|

|  Byte 31 - Reserved for future use Byte 30 - Reserved for future use       |

|----------------------------------------------------------------------------|

|  Bytes 17-47 - Reserved for future use                                     |

|----------------------------------------------------------------------------|

D-1

This page intentionally left blank.

#