3-Drive Floppy/Tape Cable

PROBLEM - I have 1.2mb & 1.44mb Floppy Drives installed

in my 386 system, and wanted to add a CMS Tape

Backup System without giving up a floppy. To

save on the high price of adding the Tape Drive

board (and using up a slot!), I decide to make

a cable that will accomplish the same job!

SOLUTION - Make a cable to switch the "B" drive connector

from the floppy to the Tape Drive. I switch to

the tape. The cost is less than $10!

DETAILS - There are 2 critical lines that require switching.

These are named "Motor Enable B" and "Drive

Select B". A Double-Pole Double-Throw switch is all

that is required to switch between devices; and

a 34-Pin Edge connector.

PARTS - Radio Shaft: Edge connector 276-1564, Switch 275-663

CONSTRUCTION -

Added Edge Connector ���Ŀ

�

34-pin connector on controller                      #

"B"     "B"   "A"

Ŀ   10 Motor Enable A                           �Ŀ     �Ŀ   �Ŀ

�� ----------------------------------------------���-----���---���

��   12 Drive Select B                           ���     ���   ���

�����������������������������Ŀ          �������ĳ�� ���ĳ����ĳ��

��   14 Drive select A        �          �       ��� �   ���   ���

�� ---------------------------)----------)-------���-)---���---���

��   16 Motor enable B        �          �       ��� �   ���   ���

�����������������������Ŀ     �          �     �ĳ�� � �ĳ����ĳ��

��                      �     �          �     � ��� � � ���   ���

��                      �   12�      12  �     � ��� � � ���   ���

��                      �     �    /o�����     � ��� � � ���   ���

�     ��#/|  Sec. A  12�     � �

�         | o����������)������ �

�         |          16�       �

16�         |/o�����������       �

��������#/   Sec. B          16�

o�������������������

#

�

DPDT Switch

Above is the art work. I made a cable with 3-34 pin Edge

Connectors. Pins 10-16 are twisted between "A" and "B"

Pins 12 and 16 are cut at a convenient point, where they

come off the controller, and between each of the 2 "B" drive

connectors. Take a wire and solder on an extension and run

it to the Double-Pole Double-Throw Switch. Next run wires from

each side of the switch to the short pieces of wire coming

from the connectors where you previously cut, and solder the

connections. (I recommend using heat shrink tubing for in-

sulation on each connection.) Now run the cable to the "B"

Tape Drive, and then to the "B" Disk Drive. Mount the DPDT

switch in a convenient place and you're set.

I have 3-drive opening in my clone case, so I mounted the tape

drive on the bottom, and the switch in a small hole drilled in

the front panel of the Tape-Drive on the left side.

Ted J. Mieske

T.M.S. Micro