AT BIOS HARD DRIVE DOCUMENT

���������������������������

Version 1.2  July 1992

Copyright (C)1991,1992 David R. Green, All Rights Reserved

The following data has been collected from a variety of sources,

mostly from drive manufacturer specification sheets.

This information is for use with the AT BIOS program written by

David R. Green only.  Do not distribute copies of this information

by itself without the full ATBIOS package.

Always give copies of the complete AT BIOS program.  Please leave

all Copyright symbols intact.

This document is protected by the same License agreement included

with the AT-BIOS program.

ANYONE WHO UPDATES OR MAKES ADDITIONS OR CHANGES TO THIS

DOCUMENT IS REQUESTED TO SEND A COPY TO DAVID R. GREEN, SO IT

MAY BE PASSED ON TO FUTURE AT BIOS USERS.

IF YOU FIND THIS SOFTWARE USEFUL, PLEASE REGISTER.  SEE THE

README.EXE FILE FOR REGISTRATION INFORMATION.

INDEX:

1. Hard Drive Specifications

Kalok

Maxtor

Micropolis

Microscience

Miniscribe

Priam

Quantum

Rodime

Seagate

Western Digital

2. ST506/412 MFM/RLL Interface

3. IDE Interface

4. BIOS Settings

5. BIOS Error Codes

6. An Actual AT BIOS

7. AT Technical Reference Fixed Disk

8. Award BIOS Reference

�����������������������������������������������������������������

AT-BIOS

The following pages are divided by drive manufacturer, and show

many of the current drives being distributed, with their drive

specifications.

Hard Drive Specifications

�������������������������

MODEL    = manufacturer's model number designation

FORM     = Form Factor, Drive Physical Sizing

H        = Drive Height  (H=Half, F=Full, 1=1")

TYPE     = Interface type

CAPACITY = Formatted drive capacity

ACCESS   = Random Seek Time

CYL      = Number of Cylinders

HDS      = Number of Heads

SEC      = Number of Sectors per Cylinder

PRCMP    = Write Precomp Cylinder

A        = Actuator Type (V=Voice Coil, S=Stepper Motor)

�����������������������������������������������������������������

Hard Drive Specifications

KALOK

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

Octagon20 3.5"  H  MFM    21.4MB    40ms   615   4   17   none   S

Octagon30 3.5"  H  RLL    32.8MB    40ms   615   4   26   none   S

Octagon40 3.5"  H  RLL    40.8MB    25ms   664   4   30   none   S

�����������������������������������������������������������������

Kalok  1

MAXTOR

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

XT-1085  5.25" F  MFM    71.3MB    28ms  1024   8               V

XT-1140  5.25" F  MFM   119.9MB    27ms   918  15               V

XT-2190  5.25" F  MFM   159.8MB    29ms  1224  15               V

XT-4170S 5.25" F  SCSI  157.5MB    14ms  1224   7               V

XT-4170E 5.25" F  ESDI  157.9MB    14ms  1224   7               V

7040A    3.5"  1  IDE    40.5MB    17ms   977   5    17     na  V

7040A    3.5"  1  IDE    40.7MB    17ms   981   5    17     na  V

7040A    3.5"  1  IDE    40.9MB    17ms   524   4    40     na  V

7080A    3.5"  1  IDE    76.5MB    17ms  1024   9    17     na  V

7080A    3.5"  1  IDE    80.0MB    17ms   497  10    33     na  V

7080A    3.5"  1  IDE    80.4MB    17ms   832   6    33     na  V

7080A    3.5"  1  IDE    81.4MB    17ms   981  10    17     na  V

7080A    3.5"  1  IDE    82.1MB    17ms  1039   9    18     na  V

�����������������������������������������������������������������

Maxtor  1

MICROPOLIS

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

1353                    75.0MB          1024   4    36

1353A                   94.0MB          1024   5    36

1354                   113.0MB          1024   6    36

1354A                  132.0MB          1024   7    36

1355                   150.0MB          1024   8    36

1373                    72.0MB          1016   4    36

1373A                   91.0MB          1016   5    36

1374                   109.0MB          1016   6    36

1374A                  127.0MB          1016   7    36

1375                   145.0MB          1016   8    36

1556-11                248.0MB          1224  11    36

1557-12                270.0MB          1224  12    36

1557-13                293.0MB          1224  13    36

1558-14                315.0MB          1224  14    36

1558-15                338.0MB          1224  15    36

1673-4                  90.0MB          1245   4    36

1673-5                 112.0MB          1245   5    36

1674-6                 135.0MB          1245   6    36

1674-7                 158.0MB          1245   7    36

�����������������������������������������������������������������

Micropolis  1

MICROSCIENCE

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

HH-1050  5.25" H  MFM    44.6MB    28ms  1024   5               V

HH-1080  5.25" H  RLL    65.5MB    28ms  1024   5               V

HH-1075  5.25" H  MFM    62.4MB    28ms  1024   7               V

HH-1090  5.25" H  MFM    80.1MB    28ms  1314   7               V

HH-1095  5.25" H  RLL    95.4MB    28ms  1024   7               V

HH-1120  5.25" H  RLL   122.4MB    28ms  1314   7               V

HH-2120  5.25" H  ESDI  121.1MB    28ms  1024   7               V

HH-2160  5.25" H  ESDI  160.1MB    28ms  1276   7               V

HH-3120  5.25" H  SCSI  121.1MB    28ms  1314   7               V

HH-3160  5.25" H  SCSI  169.5MB    28ms  1314   7               V

4050     3.5"  H  MFM    44.5MB    18ms  1024   5               V

4060     3.5"  H  RLL    66.8MB    18ms  1024   5               V

4070     3.5"  H  MFM    62.3MB    18ms  1024   7               V

4090     3.5"  H  RLL    93.4MB    18ms  1024   7               V

5100     3.5"  H  ESDI  110.3MB    18ms   855   7               V

6100     3.5"  H  SCSI  110.3MB    18ms   855   7               V

7040     3.5"  H  IDE    47.2MB    18ms   855   3               V

7100     3.5"  H  IDE   110.3MB    18ms   855   7               V

�����������������������������������������������������������������

Microscience  1

MINISCRIBE

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

8051A   3.5"  H  IDE    41.0MB    28ms   745   4               V

8450    3.5"  H  RLL    39.0MB    45ms   771   4               S

8450AT  3.5"  H  IDE    41.0MB    40ms   745   4               S

8438    3.5"  H  RLL    31.0MB    68ms   615   4               S

8051S   3.5"  H  SCSI   43.0MB    28ms   799   4               V

8450XT  3.5"  H  IDE    41.0MB    68ms   805   4               S

8225S   3.5"  H  SCSI   20.0MB    68ms   809   2               S

8225XT  3.5"  H  IDE    20.0MB    68ms   805   2               S

8450S   3.5"  H  SCSI   40.0MB    68ms   809   4               S

8425F   3.5"  H  MFM    20.0MB    40ms   615   4               S

8425    3.5"  H  MFM    20.0MB    68ms   615   4        300    S

8225    3.5"  H  RLL    20.0MB    68ms   771   2               S

8225AT  3.5"  H  IDE    20.0MB    40ms   747   2               S

8425S   3.5"  H  SCSI   20.0MB    68ms   617   4               S

3650    5.25" H  MFM    40.0MB    61ms   809   5               S

3180E   5.25" H  ESDI  157.0MB    17ms  1250   7               V

�����������������������������������������������������������������

MiniScribe  1

PRIAM

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

ID45H       5.25" H  MFM    44.3MB    25ms  1018   5               V

ID62-AT-D2  5.25" F  MFM    62.0MB    28ms  1018   7               V

ID130-AT-D2 5.25" F  MFM   132.9MB    20ms  1218  15               V

ID120       5.25" H  ESDI  121.1MB    28ms  1017   7               V

�����������������������������������������������������������������

Priam  1

QUANTUM

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

52AT        3.5"  H  IDE    52.0MB      ms   751   8    17   na    V

�����������������������������������������������������������������

Quantum 1

RODIME

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

3058T   3.5"  H  SCSI   45.3MB    18ms   868   3               V

3088T   3.5"  H  SCSI   75.6MB    18ms   868   5               V

3128T   3.5"  H  SCSI  105.8MB    18ms   868   7               V

3059T   3.5"  H  SCSI   46.7MB    18ms  1216   2               V

3089T   3.5"  H  SCSI   70.0MB    18ms  1216   3               V

3129T   3.5"  H  SCSI  105.8MB    18ms  1091   5               V

5178S   5.25" H  SCSI  144.2MB    19ms  1219   7               V

3058A   3.5"  H  IDE    45.3MB    18ms   868   3               V

3088A   3.5"  H  IDE    75.6MB    18ms   868   5               V

3128A   3.5"  H  IDE   105.8MB    18ms   868   7               V

3135A   3.5"  H  IDE   112.5MB    19ms   923   7               V

3059A   3.5"  H  IDE    46.7MB    18ms  1216   2               V

3089A   3.5"  H  IDE    70.0MB    18ms  1216   3               V

3129A   3.5"  H  IDE   105.8MB    18ms  1091   5               V

3139A   3.5"  H  IDE   112.5MB    18ms  1168   5               V

3045    3.5"  H  MFM    37.9MB    28ms   872   5               V

3055    3.5"  H  MFM    45.6MB    28ms   872   6               V

3065    3.5"  H  MFM    53.1MB    28ms   872   7               V

5090    5.25" H  MFM    74.6MB    28ms  1224   7               V

3060R   3.5"  H  RLL    49.9MB    28ms   750   5               V

3075R   3.5"  H  RLL    59.9MB    28ms   750   6               V

3085R   3.5"  H  RLL    69.9MB    28ms   750   7               V

5130R   5.25" H  RLL   114.1MB    28ms  1224   7               V

�����������������������������������������������������������������

Rodime  1

SEAGATE

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

ST125    3.5"  H  MFM    21.4MB    40ms   615   4   17   none   S

ST125-1  3.5"  H  MFM    21.4MB    28ms   615   4   17   none   S

ST125A   3.5"  H  IDE    21.5MB    28ms   404   4        na     S

ST125N   3.5"  H  SCSI   21.5MB    28ms   407   4        none   S

ST138    3.5"  H  MFM    32.1MB    28ms   615   6   17   none   S

ST138A   3.5"  H  IDE    32.1MB    28ms   604   4        na     S

ST138R   3.5"  H  RLL    32.7MB    28ms   615   4   26   none   S

ST138N   3.5"  H  SCSI   32.3MB    28ms   613   4   26   none   S

ST151    3.5"  H  MFM    42.5MB    24ms   977   5   17   none   V

ST157A   3.5"  H  RLL    44.7MB    28ms   560   6   26   na     S

ST157R   3.5"  H  RLL    49.1MB    28ms   615   6   26   none   S

ST157N   3.5"  H  RLL    48.6MB    28ms   613   6   26   none   S

ST177N   3.5"  H  SCSI   60.8MB    24ms   921   5               V

ST225    5.25" H  MFM    21.4MB    65ms   615   4   17   300    S

ST225N   5.25" H  SCSI   21.3MB    65ms   615   4   17   none   S

ST225R   5.25" H  RLL    21.2MB    70ms   667   2               S

ST238R   5.25" H  RLL    32.7MB    65ms   615   4   26   none   S

ST250R   5.25" H  RLL    42.3MB    70ms   667   4               S

ST251    5.25" H  MFM    42.8MB    40ms   820   6   17   none   S

ST251-1  5.25" H  MFM    42.8MB    28ms   820   6   17   none   S

ST251R   5.25" H  RLL    43.6MB    40ms   820   4   26   none   S

ST251N   5.25" H  SCSI   43.1MB    40ms   818   4   26   none   S

ST251N-1 5.25" H  SCSI   43.2MB    28ms   630   4        none   S

ST277R   5.25" H  RLL    65.5MB    40ms   820   6   26   none   S

ST277R-1 5.25" H  RLL    65.5MB    28ms   820   6   26   none   S

ST277N   5.25" H  SCSI   64.9MB    40ms   818   6   26   none   S

ST277N-1 5.25" H  SCSI   64.9MB    28ms   628   6        none   S

ST280A   3.5"  H  IDE    71.0MB          1024   8   17   na     V

ST296N   5.25" H  SCSI   85.0MB    28ms   818   6   34          S

�����������������������������������������������������������������

Seagate  1

ST1090A           IDE    79.5MB          1072   5   29   na

ST1090A           IDE    75.8MB           335  16   29   na

ST1096N  3.5"  H  SCSI   83.9MB    24ms   906   7               V

ST1100            MFM    83.9MB          1072   9   17   none

ST1102A  3.5"  H  IDE    89.1MB          1024  10   17   na     V

ST1111E           ESDI   94.2MB          1072   5   36

ST1111E           ESDI   88.9MB          1072   5   34

ST1126A           IDE   111.4MB          1072   7   29   na

ST1126A           IDE   106.2MB           469  16   29   na

ST1144A  3.5"  H  IDE   130.7MB          1001  15   17   na     V

ST1144A  3.5"  H  IDE   130.7MB          1024  14   17   na     V

ST1150R           RLL   128.4MB          1072   9   26   300

ST1162A           IDE   143.2MB          1072   9   29   na

ST1162A           IDE   136.6MB           603  16   29   na

ST1186A           IDE   164.1MB          1272   7   36   na

ST1186A           IDE   156.5MB           636   9   56   na

ST1201A           IDE   177.8MB          1072   9   36   na

ST1201A           IDE   169.5MB           804   9   48   na

ST1201E           ESDI  169.5MB          1072   9   36

ST1201E           ESDI  160.1MB          1072   9   34

ST1239A           IDE   211.0MB          1272   9   36   na

ST1239A           IDE   201.2MB           954  12   36   na

ST3096A           IDE    89.1MB          1024  10   17   na

ST3096A           IDE    89.0MB           836   8   26   na

ST3120A           IDE   106.9MB          1024  12   17   na

ST3144A           IDE   130.7MB          1001  15   17   na

ST4038   5.25"    MFM    31.9MB    40ms   733   5   17   300

ST4051            MFM    42. MB    40ms   977   5   17   none

ST4053   5.25" F  MFM    44.5MB    28ms  1024   5   17   none   V

ST4096   5.25" F  MFM    80.2MB    28ms  1024   9   17   none   V

ST4144R  5.25" F  RLL   122.7MB    28ms  1024   9   26   none   V

ST4192E  5.25"    ESDI  169.1MB    17ms  1147   8   36

ST4192N  5.25"    SCSI  168.5MB    17ms  1147   8   36

�����������������������������������������������������������������

Seagate  2

94155-85   5.25" F  MFM    71.3MB    28ms  1024   8               V

94155-85P  5.25" F  MFM    71.3MB    28ms  1024   8               V

94155-86   5.25" F  MFM    72.5MB    28ms   925   9               V

94155-96   5.25" F  MFM    80.2MB    28ms  1024   9               V

94155-96P  5.25" F  MFM    8O.2MB    28ms  1024   9               V

94155-135  5.25" F  RLL   115.0MB    28ms   960   9               V

94155-135P 5.25" F  RLL   115.0MB    28ms   960   9               V

94205-51   5.25" H  MFM    43.0MB    28ms   989   5               V

94205-77   5.25" H  RLL    65.OMB    28ms   989   5               V

94204-74   5.25" H  IDE    65.5MB    28ms   941   5               V

94204-81   5.25" H  IDE    71.3MB    29ms  1024   5               V

94166-182  5.25" F  ESDI  155.0MB  16.5ms   969   9               V

94166-141  5.25" F  ESDI  121.0MB  16.5ms   969   7               V

94166-101  5.25" F  ESDI   86.0MB  16.5ms   969   5               V

94161-182  5.25" F  SCSI  155.0MB  16.5ms   969   9               V

94161-141  5.25" F  SCSI  121.0MB  16.5ms   969   7               V

94161-101  5.25" F  SCSI   86.0MB  16.5ms   969   5               V

94211-106  5.25" H  SCSI   91.0MB    20ms   969   5               V

94216-106  5.25" H  ESDI   91.0MB    18ms   969   5               V

94355-100  3.5"  H  MFM    88.0MB    15ms  1072   9               V

94355-150  3.5"  H  RLL   133.0MB    15ms  1072   9               V

94356-111  3.5"  H  ESDI   98.0MB    15ms  1072   5               V

94356-155  3.5"  H  ESDI  138.0MB    15ms  1072   7               V

94354-111  3.5"  H  IDE    98.0MB    15ms  1072   5               V

94354-126  3.5"  H  IDE   111.0MB    15ms  1072   7               V

94354-133  3.5"  H  IDE   117.0MB    15ms  1272   5               V

94354-155  3.5"  H  IDE   138.0MB    15ms  1072   7               V

94354-160  3.5"  H  IDE   143.0MB    15ms  1072   9               V

94354-186  3.5"  H  IDE   164.0MB    15ms  1272   7               V

94351-111  3.5"  H  SCSI   98.0MB    15ms  1068   5               V

94351-126  3.5"  H  SCSI  111.0MB    15ms  1068   7               V

94351-155  3.5"  H  SCSI  138.0MB    15ms  1068   7               V

94351-160  3.5"  H  SCSI  142.0MB    15ms  1068   9               V

94351-133S 3.5"  H  SCSI  116.0MB    15ms  1268   5               V

94351-155S 3.5"  H  SCSI  138.0MB    15ms  1068   7               V

94351-186S 3.5"  H  SCSI  163.0MB    15ms  1268   7               V

94311-136S 3.5"  H  SCSI  120.0MB    15ms   N/A   5               V

94311-136  3.5"  H  SCSI  120.0MB    15ms   N/A   5               V

94314-136  3.5"  H  IDE   120.OMB    15ms   N/A   5               V

94316-136  3.5"  H  ESDI  120.0MB    15ms   N/A   5               V

�����������������������������������������������������������������

Seagate  3

WESTERN DIGITAL

MODEL   FORM  H  TYPE  CAPACITY  ACCESS  CYL  HDS  SEC  PRCMP  A

WD93028-X  3.5"  H  IDE    21.6MB    70ms   782   2               S

WD93038-X  3.5"  H  IDE    32.4MB    70ms   782   3               S

WD93048-X  3.5"  H  IDE    43.2MB    70ms   782   4               S

WD93028-AD 3.5"  H  IDE    21.6MB    69ms   782   2               S

WD93048-AD 3.5"  H  IDE    43.2MB    69ms   782   4               S

WD93024-A  3.5"  H  IDE    21.6MB    28ms   782   2               S

WD93044-A  3.5"  H  IDE    43.2MB    28ms   782   4               S

WD93028-A  3.5"  H  IDE    20.0MB    70ms   782   2               S

WD93048-A  3.5"  H  IDE    40.0MB    70ms   782   4               S

�����������������������������������������������������������������

Western Digital  1

ST506/412 MFM/RLL INTERFACE

���������������������������

Host Adapters usually support two Hard drives and two Floppy drives.

Controllers are available with varying Interleave values.

Transfer rates at Interleaves are approximately:

4:1  125 to 150 kBytes per second

3:1  175 to 200 kBytes per second

2:1  250 to 300 kBytes per second

1:1  512 to 600 kBytes per second

Drive Select Jumpers

��������������������

A jumper must be installed to select the desired "Drive Select"

number.  Drive Numbers may start at 1 or 0.  In the following

information, we will assume that the drive numbers start at 1.

Therefore, Drive Select 1 is the first jumper position, and Drive

Select 2 is the second jumper position, etc.

Examine the large cable that connects the controller to the drive.

In many cases the cable will have connectors at both ends and a

third connector approximately 6 inches from one end.  Check to see

if the ribbon cable is split with a group of wires twisted between

the two connectors at the drive end.

� "C" Drive Configuration

If you have a twisted cable, configure the drive as Drive Select

2.

If you have an untwisted cable, configure the drive as Drive

Select 1.

� "D" Drive Configuration

If you are connecting a second hard drive, configure the drive

as Drive Select 2.

Resistor Termination Packs

��������������������������

Most drive manufacturers tell you to remove the resistor

termination packs on all but the last drive on the cable when you

are installing multiple drives.  In most cases it will not affect

drive performance to leave the termination packs installed.  You

will also have less chance of losing or misplacing the packs for

when you decide to remove a drive.

�����������������������������������������������������������������

ST506/412 MFM/RLL Interface  1

MFM/RLL Cable Pinouts

���������������������

1 - GND                 2 - /Reduced Write Current

3 - GND                 4 - reserved

5 - GND                 6 - /Write Gate

7 - GND                 8 - /Seek Complete

9 - GND                10 - /Track 00

11 - GND                12 - /Write Fault

13 - GND                14 - /Head Select 2^0

15 - GND                16 - reserved

17 - GND                18 - /Head Select 2^1

19 - GND                20 - /Index

21 - GND                22 - /Ready

23 - GND                24 - /Step

25 - GND                26 - /Drive Select 1

27 - GND                28 - /Drive Select 2

29 - GND                30 - reserved

31 - GND                32 - reserved

33 - GND                34 - /Direction In

1 - Drive Select        2 - GND

3 - reserved            4 - GND

5 - spare               6 - GND

7 - reserved            8 - GND

9 - spare              10 - spare

11 - GND                12 - GND

13 - MFM Write Data     14 - /MFM Write Data

15 - GND                16 - GND

17 - MFM Read Data      18 - /MFM Read Data

19 - GND                20 - GND

MFM/RLL Cabling

���������������

� Single Hard Drive, Single Cable set

ڿ                   ڿ

��������������������͵�   Red Stripe is Pin 1.

��                   ��   Many connectors use a V marking to

��������������������Ĵ�   signify Pin 1 in the plastic.

��                   ��

ڿ                   ڿ

��������������������͵�

��                   ��   Red Stripe is Pin 1.

��                   ��

��                   ��   Many connectors use a V marking to

��                   ��   signify Pin 1 in the plastic.

��                   ��

��                   ��

��������������������Ĵ�

��                   ��

Interface             "C"

Drive

DS#1

�����������������������������������������������������������������

ST506/412 MFM/RLL Interface  2

� Single/Dual Hard Drive, Untwisted Cable

ڿ                   ڿ

��������������������͵�   Red Stripe is Pin 1.

��                   ��   Many connectors use a V marking to

��������������������Ĵ�   signify Pin 1 in the plastic.

��                   ��

ڿ                   ڿ

��������������������͵�   Red Stripe is Pin 1.

��                   ��   Many connectors use a V marking to

��������������������Ĵ�   signify Pin 1 in the plastic.

��                   ��

ڿ             ڿ    ڿ

��������������͵����͵�

��             ��    ��   Red Stripe is Pin 1.

��             ��    ��

��             ��    ��   Many connectors use a V marking to

��             ��    ��   signify Pin 1 in the plastic.

��             ��    ��

��             ��    ��

��������������Ĵ����Ĵ�

��             ��    ��

Interface      "D"    "C"

Drive  Drive

DS#2   DS#1

� Single/Dual Hard Drive, Twisted Cable

ڿ                   ڿ

��������������������͵�   Red Stripe is Pin 1.

��                   ��   Many connectors use a V marking to

��������������������Ĵ�   signify Pin 1 in the plastic.

��                   ��

ڿ                   ڿ

��������������������͵�   Red Stripe is Pin 1.

��                   ��   Many connectors use a V marking to

��������������������Ĵ�   signify Pin 1 in the plastic.

��                   ��

ڿ             ڿ    ڿ

��������������͵����͵�

��             ��    ��   Red Stripe is Pin 1.

��             �� >< ��

��             ��    ��   Many connectors use a V marking to

��             ��    ��   signify Pin 1 in the plastic.

��             ��    ��

��             ��    ��

��������������Ĵ����Ĵ�

��             ��    ��

Interface      "D"    "C"

Drive  Drive

DS#2   DS#2

�����������������������������������������������������������������

ST506/412 MFM/RLL Interface  3

IDE INTERFACE

�������������

Host Adapters usually support two AT interface drives and two

Floppy drives.

AT IDE Host Pinout

������������������

1 - /Host Reset         2 - GND

3 - Host Data 7         4 - Host Data 8

5 - Host Data 6         6 - Host Data 9

7 - Host Data 5         8 - Host Data 10

9 - Host Data 4        10 - Host Data 11

11 - Host Data 3        12 - Host Data 12

13 - Host Data 2        14 - Host Data 13

15 - Host Data 1        16 - Host Data 14

17 - Host Data 0        18 - Host Data 15

19 - GND                20 - KEY

21 - unused             22 - GND

23 - /Host IOW          24 - GND

25 - /Host IOR          26 - GND

27 - IOCHRDY            28 - Host BALE

29 - unused             30 - GND

31 - Host IRQ           32 - /Host IOCS16

33 - Host A1            34 - unused

35 - Host A0            36 - Host A2

37 - /Host CS0          38 - /Host CS1

39 - /Active            40 - GND

AT IDE Host Cabling

�������������������

ڿ             ڿ    ڿ

��������������͵����͵�

��             ��    ��   Red Stripe is Pin 1.

��             ��    ��

��             ��    ��   Many connectors use a V marking to

��             ��    ��   signify Pin 1 in the plastic.

��             ��    ��

��             ��    ��

��������������Ĵ����Ĵ�

��             ��    ��

Host          Master Slave

1st    2nd

Drive  Drive

�����������������������������������������������������������������

IDE Interface  1

BIOS Settings

�������������

Mfg.     Version    Drive    BIOS #

�����    �������    �������  ���������

AMI      4/9/90+    ST1102A  35

AMI      4/9/90+    ST1144A  9, 47, 45

Award    3.04+      ST1102A  33

Award    3.04+      ST1144A  9, 41

QuadTel  3.04+      ST1102A  40

QuadTel  3.04+      ST1144A  9

Phoenix  3.10+      ST1102A  35

Phoenix  3.10+      ST1144A  9, 41

�����������������������������������������������������������������

BIOS Settings  1

BIOS Error Codes

����������������

The majority of 286/386/486 BIOS' today provide full english

error messages.

The IBM Error code numbers for Fixed Disks are:

1700 Code series

1701 Fixed disk POST error

1702 Fixed disk adapter error

1703 Fixed disk drive error

1704 Fixed disk adapter or drive error

1780 Fixed disk 0 failure

1781 Fixed disk 1 failure

1782 Fixed disk controller failure

1790 Fixed disk 0 error

1791 Fixed disk 1 error

�����������������������������������������������������������������

BIOS Error Codes  1

An Actual AT BIOS

Award 286 Modular BIOS          IBM PC/AT Technical

Version 3.02m (AT)              Reference Manual

Type Cyl  Hd Sct PreCmp LZone    Cyl  Hd Sct PreCmp LZone   Drive

1   306   4  17   128    305    306   4  17   128    305   Standard 10M

2   615   4  17   300    615    615   4  17   300    615   Standard 20M

3   615   6  17   300    615    615   6  17   300    615   Standard 30M

4   940   8  17   512    940    940   8  17   512    940

5   940   6  17   512    940    940   6  17   512    940

6   615   4  17  NONE    615    615   4  17  NONE    615

7   462   8  17   256    511    462   8  17   256    511

8   733   5  17  NONE    733    733   5  17  NONE    733

9   900  15  17  NONE    901    900  15  17  NONE    901

10   820   3  17  NONE    820    820   3  17  NONE    820

11   855   5  17  NONE    855    855   5  17  NONE    855

12   855   7  17  NONE    855    855   7  17  NONE    855

13   306   8  17   128    319    306   8  17   128    319

14   733   7  17  NONE    733    733   7  17  NONE    733

15   n/a                         Extended Parameters

16   612   4  17     0    663    612   4  17   ALL    663

17   977   5  17   300    977    977   5  17   300    977

18   977   7  17  NONE    977    977   7  17  NONE    977

19  1024   7  17   512   1023   1024   7  17   512   1023

20   733   5  17   300    732    733   5  17   300    732

21   733   7  17   300    732    733   7  17   300    732

22   733   5  17   300    733    733   7  17   300    733

23   306   4  17     0    336    306   4  17   ALL    336

24   977   5  17  NONE    976    Reserved

25  1024   9  17  NONE   1279    Reserved                   Seagate 4096

26  1224   7  17  NONE   1223    Reserved                   Maxtor 2085

27  1224  11  17  NONE   1223    Reserved                   Maxtor 2140

28  1224  15  17  NONE   1223    Reserved                   Maxtor 2190

29  1024   8  17  NONE   1023    Reserved                   Maxtor 1085

30  1024  11  17  NONE   1023    Reserved                   Maxtor 1105

31   918  11  17  NONE   1023    Reserved                   Maxtor 1170

32   925   9  17  NONE    926    Reserved                   CDC 9415

33  1024  10  17  NONE   1023    Reserved                   Generic 10head

34  1024  12  17  NONE   1023    Reserved                   Generic 12head

35  1024  13  17  NONE   1023    Reserved                   Generic 13head

36  1024  14  17  NONE   1023    Reserved                   Generic 14head

37  1024   2  17  NONE   1023    Reserved                   Generic 2head

38  1024  16  17  NONE   1023    Reserved                   Generic 16head

39   918  15  17  NONE   1023    Reserved                   Maxtor 1140

40   820   6  17  NONE    820    Reserved                   Seagate 251

41     0   0   0     0      0    Reserved

42     0   0   0     0      0    Reserved

43     0   0   0     0      0    Reserved

44     0   0   0     0      0    Reserved

45     0   0   0     0      0    Reserved

46     0   0   0     0      0    Reserved

47     0   0   0     0      0    Reserved

NONE

�����������������������������������������������������������������

An Actual BIOS  1

Award BIOS Version 3.03 common drives

17  Seagate ST-151    40Mb 24ms

40  Seagate ST-251    40Mb 40ms

40  Seagate ST-251-1  40Mb 28ms

03  Miniscribe 3650   40Mb 61ms

24  Miniscribe 3053   40Mb 28ms

03  Seagate ST-157    50Mb 40ms RLL

03  Seagate ST-157-1  50Mb 28ms RLL

40  Seagate ST-277    60Mb 28ms RLL

18  Miniscribe        72Mb 28ms

29  Micropolis 1335   72Mb 28ms

29  Maxtor 1085       72Mb 28ms

25  Seagate ST-4096   80Mb 28ms

39  Maxtor 1140      110Mb 28ms

39  Maxtor 2190      150Mb 28ms

AT BIOS FD_TBL Fixed Disk Parameter Table (IBM AT Tech Ref Pg.5-185)

Address: $FE00:0401

--------------------------------------------------

INITIALIZE DRIVE CHARACTERISTICS

FIXED DISK PARAMETER TABLE

- THE TABLE IS COMPOSED OF A BLOCK DEFINED AS:

+0  (1 WORD) - MAXIMUM NUMBER OF CYLINDERS

+2  (1 BYTE) - MAXIMUM NUMBER OF HEADS

+3  (1 WORD) - NOT USED/SEE PC-XT

+5  (1 WORD) - STARTING WRITE PRECOMPENSATION CYL

+7  (1 BYTE) - NOT USED/SEE PC-XT

+8  (1 BYTE) - CONTROL BYTE

BIT    7 DISABLE RETRIES -OR-

BIT    6 DISABLE RETRIES

BIT    3 MORE THAN 8 HEADS

+9  (3 BYTES)- NOT USED/SEE PC-XT

+12  (1 WORD) - LANDING ZONE

+14  (1 BYTE) - NUMBER OF SECTORS/TRACK

+15  (1 BYTE) - RESERVED FOR FUTURE USE

- TO DYNAMICALLY DEFINE A SET OF PARAMETERS

BUILD A TABLE FOR UP TO 15 TYPES AND PLACE

THE CORRESPONDING VECTOR INTO INTERRUPT 41

FOR DRIVE 0 AND INTERRUPT 46 FOR DRIVE 1.

--------------------------------------------------

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  1

0401            FD_TBL:

- DRIVE TYPE 01 -

0401 0132           DW   0306D          ;CYLINDERS

0403 04             DB   04D            ;HEADS

0404 0000           DW   0

0406 0080           DW   0128D          ;WRITE PRECOMP CYLINDER

0408 00             DB   0

0409 00             DB   0              ;CONTROL BYTE

040A 00 00 00       DB   0,0,0

040D 0131           DW   0305D          ;LANDING ZONE

040F 11             DB   17D            ;SECTORS/TRACK

0410 00             DB   0

- DRIVE TYPE 02 -

0411 0267           DW   0615D          ;CYLINDERS

0413 04             DB   04D            ;HEADS

0414 0000           DW   0

0416 012C           DW   0300D          ;WRITE PRECOMP CYLINDER

0418 00             DB   0

0419 00             DB   0              ;CONTROL BYTE

041A 00 00 00       DB   0,0,0

041D 0267           DW   0615D          ;LANDING ZONE

041F 11             DB   17D            ;SECTORS/TRACK

0420 00             DB   0

- DRIVE TYPE 03 -

0421 0267           DW   0615D          ;CYLINDERS

0423 06             DB   06D            ;HEADS

0424 0000           DW   0

0426 012C           DW   0300D          ;WRITE PRECOMP CYLINDER

0428 00             DB   0

0429 00             DB   0              ;CONTROL BYTE

042A 00 00 00       DB   0,0,0

042D 0267           DW   0615D          ;LANDING ZONE

042F 11             DB   17D            ;SECTORS/TRACK

0430 00             DB   0

- DRIVE TYPE 04 -

0431 03AC           DW   0940D          ;CYLINDERS

0433 08             DB   08D            ;HEADS

0434 0000           DW   0

0436 0200           DW   0512D          ;WRITE PRECOMP CYLINDER

0438 00             DB   0

0439 00             DB   0              ;CONTROL BYTE

043A 00 00 00       DB   0,0,0

043D 03AC           DW   0940D          ;LANDING ZONE

043F 11             DB   17D            ;SECTORS/TRACK

0440 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  2

- DRIVE TYPE 05 -

0441 03AC           DW   0940D          ;CYLINDERS

0443 06             DB   06D            ;HEADS

0444 0000           DW   0

0446 0200           DW   0512D          ;WRITE PRECOMP CYLINDER

0448 00             DB   0

0449 00             DB   0              ;CONTROL BYTE

044A 00 00 00       DB   0,0,0

044D 03AC           DW   0940D          ;LANDING ZONE

044F 11             DB   17D            ;SECTORS/TRACK

0450 00             DB   0

- DRIVE TYPE 06 -

0451 0267           DW   0615D          ;CYLINDERS

0453 04             DB   04D            ;HEADS

0454 0000           DW   0

0456 FFFF           DW   0FFFFH         ;NO WRITE PRECOMP

0458 00             DB   0

0459 00             DB   0              ;CONTROL BYTE

045A 00 00 00       DB   0,0,0

045D 0267           DW   0615D          ;LANDING ZONE

045F 11             DB   17D            ;SECTORS/TRACK

0460 00             DB   0

- DRIVE TYPE 07 -

0461 01CE           DW   0462D          ;CYLINDERS

0463 08             DB   08D            ;HEADS

0464 0000           DW   0

0466 0100           DW   0256D          ;WRITE PRECOMP CYLINDER

0468 00             DB   0

0469 00             DB   0              ;CONTROL BYTE

046A 00 00 00       DB   0,0,0

046D 01FF           DW   0511D          ;LANDING ZONE

046F 11             DB   17D            ;SECTORS/TRACK

0470 00             DB   0

- DRIVE TYPE 08 -

0471 02DD           DW   0733D          ;CYLINDERS

0473 05             DB   05D            ;HEADS

0474 0000           DW   0

0476 FFFF           DW   0FFFFH         ;NO WRITE PRECOMP

0478 00             DB   0

0479 00             DB   0              ;CONTROL BYTE

047A 00 00 00       DB   0,0,0

047D 02DD           DW   0733D          ;LANDING ZONE

047F 11             DB   17D            ;SECTORS/TRACK

0480 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  3

- DRIVE TYPE 09 -

0481 0384           DW   0900D          ;CYLINDERS

0483 0F             DB   15D            ;HEADS

0484 0000           DW   0

0486 FFFF           DW   0FFFFH         ;NO WRITE PRECOMP

0488 00             DB   0

0489 08             DB   008H           ;CONTROL BYTE

048A 00 00 00       DB   0,0,0

048D 0385           DW   0901D          ;LANDING ZONE

048F 11             DB   17D            ;SECTORS/TRACK

0490 00             DB   0

- DRIVE TYPE 10 -

0491 0334           DW   0820D          ;CYLINDERS

0493 03             DB   03D            ;HEADS

0494 0000           DW   0

0496 FFFF           DW   0FFFFH         ;NO WRITE PRECOMP

0498 00             DB   0

0499 00             DB   0              ;CONTROL BYTE

049A 00 00 00       DB   0,0,0

049D 0334           DW   0820D          ;LANDING ZONE

049F 11             DB   17D            ;SECTORS/TRACK

04A0 00             DB   0

- DRIVE TYPE 11 -

04A1 0357           DW   0855D          ;CYLINDERS

04A3 05             DB   05D            ;HEADS

04A4 0000           DW   0

04A6 FFFF           DW   0FFFFH         ;NO WRITE PRECOMP

04A8 00             DB   0

04A9 00             DB   0              ;CONTROL BYTE

04AA 00 00 00       DB   0,0,0

04AD 0357           DW   0855D          ;LANDING ZONE

04AF 11             DB   17D            ;SECTORS/TRACK

04B0 00             DB   0

- DRIVE TYPE 12 -

04B1 0357           DW   0855D          ;CYLINDERS

04B3 07             DB   07D            ;HEADS

04B4 0000           DW   0

04B6 FFFF           DW   0FFFFH         ;NO WRITE PRECOMP

04B8 00             DB   0

04B9 00             DB   0              ;CONTROL BYTE

04BA 00 00 00       DB   0,0,0

04BD 0357           DW   0855D          ;LANDING ZONE

04BF 11             DB   17D            ;SECTORS/TRACK

04C0 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  4

- DRIVE TYPE 13 -

04C1 0132           DW   0306D          ;CYLINDERS

04C3 08             DB   08D            ;HEADS

04C4 0000           DW   0

04C6 0080           DW   0128D          ;WRITE PRECOMP CYLINDER

04C8 00             DB   0

04C9 00             DB   0              ;CONTROL BYTE

04CA 00 00 00       DB   0,0,0

04CD 013F           DW   0319D          ;LANDING ZONE

04CF 11             DB   17D            ;SECTORS/TRACK

04D0 00             DB   0

- DRIVE TYPE 14 -

04D1 02DD           DW   0733D          ;CYLINDERS

04D3 07             DB   07D            ;HEADS

04D4 0000           DW   0

04D6 FFFF           DW   0FFFFH         ;NO WRITE PRECOMP

04D8 00             DB   0

04D9 00             DB   0              ;CONTROL BYTE

04DA 00 00 00       DB   0,0,0

04DD 02DD           DW   0733D          ;LANDING ZONE

04DF 11             DB   17D            ;SECTORS/TRACK

04E0 00             DB   0

- DRIVE TYPE 15 -   RESERVED  ***DO NOT USE***

04E1 0000           DW   0000D          ;CYLINDERS

04E3 00             DB   00D            ;HEADS

04E4 0000           DW   0

04E6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

04E8 00             DB   0

04E9 00             DB   0              ;CONTROL BYTE

04EA 00 00 00       DB   0,0,0

04ED 0000           DW   0000D          ;LANDING ZONE

04EF 00             DB   00D            ;SECTORS/TRACK

04F0 00             DB   0

- DRIVE TYPE 16 -

04F1 0264           DW   0612D          ;CYLINDERS

04F3 04             DB   04D            ;HEADS

04F4 0000           DW   0

04F6 0000           DW   0000D          ;WRITE PRECOMP ALL CYLINDERS

04F8 00             DB   0

04F9 00             DB   0              ;CONTROL BYTE

04FA 00 00 00       DB   0,0,0

04FD 0297           DW   0663D          ;LANDING ZONE

04FF 11             DB   17D            ;SECTORS/TRACK

0500 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  5

- DRIVE TYPE 17 -

0501 03D1           DW   0977D          ;CYLINDERS

0503 05             DB   05D            ;HEADS

0504 0000           DW   0

0506 012C           DW   0300D          ;WRITE PRECOMP CYLINDER

0508 00             DB   0

0509 00             DB   0              ;CONTROL BYTE

050A 00 00 00       DB   0,0,0

050D 03D1           DW   0977D          ;LANDING ZONE

050F 11             DB   17D            ;SECTORS/TRACK

0510 00             DB   0

- DRIVE TYPE 18 -

0511 03D1           DW   0977D          ;CYLINDERS

0513 07             DB   07D            ;HEADS

0514 0000           DW   0

0516 FFFF           DW   0FFFFH         ;NO WRITE PRECOMP

0518 00             DB   0

0519 00             DB   0              ;CONTROL BYTE

051A 00 00 00       DB   0,0,0

051D 03D1           DW   0977D          ;LANDING ZONE

051F 11             DB   17D            ;SECTORS/TRACK

0520 00             DB   0

- DRIVE TYPE 19 -

0521 0400           DW   1024D          ;CYLINDERS

0523 07             DB   07D            ;HEADS

0524 0000           DW   0

0526 0200           DW   0512D          ;WRITE PRECOMP CYLINDER

0528 00             DB   0

0529 00             DB   0              ;CONTROL BYTE

052A 00 00 00       DB   0,0,0

052D 03FF           DW   1023D          ;LANDING ZONE

052F 11             DB   17D            ;SECTORS/TRACK

0530 00             DB   0

- DRIVE TYPE 20 -

0531 02DD           DW   0733D          ;CYLINDERS

0533 05             DB   05D            ;HEADS

0534 0000           DW   0

0536 012C           DW   0300D          ;WRITE PRECOMP CYLINDER

0538 00             DB   0

0539 00             DB   0              ;CONTROL BYTE

053A 00 00 00       DB   0,0,0

053D 02DC           DW   0732D          ;LANDING ZONE

053F 11             DB   17D            ;SECTORS/TRACK

0540 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  6

- DRIVE TYPE 21 -

0541 02DD           DW   0733D          ;CYLINDERS

0543 07             DB   07D            ;HEADS

0544 0000           DW   0

0546 012C           DW   0300D          ;WRITE PRECOMP CYLINDER

0548 00             DB   0

0549 00             DB   0              ;CONTROL BYTE

054A 00 00 00       DB   0,0,0

054D 02DC           DW   0732D          ;LANDING ZONE

054F 11             DB   17D            ;SECTORS/TRACK

0550 00             DB   0

- DRIVE TYPE 22 -

0551 02DD           DW   0733D          ;CYLINDERS

0553 05             DB   05D            ;HEADS

0554 0000           DW   0

0556 012C           DW   0300D          ;WRITE PRECOMP CYLINDER

0558 00             DB   0

0559 00             DB   0              ;CONTROL BYTE

055A 00 00 00       DB   0,0,0

055D 02DD           DW   0733D          ;LANDING ZONE

055F 11             DB   17D            ;SECTORS/TRACK

0560 00             DB   0

- DRIVE TYPE 23 -

0561 0132           DW   0306D          ;CYLINDERS

0563 04             DB   04D            ;HEADS

0564 0000           DW   0

0566 0000           DW   0000D          ;WRITE PRECOMP ALL CYLINDERS

0568 00             DB   0

0569 00             DB   0              ;CONTROL BYTE

056A 00 00 00       DB   0,0,0

056D 0150           DW   0336D          ;LANDING ZONE

056F 11             DB   17D            ;SECTORS/TRACK

0570 00             DB   0

- DRIVE TYPE 24 -

0571 0000           DW   0000D          ;CYLINDERS

0573 00             DB   00D            ;HEADS

0574 0000           DW   0

0576 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0578 00             DB   0

0579 00             DB   0              ;CONTROL BYTE

057A 00 00 00       DB   0,0,0

057D 0000           DW   0000D          ;LANDING ZONE

057F 00             DB   00D            ;SECTORS/TRACK

0580 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  7

- DRIVE TYPE 25 -

0581 0000           DW   0000D          ;CYLINDERS

0583 00             DB   00D            ;HEADS

0584 0000           DW   0

0586 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0588 00             DB   0

0589 00             DB   0              ;CONTROL BYTE

058A 00 00 00       DB   0,0,0

058D 0000           DW   0000D          ;LANDING ZONE

058F 00             DB   00D            ;SECTORS/TRACK

0590 00             DB   0

- DRIVE TYPE 26 -

0591 0000           DW   0000D          ;CYLINDERS

0593 00             DB   00D            ;HEADS

0594 0000           DW   0

0596 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0598 00             DB   0

0599 00             DB   0              ;CONTROL BYTE

059A 00 00 00       DB   0,0,0

059D 0000           DW   0000D          ;LANDING ZONE

059F 00             DB   00D            ;SECTORS/TRACK

05A0 00             DB   0

- DRIVE TYPE 27 -

05A1 0000           DW   0000D          ;CYLINDERS

05A3 00             DB   00D            ;HEADS

05A4 0000           DW   0

05A6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

05A8 00             DB   0

05A9 00             DB   0              ;CONTROL BYTE

05AA 00 00 00       DB   0,0,0

05AD 0000           DW   0000D          ;LANDING ZONE

05AF 00             DB   00D            ;SECTORS/TRACK

05B0 00             DB   0

- DRIVE TYPE 28 -

05B1 0000           DW   0000D          ;CYLINDERS

05B3 00             DB   00D            ;HEADS

05B4 0000           DW   0

05B6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

05B8 00             DB   0

05B9 00             DB   0              ;CONTROL BYTE

05BA 00 00 00       DB   0,0,0

05BD 0000           DW   0000D          ;LANDING ZONE

05BF 00             DB   00D            ;SECTORS/TRACK

05C0 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  8

- DRIVE TYPE 29 -

05C1 0000           DW   0000D          ;CYLINDERS

05C3 00             DB   00D            ;HEADS

05C4 0000           DW   0

05C6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

05C8 00             DB   0

05C9 00             DB   0              ;CONTROL BYTE

05CA 00 00 00       DB   0,0,0

05CD 0000           DW   0000D          ;LANDING ZONE

05CF 00             DB   00D            ;SECTORS/TRACK

05D0 00             DB   0

- DRIVE TYPE 30 -

05D1 0000           DW   0000D          ;CYLINDERS

05D3 00             DB   00D            ;HEADS

05D4 0000           DW   0

05D6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

05D8 00             DB   0

05D9 00             DB   0              ;CONTROL BYTE

05DA 00 00 00       DB   0,0,0

05DD 0000           DW   0000D          ;LANDING ZONE

05DF 00             DB   00D            ;SECTORS/TRACK

05E0 00             DB   0

- DRIVE TYPE 31 -

05E1 0000           DW   0000D          ;CYLINDERS

05E3 00             DB   00D            ;HEADS

05E4 0000           DW   0

05E6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

05E8 00             DB   0

05E9 00             DB   0              ;CONTROL BYTE

05EA 00 00 00       DB   0,0,0

05ED 0000           DW   0000D          ;LANDING ZONE

05EF 00             DB   00D            ;SECTORS/TRACK

05F0 00             DB   0

- DRIVE TYPE 32 -

05F1 0000           DW   0000D          ;CYLINDERS

05F3 00             DB   00D            ;HEADS

05F4 0000           DW   0

05F6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

05F8 00             DB   0

05F9 00             DB   0              ;CONTROL BYTE

05FA 00 00 00       DB   0,0,0

05FD 0000           DW   0000D          ;LANDING ZONE

05FF 00             DB   00D            ;SECTORS/TRACK

0600 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  9

- DRIVE TYPE 33 -

0601 0000           DW   0000D          ;CYLINDERS

0603 00             DB   00D            ;HEADS

0604 0000           DW   0

0606 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0608 00             DB   0

0609 00             DB   0              ;CONTROL BYTE

060A 00 00 00       DB   0,0,0

060D 0000           DW   0000D          ;LANDING ZONE

060F 00             DB   00D            ;SECTORS/TRACK

0610 00             DB   0

- DRIVE TYPE 34 -

0611 0000           DW   0000D          ;CYLINDERS

0613 00             DB   00D            ;HEADS

0614 0000           DW   0

0616 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0618 00             DB   0

0619 00             DB   0              ;CONTROL BYTE

061A 00 00 00       DB   0,0,0

061D 0000           DW   0000D          ;LANDING ZONE

061F 00             DB   00D            ;SECTORS/TRACK

0620 00             DB   0

- DRIVE TYPE 35 -

0621 0000           DW   0000D          ;CYLINDERS

0623 00             DB   00D            ;HEADS

0624 0000           DW   0

0626 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0628 00             DB   0

0629 00             DB   0              ;CONTROL BYTE

062A 00 00 00       DB   0,0,0

062D 0000           DW   0000D          ;LANDING ZONE

062F 00             DB   00D            ;SECTORS/TRACK

0630 00             DB   0

- DRIVE TYPE 36 -

0631 0000           DW   0000D          ;CYLINDERS

0633 00             DB   00D            ;HEADS

0634 0000           DW   0

0636 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0638 00             DB   0

0639 00             DB   0              ;CONTROL BYTE

063A 00 00 00       DB   0,0,0

063D 0000           DW   0000D          ;LANDING ZONE

063F 00             DB   00D            ;SECTORS/TRACK

0640 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  10

- DRIVE TYPE 37 -

0641 0000           DW   0000D          ;CYLINDERS

0643 00             DB   00D            ;HEADS

0644 0000           DW   0

0646 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0648 00             DB   0

0649 00             DB   0              ;CONTROL BYTE

064A 00 00 00       DB   0,0,0

064D 0000           DW   0000D          ;LANDING ZONE

064F 00             DB   00D            ;SECTORS/TRACK

0650 00             DB   0

- DRIVE TYPE 38 -

0651 0000           DW   0000D          ;CYLINDERS

0653 00             DB   00D            ;HEADS

0654 0000           DW   0

0656 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0658 00             DB   0

0659 00             DB   0              ;CONTROL BYTE

065A 00 00 00       DB   0,0,0

065D 0000           DW   0000D          ;LANDING ZONE

065F 00             DB   00D            ;SECTORS/TRACK

0660 00             DB   0

- DRIVE TYPE 39 -

0661 0000           DW   0000D          ;CYLINDERS

0663 00             DB   00D            ;HEADS

0664 0000           DW   0

0666 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0668 00             DB   0

0669 00             DB   0              ;CONTROL BYTE

066A 00 00 00       DB   0,0,0

066D 0000           DW   0000D          ;LANDING ZONE

066F 00             DB   00D            ;SECTORS/TRACK

0670 00             DB   0

- DRIVE TYPE 40 -

0671 0000           DW   0000D          ;CYLINDERS

0673 00             DB   00D            ;HEADS

0674 0000           DW   0

0676 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0678 00             DB   0

0679 00             DB   0              ;CONTROL BYTE

067A 00 00 00       DB   0,0,0

067D 0000           DW   0000D          ;LANDING ZONE

067F 00             DB   00D            ;SECTORS/TRACK

0680 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  11

- DRIVE TYPE 41 -

0681 0000           DW   0000D          ;CYLINDERS

0683 00             DB   00D            ;HEADS

0684 0000           DW   0

0686 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0688 00             DB   0

0689 00             DB   0              ;CONTROL BYTE

068A 00 00 00       DB   0,0,0

068D 0000           DW   0000D          ;LANDING ZONE

068F 00             DB   00D            ;SECTORS/TRACK

0690 00             DB   0

- DRIVE TYPE 42 -

0691 0000           DW   0000D          ;CYLINDERS

0693 00             DB   00D            ;HEADS

0694 0000           DW   0

0696 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

0698 00             DB   0

0699 00             DB   0              ;CONTROL BYTE

069A 00 00 00       DB   0,0,0

069D 0000           DW   0000D          ;LANDING ZONE

069F 00             DB   00D            ;SECTORS/TRACK

06A0 00             DB   0

- DRIVE TYPE 43 -

06A1 0000           DW   0000D          ;CYLINDERS

06A3 00             DB   00D            ;HEADS

06A4 0000           DW   0

06A6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

06A8 00             DB   0

06A9 00             DB   0              ;CONTROL BYTE

06AA 00 00 00       DB   0,0,0

06AD 0000           DW   0000D          ;LANDING ZONE

06AF 00             DB   00D            ;SECTORS/TRACK

06B0 00             DB   0

- DRIVE TYPE 44 -

06B1 0000           DW   0000D          ;CYLINDERS

06B3 00             DB   00D            ;HEADS

06B4 0000           DW   0

06B6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

06B8 00             DB   0

06B9 00             DB   0              ;CONTROL BYTE

06BA 00 00 00       DB   0,0,0

06BD 0000           DW   0000D          ;LANDING ZONE

06BF 00             DB   00D            ;SECTORS/TRACK

06C0 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  12

- DRIVE TYPE 45 -

06C1 0000           DW   0000D          ;CYLINDERS

06C3 00             DB   00D            ;HEADS

06C4 0000           DW   0

06C6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

06C8 00             DB   0

06C9 00             DB   0              ;CONTROL BYTE

06CA 00 00 00       DB   0,0,0

06CD 0000           DW   0000D          ;LANDING ZONE

06CF 00             DB   00D            ;SECTORS/TRACK

06D0 00             DB   0

- DRIVE TYPE 46 -

06D1 0000           DW   0000D          ;CYLINDERS

06D3 00             DB   00D            ;HEADS

06D4 0000           DW   0

06D6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

06D8 00             DB   0

06D9 00             DB   0              ;CONTROL BYTE

06DA 00 00 00       DB   0,0,0

06DD 0000           DW   0000D          ;LANDING ZONE

06DF 00             DB   00D            ;SECTORS/TRACK

06E0 00             DB   0

- DRIVE TYPE 47 -

06E1 0000           DW   0000D          ;CYLINDERS

06E3 00             DB   00D            ;HEADS

06E4 0000           DW   0

06E6 0000           DW   0000D          ;WRITE PRECOMP CYLINDER

06E8 00             DB   0

06E9 00             DB   0              ;CONTROL BYTE

06EA 00 00 00       DB   0,0,0

06ED 0000           DW   0000D          ;LANDING ZONE

06EF 00             DB   00D            ;SECTORS/TRACK

06F0 00             DB   0

�����������������������������������������������������������������

AT Technical Reference Fixed Disk  13

AWARD BIOS V3.02m FD_TBL Fixed Disk Parameter Table

0401 0132         DRIVE TYPE 01

0403 04

0404 0000

0406 0080

0408 00

0409 00

040A 00 00 00

040D 0131

040F 11

0410 00

0411 0267         DRIVE TYPE 02

0413 04

0414 0000

0416 012C

0418 00

0419 00

041A 00 00 00

041D 0267

041F 11

0420 00

0421 0267         DRIVE TYPE 03

0423 06

0424 0000

0426 012C

0428 00

0429 00

042A 00 00 00

042D 0267

042F 11

0430 00

0431 03AC         DRIVE TYPE 04

0433 08

0434 0000

0436 0200

0438 00

0439 00

043A 00 00 00

043D 03AC

043F 11

0440 00

0441 03AC         DRIVE TYPE 05

0443 06

0444 0000

0446 0200

0448 00

0449 00

044A 00 00 00

044D 03AC

044F 11

0450 00

�����������������������������������������������������������������

Award BIOS Reference  1

0451 0267         DRIVE TYPE 06

0453 04

0454 0000

0456 FFFF

0458 00

0459 00

045A 00 00 00

045D 0267

045F 11

0460 00

0461 01CE         DRIVE TYPE 07

0463 08

0464 0000

0466 0100

0468 00

0469 00

046A 00 00 00

046D 01FF

046F 11

0470 00

0471 02DD         DRIVE TYPE 08

0473 05

0474 0000

0476 FFFF

0478 00

0479 00

047A 00 00 00

047D 02DD

047F 11

0480 00

0481 0384         DRIVE TYPE 09

0483 0F

0484 0000

0486 FFFF

0488 00

0489 08

048A 00 00 00

048D 0385

048F 11

0490 00

0491 0334         DRIVE TYPE 10

0493 03

0494 0000

0496 FFFF

0498 00

0499 00

049A 00 00 00

049D 0334

049F 11

04A0 00

�����������������������������������������������������������������

Award BIOS Reference  2

04A1 0357         DRIVE TYPE 11

04A3 05

04A4 0000

04A6 FFFF

04A8 00

04A9 00

04AA 00 00 00

04AD 0357

04AF 11

04B0 00

04B1 0357         DRIVE TYPE 12

04B3 07

04B4 0000

04B6 FFFF

04B8 00

04B9 00

04BA 00 00 00

04BD 0357

04BF 11

04C0 00

04C1 0132         DRIVE TYPE 13

04C3 08

04C4 0000

04C6 0080

04C8 00

04C9 00

04CA 00 00 00

04CD 013F

04CF 11

04D0 00

04D1 02DD         DRIVE TYPE 14

04D3 07

04D4 0000

04D6 FFFF

04D8 00

04D9 00

04DA 00 00 00

04DD 02DD

04DF 11

04E0 00

04E1 0000         DRIVE TYPE 15     RESERVED  ***DO NOT USE***

04E3 00

04E4 0000

04E6 0000

04E8 00

04E9 00

04EA 00 00 00

04ED 0000

04EF 00

04F0 00

�����������������������������������������������������������������

Award BIOS Reference  3

04F1 0264         DRIVE TYPE 16

04F3 04

04F4 0000

04F6 0000

04F8 00

04F9 00

04FA 00 00 00

04FD 0297

04FF 11

0500 00

0501 03D1         DRIVE TYPE 17

0503 05

0504 0000

0506 012C

0508 00

0509 00

050A 00 00 00

050D 03D1

050F 11

0510 00

0511 03D1         DRIVE TYPE 18

0513 07

0514 0000

0516 FFFF

0518 00

0519 00

051A 00 00 00

051D 03D1

051F 11

0520 00

0521 0400         DRIVE TYPE 19

0523 07

0524 0000

0526 0200

0528 00

0529 00

052A 00 00 00

052D 03FF

052F 11

0530 00

0531 02DD         DRIVE TYPE 20

0533 05

0534 0000

0536 012C

0538 00

0539 00

053A 00 00 00

053D 02DC

053F 11

0540 00

�����������������������������������������������������������������

Award BIOS Reference  4

0541 02DD         DRIVE TYPE 21

0543 07

0544 0000

0546 012C

0548 00

0549 00

054A 00 00 00

054D 02DC

054F 11

0550 00

0551 02DD         DRIVE TYPE 22

0553 05

0554 0000

0556 012C

0558 00

0559 00

055A 00 00 00

055D 02DD

055F 11

0560 00

0561 0132         DRIVE TYPE 23

0563 04

0564 0000

0566 0000

0568 00

0569 00

056A 00 00 00

056D 0150

056F 11

0570 00

0571 03D1         DRIVE TYPE 24

0573 05

0574 0000

0576 FFFF

0578 00

0579 00

057A 00 00 00

057D 03D0

057F 11

0580 00

0581 0400         DRIVE TYPE 25

0583 09

0584 0000

0586 FFFF

0588 00

0589 08

058A 00 00 00

058D 04FF

058F 11

0590 00

�����������������������������������������������������������������

Award BIOS Reference  5

0591 04C8         DRIVE TYPE 26

0593 07

0594 0000

0596 FFFF

0598 00

0599 00

059A 00 00 00

059D 04C7

059F 11

05A0 00

05A1 04C8         DRIVE TYPE 27

05A3 0B

05A4 0000

05A6 FFFF

05A8 00

05A9 08

05AA 00 00 00

05AD 04C7

05AF 11

05B0 00

05B1 04C8         DRIVE TYPE 28

05B3 0F

05B4 0000

05B6 FFFF

05B8 00

05B9 08

05BA 00 00 00

05BD 04C7

05BF 11

05C0 00

05C1 0400         DRIVE TYPE 29

05C3 08

05C4 0000

05C6 FFFF

05C8 00

05C9 00

05CA 00 00 00

05CD 03FF

05CF 11

05D0 00

05D1 0400         DRIVE TYPE 30

05D3 0B

05D4 0000

05D6 FFFF

05D8 00

05D9 08

05DA 00 00 00

05DD 03FF

05DF 11

05E0 00

�����������������������������������������������������������������

Award BIOS Reference  6

05E1 0396         DRIVE TYPE 31

05E3 0B

05E4 0000

05E6 FFFF

05E8 00

05E9 08

05EA 00 00 00

05ED 03FF

05EF 11

05F0 00

05F1 039D         DRIVE TYPE 32

05F3 09

05F4 0000

05F6 FFFF

05F8 00

05F9 08

05FA 00 00 00

05FD 039E

05FF 11

0600 00

0601 0400         DRIVE TYPE 33

0603 0A

0604 0000

0606 FFFF

0608 00

0609 08

060A 00 00 00

060D 03FF

060F 11

0610 00

0611 0400         DRIVE TYPE 34

0613 0C

0614 0000

0616 FFFF

0618 00

0619 08

061A 00 00 00

061D 03FF

061F 11

0620 00

0621 0400         DRIVE TYPE 35

0623 0D

0624 0000

0626 FFFF

0628 00

0629 08

062A 00 00 00

062D 03FF

062F 11

0630 00

�����������������������������������������������������������������

Award BIOS Reference  7

0631 0400         DRIVE TYPE 36

0633 0E

0634 0000

0636 FFFF

0638 00

0639 08

063A 00 00 00

063D 03FF

063F 11

0640 00

0641 0400         DRIVE TYPE 37

0643 02

0644 0000

0646 FFFF

0648 00

0649 00

064A 00 00 00

064D 03FF

064F 11

0650 00

0651 0400         DRIVE TYPE 38

0653 10

0654 0000

0656 FFFF

0658 00

0659 08

065A 00 00 00

065D 03FF

065F 11

0660 00

0661 0396         DRIVE TYPE 39

0663 0F

0664 0000

0666 FFFF

0668 00

0669 08

066A 00 00 00

066D 03FF

066F 11

0670 00

0671 0334         DRIVE TYPE 40

0673 06

0674 0000

0676 FFFF

0678 00

0679 00

067A 00 00 00

067D 0334

067F 11

0680 00

�����������������������������������������������������������������

Award BIOS Reference  8

0681 0000         DRIVE TYPE 41

0683 00

0684 0000

0686 0000

0688 00

0689 00

068A 00 00 00

068D 0000

068F 00

0690 00

0691 0000         DRIVE TYPE 42

0693 00

0694 0000

0696 0000

0698 00

0699 00

069A 00 00 00

069D 0000

069F 00

06A0 00

06A1 0000         DRIVE TYPE 43

06A3 00

06A4 0000

06A6 0000

06A8 00

06A9 00

06AA 00 00 00

06AD 0000

06AF 00

06B0 00

06B1 0000         DRIVE TYPE 44

06B3 00

06B4 0000

06B6 0000

06B8 00

06B9 00

06BA 00 00 00

06BD 0000

06BF 00

06C0 00

06C1 0000         DRIVE TYPE 45

06C3 00

06C4 0000

06C6 0000

06C8 00

06C9 00

06CA 00 00 00

06CD 0000

06CF 00

06D0 00

�����������������������������������������������������������������

Award BIOS Reference  9

06D1 0000         DRIVE TYPE 46

06D3 00

06D4 0000

06D6 0000

06D8 00

06D9 00

06DA 00 00 00

06DD 0000

06DF 00

06E0 00

06E1 0000         DRIVE TYPE 47

06E3 00

06E4 0000

06E6 0000

06E8 00

06E9 00

06EA 00 00 00

06ED 0000

06EF 00

06F0 00

�����������������������������������������������������������������

Award BIOS Reference  10