Conner.Exe is a utility

to set automatic power-down for Conner 3 inch IDE-drives.

Call: Conner Power-down delay (minutes)

delay from 1 to 20: Power-down after (minutes) idle time

delay from -1 to -20: Additionally turn off drive now

delay = 0: Turn on drive, disable auto power-down

Notes: Tested with OS/2 2.0, 1.x and DOS for:

CP3024 (20MB), CP3044 (40MB), CP30104 (115MB).

DO NOT USE WITH OTHER DRIVES! USE AT YOUR OWN RISK ONLY!

Version 3 (US & German message), R.W.Saddey, Berlin, Germany; DEC-92

For the curious ones:

Source in Conner.C to be compiled using MS C6.00a

cl -G0s -Oxaz -Lp -Fb Conner.C Conner.Def /link /ali:16 /e /packc

R.W.Saddey CIS 100022,571