___________________________________________________________________________

STAC FAX                                                           STAC FAX

Title: Loading Stacker High with DOS 5.0 or Windows 3.1.

STACKER NOTE TEC015 - 6/19/92                          STAC FAX Index #2102

___________________________________________________________________________

Background.

MS-DOS 5.0 and Windows 3.1 come with  an expanded memory manager,

EMM386.EXE, that enables the loading of device drivers and TSRs into the

Upper Memory area on 386 and 486 machines.  The Stacker device driver may

be loaded into this area if the system is configured properly and if there

is an upper memory block (UMB) large enough to accommodate the driver's

initialization size.  Stacker reports the initialization size of its

device driver at boot time.   As EMM386 loads, it reports the size of the

largest UMB. (Use the pause key during system boot to view these figures.)

If you attempt to load the Stacker driver into the UMBs and they are not

large enough, the driver will simply load into the lower 640k of

conventional memory.

Optimizing upper memory usage can be a complex task.  Freeing up UMBs that

are large enough may require much trial and error.  You may even have to

experiment with the order in which device drivers are loaded.  This

document provides the necessary configuration information to get you

started.

Requirements.

1. The computer must be a 386 or 486 with at least 1MB of memory.

2. The Config.sys file must contain the following statements (in this

order):

DEVICE=C:\<dir>\HIMEM.SYS               (where <dir> is DOS or WINDOWS)

DEVICE=C:\<dir>\EMM386.EXE   switches   (switches are explained later)

DOS=UMB   (or DOS=HIGH,UMB)

3. You need to tell DOS to load the Stacker device HIGH by changing the

syntax from  DEVICE=  to   DEVICEHIGH= in Config.sys.  For example,

DEVICE=C:\STACKER\STACKER.COM  C:\STACVOL.DSK

becomes     DEVICEHIGH=C:\STACKER\STACKER.COM  C:\STACVOL.DSK

NOTES:

1. Make sure HIMEM.SYS and EMM386.EXE exist on the uncompressed boot

drive or they will not be available during system startup.

2. Stacker's SSWAP program is not memory resident.  Therefore, it is

not necessary to load its device driver high.

EMM386.EXE Switches.

The switches (parameters) tell EMM386 to perform a variety of functions

like opening the Upper Memory Area for device drivers and TSRs, or to

create Expanded (EMS) memory for applications.  The switches you choose

will depend on your applications and the type of memory they require.

1. If some applications use EMS memory, use the RAM parameter.  This

will provide access to the UMBs and create a 64k EMS page frame.

For example:  DEVICE=c:\DOS\EMM386.EXE RAM

2. If no EMS memory is required, use the NOEMS parameter.  This

will provide access to the UMBs and create 64K more usable space

than the RAM parameter.  No EMS services will be provided.

For example:  DEVICE=c:\DOS\EMM386.EXE  NOEMS

3. In order to free up the MAXIMUM amount of upper memory, you can

add the m9 parameter.  When used in conjunction with the RAM

parameter, this will inform EMM386 to position the page frame

at address E000h, freeing up an additional 64K of RAM.  When used

with the NOEMS parameter, the 64k memory block starting at E000h

will be added to the available UMBs.

For example:   DEVICE=c:\DOS\EMM386.EXE NOEMS m9

This line will free up the maximum amount of UMBs on most systems.

It is possible to specify other memory starting address with the

m# parameter.  Please consult your DOS or Windows manual.

NOTE: This option will work on most IBM-compatible machines.

However, a few machines like the PS/2 contain BIOS information

at E000.  You will not be able to use the M9 option on these

machines.

Checking Stacker's memory location.

To see if the Stacker device driverwas successfully loaded into the UMBs,

use the DOS MEM command.   Type:    MEM /C      to view the memory map.

The Stacker XT/8 coprocessor card and EMM386.

If you have the XT/8 coprocessor card, it will be necessary to

exclude the 16K upper memory address range that it uses from use

by EMM386.  For example, if the coprocessor base address is cc00,

add  x=cc00-cfff to the EMM386 device driver.  You can determine the

coprocessor address by examining the /B= parameter on the Stacker

device line in Config.sys.  For example, if /B=CC00, the EMM386

line would read:

DEVICE=C:\DOS\EMM386.EXE NOEMS m9 x=cc00-cfff

Failure to exclude the address will result in the message "Coprocessor

not functioning, invoking software driver".

The following table lists the seven possible memory ranges, based on the

base address of the XT/8 coprocessor card:

Base Address   <range> to Exclude

------------   ------------------

c400           c400-c7ff

c800           c800-cbff

cc00           cc00-cfff

d000           d000-d3ff

d400           d400-d7ff

d800           d800-dbff

dc00           dc00-dfff

NOTE: The Stacker AT/16 card uses an exclusive I/O address so

it requires no special treatment.

___________________________________________________________________________

Copyright 1992, Stac Electronics                    STAC FAX (619) 431-6712