___________________________________________________________________________

STAC FAX                                                           STAC FAX

Title: Using QEMM386 and Stacker

STACKER NOTE TEC022                                    STAC FAX Index #2105

___________________________________________________________________________

Quarterdeck Technical Note #214

By: Robert Ngan

I. Hardware Considerations.

The AT/16 coprocessor card and the MC/16 microchannel card tested were

found to work with QEMM 6 without the need to exclude any High RAM

addresses.

The XT coprocessor card needs an exclude and this address is normally at

CC00-CFFF but is changeable during the Stacker install process. This is

an 8-bit card designed for use on XTs, although one can also use it on

AT machines.  QEMM will need an EXCLUDE for this RANGE.

For example: if the card uses CC00-CFFF, then add X=CC00-CFFF to the

QEMM device line in the config.sys.

II. Using Stacker 2.0 with EMS memory and QEMM's Stealth.

a) When installing Stacker 2.0 on a machine with QEMM already

present.  The Stacker Install program will ask whether or not you

wish to use EMS.  Selecting yes would add the /EMS switch to the

Stacker device line.  If you want to use EMS with Stacker 2.0 and

QEMM's Stealth option, make sure to add DBF=2 to the QEMM device

line before installing Stacker 2.0.  Otherwise the machine will just

reboot and go into a loop saying that you need to add DBF=2. DBF is

needed because Stacker 2.0 is accessing the disk via the page frame,

and this is not allowed when using Stealth.

The only way out of this loop is to REBOOT, then hold down the ALT

key after you hear a beep.  You will be prompted to press the

Escape key to unload qemm.  Press escape, then edit the config.sys

(on the uncompressed drive) and add DBF=2 to the QEMM386.sys device

line,

Device=c:\qemm\qemm386.sys dbf=2

IMPORTANT: This change should be done on the uncompressed drive.

b) When installing QEMM 6 on a machine with Stacker 2 already

installed, if you plan to let Stacker 2.0 use EMS provided by QEMM

6, make sure to edit the config.sys and add the DBF=2 parameter to

the QEMM device line in the config.sys.  At the same time, you

could add /EMS to the Stacker device line to allow it to use EMS.

II. Running OPTIMIZE with Stacker 2.0.

a) How can I make OPTIMIZE work with Stacker?

Optimize fails because of the SSWAP.COM driver in the config.sys.

SSWAP's line in config.sys typically looks like:

DEVICE=C:\Stacker\SSWAP.COM C: D: /SYNC

SSWAP.COM is the non-resident program that switches the drive

letters. In the example above, drive C: becomes drive D: and drive

D: becomes drive C:. Optimize reboots your machine a few times and

writes data to the disk during this process. SSWAP.COM will make

Optimize think it is writing to C when in fact it was writing to D.

1. When running Optimize, this device line needs to be either

remarked or removed temporarily from the CONFIG.SYS.  Also,

your AUTOEXEC.BAT (and some lines in the config.sys that came

after SSWAP.COM) probably contains references to drive C:. All

these references (except for references to files that you know

exist on both the compressed and uncompressed drive) need to be

changed to drive D:. Conversely, all references to drive D:

should be changed to drive C:. This is necessary since Optimize

requires that the CONFIG.SYS and the AUTOEXEC.BAT reside on

drive C:.

2. When installing Stacker 2.0 on a machine with QEMM already

installed, Stacker2 will automatically copy most of the files

Optimize needs to the \QEMM subdirectory of the boot

(uncompressed) drive. As of the date of this writing, the

Stacker2 install program did not copy TESTBIOS.COM (needed for

STEALTH testing) and WINSTLTH.VXD (needed for Windows enhanced

mode if using STEALTH).  This means that you have to copy the

TESTBIOS.COM and WINSTLTH.VXD from the compressed drive to the

uncompressed \QEMM subdirectory. Go to step 4...

3. When installing QEMM on an existing Stacker drive, create a

QEMM sub-directory on the uncompressed drive. (That is C: since

we already REMOVED SSWAP.COM in step 1.) Some files need to be

in the QEMM sub-directory in order to run Optimize properly:

QEMM386.SYS, OPTIMIZE.COM, LOADHI.SYS, TESTBIOS.COM,

LOADHI.COM, BUFFERS.COM, RSTRCFG.SYS, MCA.ADL (if

you are running on a microchannel machine), WINHIRAM.VXD and

WINSTLTH.VXD (if using Windows 3 enhanced mode)

4. Once you have done this, you should reboot before running

Optimize so that the drives are set up correctly.  Now you

should be able to run Optimize normally.  After Optimize has

run, you should edit your CONFIG.SYS and restore the SSWAP.COM

line.  Also, you should edit your AUTOEXEC.BAT to restore the

drive specifications to what they were before; e.g. change all

references to drive D: to drive C: and all references to drive

C: to drive D:.  Reboot again.  You are now getting the most

out of your conventional memory.

___________________________________________________________________________

Copyright (C) 1991, by Quarterdeck Office Systems.  STAC FAX (619) 431-6712