___________________________________________________________________________

STAC FAX                                                           STAC FAX

Title: Creating a Bootable Stacker Recovery Disk.

STACKER NOTE TEC025 - 8/20/92                          STAC FAX Index #2504

___________________________________________________________________________

Background.

Stacker drives are 'mounted' at boot time by a device driver in the

Config.sys file.  If you were to boot from a "plain vanilla" system

disk, which had no Config.sys file or Stacker device driver statement(s),

your Stacker drives would appear to be no more than large hidden files

on the hard disk.  Creating a boot disk that enables access to the

Stacker drives involves making a DOS startup disk, then creating a

Config.sys file on that disk which contains the Stacker device drivers.

This note outlines the required steps to create such a disk.

Procedure.

1.  Making the startup disk:

A. Create a DOS startup disk by inserting a formatted diskette in

drive A: and typing:

SYS C: A:

Note: If you are running DOS 3.x, you will need to copy Command.com

to drive A: after 'sys'ing the disk.

B. Next, copy Config.sys from your hard disk to this diskette.

For example, at the command prompt type:

Copy C:\Config.sys A:            <ENTER>

This disk is now capable of starting the system and mounting the

Stacker drives.  (It is not necessary, but you may wish to copy

C:\Autoexec.bat to this disk as well).

2.  Making a Stacker diagnostic disk:

You may wish to make this disk more "plain vanilla" for trouble shooting

purposes.  Just edit Config.sys with your favorite text editor and

eliminate all but the Stacker device drivers.  For example, to modify

Config.sys using Stacker's SEDIT text editor, type:

C:\STACKER\SEDIT  A:\CONFIG.SYS     <ENTER>

You should now see a listing of the Config.sys contents.  Locate the

Stacker device drivers.  They will look similar to:

DEVICE=C:\STACKER\STACKER.COM C:\STACVOL.DSK

DEVICE=C:\STACKER\SSWAP.COM   C:\STACVOL.DSK /SYNC

Now delete all but these Stacker devices.  For example, if you are using

SEDIT, position the cursor at the beginning of the line using the arrow

keys. Now press  <Ctrl> K    to delete the line.  Repeat for all the

lines you wish to delete.  When you are finished, press <Ctrl> Z to save

the file.

Note: If you are using a device driven drive such as a Hardcard or

CD-ROM, or have non-DOS partitions requiring device drivers, you

will need to leave their device lines in the Config.sys file.

___________________________________________________________________________

Copyright 1992, Stac Electronics                    STAC FAX (619) 431-6712