IOLINE SYNTAX COMMANDS

IOLINE LP4000 AND 3700 ONLY

This document is a transcription of documentation of the

Ioline-specific command language for the Ioline 400 and 3700

plotters.  Transcribed by Jon Fleming, May 15 1992, with added notes,

from a paper copy supplied by Ioline.  Distributed with permission.

Ioline Plotters Connection guide: Ioline Syntax Commands

(LP4000 & 3700)

Ioline LP4000 plotters are programmed to interpret commands sent

in a special language that we use to operate the plotter from such

programs as ROLL.EXE, the Ioline passive roll feed driver program,

and KEYS.EXE, the Ioline keyboard emulation program.  These commands

are sent to the plotter in DM/PL mode, so must be prefaced with a ;:

plotter select.  (Added by JRF:  perhaps each "_" code at the end of

a keypad command string should be followed by a ";:" code in case the

plotter is programmed to go on-line in HP mode).  Here is a list of

commands:

!C  Chart hold.  This is a toggle, lifting the chartwheels if they

are down, lowering them if they are up.

!fn Frame advance.  n = advance distance in .0025 inch units (EC2).

The plotting media will slowly advance in the X axis (paper

movement) direction to a position "n" units from the Lower Left.

An optional parameter, EL, added to the command after "n", will

cause the plotter to pause for sheet cutting or inspection before

resuming after the ALT key is pressed.

!Fn Pull sheet off roll.  n - advance distance in .0025 inch units

(EC2).  The plotting media will slowly advance "n" units, to pull

the paper evenly and smoothly off the roll.  The media is then

quickly returned to the existing Lower Left point.

!H  Toggle plotter language to HP-GL.  As mentioned earlier, Ioline

syntax commands are sent in the DM/PL mode when the ";:" plotter

select command is sent to the plotter.  In order to change back

to HP-GL language, this command can be used.

!S  Keyboard command string.  Usage: !S [keypad characters]_ .  The

"_" (underline) must be used as a terminator after the keypad

commands.  With this command you can reproduce any set of

keystrokes on the plotter keypad from a file sent from the host

computer.  The following ASCII characters represent pressing the

associated key on the plotter keypad:

F    SIZE

%    ALT

I    ON LINE

Q    VIEW

V    SPEED / DELAY

E    SCALE

*    DIGIT / GOTO

.    . / ORIGIN

Z    RESET

K    CHART HOLD

U    PEN UP

D    PEN DOWN

[    LL

]    UR

0-9  Numbers 0-9

Some notes on using Ioline Syntax Commands:

General:

The plotter must be online to receive commands of this type from

the host computer.  If you are going to be advancing paper off a

roll, or setting the paper size, pressing LL UR ON LINE at the

plotter keypad will accomplish this.  Setting a paper size of some

kind is necessary to put the plotter online, and this sets a paper

size of 0x0 dimensions.  If a paper size is set with an X - axis

dimension, the paper will be automatically viewed back and forth,

which would be undesirable if the paper is being pulled off a roll.

[Added by JRF:

When sending commands to move the pen carriage or paper IN A

KEYPAD COMMAND STRING, it is sometimes desirable to use negative

numbers.  Positive numbers for movements pull paper off the roll and

move the pen carriage away from the keypad.  To move in the opposite

directions (negative numbers), add the negative number to 66,536 and

use the result in the command string.  For example, to move the pen

carriage 17 inches towards the keypad, the command string looks like:

;:EC2A!SI%*0.58736*I_

where ";:EC2A" selects the plotter, sets step size to 1/400 inch, and

sets absolute coordinate addressing.  "!S" starts a keypad command

string. "I" takes the plotter off-line.  "%*0.58736*" moves the pen,

as if ALT GOTO 0 . 5 8 7 3 6 GOTO were pressed at the keypad.  "I"

puts the plotter back on-line and "_" terminates the keypad command

string.

Negative numbers for the "F" and "f" commands are entered

directly as in "F-1000".

When using "[" to set the Lower Left corner, note that this

cancels any setting of Upper Right corner; whenever "[" is used a "]"

must follow it somewhere before the plotter is put back on-line.

End JRF addition]

For roll feed functions:

Generally, two types of command string files are used to control

roll feed operations; Start files, and End files.  The functions

performed by a Start file might include pulling paper off the roll,

setting paper size, setting plot orientation, setting plotting scale,

setting plotting speed and acceleration, etc..  The functions

performed by an End file might include advancing the Lower Left or

origin to the next frame on the roll, advancing the frame to the cut

line and pausing for the sheet to be cut off, then retreating to the

origin for the next plot.

A typical Start file looks like this:

;:EC2A!F18200!SI[%*17600.13600*]%*[I_

;:EC2A    Plotter select and setup - sets default step size at

.0025 inch, and absolute coordinate addressing.

!F18200   Pulls 45.5 inches of paer off the roll at slow speed

and acceleration, then quickly returns to Lower Left.

!SI[%*17600.13600*]%*[I_      A keypad command string, commands

used are:

I         ON LINE  The plotter must be taken offline for

some keys on the plotter keypad to be enabled.

This is true of keypad command characters sent

from the computer as well.  The ON LINE key is a

toggle, that is it puts the plotter online and

also takes it offline.

[         LL  This sets the current pen position as the plot

Lower Left.  In DM/PL mode, this is also set as

the Origin.  In HP-GL mode, this is also set as

point P1.

%*17600.13600*]%*   ALT GOTO 17600 . 13600 GOTO   This

causes the pen to move 34 inches, and

the paper to move 44 inches, to the

point we wish to set as the Upper Right.

]         UR    This sets the current location as the plot

Upper Right.

%*]       ALT GOTO LL    This causes the pen and paper to

move so that the pen will be at the Lower Left

location, prepared to plot.

I         ON LINE    This puts the plotter back online.

_         (underline)   This is the terminator, to indicate

the end  of the keypad command string.

This was a fairly basic Start command file.  There are many other

functions that can be added, by adding more keypad command strings.

There are some examples:

!S%FV20V       ALT SIZE SPEED 20 SPEED    This will set the

plotter acceleration at 2 G's.  Any value from 1

to 20 can be entered, for .1 to 2 G's

acceleration.  (1 G = 32 feet / second squared)

!SV20.V_       SPEED 20 . SPEED    This will set the plotter

speed at 20 inches per second.  Any value from 1

to 20 can be entered, for speeds from 1 inch per

second to 20 inches per second (axial speeds).

The "." after the number will make this command

absolute, maintaining that speed regardless of any

speed commands sent from the host computer, as part

of a plotfile, for example.

!S%D_          ALT PEN DOWN   This will cause the plotter to

disregard any pen change requests, and plot a

multicolor plot as a single pen.  This is useful

for check or test plots.

!SE50E_        SCALE 50 SCALE    This will cause the plotter to

plot the drawing at 50% scale.  This is also

useful for quick check plots.  Any value from 1% to

999% can be entered, for reduction or enlargement.

A typical End file looks like this:

;:EC2A!f17600!SI[]I_!f1820EL!f-1820

;:EC2A    Plotter select and setup - sets default step size at

.0025 inch, and absolute coordinate addressing.

!f17600   Advances the sheet to the x - axis (paper direction)

limit, and stops at this location.

!SI[]I_   ON LINE LL UR ON LINE    Takes plotter offline, sets

Lower Left and Upper Right (at the same spot) and puts

the plotter back online.  This is just to set a new

origin reference point, allowing removal of the

just-plotted sheet.

!f1820EL  Advances the sheet in the x-axis direction to the point

where that paper limit of the just-plotted is at the

cutoff guide line on the front of the plotter.  The EL

parameter added at the end causes the plotter to pause,

waiting for the operator to cut off the finished plot

and press the ALT key on the plotter keypad, resuming

operation at the next command.

!f-1820   This causes the plotter to pull the edge of the paper

back from the cutoff guide line on the front of the

plotter, preparatory to starting the next sheet.