The switch now contains code to support external Hardware Drivers.

The following is an outline spec of the interface. This feature is still

under development - any comments from potential users would be appreciated.

The driver is called by a software interrupt, and must be loaded before

the switch. Each driver can support multiple channels, and more than one

driver may be loaded, so long as each has its own software interrupt number.

At the moment the driver supplies only the Level 1 funtions, ie it effectively

provides a KISS-like interface. Later I hope to support external Level 2

drivers as well.

The transmit buffer will have a 5 byte header, with the length in 4th and

5th bytes. The length includes the header. The received frame passed to

the switch should have the same format.

;	THE ROUTINE IS CALLED VIA A SIMULATED INT, SO SHOULD RETURN

;	WITH AN IRET. THE DRIVER MAY SUPPORT MULTIPLE LOGICAL CHANNELS -

;	THE CHANNEL NUMBER IS PASSED IN AL

;

;	THE ROUTINE MUST PRESERVE ALL SEGMENT REGISTERS, UNLESS USED TO

;	RETURN A VALUE (IE DS IN FUNCTION 1). OTHER REGISTERS NEED NOT BE

;	SAVED

;

;	THE ROUTINE MUST PROVIDE THE FOLLOWING FUNCTIONS:

;

;

;	AH=0	INITIALISE. CALLED ONCE ON LOAD

;

;	AH=1	RETURNS POINTER TO RECEIVED FRAME IN DS:SI, LENGTH IN CX

;		RETURNS CX=0 IF NO FRAME READY

;

;	AH=2	SEND FRAME IN BUFFER IN DS:DI - LENGTH IS IN WORD AT 3[DI]

;						DATA FOLLOWS LENGTH

;

;	AH=3	STATUS. RETURNS AX=0 IF TX NOT BUSY. CX=0 IF RX NOT READY

;

The driver does not need to queue transmitted frames, but performance is

likely to be improved if it can buffer a reasonable number of frames. The

receive process must always be capable of buffering frames.

John Wiseman, G8BPQ

9/2/92

The following sample code is equivalent to the INTERNAL Loopback

driver - ie anything sent to it is received.

PAGE    62,132

;

CODE   SEGMENT PUBLIC 'CODE'

ASSUME  CS:CODE,DS:CODE,ES:CODE

;

;	INTERFACE TO SWITCH IS VIA SOFTWARE INTERRUPT

;

;	COMMANDS SUPPORTED ARE:

;

;	SUBCHANNEL IS IN AL

;

;	AH = 0	INIT Driver

;

;	AH = 1  Poll for data. Returns CX = 0 if none available,

;		otherwise CX=length, DS:SI=Data

;

; 	AH = 2	Send frame. DS:DI = Buffer header.

;

;	AH = 3  Status - Return AX=0 If OK to send a frame.

;			        CX=0 If no frame available to receive

;

ORG	100H

GO:

JMP	INIT

SWITCHINT:

CMP	AH,3

JE	STATUS

CMP	AH,2

JE	PUTFRAME

CMP	AH,1

JE	POLL

IRET

STATUS:

MOV	AX,0

CMP	CS:FRAMEFLAG,0		; TX BUSY?

JZ	STAT00			; NO

MOV	AX,1

STAT00:

CMP	CS:FRAMEFLAG,0		; RX READY?

JE	POLLRET			; NO

MOV	CX,1

IRET

POLL:

CMP	CS:FRAMEFLAG,0

JE	POLLRET

PUSH	CS

POP	DS

MOV	SI,OFFSET FRAMECOPY

MOV	CX,3[SI]

MOV	FRAMEFLAG,0

IRET

POLLRET:

MOV	CX,0

IRET

PUTFRAME:

PUSH	SI

PUSH	DI

PUSH	CX

PUSH	ES			; save regs

MOV	CX,3[DI]

MOV	SI,DI

MOV	DI,OFFSET FRAMECOPY

MOV	CS:FRAMEFLAG,1

PUSH	CS

POP	ES			; ES TO OUR SEGMENT

REP MOVSB			; SAVE FRAME

POP	ES

POP	CX

POP	DI

POP	SI

IRET

FRAMECOPY	DB	400 DUP (0)

FRAMEFLAG	db	0

ENDOFCODE	LABEL	BYTE

MSG	DB	'Sample EXTERNAL Interface for G8BPQ switch '

db	0dh,0ah,'$'

ERRORMSG	DB	0AH,'Parameter error:',0dh,0ah,0ah

db	'Format is DRIVER interrupt'

db	0dh,0ah,'Program NOT installed'

DB	0dh,0ah,0AH,'$'

INIT:

MOV	DX,OFFSET MSG

MOV	AH,9

INT	21H

;

MOV	SI,82H			; PARAM STRING

CALL	GETVALUE		; interrupt number

JC	BADPORTVAL

MOV	AL,BYTE PTR NEWVALUE

OR	AL,AL

JZ	BADPORTVAL

;

MOV	AL,BYTE PTR NEWVALUE	; VECTOR

MOV	DX,OFFSET SWITCHINT

MOV	AH,25H

INT	21H			; SET VECTOR

;

MOV	DX,OFFSET ENDOFCODE

INT	27H

BADPORTVAL:

MOV	AH,9

MOV	DX,OFFSET ERRORMSG

INT	21H

RET

NEWVALUE	DW	0

D16		DW	16

D10		DW	10

GETVALUE:

;

;	EXTRACT NUMBER (HEX OR DECIMAL) FROM INPUT STRING

;

MOV	NEWVALUE,0

LODSB

CMP	AL,'$'			; HEX?

JE	DECODEHEX

DEC	SI

VALLOOP:

LODSB

CMP	AL,' '

JE	ENDVALUE

CMP	AL,0DH

JE	ENDVALUE

CMP	AL,','

JE	ENDVALUE

;

;	ANOTHER DIGIT - MULTIPLY BY 10

;

MOV	AX,NEWVALUE

MUL	D10

MOV	NEWVALUE,AX

MOV	AL,-1[SI]

SUB	al,'0'

JC	DUFFVALUE

CMP	AL,10

JNC	DUFFVALUE

MOV	AH,0

ADD	NEWVALUE,AX

JC	DUFFVALUE

JMP	VALLOOP

DECODEHEX:

HEXLOOP:

LODSB

CMP	AL,' '

JE	ENDVALUE

CMP	AL,0DH

JE	ENDVALUE

CMP	AL,','

JE	ENDVALUE

;

;	ANOTHER DIGIT - MULTIPLY BY 16

;

MOV	AX,NEWVALUE

MUL	D16

MOV	NEWVALUE,AX

MOV	AL,-1[SI]

SUB	al,'0'

JC	DUFFVALUE

CMP	AL,10

JC	HEXOK

SUB	AL,7

CMP	AL,10

JC	DUFFVALUE

CMP	AL,16

JNC	DUFFVALUE

HEXOK:

MOV	AH,0

ADD	NEWVALUE,AX

JMP	HEXLOOP

ENDVALUE:

CLC

RET

DUFFVALUE:

STC

RET

CODE	ENDS

;

END     GO

John Wiseman, G8BPQ

21/3/92

###########