DIPOLE.DOC                       by:  Ira F. Kavaler - Aug, 1963

Major enhancements:  Sept, 1979 & Aug, 1990

Version 6.862 - 6/16/92

This program will both design and analyze the performance of

dipole (single element) and doublet (multi-element) antennas.

Although these antennas are primarily used for both transmitting

and receiving, that property only makes their performance that

much better!

I.        Number of Elements.

-------------------

You will first be asked for the number of elements.  A one

element antenna of this design is a dipole, having a total

physical length of one half wavelength at the main resonant

frequency.  To be more accurate, an exact half wavelength antenna

at resonance (operating at the design frequency) would have an

input impedance of approximately 73 ohms.

Since I use these antennas with 52 ohm coaxial cable (types

RG8/U and RG58/U), I have adjusted the design to provide antenna

lengths to match these types of transmission limes.  The errors

created for other common transmission line impedances, namely 75

and 300 ohms, are negligible.  A 300 ohm dipole would usually be

in the form of a "folded" dipole, having a one half wavelength

design formula, as would a straight 75 ohm half wavelength dipole.

No effort has been made to revise or add formulas to take

into account the height of the antenna above ground, nor mutual

coupling amongst the elements.  In fact, the main antenna

radiation resistance formula has to be approximated by numerical

methods as it consists of a calculus formula that cannot be

integrated.  Further approximations are used to speed the massive

amounts of mathematics required for each element at each

evaluation frequency.  Some other electrical characteristics are

documented in graphical form rather than by formulas, and thus

formulas had to be derived from the graphs.

A standard one element dipole will have one main resonant

frequency at which point the input impedance will be purely

resistive and match the transmission line impedance;  however, at

odd multiples of the resonant frequency the resistive portion of

the antenna input impedance will approach, but not reach, the

desired design level.  At a slightly different frequency, the

reactance portion of the impedance will pass through zero ohms.

Thus many dipoles are sometimes used at their third harmonic;

that is, three times the main resonant frequency.

An antenna will be useable from a practical standpoint over a

frequency range of from approximately 2% below the main resonant

frequency to approximately 2% higher than this frequency.  This

characteristic is determined from the level of antenna impedance

to transmission line impedance mismatch, and is calculated from

the "standing wave ration" (SWR).

A perfectly matched system (antenna and transmission line

operating at the main resonant frequency) will have an SWR of 1:1

(pronounced one-to-one).  As the operating frequency of the

transmitter is changed, the mismatch will increase to higher SWR

figures.  The higher the SWR, the higher the transmitter power

that will NOT be radiated by the antenna, but will be reflected

back down the transmission line to the transmitter output stage.

At an SWR of 3:1, half the forward power will be reflected

back to the transmitter.  High levels of reflected power will

cause the following undesirable characteristics:

1. the output stage of the transmitter will overheat and will

eventually weaken the transmitting tubes or transistors,

and could cause arcing across the plates of the

transmitter tuning capacitors,

2. the peak standing wave voltage level on the transmission

line could exceed the maximum rating of the line, causing

it to arc through the dielectric and be permanently

damaged,

3. the radio frequency (RF) fields cannot be contained within

the transmission line, and will cause the line to radiate,

becoming part of the antenna system, supporting

significant radiation at the undesirable harmonic

frequencies, which leads to RF interference, and

4. the mismatch will reduce the efficiency of any in-line

devices, such as filters, and increase the attenuation of

the line beyond the characteristic loss figures published

for that line.

It is usually stated by manufacturers that the SWR should not

exceed 2:1 for most transmitters.

If the operating frequency range (bandwidth) of a dipole is

insufficient then multiple elements can be requested, up to 5.  As

the resonant frequency difference amongst the elements is reduced,

the individual impedance of each element will "load" (detune) the

other element(s).

II.       The Automatic Design Mode and The Design Screen.

------------------------------------------------

An Automatic Design Mode is now included which can be invoked

by entering the letter "a" instead of specifying the number of

elements.  Upon supplying the evaluation frequencies (see section

IV. below), the program will determine the antenna design that has

the minimum number of elements.  This design will completely cover

the desired frequency spread while maintaining an SWR under 2:1.

Whenever the antenna design yields a wider frequency spread

than requested, the program will indicate the resultant minimum

and maximum frequencies for an SWR = 2:1.  You will also be given

the opportunity to align the antenna bandpass to the lowest or

highest evaluation frequency, or center the antenna bandpass over

the evaluated frequency spread.

The design will proceed with the resonant frequency and

physical lengths of each element being computed and displayed.

III.      Element Resonant Frequency & part of The Design Screen.

-------------------------------------------------------

Next you will be asked for the resonant frequency of each

element of the antenna.  If more than one element was requested,

the resonant frequencies need not be entered in any special order,

and the physical length of that element will be displayed in feet

and inches.

IV.    Evaluation Frequency Range & the rest of The Design Screen.

----------------------------------------------------------

Next you will be asked for the lowest frequency (starting

frequency) for the evaluation.  This need not be lower that the

lowest resonant frequency.

Next you will be asked for the highest frequency (ending

frequency) for the evaluation.  This need not be higher that the

highest resonant frequency, and the evaluating frequencies will be

readjusted if entered in reverse order.

As explained in section V. below, the program will select 41

discrete evaluation frequencies, equally spaced between the two

limiting frequencies you specify.  If any discrete evaluation

frequency happens to equal an even harmonic frequency of any one

of the elements of the antenna, the impedance of that element at

that frequency will be infinity.  This will cause a numeric

overflow in the computer, and usually result is the program

crashing (aborting).  Should this occur, simply re-enter the same

element resonant frequencies and either:

1. lower the lowest evaluation frequency, or

2. raise the highest evaluation frequency.

V.        The Table Screen.

-----------------

With all input data entered, the program will start the

evaluation.  The first screen will be a table containing a listing

of frequencies, SWR's and input impedances in polar notation

(magnitude of impedance in ohms at a phase angle in degrees).  The

listings will be displayed in red whenever the SWR exceeds 2:1, in

yellow over 1.5:1, in blue over 1.1:1, and in green below 1.1:1.

The table will be in two sections to allow the evaluation at

41 discrete frequencies in the evaluation frequency range.  At the

top of the screen will be an indication of the number of elements,

a listing of the resonant frequencies of each element, the

physical length in feet and inches of the longest element, and the

geometric mean (average) resonant frequency of the antenna.

In the full version of the program you can abort the

evaluation of the antenna during this phase by momentarily

pressing the [ESC] key.

When the table is completed a flashing prompt will indicate

when to proceed to the next screen.  Pressing the key prior to

that point will not allow the table to be readable once completed.

VI.       The Graph Screen.

-----------------

In order to graphically see the performance of the antenna

system, the next screen will plot the SWR between 1:1 and 5:1 over

the evaluation frequency range.  If one or more of the element

resonant frequencies fall in this range, they will be displayed as

dashed vertical lines on the graph.  The critical SWR level of 2:1

will be displayed as a slightly heavier line horizontally across

the graph.

When the graph is completed press any key to proceed to the

next screen.  Pressing a key prior to that point will not allow

the graph to be readable once completed.

VII.      The Chart Screen.

-----------------

The overall characteristic of the antenna system can next be

viewed from a Smith Chart which is based on a unit impedance of 52

ohms.  Only the most important lines of the chart will be

displayed along with a central circle indicating an SWR of 2:1.

The center of the chart is at a point where the SWR = 1:1, and the

outermost circle is where the SWR = infinity:1.

The starting (low) and ending (high) frequency points are denoted

on the plot.

When the chart is completed press any key to proceed to with

the next antenna design, which will also permit termination of the

program if desired.  Pressing a key prior to that point will not

allow the chart to be readable once completed.

VIII.     Frequency List.

---------------

The following list of frequencies is not meant to be

complete, nor has it been checked for accuracy with the latest FCC

issued frequency charts; its purpose is to supply information for

design purposes to users with limited technical knowledge.

Amateur Radio (Ham) Bands

-------------------------

Wavelength (name)        Low Freq.(MHz.)     High Freq.(MHz.)

160 Meters                   1.8                 2.0

80    "                      3.5                 4.0

30    "                     10.1                10.15   (WARC)

20    "                     14.0                14.35

17    "                     18.068              18.168  (WARC)

15    "                     21.0                21.45

12    "                     24.88               24.98   (WARC)

10    "                     28.0                29.7  (used to 29)

6    "                     50                  54  (used to 51.5)

2    "                    144                 148

1-1/4 "                    222 (as of 8/1/91)  225 (was from 220)

3/4   " (70 cm.)           420                 450 (used from 432)

1215                1300

2300                2350

Amateur radio bands above 2350 MHz. have been omitted.  A

complete listing of authorized frequencies can be found in the

ARRL publication, "The Amateur Radio Handbook" as well as others.

Citizens Band (CB)

------------------

11 Meters                    26.96              27.45

Radio Controlled Models      72                 76

Cordless Telephones          46                 50

Note: Including short range walkie-talkies.

Commercial/Industrial/Governmental

----------------------------------

Low Band                     30                 50

High Band                   148                174

UHF (U-Band)                450                470

T-Band (old TV ch. 14-20)   470                512

800/900/Cellular Telephone  800                960 ? *

Aviation                    110 ?              136 ?

Marine Transatlantic          1.62 ?             1.8 ?

Broadcast Radio

---------------

AM (amplitude modulation)     0.535              1.605

FM (frequency modulation)    88                108

Broadcast Television

--------------------

Channel No.

2                            54                 60

3                            60                 66

4                            66                 72

5                            76                 82

6                            82                 88

7                           174                180

8                           180                186

Channels 9 through 13 follow logically every 6 MHz.

14                          470                476

Channels 15 through 83 follow logically every 6 MHz. although

channels 14 through 20 have been reassigned for commercial use and

are no longer available for TV service except under special

circumstances; for example, channel 20 in Connecticut.  Similarly,

channels 70 through 83 have been reassigned for commercial use and

are no longer available for TV service.

Note: The High Freq. is always 6 MHz. above the Low Freq.

The picture or video carrier frequency is always 1.25 MHz.

above the Low Freq.

The color (chroma) subcarrier is always 3.58 MHz. above the

picture carrier.

The audio (sound) subcarrier is always 4.5 MHz. above the

picture carrier.

IX.       Printing.

---------

The design screen and the table screen can be printed by

using the computer's screen print function (pressing the keys

[SHIFT] and [PRT SC] simultaneously; however, since both the graph

screen and the chart screen require printer graphics you must

first execute under DOS your DOS utility GRAPHICS.COM before you

execute this program.  Each of the four screens can now be printed

using the screen print function described above, just wait until

the screen has been completed.

X.        Here's the small print.

-----------------------

This is a demonstration/trial version of the program.  It is

fully operational with all features;  however, when in the demo

mode, the data that you enter for the antenna parameters will be

used as design criteria but the performance evaluation will be

that of a two element doublet antenna designed to cover the entire

80 meter amateur radio band.

All versions of this program including its related files are

being distributed on an "AS IS" basis.  There is absolutely no

stated or implied guarantee or warrantee of usability for any

purpose or correctness of the formulas and procedures contained in

any file.

If you happen to discover an error in the program I will make

every attempt to correct the error as quickly as possible.  I am

under no obligation to replace nor make refunds for defective full

versions or demonstration/trial versions of the program.  I have

to take this posture as my cost to make even the simplest of

corrections far outweighs any monetary compensation received for

the full version of the program.

If you require any special modifications to the program I

will be happy to discuss on an individual basis the cost of

supplying modified programs and documentation.

The program was tested on a Tandy model 1000 using MS-DOS

3.2 and 3.3, a Tandy model 1000SX using MS-DOS 3.2 and 3.3, and an

IBM PS/2 model 50Z and 80, using PC-DOS 3.3.  All computers had

either CGA or VGA color graphics.  If your system has a Hercules

graphics adaptor, I suggest you obtain a color simulation program

such as SIMCGA.

XI.       And now a word from our sponsor.

--------------------------------

You can purchase the latest full unprotected version of this

program.  I will also include any other demonstration/trail

programs that I have available.  Please send $10.00 for one of the

following:

1. IBM compatible 5-1/4 inch 360 kilobyte or 3-1/2 inch 720

kilobyte disk, your choice,

2. TRS-80 Model I or III 5-1/4 inch disk (only $5.00 as there

is no Smith Chart nor Automatic Design Mode!)

to:

IRA F. KAVALER

671 East 78 Street

Brooklyn, New York 11236

I reserve the right to discontinue support for, change the

terms, or withdraw any part or all of this offer including but not

limited to the programs and its associated files at any time

without giving prior notice.

No form of this program may be used in commercial,

educational, nor governmental applications without written

authorization from the author;  such authorization may require

that a fee be paid to the author.

XII.      73's, de WA2ZIR.

----------------

In 1963 while attending the RCA Institutes in New York City,

I was taught the most crucial equation to this program, the

radiation resistance calculus formula.  I was astonished at the

amount of time required to make even the simplest of calculations

with my K+E Log Log Duplex Decitrig slide rule, four man-hours.

As an undergraduate at the Polytechnic Institute of Brooklyn

(E.E. class of '67), I became intrigued with digital computers.

My first computer program was written in machine language, along

with all required trigonometric subroutines, to solve this

formula.  The program was subsequently written in Fortran IV and

run on Polytech's IBM 7040 massive computer system.

As soon as the first scientific calculator became available,

the Hewlett Packard HP-35, I was able to reduce the time to five

minutes, but there were a lot of keystrokes to enter.  Later I

purchased the programmable version using the famous chewing gum

magnetic cards, the Hewlett Packard HP-65 calculator.  The time

was now reduced to an amazing 45 seconds, and no more hundreds of

keystrokes.

Through the years I have used this program, primarily written

in BASIC, to test the speed of various personal computers.

Believe it or not, for many years the Timex-Sinclair held the

speed record of 8 seconds.

Recently I was amazed once more by a friend who allowed me to

use a Compaq 25 MHz./386 system in his office.  The table screen

was completed in a fraction of a second, and I gasped!

I welcome your suggestions and comments about this product

and others.  I won't promise that good suggestions will be added

to the program, but they will be considered.

Thank you.

Appendix.

---------

The operating systems, programs and companies mentioned in this

file:  MS-DOS, PC-DOS, LIST, BROWSE, SIMCGA, Hercules, Tandy and

IBM are all copyrights, trademarks, and/or service marks of other

individuals or other corporations.

>>>>>  End of File  <<<<<