PCTRACE

Version 5.0

Users Manual

June 21, 1992

by

Douglas E. Ehlers

Ehlers Technical Consultants

4520 S. 58th St.

Lincoln, NE   68516

(402) 435-8440  Voice (evenings)

(701) 642-3846 BBS 9600,N,8,1 V.32, V.42bis

2

Table of Contents

Notice of Copyright............................................3

Introduction...................................................4

Installation...................................................4

System Requirements............................................5

Software Setup.................................................5

Old file Upgrades..............................................6

User support Methods...........................................6

Voice.....................................................6

BBS.......................................................7

Input/Output Menu..............................................8

Board Definition Menu..........................................9

Chip Placement...........................................10

Interactive Router............................................14

Mouse use................................................18

Autorouter....................................................18

Board Statistics..............................................19

Printer Menu..................................................21

Export Menu...................................................21

Exiting the program...........................................22

Tutorial......................................................24

RPRINT Subprogram.............................................26

DEFGEN Subprogram.............................................28

PC-Trace Keyboard control summaries...........................32

Schematic CAD.................................................33

Drawing Symbols..........................................36

Entering Components/Symbols..............................40

Making Connections.......................................43

Additional Drawing Commands..............................44

SCAD active keys.........................................49

Conclusions...................................................50

Appendix A - .NLT file format.................................51

PC-Trace Users Manual

3

Notice of Copyright

This program is protected by a copyright held by the author

and filed with the United States Government.  Copying of this

software is forbidden and a criminal offense.  Anyone caught

distributing this software without the author's consent will be

prosecuted to the fullest extent of the law.

Note:     This program was originally marketed under the name

PCROUTE but for copyright reasons has been renamed to

PCTRACE.  SCAD is short for Schematic CAD and is a

schematic capture program that is part of the PCTRACE

system.

PC-Trace Users Manual

4

Introduction

PCTRACE is a computer aided design program for printed

circuit boards.  It contains programs that can take the designer

from a schematic, to a board layout, to computer control of the

machines needed to make high quality circuit boards.  The program

should be quite adequate for most purposes.  This program is not

meant for IBM's use to design the next PS/2 motherboard.  It is

intended for the late-night engineer to build home projects.  It

contains a full featured autorouter that averages a 93%

completion rate on low density boards and 70% on higher density

boards.  It also supports  most popular printers for its output.

PCTRACE is the general name given to all the programs that

make up this system.  For example, PCTRACE includes SCAD, RPRINT,

and PCTRACE itself.

Key features include:

1.  Autorouting

2.  Graphic interactive routing

3.  Board layout functions

4.  Flexible pinouts for devices

5.  2X and 1X output for use in making PC boards

6.  Disk functions built into the software

7.  Sophisticated component and connection entry systems

8.  General statistics

9.  Up to 300 components and 1800 connections

10.  Mouse support in the interactive router

11.  Variable pad and trace sizes

12.  Single and double sided boards

13.  Manual placement of traces

14.  CGA, EGA, and VGA graphics support to display the

board. CGA:80x25 EGA:80x43 VGA:80x60

15.  Gerber, DXF, and Autocad Script file output

16.  Actual size silkscreen creation

17.  Error handling for critical errors

18.  Schematic Capture for a complete cad system.

19.  The authors continued support

Installation

Contents of PC-Trace directory:

PCTRACE   .EXE - The board layout/main program

SCAD      .EXE - Schematic CAD

RPRINT    .EXE - The PC-Trace printer output program

SPRINT    .EXE - The SCAD printer output program

CONVCDT   .EXE - Converts 4.0 and older files to 5.0

CONVDEF   .EXE - Converts 4.0 and older files to 5.0

PC-Trace Users Manual

5

*         .DEF - PC-Trace part layout files

*         .DIA - SCAD symbols

System Requirements:

To run this program requires:

1. CGA, EGA, or VGA graphics

2. 640k of memory with 520k free

3. 1 floppy disk drive

4. Hard Disk Drive or Large capacity floppy

5. One of the following printers:

a. IBM PC or compatible printer

b. EPSON LQ-2500 or compatible printer

c. HP Laserjet II or compatible

d. Adobe Postscript Compatible printer

e. HP Deskjet or Deskjet +

f. Epson MX-80 or compatible

g. IBM Proprinter 24pin must be in AGM mode

h. HP and Roland Plotters

6. Mouse is required for SCAD and usable in PC-Trace

A fast processor and math coprocessor are also beneficial.

Software Setup:

For PCTRACE to run reliably, a line must appear in the

config.sys files that says FILES=20.  If any other major

commercial software applications are in use this command probably

exists, but it should be checked.

To install the software insert disk 1 in either drive A: or

B:.  Change to the drive that the floppy disk is in.  To do this

type A: or B: from the dos prompt.  To start the installation

process, type INSTALL from the DOS prompt.  The installation

program will run and prompt for the drive that the install disk

is in.  Enter the letter and verify that it is correct.  The next

prompt is for the destination drive.  This should be the letter

of a hard-disk or alternate floppy disk that has at least 1mb of

free space.  Verify that the destination of the program is

correct.  The installation program will notify the user that it

is ready to start the installation process and that Disk 1 should

be placed in the disk drive.  This should already be the case.

Press SPACE to begin.  After a few moments, a prompt will appear

for the user to insert disk 2.  If PCTRACE was shipped on a

single floppy (3.5 inch) disk 2 is the same as disk 1.  If the

program came on two floppies (5.25 inch) insert the second disk.

Press space to finish the installation.

To execute the programs the user must change into the

PCTRACE5 directory using the command CD \PCTRACE5.  The programs

PC-Trace Users Manual

6

will not run properly if run from some other directory than

PCTRACE.  To run the programs type PCTRACE, SCAD, or RPRINT from

the DOS prompt.  Install will also create two subdirectories

called DEFILES and DIAGRAMS to store the definition files for

PCTRACE and symbol files for SCAD.

Two programs are included to help with the update process.

CONVDEF.EXE is used to modify the old .DEF files that the user

might already have created.  To use this program, copy it into

the subdirectory that holds the old .DEF files.  Type CONVDEF,

the program will run and update the file format of all the old

files.  CONVDEF can now be deleted from the hard-disk anywhere

that it appears.

The second program is used to update the .CDT files.  This

program requires that the .CDT file be in the same directory as

CONVCDT.EXE.  To convert the file, type:

CONVCDT file1.CDT file2.CDT file3.CDT.

The program will update the file from the 3.0 and 4.0 format

to the 5.0 format.  The old file will be saved in a .BAK file.

One major consideration when moving a 4.0 file to the 5.0

system is the advanced silkscreening.  The parts are referenced

to a most upper left corner for placement, this includes

silkscreen parts.  If a part exists on an old board and is

converted to the 5.0 program, that part should be left unchanged

as far as silkscreen.  If the silkscreen is drawn around that

part, it will offset the location of the pads and the existing

traces will not meet the pads in the correct place.  This will be

immediately visible as the affected parts will appear to have too

many pads.  If the user desires the silkscreen on the old board

with the 5.0 software, they should be prepared to move components

around in the interactive router to get the pads in the popper

places.  Traces that were overwritten by the incorrectly placed

pad may have to be repaired as well.

User Support Methods

Phone Calls:

All registered users are free to call anytime to notify

the author of bugs, perceived bugs, program usage questions,

suggestions of additions, and anything else that the end

user feels the need to discuss with the author.  The BBS is

also a good way to receive support for the registered user.

Un-registered users are also welcome to call during the

evening when having problems getting the demonstration

version of PCTRACE running.  The support will be more

limited.  A better, and preferred method by the author, is

for these users to use the BBS.

PC-Trace Users Manual

7

BBS Support:

Perhaps the best reason to register the demo version is

to get the support of the registered user section on the

BBS.  The complete PCTRACE system is posted in this area for

downloading.  Upon registration all users receive a password

on this system that allows them to download the most current

bug fixes any time they like.  The user name and password

appear on the first page of the manual.  Part and symbol

libraries will also be posted and hopefully uploaded by

users of the system.  Other programs deemed useful by the

author will be posted for all users as well as the demo

version of the software.

To access the BBS, call the number listed on the

opening screen of the software with a modem control package

such as PROCOMM or QMODEM.

Support is provided courtesy of the author.  Please

feel free to log in to the BBS anytime.  Voice calls should

be at a decent hour and evenings are best.  Please exhaust

all possibilities before making the call and be sure to try

and understand the problem before making the call to better

explain it to the author.  If it is possible, please try and

isolate the problem to the point that it can be repeated by

executing a certain sequence of events.  By doing this, it

becomes much easier to find and fix the errors in

programming.

PC-Trace Users Manual

8

PC-Trace Manual

Input/Output Menu

1. Load Data:

This function is used to retrieve from a disk necessary

information to design a printed  circuit board.  Select this

function  from the menu  and when  prompted, enter  the file

name.  No extension  is used, the program will add  the .CDT

extension.   The  path should  not be  used on  the filename

itself.  There is a function that will be discussed later to

set  the  path name.    After entering  the  filename, press

Return.   The program will ask if  this is the correct name,

enter Y  or  N to  answer.   If the  file is  not found  the

program will reprompt for file name.  The ESC key is used to

jump  back to  the main  menu instead  of entering  the file

name.

2. Save Data:

This function  will save a complete  description of the

design layout,  component list,  and connection list.   When

prompted, enter the filename.  Again do not specify the file

extension  or path name.   PCTRACE will  ask, after entering

the  file name, if everything is correct.   If so, enter a Y

or N.   If  the file exists  the program  will ask if  it is

alright to overwrite the file.  N will prompt for a new file

name.  ESC will bypass the prompt back to the I/O menu.

3. Erase Data:

This  routine  clears the  memory of  the program.   It

erases the boards, component list, and connection list.  The

program will ask for confirmation before erasing the data in

memory.   Answer  Y or  N.   The  program will  respond with

"Erasing..."  and after a few  seconds drop back  to the I/O

menu.

4. Import Data:

This load  routine works the  same as the  primary load

command in this menu.  The difference is that it  is made to

load a netlist  generated by SCAD.  Creation of this file is

covered in the SCAD manual.  Once this file has been loaded,

assuming that it is  a complete design, all that  remains is

to  place the  parts,  make any  minor  design changes,  and

autoroute the board.  This is a best case scenario and  more

work is usually required.

5. Disk Directory:

This function displays a listing of the all the circuit

data files on the current drive and path.  Use the space bar

to  view twenty file  names at a  time.  When  no more files

PC-Trace Users Manual

9

exist, the space bar returns to the I/O menu.

6. Change drive/path:

This  function changes the  current path.   The ESC key

will back  out of  the function  without changing the  path.

The function  will display  the current  drive and  path and

then prompt for a new one.  Just enter the new one and press

Return.

Define/Edit Board Menu

1. Component Edit/Entry:

This function is  used to enter components into  the system.

Selecting this function  will place the user in the entry/editing

environment.    If  no  components have  been  entered  only  two

possible commands  will  be shown,  if  there are  components  in

memory  the programs  shows options to  delete, copy  and perform

other functions.

New Component Entry:

Typing  an  "A" will  allow entry  of  new components.   The

program  will display the component number being entered and then

prompt for the  name.  The name  may be up to  15 characters long

and serves  only to remind  the designer  of what the  device is.

ESC can  be used at this point to return you to the entry/editing

environment.

The  next  question is  for the  designation.   This  is the

unique name  that is given to  the component and will  be used in

the  future to refer to it.   Typical names include IC1, R1, IC2,

and A1.  There are two reserved designations.  The first reserved

designation starts with "V" and then a number, so V1 would not be

accepted  but VI1 would be.   The second  reserved designation is

"GRNDP".  If this is specified  as a source or destination pad in

the  autorouter,  a  connection  to  the  ground  plane  will  be

attempted if  one is defined.   ESC may be used to  back up while

entering the designation name to reenter the component name.

Third, the program will prompt for the definition file name.

This is the name of a file that tells the program how the pins on

the device being entered are situated.  The program first shows a

list of  possible definition file names  at the lower right.   At

the end of  the list the program will prompt for  a name.  Do not

enter a file extension, the program will add it for you.   If the

name of  the  file entered  exists,  it will  read  the file  and

display the chip definition to see if it is correct.  If the user

answers "yes" the program will continue to the next question.  If

the  user  answers  "no",  the  program  will  reprompt  for  the

definition  name after displaying the possible files.  ESC may be

used  to  back up  and reenter  the  component designation.   New

definition  files can be created with the subprogram DEFGEN.  See

PC-Trace Users Manual

10

that section of the manual for information on its usage.

The  fourth prompt asks for the pad  size for the device.  A

small window will  pop up and prompt for a  pad size between 0.05

inches  and  0.15 inches.    All pads  are  circular.   Enter the

corresponding number to the pad size.

The  next question  the program  will ask  if everything  is

correct.   Answering  "no" will  restart  this process  with  re-

prompting  for the  name.   Answering  "yes"  will get  the  next

prompt.

The next prompt is to keep this device.  Answering "no" will

skip the next prompt and ask if you wish to enter another device.

Answering "yes" displays the next prompt.

This prompt asks if the user wishes to place the device now.

Entering  "no" will allow  the delay  of this  until later.   All

devices must be placed somewhere on the board before they  can be

connected.  Placing a chip goes as follows:

Placing Chips:

When the  graphic screen  comes up a  representation of

the current  circuit board with all  other placed components

and ground planes are shown.  The chip being entered will be

shown in the upper left corner of the board.  The arrow keys

will  move this chip around on the  board. By using a shift-

arrow  combination the chip can  be moved faster.   Move the

chip to its final place  and press return.  Chips cannot  be

overlapped.   The chip is now placed  in this location.  ESC

will  exit  without  placing  the  chip.    Confirmation  of

abandoning the placement  is asked.  Yes will go to the next

prompt, no will return to the place before hitting ESC.

The last prompt is to enter another chip or  not.  Answering

"yes" will put the user back at the name prompt  to enter another

chip.    No  will   put  the  user  back  in   the  entry/editing

environment.

Editing Component Definitions:

Selecting  E  from  the  entry/editing   environment  allows

editing of any part of each definition.  Simply select the letter

of the item to change.  For example, to  change the name, press A

after pressing  E and enter the  new name.  Either  enter the new

name, or press  ESC to leave  it unchanged.   When done  editing,

press the key that corresponds to exiting the edit.

If editing the placement of chips, the  chip definition file

must  be present  in the DEFILES  subdirectory for  the placechip

function to work.   The  only time this  is a  problem is if  the

board being edited is not your own, such that the chip definition

files have different names from your own.

PC-Trace Users Manual

11

Deleting Component Definitions:

This function allows removal of a  component.  The component

that  will be  deleted  is the  one  currently displayed  on  the

screen.   Press  D  and confirmation  will  be asked  for  before

deleting  the component.   Answer  accordingly.   All connections

that contained the deleted function will also be deleted.

Copying Components:

This function  will allow  copies of  a  chip definition  to

speed  the entry  of chips.    It will  copy everything  from the

displayed  definition   to  a  new  definition   except  for  the

designation  from  which it  will take  the  alpha part  and then

increment the numerical part to the next available part.  It also

does not put the chip in the same location.  The program will ask

if the user wishes to place the chip now or later.

Searching for a Component:

Sometimes finding the component  that needs to be edited  or

deleted  can  take some  time on  a large  board.   This function

allows a search  for components by  designation.  When  prompted,

enter the designation.  PCTRACE will change the case to upper. If

it exists,  the program will display that device's number.  If it

doesn't exist, nothing happens.

Incrementing and Decrementing the displayed Component:

Using  + and  -  will change  the  displayed component.    +

increments  and - decrements.  The display is circular in nature,

therefore hitting  + at  the last component  will move it  to the

first and hitting - at the first will move it to the last.

Exiting the Entry/Editing Environment:

Use the N key to end these functions and return to the board

definition menu.

2. Connection Edit/ Entry:

This  system  allows  for  entry  and  editing  of  all  the

connections  that need to be  made between the  components in the

system.    The  functions  are  very  similar  to  the  component

functions.

New Connection Entry:

Typing  an "A"  will allow  entry of  new connections.   The

program will display the connection number being entered and then

prompt  for the  name of the  source designation.   The name must

exist in memory as a component.  ESC can be used at this point to

return to the entry/editing  environment.  If a ground  plane has

been defined, the  reserved designation  "GRNDP" can  be used  to

connect to it.  The associated pin is ignored and defaults to 1.

The  next question is for the source chip pin number.  Enter

PC-Trace Users Manual

12

the number of the pin that the  trace is to connect to.  ESC will

back up to the previous prompt.

The  program will now prompt for the name of the destination

designation.    The name  must exist  in  memory as  a component.

"GRNDP" can be used as a destination as well.  ESC can be used at

this point to return to the previous prompt.

The  next question if  for the destination  chip pin number.

Enter the number of the pin that the other end of the trace is to

connect to.  ESC will back up to the previous prompt.

The program will now ask if this is a priority route or not.

By  answering "yes", this trace  will be routed  before all other

non-priority  routes no matter how long it  may be and what other

routes it may block.  This might be used for making sure a memory

access  bus is all done in copper  instead of using jumpers for a

vital connection.

Next, the program will want to know the desired trace width.

Traces  from 0.01  to  0.07 are  supported.   Select  the  number

corresponding  to the  size required.   The following  table will

show what size trace will  go between two pads of some  size that

are spaced on 0.1 inch centers.

Pad Size (inches)

Trace width    0.05      0.06      0.07

0.01 Yes       Yes       No

0.02 Yes       No        No

0.03 Yes       No        No

0.04 No        No        No

0.05 No        No        No

0.06 No        No        No

0.07 No        No        No

These tolerances  can be overridden using  the interactive router

which will be discussed later.

The next question the  program will ask is if  everything is

correct.  Answering "no" will restart this process by reprompting

for the source designation.  Answering yes will bring up the next

prompt.

The  next  prompt  is  if  the  user  wishes  to  keep  this

connection.   Answering "no" will skip the next prompt and ask if

you  wish to enter another connection.  Answering "yes" brings up

the next prompt.

The  last  prompt asks  if  another connection  needs  to be

entered.   Answering "yes", will put  start back up at the source

designation prompt to enter another connection.  "No" returns the

program back to the entry/editing environment.

Editing Connection Definitions:

Selecting  E from the  entry/editing environment  will gives

the  user  the ability  to change  any  part of  each connection.

Simply select  the letter of the item to change.  For example, to

PC-Trace Users Manual

13

change the source designation, press A after pressing E and enter

the  new designation.  Either enter the new designation, or press

ESC to leave it unchanged.  Press N to exit the edit mode.

Deleting Connection Definitions:

This  function   allows  removal  of  a   connection.    The

connection that will be deleted is the one currently displayed on

the screen.  Pressing D will confirm before actually deleting the

connection.  Answer accordingly.

Searching for a Connection:

Finding  the connection to edit or delete can take some time

on a large board.  This  function allows a search for connections

by  designation.   When prompted,  enter the  designation sought.

PCTRACE will change the case to upper for you.  PCTRACE then will

search from  the current connection to the end.  It looks at both

the source and destination designations for a match.  If it finds

the  search string  the  program will  display that  connection's

information.  If it doesn't exist, nothing changes.

Incrementing and Decrementing the displayed Connection:

Using  +  and -  will change  the  displayed connection.   +

increments  and - decrements.  The display is circular in nature,

therefore typing +  at the last connection moves it  to the first

connection and pressing - at  the first will move it to  the last

connection.

Exiting the Entry/Editing Environment:

Use the N key to end these functions and return to the board

definition menu.

3. Ground Plane:

This function is used to set up a ground plane.  The program

will display a picture of the board.  It will prompt for the

width of  the ground  plane on  all four  sides on  both the

component side and  solder side  of the board.   Each  digit

entered corresponds to .05 actual  inches.  The ground plane

may  be changed at any time.   ESC will also back out of the

first prompt only (top-solder side).

***  Please note that  this function erases any work done in

the interactive router (discussed later).

4. Define Board Size:

This  function is the first  function used.   It defines the

dimensions  of the board.  The  maximum board size supported

by  PCTRACE is  8" X  10".  When  prompted for  numbers, the

PC-Trace Users Manual

14

numbers  are  entered  as  multiples  of  5.    Each  number

represents 0.05" of  actual size.  So an entry  of 100 would

correspond  to 5".   Board sizes can be  changed at any time

during  the design process by calling the routine again.  If

the board is made  smaller, the chips that were  outside the

new border  will have to be  replaced.  ESC will  back up to

the  previous prompt, or jump out of the routine leaving the

board size unchanged.

5. Spider Web:

This function  is useful for checking to see if a chip would

be more logically placed somewhere else.  By using the + and

-  keys,   various  chips  and  their   connections  can  be

displayed.   As each  chip is selected,  it, as  well as all

traces connected to it are highlighted in red.  Once  a chip

is highlighted, selecting M will allow movement of the chip.

After pressing M, the arrow keys become active and the chip,

as  well as all its connections will be rubber banded around

the  board.   Pressing  RETURN will  place  the chip  at the

current location, or press ESC to not place the chip and put

it back  at its original  location.  The  ESC or E  key will

exit from this function.

*** Please note, if  chips are moved, the  work done by  the

interactive router is erased.

Interactive Router

This   board  editing   system   allows   the  designer   to

interactively design a board.  It can be very useful  for finding

possible routes that  the autorouter  was unable to  find.   Upon

entry  to this  section, the  program generates  the  borders and

places  the  pads  of all  the  chips.    If  a large  number  of

components has  been placed  on the  board, it  could take a  few

moments.  It will also  ask the user to place all the  chips that

have not  been placed  yet.   If you intend  to route  the entire

board this way,  place your chips  and then leave  them.   Moving

them  around  after  placing the  chip  will  distort the  board.

PCTRACE  has  no way  of  connecting  traces up  to  a chips  new

location.   The  screen  will  show  all  the  chips  with  their

designations centered  amid each  component pin unless  DEFGEN is

used to  lock the location  of the designation.   This system has

many  keys that  are  active  while  in  this  environment.    An

explanation of each follows.

S - switch sides:

This function changes the side of the board displayed.  When

S is pressed, the  other side of the board  becomes visible.

The program supports only 2 sides, solder and component.

PC-Trace Users Manual

15

K - Silkscreen toggle:

Pressing this key toggles the silkscreen  display on and off

while in the  interactive router.   It defaults  to ON  upon

entry to the interactive router.

C - move a component:

Pressing this key will give the  user the option of moving a

component within the interactive  router.  After pressing C,

the bottom  of the screen  will show  a brief  listing of  a

component.  The  name and  designation are listed.   +  will

move to the next component, and - to the previous  one.  ESC

will exit the component movement function.  Pressing  M will

erase the  displayed component  and redraw the  screen.   At

this point, move the cursor to the upper  left corner of the

area where the component will be placed and press G to place

it.    It  is  important  to  understand   that  the  cursor

represents  the  absolute  upper  left corner  of  the  part

including any silkscreen extensions.

G - grab location:

The key  is only active at  certain times.  The  F1, F2, F4,

F5, F6, F7, and F10  function all use this key.  The  key is

normally used to mark a location on the board or  a piece of

a trace.

B - Turn on background:

This  key is used to  turn on and  off the background layer.

If  editing  the  solder  side,  a  lightened  view  of  the

component side  can be turned  on.   On a color  monitor, it

will appear dotted and  red.  Black and white  monitors will

show dotted graphics only.  Pressing B again turns this mode

off.

E or ESC - end:

Pressing E or ESC will exit from the interactive router.

Cursor Keys:

The  cursor keys move the  solid block around  on the board.

Each time an  arrow key is pressed,  the cursor will  move 1

space.   Relative coordinates are displayed at the bottom of

the  screen.  Using Shift-Arrow  moves the cursor 10 spaces.

If  the edge of the screen  is encountered, but not the edge

of the board, the screen will shift the board over.

Function Keys:

The function  keys numbered  F1-F10 do  the majority  of the

work in PCTRACE.   They allow placement of vias,  running traces,

deleting traces, resizing traces, and deleting vias.

PC-Trace Users Manual

16

F1 - Set Source:

By  pressing  F1,  the bottom  of  the  screen  will say  to

position  the cursor on the  pad that will  be designated as

the source and then press  G to grab it.  The  bottom status

line will display exactly what chip and pin number have been

selected.

F2 - Set Destination:

This function works exactly  as the F1 function but  selects

the destination pin.

F3 - Run Trace:

Once a source and destination have been defined, pressing F3

will  attempt  to connect  the two  pins  with a  trace 0.01

inches  wide.   If  it is  successful,  the screen  will  be

redrawn  and the trace will  appear.  If  it cannot complete

the route, a  message will be displayed at the bottom of the

screen.   At this point try  placing a via and  routing from

the source  to  it on  one  side and  from  the via  to  the

destination on the other.

F4 - Delete Trace:

This  function  will delete  any part  of  any trace  on the

board. Pressing F4 enters the  delete selection mode.  After

pressing F4 the screen will  say to use G to grab  the parts

to  be deleted.  As the user  presses G on valid traces, the

pieces will  be highlighted.  Changing screens by moving the

cursor off  the screen  will not  lose the  already selected

traces.  Once all  the pieces to delete have  been selected,

press F4  again  and  the  program will  prompt  to  confirm

deleting of the  selected traces.   Answer Y  to finish  the

delete operation.

F5 - Add Via:

This function  key will  add a  via at  the location  of the

cursor when G  is pressed.  It  will be V and  then the next

available number.   Use these for multilevel  routes or just

extra  pads needed.  A  via cannot be  placed adjacent to an

existing pad.  One can be placed diagonally from  a pad .  A

via  also may not be  placed in the middle of  a trace.  All

vias are deleted by the autorouter when it is invoked.

F6 - Del Via:

This  function deletes the via at the cursor location when G

is pressed if a via exists there.

PC-Trace Users Manual

17

F7 - Resize trace:

This function will resize  a trace to any valid  trace size.

Select F7  and use G to  select the traces to  resize.  Once

the traces to  resize have  been selected press  F7 and  the

program will ask for  confirmation of the resize.   Answer Y

to resize.   The program  will prompt for  the trace  resize

value.  1 corresponds to 0.01  up to 7  which corresponds to

0.07 inches.  Be careful not to overlap other traces or pads

with this function.

F8 - Check Route:

This  function will check to see if  a selected pair of pads

can be connected.  It will report back whether the route can

be completed, the route  exists already, or if no  route can

be found.  This is useful for doing multi-layer routes using

vias.   Checking for  an available  route before  laying the

actual trace will save a number of deletes if later the path

is blocked.

F9 - Manual Route:

This  function allows  the  user to  manually route  traces.

Place the  cursor on the starting pad or trace and press F9.

A prompt will appear  at the bottom of the screen asking for

the trace width.   Enter the number that corresponds  to the

desired size and press  return.  Now use the  cursor control

keys to  route the trace.  Every  time the cursor moves, the

trace  is lengthened.   When  done, either  stop in  an open

space and press F9  or stop on a trace or  pad and press F9,

the  correct  connecting piece  will  be added  to  make the

connection.  Please note that moving over an  existing trace

will cause  PCTRACE to cut  that trace in  favor of  the new

path.   This  works well  for putting  short jumpers  on the

board.   This is very  useful for routing  traces that would

normally  block other routes  if they  were not  placed this

way.   With  the interactive  router any  size trace  can be

placed anywhere.  It is possible to over-ride the autorouter

and  run a 0.01  inch trace between  two 0.07 pads.   If the

board-making process used is accurate to 0.01 many low power

traces can be completed this way.  The manual router is also

used  in  many  advanced  "tricks"  for  making  specialized

boards.

F10 - Write Text:

Pressing  F10  allows additional  text  to be  added  to the

silkscreen layer.   After pressing F10, the program will ask

the user to move the cursor to the starting position for the

text  and  press G.   After  pressing  G, the  software will

prompt  for  the text  string.   It  will  accept  up to  20

characters.   When done  entering text, press  return.   The

PC-Trace Users Manual

18

next prompt is for  double sized text.  Answer "yes" and the

text  will  appear double  sized.   The  board will  then be

redrawn with the text added in.   The only thing to be aware

of is that normal silkscreen text takes precedence over user

added text.

Using a mouse in the interactive router:

The mouse allows  a faster  method to do  all the  functions

mentioned  previously except  for the  manual router.   The mouse

must  be installed  before  running the  PCTRACE  program.   When

entering the interactive program  four arrows will appear  in the

lower right corner of  the screen.  By  using the mouse,  putting

the cursor on one of these  arrows, and pressing the left button,

the screen will  be moved by  half in that  direction.  The  same

thing was accomplished by  moving the cursor off the  board using

the arrow keys.

Pressing  the  right button  will pop  up  a menu  along the

bottom of the screen.  The  menu corresponds to function keys F1-

F8, and F10.  The exit command at the far right, exits the  menu.

By placing the cursor anywhere on  the command desired, execution

of that  command is  done by  pressing the left  button.   If the

command selected is F1, F2,  F4, F5, F6, F7, or F10,  the program

will be using the "Grab" method.  The left button  works the same

as the G  key in these cases.   Use it to highlight  sections and

locations on the board to  work with.  When using the  delete and

resizing  functions (F4 and F7)  the user must  press and release

the left button for each piece of trace.

Autorouting

Selecting this from the menu will enable the user to connect

all  or most of the connections  desired.  The program will first

draw borders, place  pads, and set  up the ground  planes.  If  a

component has not been placed,  the program will ask the  user to

place it.   The program requires all chips to be placed before it

can  route.    There are  four  routing  strategies  that can  be

employed.  Select each strategy by  pressing Y or N as the cursor

moves to each option.  If Y is selected for single or  double via

routing a prompt will also appear for the maximum number of tries

for each.   Just enter a  number.  Bigger numbers  are better for

bigger boards, smaller numbers for small boards.

The  PCTRACE  system  is  totally  re-entrant.    The  first

question  asked  is  if the  user  wishes  to  clear all  traces.

Answering  "yes" clears anything  currently on  the board.   "No"

would keep the screen as it was.   ESC can  be pressed during any

routing  strategy to exit that  routine and the  autorouter.  The

program will then show  the list of all unconnected  routes still

existing.   If the router  has not completed  one run-through the

connection list,  traces may have  been completed by  the earlier

route and still be  reported as unconnected.  A  design check can

PC-Trace Users Manual

19

be done  to see which routes in the connection list have not been

connected,  either by hand or  by the autorouter,  by running the

autorouter and not erasing  the current traces.  This  is because

the autorouter  will not  reroute a connection  that has  already

been completed.   Just select  the double or  single sided  route

system.   It  will report  any routes  that are incomplete.   Any

route  that was completed before  will be marked  as complete and

not reported.   The program will  report whatever routes  weren't

done and the ones it couldn't complete.

Single Sided Route:

This  function will attempt to  place all the  routes on the

solder  side  of  the board.    It  is  the least  efficient

strategy  but  is great  for  small projects  where  ease of

making the board itself is a priority.  This function cannot

be used in conjunction with the next three strategies.

Double Sided Route:

This function will do its routing on two sides of the board.

It has a higher completion percentage  than the single sided

strategy.

Single Via Route:

This  function requires  that the  double sided  strategy be

selected first.    This function  is used  after the  double

sided strategy  to try  to complete any  uncompleted routes.

It places  a via  and tries  to do the  single route  with a

trace on  each side of  the board.  A  number such as  50 is

good for the maximum number of tries prompt.

Double Via Route:

This  function also  requires that  double sided  routing be

selected to work.  It  uses two vias to attempt  to complete

the route.   A good  maximum number  of tries is  30 or  so.

This function can take a great deal of time so be patient.

Board Statistics

This function displays 5 statistics.

Equivalent Integrated Circuit Count:

This function is the total number of pins divided by 16.

Board Density: This is  the amount of space total  that each chip

has  to occupy.   It  is calculated by  taking the

area of  the board and dividing  by the Equivalent

IC count.

PC-Trace Users Manual

20

Total Trace Length: The function  gives the  total length  of all

the  traces on  the board.   The  smaller the

number, the better the design.

Board size:    The board dimensions in inches.

File name:     This is the file name currently in memory.

PC-Trace Users Manual

21

Printer Menu

1. Print Components:     This  function will  generate a  list of

all the components.  It can be sorted by

name or designation.

2. Print Connections:    This  function will  generate a  list of

all the  connections in memory.   It can

be sorted  by the source  or destination

designation.

Export Menu

The  export menu allows for files to be created that various

other  cad software and board producers use.  After selecting the

Export menu from the main menu, four export choices will be seen.

1. Write Autocad Script file:

This function will  write a  file that can  in turn  be

read into and edited with Autocad.  The process is one  way,

no  way is provided to read autocad files back into PCTRACE.

Call this function from the Export menu and give a name when

prompted.  Specify no path or file extension.  The extension

.SCR will  be added to the  file.  Once the  program is done

writing the file, Autocad  may be started.  Use  the command

"SCRIPT" from the "Command:"  prompt.  Autocad will ask  for

the script file name.  Specify  the same name (with path  if

needed)  as the one created in PCTRACE and Autocad will read

in the  file.  Autocad  will display  three layers:  solder,

component,  and silkscreen.  Each may be edited and saved as

Autocad  drawing files.   Doing  this greatly  increases the

number  of printers that can  be used for  output as Autocad

supports a large number of printers.

2. Write a DXF file:

The  DXF  file   is  used  by  many   cad  packages  to

incorporate data into their  systems.  Autocad is  one prime

example.   The  DXF file  includes all  three layers  in its

output (Solder,  Component,  and Silkscreen).   The  command

will prompt for  a file name.   Enter  the filename with  no

extension.  The  extension .DXF  will be added.   After  the

filename has  been  entered, the  DXF  file output  will  be

created.   Autocad will  load  this file  using the  command

"DXFIN"  from  the Autocad  "Command:" prompt.   Wordperfect

will also read a DXF file and convert it to its own graphics

format (WPG files).

PC-Trace Users Manual

22

3. Write a Gerber file:

A Gerber file is the standard file type that is used by

professional  board manufacturers.    The PC-Trace  software

will  generate this file format and is accurate to 3 decimal

places.  Upon selection of this option, the user is prompted

to enter aperture names to be used.  The aperture is used to

describe an entity on the board.   If the photo plotter that

the board maker is using requires that  a pad 0.05 inches in

diameter  be specified  as  D23,  then  the  0.05  inch  pad

aperture name must be changed to reflect this.  The software

will ask if the aperture  list is OK.  If it is, press Y for

yes and the program will continue.   If N for no is pressed,

each aperture must be re-entered as the cursor goes down the

column.    ESC  will  bypass a  selection  without  erasing.

Retyping will change the selection and return will erase the

selection  which  would make  it  necessary  to re-edit  the

aperture list.  Once  the list is determined to  be correct,

the program prompts for the board  side to be plotted to the

file.   Press  the number  that corresponds  to the  desired

selection.   Last of all the  program will ask for an output

filename.   Enter  the  filename with  no  extension.   Upon

pressing  the  return key,  the  program  will generate  the

Gerber file.   Two files  will be created.   The first  file

will  be the filename plus the extension .GER for the actual

Gerber  file.   The  second file  is  the filename  plus the

extension  .AP for the aperture  list.  Both  of these files

should  be sent  to  the  manufacturer.    A  file  must  be

generated  for  each  board  side  that  is  desired  to  be

manufactured.  Only one aperture file need be sent.

4. Write a CNC Drill file:

A  CNC  drill  file  contains  the  information  for  a

Computer Numerically  Controlled  drill press  to drill  the

holes  for the pins of  the components to  be placed though.

The  software will  prompt for the  drill bit  (tool number)

size that should be used for each different  sized pad.  The

entry system works the  same as that of the  Gerber aperture

definition screen.  The  user will next be prompted  for the

output  file  name.    It  should  be  entered   without  an

extension.  After pressing return at the end of the filename

the  computer will  create the  filename with  the extension

.NC.  The file contains the X,Y coordinates of all the pads.

Exiting the Program

Selecting 8 from the  main menu will exit from  PCTRACE back

to DOS.   If there is data in memory the  program will ask if you

wish to  save it.  Answering  "Yes" will prompt for  a file name.

PC-Trace Users Manual

23

Enter  the file name as was done in the save data function in the

Input/Output menu.   ESC  will abort  the save,  but it  will not

return the user to the program.  Answering "No" to the save query

will cause the program to go immediately to DOS.

PC-Trace Users Manual

24

Brief Tutorial

The  following is  intended as  a method  of getting  up and

running with the program.  It will give a general idea of how the

program works.

1.   Begin by  running PCTRACE.   This  information can  be found

back in the section on setting up PCTRACE for your machine.

2.   Once  at the main menu,  press 3 for  define/edit board, and

then select 4 for define board dimensions.  For our purposes

we will use a board size of 80 x 80, which translates into a

4 inch square board.

3.   Select function 1  to enter  new components.   You will  see

that no data has been  entered yet.  Press A to  enter data.

We  will put  three chips  on the  board and  two resistors.

Answer  each prompt  below with  the following  data.   When

asked  to place each chip  answer Y and  place the chip with

the  cursor keys.   After  you have  entered the  last item,

answer N to the "Enter another component?" prompt.

Name: 8051 uP                 Designation: IC1

Definition file: DIP40V6      Placement: Left side

Pad Size: 0.05

Name: 74ls373                 Designation: IC2

Definition file: DIP20V3      Placement: Upper right

Pad Size: 0.05

Name: 2716                    Designation:  IC3

Definition file: DIP24V6      Placement: Lower right

Pad Size: 0.05

Name: 100k resistor           Designation:  R1

Definition file: 2PAD50V      Placement: Upper middle

Pad Size: 0.06

Name: 10k resistor            Designation:  R2

Definition file: 2PAD50V      Placement: Lower middle

Pad Size: 0.07

N exits to the board definition menu.

4.   Next,  enter the list of connection between chips.  Select 2

from  the menu to enter the connection entry section.  Enter

the following connections.  All traces should be 0.02 inches

wide.    Answer N  to each  prompt  of "Is  this  a priority

connection?".   When  the last  connection has  been entered

answer N to the "Enter another connection prompt?".

Source Device  Pin  Destination Device  Pin

PC-Trace Users Manual

25

IC1  2         IC3       2

IC1  3         IC2       16

IC3  22        IC2       6

R1   1         IC2       4

R2   1         IC1       8

R2   2         IC3       6

R1   2         IC1       31

IC2  8         IC3       17

IC1  21        IC1       15

IC2  8         IC3       8

IC2  9         IC3       9

IC2  10        IC3       10

IC1  14        IC2       9

N returns to the menu.

5.   Ok, let's autoroute the board.  Go back to the main menu and

select  4  to autoroute  the board.    When prompted  to use

single sided routing or not, answer  Y.  Now hit the ESC key

and  watch  the machine  route.   When  it is  done  is will

display the  list of uncompleted connections  to the printer

or  screen.   Select whichever  you  wish.   We will  now be

dropped back to the main  menu.  We can take a look  at what

we have done by  selecting number 5 from  the menu.   Cursor

around  and see a representation of the board on the screen.

Hit ESC when done.

Now let's re-route the  board using double sided techniques.

This time  say N to single  sided and Y to  double sided and

then hit ESC.  The machine will route double sided and again

give  a list of incomplete connections if any exist.  Select

printer or screen for this  list.  Let's take a look  at the

board again.  This time we may also want to use the S key to

change sides of the board that we are viewing.  When you are

done hit ESC.

6.   From the main  menu, hit  6 for statistics.   Displayed  are

five statistics that tell you how good your  design is.  Hit

space when you are done with this.

7.   Last of all we'll print out our design.  Follow instructions

in the section on the RPRINT Subprogram to do this.

PC-Trace Users Manual

26

RPRINT.EXE Subprogram

Execute  this program by typing RPRINT  from the dos prompt.

Upon  entry into  this program the  user will  be prompted  for a

printer type.  Enter the corresponding number.  The second prompt

is for  the board name.   Type the  name with no  extension.  The

main control  panel will  now explode  onto the  screen.   A math

coprocessor is particularly useful in this program.

1. File name:  Selecting 1  will allow changing of the file to be

printed.  Enter the filename with no extension.

2. Board side: This selects  the board side to  print.  Selecting

this  number will  toggle  between three  possible

sides   to   print    (Solder,   Component,    and

Silkscreen).  Single sided  boards are done on the

solder side.

3. Mirroring:  This  selection toggles between  mirroring and not

mirroring the board.

4. Negative:   This  function  will  allow  the  user  to  select

between printing  a negative or a  positive of the

actual board.

5. Actual size:     Laser printers and 24 pin dot matrix printers

can output the images at actual size.  If the

selected  printer  lacks  the  resolution  to

achieve acceptable results, this  menu choice

will  not exist.   Selection  of  this choice

allows  toggling  between  actual   size  and

double sized printing.

6. Pin1 square:     Toggling this  to the on position  will print

the  number one  pin of  all components  as a

square.  This  is useful for identifying  the

number one pin when placing the components on

the finished circuit board.

7. Output file:     After selecting this menu choice, the user is

prompted for an output  file name.  Enter the

filename without an  extension.  The printout

of the file will now be sent to the file that

was designated.   The file will  be given the

extension  .PRT.    Be  careful  if  printing

multiple  files not  to overwrite  a previous

printing.

8. Print Board:     This  selection starts  the  printing of  the

board.

9. Exit program:    This  selection exits the RPRINT program back

to DOS.

Plotter Notes:

In using RPRINT with plotters, no provision is made to print

to a serial port.   With DOS 3.3 and later, the MODE  command can

be used  to redirect  printer output  to  the serial  port.   Two

PC-Trace Users Manual

27

commands are needed.

MODE ComX:9600,N,8,1,p (press return) X is the com number

MODE LPT1:=ComX: (press return)

MODE LPT1: (press return) will return to normal operation

PC-Trace Users Manual

28

DEFGEN

Subprogram

The  program needs  to  know the  exact  pin layout  of  any

component  placed on the board.  This function allows creation of

these definitions.   The largest difference  between this version

and previous versions  of PCTRACE is  that surface mount  support

can be acheived more easily.   When a device is defined, all pins

of the surface mount  part must be terminated at  a through-holed

pad.   That is  the same as  before.  With  version 5.0, pads and

traces can  be defined within  the definition  file allowing  the

user to draw small traces with flat pads to serve as locations to

mount  a surface mount  part.  A  silkscreen outline  can also be

drawn with the manual router in "SILKSCREEN" mode.

The program  is  started by  typing  "DEFGEN" from  the  dos

prompt in the PCTRACE subdirectory.

***Note:  Most chip  dimensions are available in  data books such

as RCA cross reference manuals.  Radio Shack  also puts

device dimensions on the back of their packages.

When running DEFGEN to create  new chip layout patterns many

keys are active.  A summary of each key follows:

S - switch sides:

This function changes the side of the board displayed.  When

S is entered from the keyboard,  the other side of the board

will  be displayed.    The program  supports  only 2  sides,

solder and component.

K - Silkscreen toggle:

Pressing  this key  toggles  the drawing  on the  silkscreen

layer as opposed to the solder  or component layers.  When K

is pressed a green "SILKSCREEN" appears at the bottom of the

screen.   The only commands that are available at this point

are  the F9, manual router,  which is used  to draw the part

outline, F10  which allows placing  text in addition  to the

component designation, F1  which is Load, F2 which  is Save,

and F3 which is erase.

G - grab location:

The key is only active at certain times.  The F4, F5, F7 and

F10 functions all use this key.

B - Turn on background:

This key is  used to turn  on and off the  background layer.

If the user is editing the solder side, this key will show a

PC-Trace Users Manual

29

lightened view of the  component side.  On a  color monitor,

it will appear  dotted and  red.  Black  and white  monitors

will show dotted graphics only.  Pressing B again turns this

mode off.

E or ESC - end:

Pressing E  or ESC will exit  from the DEFGEN program.   The

exit is immediate and should be used carefully.

Cursor Keys:

The  cursor keys move the  solid block around  on the board.

Each time the  arrow key is pressed, the cursor  will move 1

space.  Relative  coordinates are displayed at the bottom of

the screen.   Using Shift-Arrow moves the  cursor 10 spaces.

If the cursor comes to  the edge of the screen, but  not the

edge of the board, the screen will shift the board over.

Function Keys:

The function keys  numbered F1-F10  do the  majority of  the

work in  DEFGEN.   They allow the  user to  place vias,  run

traces,  delete traces,  resize traces,  add pads,  and draw

component outlines.

F1 - Load Definition:

By pressing F1, the bottom of the screen will prompt  for an

input definition  file name.   Enter  the  filename with  no

extension.  If the file is found it will be displayed on the

screen  for editing.  Please  feel free to  use the provided

definition files in creating others.

F2 - Save Definition:

This  function  will prompt  for  a  save filename  for  the

definition.  Enter the save filename with no extension.  The

program  will notify the user if the file already exists and

check to make sure that  it is alright to rewrite the  file.

Once the file has been written with this command, DEFGEN can

be exited and the definition used in PCTRACE.

F3 - Erase:

This  command has  four options.   Option  1 will  erase the

component layer of  everything except the pads.   The second

option deletes everything but the pads off the solder layer.

The third  layer will  remove everything off  the silkscreen

layer  except for  the  placement of  the designation  text.

Option 4 deletes  everything, including the pads.   Option 5

exits without deleting anything.

PC-Trace Users Manual

30

F4 - Delete Trace:

This  function will delete any part of any trace that exists

in the  definition. Pressing F4 enters  the delete selection

mode.   After pressing  F4 the screen  will say to  use G to

grab  the parts  to be deleted.   As  G is  pressed on valid

traces, the pieces will be highlighted.  The user can change

screens by moving the cursor  off the screen without  losing

the  already selected traces.  Once all the traces to delete

have been  selected, press  F4 again  and  the program  will

prompt  to make sure you wish to delete the selected traces.

Answer Y to finish the delete operation.

F5 - Add New Pad:

This  function key will  add a  pad at  the location  of the

cursor  when G  is pressed.   All  components must  have pad

definitions.   The router is unable to route to a trace that

cannot be specified by an attached pad.  The pads are placed

in  sequence so  be  sure  to  place  them  in  the  package

numbering order.

F6 - Delete Last Pad:

This function deletes  the last placed pad from  the screen.

It requires that the definition be regenerated so the screen

will be redrawn.

F7 - Resize trace:

This function will resize  a trace to any valid  trace size.

Select F7  and then  use G to  select the traces  to resize.

Once  you have selected the  traces to resize,  press F7 and

you will be asked  if you really wish to resize  the traces,

answer Y  if you do.   Next the program will  prompt for the

trace  size. 1 corresponds to 0.01 up to 7 which corresponds

to 0.07 inches.  Be  careful not to overlap other  traces or

pads with this function.

F8 - Coordinates for Designation:

This  function   will  anchor  the  location   of  the  part

designation.  Normally the designation moves to the centroid

of the area  defined by  the pads.   For most  applications,

like  DIPs,  this  is  acceptable.    Some  parts,  such  as

transistors, benefit from having the designation text placed

above  or below the pads.  Selecting this option will prompt

for a location for the center of the designation text.  Move

to  that location and press G.   The next prompt asks if the

user  desires double sized text.   If "yes"  is entered, the

designation will be drawn 0.1  inches high on the silkscreen

layer.  "No" will draw text 0.05 inches high.

PC-Trace Users Manual

31

F9 - Manual Route:

This  function allows  the  user to  manually route  traces.

Place  the cursor on the  starting pad, trace,  or open area

and press  F9.  Now use the cursor control keys to route the

trace.     Every  time  the   cursor  moves,  the  trace  is

lengthened.  When done, press F9.   If the end is on another

trace, pressing F9 will add the proper connecting point.  If

the green  SILKSCREEN indicator  is  on, the  trace will  be

routed in green.  This should be used to draw the outline of

the part.  This outline will be displayed in the interactive

router and when printing the silkscreen.

F10 - Write Text:

Pressing  F10 allows the addition  of text to the silkscreen

layer.  After  pressing F10, you  will be asked to  move the

cursor  to the starting position  for the text  and press G.

After pressing  G, the  software  will prompt  for the  text

string.   It will  accept up  to 20  characters.   When done

entering  text, press return.  The next prompt is for double

sized  text.  Answer "yes"  and the text  will appear double

sized.  The  board will then be redrawn  with the text added

in.

Using a mouse in the DEFGEN program:

The  mouse  allows  usage  of all  the  functions  mentioned

previously except for the  manual router.  The mouse  driver must

be installed  before running the  DEFGEN program.   When entering

the interactive  program, four  arrows will  appear in  the lower

right corner  of the  screen.   By using  the mouse,  putting the

cursor on one of  these arrows, and pressing the left  button the

screen will be moved by  half in that direction.  The  same thing

was accomplished by  moving the  cursor off the  board using  the

arrow keys.

Pressing the right mouse button will pop up a menu along the

bottom of the screen.  The  menu corresponds to function keys F1-

F8, and F10.  The exit command  at the far right, exits the menu.

By  placing the  cursor  anywhere on  the  command desired,  that

command  can be  executed by  pressing the left  button.   If the

command selected  is F4,  F5, F7, F8,  or F10, the  "Grab" method

will be in use.  The left button  works the same as the G key  in

these cases.  Use  it to highlight sections and locations  on the

board to work with.  When using the delete and resizing functions

(F4 and F7) the user  must press and release the left  button for

each piece of trace.

PC-Trace Users Manual

32

Keyboard Summaries

for PCTRACE

Chip Placement Screen:

ESC       - Option to Abandon Placement

RET       - Place chip at current location

Cursor Keys    - Moves chip one space for each depression

Shift Cursor Key - Moves chip 10 spaces for each depression

Spider Web System:

M*        - Set system to move highlighted chip

Cursor keys    - Move chip one space on board

RET       - Place chip at current location

ESC       - Abandon placement

+,-*      - Increment and Decrement through chips

E*        - Exit Web System

* -  Denotes that  this selection is  displayed on screen  at all

times.

Interactive Router:

S         - Display other side of board

G         - Grab a piece of trace at cursor

E,ESC          - Exit interactive router

B         - Turns on and off background

C         - Allows movement of devices

K         - Turns silkscreen layer on and off

F1        - Select source pad

F2        - Select destination pad

F3        - Attempt to connect source and destination

F4        - Delete traces

F5        - Place via at cursor location

F6        - Delete via at cursor location

F7        - Resize selected trace

F8        - See if route is possible between source

and destination pads

F9        - Enters and exits the manual router

F10       - Puts user text on the silkscreen layer

PC-Trace Users Manual

33

SCAD

Schematic Capture

Version 1.0

Users Manual

June 21, 1992

by

Douglas E. Ehlers

Copyright (C) 1991,1992

Introduction

Schematic   CAD  is   a  program   for  drawing   electrical

schematics.  While this drawing is being made, the software keeps

track of all components and connections between them.  It is then

possible to create an  export file (.NLT) that PC-Trace  can load

and create  a board from.   This allows  the designer a  complete

cycle  for production of a circuit board.  From conceptual design

to computer controlled manufacture of the board,  this system has

it all.

Menu Usage:

The menu  system in  SCAD requires  the use of  a mouse  for

complete use of the  system.  Some functions can  be accomplished

with  the use  of the  keyboard only.   Using  a mouse,  move the

cursor  to the area  of the desired menu  selection.  By pressing

the left  mouse button, a pull  down menu will appear.   Move the

mouse down the menu  until the desired choice is  highlighted and

release the button.  The menu  choice will be executed.   Another

way to make menu choices is to open  the menu by pressing the ALT

key and the  highlighted menu area key.   For example ALT-F  will

open the Files  Menu.  The cursor  keys can be  used to move  the

highlight bar up and down the menu.  Pressing return will  select

the  menu  choice.   Once the  menu  appears, pressing  the first

letter of the desired  choice will move the highlight bar to that

choice.

Some menus have  small arrows  that point to  the right.  By

moving the  mouse across that arrow a  submenu will appear.  That

menu  responds the  same way  as  the rest  of the  menus.   Many

functions  use this  concept including  Print, Draw,  Delete, and

Move.

File Menu:

New Diagram:

The New Diagram function from the FILES menu will clear

the drawing space that SCAD uses to store files.  A dialogue

box is used to confirm this selection.

Load Diagram:

The Load Diagram  file will load  a schematic that  was

saved  previously.   The  program will  prompt for  the file

name.  Enter  the name  without the extension  (.SCH).   The

program  will add the extension.  If the program cannot find

the file it will report it and give the  user another chance

to enter the filename or to exit back to the drawing area.

Save Diagram:

The  Save Diagram  function  saves whatever  is in  the

memory of the system at a given point.  When  prompted for a

save name, enter the name with no extension.  SCAD  will add

the extension (.SCH).  If the program can't find the entered

file name, it will save the information from the given mode.

If  the program finds that the save filename exists, it will

prompt to see  if the old file  should be overwritten, or  a

new file name should be specified.

PC-Trace Users Manual

35

Export DXF:

The Export  DXF function  is used  for creating  a file

format that can  be read  by many popular  CAD programs  and

word-processing packages.  The DXF format used is compatible

back to Autocad Version 2.6.  The program will  prompt for a

save  name.   Give the  file  name with  no extension.   The

program will add it  (.DXF) automatically.  If  the filename

already  exists on the disk,  the program will  prompt as to

whether  or not to overwrite  the file.   Answering yes will

overwrite the old filename, and answering no will cause SCAD

to prompt for a new file name.

Export PC-Trace:

The function is used  to create a netlist that  PCTRACE

can  load.   It  contains  a  list  of  all  components  and

connections  that have  been  entered while  using the  SCAD

software.  All that will be required on the PCTRACE side  is

to place  the  components and  start  the autorouter.    The

program prompts for the export filename.  Enter the filename

with no  extension, the  program will add  it automatically.

If the program finds that  the entered filename exists,  the

program will allow the option of overwriting the file or re-

entering a new filename.

Print Total/Window:

Print has  two options,  one, the option  called Total,

that will  find the extents  of the drawing and  scale it to

fit  on a page.   The second, or  Window, essentially prints

what  is currently  being viewed  on the  screen.   Once the

program is told to  print, it will create a print file, save

the  current drawing  and chain  to the  sprint.exe program.

The  first prompt in this  program is for  the printer type.

The  user should  select  their printer  or  one that  their

printer emulates.  The  sprint menu can print to  a file and

that is toggled  with option 1.   When prompted for  a print

filename,  enter the filename with no extension.  The second

option will  allow printing  in portrait or  landscape mode.

Option 3  actually prints  the drawing,  and option  4 exits

sprint.exe and returns to SCAD.

Show Drawing Info:

This  function  will  show  basic  drawing  information

including the  current SNAP value, the  current window size,

the  amount  of  free   memory,  and  the  currently  loaded

filename.

Exit:

The exit  function exits whichever mode  the program is

currently running  in.  This function can  also be activated

by  pressing ALT-X.  If  the user is  currently operating in

PC-Trace Users Manual

36

the Component  Drawing area, it  will exit the  drawing pad.

If the user is editing the  main drawing, it will allow exit

of the program.  No option is given to the user  to save the

file once exiting the program is answered with an OKAY.

Drawing Symbols:

When Draw Component is selected from the Components menu,  a

box appears  in the center  of the screen.   This is  the drawing

area  for creation of  components.  A  component in this  case is

defined to  be the  schematic symbol  for any part  that will  be

placed in  the drawing.   A  small library  of common symbols  is

included with the system for the user.  Invariably, a symbol will

be  needed that  is not included.   That  is the  purpose of this

editor.

Once  the box appears in the middle of the screen, the Files

menu  and the  Entities  menu  come  into  play.    Only  certain

functions may  be used in each.   The unusable functions  will be

grayed out and the  user will be unable to  use them.  The  Load,

Save, New, and Exit functions in the Files menu work as described

previously, except  that they only  apply to  the current  symbol

being drawn.   For example, if an AND gate is drawn in this area,

saving  it will only save the representation  of the AND gate and

not the overall drawing.

The  save routine itself is different at this point as well.

It includes a  short routine that  asks the user  to outline  the

symbol with a bounding  box before the save takes  place.  Select

the  upper left  and lower  right of  the drawing  to save.   The

entire drawing will  be shifted to call the upper  left corner of

the bounding box the origin for later use.

The  following  overview  tells  how  the  various  Entities

commands work and what they do.  Their usage here is  essentially

the same as their usage in the main drawing area.

Draw-Line:

This function  is used to draw lines within the drawing

window.   At the bottom of the  screen, SCAD will prompt the

user  as  to  what   the  program  expects.     The  current

coordinates  are shown  at  the bottom  right.   Select  the

location of the  starting point  of the line  and click  the

left  or right button.  Clicking the left button will anchor

the  start  of  the  line at  the  current  mouse  location.

Clicking the right button will anchor the start point at the

current  point, plus it will  draw a connection  dot at this

point.   The dot  will not  appear until  the start and  end

points  of the line have been specified.  After clicking the

left  or right button, the  program will prompt  for the end

point.  Click either the right or left button.   The buttons

have the same effect  as when picking the start  point.  ESC

will exit or back up while  drawing lines.  To draw  another

line, select this function again.

PC-Trace Users Manual

37

Draw-Arc:

This function is used to draw  an arc of some size.  It

can be used to draw 1/4 arcs from any start point to any end

point and  at any  diameter.   The first  prompt is  for the

center  point of the  arc.  Move the  mouse to this location

and click  a button.  The second prompt is for the radius of

the  arc.  Move the  pointer to the  location that shows the

desired radius.   A floating line reminds the user as to the

current length.  The next point is for the start of the arc.

Pick  this point as before.  The  last prompt is for the arc

ending point.   Click on this point.  The  arc will be drawn

from the start point, counter-clockwise, to the end point at

the  given radius.   ESC  will back  up at  any time  to the

previous  prompt or will exit the function.  To draw another

arc, pick this function again.

Draw-Circle:

This function draws a circle  at some center point  and

some radius.  When prompted, pick the center point and click

a mouse button.  The second  prompt is for the radius of the

circle.  Click on this point and the circle will be drawn on

the screen.  ESC will back up to the previous prompt or exit

the function.   To draw  another circle, pick  this function

again.

Draw-Text:

This  function is used place  text on the  screen.  The

first prompt is for the starting location of the text.  This

point is located at the top, left corner of the text string.

Select  this  point  by  moving the  mouse  to  the  desired

location and click  a button.  The second prompt  is for the

scale of the text.  This number is the size of  the text.  A

value of  0.25 will  create a string  with characters  every

0.25  units.   The  third  prompt is  for  horizontal versus

vertical text.  Answering yes  will draw the text horizontal

and no will  draw the  string vertically.   The letters  are

always upright, they are not rotated for vertical text.  The

last prompt is for the text string itself.  This string will

be displayed on  the drawing  itself.    When entering  text

pressing  return  at the  end of  the  string will  draw the

normal  text.  If the up-arrow cursor key is pressed instead

of return, the text will be drawn with a line over it.  This

is useful for digital systems when a signal is active low.

Delete-Line:

This function is used to delete lines from the  current

drawing area.  A  line will be highlighted  in red and  made

dotted.   If this line  is to  be deleted press  Y for  yes.

Another line will be  highlighted.  Pressing Y for  yes will

delete that line  as well.   Other options for  keys are  to

press N for no.  Saying no will move to the next line, or if

no  other lines exist, will  exit the function.   Pressing B

for back will back up to  the previous line.  This was added

in  the  case  of   many  lines  on  the  screen   and  user

inadvertently bypassed  the desired line.  ESC  will exit at

any  time, leaving the current  line shown on  the screen in

PC-Trace Users Manual

38

the correct color.

Delete-Circle/arc:

This function  is used to delete  circles/arcs from the

current  drawing area.  A  circle/arc will be highlighted in

red and made  dotted.  If this  circle/arc is to  be deleted

press  Y for yes.   Another circle/arc  will be highlighted.

Pressing Y  for yes  will  delete that  circle/arc as  well.

Other options  for keys are  to press N  for no.   Saying no

will  move   to  the  next   circle/arc,  or  if   no  other

circles/arcs exist, will exit the function.   Pressing B for

back  will back  up to  the previous  circle/arc.   This was

added in the case of many circles/arcs on the screen and the

user  inadvertently bypassed  the  desired circle/arc.   ESC

will exit at any time,  leaving the current circle/arc shown

on the screen in the correct color.

Delete-Text:

This function is  used to delete text  from the current

drawing area.  A text string will be highlighted  in red and

made dotted.  If this text is to be deleted press Y for yes.

Another text string will be highlighted.  Pressing Y for yes

will  delete that text as well.   Other options for keys are

to  press N for  no.  Saying  no will move  to the next text

string,  or if no other text exists, will exit the function.

Pressing B for back will back up to the previous text.  This

was added in the case of many text strings on the screen and

the user inadvertently bypassed the  desired text.  ESC will

exit  at any  time, leaving  the current  text shown  on the

screen in the correct color.

Regenerate:

Selecting this function will redraw the current screen.

SCAD sometimes creates a little  garbage on the screen  when

adding and deleting lines.  This function will clean it up.

Set Snap:

The Snap function  is used  to set  absolute units  and

endpoints for drawing operations.  If the Snap  function has

already been turned  on, selecting it again will  prompt the

user  that it will now be turned  off.  If the Snap function

is  currently off, a text entry dialogue box will appear and

the snap  value will  be given.   Common values  are 0.1  or

0.25.   Any value between 0  and 1 can be  entered and used.

All subsequent usage of drawing commands will move endpoints

of lines, arcs, circles and text to the grid points shown.

Scale:

The  scale menu selection is used to change the size of

the current symbol  uniformly across its  entire area.   The

value  that  is  entered  when the  dialogue  box  opens  is

multiplied by  the  dimensions of  all the  entities in  the

symbol.   For example, if the  symbol was 1 unit  long and a

scale factor of  0.5 was  entered, the final  length of  the

PC-Trace Users Manual

39

symbol  would be 0.5 units.  Pressing  ESC will exit with no

changes.

Shift Space:

The shift  space routine  will move the  entire drawing

some  offset specified  by  entering a  starting and  ending

point of a  line.  If a line  from 1,1 to 2,2 is  drawn, the

entire drawing will be shifted 1 unit right and 1 unit down.

To draw the line, when prompted, click on the starting point

and ending point.   After the second click the  drawing will

be shifted.  ESC will exit backwards or out of this routine.

Extend/Trim:

This  function  allows a  line  to be  extended/trimmed

to/by another line, or a circle to be extended/trimmed to/by

a line, or  a line  to be extended/trimmed  to/by a  circle.

This   function  is  useful   for  designing  symbols  whose

intersection of  points aren't on convenient  dimensions.  A

good example of this is the  OR gate.  The function works by

first prompting  to see if the  object that will  be used to

extend or trim with is a line  or a circle/arc.  Press L for

line or  C for circle.   The software will now  use the same

procedure as  is  used for  deleting objects  to select  the

extend/trim edge.  Once it is selected it will be colored in

Magenta.  The second  prompt is for the object  to extend or

trim.   This  is the  object that  will have  its dimensions

modified.  If  the trim object was a line,  the program will

give the option of a line or a circle/arc to modify.  If the

trim object was a  circle/arc the program will automatically

select  the second object  to be a  line.  The  program will

highlight each object in succession until the user picks the

desired  object to modify.  The next  prompt is for the user

to select a point, with the mouse, near the end point of the

object to modify.  One more prompt exists if a circle is one

of the affected  entities.  This  prompt is to pick  a point

near  the projected intersection.  At the end of the prompts

the entity will be modified as per the user instructions.  A

regeneration may be required as well to clean up the screen.

The tools that have been described above  should be adequate

for drawing most objects.   Sometimes some creative usage  of the

Snap function and Extend/Trim is required.

Placing Components/Symbols:

This function is used  to place symbols on the  main drawing

area.   This area is separate  from the previous area  and has no

limits in size  for practical use.   The only limiting  factor is

the amount of available memory.

When a set  of graphic symbols  are put  together to make  a

meaningful  electrical symbol,  they  are grouped  as an  object.

They can only be edited by choosing from the Components menu  the

PC-Trace Users Manual

40

Place Component command.  The Move, Delete, and other commands in

the Entities section only affect the objects that have been drawn

not using this menu selection.

When Place Components is selected with the mouse or keyboard

a  window  will appear  in  the  center of  the  screen.   If  no

components  have been entered, only two choices will appear: N to

exit or A to enter new symbols.  ESC will also exit the function.

A-Enter new Components:

This  function will clear the window and prompt for the name

of the symbol that will be placed.  This name will not  appear on

the  symbol itself and is used only  to tell the designer exactly

what the part is.

The  second prompt  if for  the part  designation.   This is

later placed on  the name of the  part and is used  by PCTRACE to

specify  autorouter connections.   This  designation need  not be

unique  as it must be  in PCTRACE.   The reason for  this is that

some parts, such as AND gates  like the 74LS08 have more than one

part in them.   This allows  the user  to connect multiple  parts

under  one  name.    No  automatic   provision  is  provided  for

specifying  which part of the overall part is being connected to,

other than the pin number.  The program will notify the user if a

duplicate  designation  is  specified  to  verify  that  this  is

desired.

The  next prompt is a  filename for the  actual parts layout

information.   This is the same  file as that used  by PCTRACE to

specify  pad locations.  The  program will pop  up a small window

with lists of names that  are available for use.  Enter  the name

of the part without the extension and the program will display it

on the screen  in a  small window.   Information  about the  part

appears on the bottom  of the screen.   If it is correct  press Y

for yes or click in the Yes box.   If it is not correct, click on

the No  box or press N,  the program will reprompt  for the name.

This entry is not significant  for drawing the schematic  itself,

but is needed for export to PCTRACE.

The next prompt  is for the Pad  size.  Enter 1  for 0.05, 2

for 0.06, 3 for 0.07 and so forth.

The last text prompt is for the name of the schematic symbol

that will  be drawn on the  main drawing area.   The program will

open a window showing a  list of valid diagram files.   Enter the

diagram file with no extension and it will be drawn  to check for

user approval.   Answering Y  will move to the  main drawing area

for placement, and N  will go back  and reprompt for the  diagram

file name.

Once the diagram file has been selected the text window will

disappear and two  crosshairs will be shown on the  screen.  This

is  the size  of the  part that  has been  selected and  is being

placed.  Moving the mouse to the desired location and  pressing a

mouse  button will  anchor the  location of the  part.   The next

PC-Trace Users Manual

41

prompt  appears at  the bottom  of the  screen and  asks  for the

location  of  the text.    The text  that  will be  drawn  is the

designation  name  entered  earlier.   Remember  that  the  point

selected is  the upper left corner of  the first character in the

string.  The next prompt is for the  text scale and is the actual

size, in units,  of the characters that will be  drawn.  The last

prompt is for horizontal text.  Answering Y for yes will draw the

text  in  a horizontal  line  and N  for  no will  draw  the text

vertically.

At this point the user is prompted as to whether or not they

want to enter another component.  Click on the appropriate space.

Answering Y will start the process again, and N will put the user

on the last  entered component and give other options  as well as

display the current data.  At any point in the previous component

entry routine, ESC will back the user up.

From a Display of Component Data:

Once a  component has  been entered various  editing options

can be used on it.  Those options are discussed below:

E - Edit Component:

This will allow the user to change things such as the  name,

designation, pad  size, actual part layout  filename, location of

the  diagram, orientation of the diagram, and the location of the

text that is associated with this component.  The letters that do

each of these are discussed below:

A-Name:   This  will prompt for a new name, enter the new name or

press ESC to leave it unchanged.

B-Designation: This will prompt for a new designation name, enter

the name or ESC to exit.

C-Definition:  This will  prompt for  a new component  pad layout

definition.  Enter the new file or ESC to exit.

D-Pad Size:    This will  open a window with  three different pad

sizes.  Enter the desired value.

E-Move Comp:   This  will  allow the  user  to  move the  current

symbol  to a  new location.   The  crosshairs will

appear and mouse will move them.  Click the button

on the  location for the  new location.   ESC will

exit.

F-Rotate: This function will rotate all  elements associated with

this  component.  The device can be rotated 90, 180, or

270 degrees  counter-clockwise.   It  may require  some

movement of text to make everything look right.

PC-Trace Users Manual

42

G-Move Text:   This function allows the  user to first select the

text,  using  the previously  mentioned  method or

moving forward, backward,  and selecting.   A  box

appears  that is the  outline of the  text.  Using

the mouse,  move the text to  the desired location

and click a  mouse button.  The  new text location

will now be at this point.

H-Exit Edit:   Selecting H, will exit the edit function.

A-Enter new Components:

This function  allows entry of  more components  and is  the

same routine that was discussed at the beginning of this section.

Delete:

This function will allow deletion  of the component that  is

currently on screen.  The entire component can be deleted or just

the text.  Enter the  number that is desired.  An entry of 3 will

exit  with  no  deletions taking  place.    If  the component  is

deleted, all associated connections will be deleted as well.

Search:

This will allow the user to search from the current location

to the end  of the list of components.   Only the Designation can

be searched on.  Follow the prompts to do either.

+ - Next:

Pressing  the + key will  display the next  component on the

screen.   If at  the  end of  the list,  the  first component  is

displayed.

- - Previous:

Pressing  the - key will  display the next  component on the

screen.  If that the beginning of the list, the last component is

displayed.

Text Entry:

This  function allows text entry  that can be  placed on the

main drawing area.  This text will be linked in  to the component

that is currently displayed.  This text entry should only be used

for text that is associated  with the component.  It can  only be

modified  from this component.  More general text entry should be

done from the  Entities menu  while running in  the main  drawing

area.

PC-Trace Users Manual

43

N to End Component Entry and Editing:

N or ESC will move the user  back to main drawing area.  All

work will be held in memory.

Connection Entry:

Only one choice  exists from the Nconnections menu  and that

is to make  connections between existing  components.  The  input

routine was created to  minimize the amount of time  spent typing

and  maximize  the  ease  with which  connections  are  drawn and

recorded.  When the selection is executed it automatically starts

entry of a new connection.

The  first prompt  is  to pick  a  point inside  the  source

component.  The  crosshair will  show on the  screen under  mouse

control.  Place the crosshair in the "bounding box" of the source

component  and click a button.  When prompted for the destination

component  do the same  thing.   This will  tell the  program the

source and destination components.

The  next step is to tell the program the source pin number.

Enter it at  the bottom of the  screen when prompted.  The second

step is  to use the  mouse to specify  the starting point  of the

number just entered.  The third step is to enter the scale of the

pin number.  The destination pin number is done  exactly the same

was as the source pin.

The  source  and destination  pin  number  that are  entered

during this process will  be normal text.  The source number will

become  part of  the  source component  object  and will  not  be

editable from  the main drawing area.  It will be necessary to go

into the Place Components section to edit or move the  text.  One

reason for this is to move  the pin numbers or change the letters

to  have an  overline.   The  same is  true  for the  destination

number, it will be attached to the destination component.

After  both the source and destination  have been entered, a

dialogue   box  appears  to   show  the  user   that  source  and

destinations that  have  been  chosen.   If  the  information  is

incorrect click  on the No box  or press N,  the connection entry

will be  restarted at the beginning.  If yes is selected the line

draw  routine will started and  the user can  draw the connection

with as many  straight lines as desired.  The  e key for endpoint

and  right mouse  buttons are  active for  finding endpoints  and

making  connection dots.  The lines drawn will be attached to the

current connection object.  Deleting this object will  remove the

lines that are associated  with this connection.  The  only other

difference between  connections and components is  that the lines

drawn in  a connection can  be edited in  the main  drawing area.

After the last  line has been entered, pressing  ESC will end the

line drawing routine.   After pressing  ESC, the program  prompts

asks  if the  user  wants to  enter  another connection  or  not.

Answer  yes   to  restart  this  procedure   and  create  another

connection.    No will  put the  user  at the  connection editing

facilities.

PC-Trace Users Manual

44

Connection List Menus:

Several  options are available from  this point.   A list of

commands is shown as the last line in the window.      The   edit

command  allows the  user to  change  the source  and destination

names and pin numbers.  Changing this information will not change

the  graphic  data  that was  entered.    To  change the  graphic

representation of the number that are on the drawing will require

finding the component that they are associated with.

A)enter  new  will  allow  entry of  other  connections  are

described above.

Delete will delete the listed connection  and the lines that

are associated with it.

Search  will prompt for  a designation name  and search from

the current location to the end of all connections for a matching

source or destination designation.

'+' and  '-'  move to  next  or previous  connection  entry.

These keys  loop from start to  end or end to  start depending on

the key.

Line will allow the user to draw  lines in addition to those

that were drawn during the initial connection entry.  These lines

are drawn the same as during connection entry.  ESC will exit the

line drawing routine.

N for end or ESC will exit  this menu and return the user to

the main drawing area.

Drawing Entities:

Draw-Line:

This function is  used to  draw lines  within the  main

drawing area.  At the bottom of the screen, SCAD will prompt

the  user as  to  what the  program  expects.   The  current

coordinates  are  shown at  the  bottom right.    Select the

location of the  starting point  of the line  and click  the

left  or right button.  Clicking the left button will anchor

the  start  of  the  line  at  the  current  mouse location.

Clicking the right button will anchor the start point at the

current  point, plus it will  draw a connection  dot at this

point.   The  dot will  not appear until  the start  and end

points  of the line have been specified.  After clicking the

left  or right button, the  program will prompt  for the end

point.  Click either the right or  left button.  The buttons

have  the  same effect  as  when  picking the  start  point.

Another option for selecting the  desired point is to  press

'E' while  the program is asking for the start or end point.

Pressing  'E' at  this time  will draw  a small  box  on the

screen.  Placing this box at or near the endpoint of another

line will select  the exact  endpoint of that  line or  arc.

The  mouse buttons have the  same effect.   This function is

active much of time when in the main drawing area and should

PC-Trace Users Manual

45

be tested whenever it is  needed.  ESC will exit or  back up

while drawing lines.   To draw another line, select  a start

and  end point  again.   This  function  will continue  this

sequence until ESC is pressed to exit it.

Draw-Arc:

This function  is used to draw an arc of some size.  It

can  be used to draw any length  arc from any start point to

any end point and at any  diameter.  The first prompt is for

the  center  point  of the  arc.   Move  the  mouse  to this

location and click a button.   The second prompt is  for the

radius of the arc.   Move the pointer  to the location  that

shows  the desired radius.  A floating line reminds the user

as  to the current length.  The  next point is for the start

of the arc.  Pick this point  as before.  The last prompt is

for  the arc ending  point.  Click  on this point.   The arc

will  be drawn  from the  angle of  the start point,  at the

selected radius,  counter-clockwise, to the end  angle.  ESC

will back up at any time to the previous prompt or will exit

the  function.   To  draw  another arc,  pick  this function

again.

Draw-Circle:

This function draws  a circle at some  center point and

some radius.  When prompted, pick the center point and click

a mouse button.  The second prompt is for the  radius of the

circle.  Click on this point and the circle will be drawn on

the screen.  ESC will back up to the previous prompt or exit

the function.   To draw another  circle, pick this  function

again.

Draw-Text:

This  function is used place  text on the  screen.  The

first prompt is for the  scale of the text.  This  number is

the size  of the text.  A value of 0.25 will create a string

with  characters every 0.25 units.  The second prompt is for

horizontal versus  vertical text.   Answering yes  will draw

the text horizontal and no will  draw the string vertically.

The letters  are always upright,  they are  not rotated  for

vertical  text.   The third  prompt is  for the  text string

itself.   This  string  will be  displayed  on  the  drawing

itself.  The last prompt is for the starting location of the

text.   This point is located at the top, left corner of the

text string.  Select this  point by moving the mouse  to the

desired  location and  click a  button.  When  entering text

pressing  return  at the  end of  the  string will  draw the

normal  text.  If the up-arrow cursor key is pressed instead

of return, the text will be drawn with a line over it.  This

is useful for digital systems when a signal is active low.

PC-Trace Users Manual

46

Move-Line:

The move line  is used  to change the  placement of  an

existing line.   When this  selection is  made, the  program

will highlight  lines in succession until  the user responds

with a Y for yes.   At this point the user will be  asked to

select, with the  mouse, a base point of movement.  Click on

that point. E is active to  select the endpoint of the line,

or  any other, for movement.   After the  basepoint has been

selected and  the mouse button clicked, the entire line will

move with the  movement of the mouse.   Position the line at

the desired location and click a button to permanently place

the line.  E is active to select an endpoint.  ESC will back

up or exit the function.

Move-Circle/Arc:

The  move circle/arc  function  is used  to change  the

placement of an existing circle/arc.  When this selection is

made,  the  program  will  highlight  circles  and  arcs  in

succession until the  user responds with  a Y  for yes.   At

this point the user will be asked to select, with the mouse,

a base point of movement.   Click on that point. E is active

to  select  the  endpoint of  the  arc,  or  any other,  for

movement.   After the  basepoint has  been selected and  the

mouse button  clicked, the entire circle/arc  will move with

the movement of the  mouse.  Position the circle/arc  at the

desired position and click a button to permanently place the

circle/arc.   E  is active  to select an  endpoint.   ESC is

active to back up or exit the function.

Move-Text:

The  move text function is used to change the placement

of  an existing text string.   When this  selection is made,

the program will highlight  text strings in succession until

the user responds with a Y for yes.  At  this point the user

will be  asked to select,  with the  mouse, a base  point of

movement.   After  the basepoint  has been selected  and the

mouse button clicked, a box  representing the text will move

with  the movement of the  mouse.  Position  the text box at

the desired position and click a button to permanently place

the text.  E is active to select an endpoint.  ESC is active

to back up or exit the function.

Delete-Line:

This  function is  used to delete  lines from  the main

drawing area.  A  line will be highlighted  in red and  made

dotted.   If this line  is to  be deleted press  Y for  yes.

Another line will be  highlighted.  Pressing Y for  yes will

delete that line  as well.   Other options for  keys are  to

press N for no.  Saying no will move to the next line, or if

PC-Trace Users Manual

47

no  other lines exist, will  exit the function.   Pressing B

for back will back up to the previous line.   This was added

in  the  case  of   many  lines  on  the  screen   and  user

inadvertently bypassed the  desired line.  ESC will  exit at

any  time, leaving the current  line shown on  the screen in

the correct color.

Delete-Circle/arc:

This function  is used to delete  circles/arcs from the

current drawing area.   A circle/arc will  be highlighted in

red and  made dotted.  If  this circle/arc is  to be deleted

press Y  for yes.   Another circle/arc will  be highlighted.

Pressing  Y for  yes will  delete  that circle/arc  as well.

Other options for  keys are to  press N for  no.  Saying  no

will  move   to  the  next   circle/arc,  or  if   no  other

circles/arcs exist, will exit the function.   Pressing B for

back  will back  up to  the previous  circle/arc.   This was

added in the case of many circles/arcs on the screen and the

user inadvertently  bypassed  the desired  circle/arc.   ESC

will exit at any time, leaving  the current circle/arc shown

on the screen in the correct color.

Delete-Text:

This function is  used to delete text  from the current

drawing area.  A text string will be  highlighted in red and

made dotted.  If this text is to be deleted press Y for yes.

Another text string will be highlighted.  Pressing Y for yes

will delete that text as well.   Other options for keys  are

to press N  for no.   Saying no will  move to the next  text

string,  or if no other text exists, will exit the function.

Pressing B for back will back up to the previous text.  This

was added in the case of many text strings on the screen and

the user inadvertently bypassed the  desired text.  ESC will

exit  at any  time, leaving  the current  text shown  on the

screen in the correct color.

Regenerate:

Selecting this function will redraw the current screen.

SCAD  sometimes creates a little garbage  on the screen when

adding and deleting lines.  This function will clean it up.

Set Snap:

The  Snap function  is used to  set absolute  units and

endpoints for  drawing operations.  If the Snap function has

already been turned  on, selecting it again  will prompt the

user  that it will now be turned  off.  If the Snap function

is  currently off, a text entry dialogue box will appear and

the  snap value  will be  given.  Common  values are  0.1 or

PC-Trace Users Manual

48

0.25.   Any value between 0  and 1 can be  entered and used.

All subsequent usage of drawing commands will move endpoints

of lines, arcs, circles and text to the grid points shown.

Zoom In:

This function requires the  use of the mouse to  pick a

bounding box for an area  to zoom in on.  Click the mouse on

one corner of the area to zoom in on and  click the mouse on

the opposite corner.  SCAD will show the boxed  in region on

screen  as  the full  screen  while  maintaining the  aspect

ratio.

Zoom Out:

The Zoom out  function prompts for  a zoom out  factor.

Enter a value.   A value of 2, for  example, will double the

area  being viewed.  A  value of three  will triple the area

and so forth.

Zoom Total:

The Zoom Total will zoom out so that all of the drawing

is shown on  the screen.   This function  finds the  overall

size   of  the   drawing  and   adjusts  the   viewing  area

accordingly.

Shift Space:

The shift  space routine  will move the  entire drawing

some  offset specified  by  entering a  starting and  ending

point of a  line.  If a line  from 1,1 to 2,2 is  drawn, the

entire drawing will be shifted 1 unit right and 1 unit down.

To draw the line, when prompted, click on the starting point

and ending point.   After the second click the  drawing will

be shifted.  ESC will exit backwards or out of this routine.

Extend/Trim:

This  function allows  a  line to  be  extended/trimmed

to/by another line, or a circle to be extended/trimmed to/by

a line, or  a line  to be extended/trimmed  to/by a  circle.

This  function  is   useful  for  designing  symbols   whose

intersection of  points aren't on convenient  dimensions.  A

good example  of this is the OR gate.  The function works by

first prompting to  see if the object  that will be used  to

extend or trim with is a line or a circle/arc.   Press L for

line or  C for circle.   The software will now  use the same

procedure as  is used  for  deleting objects  to select  the

extend/trim edge.  Once it is selected it will be colored in

Magenta.  The  second prompt is for the object  to extend or

trim.   This  is the  object that  will have  its dimensions

modified.  If  the trim  object was line,  the program  will

give the option of a line or a circle/arc to modify.  If the

PC-Trace Users Manual

49

trim object was a  circle/arc the program will automatically

select the  second object to  be a  line.  The  program will

highlight each object in succession until the user picks the

desired object  to modify.  The next  prompt is for the user

to select a point, with the mouse, near the end point of the

object to modify.  One more prompt exists if a circle is one

of the  affected entities.  This  prompt is to pick  a point

near  the projected intersection.  At the end of the prompts

the entity will be modified as per the user instructions.  A

regeneration may be required as well to clean up the screen.

Keyboard Summaries

for SCAD

When Entering Text on the schematic:

RETURN         - enters the text normally

Cursor Up Arrow - overlines the text

When drawing points, lines, arcs, and text:

E         - gives user box to find endpoints

R. Mouse Button - puts connection dot on point of line

PC-Trace Users Manual

50

Conclusion:

The PCTRACE circuit design system was written by Doug Ehlers

for use  by himself.   This  was  done without  ever having  used

another design package.   The  decisions made were  chosen to  be

intuitive  to   the  author.    The  program  is  under  constant

refinement,  so the user should try and maintain contact with the

author through  one of the previously  mentioned support methods.

As always the author is interested in user input on the software.

The program has evolved as it  has through the input and  support

of its users.  In the words of a famous gentleman (almost) "Thank

you for your support."

PC-Trace Users Manual

Appendix A

.NLT File Format

The  file created by the export function  of SCAD and read by the

import  function of  PC-TRACE  can be  used  as a  mechanism  for

importing  netlists from other CAD software.  It is currently not

feasible  for the author to  offer this support.   If conversions

are written by users, they are encouraged to  let the author know

so that they  can be shared by  other users.  The ->  signifies a

line in the file.  The following is the file format:

->Number of Components in list

Each component looks like:

->Name of component           15 characters

->Component Designation        5 characters

->Definition file name        12 characters

->Pincount  & Padsize          2 Integers with 2 spaces between

Pincount is an integer signifying the number of pins in the file.

The  Padsize is a 3 bit  number in the 5th, 6th,  and 7th bits of

the number.  0  is 0.05, 1 (=32) is  0.06, and 2 (=64) is  a 0.07

inch diameter pad.   The  lower five  bits of  Padsize should  be

zero.   The bit 0 signifies  if the component has  been placed on

the board yet.

->Number of Connections in list

Each Connection looks like:

->Source Designation          5 characters

->Source Pin Number           Integer

->Destination Designation     5 characters

->Destination Pin Number      Integer