�������۰ ������۰  �����۰ ��۰  �۰ ��������۰ ������۰  ۰     ��۰  �۰

۰    ۰ ۰    ۰  ۰   ۰  ۰  �۰  ۰  ۰  ۰ ۰    ۰  ۰      ۰  �۰

������۰ ������۰  ۰       ����������������ͻ  ������۰  ۰      ����

۰       ۰    ۰  ۰       ۺ MESSAGE THIEF �  ۰    ۰  ۰      ۰  ��

۰       ۰    ۰  ۰   ۰  ����������������ͼ  ۰    ۰  ۰   ۰ ۰   ۰

��۰     ��۰  ��۰ �����۰ ��۰  �۰    ��۰   ��۰  ��۰ �����۰��۰  �۰

(R).

This manual covers versions 1.04 to 1.09-J.

PACKTALK - PBBS uses SOFTWARE FLOW CONTROL.

A three wire hookup is all that is necessary and MUST be run at a computer

speed NO faster than 12mc. If your using a 286 or 386 set your speed to the

slowest speed.

If you MUST run the computer at a faster speed make sure that you

open the com ports to at least 9600 baud. Also, PACKTALK should not print

more than 1 line per second to the TNC when doing an UP or DOWNLOAD otherwise

you will loose data. This can be controlled by the settings of the parameters

in line 1 and 23 of the PBBS.CNF file. Set in increments of 5 until you get

the proper speed. Set for a little slower than 1 line per second to be safe.

IF YOU HAVE PT-109J OR HIGHER, THE ABOVE NO LONGER APPLIES.

PACKTALK is written in COMPILED BASIC and needs BRUN45.EXE run time

module in order for it to work.  Do not use any file compression programs

on any of the files associated with this software. PACKTALK IS NOT GURANTEED

TO WORK WITH ANY DOS ABOVE 3.3 OR WITH ANY TNC OTHER THAN A KANTRONICS!

If you make the GATEWAY available to your users keep this in mind. When

a user accesses the Gateway, the USERS callsign is the one that will be used

on the Gateway, NOT the call of the PBBS.

The first thing to do after acquiring PACKTALK is to make a directory

called PACKTALK and copy ALL files into the PACKTALK directory.

**** The EXE files ****

=============

PACKTALK EXE          [ the terminal program ( see note below ) ]

PBBS     EXE          [ the BBS program ]

GATEWAY  EXE          [ the gateway routine when user enters a <G> ]

MFWD     EXE          [ the mail forwarding routine ]

REVFWD   EXE          [ this does the reverse poll for mail ]

BRUN45   EXE          [ the runtime module ]

EDITOR   EXE          [ an extremely good text editor ]

Note:

All EXE files are needed to operate the BBS but should you want

to run just the terminal program you may do so, to use just

the terminal mode you need PACKTALK.EXE, BRUN45.EXE and the first

20 lines of the PBBS.CNF file.

Page 2

You will have to make any directories for the PBBS. These are the

DIRECTORIES that are displayed when a user  sends a  <W>  to the BBS.

At this point you can make the necessary directories needed to RUN the

pgm.  Create these directories from the ROOT of your hard drive:

MSG

BBS

BBS\LOG

BBS\HOME

BBS\NAME

BBS\DATABASE

BBS\MFWD

BBS\STATS

BBS\MFR

ANY additional directories needed for user downloads will be made as sub-

directories of the BBS directory, ie.:

BBS\PGMS

BBS\BASIC

BBS\MODS

ALL directories must have in them an ASCII representation of the directory

except for the MSG directory. You can create these ASCII directories in 2

ways, 1 by running the pgm. SORTBBS or by going into each and every dir.

and typing DIR>DIR. DS.COM is used to sort the directories and should be

used before creating the ASCII directories manually by typing from the

ROOT dir. DS N/S. If you use SORTBBS.EXE this step is not necessary.

If someone is on the PBBS and they send a W or a W directoryname and

the pgm. sends back a prompt, no such directory exists and you know that it

does it is probably because you did not create the ASCII representation file

for that directory. When a file is uploaded to a directory PACKTALK will sort

the directory and then create a NEW ASCII directory for that directory, so

once you set it up properly you no longer need to maintain it. To find out

what the PBBS can do access it your self and download the BBS-CMDS.DOC file

which you should have put in the BBS directory also you should have HELP.BBS

and SHELP.BBS in the BBS directory.

THE ABOVE FOR PT-103 AND LOWER.

You may operate this PBBS as a CLOSED system. This means that only

stations that are in your MFWD.BBS file will be able to connect. This is

a parameter that you set in the PBBS.CNF file. The options you have are

OPEN or CLOSED.

Page 3           Example of the PBBS.CNF file.

0  <delay for the faster computers to prevent data lose on file xfers>.

9600,N,8,1  <9600 baud for PT-104>.

N3DFD    <your call, no SSID's>.

N3DFD-4  <the call that will be assigned to the TNC>.

***  CONNECTED

***  DISCONNECTED

JUNK.TXT  <default file name for downloads, may include path>.

TNC=2  <if you are utilizing COM2, if not then TNC=1>.

SOUND=Y    <or SOUND=N>.

CONOK OFF  <setting for TERMINAL mode, PBBS will always be CONOK ON>.

BEACON E 240   <for tnc 1>.

BEACON E 120   <for tnc 2>.

BT PACKTALK, Version 1.0.4.

CT Accessing PBBS, please wait..

U PBBS V ABX    < unproto >

TXD 75

K    <K for Kantronics, O for other.  Sends RESET or RESTART to TNC>.

10 20 45 <times within any 1 hour to forward mail>.

F3 to Exit..<line 25 prompt for PBBS ready mode, can be no longer than 80>

[PBBS back on line].<sent over the air when returning to PBBS ready mode>

TIMER/S <the command to run your timer pgm. ,to set the computers clock>.

*** eof

0    <delay for the faster computers to prevent data lose on file xfers>.

Please enter your HOME PBBS, (where you get mail). ie., (NH N3DFD).

361     <time out for PBBS in seconds>.

Philadelphia, Pa.

19124.   <your zip code>.

- (B,CB,D,F,FS,G,H,HA,I,J,JH,K,L,N,NH,R,S,T,U,V,W,WH,X,?,)>

[You have mail, use <RM> to read it and <KM> to kill it].

thanks for connecting, ya'll come back now, hea! < signoff text >

===> [Timeout] <===       <prompt sent when PBBS times out>.

Please stand by...        <prompt for Talk mode, PBBS>.

18 19 20 21 22 23       <download prohibit hours for files over 25k>.

N3DFD N3FOA WB3EUF        <list of remote SYSOPS>.

144.990   <COM1 freq.>.

29.2500   <COM2 freq.>.

15     <amount of things a user can do on the pbbs before kick off>.

Y      <limit MAIL FOR Btext to CALL signs only>.

No - already have that BID.

No - don't want it.

You are being denied access to this PBBS at this time!  <jerk text>.

NOCALL PK232 <jerk list>. (lids)

PA.USA.NA  <your "H" address for the R: headers>.

OPEN    <OPEN or CLOSED BBS, if CLOSED then BBS connects only>.

*** eof

### DISCONNECTED

FAIL                     < prompts that you get back >

RETRY                    <  from the various nodes.  >

LINK MADE

> CONNECTED              < you may edit them if needed. >

CONNECTED

Y  <if N then all messages will be forwarded without the R: headers>.

Page 4

**** REMOTE SYSOP ****

============

REMOTE SYSOP: You can place up to 5 callsigns in the PBBS.CNF file

and these will have remote sysop privileges.

By a call being in the PBBS.CNF file as a remote sysop the first thing

this allows is that when that particular person when connected to your PBBS

will be able to see private messages, he will be able to kill ANY message,

He will also be able to over write existing files in your directories, kill

files in any directory, create new ASCII directories in any directory

on any drive, he can up and download files to and from ANY directory on ANY

drive, so be careful who you give privileges to. He can also shut your system

completely down. By creating an ASCII directory in the ROOT directory, then

downloading that ASCII directory he will have a full map of your disk layout.

Can you see the possibility? To find what the remote commands send a  X  and

you will get the menu listed below:

Summary of SYSOP commands for the PBBS

[LF] ------- Used to see what mail is left to be forwarded.

[STATS] ---- Used to download the daily STATS file, who con. and did what.

[DEL STATS]  Used to kill the STATS file.

[*] -------- Used to see how many messages on the PBBS.

[XD] ------- Used to download files, ie.,      <XD C:\BBS\MFWD\BID.NUM>.

[XU] ------- Used to upload a file, ie.,       <XU C:\BBS\MFWD\REJECT.BBS>.

[SHELL] ---- Used to shell a pgm, ie.,         <SHELL SORTBBS>.

[DIR] ------ Used to create an ASCII dir. ie., <DIR C:\WORK>.

[DEL] ------ Used to delete a file, ie.,       <DEL C:\WORK\M.AA>.

[KF xx-xx]-- Used to kill messages, ie.,       <KF 30-70>.

[KS] ------- Used to kill messages, ie.,       <KS AMSAT>.

[K ALL ----- Used to kill ALL messages, ie;    <K ALL>.

[PURGE] ---- Used to PURGE the PBBS, ie.,      <PURGE>.

[PS xxxxx] - Used to reset tnc parameter, ie., <PS TXDELAY 50)

[CREATE] --- Used to create a .BAT file that will be executed as soon as

you complete it. Type CREATE and then follow the prompts.

[*** SHUTDOWN ] -- Used to shutdown the PBBS. Not recoverable by SYSOP.

***** C A U T I O N *****

=============

The SHELL command may be used remotely but be careful with it.

if you SHELL a .BAT file or a pgm. that at any time requires keyboard

input your stuck. Presumably you won't be there to press any keys.

Example of the SHELL command usage:   SHELL DO.BAT   When the task is

completed you will get the prompt back, Completed!

Page 5

*** FORWARDING ***

==========

PACKTALK can forward BULLETINS to as many  BBS's you want to, just

enter callsigns in the MFWD.BBS file. It can forward messages addressed to

a specific call sign as long as they are entered in the MFWD.BBS file. You

can enter any amount of calls you wish to forward to.

The first thing you will need to do to set up the BBS for auto-forwarding

is to create a file in the directory you are running PACKTALK from and name

it MFWD.BBS.  This file will contain the call signs of the stations you

will forward to, the PRIMARY BBS is POSITION SENSITIVE and has to be the last

entry in the file, this is normally the PBBS that forwards to you. If you get

a message and it can not go in any other PBBS's slot then it will always go

to the last PBBS in the MFWD.BBS file. It can forward either on COM1 or COM2

ports. Forwarding on COM2 is determined by the PORT2.BBS file.

Following is an example of what the MFWD.BBS file will look like:

KA2EZN ALLBBS ALLUSA ALLUS AMSAT N3FVG NO2V NJNET

WA3TSW ALLBBS MDCBBS EPA SEPA PHILA

N3FOA  ALLBBS ALLUSA PANET N3FVG N3INY AMSAT ARRL .CA .OR

N3ACL  ALLBBS ALLUSA MDCBBS ATLDIV

123456789  <not part of the file>.

NOTICE: all the text following the callsign begins at location 8 on the

line. THIS IS VITAL for the file to work as PACKTALK at times reads only the

first 6 characters to compare it to other strings.

The text following the callsigns are the distribution designators,

(the @BBS field) and there can be as many on the line as you need. The line

can be up to 32k long, (an unbroken line, meaning no CR/LF except at the

end of the line).  In the first example, if a message came in for N3FVG, it

would be forwarded to KA2EZN, etc...  This file is kept in the \PACKTALK

directory with another file you create called FWD.TIM. In this file, on a

single line you place the hours your BBS will be allowed to forward. For

example, here is what the FWD.TIM file might look like;

00 01 02 03 04 05 06 07 08 09 10 11 12 13

During the hours that are in this file, your BBS will forward mail.

If you desire to forward 24 hours a day then put an * anywhere on the line.

If you don't create this file PACKTALK will create it automatically and

place a single * in the file and nothing else.

Page 6

Now you must create the CONNECT files and they will be put in the

\BBS\MFR directory. The name of the file will be the callsign of the BBS

you are forwarding to and the extension .CON. Here is an example of a few:

N3ACL.CON, WA3TSW.CON, N3FOA.CON.  What you place in these files is the

connect request you would normally type to connect to a station, ie;

C N3FOA, You may place as many connect requests as you need, for example,

if you wanted to connect to WA3TSW via a node you would on the first line

place the connect request to the node and on the second line the connect

request to WA3TSW. Following is an example of what the file would look like:

C WA3YUE-1

C WA3TSW

That's all there is to it, just remember you MUST have a correspon-

ding .CON file for every BBS you list in the MFWD.BBS file.

The MFWD.BBS and FWD.TIM files can be edited from the program. As

messages come in they are put into files corresponding to the callsigns you

have listed in your MFWD.BBS file. For example if a bulletin comes in to

ALL @ ALLBBS, its corresponding number will be put into a file with the

callsign of the BBS that is forwarding to you and the extension .MFD. ie;

N3ACL.MFD. The .MFD files are created as messages come in and will have only

1 number on each line. These files also can be edited from the terminal mode

also if necessary with the ALT-F function.

WARNING! There must NEVER be a blank line in any file pertaining to this

PBBS and also there must never be a blank line as the LAST line in any

file. PT-104 will read all lines in a file and if any are blank this will

leave a string variable in the software blank and that will cause problems.

Also kept in the \BBS\MFR directory is a file named in numerals,

representing the day and an extension .MSG. This is a permanent record of

the messages that are forwarded and to what BBS. This file can also be

accessed by the ALT-F function. Any messages that are not forwarded due to

not having a bid number are listed in a file named NO-BID.MSG and can be

viewed by the ALT-F function. This file is not automatically reduced so it

can grow quite large over a period of time.

ALL private messages are backed up in the \BBS\MFWD directory and

once a month are archived into a ARCHIVED file with the first name being

the month and the extension .MSG. This way there can only be 12 files at

any given time.

If for one reason or another you need to abort the MF routine, just

hit the ENTER key. [not a good practice, only do this if necessary].

Page 7

PACKTALK will handle "H" addresses. Addresses are kept in a file named

ADDRS.BBS. This file is kept in the \BBS\MFWD directory. PACKTALK will now

keep this file updated. Here is a sample of an entry: WA3ABC.PA.USA.NA.

PACKTALK also supports BID's and MID's.

You can have your BBS reject any type of message you wish by entering

the type of message in a file named \BBS\MFWD\REJECT.MSG. For example if you

wanted to reject messages addressed to @AMSAT you would enter AMSAT into the

REJECT.MSG file. You can also have PACKTALK reject messages from a particular

call by entering the persons call sign in the file also. Each entry on a

separate line.

If you wish to have PACKTALK hold the mail instead of forwarding it

automatically you can do this by entering the terminal mode and press ALT-M

and you will be prompted if you want to hold the mail. If you enter Y to the

prompt it will put the mail on hold.

The BID numbers (BULLETIN IDENTIFICATION NUMBERS) for bulletins are

kept in the \BBS\MFWD directory in a file named BID.NUM. Packtalk keeps the

last 4k of BIDS in it at all times, there is no need to do anything to this

file.

PACKTALK keeps all of the R: headers in each message although

the user will not see them.  What this does is when a message is read the

program uses the R: headers to show the user the PATH that the message took

to get to your BBS. Here is an example of what it may look like:

PATH: N3FOA!N3DFD!WA3TSW!WB3JOE!

FORWARDING TO TNC MAILBOXES: You no longer need to do anything about that,

just put the persons callsign in the MFWD.BBS file, make up a .CON file for

it in the \BBS\MFR directory and that's all there is to it. PT-104 will

know if it's a tnc's mailbox or a regular PBBS. If you want to forward

more than just personal messages to him then place the categories on the

same line as his call sign, ie; AMSAT, ALLBBS, ARRL, etc...

If PACKTALK doesn't find a BID number for a BULLETIN that it is attempting

to forward, it will NOT forward that BULLETIN.

MORE ON 'H' ADDRESSING FURTHER ON IN THIS MANUAL.

Page 8

*** PACKTALK UTILITIES ***

==================

BTRENUM.EXE --- Used to renumber messages.

BUBBS.EXE    -- Used to backup the entire PBBS on to another

hard drive ONLY! (mirror backup).

SORTBBS.EXE  -- Used to sort all the directories associated

with the PBBS and create a new ASCII directory

in each directory.

CHANGE.EXE   -- Used to automatically alter the TO and AT

fields in the MESSAGE.DAT file and delete old

messages set by the LIMIT.MSG file.

SERVER1.EXE  -- Used to remove duplicate and bad 'H' addresses

from the ADDRESS.BBS and ADDRS.BBS files that

are kept in the \BBS\MFWD directory. This pgm.

can be run automatically by inserting the com-

mand in the OPTION.BAT file which is run once

a day at 7pm.

ALL the above support pgms. for PACKTALK must be available to MS-DOS by

the PATH command. It's probably a good idea to keep all these pgms. in

the \PACKTALK directory.

------------------------------------------------------------------------------

**** BTRENUM.EXE ****

===========

A word about BTRENUM.EXE. This is a separate program that you

must run from DOS.  What this pgm does is RENUMBERS your messages

that are in the PBBS. BTRENUM MUST be in the PACKTALK directory .

After you do a renumber you can move BTRENUM.EXE to any place you like to

store it until the next time you need it. The highest message number allowed

is 99999.

All the messages and the MESSAGE.DAT file are kept in a directory

named \MSG, off the ROOT directory of the hard drive that you installed the

program on. Also kept in this directory is the MESSAGE.CNT file that keeps

track of the message numbers.

NOTE: CHANGE.EXE and SERVER1.EXE are part of the PACKTALK.EXE file for

versions 1.0.5 and above.

Page 9

The following files are to be kept in the same directory you are running

PACKTALK from.

PORT2   .BBS       [ BBS's that you forward to on port 2, see note below ]

GATEWAY .HLP       [ text sent to user going into the Gateway ]

DBASE   .INF       [ see note below ]

TNCPARMS.SET       [ parms to set up TNC, ALT-L ]

LOGO    .TXT       [ displayed upon bootup of program ]

INFO    .TXT       [ the info about your system, "I" command ]

SIGNON  .TXT       [ text sent to the user accessing the BBS ]

MFWD    .BBS       [ this file controls the mail forwarding ]

PBBS    .CNF       [ this is how the BBS is setup to run ]

BTRENUM .EXE       [ renumbers messages, optional ]

PORT2.BBS note:  ENTER 1 PBBS per line only, no SSID's, Example follows:

WB3EUF

N3DFD

KA2EZN

===> NOTE: See page 27 for documentation on PT-109-E and using SUB ports.

LIMIT.MSG note:  the default file will be as follows:

SYSOP

NONE

NONE

NO KILL

250

This can be set up for anything in the TO field and you may have it delete the

message after it has been copied into the BBS\DATABASE directory. It can be

very handy if you want to take a poll. When you send a bulletin you would

have replies sent to POLL @ N3DFD and setup the LIMIT.MSG file as follows:

POLL

whatever other TO field you want here.

whatever other TO field you want here.

KILL

250

Any message addressed to POLL will be copied into the BBS\DATABASE directory

then message will be killed and be removed from the MESSAGE.DAT file.

The number 250 in the LIMIT.MSG file determines the number of messages

CHANGE.EXE will allow on the PBBS. When that number is reached CHANGE will

KILL the oldest messages up to that number. If you wish to PROTECT any msg.

from being killed place a * after the DATE in the MESSAGES.DAT file for that

message. You may edit this number to be any number up to 505.

Page 10

The following files are to be kept in the \BBS\MFWD directory.

ADDRS    BBS     [PACKTALK now compiles this file]

ADDRESS  BBS     [for mail addressing and the CALL BOOK feature].

CHANGE   TO      [ optional ]

CHANGE   AT      [ optional ]

REJECT   MSG     [ optional ]

The following should be kept in the \BBS directory:

SHELP.BBS

HELP.BBS

BBS-CMDS.DOC

This is the directory the user sees when he sends a W to the BBS.

==============================================================================

**** TERMINAL MODE ****

=============

There is one menu that may be brought up by

entering ALT-H in the terminal mode only.

Here is a list of the keys and their function:

ALT-Z          [ copy a message into a directory to use as a file ]

ALT-F          [ clean MFWD.BBS file, DELETES ALL MESSAGE NUMBERS ]

ALT-M          [ to put mail on or off HOLD / to force mail forwarding ]

ALT-N          [ reverse polling to BBS for mail forward ]

ALT-C          [ to clear screen ]

ALT-E          [ to activate CHANGE.EXE ]

ALT-R          [ access DR.COM to read messages in the \MSG dir. ]

ALT-J          [ calls up EDITOR.EXE to edit the MFWD.BBS file ]

ALT-P          [ toggles dual screen when running dual ports only ]

ALT-L          [ upload parms to TNC ]

ALT-X          [ view/kill STATS files ]

ALT-B          [ message thief ]

HOME           [ toggle between COM ports ]

PgUp           [ to upload a file ]

PgDn           [ to download a file to disk ]

Page 11

At the bottom of the screen in the terminal mode there is a

list showing the function of the F keys. Listed below is a run down on

what they do.

F-1   < to set path and file name for download >

F-2   < shells to DOS, type EXIT to return to program >

F-3   < toggle between the TERMINAL and PBBS READY MODE >

F-4   < to access your BBS >

F-5   < calls up DR.COM to read files >

F-6   < filter the headers, on the bottom of the screen shows the HEADERS >

F-7   < calls up EDITOR.EXE, you name the path and file >

F-8   < shows who connected to the BBS, STACK and DATA space left >

F-9   < does a hard disconnect and restores screen >

F-10  < calls up EDITOR.EXE to edit the MESSAGE.FWD file >

F-8 note:

the program will delete the WHO.CON

file after date change and somebody connects to the PBBS.

=============================================================================

At this point it might be a good idea to bring up the editor and

edit the SIGNON.TXT file. (no more than three lines, each line followed by a

carriage return except the last line). and then create or edit the INFO.TXT

file. This is the file that is read when an I is sent to the BBS.

Page 12

*****  Final Comments ****

==============

Don't forget, you will need BRUN45.EXE for the program to run.

You may keep it in the PACKTALK directory or any directory covered by

your PATH command.

Anyway, that's about it, so hope you can get the thing up and running

with out too much difficulty. There are a few here in this area who have

been running PACKTALK for some time now so just about all the bugs are out of

the software. This software runs perfectly with any TNC.

PACKTALK.EXE uses SOFTWARE FLOW CONTROL, so set your TNC parms accordingly.

If you have any problems you might try running SORTBBS.EXE before

running PACKTALK and after you create all the directories and subdirectories you

need.

ALWAYS use the RUN.BAT file to load and run PACKTALK as the RUN.BAT

file sets a few DOS conditions.

==============================================================================

/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/

==============================================================================

Any support pgms. for PACKTALK you do not have may be obtained from

the N3DFD PBBS on 144.990/29.2500 Mhz. in Philadelphia or from:

The Software Exchange Xpo (S.E.X)

912 E. Ramona Ave.

Phila. Pa. 19124.

de N3DFD (Joe).

=============================================================================

/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\/\

=============================================================================

NOTICE: Whatever TNC is used, the following MUST be observed:

The TNC must NOT send added LINE FEEDS to the computer. Carriage return &

line feed must be appended to each outgoing packet. ALL time and date

stamping must be turned OFF.  DO NOT SET UP YOUR TNC FOR "BLOCK TRANSMISSION

OF PACKETS". Streamswitch must be set for $00. These are all parameters set

by your tnc but all tnc's don't use the same name for the particular parm.

in question, that's why this explanation!

Page 13

TNC parameter settings for the AEA tnc's for running PACKTALK.

COMMAND    MNEMONIC    DEFAULT                          FUNCTION

_______    ________    ____________________   ________________________________

8BITCONV     8B        ON                     Strip bit 7 in CONVERSE

ABAUD        AB        110 baud               ASCII baud rate

ACRDISP      ACRD      256 OR 0               Screen width

ACRPACK      ACRP      ON                     Add <CR> to packet

ACRRTTY      ACRR      71                     Auto <CR> column in RTTY

ALFDISP      ALFD      OFF                    Send LF after <CR> to terminal

ALFPACK      ALFP      ON                     Send LF after <CR>, packet

AWLEN        AW        8                      Data bits per word, to terminal

AX25L2V2     A         ON                     Operate as AX.25 Version 1.0

AXDELAY      AXD       0 (zero)               Repeater key-up delay (X ms)

AXHANG       AXH       0 (zero)               Repeater hang time (X10 ms)

BKONDEL      BK        OFF                    Send BS SP BS for DELETE char.

CANLINE      CAN       $18 <CTRL-X>           LINE DELETE character

CANPAC       CANP      $19 <CTRL-Y>           PACKET DELETE character

CASEDISP     CAS       0                      Display case; as is/lower/upper

CBELL        CB        OFF                    Enable "Connect" bell

CCITT        CC        ON                     Select CCITT ITA#2 or US Baudot

CFROM        CF        Empty                  Connect request/accept list

CHCALL       CHC       OFF                    Show call sign after channel ID

CHDOUBLE     CHD       OFF                    Show CHSWITCH character twice

CHECK        CH        30                     Idle link timeout (X10 seconds)

CHSWITCH     CHS       $00                    Channel-select (Link) character

CMDTIME      CM        1                      Transparent Mode escape timer

CMSG         CMS       ON                     Send CTEXT message to caller

COMMAND      COM       $03 <CTRL-C>           Character escape to COMMAND Mode

CONMODE      CONM      CONVERSE               Mode to enter when link starts

CONPERM      CONP      OFF                    Never disconnect this link

CONSTAMP     CONS      OFF                    Mark connections with time/date

CPACTIME     CP        OFF                    Use packet timeout in Converse

DAYSTAMP     DAYS      OFF                    Include DATE in time-stamp

DAYTIME      DA        None                   Set/read PK-232 internal clock

DCDCONN      DC        OFF                    RS-232 Pin 8 follows "CON" LED

DELETE       DEL       OFF                    Use BS ($08), not DEL ($7F)

DFROM        DF        Empty                  Digipeat call sign list

DWAIT        DW        16                     Delay for digi repeat (X10 ms)

ECHO         E         OFF                    Echo typed keyboard characters

ESCAPE       ES        OFF                    Send ESC character $1B as $24

FLOW         F         ON                     Stop echo to screen while typing

FRACK        FR        4                      Time (X1 sec) to wait for ACK

FULLDUP      FU        OFF                    Full-Duplex terminal operation

HEADERLN     HE        ON                     Insert <CR> after headers

HID          HI        OFF                    Send ID UI packet every 9.5 mins

HOST         HOST      OFF                    Host/terminal interface

HPOLL        HPOLL     OFF                    Host poll, packet mode

ILFPACK      IL        OFF                    Ignore linefeeds in packet xmit

Page 14

MAXFRAME     MAX       7                      Maximum un-ACK'd frames allowed

MBX          MB        none                   Monitor channel without headers

MCON         MC        0 (zero)               Monitor while connected (0-6)

MDIGI        MD        OFF                    Monitor digipeated frames

MFILTER      MFI       $80                    Filter received characters

MFROM        MF        Empty                  Monitor MFROM call signs

MONITOR      M         4                      Monitor mode level select (0-6)

MRPT         MR        ON                     Show digipeaters in headers

MSTAMP       MS        OFF                    Time-stamp monitored frames

MTO          MT        Empty                  Monitor MTO call signs

MYALIAS      MYA       none                   Alternate MYCALL

MYALTCAL     MYALT     Empty                  Alternate AMTOR SELCAL

MYCALL       MY        PK232; enter yours     Your call sign for packet adrs

NEWMODE      NE        ON                     Return to Command mode at DISC

NOMODE       NO        OFF                    Never switch modes

NUCR         NU        OFF                    Nulls to terminal after <CR>

NULF         NUL       OFF                    Nulls to terminal after <LF>

NULLS        NULL      0 (zero)               Number of nulls for NUCR & NULF

PACLEN       PACL      0                      # of user-typed bytes in packet

PACTIME      PACT      AFTER 10               Packet transmit timer (X100 ms)

PARITY       PAR       0                      Parity, none/odd/none/even

PASS         PAS       $16 <CTRL-V>           Pass input editing character

PASSALL      PASSA     OFF                    Ignore CRC in HDLC ("Junk Mode")

RAWHDLC      RAW       NO                     Raw HDLC Mode

RECEIVE      REC       $04 <CTRL-D>           Receive-mode character in text

REDISPLA     RED       $12 <CTRL-R>           Re-display current input buffer

RELINK       REL       OFF                    Reconnect after retrying out

RESET        RESET     Immediate Command      RESET bbRAM to factory defaults

RESPTIME     RES       10                     Minimum delay before sending ACK

RESTART      RESTART   Immediate Command      Same as power-on/off reset

RETRY        RE        10                     Maximum number of frame repeats

RXREV        RXR       OFF                    Reverse received data sense

SENDPAC      SE        $0D <CTRL-M>           Character that "sends" a packet

SQUELCH      SQ        OFF                    Select receiver squelch polarity

START        STA       $11 <CTRL-Q>           Resume sending data to terminal

STOP         STO       $13 <CTRL-S>           Stop sending data to terminal

TBAUD        TB        9600 baud              Set terminal data rate

TIME         TI        $14 <CTRL-T>           Insert "send time" in text

TRFLOW       TRF       OFF                    Terminal flow ctrl - Transparent

TRIES        TRI       0 (zero)               Show or force retry count

TXDELAY      TXD       75                     PTT key-to-data delay (X ms)

TXFLOW       TXF       ON                     PK232 flow control - Transparent

UNPROTO      U         CQ                     Path/address to send UI frames

USERS        US        1                      # of multi-connections allowed

USOS         USO       ON                     RTTY "unshift on space"

VHF          V         ON                     Packet - wide or narrow shift

XFLOW        XF        ON                     Software flow control

XMIT         X         Immediate Command      Start transmission - key PTT

XMITOK       XMITO     ON                     PTT line can be keyed

XOFF         XO        $13 <CTRL-S>           Stop sending data to terminal

XON          XON       $11 <CTRL-Q>           Resume sending data to terminal

Page 15

TNC-PARMS for the KANTRONICS TNC's.

8BITCONV ON

ABAUD    9600

ALIAS    OFF

AUTOLF   ON

AXDELAY  0

AXHANG   0

BBSMSGS  ON

BKONDEL  OFF

CANLINE  $18

CANPAC   $19

CCITT    OFF

CHECK    24

CMDTIME  1

CMSG     ON

COMMAND  $03

CONLIST  OFF

CONMODE  CONVERS

CONOK    OFF

CPACTIME OFF

CR       ON

CSTAMP   OFF

DAYUSA   ON

DBLDISC  OFF

DELETE   $08

DIGIPEAT OFF

DWAIT    1

ECHO     OFF

ESCAPE   OFF

EXCARDET OFF

FLOW     ON

FILTER   ON

FRACK    5

FULLDUP  OFF

HEADERLN ON

HID      OFF

KISSMODE OFF

KNTIMER  3

LCOK     ON

LCSTREAM ON

LFADD    OFF

Page 16

MALL     ON

MAXFRAME 7

MAXUSERS 1

MCON     OFF

MCOM     ON

MODEMENA OFF

MRESP    ON

MRPT     ON

MSTAMP   OFF

NDMON    OFF

NEWMODE  ON

NOMODE   OFF

NUCR     0

NULF     0

PACLEN   0

PACTIME  AFTER 1

PARITY   4

PASS     $16

PASSALL  OFF

PERSIST  255

PID      OFF

REDISPLA $12

RELINK   OFF

RESPTIME 1

RETRY    10

RNRTIME  0

SCREENL  0

SENDPAC  $0D

SLOTTIME 1

START    $11

STATSHRT OFF

STOP     $13

STREAMSW $00

STREAMCA OFF

STREAMDB OFF

STREAMEV OFF

SUPLIST  BOTH

SWDETENA OFF

TRFLOW   OFF

TRIES    0

TXDELAY  75

TXFLOW   ON

USERS    1

XFLOW    ON

XMITOK   ON

XOFF     $13

XON      $11

Page 17                 Latest improvements:

*** CHANGE..EXE, Version 1.0.4. ***

CHANGE will now provide FULL maintenance of your PBBS and is part

of the PACKTALK.EXE file for versions 104 and higher. (no longer seperate).

To change the TO or AT fields you need to create 2 files for the

\BBS\MFWD dir. CHANGE.TO and CHANGE.AT. Here is a sample CHANGE.TO file:

old    new  <----> (not part of the file).

SAT   -AMSAT

4SALE -SALE

WANT  -WANTED

NOTE: The first 6 characters MUST be used even if the entry is not

6 characters long. The - will ALWAYS be the 7th character. This applies to

both files.

On every day then ends with the number 1 or 5 CHANGE will repair and

remove dupes from the ADDRESS files in the \BBS\MFWD directory and every so

often will ARCHIVE the backed up messages that are stored there.

If you look at your LIMIT.MSG file, which CHANGE reads every time it

is activated, you will see a new entry.  The last entry will be a number.

The default is 250. This number represents the number of messages CHANGE

will allow in the \MSG directory. If, for example, you have the number set

to 50, and you have 100 messages on your BBS, CHANGE will delete the first

50 messages on the BBs, leaving you with 50 messages. The highest number of

messages permitted in the \MSG directory is about 508. This is dictated by

MS-DOS which will not allow more than 512 files in a directory (hard disk),

for floppy disk the max number is 112.

CHANGE will not affect any messages that have not been forwarded or

any messages with your call sign in the TO, AT or FROM field.

IF YOU MANUALLY RUN CHANGE AND THE MESSAGE COUNT DOES NOT EXCEED THE LIMIT

YOU HAVE PLACED IN THE LIMIT.MSG FILE BY 3, CHANGE WILL NOT DO ANYTHING.

You may PROTECT any message from being killed by CHANGE by entering

a * after the date in the MESSAGE.DAT file for the message you want to

protect. ANY message with your call in it will not be killed by CHANGE.

With these new additions to CHANGE you do not have to do any

maintenance to your PBBS at all except when your last message number gets

to 99999. Then you must renumber your messages with the RENUM.EXE pgm.

This new version of CHANGE.EXE still does what the older version

does and also takes care of the ADDRS.BBS and ADDRESS.BBS files such as

removing bad addresses and removing dupes, etc.

Page 18                Latest improvements:

Here are some of the new features for PACKTALK-104.

From the PBBS if you enter a * you can get a count of the messages

on the PBBS.

All bulletins when read by a user will now be tagged at the end of

the message. The tag will tell the user how to address any replies to the

bulletin. If a bulletin does not contain a callsign in the BID number assigned

to it the message will not be tagged, however the PATH: line is still given

and a user can glean from that how to address a reply. Messages addressed to

ARRL or from ARRL for example do not contain a call sign in the BID number

so these types of messages will not be tagged. There is no need for a user

to manually address a message with a 'H' address, however if it should

become necessary he can do so and the 'H' address will be placed in the

ADDRS.BBS file for future use.

In the PBBS ready mode if there is no activity after 30 seconds the

screen will clear. You can get the screen back by pressing F3 or if any com

port activity takes place the screen will come back. This is only active

when you are in the PBBS READY mode.

The screen buffer has been enhanced and you can save the screen

buffer to disk by pressing the END key from the TERMINAL mode. If your in

the PBBS ready mode and you press the END key the screen buffer will be

displayed but not saved to disk. The screen buffer is maintained even when

someone connects to your PBBS. The SCREEN buffer is maintained at about

20k at all times and is kept in the \PACKTALK directory.

The STATS feature keeps a record of who connects and what commands

they issue to the PBBS. The STATS file is renewed at midnight when the date

changes. From the TERMINAL mode, if you press ALT-X, you can view the STATS

file. The STATS file can also be download remotely by sending the command

STATS to the PBBS.

Page 19                  Latest improvements:

If you have any problems, refer to the PACKTALK.DOC file, if you

can't resolve it then let me know...  N3DFD @ N3DFD.#PHILA.PA.USA.NA

You no longer have to update any ASCII directories for your user

directories when you add new files in them for download.  When ever a user

uses the W command, PT-104 creates a new ASCII directory before the dir-

ectory is accessed, however it does not SORT the directory automatically so

it's still a good idea to run the SORTBBS program every now and then.

For 'H' routing of mail edit the STATES.ABB file in the \BBS\MFWD

directory. Place 1 state abbreviation per line. ie;

CA

NY

OH

NJ

Then in the MFWD.BBS file you make the entries you need, ie; .CA,

.NY, .NJ, etc...

HOLDING A MESSAGE: If you desire to hold certain messages from being

forwarded edit the MESSAGE.HLD file. On a SINGLE LINE only, you can place

a callsign, the TO or AT field string. When PT-104 tries to forward this

message, instead of forwarding it, it will put the message number in a file

named HOLDING.MSG. If at a later date you want to forward the message just

kill the MESSAGE.HLD file and place the message number in the file of the

BBS you want to forward it to. (\BBS\MFR directory). If you have messages

that have been put on hold you will see an * preceding the MAIL NOT ON

HOLD prompt at the bottom of the screen when you are in the PBBS READY mode.

PURGING THE BBS...

If you ever desire to start your PBBS 'new' again, go into the BBS

and use the K command to kill all the messages. You do this by typing:

K 0-x , x being 1 number higher than the number of the last message on the

board. Then send a B to leave the BBS. You will then see a prompt come up

on the screen that says, Purged BBS. This action will set everything pretty

much back to the way it was the first time you ran the BBS. This action will

not effect any user directories or the ADDRESS files. You can also use the

PURGE command.

The new CALLBOOK feature for the PBBS. PACKTALK compiles a list of

'H' addresses for any user or PBBS that it gleans from the R: headers sent

with every message that is forwarded through the BBS. PACKTALK will correctly

address a message even if the user doesn't. A user can look up anybody's 'H'

address by using the CB command and following the prompts. These addresses

are kept in the ADDRES.BBS and ADDRESS.BBS files in the \BBS\MFWD directory.

Page 20

=============================================================================

Transferring BINARY files

=============================================================================

BINARY files can only be transferd between PACKTALK, BACKTALK  and

the new LAP-TOP terminal software! (released in NOVEMBER of 1991).

When the receiving station sees that the file extension name ends in

.EXE, .COM or .ARC, it knows this will be a BINARY transfer and will act

accordingly. The same is true for the sending station.

Software FLOW control is used in these transfers, just as with an

ASCII transfer so the reliability of the BINARY transfer should be as good

as an ASCII transfer.

Doing a BINARY transfer using PACKTALK or BACKTALK is as easy as

doing an ASCII transfer but the files will be much shorter since the files

do not have to be converted to ASCII before transfer.

Unlike previous versions of PACKTALK, version 1.0.4 runs in the

TRANSPARENT mode, (BBS only). The terminal section of the program still

runs from the cmd: mode as does the MAILFORWARDING routines.

The only difference between version 1.0.3 and 1.0.4 is that 1.0.4

can do BINARY transfers.

Critical TNC parameters for version 1.0.4 are as follows:

PARITY 4  (for Kantronics, 0 for others) = no parity.

CMDT 1

PACTIME E 30

TRFLOW OFF  all other FLOW parameters must be set to ON.

PACKTALK, version 1.0.4 should open Com ports with NO PARITY. This

is determined by the setting in the PBBS.CNF file.

If TNC's PARITY or the Com ports PARITY are set to anything other

than NONE then a BINARY transfer will not be possible.

All commands on the PBBS generally consist of a letter a space

and then the qualifier and then a Carriage Return or on some computers the

Return key. eg.   N Joe<C/R)    will place your name into the system as Joe.

For most of the commands if you send just the letter without any qualifier,

you will get an explanation back on the use of the command. Sending the

letter D plus <C/R> for example you will get back from the PBBS;

Use: D directoryname\filename.ext

Page 21

Summary of COMMANDS on BBS:

[B] ---------- to LOG off from BBS. (DISCONNECT). Sending the letter B plus

a carriage return will disconnect you from the PBBS.

[CB] --------- this BBS keeps a running log of 'H' addresses of anyone

who's bulletin or message passes through it. If you want to

send a message to someone and you don't know what BBS they

get their mail on, this feature may help. It will give you

the BBS's call plus the 'H' address for it. The file that

holds this information is allowed to grow to a half a million

bytes before it is reduced.

[? or H] ----- to get HELP, these instructions. Sending the ? or the letter

H will get you the Summary of commands list. (short version).

[HA] --------- to get instructions on how to address mail to be forwarded.

[F] ---------- to FIND any file on the PBBS. Send an F to the BBS and then at

the prompt you get back you can enter the first name of the

file you are looking for, eg., MORSE, MORS, MOR, CODE, COD,

etc. The shorter the string you enter the less selective the

search will be. If you entered MORSE you would get back a

listing of all the files on the BBS that has the string MORSE

in it and what directory it is in.

[L] ---------- to LIST LAST 20 messages on BBS. Sending just the letter L

will list only the last 20 messages on the PBBS. To list more

than that you can use LL #, L>, L<, LB or L ALL.

[LB] --------- to LIST only msgs. addressed to ALL. Sending LB to the PBBS

will get you a list of all the messages addressed to ALL only.

[L ALL] ------ to LIST ALL the msgs. on the BBS. Sending L ALL will get you

a list of every message on the PBBS except those that were

sent as private mail with the SP command.

[L>] --------- to FIND mail addressed to you.  L> your callsign. Use this to

find a particular message or group of messages, for example,

if you saw in the BEACON that there was mail on the PBBS for

N3XYZ OR SALE, you can list only mail addressed to them by

the L> command. eg.   L> SALE   will get you a list of all the

mail addressed to SALE and nothing else.

[L<] --------- to FIND mail addressed from you.  L< your callsign.

This command is just the opposite of the L> command, this one

will find and list mail from someone rather than to someone.

if for example you wish to see how many messages are on the

PBBS from you, you would send   L< yourcallsign.

Page 22

[LC] --------- to LIST messages by category, eg., < LC DX, LC ARRL >, etc,

This command will search the TO and @BBS fields and also

the SUBJECT field for the STRING you enter for search.

[LM] --------- to LIST msgs. addressed to you. If you have mail on the BBS,

when you first connect to the system,  you will be notified

you do. It is only at this time you can use the following:

LM, RM, KM. These commands eliminate the need for you to deal

with message numbers at this point.

[LL #] ------- to LIST # of messages. This command will list the number of

messages you desire, eg.    LL 7    will list the last 7 msgs.

[LT] --------- to LIST messages for TODAY only.

[I] ---------- to read INFOrmation file about this station. Sending a letter

I to the system will download to you the Information file con-

taining all the pertinent information about the PBBS system

and radio station.

[N] ---------- to put your NAME into the system. <N yourname>. This command

puts your name in the system permanently so that the system

may address you by name.

[NH] --------- to Enter your HOME PBBS into the system. This puts your home

PBBS in the system files so the sysop will know where to notify

you in the event you have mail here or he can forward it if

need be.

[R] ---------- to READ a message, <R messagenumber>, <R 5>.  You may also

stack this command, for example if you wanted to read more

than 1 message you could send: <R 2 234 236 305>. This syntax

would result in the BBS sending you message 2, 234, 236 and

305 with just the one command.

[RM] --------- to READ your message(s). This applies only if you have mail on

the system. RM will read ALL your messages.

[S] ---------- to SEND a msg to someone. <S callsign>, <S N3XYZ>.

[SP] --------- to SEND PRIVATE MAIL.  Use this command to send mail to

someone that you want to be private. It will be only able to

read by the sender or the person it is addressed to.

[S ALL] ------ to SEND a message or bulletin addressed to ALL. <S ALL>.

[S xxx @ xxx]  to SEND a message or bulletin to another PBBS or USER at

another PBBS. eg., <SB ALL @ ALLBBS>, <S N3ABC @ W3XYZ>.

Page 23

[JH] --------- to SHOW LOG.(Who has connected to BBS).

[J] ---------- to show what stations the TNC has heard.

[G] ---------- to access the Gateway if it is available.

[KM] --------- to KILL your message(s). Kills ALL mail addressed to you.

[W] ---------- to see WHAT directories/files are on the PBBS.  Files of info-

rmation and computer software are stored on the disk in SUB-

DIRectories. When you send the command W to the system, what

you get back is a list of what's in the various directories.

You may see files there and also SUB-DIRectories. A file will

have a name like, BBS-CMDS.DOC followed by its size in bytes.

If you see a name like BASIC <DIR>. then you know that BASIC

is a SUB-DIRectory and in it are probably computer pgms written

in BASIC. If you wanted to download any files that might be

there, you must first be able to see what's there and you may

do this by sending the command  W BASIC  and this will show you

what's in the BASIC DIRectory. then if you wish you may down

load any file you see there by sending the following command:

D BASIC\MINITERM.BAS, if indeed that is the file or pgm you

wanted.

[WH] --------- to find some ones Name, Home PBBS and when they last logged

into the PBBS.  <WH callsign>.

[D] ---------- to DOWNLOAD a file. (D dir\filename.ext).(CTRL-Z to ABORT).

After starting a download you may Abort it at any time by the

sending of the CTRL-Z character. Upon the PBBS receiving the

CTRL-Z the pbbs will immediately stop sending data to the TNC.

However there may be up to 3k of data still left in the TNC

that will be sent to you so you won't see an immediate cease

of incoming data, but be assured it will stop as soon as the

TNC has purged itself. It's rather like trying to stop a woman

from completing a tirade in mid stream, it'll never happen

unless your prepared to shoot her.  BINARY downloads can ONLY

be done if you are using BACKTALK-100 or PACKTALK, version

1.0.4 and the filename extension ends in .EXE, .COM or .ARC.

[V] ---------- to show what VERSION of software and author.

Page 24

[U] ---------- to UPLOAD a file. (U dir\filename.ext). You can upload ASCII

or BINARY files to this system. Just do the same as you would

for a download except you replace the D with a U, for example,

if you wanted to upload a pgm to the BASIC directory and the

programs name was MINITERM.BAS then you would type the fol-

lowing:  U BASIC\MINITERM.BAS.  BINARY uploads can ONLY be

done if you are using BACKTALK-100 or PACKTALK, ver. 1.0.4.

and as long as the filename extension ends in .EXE, .COM or

.ARC.

[T] ---------- to TALK to SYSOP. Sending a T to the PBBS sets in motion a

series of events that take place in the shack where the PBBS is

located. The PBBS pgm moves to an area in its pgm that will

allow whatever the sysop types on the keyboard to be sent to

the TNC for transmission and whatever comes in from you to be

read by him and having no effect on the PBBS. When you sent

that T it made the computer BEEP rather loudly to catch the

attention of the sleeping sysop, so if he comes back to you a

little disoriented don't be too concerned. If he does not reply

you will be put back to the PBBS in about 15 seconds.

[X] ---------- to get the command list for remote SYSOP's.

[CTRL-Z] ----- This control character is used for closing files and also to

ABORT any download, this also includes messages you are

reading.

=============================================================================

Page 25                   *** MESSAGE THIEF ***

=============================================================================

MT.EXE is the newest feature to be added to PACKTALK PBBS, version 1.0.6.

This program gives the PACKTALK PBBS software the ability to capture

messages being forwarded to another PBBS without PACKTALK having to be

connected to anybody. MT.EXE will capture ONLY bulletins and private messages

addressed to SYSOP.

MT.EXE utilizes the REJECT.MSG and BID.NUM files that were created for

PACKTALK so it can reject certain types of messages and also check for

duplicate messages.

There are 2 modes you can run this software in and what mode you run it

in is determined by the way you have the MT.CNF file set up. In mode 1, it

will be possible only to capture messages that do not have any missing

FRAMES. Mode 2 will allow capture of messages with missing frames, this is

not a recommended mode.

Messages that are captured with missing frames CAN NOT and MUST NOT be

forwarded back into the PACKET SYSTEM! Messages that are captured in mode 1

may be forwarded only if you have not changed the BID.NUM and the AT field.

The AT field CAN be changed to something else, determined by the MT.CNF

file.

Capturing messages in MODE 1 DOES NOT INSURE THAT THE MESSAGE WILL BE

AN EXACT COPY OF THE ORIGINAL MESSAGE so it is a good idea to check these

messages before forwarding them back into the SYSTEM. MT.EXE does a good

job of keeping the integrity of the messages captured but there can be

errors. Remember you are not connected to the station forwarding messages

so there is the chance your messages will not be error free although great

measures have been taken to prevent contamination of the message you are

'stealing' off the air.

MT.EXE can be set up to run automatically or manually, determined by the

MT.CNF file.

Following is a sample of the MT.CNF file:

ON

BUCKET

N3DFD

WB3EUF

Y

03

�Q�

###

Page 26                    Latest improvements:

The following is an explanation of each entry made in the MT.CNF file:

ON  <ON or OFF, this entry will determine whether MT.EXE will run manually

or automatically when running with PACKTALK-PBBS. ON = AUTO>.

BUCKET  <BUCKET can be any 6 character name or if you do not want to change

the AT filed, enter a * instead of a name>.

N3DFD    <YOUR callsign>.

WB3EUF   <call sign of the PBBS you want to forward captured messages to. If

you do not want to forward any messages then enter a * in place of

the callsign>.

Y   <Y or N entry determines whether you will capture messages with or with

out missing frames. (data).  Y = mode 1, N = mode 2>.

03  <hour to force an exit from MT.EXE. IF you do not wish to force an exit

at any given hour enter a * in place of the hour>.

10  <outstanding packets allowed before aborting>.

�Q�  <the escape sequence to exit the HOST mode, Kantronics>.

###  <the escape sequence to exit the HOST mode, Non Kantronics>.

When running MT.EXE in the automatic mode with PACKTALK, PACKTALK will

no longer transmit the BACK ON LINE prompt over the air.

The MT.CNF file MUST be placed in the \PACKTALK directory in order for

MT.EXE to run properly.

The main improvement to PT-109-E is that it will handle the SUB ports

of the Kantronics KAM and KPC-4. This is accomplished by streamswitching.

The STREAMSW parameter is set to $7E(~) for port 1 and $7C(|) for port 2.

and the stream will be 'A'. This means ~A for port 1 and |A for port 2.

To have PT forward to a station on the sub port of your TNC you must

enter the callsign of the PBBS in the SUBPORT.TNC file. Thats all there

is to it.

A callsign placed in the PORT2.BBS file will make PACKTALK access COM2

and a callsign in the SUBPORT.TNC file will access the sub-port of the KAM

or KPC-4.

NOTICE: When using the KPC-4 the TNC SUBPORTS may be reversed.  You will

know this if when you try to forward mail and PT tries to access the wrong

SUBPORT. If this is the case add the stations callsign you are trying to

forward mail to, to the SUBPORT.TNC file or delete the call from the file,

which ever works.

Page 27                    Latest improvements:

PACKTALK, version 109-H and higher will now allow operation on any com

port from 1 to 4. This is accomplished by editing the COMPORT.SET file.

The default settings for this file are as follows:

COM1:

COM2:

1

2

The file as is will asccess com1 and com2 if you have the PBBS.CNF

file set for 2 port operation or if set for 1 port operation it will

access only comport 1.

The following set up will access comports 2 and 3.

COM2:

COM3:

2

3

If you wanted to access only comport 2, you would have to change the

line in the PBBS.CNF file to read 'TNC=1' instead of 'TNC=2'. This tells

PACKTALK that you only wish the 1 comport. You can only access 2 comports.

Any ports from 1 to 4.

POLLING TNC MAILBOXES:

PACKTALK will not POLL TNC mailboxes automatically unless the PBBS

has mail for that particular mailbox. It will POLL the TNC mailbox when it

has delivered the mail. If you wish to have PACKTALK POLL a TNC mailbox,

you may do so by utilizing the OPTION.BAT file to have PT-109 POLL a TNC

mailbox once every 24 hours.  Do the following:

(1)  In the \PACKTALK directory create the following files:

MAIL.

callsign.MFD   <callsign = the call of the TNC mailbox, no SSID>.

In the MAIL. file place the word YES (in CAPS).

In the callsign.MFD file place the number 0

(2)   Place these lines in the OPTION.BAT file:

COPY MAIL. \BBS\MFR

COPY \BBS\MFR\callsign.MFD+callsign.MFD \BBS\MFR\callsign.MFD

NOTE: If you are forwwarding this TNC mailbox any bulletins the above steps

will not be neccessary as PT-109 will POLL it everytime it sends it

bulletins...

Page 28                      Latest improvements:

There is a new server program named MAKEFILE.EXE available for all

versions of PACKTALK-BBS. This program will be run from the OPTION.BAT file

which is accessed by PACKTALK once every 24 hours. What this program will

do is create FILES from your messages and install them in a new user dir-

ectory. When MAKEFILE is run it looks at the MESSAGE.DAT file and searches

for a 'string' that you named when you ran the program. For example, if you

wanted to make files out of all for SALE messages you would invoke the

server in the following fashion:

MAKEFILE SALE

MAKEFILE will now search each line in the MESSAGE.DAT file and if

it finds a match it will create a new user directory named SALE and then

copy all the messages to that directory that it finds a match for.  There

will NEVER be more than 50 messages allowed in these new directories at

any time. If there are more than 50 messages in the SALE directory, for

example, MAKEFILE will delete all the files in there before putting new

ones in there. This way you are assured of always having up to date files

in the directory. The search MAKEFILE makes includes the TO, FROM, AT and

SUBJECT fileds of each message if you format the command in this manner:

MAKEFILE 1 SALE

MAKEFILE.EXE can, like I said, be run from the OPTION.BAT file. To

do this just insert the filename + string on a line in the .BAT file. ie.,

MAKEFILE AMSAT     <searches the TO field only>.

MAKEFILE 1 AMSAT   <searches all fields>.

You can run this program any time you want to by shelling to DOS and

type the proper command or you can run it from the PBBS by typing at the

command prompt: SHELL MAKEFILE xxxxxx.  xxxxxx being the 'string' you wish

to search for.

REMEMBER, the new directory will always be the same name as the

'string' you search for. The files it creates will have the same name as the

'string' also with the extension of what ever the original message number

is. ie., SALE.345.

You can now, for PT-109J only, delete user directories from the

PBBS. If for example you wanted to delete the new SALE directory you would

simply access the PBBS and at the command prompt type: DD SALE.