��������������������������������������ͻ

�                                      �

�               R8 Tuner               �

�           Drake R8 to IBM            �

�       Tuner Interface Program        �

�                                      �

�                                      �

�                                      �

�                                      �

�             Version 2.0              �

� Copyright (c) 1992 by Mike Musielski �

��������������������������������������ͼ

Requirements:

-IBM 286 compatible computer or higher running DOS

3.3 or higher.

-COM1, 2, 3 or 4 available. This version makes

provision for adjusting the port to be used. Cable length is to

your preference and should be dependant on the amount of RF

coming off you computer.

-DRAKE R8 Receiver obviously.

Files Included in this package:

* R8_TUNER.EXE

* R8INIT.IN1

MEMLOADR.EXE

* QSL.LOG

R8_BBC.DAT

R8_SPAIN.DAT

R8_SRI.DAT

R8_RFI.DAT

R8_NEDR.DAT

R8_SWED.DAT

*Both the above files must be in the same directory. The others needn't but it is

recommended.

Disclaimer:

R8_Tuner is provided AS IS. The author specifically disclaims

any and all warrantees, expressed or implied, including fitness

for a particular purpose. Use this product at you own risk.

This software is copyrighted by the author and all rights are

reserved. The Drake R8 is manufactured by the R.L. Drake Company

of Miamisburg. Ohio. The author of this software is not affiliated

with the R.L. Drake Company and does not claim to be representing

the interests of that company.

Registration:

This product is intended as shareware (user supported

software). This concept means you can evaluate the

program on your own machine and determine if you want

to purchase it. It is not free software and it is not

in the public domain.  You may distribute

it freely. But we ask that you also distribute this readme

file with it when you do. If you find that it meets your

needs and would like to purchase this software and be placed

on our mailing list a donation to the project of $35.00 sent

to the author whose address is discretely provided below is

requested.

It is our intention to upgrade this product in the future

with a mouse driven interface, complete simple to use

Database manipulation, Multi-event timer and automated file

inport-export capabilities. This will be available only to registered users.

If you have any questions regarding this product or suggestions

about the future product, please contact the author at:

Compuserve -71600,614

U.S. Mail  -Mike Musielski

16 Academy Rd.

Madison, N.J.

07940

R8_TUNER  Ver2.0

File Descriptions:

R8_TUNER.EXE  -This is the main program. This is a standalone,

full screen, menu driven, front-end driver for the

Drake R8 communications receiver. Once initiated

the program is designed to take-over as the radio's

front panel for all operations except analog inputs

such as volume control, notch adjustment and passband

offset tuning. The program can be suspended revoking

front panel control back to the radio for such

operations as timer (to be implimented in a future

version), scan operation (also to be implemented

later) and minor adjustments to tweek in a DX signal.

R8INIT.IN1    -must be present in the same directory as R8_TUNER.EXE,

if it is not, the program will automatically select COM1.

Use ALT-C to change to the desired COM port.

If it is necessary to scroll past COM4 and return to

COM1, COM2 or COM3 the correct procedure is to scroll

to the desired COM port then press ALT-S to suspend

the program and then reset the COM port by pressing

the SPACEBAR to resume the program.

QSL.LOG       -This file is a straight ascii text file created

by the R8_TUNER.EXE program. The file is created

(if not already present in the same directory) or

appended whenever ALT-D is selected in the

R8_TUNER.EXE program. The formating of the text

appears below.

���������������������������������������������������������������������������ͻ

�                     R8 listing format for QSL.LOG                         �

���������������������������������������������������������������������������ĺ

�         �        �M �          � � � � �   � � � � �                      �

�         �        �E �          � � � � �   � � �N� �                      �

�         �        �M �   F      � � � � �   �M� �O�S�                      �

�         �        �  �   R      �V�A� �A�   �O� �T�Y�                      �

�         �        �C �   E      �F�N�R�G�   �D�N�C�N�                      �

�  DATE   �  TIME  �H �   Q      �O�T�F�C�BW �E�B�H�C� PRGM NOTES           �

���������������������������������������������������������������������������ͼ

12-23-1991�16:44:48�14� 15.26000*�B�2�P�O�6.0�A�O�O�F�bbcnews program

12-23-1991�17:12:41�28� 11.62000*�A�1�A�S�4.0�C�F�F�O�SRI to n. euro-dateline

12-23-1991�18:33:30�99� 17.08000*�B�2�O�F�2.3�L�O�O�F�

������������������������������������������������������������������������������

Whenever Alt-D is pressed you will be prompted for a note to add to the

end of the current radios settings in the output log. After adding your

note, pressing ENTER will add, or create and add, the complete radio setup

and your note to the file QSL.LOG. TIP: Rename this file with a unique

name after each days sessions and build a database. More on this later.

MEMLOADR.EXE  -Is a stand alone program. When initiated, you are prompted

for a filename to load. This file is a straight ASCII text

file formatted exactly like the output file from R8_TUNER.

It will be search for a + sign immediately following the

second memory channel number. Any listings with this

charecter in this exact position will be loaded into the

radio's memory. The program first scans the input file

for this designator. Loads all such marked listings into

a buffer and counts them. If it finds exactly 100 it will

load the radio's memory channels sequentially. If there are

less than 100 listings marked, the program will provide you

with the number marked, prompting for which radio channel

to start the sequentially load from. If you inadvertantly

give it a number which when added to the marked listing

count totals more than 100 the program will abort.

(The radio's first memory channel is 00 not 1.)

The listings marked R8_XXX.DAT are files which can

be used with MEMLOADR.EXE. They contain a basic radio

setup and all of the frequencies listed in the World

Radio and TV Handbook for 1992 for the various

stations. SRI is Swiss Radio International,

NEDR is Radio Nederland, RFI is Radio France International,

the BBC is of course the BBC and the rest are self

explanitory. A simple text editor such as QEDIT,

MULTIEDIT or even EDLIN can be used to modify these

files directly. Be sure to overtype and not insert.

It is VERY import that the data's positioning on the

line remain as is.

There should be no space before the start of the

first date number. One space between date and time,

time and memory channel, vfo and ant, ant and rf,

rf and agc, agc and bw, bw and mode, mode and nb,

nb and notch, notch and sync, sync and notes as

illustrated in the above example. This sounds

more difficult than it is. Actually use the

overtype mode and Replace the choices with the

new ones or in the case of making a selection

place a + immediately following the second number

of the memory channel. Make sure nothing moves on

that line when you do and save the file as a pure

text file. Once saved, from the dos prompt type

MEMLOADR and press ENTER the program will ask

for a filename and enter it just as you would

from the dos prompt, path and all. Just make

sure you do not overrun the highlighted box.

THE FOLLOWING CHOICES ARE AVAILABLE FOR EACH FIELD

��������������������������������������������������

MEM CH:   At this time and for the purposes of memory channel loading

this field has no meaning but must be a 2 place number.

(I.E. 01 and not just  1). A plus sign immediately following

the second memory channel number is used as a selector symbol

by the program memloadr.exe to stuff it to the radio.

FREQ.:    This can be changed but not its postion on the line. There

should always be two spaces between the last memory channel

number and the first number of the frequency. (One if a plus

sign is present and the frequency is above 10 MHz.)

If the frequency is less than 10 MHz a third space or a zero

can be used. It is important that the number be carried out

to five decimal places followed by either two spaces or an

asterisc and a space.

VFO:    A or B.

ANT:    1 or 2.

RF:     O(off), P(preamp) or A(attn).

AGC:    O(off), S(slow) or F(fast).

BW :    6.0, 4.0, 2.3, 1.8 or 0.5 (The decimal point is required)

MODE:   A(am), F(fm), C(cw), R(rtty), L(lsb) or U(usb).

NB:     O(off), N(narrow) or W(wide).

Notch:  F(off) or O(on) (sorry but with two positions... well it was late)

Synch:  F(off) or O(on).

Notes:  This field is just for your notes.

(When finished press a return and start on a new line. There

can be no blank lines in the file.)

�����������������������������������������������������������������������������ͻ

�                     R8TUNER.EXE OPERATION MANUAL                            �

�����������������������������������������������������������������������������ͼ

To START:  -the program type R8_TUNER and hit carriage return.

To END:    -exit program press ALT-X.

When the program starts, it looks for the file R8INIT.IN1. Not

finding this file in the current directory will cause the screen

to loadup with garble. To correct this hold down the ALT key while

pressing the letter C (ALT-C). Then press ALT-X to exit the program.

Which will create the file. Then restart the program. Pressing ALT-C

will scroll through the four available COM ports. Stop when the

correct port for your installation is displayed in the lower left

portion of the base screen.

Assuming you are now on the base screen with the correct COM port

displayed in the lower left portion, pressing any highlighted key

will open that menu. All the radio's functions can be controlled

in this manner except certain scan functions. The menus can

all be exited by pressing the escape key. The time and date displayed are

whichever your computor is set for (Local or GMT).

FUNCTIONS:

F   -Opens the frequency window and prompts for the frequency

desired. Enter only numbers in memgahertz. No period.

(I.E. 9.500 should be entered: 09500 or 0950000)

The number can be corrected by using the arrow keys or

the backspace key and reentering the number. Pressing

ENTER or RETURN stuffs the selected frequency to the radio.

R   -Opens the power menu. Select either O for on or F for OFF.

In all two selection menus the above will hold true.

M   -Opens the mode menu. Selections can be made by by pressing

just the highlighted letter (changing the radio immediately).

Or using the arrow keys (which waits for you to press RETURN

or ENTER before executing the command)

-Pressing ESC exits from this menu returning you to the base

screen with updated information.

W   -Opens the Bandwidth menu. Selections can be made the same way

as in the mode menu. Press ESC to exit.

G   -Opens the AGC menu. Selections can be made the same way as in

the mode menu. Press ESC to exit.

P   -Opens the RF menu. Selections can be made the same way as in

the mode menu. Press ESC to exit.

B   -Opens the Noise Blanker menu. Selections can be made the same

as in the mode menu.

--cont--

V   -Opens the VFO menu. Selections can be made the same

as in the mode menu.

A   -Opens the Antenna menu. Selections can be made the same

as in the mode menu.

S   -Whenever this is pressed the frequency step size will be

immediately changed.

(I.E. if the Frequency displayed on the radio is 15.07000

pressing this will change the display to 15.0700

pressing again will change the display to 15.070)

N   -Opens the Notch menu. Selections can be made the same

as in the mode menu.

Y   -Pressing this will immediately turn on the Synchro feature

on the radio.

ALT FUNCTIONS:

F1     -Help file.

ALT-C  -Changes the com port selected. Displays the current

choice in the lower left portion of the screen. Pressing

ALT-S to suspend the program operation and then pressing

the SPACEBAR to resume the program's control will usually

re-establish the link using the newly selected COM port.

If this doesn't work use ALT-X to exit the program and

re-enter the program normally.

ALT-M  -Puts you into memory mode allowing for scrolling

through all the current memory channels. The use of

U or the up arrow key increments the memory channel

on the radio. The use of D or the down arrow will

decrement the memory channel on the radio. Pressing

the letter G while in this mode will prompt for which

channel to GO to directly upon the pressing of the

Enter or Return key.

ALT-D  -Opens a prompt window asking for any notes

pertaining to the current radio setup or program

being listened to before saving the record to the

file QSL.LOG. If QSL.LOG does not exist it will be

created in the current directory.

ALT-L  -Will open a prompt window asking for the channel you

would like to load the current radio setup into.

ALT-S  -Will suspend the program allowing you to manually

tweek the radio. After you are finished pressing the

spacebar will resume the program and refresh the

screen with the current radio information.

ALT-X  -Exits the program and closes all open files and ports.

�����������������������������������������������������������������������������ͻ

�                     MEMLOADR.EXE Program Manual                             �

�����������������������������������������������������������������������������ͼ

Discription:

This is a stand alone program designed to allow the radio's

memory channels to be automatically loaded from an input file

provided by the user. It will prompt for the path and filename.

Unpredictable results may occur if the maximum length is exceeded.

If the file requested is not present, the program will terminate

normally with the message:

Cannot open the file

PROGRAM ABORTED NORMALLY

If the file is present but more than 500 lines the program will

terminate normally with the message:

PROGRAM TERMINATED DUE TO MEM_CH.DTA HAVING MORE THAN 500 LINES

If the file has less than 500 lines the file will be loaded and

checked. If a + sign is present next to the memory channel number

the line will be prepared for loading into the radio. A counter

will keep track of the number of selections. If the number is the

maximum the receiver can handle, (100 selections), the program will

automatically load the radio beginning with channel number 00 and

continue until all 100 channels have been loaded. While the

loading is in progress you will be reminded by a flashing White

on Red window reading:

THE MEMORY CHANNELS ARE BEING LOADED

Please wait until the radio stops updating

then PRESS ANY KEY TO CONTINUE

Example of correct input format:

12-23-1991 17:12:41 28+ 11.62000* A 1 A S 4.0 C F F O

(notes field is not read and is optional

On the conclusion of reading the input file you will be prompted

for where to start loading the memory channels on the radio.

(Provided 100 channels were not selected). Enter the channel

number. This will be the first channel loaded and the program

will sequentially load the selected programs.

If you inadvertantly provide a channel number which when added

with the number of programs waiting to be loaded exceeds the

100 channel maximum the program will abort normally with a

window reading:

DATA OVERRUN

UNABLE TO LOAD CHANNELS

TIPS:

Use a ASCII text editor such as QEDIT or MULTIEDIT to edit

the QSL.LOG file to keep just those programs likely to be

loaded into memory and save the file under unique names.

Such as all the frequencys and radio setups you use for

the BBC could be save in a file named BBC.LOG then when you

wanted to load the memorys with BBC frequencys just edit

the file with an ASCII text editor of choice adding a + sign

next to the memory channel number. Go to the end of the

file and look at the line count for the file. Make sure this

isn't larger than 500 lines. A separate file of channels and

setups can be kept individually for all the frequencys and their

associated radio setups in uniquely named files.

When wanting these in memory just rename it to preserve the

original files add the + signs for the active channels and

load it. The program will prompt you for where to start the

sequential load.

Also rename qsl.log at the end of a day's sessions so each is

a unique name. I.E.:

r8122391.log for the qsl.log for dec. 23, 1991