T R T Y   4 0 0 0 . 1

for ZENITH Z-100 and IBM COMPUTER SYSTEMS

BY SKYLINE ASSOCIATES

JANUARY 1ST, 1992

The  TRTY.EXE  program  is  a  split-screen,  dual-port,  Amateur  Radio

communication control program.  The  specifications  and  the  program  were

developed  after many years of Amateur Radio Teletype operational experience

on the Heath Computer Amateur Radio Teletype Net (14082.5 mcs, mark).

Features of TRTY.EXE 4000.1

Supports two serial ports simultaneously,  in  half  or  full  duplex.

Control   AMTOR,   PACKET   or  RTTY  communications,  in a single or

any  double combination.

Supports BAUDOT, with 4 shift levels, and  7  or  8  bit  ASCII.  Baud

rates 45 to 9600.

Software and Hardware handshaking.

Colors may be set for all text, background, and window borders.

Separate Radio call signs for each session.

Beep on recognition of call sign, and upon receiving an Alert Bell call.

Remote station may retrieve messages from a 'MailBox' of a station.

Operator  may  scroll through text previously shown but rolled off the

screen.

Operator may resend a received message, across bands, or modes.

Operator  may  edit  text  in  one window while monitoring one or both

ports in other windows.

Operator may search a window for a 'character string' in a window.

Same program runs on IBM PC or ZENITH Z100, and Z100 with the ZPC  IBM

emulator. The ZENITH requires 768k RAM.

Program  may  be  configured  for  special requirements of Packet and

Amtor.

Software squelch programmable, with a 'DICTIONARY' type lookup.

MILITARY AMATEUR RADIO SERVICE (MARS) protocol requirements possible.

... . TRTY OPERATIONS MANUAL - Page 2

CONTENTS                                         pages/s

24/25th LINE . . . . . . . . . . . . . . . . .    6

3RD AND 4TH LEVEL OPERATION. . . . . . . . . .   15

ALTERNATE PORT SETUP . . . . . . . . . . . . .   14

ANSWERS TO QUESTIONS . . . . . . . . . . . . .   33

AUTO 'SHUTOFF' OF THE TRANSMITTER. . . . . . .    4

BELL REMINDER. . . . . . . . . . . . . . . . .   17

BREAK MODE . . . . . . . . . . . . . . . . . .   15

CHANGE CONSTANTS MENU. . . . . . . . . . . . .    7

COLOR  CONFIGURATION . . . . . . . . . . . . .   19

CONFIGURATION MENU . . . . . . . . . . . . . .   19

CONFIGURING THE PROGRAM. . . . . . . . . . . .    5

DEMODULATOR TYPES  . . . . . . . . . . . . . .    3

DISK I/O MENU. . . . . . . . . . . . . . . . .   18

DUAL SCREENS . . . . . . . . . . . . . . . . .   17

EDITING TEXT . . . . . . . . . . . . . . . . .   27

END OF LINE SEQUENCES (EOL). . . . . . . . . .    8

ERROR INDICATIONS. . . . . . . . . . . . . . .   21

FANCY MARGIN . . . . . . . . . . . . . . . . .   16

FLOW CONTROL . . . . . . . . . . . . . . . . .   14

HERCULES MONITORS  . . . . . . . . . . . . . .   33

HELP MENU. . . . . . . . . . . . . . . . . . .   12

HOT KEYS . . . . . . . . . . . . . . . . . . .   12

IBM SYSTEM CONNECTIONS . . . . . . . . . . . .   29

KANTRONIC  KAM . . . . . . . . . . . . . . .   3/28

LOGS . . . . . . . . . . . . . . . . . . . .     13

MAILBOX  . . . . . . . . . . . . . . . . . 16/24/25

MASTER CONFIGURATION FILE. . . . . . . . . . .   30

MISCELLANEOUS OPTIONS MENU . . . . . . . . . .   15

PACKET/AMTOR OPERATION . . . . . . . . . . . .   27

PARTNER CALL . . . . . . . . . . . . . . . . .    8

PICTURES . . . . . . . . . . . . . . . . . . .   17

PK-232 . . . . . . . . . . . . . . . . . . .   3/28

PORTS  . . . . . . . . . . . . . . . . . . . . 3/13

PRINTER. . . . . . . . . . . . . . . . . . . .   17

PROGRAMMABLE STRINGS . . . . . . . . . . . . .   20

QUICK TRIAL OF THE PROGRAM . . . . . . . . . .    4

RECONFIGURING THE KEYBOARD . . . . . . . . . .   21

RELAY TIME OUT . . . . . . . . . . . . . . . .    8

RELAY. . . . . . . . . . . . . . . . . . . .  15/16

REMOTE CONTROL . . . . . . . . . . . . . . . .   24

REMOTE WRU FUNCTION. . . . . . . . . . . . . .   26

RS232 CONNECTIONS  . . . . . . . . . . . . .  28/29

SCREEN SAVER . . . . . . . . . . . . . . . . .   17

SEARCH STRING. . . . . . . . . . . . . . . . .   13

SENSITIVITY, software squelch  . . . . . . . .   11

SESSIONS, defined  . . . . . . . . . . . . . .    3

SET PORT MENU. . . . . . . . . . . . . . . . .   13

SHOW FILE USAGE MENU . . . . . . . . . . . . .   13

SOFTWARE SQUELCH . . . . . . . . . . . . . .  11/14

ST-5, ST-6 . . . . . . . . . . . . . . . . . . .  3

STARTUP DIDDLES. . . . . . . . . . . . . . . .    8

STRINGS  . . . . . . . . . . . . . . . . . . . 9/20

SYSTEM REQUIREMENTS. . . . . . . . . . . . . .   27

TIME ZONE and CALL SIGN. . . . . . . . . . . .    7

TNC UNITS  . . . . . . . . . . . . . . . . . .   28

TRTY PROGRAM OVERVIEW. . . . . . . . . . . . .    3

WIDTH, transmitted lines . . . . . . . . . . .    8

WINDOWS 3.0 . . .. . . . . . . . . . . . . . .   33

WINDOW MANAGEMENT MENU . . . . . . . . . . . .   18

WRU  . . . . . . . . . . . . . . . . . . . . . 9/16

ZENITH SYSTEMS . . . . . . . . . . . . . . . . 3/28

... .  TRTY  OPERATIONS MANUAL - Page 3

-:DISCLAIMER:-

Skyline  Associates  hereby  disclaims  all  warranties relating to this

product, whether  express  or  implied,  including  without  limitation  any

implied  warranties  of merchantability or fitness for a particular purpose.

Skyline  Associates  cannot  and  will  not  be  liable  for  any   special,

incidental,  consequential,  indirect or similar damages due to loss of data

or any other reason, even if Skyline Associates  has  been  advised  of  the

possibility  of such damages.  The user of this program bears all risk as to

the quality and performance of the software.

-:TRTY PROGRAM OVERVIEW:-

The  TRTY.EXE  program  is  an MSDOS program and was developed to fill a

need in the Amateur Radio Communication Hobby.  We tried to include as  many

of  the features that are needed for a basic Teletype Communication Program,

and then we added some. This program will function with either a ZENITH Z100

with at least 256k ram,or an IBM PC compatible  computer.   A  command  line

indicator  is  sometimes  needed  to  assist in this system recognition. The

program will also work with the ZPC emulator, on the  ZENITH  Z100  with  at

least  256k  ram.   A  color monitor is desirable, but not necessary for the

program.  Normal RTTY communication compatibility, with other mechanical and

electronic systems, has been maintained.

TRTY was  developed  primarily  to  control  Irv  Hoff's  ST-5  or  ST-6

MAINLINER  RTTY  demodulators.   It  of  course will control the HAL ST-6 or

ST-6000 copies of  Irv's  fine  demodulators.   Any  RTTY  demodulator  with

simuliar  specifications  will work just fine.  A later addition to TRTY was

the features needed to control the PK232 and the KAM type units.

Many  of  the advanced features of TRTY will only work with another TRTY

using system.  This program is not a trivial communication control  program,

and takes some thought and effort to properly configure for use. The default

configuration allows normal Baudot use at 45.5 baud. Only a simple choice of

Com Ports is needed to start communicating..

Each serial port on an IBM or Zenith Z100 system is  controlled  from  a

SESSION split-screen.  SESSION 1 controls Port Com1 or Port A, and SESSION 2

controls Port Com2 or Port B.  Each SESSION may be configured for Baud rate,

Baudot  or ASCII, half or full duplex, and many other configurable items, to

tailor the program to your needs and  communication  system.   Each  SESSION

screen consists of two windows, a DIALOG window to show incoming data, and a

PRETYPE window, to buffer the outgoing data. These windows can be configured

to  overlay,  or appear side by side, or stacked , in a three or four window

display called DUAL mode.  More on DUAL mode, later.

Typing errors are correctable in the pretype area, at any time, prior to

the characters being transmitted. An added feature is one that stops the out

put   of   characters  from  the  pretype,  15  characters  prior  to  being

transmitted. This allows for corrections of the last word or few words being

made.  The pretype will be emptied when a Ctrl-g or Ctrl-v is  entered  into

the pretype, finishing the transmission.

... .  TRTY  OPERATIONS MANUAL - Page 4

-:QUICK TRIAL OF THE PROGRAM;-

For  those  who are familiar with RTTY programs, and want to rush into a

first trial of TRTY, here are some basic directions. I  assume  you  have  a

demodulator unit attached, per connections given later in this document.

1.   Of course, backup your copy of TRTY.EXE.

2.   Call up the program, with TRTY.EXE and TRTY.HLP in the directory.

TRTY <cr>

3.   The first screen, the LOGO, will clear if you hit a key.

4.    The  next  screen  is  the  operating  screen,  and is for COM 1

operating at 45.5 baud of 60 WPM Baudot.

A dull black screen with white characters. Colors come later.

Push F10 to start with SESSION 2, COM 2.

5.   Type a message into the Pretype area, which is on the lower part of

the  screen.  I  assume  that  you have your control unit on proper

port.  Correct typing errors by  backspacing,  or  use  the  arrows

keys, to move the cursor, and make the correction.  The transmitted

characters  are  correctable  up  to  about  15 characters prior to

transmission.  The output will stop, and wait if the output catches

up to the input, with about 15 characters waiting. A  Ctrl-g  or  v

will complete the transmission to the end of the pretyped data.

6.   Hit Ctrl-X, (both keys at same time), and the message will  be sent

out to the Session port.   (Ctrl-X is a toggle, a  second  hit will

toggle the system back to 'RECEIVE)' The message will appear in the

upper window as each character is transmitted, with a 'GIMMICK' at

the left margin of each line, indicating 'Transmitted' lines.

7.   To  end the transmission, hit Ctrl-G, (sends NNNN's) and return to

the system.

8.   To view the 'HELP' file, hit SHIFT-F1.

9.   To return to the operation windows, hit F1 twice. If the HELP file

was not in the  directory, the system will BEEP, hit F1 three times

to return to the operations window.

10.   To 'EXIT' the program, at any time, hit Ctrl-C or ESC twice.

If  you  want  to  try Packet, Amtor, or ASCII communications, please be

patient, follow the configuration procedure, and then proceed. Now that you

have tried the program, got  it  to  work,  have  fun  configuring  all  the

possible features and colors. Enjoy the fun.

AUTO 'SHUTOFF' OF THE TRANSMITTER:-

The program has a timer that will  shut  off  the  transmitter,  if  the

program  is left in the 'Sending' or 'transmit' condition, for more than one

minute without input from Pretype.   This  is  intended  to  cut  off  stray

transmissions,  or  accidental  activation of the 'TRANSMIT' switch.  At the

end of the one minute inactivity, the station WRU (Who  aRe  yoU),  will  be

sent, and the transmitter turned off.

... .  TRTY  OPERATIONS MANUAL - Page 5

-:CONFIGURING THE PROGRAM:-

All  of the operator and system interfaces can/may/should be configured.

This configuration task is not difficult, so let's get started and learn  at

the  same  time.   Type  out this document file, TRTY.DOC, to facilitate the

configuration and first use of the program. A sample configuration  file  is

shown  at  the  end  of  this document. Comments are shown for help and your

information.  They are not needed nor used by the program.  If  you  include

such  comments  in your configuration file, they will be lost the first time

you save the file.

It  is  important to note that the editing of the HELP and CONFIGURATION

screens, from within TRTY, changes the screen, but does not change the files

or the TRTY program.  These two screens must be SAVED,  after  editing,  and

installed,  to  implement  the  changes.  The remainder of the configuration

menus are actually changing the TRTY program configuration variables.  These

changes must be saved in  the  TRTY.CFG  file,  (pushing  F4)  to  become  a

permanent part of the configuration file. I will point out this requirement,

as  we  progress  through  the  configuration menus. Nothing you can do will

change the TRTY.EXE file that exists on the default  device.   A  messed  up

configuration file can be deleted, and a new start made, at any time.

The  program  can  be  configured  completely with your ASCII editor, by

modifying the CONFIGURATION file.  Unless you are familiar with the program,

a more successful configuration will result by starting with the MAIN  menu,

from within TRTY.

One point of information, to exit the program for any reason hit CTRL- C

or  ESC  twice,  and  the  program will close all files, return the ports to

their original initialization and exit to the operating system.

First,  with  TRTY.EXE and TRTY.HLP file in the directory, call the file

TRTY <cr>.

In  most  cases  some 'WILD INTERRUPTS' will flash on the screen, as the

program determines the machine type.

The first configuration of the program will be the default configuration

from within the program. If the file TRTY.CFG exists on the drive, that will

be used to configure the program.   The  configuration  file  may  have  any

fname,  and  must be called on the COMMAND line.

In  ALL  cases  it  is  necessary  to  identify  the system type when the

ALTERNATE configuration file is called.  Add 'IBM', 'ZEN' or  'ZPC'  to  the

command line, and the fname of a configuration file if other than TRTY.CFG.

The use of this program with the ZPC (by Heathkit),  requires  that  the

command line have the form, 'TRTY ZPC [fname.cfg] <cr>'.

Call up the program, with any of the above  commands,  the  system  will

'BEEP'  if  no  configuration  file is present in the default directory. The

first window visible will be the LOGO of the TRTY.EXE program.  Hit any key,

to proceed to the operation screen.  The LOGO  will  still  flash,  even  if

configured as 'N' in the 'OPTIONS" menu. We do not want anyone to forget the

name  of this work of love.

... .  TRTY  OPERATIONS MANUAL - Page 6

For this first configuration discussion, I will assume that there was no

TRTY.CFG  file  in  the default drive/directory.

The second screen that pops up, is the  SESSION  1  split  window,  with

borders  around  the upper or DIALOG1 window, and the lower PRETYPE1 window.

At the bottom of the window,  on  the  24th  and  25th  lines  will  be  the

following:

_______________________________________________.............................

SESSION 1 Baud Rate 45.5 Bit Mode Baudot Log None Receiving  Relay SSxx08:00

F1=Setup 1995 Available Buffers EE   12 Lines   12  1   gggg  Pretype 1

_______________________________________________________________________|

' SESSION 1' means, this window is for control of PORT A, or Com 1.

PUSH F10 and the second window SESSION 2, will appear. Push F9,

and return to SESSION 1.

' Baud Rate 45.5 ' is the default baud rate.

' Mode Baudot ' means 5 bit Baudot code. Alternate setup shows in reverse

video. Hitting F3 Toggles the Alternate setup.

' Log None ' means that the logging function has not been set.

' Receiving' means the Port is set to receive. The default mode.

'  SSxx' Presence  means  the squelch  set Session 1and/or 2

The xx means XOFF HANDSHAKING has been received.

'08:00'  is system time.

The line of 'dots' indicates that the remote ALERT feature is set, a

callsign, or name here, indicates who made the ALERT call. Lower-

case 'x's indicates that the feature has been reset by CTRL-q.

On the 25th line:

'F1=Setup' means, Push F1 to get the SetUp Menu.

'1995 Available Buffers' means, number of line buffers available.

Both incoming and outgoing data is buffered

in RAM, for each Session.

'EE' Presence means, Parity and Framing errors in SESSION 1 and/or 2.

'12 Lines' means, number of lines in the buffer with the active cursor.

'12'  after 'Lines' means, line number of the active cursor.

'1'  means, character number at active cursor line.

'gggg' GIMMICK shows  when  data  is  incoming,  or outgoing, for each

SESSION.  A  flashing  cross for incoming, and a musical note

for outgoing.

'Pretype1' indicates where the active cursor is located. Hit F2 to

toggle active cursor into/out of Upper or DIALOG  window.

... .  TRTY  OPERATIONS MANUAL - Page 7

Push F1 to view the MAIN configuration menus.

-:MAIN MENU:-

____________________________________________

|                                 Main     |

|   1.  Help.                  (Shift-F1)  |

|   2.  Configure.             (Shift-F2)  |

|   3.  Disk I/O.              (Shift-F3)  |

|   4.  Window Management.     (Shift-F4)  |

|   5.  Miscellaneous Options. (Shift-F5)  |

|   6.  Port Setup.            (Shift-F6)  |

|   7.  Show file usage.       (Shift-F7)  |

|   8.  Change constant        (Shift-F8)  |

|                                          |

|       Select a function from this list.  |

|__________________________________________|

This is the Main Menu, and is the starting point to configure all of the

interface items and conditions. Screen colors, STRING  lines  and  'software

SQUELCH' dictionary 'WORD's are configured by editing the configuration file

itself. More on this file editing, later.

Pushing the number keys, 1-8 will pop  up  the  appropriate  menus.  The

right  hand  listing  of  Shift-f  keys,  are  the commands from the SESSION

windows, to go directly to the a specific menu, bypassing the MAIN menu.

Keep  in mind the differences between editing the HELP and CONFIGURATION

files, and editing the other direct operating menu items, proceed  with  the

configuration. Let's start, assuming you know little about the program, in a

way  that  will enhance your learning process. If in doubt as to whether the

changes are saved, push the indicated keys and save  the  configuration.  As

you  finish  configuring all the items in each menu, save the configuration.

Key F4 is usually the correct key.

-:CHANGE CONSTANTS MENU #8:-

Assuming  that  the  MAIN  menu  is  showing,  PUSH  number  8,  'CHANGE

CONSTANTS',  and  set  up  the  items  on  that  menu.  After completing the

editing, PUSH F4 to bring up the 'WRITE FILE' menu.  The  default  fname  is

'TRTY.CFG', but any fname.cfg may be used, or the configuration may be saved

in  each of several files. If 'TRTY.CFG' as a fname is appropriate hit <cr>,

and the configuration file will be saved.

TIME ZONE and CALL SIGN are the usual items and are self explanatory.

The  FANCY margin is the special character string that you might want to

have inserted at the start of each line of data, as you send messages.   Any

characters,  or  even  a  callsign  may  be used. The 'FANCY MARGIN' is made

active in Menu 5, item 5, by toggling the default 'N' to 'Y'.

If  you  would like to have lines you transmit start with your callsign,

hit key #3, and the EDIT CONSTANT submenu will appear.   The  default  fancy

margin  of  '-'  will be over the flashing cursor. Type in your call, W0xxx,

with a space of two following and hit the <cr>, the menu #8  will  reappear.

Set  the  item  in menu #5 to 'Y' and your call will be sent at the start of

each line sent. Any other string can be programmed as a fancy  margin,  your

choice. If no fancy margin is desired, don't set it to 'Y'.

... .  TRTY  OPERATIONS MANUAL - Page 8

CHANGE CONSTANTS MENU #8 continued:-

END OF LINE SEQUENCES (EOL):-

This controls the sequence sent whenever a new line is started from  the

transmitted  text.   It  is triggered by the RETURN key, automatic wrap of a

line being entered into the buffer, or by a NEWLINE (0AH, same as LINE FEED)

character from a disk input file.  Baudot allows CARRIAGE RETURN  (C),  LINE

FEED  (N), LETTERS (L), or FIGURES (F). ASCII allows CARRIAGE RETURN (C) and

LINE FEED (N). ASCII mode will ignore any other end of line characters.   If

left blank, the default sequence is CCN. After configuring the items in this

menu,  hit  F4,  to  save  the  configuration.   Follow  instructions on the

following  'EDIT  CONSTANTS' screen, replacing the default CCN with what you

need. The use of C,N,L and F are unique to the EOL sequences, do not use nor

confuse them with similarly named commands in the STRING setup. Sorry if it

is a cause of confusion.

RELAY TIME OUT:-

This  item  is  the  number of seconds that a message will be allowed to

remain in the Pretype, during a remote Relay. Usually set to twenty  minutes

(1200  seconds), limiting the total time for a relay to ten minutes receive,

and 10 minutes to resend the message. Default is one hour (3600 seconds).

STARTUP DIDDLES:-

This is set to the number of Startup Diddles, sent as the first line  of

a  transmission.  They  are Letters Characters, and are defaulted to 32, for

use by mechanical Teletype units to get synchronized.  Must be enough to open

the Autostart of users Demodulators. Not less than about 10 is recommended.

WIDTH:-

Sets the width of the transmitted line, can be  up  to  80,  but  72  is

recommended, mechanical Teletype units are limited to 72 character lines.

PARTNER CALL:-

Place the callsign or other string, that is  used  regularily,  in  this

menu  by  hitting  key  #9.   This  'PARTNER'  call  or  string, of up to 12

characters, can be called in a STRING with a  '\Z'  as  shown  in  STRING(Z)

number 26, on page 31 of this document.

During normal operation, the 'PARTNER CALL' can be modified  by  hitting

Ctrl-r, after  positioning  the cursor on or just after the word you want to

place in the PARTNER string. Everything back to the preceding blank  or  CR,

is put into the 'PARTNER' string (up to 12 characters).  If the cursor is at

the  start of a line or after two blanks the string is not inserted. You can

type a callsign of string, hit Ctrl-R, then  backspace  to  erase  the  word

before going on.  The word is moved to the 'PARTNER' string and is no longer

dependent on what is in the location that you got it from.

... .  TRTY  OPERATIONS MANUAL - Page 9

CHANGE CONSTANTS MENU #8 continued:-

USER PROGRAMMABLE STRINGS:-

The  default  Constants,  Start-Up  String,(item A), End Of Transmission

(item  B)  and  WRU  String  (item  C)  may  be  reconfigured  for   special

requirements,  such as Packet and Amtor. The default numbers 1, 2 and 5, are

for 'normal' RTTY operation.

These strings, 1-9, are in  addition  to  A-Z  for  each  session.   The

strings  are  fixed  and can not be changed, as they are reserved for use by

the program.

STRING           Value       Normal Use

______           _____       ___________________________________________

1                "\D"        Plays 'diddles' at startup. This gives the

same function as normal startup.

2                "\J"        Plays 'line-feed', before going to receive

from transmit.  This gives the CTRL-V.

3                "\$03\M"    Plays Control-C, then "X", then carriage

return.  Makes PK232 go to transmit mode.

4                "\$04"      Plays Control-D, makes PK232 go to receive.

5                "\J\U\C  \K\Y\J"   Plays  WRU (Who aRe yoU) message

followed by line feed.

6                "\J\U\C  \K\Y\J\R" Plays WRU message followed by Ctrl-V.

7 to 9          Reserved for program and future use.

From the "Constants Menu, #8" these or other programmable strings can be

chosen for the functions of Start-Up, End of  Transmission  (EOT),  or  WRU.

Configure  new  strings  for these functions in the "EDIT Config" menu, #2",

and call for that string in the setup in menu #8.  For  instance,  configure

STRING(x),  in  the  Session being configured, as follows, for all SPACES at

Start-Up.

STRING(x) "                                 "

Then in menu #8, edit in the  STRING  letter  given  to  the  above  new

Start-Up  string,  as the letter for Config Item "A".  This string will then

be sent in placed of the default Start-Up string #1.

Continue for the remainder of needed customization of the strings.

The string functions for these menu #8 items are:

\C place callsign of current SESSION here.

\D works only in the Start-Up String, and consist of blanks.

\G works only in Strings(a-z), just BEEPS here.

\J linefeed.

\K Puts "DISK" in string if copying to disk.

\M carriage return prior to next characters or words.

\R stop sending, same as Ctrl-V.

\U puts 'Time-stamp' in string (works in any string).

\Y puts "Relay" in string if RELAY MODE enabled.

... .  TRTY  OPERATIONS MANUAL - Page 10

CHANGE CONSTANTS MENU #8 continued:-

To assure a complete understanding of the above  strings  and  controls,

here are some additional points of information.

1.  'Startup' is played when Ctrl-X is hit while in the  'RECEIVE'  mode.

The  string  is sent to the connected unit before any data in the

pretype is sent. The default #1 is for normal RTTY, #3 for Packet.

2. 'EOT' string is sent when Ctrl-v is encountered in the pretype  buffer

while  transmitting,  to  allow  the command, ' return to receive', to

be sent to the TNC type  units.  (normally  the  'RETURN  TO  RECEIVE

AFTER EMPTYING  BUFFER' command  if  your TNC unit supports such a

command.) #2 for RTTY, #4 for Packet.

3. The  'WRU'  string  is  sent  (preceded by the 'STARTUP' string) when

Ctrl-W is hit on the keyboard or in response  to  a  remote  WRU

request (callsignZW,  :TIME,  :xDISK or any other place where the proper

response to a remote command is the WRU sequence).

4. The strings may be customized for your unit  commands  using  STRING(a)

through  (z)  configured as normal strings for the proper SESSION or you

may use strings #3 or #4, which are for the PK-232.

5. To disable the 'STARTUP' and 'EOT' strings, designate a 'NON-EXISTING'

STRING letter in the menu selection.

Note:-

In this document, the letters 'WRU' refer to the message that is shown

or  sent  in  response  to  a  remote  request of 'callsignZW'.  The message

normally consists of the 'TIME, DATE and Callsign' of  the  sending  station,

plus  the  words  'Disk and/or Relay'.  The latter two indicates the status of

the sending station. See the 'HELP' file for local key command to  send  the

'WRU'.

(remainder of page left blank for readability)

... .  TRTY  OPERATIONS MANUAL - Page 11

CHANGE CONSTANTS MENU #8 continued:-

SENSITIVITY ITEM D:-(part of Software Squelch)

Software squelch is provided by TRTY, if so configured, to overcome  the

limitations of TNC packet and Amtor units.  Some RTTY demodulators also have

limited   ability  to  open  or  close  the  Autostart  or  squelch,  for  a

signal/no-signal condition. Software Squelch mode is activated in  Menu  #5,

which will be done later, the main discussion will be done here.

Three methods of opening the Software Squelch are provided.

1)  by  receiving  a number (x) of  repetitive  characters.

2)  by  matching incoming characters or 'WORDs' to a listing of 'WORDs'

in a configurable dictionary in the configuration file.

3)  when switching  from 'Sending to 'Receiving', to allow fast break in.

The  Software  Squelch is toggled 'Y/N' in the Port Set-Up menu, #6. The

two  number  setting  for  the software  squelch is set in the menu #8D. The

following software squelch functions are only operational, if the  above  is

set to 'Y'.

SENSITIVITY:-

Hit key 'D', bringing up the Edit Constant menu, with 4   10 as default.

The first number is the number of received characters that  will  'open'

the  squelch.   If  set  to 4, then 4 diddles or 4 a's, or 4 b's, etc.  will

'open' the squelch.  The normal number of 'diddles' was  32,  in  'old  time

RTTY',   but   can   be  any number. Set this to 99, to effectively open the

Software Squelch only upon receiving WORDs in the squelch dictionary.

The  squelch  will  open when 'WORD' strings, configured in a dictionary

(case insensitive) added to  the  Configuration  file,  are  received.   The

strings may contain any number of characters up to 7. The squelch dictionary

will  accept  over  750 WORDS (more if short words).  The configuration file

would have the following type of entries for each word:

WORD "abcd"

WORD "W0NW"

WORD "Tom"

WORD "de"        (reverse slant C for 'preceded by a carriage return')

WORD "FROM"      (reverse slant N for 'preceded by a line feed')

The  default  dictionary, with software squelch, will have the following

additional 'WORDS' forced into it, by the program, if  you  do  not  include

them.

WORD "\Ccallsign"        (carriage return callsign)

WORD "\Ncallsign"        (linefeed callsign}

WORD "callsign"          (the above N and C are from the EOL command

characters)

This  is  done to assure that any remote command such as 'callsignZL' or

'callsignZW' will be recognized.

... .  TRTY  OPERATIONS MANUAL - Page 12

CHANGE CONSTANTS MENU #8 continued:-

The dictionary is usually added at the end of  the  configuration  menu,

following  the  last  item.  The  same  dictionary works for either session.

These 'WORDs' can be added from the configuration editing menu, or  with  an

ASCII editor.

Squelch  is  closed 4 methods:

1). lack  of signals for x seconds, set by SENSITIVITY's second number.

2). receipt of 4 N's at the end of a transmission.

3). by parity or framing errors on the port UART. The receipt of 4 or

more framing errors in 2 seconds  is  the  threshold used  in  TRTY.

4). One  last  squelch  closing  method  is  a SPACE signal of more than

one character length.  This program provides such a signal  as  the

command to' end  a  transmission'  (Ctrl-G  or Ctrl-V) is completed.

Parity errors are detected when the characters lack parity that conforms

to the configured parity of Menu #6.

Framing  errors  are  those  that are detected when a received character

does not have the  proper  number  of  start,  mark,  space  or  stop  bits.

Normally,  'open' squelch consists of garble that seldom has properly framed

characters, and thus is a good way to indicate a need to close the  squelch.

SENSITIVITY's  second  number is the number of seconds of 'no characters

received' that will 'close' the squelch, and stop printing to the monitor.

An 'E' shows on the 25th line when either a Parity or a Framing Error is

detected on a Session Port, one for each Session. These errors are  detected

and indicated, whether or not the Software Squelch is set for that Port. The

error indications are reset after 10 seconds.

'HOT KEYS' are provided for fast control of the software squelch.

For IBM type keyboards.     .

ALT-J toggles squelch mode enabling.

ALT-K opens the squelch.

ALT-L closes the squelch (if enabled).

For Zenith Z100 type keyboards.

Ctrl-K,J (two key sequence) toggles squelch mode enabling.

Ctrl-K,K (two key sequence) open the squelch.

Ctrl-K,L (two key sequence) sets the squelch (if enabled).

(on these, it makes no difference whether it is Ctrl-K then

Ctrl-L or Ctrl-J of Ctrl-K then J. The result is the same.

... .  TRTY  OPERATIONS MANUAL - Page 13

CHANGE CONSTANTS MENU #8 continued:-

SEARCH STRING:-

Item E in menu #8 is the "string" to be used in a search of  the  active

window.   Type a string here, such as W0NW, and then use Ctrl-Shift-"minus",

(Ctrl-Shift--) to find the string 'W0NW' in the buffer for that window  with

the active cursor. The minus is the keyboard '-' not the keypad '-'...

That  was some task, wasn't it?  Might get easier as we learn more about

the program.

-:SHOW FILE USAGE MENU #7:-

Item  7  is  a menu to show files open for reading and writing, for each

window.  This menu is seldom used, it is for information only.

Item 6 is the PORT configuration  menu,  push  number  6  and  continue.

-:SET PORT MENU #6:-

When items are selected in the PORT menu, the  possible  configurations

will change/toggle on the 24th line, indicating the new configuration. Prior

to a first configuration, the default for the Port is 'Receiving'.

If you want to use only one screen, or  Session,  of  the  program,  use

Session 1 for Com1 and Session 2 for Com2. The unconfigured Session will not

affect the other Com port. If you are configuring the program for DUAL-PORT,

configure  SESSION1  port,  then  hit  F10, and configure SESSION2. Note the

SESSION being configured will be shown on the 25th line.  F9  and  F10  will

toggle  between  sessions.   Switching sessions may be done at any time, and

only one F4, 'Save Configuration' will be needed.

When the 'Change Log Option' is first configured, a WRITE FILE menu will

appear.  No fname will be shown. Hitting <cr> will  force  a  default  fname

consisting  of  MONth, DAy and Session number (NOV29Sx) with an extension of

.001.  If you insert a fname of your choice, such as 'A:\MYLOG' ,do not  use

an  extension,  as  the  program will assign .001, and increment that as new

log files are created on that same day.

The default LOGNAME shown in the CONFIGURATION FILE will be  "*",  which

is  shorthand  for the MONth-DAy-Session fname.  If you want to log to other

than the default drive, type a '*' and then the drive letter, such  as  '*A'

(with  out  the quotes, or course), and the log will be written on drive A:,

with the fname of 'NOV29S1.00x'. No colon is needed after the drive name.

When  you  type the 'A' following the '*', the menu will blank out for a

moment, and then the SET PORT menu will  reappear.   Hit  F4,  to  save  the

configuration.   Any other fname can be shown as 'drive:\fname'. The size of

the log file is set directly by editing the CONFIGURATION file, itself,  and

is set by default at 5000 characters.

Dialog  Mode  should  be set to 'Receiving' for half-duplex BAUDOT.  For

ASCII communications set the mode to 'Full-Duplex'or 'Full-Echo'.

'Stop  Bits' default is 1.5 for BAUDOT, and 1 for ASCII. Change to other

possible settings by hitting key #6.

... .  TRTY  OPERATIONS MANUAL - Page 14

SET PORT MENU #6 continues:-

'Parity' should be set to the need of  the system. Parity can be set for

BAUDOT communications, but is normally set to 'NONE'.  For ASCII  data,  the

parity  can  be set to ODD or EVEN, and two additional settings of STUCK ODD

and STUCK EVEN. These last two settings mean that the parity bit is  set  to

either a binary 1 for ODD or a binary 0 for EVEN, and does not change.

Flow Control is set for one of the following types by toggling key #8:

None     No flow control.

Xon/Xoff Software flow control.

RTS/CTS  Hardware flow control.

DTR/DSR  Hardware flow control.

'Software  Squelch' should be toggled to 'Y', if the function is needed.

Hit key #9 to toggle from 'N' to 'Y'. A complete discussion of the  SOFTWARE

SQUELCH feature is available starting on page 9 of this document.

Only the Log option is common to both the ALTERNATE and PRIMARY Port.

Push F4 to Save the configuration.

ALTERNATE PORT SETUP:-

There are times when it is necessary or desirable to quickly  reset  the

mode  and  baud  rate  for  a  Session, shifting from 45.5 Baudot to 110 (or

whatever) ASCII. This could be to copy a signal that is not being copied  at

the   set   mode  and baud rate. TRTY provides an ALTERNATE PORT SETUP and a

'HOT' key to rapidly switch between two predetermined  conditions  for  each

SESSION..

If you wish to configure for an ALTERNATE PORT Mode and  baud  rate,  for

each  SESSION, push F3, and program an alternate setup, using the same items

in the menu.  The mode items will be shown in reverse  video.   The  program

has  this capability, maintaining an alternate setup for each port, callable

by F3. Make sure that the setup you desire for the normal start  up  of  the

program, is showing on the 24th line prior to saving the setup. Push F3, set

ALTERNATE  port  setup. Note that BAUDOT or ASCII is in reverse video.  Save

the port setup/s by pushing F4. Hit F3 to return  to  the  normal  operation

setup.

Push F4 to Save the configuration.

... .  TRTY  OPERATIONS MANUAL - Page 15

-:MISCELLANEOUS OPTIONS MENU #5:-

Bring  up  the  MAIN  menu,  hit  the  number  5, for the 'MISCELLANEOUS

OPTIONS' menu.

Some of the "MISCELLANEOUS OPTIONS' can be configured for  each  SESSION

and some are items applicable to both SESSIONS. Switch SESSIONS, if both are

being  configured,  and  save  with F4.  Always return to the SESSION screen

that you wish to be the default or start up  screen,  prior  to  saving  the

configuration because the start SESSION is part of the configuration record.

3RD AND 4TH LEVEL OPERATION:-

One  of  the  features  of  this  program  is  called "3rd and 4th level

BAUDOT". This is a programming method of  sending  and  receiving  the  full

ASCII  character  set  over  a BAUDOT channel. This program always transmits

with 3rd and 4th level characters, and is  always  available.   The  program

automatically  sends  a 'Blank-Figs', or a 'Blank-Ltrs', to make the 3rd and

4th shifts. Both sending and receiving stations must  be  using  this  or  a

compatible program, to show the ASCII, when sending BAUDOT.

Unlock  the  "CAPS  LOCK",  to send lower case data, to be received on a

system using this same TRTY program. The print on a TTY  machine,  or  on  a

monitor  (with  other  than  this  program),  will show as if it were normal

Baudot. Of course, normal ASCII at 100 WPM can also be used, if desired.

3rd and 4th level 'receive' is set as a configuration item  and  may  be

set on or off for each session.

BREAK MODE:-

Setting  item  2  to 'Y', shortens the startup 'DIDDLES' and changes the

CTRL-G, for end of transmission to '-BK-'  for  fast  break-in.   This  mode

should  only be used when you are indeed working 'fast-break-in'.  Many RTTY

demodulators will not open the squelch  on  the  short  number  of  start-up

diddles.

RELAY:-

TRTY  features  the  ability  to  control a remote TRTY using station to

retransmit a message to a third  or  more  stations.  This  mode  is  called

'RELAY'  mode,  and  is  set  ON by item 3 in the Options #5 menu.  The word

'RELAY' appears on the 24th line, when the mode is active.

In the "RELAY" mode, there is a timer  to  prevent  untransmitted  relay

messages  to linger in the Pre-type buffer.  This timer can be set up to one

hour in length, which prohibits any relay message longer than  1/2  the  set

time.  If  the  timer  expires,  the  relay message is cleared and the relay

function is armed for another usage.  The default time out is 3600  seconds,

one  hour,  which  may  be changed in the configuration record.  A time less

than 1200 seconds (twenty  minutes  total)  is  near  the  lower  limit  for

practical  use  of  the  relay  function. If a relay has been initiated by a

remote station, and the command to the station to execute the relay has  not

been received, this time out will clear the Pretype buffer, and allow normal

station operation. This timer is set in CONSTANTS MENU #8, item 6.

... .  TRTY  OPERATIONS MANUAL - Page 16

MISCELLANEOUS MENU #5 continued:-

NOTE: To use the relay system, follow these steps:

1.  Send  call  of  station  having a RTY or TRTY program operational

with the capital characters  ZC,  appended  to  the  call  of  the  relay

station.

callsignZC <ret> (puts relay station into RELAY MODE)

callsignZC <ret> (send a second time to assure reception)

2. Send normal message to station: "W0NW de W0SE etc."

3. Send <ret> then "four N's" and a <ret>. Do not use Ctrl-G.

4. Send call of station being used for relay, with ZR appended.

callsignZR (tells relay station to relay)

5. Hit CTRL-V to leave the air immediately

6. Watch relay being made.

WRU on CALLSIGN:-

Item 4 in this menu has a four-way toggle. Consecutive hits of the 4 key

will toggle the 'WRU on CallSign only' from Yes to No, and a second item, ZF

from ON to OFF.

If  set to "Y" , for each session, the WRU will not respond to the :DISK

of :TIME command, but only to callsignZW.

The 'ZF' is the control for the 'MAILBOX' feature of TRTY.   If  set  to

'ON',   then   the   station   will   respond   to   a   remote  command  of

'callsignZFfname', by sending that file from a subdirectory in  the  default

directory  named  'BOX',  if it exists. Because TRTY can not read the actual

directory of subdirectory 'BOX', in our net we normally have  a  file  named

'DIR',  which  contains the fnames of any files in the directory 'BOX'. That

way, a call to the station, with 'callsignZFDIR', will transmit the  listing

of  files  in  the  station  'BOX' directory.  Any prearranged scheme can be

implemented, to alert callers to a message awaiting delivery.  If  no  local

subdirectory  exists  called  'BOX'  or  no  'called  for  fname'  is in the

directory, the responding system will just send the  local  'WRU'  response.

The subdirectory must be named 'BOX'

FANCY MARGIN:-

Setting item 5 to 'Y' means, send 'FANCY MARGIN' (the string set in Menu

8 above), at the beginning of each AUTOWRAPed line. \Sx in the FANCY margin,

will call up STRING(x). This allows  any  STRING  as  a  FANCY  margin.

... .  TRTY  OPERATIONS MANUAL - Page 17

MISCELLANEOUS MENU #5 continued:-

PICTURES:-

In  the  "PIX MODE", the "UNSHIFT ON SPACE" is disabled. The shift stays

the same as it was before the space. A <CR> does not line feed. When sending

long PIX files, a pause will be initiated at ten minute intervals, asking if

a 'station identification line should be sent?'. This  can  be  bypassed  by

suitable  response  to the query. A PIX file will not be seen as sent, or as

printed on a teletype printer, because of the inability of  the  monitor  to

overprint  a character. The PIX file will be sent to a file, as received, so

that it may be later be played back to  a  mechanical  printer,  for  proper

viewing.

DUAL SCREENS:-

One  item  called  'DUAL'  in  the 'MISCELLANEOUS OPTIONS' menu requires

special explanation.  This item makes possible the viewing of both  SESSIONS

on a split screen of either three windows, or four.  The DUAL windows may be

configured  as  three  windows,  with  two  dialog  windows, and the PRETYPE

windows overlaid, or as four windows, with the DIALOG and PRETYPE windows in

each quadrant of the screen, or four windows  from  top  to  bottom  of  the

screen.   A  setting  of  'DUAL=3",  will  allow  the  PRETYPE windows to be

overlay, and activity on one PRETYPE window will not bleed  through  to  the

overlay  window.   A  setting  of  'DUAL=4'  will  make  the PRETYPE windows

transparent, and activity in either window will show. The placement and size

of the windows is done in the 'WINDOW MANAGEMENT' menu.

SCREEN SAVER:-

Set item to 'Y' to blank or save the screen,  after  10  minutes  of 'no

input' to  any window.  Hit any key to refresh the window. Any received data

will un-blank the window. Saver action is erratic with Software Squelch.

PRINTER:-

The printer may be programmed to print only one of the two sessions.  If

the  printer  has  been programmed and not made ready, the program will hang

up, not indicating what the problem is, when printing is called.

BELL REMINDER:-

This feature has

To set or check the status of the MODE, hit Ctrl-q. The lower right line

will show dot's for 'Y' as it turns  'ON'. If already active, the  line  will

turn to x's as it is turned OFF.

Hit ctrl-q to set the MODE to 'Y'or 'N' as desired.

If  Ctrl-q  is  hit  while the 'Bell Reminder' is enabled, during normal

operations, there are two possibilities:

1.  Any beeping reminders (see explaination) are cleared.

2.  If no outstanding reminders, it disables the function.

Push F4 to Save the configuration.

... .  TRTY  OPERATIONS MANUAL - Page 18

-:WINDOW MANAGEMENT MENU #4:-

If  want  or need to change the default window arrangement, and have set

either DUAL=3 or DUAL=4  in  the  OPTIONS  menu  #5,  call  up  the  'WINDOW

MANAGEMENT' menu from the MAIN menu.

1). Configure  the  window  placement  and size by selecting the appropriate

window, (the cursor will move to the selected window).

2). Move the borders with the ARROW KEYS.

3). Save the configuration by hitting F4.

The number and placement of the windows should be in accordance with the

previous  setting  for  DUAL.  By moving the borders off screen, the maximum

lines and line length can  be  achieved.   80  character  line  lengths  are

possible,  but  undesirable  when  the  signals  are  printed  by a Teletype

machine.  There are some who still  use  the  old  mechanical  systems,  for

communications  and  monitoring.   The  72  character  line  length  is  the

preferred length.

One  of  the more popular window arrangements is to have DIALOG 1 window

sized to occupy the upper third of the screen, DIALOG 2 window in the second

third of the screen, and the PRETYPE for both windows in the last third.  In

this  way, both incoming windows can be seen, and the active PRETYPE will be

switchable to SESSION 1 or 2, in the lower part of the screen.

Any  other preferred arrangement can be used, even with each of the four

windows occupying a quadrant of the screen.  Let your  preferences  be  your

guide.

-:DISK I/O MENU #3:-

Item 3 in the MAIN menu, is used to READ, WRITE  files  and  to  view  a

directory.   Experiment  with this item after finishing the configuration of

he program. The window with the cursor, is the active read/write window, and

is the window that will be written to a file, of where a file will be shown.

(remaining lines left blank for readability)

... .  TRTY  OPERATIONS MANUAL - Page 19

Hit number 2, from the MAIN MENU, and the  following  menu  will  popup.

-:CONFIGURATION MENU #2:-

___________________________________________

|                          Configure      |

| 1. Edit Configuration File.             |

| 2. Read Configuration File.             |

| 3. Write Configuration File.            |

|                                         |

| 4. Reconfigure Keys.                    |

| 5. Toggle Sample Colors.                |

|                                         |

| F2 Install and Run Configuration.       |

|                                         |

| F4 Create Configuration File from       |

|    Installed Configuration.             |

|_________________________________________|

The  first  time  you  try  this, without a TRTY.CFG file on the default

drive, the program default configuration will be shown. If  TRTY.CFG  exists

on  the  drive,  that will be shown.  If fname.CFG file has been called from

the COMMAND line, that will be shown.

Hit   key  #1  to  Edit the Configuration file. The file that is on view

is  the  current  program  configuration.  Add  new  items,  or change those

required.

The WINDOW COLORS and the PROGRAMMABLE  STRINGS  can  be  added  to  the

configuration file ONLY from this menu, or with an ASCII editor.

COLORS CONFIGURATION:-

The default colors for the menus and screens are a white foreground  and

a  black  background.  (white  characters  on a black screen) The colors, or

intensity levels on a monochrome screen, are shown as numbers, on the bottom

of the screen, when the CONFIGURATION file is viewed.

All  colors are possible, space precludes showing all colors.  Try color

numbers that fall between the numbers shown.  An EGA or VGA  screen  can  be

very  colorful.  To change the colors of the text, background or  borders of

any window,  edit  the  configuration  record  directly,  adding  the  color

keywords and numbers to each window section.  Each window must be configured

separately.

See  the  MASTER  CONFIGURATION  FILE,  at  the end of this document for

proper placement of the 'key words' for window color configurations.

Keywords in configuration file require the number of the color shown  at

the bottom of the screen to display that color.

In these items, the 'n' indicates one of the color numbers.

COLOR2425 n         Sets color for 24th and 25th lines. Try 4.

COLOR_HI n          Sets color for variable data on screen. Try 11.

COLOR MENU n        Sets menu colors. Try 12.

... .  TRTY  OPERATIONS MANUAL - Page 20

CONFIGURATION MENU #2 continued:-

Add  the  following KEY WORDS and numbers for each window color.

BACKGROUND n        Sets background color for windows. n must be 0-7.

FOREGROUND n        Color of the normal text, and GIMMICK on 25th line.

HIGH_LITER n        Color for 'BLOCK' identification.

BORDER n            Color for border of window.

MOVE_BORDER n       Color for border when being moved in WINDOW menu.

PROGRAMMABLE STRINGS:-

This  program  allows  26  callable  STRINGS  to be programmed, for each

SESSION, and to be called with the 'CTRL-A' command. See the examples in the

configuration file at the end of this document. The number of characters  in

a  STRING, including the STRING name may not exceed 80 characters.  Use your

normal ASCII editor or edit the configuration record  directly  from  within

the  TRTY  program.

Format control, for data in  the  STRINGS,  is  done  by  including  the

following control characters along with the data. The STRINGS may consist of

just control characters. Be sure in enclose the STRINGS in double quotes.

"\C"       means Put current SESSION callsign in STRING. In EOT strings

this  means  'CARRIAGE  RETURN',  sorry  about the two

different command actions.

"\D"       means play 'diddles', how many set in menu #8.

"\G"       means play 4 N's and end transmission.

"\K"       means Insert "Disk", if copying to disk.

"\M(or m)" means Insert CR here. Note:- In these strings 'M' means CR.

Actually means, insert CR prior to next characters.

"\J(or j)" means Insert LF here. Note:- In these strings 'J' means LF.

Actually inserts the EOL sequence from menu #8,4.

"\N(or n)" means Insert NNNN , do not end transmission.

"\Sx"      means Play STRING(x) here. Only four "Plays" in a row.

"\\"       means Send ESCAPE character.

"\L"       means Insert Fancy Margin.

"\$xy"     means Send out hex characters x and y. x and y must be

one of 0123456789ABCDEF. y is optional. Invalid

characters will be put out as is, and not as hex.

Hex FD,FE, and FF are not allowed.

"\R(OR r)" means Stop Sending when this point is reached in string.

Same as CTRL-V, end with no N's..

"\T(OR t)" means Turn on transmitter when reached in string, if

transmitter is not already turned on.

"\U"       means Insert "Time Stamp", works in any string.

"\Y"       means Insert "Relay", if relay mode is enabled.

"\Z(OR z)" means Insert the 'PARTNER' string from menu 8.

These control sequences are intended to reduce the number of  keystrokes

needed  in normal RTTY communications, and for Macro sequences in Packet and

Amtor communications. Experiment with these controls, see  the  examples  in

the file at the end of this document.

... .  TRTY  OPERATIONS MANUAL - Page 21

CONFIGURATION MENU #2 continued:-

After  Editing  the  file,  remember  that only the file in RAM has been

changed, the file on the disk must be saved. Following these steps  to  save

and implement the new configuration.

1. Hit F1, to return to the 'Configure' menu.

2. Hit key #3, to save the new configuration file. Hit <cr>.

The default fname is 'trty.cfg'. If you wish to change the fname,

do so prior to hitting key #3.

3. Hit F1, to return to the 'Configure' menu.

4. Hit F2, to install the new configuration.

CAVEAT:-   Do not hit F4 to 'Save the installed file', before you  hit  F2

to install the new file. You will lose the new configuration.

ERROR INDICATIONS:-

If  an  unacceptable character or number has been edited into any of the

KEYWORDS of the configuration file, in most cases, the program will stop  at

this  point,  with  the  configuration file on the screen.  The unacceptable

line will be shown in reverse video, and the problem listed at the bottom of

the screen.. Hit any key, edit the the character or number  and  resume  the

procedure on line #3 above.

DVORAK KEYBOARD ANYONE? or RECONFIGURING THE KEYBOARD:-

Each key or key combination can be remapped by hitting  number  4,  from

the  CONFIGURATION  menu. If you would like to exchange the functions of the

key combinations CTRL-W and CTRL-U, (WRU sent with the CTRL-U, in  place  of

CTRL-W and 'unmark the MARKED BLOCK sent by CTRL-W), call up the RECONFIGURE

KEY  menu.  As requested by the menu, hit the CTRL-U combination, as the key

combination  to  be  remapped,  and  at  the  next  menu,  hit  the   CTRL-W

combination. Both the CTRL-W and CTRL-U will now send the WRU, so go back to

the  RECONFIGURE  KEYS menu, and hit CTRL-W at the first menu, and CTRL-U at

the second menu. The configuration is now reversed for the two keys, and has

replaced the default mapping in the program. Be sure to  save  the  file  by

hitting  F4,  to make the exchange a part of the configuration file. Install

the  change  by  hitting  F2.   Any  letter  key,  number  key,  or  control

combination  can  be  remapped  with  this  feature. The sequence of keys is

important, first the key to be remapped  ,  and  then  the  function  to  be

redirected,  then  F4  to  save  the installed configuration.  Make sure you

redirect both  key  control sequences, or  there  will  two  keys  doing  or

printing the same thing.

... .  TRTY  OPERATIONS MANUAL - Page 22

-:HELP MENU #1:-

The remaining configuration menu is the HELP menu. Hit key #1, from  the

MAIN menu, the HELP MENU will appear, if the TRTY.HLP file is on the default

drive.   This file may be edited/written either from this menu, or by use of

any ASCII editor.  The format of the Help file is  not  fixed,  and  can  be

customized  as desired.  If any the Keyboard Keys are redirected, changes to

the HELP file should be made to indicate the latest options.

In case the HELP file has not accompanied the program and DOC file, here

are  the  items  typically contained in  a HELP file. The HELP fname must be

TRTY.HLP, to be callable by the HELP file menu in TRTY.

KEY         Function

----------  --------------------------------------------------------------

F1          Activate function menus.

F2          Toggle from pretype to dialog and vice versa.

F3          Toggle alternate port setup.

F4          Write new configuration file configuration from menus.

F5          Force disk write and start new log file.

F6          Place a control key in text.

F7          Force Relay recording to start. (Clear Pretype buffer.)

F8          Zoom/UnZoom Window with cursor.

F9          Select Session 1.

F10         Select Session 2.

S-F1        Help screen.

S-F2        Configure screen.

S-F3        Disk I/O menu.

S-F4        Window management menu.

S-F5        Miscellaneous options menu.

S-F6        Port Setup menu.

S-F7        File usage display menu.

S-F8        Change constants menu.

Ctrl-^ (Ctrl Shift 6) Copy BASE to ALTERNATE port setup for session.

Ctrl-a      Select (A-Z) string and place at end of text.

Ctrl-b      Mark first or last line of block. (one block at a time.)

Ctrl-c      Two times in succession causes program to terminate.

Ctrl-d      Delete the block marked by Ctrl-b.

Ctrl-e      Erase active window.

Ctrl-f      Re-send last transmission or Force 'Relay' to play.

Ctrl-g      Mark end of text with NNNN and Ctrl-v.

Ctrl-h      Back space.

Ctrl-i      Tab.

Ctrl-j      Line feed.  Also causes line to be inserted when editing.

Ctrl-k      Not used, for IBM systems, used for ZENITH systems, see below.

Ctrl-l      New line with fancy margin.

Ctrl-m      Carriage return.

Ctrl-o      Toggle Port On/Off.

Ctrl-p      Not used.

Ctrl-q      Toggle Set/Reset Remote Bell Reminder.

... .  TRTY  OPERATIONS MANUAL - Page 23

HELO MENU #1 continued:-

Ctrl-r      Modify  'PARTNER' call. Place active cursor at end of string.

Ctrl-s      Not used.

Ctrl-t      Place time  and  date  at  cursor.

Ctrl-u      Unmark the marked block.

Ctrl-v      Mark 'end of text' and end tramission with out NNNN's at mark.

Ctrl-w      Play WRU message.

Ctrl-x      Toggle transmitter ON/OFF. RS232 pin #4 goes Plus, ON.

Ctrl-y      Move marked block and insert following line cursor is on.

Ctrl-z      Copy block  and  insert  following  the line  cursor is on.

Ctrl-\      Place Bell in text.  (Ctrl-reverse  slash).

Home        Beginning of line.  Top of window.  Top of  file.

Cursor keys. Up,down,left,right as marked.  (May also force  scrolling)

Ctrl-Shift-'-' Search buffer of window with cursor,using 'keyword' from

menu #8, item 'E'. (keyboard minus)

Esc         Two times in succession causes program to terminate.

The following keys are unique to the IBM PC.

Ins           Toggle insert mode.

Del           Deletes a character.

PgUp          Scrolls window up one page.

PgDn          Scrolls window down one page.

End           End of line.  End of screen.  End of file.

Ctrl-Bksp     Deletes a line.

ALT-J         Toggles SOFTWARE squelch mode enabling.

ALT-K         Opens the squelch, if enabled.

ALT-L         Closes the squelch, if enable

The following keys are unique to the Z100.

Ichr          Toggle insert mode.

Shift-Ichr    Deletes a character.

InsLine       Insert a line.  (Same as Ctrl-j.)

DelLine       Deletes a line.

F0            End of line.  End of screen.  End of file.

F11           Scrolls window up one page.

F12           Scrolls window down one page.

Help          Help.

Ctrl-K,J      Toggles squelch mode enabling. (two key sequence)

Ctrl-K,K      Opens squelch. (two key sequence)

Ctrl-K,L      Closes Squelch. (two key sequence)

(on  these, it makes not difference whether it is Ctrl-K

then Ctrl-J or Ctrl-k then J. The result is the same, and

not case sensitive)

(remainder of page left blank for readability)

... .  TRTY  OPERATIONS MANUAL - Page 24

-:REMOTE CONTROL:-

Assuming  two  TRTY using stations are in communication, this version of

TRTY.EXE (3003.2) will respond to following commands if preceded by a CR and

a LF, and the Pre-type buffer of the called station, is clear or empty.

COMMAND by sending       Action at (callsign) receiving station.

station.

callsignZAyyyyyy -  Resets a 'callsignZBzzzzzz' reminder where

the yyyyyy matches the zzzzzz described below.

The 'callsign' station will respond with the

WRU, to acknowledge receipt of the command.

If no match is found, the reset is ignored.

callsignZBzzzzzz -  Sets remote bell reminder with zzzzzz displayed

on remote screen. The 'callsign' station will

respond with the WRU, to acknowledge receipt

of the command. The 'zzzzzz' indicates text of

1 to 6 characters, which is displayed on the

right end of the dividing line in the dots, above

the pretype area.

Example for 'zzzzzz': Your Call or name.

Each session can store and display up to two

different 'zzzzzz' strings. If duplicate 'zzzzzz'

strings are received, the second is ignored.

callsignZC - Set RELAY station to copy incoming messages. See page

#15 for complete discussion of the 'RELAY' function.

callsignZD - Write receive buffer of 'callsign' station, to disk.

callsignZFfilename  - When received by the station 'callsign', the

file name is retrieved from the sub-directory

'BOX' in the default directory and is transm-

itted by station 'callsign', followed by  the

WRU   of  the  station. If the file does not

exist, the 'callsign' station will send it's

WRU. This is also known as a MAILBOX, as the

fname could  be  the callsign of a remote

station, for a file left there for later

transmission.

Not performed if the station has anything in

pretype, and not performed if the receiving

station has the function disabled. This can

be set or disabled directly in the config

file, or by toggling the item, in the WRU

setup, Menu #5.

... .  TRTY  OPERATIONS MANUAL - Page 25

REMOTE CONTROL continued:-

Using the COMMAND callsignZFfilename:-

Make  a sub-directory in the default directory (the one with TRTY), with

a name of 'BOX'.  This sub-directory is  where  the  ZF  command  looks  for

files.   I  suggest  that  one  of the files be named 'DIR', and include the

fnames of all the files in the sub-directory 'BOX'.  Then a calling  station

can call for the file 'DIR', and find out what files are available.  Make up

the  files  using  MAIN  MENU  item  #3 (Disk I/O) and write a file from the

Pretype area. Be sure to have the message ready in Pretype before writing to

a file. When the Write function is called from the Disk  I/O  menu,  put  in

'BOX\filename',  as  called  for  and hit <cr>. That will write the pretyped

file to the BOX directory, with that filename.

callsignZR - Request to relay station to Relay message, with VIA etc.

See page #15 for complete discussion of 'RELAY'.

callsignZP - Replay previous transmission, WRU if pretype empty.

callsignZW - Play WRU

callsignZLcallsign - Inquire remote station status. See page #25.

End sequence with CTRL-G (NNNN).

callsignZMcallsign - Relay station call to Remote. See page #25..

Automatic response from relay station to

activate remote, unheard station.

:TIME - Play all WRU'S

:DISK - Play all WRU'S if in DISK COPY

:xTIME - Play x call area WRU's (use 0 to 9 for x)

:xDISK - Play x call area WRU's (use 0 to 9 for x)

(remainder of page left blank for readability)

... .  TRTY  OPERATIONS MANUAL - Page 26

-: REMOTE WRU FUNCTION:-

To determine if a remote station is active, when you can  not  hear  the

station  on  a  direct  path,  the  REMOTE  WRU  function is available.  The

function will only work when a version of the RTY.EXE or TRTY.EXE program is

in use at all stations involved. By the  use  of  specific  "Z"  signals,  a

remote,  but  unheard  station,  can be triggered to send its WRU, through a

RELAY station, and thus be heard by the originating station.

Following is the procedure to activate this feature:

(assuming the Relay Station can hear both other stations)

------------        -----------          -----------

: Your     :        : Relay   :          : Remote  :

: Station  :----\---: Station :-----\----: STATION :

:          :        :         :          :(unheard):

------------        -----------          -----------

Callsigns:

aaaaa              bbbbb                ccccc

Action:                             Comments:

aaaaa Sends:                        Calling station

bbbbbZLccccc<ret>                   Request bbbbb to call ccccc's WRU

bbbbb Automatically Sends:          Response from bbbbb

cccccZMbbbbb                        Ask ccccc to relay WRU to bbbbb

-

ccccc Automatically Sends:          Response from ccccc

bbbbbZC                             Set relay at bbbbb

WRU for ccccc                       Normal WRU from ccccc

bbbbbZR                             Request bbbbb relay

-

bbbbb then relays the WRU of ccccc, for all to hear, including aaaaa.

-

No operator intervention is required at any station, other than  at  the

originating  station.   The  remote  stations must have RELAY mode "ON", and

each Pre-type buffer empty.

(remainder of page left blank for readability)

... .  TRTY  OPERATIONS MANUAL - Page 27

-:EDITING TEXT:-

Text can be edited, deleted and/or moved from any window  to  any  other

window. The feature is useful when you want to or  need  to  manually  relay

data or notes from one station or SESSION to another.

Move  the cursor to the window with the data to be edited, by hitting F9

of F10, for the SESSION, and F2 to put the cursor in  the  window  with  the

data.

Hit  Ctrl-B  at  the  start  and finish of a block to be moved, move the

cursor to the place in the window to receive the text, and hit either Ctrl-Y

or Ctrl-Z and the data will be moved or copied. (see HELP file  for  control

key  action)  Note  that  the  blocked  data,  is  highlighted  in the color

configured as 'HIGH_LITER n" for that window, in the configuration file.

-:SYSTEM REQUIREMENTS:-

This version of TRTY requires 300k or more of RAM, and  version  2.0  or

higher of PC-DOS or MSDOS. The FILES parameter in the CONFIG.SYS file should

be  reviewed.   This  program  can  use  as  many as four files at one time.

FILES=20, BUFFERS=20 are suggested. Some versions  of  MSDOS  and  some  IBM

system copies may give trouble, not all combinations and systems have or can

be tested.

The program will start in a default mode, or in the configuration called

in  the  TRTY.CFG  record.  Other  configuration  files may be called on the

command line. I.E., TRTY IBM MYCONFIG.CFG <cr>.

As mentioned earlier, TRTY  is  primarily  a  control  system  for  RTTY

demodulators  of  the ST-6 MAINLINER type, and has been updated to be useful

with the PK232 AND KAM units.

-:PACKET/AMTOR OPERATION:-

Because users have the most trouble getting PACKET  and  AMTOR  properly

configured, I am including a detailed discussion of this area.

Four handshaking  modes  are  available.   None,  DTR/DSR,  RTS/CTS  and

software  XON/XOFF.  An  'x' will be shown, just before the TIME display, on

the 24th line, when an XOFF has been received.  Normally this will be reset,

as the XON is received from the external unit.  The  'x'  can  be  reset  by

hitting  F6  and  then letter 'Q', if the system hangs up with no XON.

Data  may  be  held  in  the  Pretype buffer, while using either of the

Duplex  modes,  until  ready  to  send  it,  by  hitting  CTRL-X. This turns

the system to 'Receiving' while  filling  the  Pretype.  Send  the  data  by

hitting Ctrl-x.

If  you  want  to  have  the  system start in the 'Duplex' mode with the

system in the 'Receiving' condition, then save that configuration, by  doing

so while in the 'Duplex Receiving' condition.

The  configuration  file  will  have  the  Key  words 'FULL-RECEIVE' and

'ECHO-RECEIVE' in the appropriate  Session  areas,  to  indicate  the  above

condition.

One of the common problems when using a TNC, is the EOL  requirement  of

the  TNC's.   RTTY and the default, is two carriage returns and a line feed,

(CCN), where the TNC normally require just a carriage return  (C).  Set  the

EOL properly, or experiment until you get it to work.

... .  TRTY  OPERATIONS MANUAL - Page 28

PACKET/AMTOR OPERATION continued:-

The  TRTY  program has been tested on packet nets, with the TNC-1, TNC-2

and with the Kantronics KPC-2400. Make sure  that  the  serial  cable  lines

connect  the  proper  inputs  and  outputs,  especially  if  using  hardware

handshaking. The use of DTE and DCE serial port  configurations,  can  cause

some  connection  problems.   I  have  found  that with the ZENITH Z100, the

following unique cable changes are necessary to get hardware handshaking  to

work with the Kantronics KPC-2400.  Set TRTY to DTR/DSR handshaking.

J1 RS-232 ports.         J2 RS-232 ports.

Z100   TNC               Z100   TNC

PIN    PIN               PIN    PIN

1      1 Ground          1      1 Ground

2  <   3 RXD             2  >   2 TXD

3  >   2 TXD             3  <   3 RXD

20 <   5 CTS             6  <   5 CTS

6  >   4 RTS             20 >   4 RTS

7      7 Ground          7      7 Ground

4 (open at J1)           8 (open at J2)

(Normally Pins 1 and 7 are tied together on both plugs)

No  other  wires or jumpers connected in the cable.  4 on J1 and 8 on J2

"MUST" be open (no connection).  (Due to the 2661 UART having to have the CD

signal in a spacing condition to receive signals)

For the above unique arrangement with the Z100, Set handshaking in TRTY

for DTR/DSR if Hardware Handshake is to be utilized.  (Note that CTS/RTS  of

the  TNC  go  to  DTR/DSR  of  the Z100, fooling the TNC into thinking it is

seeing CTS/RTS handshake.)

For IBM and compatibles the following is usually standard

DTE             DCE                    DTE              DCE

IBM DB-25   To   TNC DB-25               IBM DB-9  To    TNC DB-25

Pin 1  Ground    Pin 1 Ground            Pin 1  CD       Pin 8  CD

Pin 2  TXD  >    Pin 2 TXD               Pin 2  RXD  <   Pin 3  RXD

Pin 3  RXD  <    Pin 3 RXD               Pin 3  TXD  >   Pin 2  TXD

Pin 4  RTS  >    Pin 4 RTS               Pin 4  DTR  >   Pin 20 DTR

Pin 5  CTS  <    Pin 5 CTS               Pin 5  Gnd      Pins 1 & 7

Pin 6  DSR  <    Pin 6 DSR               Pin 6  DSR  <   Pin 6  DSR

Pin 7  Ground    Pin 7 Ground            Pin 7  RTS  >   Pin 4  RTS

Pin 8  CD   <    Pin 8 CD                Pin 8  CTS  <   Pin 5  CTS

Pin 20 DTR  >    Pin 20 DTR

Above is actually to Kantronics TNC DB-25  but  with  signal  names  and

directions (< >) you can figure out the connections to you particular TNC if

different from the Kantronics.

See the included documents and configuration files  for  KAM  and  PK232

units.  These are furnished by users, and have been working for them.

... .  TRTY  OPERATIONS MANUAL - Page 29

-:IBM SYSTEM CONNECTIONS:-

Follow  are  the requirements and connections for the normal RTTY units,

such as the ST-5, ST-6 and DT600 type units.

The  input  and  output  levels  from  IBM compatible systems are RS-232

levels, +5/-5 volts.  The input/output levels of older type Terminal  Units,

may  be  TTL  levels,  and not compatible with computer systems.  The levels

must be converted to a common standard, either at the computer system or the

terminal unit interface.  There are integrated  chips  available  to  change

levels  from  and  from TTL levels, and most usually is done in the Terminal

Unit.

DTE       Connection to       DTE       Connection to

IBM DB-25        TU             IBM DB-9        TU

Pin 1  Gnd      Gnd             Pin 3  TXD >  Send Data

Pin 2  TXD  >   Send Data       Pin 2  RXD <  Receive Data

Pin 3  RXD  <   Receive Data    Pin 7  RTS >  Push To Talk

Pin 4  RTS  >   Push To Talk

*****IMPORTANT******

Unless  the  TU  specifically  states  it's terminal interface is RS-232

signal levels - - - - Do not attempt to hook your RS-232 Computer connectors

directly into the older Terminal Units without level converters to match the

RS-232 and Terminal Unit requirements.

(again, for readability)

... .  TRTY  OPERATIONS MANUAL - Page 30

-:MASTER CONFIGURATION FILE:-

The following pages consists of a Master Configuration File, showing all

the  possible  configuration items, and most of the possible variations. Use

this listing to compare with  the  program  configuration  file,  to  better

understand and configure the line items.

Order of 'key words' or items for each window not critical, and are  not

case sensitive. Spelling and arguments are critical.

Keywords:-                    Comments:-

NOLOGO              /* Prevents logo from showing at program startup.

/* For autostart from cold REBOOT, and AUTOEXEC.BAT,

/* configure to LOGO=N in OPTIONS menu.

DUAL 3 (or 4)       /* Screen setup for viewing both ports at same time.

/* Requires setting borders to non-overlap condition.

/* Three windows, from top to bottom of screen or

/* Four windows, side by side or tiled on screen.

/* Delete for separate screens.

SAVE_SCREEN         /* Blanks screen after period of inactivity.

/* Data is not lost.

/* Delete for not save screens.

COLOR2425 4         /* Color of items on the 24th and 25th lines.

COLOR_HI 11         /* Highlight color of configured items.

COLOR_MENU 12       /* Base color of menus.

DELAY_A 1000        /* Delay count for Ctrl-A when calling STRINGS(x).

SESSION1            /* Controls PORT A or COM1.

CALL XXXXXX      /* Call sign for Port A or COM1.

ZONE "CST"       /* Could be UTC, CDT, EDT, etc.

RELAY_TIMER 3600 /* Relay timeout seconds. Time the RELAY can be tied

/* up in any one relay. Reset clears Pretype.

STARTUP 32       /* Number of diddles when transmitter turns on.

MARGIN "'  "     /* Text for Fancy Margin.

RELAY            /* RELAY mode active for BASE PORT SETUP.

NOFANCY          /* Suppress Fancy Margin. Default = FANCY.

WIDTH 72         /* Line length for communication.

EOL CCN          /* For Custom end of line sequence.

/* C = CR, L = Letters, N = LF, F = Figs (for BAUDOT)

/* C = CR, N = LF ( for ASCII)

PIXON            /* Run this session in Pix mode. Delete for normal use.

BREAK            /* Set break mode. Nothing for normal RTTY.

WRUONLY          /* Play WRU only when CallsignZW is received.

/* Nothing for WRU on :TIME, :DISK or callsign requests.

BPS 45.5         /* Baud Rate for Port A (COM 1)

BAUDOT           /* Mode will be Baudot

STOPS 1.5        /* 1.5 Stop Bits, can be 1, 1.5 or none. (1.5 for BAUDOT)

LOGBOTH          /* Option for disk logging. LOGTX, LOGRX, LOGNONE

LOGNAME "*d"     /* Disk File name for logging generated by system.

/* The log drive for the system  generated  fname is "d",

/* and may be inserted directly in the config file.

LOGSIZE 5000     /* Size of log file.  (Will be approximate.)

RECEIVE          /* Port A Dialog Mode after configuration is set.

/* Could be SENDING, RECEIVING, FULL DUPLEX, FULL-RECEIVE

/* FULL-ECHO or ECHO-RECEIVE.

STUPSTR "1"      /* Default startup string.

EOTSTR "2"       /* Default End Of Transmission string.

... .  TRTY  OPERATIONS MANUAL - Page 31

MASTER CONFIGURATION FILE continued:-

WRUSTR  "5"      /* Default WRU.  ***These same items will appear in

other window areas as you configure them configured.***

SENSITIVE 4 10   /* Default sensitivity for software squelch.

SQUELCH          /* Squelch ON.

RELAY            /* Relay mode enabled for BASE PORT SETUP.

/* Nothing for no relay mode.

XON              /* Turn on XON/XOFF handshaking. Nothing for OFF

NOLEVEL34        /* Suppress receiving of 3rd and 4th level BAUDOT.

/* Delete for receiving 3rd and 4th level BAUDOT.

/* Program always sends 3rd and 4th level BAUDOT.

PARTNER "nnnnn"  /* Partner call for BASE SETUP. See DOC file.

ALTERNATE           /* Sets program to config ALTERNATE PORT.

RELAY            /* Relay active for ALTERNATE PORT SETUP.

ZONE "UTC"       /* Alternate ZONE

STARTUP 32       /* Startup diddle number.

MARGIN " .   "   /* Alt FANCY MARGIN.

EOL CCN          /* Alt End Of Line.

BPS 45.5         /* Alt Bits Per Second.

BAUDOT           /* Alt Mode.

STOPS 1.5        /* Alt Stop Bits.

RECEIVE          /* Mode shows here, as shown above.

STUPSTR "1"      /* Start up string default.

EOTSTR "2"       /* End Of Transmission string default.

WRUSTR "5"       /* WRU default string.

SENSITIVE 4 10   /* Default Sensitivity numbers.

PARTNER "nnnnn"  /* Partner call for ALTERNATE SETUP.

ALTERNATE           /* Resets to BASE SETUP, after config Alternate Port.

/* If the file retrieval ZF has been set 'Off', then

/* the words ZFOFF will follow this word ALTERNATE.

/* The indicators for Alert Bell will also follow this

/* word ALTERNATE, and be, as appropriate, BELLMIND or

/* NOMIND. This feature is set in the options menu, #5

/* from the MAIN menu.

/* Each Session may have 26 Strings available on Ctrl-a, plus a letter.

/* Customize as desired, but only one line per STRING.

STRING(A) "This is the first of a possible 26 strings."

STRING(B) "The quick brown fox jumped over the lazy dog's back.  0123456789"

STRING(C) "RYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRYRY"

STRING(D) "Each string must fit on one line of the configuration file."

STRING(E) "The lines of each string must be enclosed in double quotes."

STRING(F) "The strings are selected by typing Ctrl-a and then"

STRING(G) "the letter in parentheses."

STRING(H) "This is the eighth string."

STRING(I) "\jString nine"  /* Line feed prior to line input. (\j)

/* \j may follow the line for Line feed after the string.

STRING(J) "\m\jString ten" /* CR, LF prior to line, (\m\j)

/* \m\j may follow the line for action after line.

STRING(K) "String eleven\n"  /* Send NNNN after line. (\n)

STRING(L) "String twelve\sA" /* Play STRING(A) after line. (\sx)

STRING(M) "String thirteen"  /* Format controls are not case sensitive.

STRING(N) "String fourteen\C" /* Place current session callsign in string.

STRING(O) "String fifteen\$xy" /* Send hex chars xy in string. ($xy)

... .  TRTY  OPERATIONS MANUAL - Page 32

MASTER CONFIGURATION FILE continued-

STRING(P) "String sixteen\R"  /* Stop sending, same as Ctrl-V. (\R)

STRING(?) "\TString Q to Y"   /* Turn on transmitter at this point.(\T)

STRING(Z) "String twentysix\Z" /* Insert PARTNER string here.

SESSION2      /* Configure as shown above for PORT B, or COM2.

/* Items as appropriate for SESSION 2.

ALTERNATE          /* As above in SESSION 1.

etc etc etc (note, beginning and ending word ALTERNATE, to ALT mark setup.

ALTERNATE          /* As above for SESSION 1.

/* Each Session may have 26 strings available. Following for Session TWO.

/*   with same restrictions as shown in Session ONE.

DIALOG1

TOP 2           /* Screen row of top of this window  (Possible 1-21)

BOTTOM 16       /* Bottom line of DIALOG1 window. (Possible 3-23)

LEFT 2          /* Left side of border. Moved with Window menu. (1-78)

RIGHT 79        /* Right side of border. Moved from Window menu. (3-80)

BACKGROUND 6    /* Color of background, colors 0 to 7.

FOREGROUND 7    /* Color of text and GIMMICK in this window.

HIGH_LITER 112  /* Color of high lighted text in this window. Ctrl-B.

BORDER 15       /* Color of border.

MOVE_BORDER 240 /* Color of border being moved by WIndow Management.

PRETYPE1           /* Screen config in same order as above.

BOTTOM 22       /* Can have same items as PRETYPE window above.

DIALOG2            /* Can have same items as DIALOG window above.

LEFT 2          /* Can have same items as DIALOG window above.

PRETYPE2           /* Can have same items as PRETYPE window above.

RIGHT 79        /* Same song, second or more verse.

REMAP xx yy        /* Remap key (control-x) xx to yy.

REMAP yy xx        /* Remap key (control-y) yy to xx.

/* The REMAP lines appear if keys have been remapped.

PRETYPE1           /* Cursor placement upon callup of program.

WORD "de"        /* Search string for software squelch.

WORD  "Tom"      /* Search  string  for software squelch, used by both

/*    Sessions if both configured for Squelch..

WORD "from"      /* Words are not case sensitive.

WORD  etc..      /* As many as 750 seven character dictionary.

WORD "/Ccallsign" /* Default by program when using Software Squelch.

WORD "/Ncallsign" /* Default by program when using Software Squelch.

WORD "/callsign"  /* Default by program when using Software Squelch.

/* The above three WORDs are written to the Configuration

/* file, when the file is saved in Software Squelch Mode.

(same song and verse, for readability)

... .  TRTY  OPERATIONS MANUAL - Page 33

-:ANSWERS TO QUESTIONS:-

1. Source code will NOT be made available.

2. Buffer space will always be available, for any need.  If  the  buffers

all  get  filled,  then  the  buffers will FIFO, and the oldest data will be

lost. The available buffers are  divided,  by  the  program,  half  to  each

SESSION.   Clear  the  buffers,  for  each window, when no longer needed, by

hitting CTRL-E. At startup, the program claims as many lines of  buffers  as

the RAM will allow, up to about 2000 lines.

3.   This  program  is not compatible with the Hercules monitors, for two

reasons. One is that none of the  developers  had  a  Hercules  system,  and

secondly,  we  did  not  feel  like  winging  it. There are simulators  that

will  allow HERCULES systems to use this program. SIM-CGA is such a program,

and is available on many BBS. It is included in the ZIP package for TRTY.

4.  The program is written in 'C', and has taken about six years to reach

this  level  of  performance.  It was written by Perry Taylor W0SE, a top

programmer, and tested by a great group of  'DEBUGGERS", and 'BETA TESTERS'.

The TRTY  program is based on an older program, developed by the same group,

called RTY.COM.

6.  Yes, TRTY is Windows 3.0 friendly, PIF and ICO files are provided.

5. The net members who did the majority of the testing are:-

KG6YY, Peter Ellena, Paso Robles California

W5ESX, T.H. Bright, Midwest City, Oklahoma

W8EXG, Paul Wood, Griffin, Georgia

W0LTO, 'Abe' Rademaker, West Union, Iowa

W0NW , Tom Vinson, Pine Island, Minnesota

W0SE , Perry Taylor, Waterloo, Illinois

To these  good  friends,  and  very  dedicated  Amateur  Radio  Teletype

operators, We  give  a  most  heartfelt  THANKS.  They spent many hours, off

and on the air, digging out the bugs and testing the many versions of  TRTY.

All  computer programs have bugs, We hope we have found the most obvious and

destructive ones. We give no warranties, just wish all a good time.

-30-