The KAM on RTTY with TRTY.

-----------------------------------------------------------------------------

The KAM can be used with TRTY in RTTY, AMTOR, ASCII and PACKET modes

by proper configuration of the program.  Probably the mode that TRTY

adds the most to is the RTTY mode.  All of the 'Remote Functions'

of TRTY become useable with the KAM when TRTY and the KAM are configured

properly.  That is, a remote station can use you for "Relay" purposes,

can call your WRU string, read the 'Message Box' etc.

The following example is just one way the job can be done, I'm sure

with experimentation, additions can be made to satisfy your own

needs.  The Hardware connections are covered elsewhere in this

documentation, the following covers Configuration of the KAM and

TRTY for RTTY operation.

The important strings that affect RTTY operation are explained below.

String "T" is the "Startup" string designated in F1, 8, A of the menu.

String "R" is the "EOT" string designated in F1, 8, B of the menu

String "A" is the WRU string designated in F1, 8, C (Customize your own)

Control Characters are shown spelled out in parenthesis for clarity.

Actually code them into the strings using F6 and the character. (For

instance Control-C - type F6 then C).

Also be sure to note the slash is a "backslash" (\) and not a normal

forward (/) slash.

STUPSTR "T"

EOTSTR "R"

WRUSTR "A"

STRING(A) "DE W1XXX Podunk, Ia. - \U \K,\Y \M\J"

STRING(R) "(TWO CTL-L)(CTL-C)E\M\J"

STRING(T) "(CTL-C)T(THIRTY TWO CTL-L'S)\M\JZCZC\M\J"

STRING(X) "(CTL-C)X\M\JFLOW ON\M\JAUTOLF ON\M\JECHO ON\M\JXMITECHO OFF\M\J"

STRING(Y) "\M\JFLOW OFF\M\JAUTOLF OFF\M\JXMITECHO ON\M\JECHO OFF\M\JR\M\J"

STUPSTR, EOTSTR and WRUSTR letters are designated in the menu written

to the .CFG file by F4 while in that menu items mentioned previously.

------------------------------------------------------------------------------

String A:  (WRU)

This string is sent when CTL-W from the keyboard is struck or when

a remote station asks for Who aRe yoU (WRU) with a W1XXXZW sent as

the first characters of a new line.  The STUPSTR is inserted

automatically to key up the KAM radio side prior to sending this

string.  The content can be personalized.

String R:  (EOTSTR)

This String is sent as the last thing to the KAM each time a

Control-G or Control-V is encountered while transmitting

the Pretype Buffer, or transmitting in response to a remote

command.

CTL-L produces a letters character on the RTTY radio side of the KAM

in BAUDOT code.  Two letters are used just to make sure lower case and

Sync on the distant end.

CTL-C followed by an E causes the KAM to cease transmission on the

Radio side (drop PTT) "after" the KAM send buffer is emptied.  This

is important to ensure all data gets sent from the KAM buffer before

returning to RTTY Recive mode.  [You will note that TRTY sends data

to the KAM (after you hit Control-X) very fast and returns to receive

status.  The KAM buffer loads at the Baud Rate between the computer

and the KAM and then TRTY returns to Recevie state.  The Rate may

be 2400 Baud or higher.  The Buffer in the KAM then must send out

all the data to the radio side at 45.5 before returning to receive

itself.  This command allows the KAM buffer to empty before it

returns to receive state].

The \M\J is a CR and LF to the KAM, all commands must be followed by

a CR before the KAM recognizes the line.

String T: (STUPSTR)

This string is sent as the first thing to the KAM when a Control-X

(begin transmitting pretype buffer) is hit.  It is also automatically

inserted as the first item each time the KAM is remotely triggered

to begin transmitting.

Control-C followed by a "T" causes the KAM to key up the transmitter

on the radio side (Activate PTT).  The 32 CTL-L characters send a string

of letters out on the radio side to allow slow autostart circuits in

regular RTTY TU's to activate.  (Adjust to your own desires).

The \M\J inserts a CR-LF sequence

The ZCZC is an abitrary sequence selected to turn on the Squelch

(Software TRTY) by the 'Word' function of TRTY.  (At the end of

the .CFG file).  This can be eliminated if desired, although the

Software Squelch feature of TRTY is not as effective with a

Multi-mode unit (such as the KAM) as it is with a normal Mod/Demod

Tuning Unit.  All data between the Computer and KAM is framed

correctly (even though it may be garbage, each character has the

proper framing in ASCII to the computer).  So the software squelch

feature can never detect a framing error.  The KAM will not send

a 'Break' (long space) character to the computer either, so those

two methods of activating squelch are inactive with a KAM.  Word

recognition to squelch and unsquelch is the reliable method to use

with the KAM.  With a normal Mod/Demod only Tuning Unit such as

The HAL ST-6 (with AutoStart off) or some of the older units without

AutoStart Hardware, the software squelch has more ways of squelching

noise copy.  (This is explained elsewhere in the Documentation).

The \M\J inserts a final CR-LF sequence in the String.

String Y and X are strings optionally used to get into RTTY mode

from the Packet mode and to return to Packet mode from RTTY mode.

The TRTY configuration file for RTTY operation should be configured

for "FULL-RECEIVE" in the .CFG file.  One method to set this

by menu is while TRTY is in the FULL DUPLEX mode hit a Control-X

to go to receive.  Then Hit F1, 4, F4 to write the .CFG file.  At

this point "FULL-RECEIVE" should be in the CFG file.  (Check to be

sure).

---------------------------------------------------------------------------

*********

*****Important*****

*********

---------------------------------------------------------------------------

The sequence of Power Up Etc. to initially send the Y string to

get to the RTTY mode is important.  If you hit a Ctl-X from

the 'receive' state of TRTY while at the CMD: prompt of the KAM

the starup string will be sent.  It contains a Ctl-C followed

by a T which will put the KAM into transparent mode and cause

you grief in getting configured for RTTY.

The following sequence is recommended:

1.  Power off the KAM (configured for normal Packet operation).

2.  Bring up the TRTY program, Hit Ctl-X to get into Full Duplex

mode from receive if the program comes up in receive.

Clear the TRTY transmit buffer with a Ctl-E.

3.  Turn Software Squelch "off" with the Alt-J toggle (if it is

on).  No "S" should appear in the 25th line before the 'TIME'.

4.  Turn on the KAM.  (Sign-on message should appear).

5.  Make sure you are communicating with the KAM by hitting CR

and get CMD: return.

6.  Send String "Y" by hitting CTL-A then Y.

7.  KAM should respond with command changes and "-RTTY 45-"

if the speed is set at 45 Baud in the KAM parameters.

8.  Hit Ctl-V to go to receive mode with TRTY, clear the transmit

buffer and begin transmitting with Ctl-X after pretyping

anything you wish.  (During the pretyping you may turn on

the transmitter at any time with Ctl-X).   End the pretype

buffer with Ctl-G at the point you wish the transmitter to

cut off.

9.  Re-activate Software Squelch in TRTY with Alt-J, if desired.

10.  When RTTY operation is to be terminated, the KAM may be

returned to Packet configuration by sending String X.

(Hit Ctl-A followed by X, then Ctl-X to release the string

to the KAM.  The KAM should respond with changes made by

String X and be at CMD: prompt.

----------------------------------------------------------------------------

String Y: (Put the KAM in RTTY mode from Packet Mode)

This string, sent to the KAM while at the CMD: prompt will issue

commands to the KAM to configure it for RTTY operation with the

TRTY program.

Begins with \M\J CR-LF sequence to clear command line to the kAM.

Turn FLOW 'OFF' to allow XMITECHO mode in the KAM to operate

properly.

Turn AUTOLF 'OFF' for no double line feeds.

Turn XMITECHO 'ON' so that echo to the dialog screen only occurs as

the character is being sent out to the Radio side.

Turn ECHO 'OFF' to disable the normal 'echo as sent to KAM buffer'.

Send "R" to put the KAM into RTTY mode.

Note that each command is followed by \M\J to insert the command

to the KAM.

String X:  (Return KAM to Packet mode from RTTY mode)

This string sent to the KAM while in RTTY mode will return the KAM

parameters to Packet operation.

Ctl-C followed by X returns the KAM to CMD; prompt.

The remaining commands reverse the sequence in String Y used to

put the KAM into RTTY mode.  (Flow on,  Autolf on, Echo on,

Xmitecho off).

-----------------------------------------------------------------------------

Following is a 'Typical' KAM parameter setting "before" and "after"

configuring to RTTY from Packet.  The KAM is using Version 4.0

Firmware.

BEFORE:::  This is normal Packet operation.

KANTRONICS ALL MODE COMMUNICATOR VERSION 4.00

(C) COPYRIGHT 1988-1990 BY KANTRONICS INC.  ALL RIGHTS RESERVED.

DUPLICATION PROHIBITED WITHOUT PERMISSION OF KANTRONICS.

cmd:DISPLAY

8BITCONV OFF

ASCBAUD  110

ALIAS    OFF/OFF

AX25L2V2 ON/ON

ABAUD    2400

AUTOCR   80

AUTOLF   ON

AUTOSTRT OFF

AXDELAY  0

AXHANG   0

BEACON   EVERY 0/EVERY 0

BKONDEL  ON

BTEXT

BUDLIST  OFF/OFF

BUDCALLS NONE

A/V Link state is: DISCONNECTED

CANLINE  $18

CANPAC   $19

CCITT    OFF

CD       INTERNAL/INTERNAL

CHECK    0/0

CMDTIME  1

CMSG     OFF/OFF

COMMAND  $03

CONLIST  OFF/OFF

CONMODE  CONVERS

CONOK    ON

CPACTIME OFF

CR       ON

CRADD    OFF

CRLFSUP  OFF

CSTAMP   OFF

CTEXT

CWBAND   200

CWID     EVERY 0/EVERY 0

CWSPEED  25

CWTONE   750

DAYTIME  12/06/91  12:36:38

DAYTWEAK 8

DAYUSA   ON

DBLDISC  OFF

DELETE   $08

DIDDLE   OFF

DIGIPEAT ON/ON

DWAIT    12/12

ECHO     ON

ESCAPE   OFF

EXTMODEM OFF

FLOW     ON

FILTER   OFF

FRACK    5/5

FSKINV   OFF

FULLDUP  OFF

GATEWAY  OFF

HBAUD    300/1200

HEADERLN ON

HID      OFF/OFF

INTFACE  BBS

INVERT   OFF

KNTIMER  1

LCOK     ON

LCRTTY   ON

LCSTREAM ON

LFADD    OFF

LLIST    OFF

LOWTONES OFF

MONITOR  ON/ON

MALL     ON/ON

MARK     1600

MAXFRAME 7/7

MAXUSERS 1/1

MBEACON  ON/ON

MCON     OFF/OFF

MCOM     ON/ON

MISSCHAR $20

MORSE

CODE     TX      RX         CODE     TX      RX

**--    $00    $00          *-*-    $2A(*) $4141(AA)

---*    $00    $00          ----    $00    $00

***-*   $21(!) $534E(SN)    **-**   $00    $00

**-*-   $00    $00          **--*   $00    $00

*-***   $25(%) $4153(AS)    *-**-   $00    $00

*-*-*   $2B(+) $4152(AR)    *-*--   $00    $00

*--**   $00    $00          *--*-   $00    $00

*---*   $00    $00          -**-*   $2F(/) $2F(/)

-**--   $00    $00          -*-**   $00    $00

-*-*-   $26(&) $4B41(KA)    -*--*   $28(() $28(()

-*---   $00    $00          --**-   $00    $00

--*-*   $00    $00          --*--   $00    $00

---*-   $00    $00          ***-*-  $23(#) $534B(SK)

*-***-  $00    $00

MRESP    ON/ON

MRPT     ON/ON

MSTAMP   OFF

MYCALL   W1XXX-1/W1XXX-1

MYALIAS  W1XXX-1/W1XXX-1

MYAUTOST W1XXX

MYNODE   W1XXX-1

MYPBBS   W1XXX-10

MYGATE   W1XXX-1

MYSEL4   (WXXX)

MYSEL7   000000000 (VVVVVVV)

NAVERR   5

NAVMSG   ALL

NAVSTA   ALL

NDWILD   OFF

NEWMODE  ON

NOMODE   OFF

NTEXT

NUCR     0

NULF     0

NUMNODES 0

PACLEN   0/0

PACTIME  EVERY 30

PARITY   4

PASS     $16

PASSALL  OFF/OFF

PBBS     0

PBPERSON OFF

PERSIST  192/63

PID      OFF/OFF

PMODE    NONE

PORT     VHF

POSTKEY  12

PREKEY   12

PROSIGN  $5C

PTEXT

RBAUD    45

REDISPLA $12

RELINK   OFF/OFF

REPHASE  ON

RESPTIME 1/1

RETRY    10/10

RING     ON

RNRTIME  0

SCREENL  0

SENDPAC  $0D

SHIFT    170

SLOTTIME 1/1

SPACE    1800

START    $11

STATSHRT OFF

STOP     $13

STREAMSW $7E/$7C

STREAMCA OFF

STREAMEV OFF

SUPLIST  OFF

SUPCALLS NONE

SWP      25,8,75/17,17,108

TRACE    OFF

TRFLOW   OFF

TRIES    10

TXDELAY  25/25

TXDAMTOR 5

TXDFEC   0

TXFLOW   ON

UNPROTO  CQ/CQ

USERS    1/1

USOS     OFF

WTEXT    W1XXX, HIRAM IN PODUNK, MA.

XFLOW    OFF

XMITECHO OFF

XOFF     $13

XON      $11

cmd:

AFTER:::  (After sending String Y to the KAM for RTTY operation)

KANTRONICS ALL MODE COMMUNICATOR VERSION 4.00

(C) COPYRIGHT 1988-1990 BY KANTRONICS INC.  ALL RIGHTS RESERVED.

DUPLICATION PROHIBITED WITHOUT PERMISSION OF KANTRONICS.

DISPLAYcmd:8BITCONV OFFASCBAUD  110ALIAS    OFF/OFFAX25L2V2 ON/ONABAUD    2400AUTOCR   80AUTOLF   OFFAUTOSTRT OFFAXDELAY  0AXHANG   0BEACON   EVERY 0/EVERY 0BKONDEL  ONBTEXT    BUDLIST  OFF/OFFBUDCALLS NONEA/V Link state is: DISCONNECTEDCANLINE  $18CANPAC   $19CCITT    OFFCD       INTERNAL/INTERNALCHECK    0/0CMDTIME  1CMSG     OFF/OFFCOMMAND  $03CONLIST  OFF/OFFCONMODE  CONVERSCONOK    ONCPACTIME OFFCR       ONCRADD    OFFCRLFSUP  OFFCSTAMP   OFFCTEXT    CWBAND   200CWID     EVERY 0/EVERY 0CWSPEED  25CWTONE   750DAYTIME  12/06/91  12:44:35DAYTWEAK 8DAYUSA   ONDBLDISC  OFFDELETE   $08DIDDLE   OFFDIGIPEAT ON/ONDWAIT    12/12ECHO     OFFESCAPE   OFFEXTMODEM OFFFLOW     OFFFILTER   OFFFRACK    5/5FSKINV   OFFFULLDUP  OFFGATEWAY  OFFHBAUD    300/1200HEADERLN ONHID      OFF/OFFINTFACE  BBSINVERT   OFFKNTIMER  1LCOK     ONLCRTTY   ONLCSTREAM ONLFADD    OFFLLIST    OFFLOWTONES OFFMONITOR  ON/ONMALL     ON/ONMARK     1600MAXFRAME 7/7MAXUSERS 1/1MBEACON  ON/ONMCON     OFF/OFFMCOM     ON/ONMISSCHAR $20MORSE    CODE     TX      RX         CODE     TX      RX**--    $00    $00          *-*-    $2A(*) $4141(AA)---*    $00    $00          ----    $00    $00***-*   $21(!) $534E(SN)    **-**   $00    $00**-*-   $00    $00          **--*   $00    $00*-***   $25(%) $4153(AS)    *-**-   $00    $00*-*-*   $2B(+) $4152(AR)    *-*--   $00    $00*--**   $00    $00          *--*-   $00    $00*---*   $00    $00          -**-*   $2F(/) $2F(/)-**--   $00    $00          -*-**   $00    $00-*-*-   $26(&) $4B41(KA)    -*--*   $28(() $28(()-*---   $00    $00          --**-   $00    $00--*-*   $00    $00          --*--   $00    $00---*-   $00    $00          ***-*-  $23(#) $534B(SK)*-***-  $00    $00MRESP    ON/ONMRPT     ON/ONMSTAMP   OFFMYCALL   W1XXX-1/W1XXX-1MYALIAS  W1XXX-1/W1XXX-1MYAUTOST W1XXXMYNODE   W1XXX-1MYPBBS   W1XXX-10MYGATE   W1XXX-1MYSEL4   (WXXX)MYSEL7   000000000 (VVVVVVV)NAVERR   5NAVMSG   ALLNAVSTA   ALLNDWILD   OFFNEWMODE  ONNOMODE   OFFNTEXT    NUCR     0NULF     0NUMNODES 0PACLEN   0/0PACTIME  EVERY 30PARITY   4PASS     $16PASSALL  OFF/OFFPBBS     0PBPERSON OFFPERSIST  192/63PID      OFF/OFFPMODE    NONEPORT     VHFPOSTKEY  12PREKEY   12PROSIGN  $5CPTEXT    RBAUD    45REDISPLA $12RELINK   OFF/OFFREPHASE  ONRESPTIME 1/1RETRY    10/10RING     ONRNRTIME  0SCREENL  0SENDPAC  $0DSHIFT    170SLOTTIME 1/1SPACE    1800START    $11STATSHRT OFFSTOP     $13STREAMSW $7E/$7CSTREAMCA OFFSTREAMEV OFFSUPLIST  OFFSUPCALLS NONESWP      25,8,75/17,17,108TRACE    OFFTRFLOW   OFFTRIES    10TXDELAY  25/25TXDAMTOR 5TXDFEC   0TXFLOW   ONUNPROTO  CQ/CQUSERS    1/1USOS     OFFWTEXT    W1XXX, HIRAM IN PODUNK, MA.XFLOW    OFFXMITECHO ONXOFF     $13XON      $11cmd:

It is beyond the scope of this document to go over each of the

KAM parameters, but the above should give you something to work

from.  Many of the parameters are unique to your own setup, such

as, Baud Rate between computer and KAM, Flow Control used between

computer and KAM etc. etc.

Following is an actual .CFG file from a TRTY setup to work with the

above KAM configuration.  Again, many settings are unique to your

own environment and this is presented as a 'base' to work from.

This particular configuration file uses COM1 with the KAM in RTTY

mode (Session one of TRTY) and has COM2 (Session two) configured

for use with a normal Packet TNC.  This configuration can monitor

and operate Packet on another TNC simultaneously with a KAM on

session one in RTTY mode.  (This program does not make use of

"Host Mode" operation and since there is only one computer interface

to the KAM, will not allow operation of RTTY and Packet with

the KAM "alone".

NOLOGO

SAVE_SCREEN

COLOR2425 11

COLOR_HI 14

COLOR_MENU 10

DELAY_A 1000

SESSION1

RELAY_TIMER 3600

WIDTH 72

LOGBOTH

LOGNAME "* "

LOGSIZE 30000

RELAY

FLOW_CTRL RTS

ZONE "EST"

CALL W1XXX

STARTUP 32

MARGIN "  "

EOL C

BPS 2400

ASCII8

STOPS 1

FULL-RECEIVE

EXPECT_ECHO

PARTNER "Wxxx"

STUPSTR "T"

EOTSTR "R"

WRUSTR "A"

SENSITIVE 6 8

ALTERNATE

ZONE "UTC"

STARTUP 32

MARGIN " .  "

EOL CCN

BPS 45.5

BAUDOT

STOPS 1.5

RECEIVE

STUPSTR "1"

EOTSTR "2"

WRUSTR "5"

SENSITIVE 4 10

ALTERNATE

BELLMIND

STRING(A) "W1XXX IS MONITORING \U\M\J"

STRING(R) "^L^L^CE\M\J"

STRING(T) "\M\J^CT^L^L^L^L^L^L^L^L^L^L^L^L^L^L^L^L^L^L^L^L\M\JZCZC\M\J"

STRING(X) "^CX\M\JFL ON\M\JAUTOLF ON\M\JE ON\M\JXMITE OFF\M\J"

STRING(Y) "\M\JFL OFF\M\JAUTOLF OFF\M\JXMITE ON\M\JE OFF\M\JR\M\J^X"

SESSION2

RELAY_TIMER 3600

WIDTH 80

LOGBOTH

LOGNAME "* "

LOGSIZE 5000

FLOW_CTRL RTS

ZONE "EST"

CALL W1XXX

STARTUP 0

MARGIN " "

EOL C

BPS 2400

ASCII8

STOPS 1

FULL-DUPLEX

EXPECT_ECHO

STUPSTR "Z"

EOTSTR "Z"

WRUSTR "Z"

SENSITIVE 4 10

ALTERNATE

ZONE "UTC"

STARTUP 32

MARGIN " .  "

EOL CCN

BPS 45.5

BAUDOT

STOPS 1.5

RECEIVE

STUPSTR "1"

EOTSTR "2"

WRUSTR "5"

SENSITIVE 4 10

ALTERNATE

BELLMIND

STRING(Q) "C W1AW V W1AW-1\M\J"

STRING(S) "C SNELLV\M\J"

DIALOG1

TOP 2

BOTTOM 15

LEFT 2

RIGHT 79

FOREGROUND 15

HIGH_LITER 14

BORDER 15

BACKGROUND 1

PRETYPE1

TOP 17

BOTTOM 22

LEFT 2

RIGHT 79

FOREGROUND 15

HIGH_LITER 14

BORDER 15

BACKGROUND 1

DIALOG2

TOP 2

BOTTOM 15

LEFT 1

RIGHT 80

FOREGROUND 14

HIGH_LITER 15

BORDER 14

BACKGROUND 4

PRETYPE2

TOP 17

BOTTOM 22

LEFT 1

RIGHT 80

FOREGROUND 14

HIGH_LITER 15

BORDER 14

BACKGROUND 4

PRETYPE1

WORD "\NW1XXX"

WORD "\CW1XXX"

WORD "AL"

WORD "CMD"

WORD "CQ CQ"

WORD "CQ DX"

WORD "KG1YY"

WORD "PAUL"

WORD "CARL"

WORD "PETE"

WORD "RYRY"

WORD "T.H."

WORD "TOM"

WORD "W1LTO"

WORD "W1SB"

WORD "W1ESP"

WORD "W1XXX"

WORD "ZCZC"

*Note:

Limitations of Word Processing do not allow showing the control

characters in the .CFG file as they would actually appear in TRTY.

Control-L, for instance is ^L above.  Control-C is ^C etc. etc.

In TRTY a Control-C is a heart shaped graphic character and Control-L

is a graphic character similar to a 'stick man'.

Note in Session Two above (COM2) for Packet the "EOL" is "C" alone.

This is necessary in the session used for Packet operation.  Set

this in Menu F1, 8, 4 with a C "Only".

From the above examples using a KAM with TRTY in the RTTY mode, you

should be able to adapt strings to provide similar functions to make

up Configurations for AMTOR or ASCII.  Strings provide the Macro's

to give you some very useable shortcuts in other modes with the KAM.

Following are some AMTOR suggestions:

Here are some possibilities for Strings with the KAM in AMTOR

operation.

STRING(A) "#X\M\JE OFF\M\JFL OFF\M\JXM ON\M\JAUTOL OFF\M\JAM\M\J"

This string would set the KAM from CMD: prompt into STANDY RECEIVE

MODE where you can Receive FEC, your unit will respond to an ARQ

call or you may enter FEC transmit mode.

STRING(C) "#X\M\JFL OFF\M\JE OFF\M\JXM ON\M\JAUTOL OFF\M\JAM "

This string duplicates the above, but leaves a space and no CR-LF

so that you may insert a SELCAL to call another station ARQ mode.

STRING(E) "#X\M\J"

This string merely returns the KAM to CMD: prompt.

STRING(M) "#X\M\JFL OFF\M\JE OFF\M\JXM ON\M\JAUTOL OFF\M\JLAM\M\J"

This string enters LISTEN mode allowing you to monitor ARQ and

FEC transmissions.  It does not support transmitting.

STRING(O) "\M\JDE \C BTU +?\M\J"

This string gives the "over" signal to make the other end the

Master station and allow him to transmit to you.

STRING(R) "#E\M\J"

This string returns the KAM to receive "after" emptying it's buffer

such as with an FEC transmission.

STRING(T) "#T\M\J"

This enters the FEC transmit mode from Mode STANDBY RECEIVE (String

A above.

STRING(X) "#X\M\JE ON\M\JXM OFF\M\JAUTOL ON\M\JFL ON\M\J"

This string returns to Packet operation from AMTOR operation.

All the above AMTOR modes set the KAM for XMITECHO ON and

ECHO OFF so an echo to the dialog (receive) screen only occurs

when the character is sent to the Radio.

For convenience, an actual KAM configuration file is included, (called

KAMPKT.CFG) which can be called up with trty by:

TRTY IBM KAMPKT.CFG

typed at the dos command line of your TRTY directory.  You will have

to change the Callsigns and set any port baud rates to match your own.

This file uses 2400 Baud between the Computer and the KAM and also

2400 Baud between the Session Two Packet unit and TRTY.

The file uses Com1 (Session 1) for the KAM and Com2 (Session 2) for

Packet.

#####################################