ViewSat

Satellite Viewing Program

December 8, 1992

Copyright (c) 1992

By David J. Rasmussen

All Rights Reserved

David J. Rasmussen

10374 Palmento Dr.

Riverside, Ca. 92505

_______

____|__     |               (tm)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

ALL VERSIONS OF VIEWSAT ARE NOT PUBLIC DOMAIN SOFTWARE, NOR ARE

THEY FREE SOFTWARE.

Non-registered users are granted a limited license to use ViewSat

on a trial basis for the purpose of determining whether ViewSat

is suitable for their needs. The term of the trial basis shall

not exceed 30 days. Use of ViewSat, except for this limited

purpose, requires registration. Use of non-registered copies of

ViewSat by any business, corporation, governmental agency, or

other entity institution is strictly forbidden. The program will

notify you when the evaluation period has elapsed.

This document is a complete description of the program, including

the license that will be granted upon registration.  There are no

surprises.  If you have questions or suggestions, or if you need

assistance, you may send EMail on Compuserve to 75176,1775 or

write to the address at the end of this document or phone me at

(909) 687-4344.

We encourage you to register your use of ViewSat. Registered

ViewSat users receive a diskette containing the current version

of the registered program, notification of new releases, and

enthusiastic support from the author when needed for a period of

at least 3 months from the date of registration. The registered

version of ViewSat is functionally identical to the shareware

distributed evaluation version, but it lacks registration

reminders and it is branded with the registered user's name.

Beyond the immediate tangible benefits, registration also

provides support for the shareware software distribution concept.

Shareware is not free software, nor is it in the public domain.

Shareware means quality software that you try before you buy,

generally at a far lower cost than a similar retail product, if

one exists. We all benefit, but only if the users do their

ultimate part of registering. If ViewSat is a useful addition to

your system, please register your use.

To register, fill in the form at the end of this document and

send it with $20.00 (US) for a single machine to D. J. Rasmussen

Consulting, 10374 Palmento Drive, Riverside, Ca. 92505-1551, USA.

You may also register through CompuServe (go SWREG) using my

registration ID of 260. Site licensing is also available.

ViewSat makes no warranty of any kind, either express or implied,

including but not limited to implied warranties of merchantability

and fitness for a particular purpose, with respect to this

software and accompanying documentation.

David J. Rasmussen, author of ViewSat, SHALL NOT BE LIABLE FOR

ANY DAMAGES (INCLUDING DAMAGES FOR LOSS OF BUSINESS PROFITS,

BUSINESS INTERRUPTION, LOSS OF BUSINESS INFORMATION, OR OTHER

INCIDENTAL OR CONSEQUENTIAL DAMAGES) ARISING OUT OF THE USE OF OR

INABILITY TO USE ViewSat, OR FOR ANY CLAIM BY ANY OTHER PARTY.

****************************************************************

Introduction:

ViewSat is a program to aid the user in visually locating

Earth orbiting satellites as they pass overhead. ViewSat is menu

driven and supports mouse operation. It calculates and displays

the positions of the satellite at a user selectable interval. It

uses NASA 2-Line element sets for the necessary data to perform

the calculations.

*** Note: Obtaining Satellite Element Sets.

To achieve maximum satellite prediction accuracy, your element

set should be as current as possible. NASA two-line element sets

can be found on the Rancho Palos Verdes Bulletin Board System

(RPV BBS), (310) 541-7299, operated by Dave Ransom, Jr. These

files are located in file area #4, are of the form NASAxxx.ZIP

and are updated approximately twice weekly in the ZIP compression

format. ViewSat is also compatible with elements provided by Ted

Molczan on the Canadian Space Society bulletin board. These

elements are updated less often, but also appear in compressed

format in file area #4 of the RPV BBS.  Their file names are of

the form N2L-XXX.ZIP.

Commands:

The software's commands are listed in their order of

appearance on the display screen, from left to right, with the

appropriate sub-menus as an indented sub-list below each menu

item. Whenever a user entry is required, the keys are shown in

angle brackets and represent the available values or commands to

be entered. For example, Function key 2 is listed as <F2> and the

R key is listed as <R>. The top menu bar commands may be selected

by pressing <Alt> and the first letter of the menu desired. Other

menus and commands in ViewSat may be selected by either using

"Hot Keys", that is, the character that is highlighted in the

command or by using the arrow keys and moving the reverse video

bar to the desired command and pressing <Enter>. The available

"Hot Keys" will be shown in this manual in angle brackets. To

exit out of any command or menu, use the <Esc> key. This will

allow you to "back out", one command at a time, all the way to

the QUIT menu. Mouse support is also built into the program with

the left button used as the <Enter> and the right button user as

the <Esc> key. Moving the mouse with the left button pressed will

"drag" the reverse video bar to the desired menu entry.

Screen Layouts:

The top line contains the program name, version number, date

of that version, copyright information and the time (either local

or UTC). The next line displays the current active satellite name

and element set filename. The fourth line is the "Main Menu Bar".

The bottom line contains helpful information about current

possible options. The following is a sample of the display screen

with the ABOUT screen displayed:

ViewSat v1.08   12/08/92 (c) 1992 David J. Rasmussen            18:08:57 loc

Satellite: Mir                     Element file: JSCEL103.TXT

������������������������������������������������������������������������������ͻ

�  About   Run   Setup   Utilities   Quit                                      �

������������������������������������������������������������������������������Ķ

�       ��������������������������������������������������������������ͻ       �

�       �                                                              �       �

�       �                   ViewSat v1.08  12/08/92                    �       �

�       �                                                              �       �

�       �                         Unregistered                         �       �

�       �                                                              �       �

�       �                                                              �       �

�       �            Copyright (c) 1992 David J. Rasmussen             �       �

�       �                      all right reserved                      �       �

�       �                                                              �       �

�       �      Please register ViewSat by sending $20.00 (US) to:      �       �

�       �                 D. J. Rasmussen, Consulting                  �       �

�       �                     10374 Palmento Drive                     �       �

�       �                     Riverside, Ca. 92505                     �       �

�       �                                                              �       �

�       ������������������ Press any key to continue �����������������ͼ       �

�                                                                              �

�                                                                              �

������������������������������������������������������������������������������ͼ

LTR-cmd  <Esc>-exit                                     <>-hilite  <�-select

Main Menu Bar

The main menu bar presents several menu options. They may

be selected using the <Alt> key and the first letter of the

command.

About                                            <Alt><A>

This menu selection displays information about the program and

the author, along with registration status. When you register the

program the user's registered name will be displayed in place of

the "Unregistered" line.

*** Note: There is a 30 day evaluation period. After that time

whenever this menu selection is brought up a message will be

displayed to remind you to register your use as soon as possible

(this is refered to as a "nag screen" is shareware circles).

Run                                              <Alt><R>

This menu structure pulls down a sub-menu with the possible

simulation modes. They are as follows:

Print data to Screen

Print data to Disk

Run STS Orbit Plus

Setup                                            <Alt><S>

This menu structure is used to configure the operational

parameters pertaining to the satellite; element set; starting

date, time, length and resolution of simulation; viewing

location; and  data restrictions.

Utilities                                        <Alt><U>

This menu structure pulls down a sub-menu with various useful

disk and system functions. They are:

File maintenance

OS shell

Set Time

Quit                                             <Alt><Q>

This menu item is used to exit the program. Possible

selections are:

Stay

Quit

The Run sub-menu contains the selections which start the

simulation (this is the main function of the program... to run

simulations).

Print data to Screen                             <S>

This starts the computer calculating the positions of the

selected satellite from the viewing location for the time period

with the data restrictions applied and each data point which

passes the restrictions is printed to the screen. The following

is a sample run for the Mir space station:

Viewing location: Riverside, CA, Lat: 33.980, Lon: -117.365

Date and time at start of run: 13 Mar 1992 18:02:00 local

Mir

1 16609U 86 17  A 92069.11824684  .00032647  00000-0  43671-3 0  1341

2 16609  51.6006  29.9062 0016242 159.2562 200.9020 15.57566868346660

Element epoch age at start of run: 4/23:11:43 (days/hr:mn:sc)

Date       Time     Azim   Elev   RA     Dec     Alt     Range   Mag

local    deg    deg   hr:mn   deg     km       km

17 Mar 1992 20:25:00 324.56   8.79 00:21  48.85   394.49  1491.82  3.7

18 Mar 1992 19:29:00 335.65   7.07 22:28  54.93   396.44  1616.16  3.5

19:30:00 346.62  12.17 21:37  65.01   395.30  1291.98  2.7

19:31:00   4.43  17.77 18:29  73.31   394.14  1041.25  1.8

19 Mar 1992 18:33:00 343.37   5.36 20:43  57.49   398.28  1752.88  3.4

18:34:00 355.89   8.72 19:13  64.46   397.19  1501.06  2.7

18:35:00  12.60  11.38 16:37  64.64   396.06  1336.56  2.2

18:36:00  32.28  12.14 14:33  53.41   394.89  1294.08  1.8

18:37:00  51.19  10.50 13:35  37.80   393.70  1385.23  1.8

19 Mar 1992 20:09:00 310.07   8.73 01:03  37.76   394.15  1497.62  4.1

20:10:00 308.53  16.47 01:42  40.83   392.95  1091.43  3.1

The top 10 lines do not scroll and they show information about

the viewing location, starting date and time, the satellite

elements, the age of the element epoch at the start of the run

and column headings for the position data.

*** Note: The age of the element epoch is important because with

many low altitude satellites (or with satellites which make many

orbital maneuvers, such as the space shuttle) the position data

accurracy degrades with time. For the most part elements which

are less than a month old will be accurate to about two minutes

time for the given position.

Each data line contains the following information:

Date (shown on the first line of data for an orbit which passes

the data restrictions).

Time (local or UTC as selected).

Azimuth (compass angle increasing clockwise from north).

Elevation (angle above mean horizon, 90 degrees is straight up).

Right Ascension (hours and minutes measured from the prime celestial

meridian).

Declination (angle above or below the celestial horizon).

Altitude (satellite distance the earth's surface).

Range (linear distance from the viewing location to the satellite).

Magnitude (apparent brightness "guess"-timate, for reference

Jupiter is -2.5 and polaris (the north star) is 2.1).

*** Note: While the data is being printed the user may press the

<Space> key (or the left mouse button) to temporarily stop the

display. Press any key to restart it. Press the <Esc> key to stop

the simulation where it is at.

Print data to Disk                               <D>

This starts the computer calculating the positions of the

selected satellite from the viewing location for the time period

with the data restrictions applied and each data point which

passes the restrictions is printed to a disk file which then can

be viewed or printed to a printer. The data is the same as above.

The filename used will be @VIEWSAT.xxx (the xxx is replaced by a

3 digit number starting with 000. If a file with this extension

already exists, the number will be incremented. If all possible

files already exist the default @VIEWSAT.000 will be used).

Run STS Orbit Plus                               <R>

This is a convenient way to access Dave Ransom's excellent

graphic satellite viewing program (it draws a map projection of

the earth and the ground track of a selected satellite). I think

this is the best graphic program available, so I figured I might

as well make it easy to use both his program and mine. It is

available from the Rancho Palos Verdes BBS (310-541-7299) which

he operates. The file is SOP92xx.PAK in file area #4.

*** Note: see list at the end of this document for other BBS

where tracking programs and element sets are available.

The Setup menu structure is used to configure the operational

parameters pertaining to the satellite; element set; starting

date, time, length and resolution of simulation; viewing

location; and  data restrictions. The structure is an "entry

screen", where the data is directly entered in the highlighted

field. The highlighted field is selected using the arrow keys,

mouse, <Tab> key, or <Enter> key. The data is then entered and

the <Enter> key is pressed to save the data (the <Alt><R> key

will abort the current data entry). The <Esc> key exits this

screen back to the main menu bar and the current entries are

saved as the new defaults.

Some of the fields have associated lists of possible entries

to be picked from. The message "<F2>-pick from list" will appear

on the help line for these fields. The user may then press the

<F2> function key to bring up the pick list. The arrow keys,

<PgUP>, <PgDn>, <Home> and <End> keys or mouse may then be used

to highlight the desired entry then press the <Enter> key to

make the selection. As an alternative, the first few letters of

the desired entry may be typed and the cursor will move to

highlight the next entry in the list which matches the characters

typed. The current matching characters are displayed across the

top of the pick list window. The following is a sample Setup

entry screen with a pick list brought up and the matching

characters displayed:

���������������������������� System Setup ��������������������������ͻ

� Element data file           JSCEL103.TXT                           �

� Current satellite name      Mir                    NORAD #: 21255  �

� Start date of simulation:   03/14/199�������������RIV������������Ŀ�

� Length of run:(dd/hh:mm)        10/00�  Richmond, KY              ��

� Time resolution:(minutes)        1.00�  Richmond, UT              ��

�                                      �  Richmond, VA              ��

� Viewing location name       Riverside�  Rifle, CO                 ��

� Viewing locaton latitude:       33.98�  Rio de Janeiro, Brazil    ��

�               Elevation:       249.1 �> Riverside, CA             ��

� Use local timezone:         Y        �����������������������������ٺ

� Minimum sat. elevation:          5.0   Maximum sun elevation: -6   �

� Maximum sat. magnitude:          6.5   Minimum Mean Motion:    4.0 �

� Print satellites skipped:   N                                      �

�                                                                    �

��������������������������������������������������������������������Ķ

�          enter Location name         <F2>-pick from list           �

��������������������������������������������������������������������ͼ

The first field is the Element data filename. This may be

directly entered or picked from a directory list. When picking

from a list, the user may change directory (or disk) by

highlighting the appropriate item and pressing enter. The new

directory list will then be displayed. Once the desired filename

is highlighted, press <Enter> to load that file. The file is then

searched for the current satellite name and, if found, the new

elements are loaded and displayed.

The next field is the Satellite name. This again may be

directly entered or picked from a list of all the satellite names

in the current element data file. If the name is typed in, a

search is then made for the first occurrance in the element data

file for a satellite name that matches at least the number of

characters entered (ie. if you enter COSM, the first Cosmos entry

will be selected from the data file). If this is not the desired

file, press <F2> and the list of available satellites will be

displayed (it may take a few seconds to load, sort and display

the list, so please be patient). Once the desired satellite name

is highlighted, press <Enter> to load the elements for it. Once a

satellite has been selected, the elements will be loaded and

displayed. The following is a sample element data display:

����������������������������������������������������������������ͻ

� Satellite Name:              Mir                               �

� Satellite NORAD Number:      16609                             �

� Satellite Visual Magnitude: �1.7 (@ 1000 km, 50% illuminated)  �

�                                                                �

� Launch Year:                 1986                              �

� Launch Number:               17                                �

� Launch Piece:                A                                 �

�                                                                �

� Elements Epoch:              09 Mar 1992 @ 02:50:17 UTC        �

� Orbit Number at Epoch:       34666                             �

�                                                                �

� Orbital Inclination:         51.6006�                          �

� RA of Ascending Node:        29.9062�                          �

� Eccentricity:                0.0016242                         �

� Argument of Perigee:         159.2562�                         �

� Mean Anomaly:                200.9020�                         �

� Mean Motion:                 15.57566868 rev/day               �

� BSTAR Drag:                  0.000436710                       �

�                                                                �

������������������� Press any key to continue ������������������ͼ

*** Note: The file TRAKSAT.DOC by Paul E. Traufler contains a

very good description of these classical orbital elements and the

calculations used in this program to display the satellite

positions.

The next field is the NORAD # (number). This again may be

directly entered or picked from a list of all the NORAD numbers

in the current element data file. If the number is typed in, a

search is then made for the first occurrance in the element data

file for a satellite with that number. If this is not the desired

file, press <F2> and the list of available satellites will be

displayed (it may take a few seconds to load, sort and display

the list, so please be patient). Once the desired satellite

number is highlighted, press <Enter> to load the elements for it.

Once a satellite has been selected, the elements will be loaded

and displayed.

The next two fields are the starting date and time of the

simulation. The default value is the system time and date when

ViewSat is started. They may then be changed to any other valid

date (01/01/1600 to 12/31/3999) and time (00:00 to 23:59). They

reflect the time zone selected (either local or UTC).

Then comes the length of run field. This is in days, hours and

minutes from the starting date and time. On my 16Mhz 386 sx it

takes about 1 second per day to calculate and display the

positions for a typical satellite. The initial default is 10

days. I set this field to 00/02:00 (2 hours) and the starting

time to about 30 minutes past sundown, then I select all

satellites in the element data file to find any good satellites

in an evening.

The next field is for time resolution (expressed in minutes).

This is the resolution of the data to be displayed. In other

words, if it is set to 1.0 then each line of data will be in 1

minute increments, or if it is set to 0.5 then each line will be

in 30 second intervals. The initial default is 1 minute.

The next field if the viewing location name. This can be a

city name picked from a valid city file (STSPLUS.CTY is included

with this program and comes from Dave Ranson's STSPLUS program)

or anything the user wishes may be entered (for instance if your

city is not in the city file). This is only used when printed out

in the heading of the simulation data. If the user presses the <F2>

key then a window pops up for the filename of the city file to

use (the default is STSPLUS.CTY) enter a valid file name and

press <Enter> and then the pick list will be brought up to make a

selection from. The current (closest matching) city name is

highlighted, then highlight the new city and press enter. The

longitude, latitude and height (elevation) are then set from the

data in the file.

*** Note: Most heights are zero in the city file. Either the city

file may be edited to change this or the user may enter their

city's elevation (in meters) directly in this program.

If the user can not find a match in the city file and they

entered their own viewing location name then the next three

fields should also be set.

Below is an example of the STSPLUS.CTY file. The format of the

tracking station file is;

City Name         Long.  Lat.  Ht.

(deg)  (deg) (m)

"Riverside, CA",-117.365,33.98,249

"Riverton, WY",-108.385,43.025,0

"Riyadh, Saudi Arabia",46.7833,24.5167,0

"Roanoke, VA",-79.9455,37.2703,0

"Rochester, MN",-92.4675,44.0225,0

"Rochester, NH",-70.975,43.3033,0

The city name can be up to 25 characters long while the

longitude (negative = west), latitude (negative = south), and

height can be up to 10 characters long. A comma or a space MUST

separate the data and the city name MUST be enclosed in double or

single quote marks. The height measurement is the elevation above

mean sea-level and is in meters.

The STSPLUS.CTY file can hold a maximum of 1450 viewing

locations in it. The user can enter city data into the

STSPLUS.CTY file in any order but it would be wise to include the

data in an alphabetical order by states or country.

The next three fields contain the viewing location longitude,

latitude, and height. These are set automatically when a city if

picked from the city file, or they may be directly entered (or

altered) by the user here. They are then saved as the new default

values.

The next field is the "Time from UTC". This is the number of

hours from UTC to the viewing location local time:

PST = UTC - 8.0 hours, MST = -7.0, CST = -6.0, EST = -5.0

PDT = -7.0

*** Note: Remember to change this when "Daylight Savings Time"

begins and ends. Also note that the time change is not taken into

account if you run a simulation which overlaps the start (or end)

of "Daylight Savings Time".

The next field is Yes/No (boolean) type. It requires either a

<Y> or <N> key, or click the left mouse button while the field is

highlighted to change it to the other state.

The field is "Use local timezone". If Yes then all data will be

displayed as local time, if No then the data will be UTC (Universal

time). Note that all calculations are performed using UTC and the

conversion is done (if necessary) just prior to displaying it.

The next field sets the Minimum Satellite Elevation. This is the

elevation angle (angle above the mean horizon) that the satellite

must be above for position data to be printed. I like to use 5.0

degrees (ten degrees is about the height of your closed fist with

you arm outstretched). Valid range is +/- 90 degrees.

The next field is the Maximum Sun Elevation. This is how far

below the horizon the sun must be for visible pass data to be

printed. This represents how dark the sky will be and therefore

how easy it will be to pick out the satellite from the background

sky. For bright satellites (Mir, HST, and the space shuttle) I

use -6 degrees (civil twilight), but for most others I use 12

degrees (nautical twilight). Valid range is +/- 90 degrees.

The next field is the Maximum Satellite Magnitude. The satellite

magnitude calculation uses the sun position, satellite range, and

phase angle (angle between the sun, satellite and observer). If a

satellite's magnitude drops below (brightens) the entered value

during a pass then the entire passes data will be printed. This

helps eliminate poor viewing passes from the data printed. Novice

satellite viewers should look for satellites with maximum magnitude

of 4.5 (experienced viewers can probably find them up to 8th

magnitude depending on "seeing" conditions).

The next field is the Minimum Mean Motion. A satellite with a

Mean Motion of less than 4.0 will probably have a highly eliptical

orbit and/or an altitude of greater than 2000 km and therefore a

visual magnitude of greater than 6. This then is the quickest means

to filter out satellites from the element data file that would be

least likely to be visible.

The next field is another Yes/No field to select whether the

satellite NORAD Number and Name should be printed to the output

file if it is skipped because of it's Mean Motion being less than

the Minimum Mean Motion value entered above. This can be useful if

you expect a selected satellite to make a visible pass and it does

not show up in the output (it may have been skipped due to Mean

Motion too small).

The Utilities menu structure pulls down a sub-menu with

various useful disk and system functions. They are:

The first utility is file maintenance. This brings up a

directory list with all files in the current directory shown,

followed by any sub-directories and any other available drives.

The user may use the arrow keys, <PgUp>, <PgDn>, <Home>, and

<End> keys or a mouse to move the highlight bar around. The user

may then Copy, Delete, Rename, Print, or View the highlighted

file by pressing the associated "hot key" (the first character in

each function, ie. to Copy press <C> key). For Copy the entire

path and filename must be entered. Delete requires verification

before the file is deleted. To Rename just enter the new filename

and extension. Print sends the file directly to the system PRN

device. File View opens a new window with the file displayed in

ASCII (press <Ctrl><H> to view the file in Hexadecimal mode). The

arrow keys, <PgUp>, <PgDn>, <Home>, <End> and the mouse may be

used to move around in the file. <Esc> to go back to the

directory list.

The next utility is "OS Shell". This temporarily shuts down

ViewSat (and save most of it to EMS memory or disk) to allow the

user to run other DOS programs. Since most of the program memory

is saved to EMS memory (if available) or to disk, there should be

plenty of memory to run any program you are currently able to

run. Type EXIT at the DOS prompt to return to ViewSat.

*** Note: Do not try to load TSR programs from the OS Shell as

they may cause your system to "hang" requiring a "cold boot".

However you should be able to run any TSRs which you had

previously loaded.

The last utility allows the user to set the system time. Enter

the time in 24 hour (military) format and press enter when ready

to set the clock to the entered time.

*** Note: THE SYSTEM HARDWARE CLOCK MAY BE RESET ALSO IF THE USER

HAS DOS VERSION 3.3 AND ABOVE. Use some caution with this option.

The Quit menu item is used to exit the program. Possible

selections are:

Stay (this exits the quit menu, staying in ViewSat).

Quit (this exits ViewSat, returing control to DOS).

My thanks go out to the following people who helped me either

directly or indirectly on this program.

David H. Ransom, Jr., of Rancho Palos Verdes, CA. Dave is the

SYSOP of the RPV BBS. I included the city data file from his

program STS Orbit Plus (STSPLUS). I would highly recommend his

programs to any person interested in astrodynamics. Other program

he wrote are: STSOrbit, JPLClock, and AstroClock.

T.S. Kelso, SYSOP of the Celestial BBS where current satellite

data can be downloaded and where the core for this program was

obtained (SGP4SDP4.PAS the satellite position propogation

models). Several other satellite tracking programs are also

available on his BBS along with a vast amount of satellite

information.

Paul Traufler, author of TrakSat, whose documentation I used

in preperation of this documentation. TrakSat is another very

good satellite tracking program that combines text and graphics

output all in one program.

Please feel free to contact me to discuss ViewSat or other

computer problems. I can reached through the following BBS:

RPV BBS (owned and operated by Dave Ransom, Jr)

Rancho Palos Verdes, Ca.

310-541-7299

24 hours, 9600/2400/1200 baud.

Celestial RCP/M (owned and operated by T. S. Kelso)

Fairborn, Ohio

513-427-0674

24 hours, 9600/2400/1200 baud.

I can also be reached by E-Mail on Compuserve at 75176,1775 or

throught the Astronomy and Space Forums on Compuserve.

I call up these BBS about three times a week to check my mail

and do some file transfers. These BBS are geared towards

astronomical interests. The latest version of ViewSat and Dave

Ransom's STSPLUS can be downloaded from these BBS also.

This program is produced by a member of the Association of

Shareware Professionals (ASP). ASP wants to make sure that the

shareware principle works for you. If you are unable to resolve a

shareware-related problem with an ASP member by contacting the

member directly, ASP may be able to help. The ASP Ombudsman can

help you resolve a dispute or problem with an ASP member, but

does not provide technical support for members' products. Please

write to the ASP Ombudsman at 545 Grover Road, Muskegon, MI 49442

or send a CompuServe message via CompuServe Mail to ASP Ombudsman

70007,3536.

The REGISTERED user of ViewSat will receive notification when a

new version becomes available. To obtain any future versions of

ViewSat will require the user to send $5.00 (US) and I will send

you the new version on a disk size you specify.

(NON-REGISTERED users are required to send $20.00 (US). They then

become REGISTERED users.)

*** Note: PLEASE STATE THE DISK SIZE YOU REQUIRE!

The REGISTERED versions of ViewSat are for PERSONAL USE and are

NOT to be used COMMERCIALLY (ie. selling the output for profit).

Custom Versions of the Software:

If you require any modifications or changes to the software,

please include detailed information describing all changes you

are interested in. Customization fees will be based on the extent

of the modifications required to the software and any time

constraints, or they may just be incorperated in the next general

release of the software (at the author's discretion).

The following files should have been included in the archive

file:

ViewSat.EXE   The program.

STSPLSUS.CTY  The city file for tracking stations.

ViewSat.DOC   ViewSat program documentation.

JSCELxxx.TXT  This is the latest orbital element data set for

three bright easily viewed satellites:

Mir Soviet (???) space station,

HST Hubble Space Telescope,

GRO Gamma Ray Observatory.

ORDER.FRM   This file contains ViewSat order forms.

ViewSat                         v1.08

Satellite Viewing Program

ViewSat Order Form (PERSONAL USE ONLY)

----------------------------------------------------------------

MAKE CHECKS PAYABLE TO:

David J. Rasmussen

10374 Palmento Drive

Riverside, Ca. 92505

----------------------------------------------------------------

Qty

____ ViewSat REGISTERED Version     @ $20.00 each     $_________

* Price includes shipping and handling *  Total    $_________

Diskette type (check one):   [ ] 5-1/4" 360K  [ ] 3-1/2" 720K

[ ] 5-1/4" 1.2M  [ ] 3-1/2" 1.44M

Numeric Coprocessor Support:

[ ] Currently have (or plan to get)    [ ] No Numeric Coprocessor

Payment: (check one):

[ ] Check  [ ] Money Order  [ ] Other

Checks and Money Orders must be drawn on U.S. banks in U.S. funds.

** PLEASE PRINT **

Name    _______________________________________________________

Company _______________________________________________________

Address _______________________________________________________

_______________________________________________________

Phone   _____________________  Bus. ___________________________

Comments ______________________________________________________

______________________________________________________